3GPP TSG RAN WG1 #122bis			R1-2507765

Prague, Czech Republic, October 13th – 17th, 2025

Source:	Sharp

Title:	Overview of 6GR air interface

Agenda Item:	11.1

Document for:	Decision

# Background

The chairman has provided guidance on contribution submissions under AI 11.1 as follows [1]:

High level design proposals/principles/target, including scalable 6GR design (e.g., what design is scalable, what design is unscalable), support of minimum spectrum allocation, coverage, MRSS, synchronization signal structure and periodicity, operation of bandwidth/band adaptation, spectrum utilization and aggregation framework, harmonization of TN and NTN, and others (if any). Note: To avoid distributing proposals of a same topic to different sub-agendas, please organize the proposals according to above highlights.

In this contribution, our views on 6GR study are presented according to the guidance above.

# Discussion

# Scalable 6GR design

The following agreements on scalable 6GR design were reached at RAN1#122 meeting [2].

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

In scalable 6GR design, diverse device types with respective levels of capability should be supported with a single technology framework. The technology framework should aim at common 6G radio design which meets mobile broadband services as high priority.

Therefore, for a scalable 6GR, different levels of capability should be considered. In this sense, availability of UE’s capability at the RAN node (6G Node B) is a key factor in designing the scalable 6GR.

Figure 1 shows a concept level diagram showing different levels of scalability of channels by respective granularity level of UE capability. At the first step of UE procedures (e.g., initial cell selection, Msg1 transmission), if the RAN node has no information about the UE capability, common channels should be defined to support all the device types deployed under the network operation. On the other hand, once a capability notification with granularity level 1 has been received from the UE, scalable channel design to support device types by a set of configurations should be available. Scalability should provide benefits to higher tier devices e.g., for eMBB, to serve improved latency or reliability of capacity.

Furthermore, the UE can send another capability notification with granularity level 2, to notify more granular capability. For example, in level 2, the UE can indicate type A1 or A2 if the UE has reported type A in level 1, can indicate type B1, B2, or B3 if the UE has reported type B in level 1 and so on.

Granularity of capability signaling provides benefit for higher tier devices in an early stage of communication like initial access. For example, during 5G, some early indication mechanisms have been supported (e.g., RACH partitioning, Msg3 indication). The early indication mechanism was utilized to support various services and to serve UEs with different capabilities in a single cell.

Therefore, considering the potential benefit and achieving the scalable 6GR design, we have the following proposal.

Proposal 1: Study efficient multi-level capability notifications with different granularity levels for scalable channel design.

Figure 1: Different levels of scalability of channels

At the first step of UE procedures where the RAN node has no information about the UE capability, the RAN node cannot make any assumptions on which device type is trying to access the node. Therefore, 6GR procedures should be commonly applicable to all the device types before capability notification with granularity level 1, all the procedures should be commonly applicable. Thus, our proposal is as follows:

Proposal 2: Study the appropriate timing of 1st level capability notification with the minimum set of contents.

Proposal 3: Common channels should be designed before the timing of 1st level capability notification.

# Coverage

The following agreement on coverage was reached at RAN1#122 meeting [2].

Agreement - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

On the other hand, the conclusion section in RP-252883 was endorsed in the RAN chair note [3] for RAN#109 meeting, including the following sentence.

- Conclusion Following proposals were agreed online and to be endorsed: Proposal 1: For 3GPP internal evaluation, define energy efficiency as a quantitative KPI in 3GPP for both network and device. Proposal 2: For 3GPP internal evaluation, at least empty load and partial load cases should be evaluated in 6GR study. - Evaluate via system level simulations (SLS) - Other evaluation methods are not precluded when relevant Proposal 3: For 3GPP internal evaluation, UE/network power model is to be discussed in RAN1. Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

As in Proposal 4, link budget will be used as the evaluation methodology for coverage when applicable, and the target value will be determined by RAN. Therefore, in our understanding, RAN1 should wait for the exact target provided by RAN.

From RAN1 point of view, one of the major targets for coverage evaluation is to identify potential coverage bottleneck. For this, a complete set of link budgets should be evaluated for each frequency, each device type, and each channel/signal, including both TN and NTN. Potential coverage bottleneck can be identified using the target value to be provided by RAN.

Proposal 4: Link budget should be evaluated for each frequency, each device type, and each channel/signal, including both TN and NTN.

# MRSS

The following agreement on MRSS was reached at RAN1#122 meeting [2].

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

Furthermore, the following agreements were reached on waveform design at RAN1#122 meeting [2].

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Considering that the same waveform is the basis for the 6G study, MRSS for downlink can exploit rate-matching of NR resources as much as possible. Considering that NR carrier is lean design, rate-matching is a suitable technique to share a carrier bandwidth by NR and 6GR for downlink.

Proposal 5: Rate-matching is applicable to share a carrier bandwidth by NR and 6GR for downlink.

On the other hand, for uplink, especially if DFT-s-OFDM is used in 6GR, rate-matching increases complexity or increases PAPR. Considering that a unified procedure is preferred for CP-OFDM and DFT-s-OFDM, avoiding overlap of the NR’s and 6GR’s resources by 6G Node B’s scheduling is preferred.

Proposal 6: Avoid overlap of the NR’s and 6GR’s resources by 6G Node B’s scheduling for uplink.

# Synchronization signals and periodicity

The following agreement on initial access signals and periodicity was reached at RAN1#122 meeting [2].

Agreement - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Synchronization signals are the very first signals to detect when a UE wants to access the network after power-on, and thus, all the device types should be able to detect from anywhere in the coverage area.

In terms of its functionality, synchronization signals should provide course/fine synchronization in the time/frequency domain resource grid, and provide minimum essential system information, including scheduling information for the next step of access procedures.

In NR, synchronization signals are defined as a block called SS/PBCH block which includes PSS, SSS, and PBCH and its DMRS. PSS is to provide course sync., SSS is to provide physical cell ID, PBCH DMRS is used for PBCH channel estimation, and additionally conveys SS/PBCH block index, and PBCH conveys physical layer information and MIB. MIB includes information for scheduling SIB1, barring information, and frame number. Although RAN1 may not be a responsible group for making decisions on what kind of information should be provided, to identify high-level aspects which impacts on the 6GR sync signal structure, RAN1 should assume at least PSS-like signal, SSS-like signal, and PBCH-like signal are also necessary for 6GR.

With the assumption above, high-level aspects which impact on the 6GR sync signal structure can be studied based on comparison with NR. Our view is as follows:

- Sync signal periodicity which impact on coverage

- Common design for diverse device types which impact on coverage and bandwidth

Sync signal periodicity was assumed 20 ms when a UE performs initial cell selection and was repeating 4 times in 80 ms MIB TTI. Therefore, maximum of 4 repetitions can be combined in the UE’s detector and can improve the detection performance. However, if the periodicity becomes longer (e.g., 80 or 160 ms), 4 times repetition will increase the latency and thus, not recommended. Rather than relying on repetition, targeting improvement of one-shot sync signal detection is preferred.

Proposal 7: One-shot detection should be studied for sync signals.

Another aspect is common design. Since sync signal should be available at the 1st step before sending 1st level capability notification, design should be commonly applicable to all the device types, which means the design should be available for the lowest tier device (e.g., IoT).

The conclusion section in RP-252943 was endorsed in the RAN chair note [3] for RAN#109 meeting, including the following sentence.

- Conclusion Proposal 1 and proposal 2 were agreed online are to be endorsed. Proposal 1: 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT Proposal 2: For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR

Therefore, sync signal should be designed to meet the requirements for the lowest tier device, including UEs with single receive/transmit antennas.

Proposal 8: Sync signals should be designed to meet the requirements for the lowest tier device, including UEs with single receive/transmit antennas.

# Bandwidth adaptation

The following agreement on bandwidth adaptation was reached at RAN1#122 meeting [2].

Agreement - Study and identify the lessons learned from NR BWP framework

In 5G NR, the bandwidth part (BWP) plays a fundamental role in defining frequency-domain resources. One key aspect is the definition of baseband processing, such as FFT/IFFT operations, which can be adapted to the size of the active BWP. Another key aspect is numerology switching via the BWP framework. Numerology switching was possible only through BWP switching, as it was tied to the BWP configuration. Furthermore, due to the fundamental independence of PHY-layer configuration among numerologies, each BWP is associated with numerous RRC parameters. There is little cell-specific RRC configuration in 5G NR, and most of it is BWP-specific. However, as stated in the SID, numerology switching will not occur, as multiple numerologies per band were excluded. Therefore, in 6GR, the motivation for BWP switching needs to be revisited.

Proposal 9: The motivation for BWP switching from the UE’s perspective should be clarified in 6G.

# Spectrum aggregation

According to the RAN1 SID on 6G Radio [2], the 5G mid-band site grid should be reusable for 6G deployment, at least for frequencies around 7 GHz. In collocated scenarios, RAN-level aggregation is suitable to enhance efficiency and flexibility. In the past, carrier aggregation was specified and subsequently enhanced. However, although carrier aggregation will remain one of the key study areas in 6GR, it has certain drawbacks in terms of spectral efficiency and UE capability. Some examples are as follows:

- A hard split of frequency resources between carriers reduces frequency diversity gain.

- A hard split of HARQ mechanisms between carriers reduces HARQ gain and increases retransmission latency.

- Dedicated SSBs for each carrier can be redundant in some cases.

Therefore, it is proposed to initiate an RAN1 study on enhanced spectrum aggregation mechanisms to address at least the potential drawbacks of carrier aggregation.

Proposal 10: Initiate a RAN1 study on enhanced spectrum aggregation mechanisms to address at least the potential drawbacks of carrier aggregation.

# Harmonization of TN/NTN

The following agreement on bandwidth adaptation was reached at RAN1#122 meeting [2].

Agreement - For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

In our view, NTN characteristics potentially affect the following technical aspects:

- Larger pathloss compared to TN

- Larger propagation delay compared to TN

- Larger timing error due to the location estimation error and/or GNSS-less operation

- UE side beamforming capability and larger form factor compared to TN

Items 1 affects NTN coverage. Therefore, NTN coverage evaluation should be essentially performed like in TN, to design signals and channels. Item 2 affects scheduling/HARQ timeline. In NR, time domain offset for scheduling and HARQ disabling were exploited to solve the issue. Item 3 affects the TA management and CP length optimization since timing error can be absorbed by the CP if the error is shorter than the CP length. Item 4 is to compensate coverage in higher frequency like in Ka or Ku band. In those frequencies, VSAT with circular antennas or phased arrays are utilized for higher order beamforming. Furthermore, for phased array, HD-FDD is necessary to be supported [4].

Proposal 11: NTN characteristics potentially affect the following technical aspects:

- Larger pathloss compared to TN

- Larger propagation delay compared to TN

- Larger timing error due to the location estimation error and/or GNSS-less operation

- UE side beamforming capability and larger form factor compared to TN

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Study efficient multi-level capability notifications with different granularity levels for scalable channel design.

Proposal 2: Study the appropriate timing of 1st level capability notification with the minimum set of contents.

Proposal 3: Common channels should be designed before the timing of 1st level capability notification.

Proposal 4: Link budget should be evaluated for each frequency, each device type, and each channel/signal, including both TN and NTN.

Proposal 5: Rate-matching is applicable to share a carrier bandwidth by NR and 6GR for downlink.

Proposal 6: Avoid overlap of the NR’s and 6GR’s resources by 6G Node B’s scheduling for uplink.

Proposal 7: One-shot detection should be studied for sync signals.

Proposal 8: Sync signals should be designed to meet the requirements for the lowest tier device, including UEs with single receive/transmit antennas.

Proposal 9: The motivation for BWP switching from the UE’s perspective should be clarified in 6G.

Proposal 10: Initiate a RAN1 study on enhanced spectrum aggregation mechanisms to address at least the potential drawbacks of carrier aggregation.

Proposal 11: NTN characteristics potentially affect the following technical aspects:

- Larger pathloss compared to TN

- Larger propagation delay compared to TN

- Larger timing error due to the location estimation error and/or GNSS-less operation

- UE side beamforming capability and larger form factor compared to TN

# References

- R1-2506700, Draft Agenda of RAN1#122bis meeting, October 2025

- RAN1 Chairman note at RAN1#122 meeting, August 2025

- Draft_MeetingReport_RAN_109_250918_eom.rtf, September 2025

- R1-2505983, August 2025