3GPP TSG RAN WG1 #123	R1-2508335

Dallas, USA, 17 – 21 November 2025

Agenda item:		11.2

Source:	Nokia

Title:	On Evaluation Assumptions for Study of 6G Radio Air Interface

WI code:	FS_6G_Radio

Release:	Rel-20

Document for:		Discussion and Decision

# Introduction

One key task of the 6G SI is to “Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study”. To do that, basic evaluation methodology and assumptions need to be agreed upon.

This document further discusses the evaluation assumptions for the 6G Radio study and focuses on the following aspects:

- Consideration of UE and BS-specific system level evaluation parameters for Terrestrial Networks (TN)

- Traffic models to be used in 6GR Evaluation study

- Initial considerations on Link budget evaluation

- Assumptions for NTN.

In addition to this paper, we are also consigning a paper on Fixed Wireless Access Scenarios [1] that is submitted to the same agenda item.

# Discussion

## BS-related parameters

### BS antenna modelling

At RAN1#122bis meeting an extensive discussion of possible BS antenna configurations took place. For some of the bands, like around 700 MHz and around 2 GHz, the baseline and optional configurations were agreed, however, for the rest of the bands there is still a wide list of potential configurations. A need for higher level of alignment between the companies is beneficial and was also emphasized by the chair during the online discussions. Selection of a limited number of baseline antenna configurations (one in existing band or max two for the new bands) will allow direct comparison of network performance across the companies and bands.

Below, in Table 1, we summarize the BS antenna configurations per band based on RAN1#122bis agreements and feature lead proposals (for around 15 GHz carrier). Additionally, the last column contains physical antenna sizes.

Table 2-1: Summary of agreed and considered antenna mode configuration in different frequency bands.

| Frequency, around, MHz | λ, m | BS antenna model | Configuration | AEs | TXRUs | Sub-panels | Vertical ratio | dH, dV, λ | Antenna size, m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 700.0 | 0.428 | Outdoor (baseline) | (8, 2, 2, 1, 1; 1, 2) | 32 | 4 | 1 | 1:8. | (0.5, 0.5) | (1.713 x 0.428) |
| 700.0 | 0.428 | Outdoor (optional) | (8, 4, 2, 1, 1; x, y) | 64 |  | 1 |  | (0.5, 0.5) | (1.713 x 0.857) |
| 2000.0 | 0.150 | Indoor, Combination 1(Optional) | (2, 2, 2, 1, 1; 1, 2) | 8 | 4 | 1 | 1:2. | (0.5, 0.5) | (0.15 x 0.15) |
| 2000.0 | 0.150 | Indoor, Combination 2 (Baseline) | (4, 4, 2, 1, 1; 1, 4) | 32 | 8 | 1 | 1:4. | (0.5, 0.5) | (0.3 x 0.3) |
| 2000.0 | 0.150 | Outdoor, Combination 1(Optional) |  | 32 | 4 |  |  | (0.5, 0.8) |  |
| 2000.0 | 0.150 | Outdoor, Combination 2 (Baseline) | (12, 8, 2, 1, 1; 4, 8) | 192 | 64 | 1 | 1:3. | (0.5, 0.5) | (0.899 x 0.6) |
| 4000.0 | 0.075 | Indoor, Combination 1 | (4, 4, 2, 1, 1; 4, 4) | 32 | 32 | 1 | 1:1. | (0.5, 0.5) | (0.15 x 0.15) |
| 4000.0 | 0.075 | Indoor, Combination 2 | (8, 8, 2, 1, 1; 2, 8) | 128 | 32 | 1 | 1:4. | (0.5, 0.5) | (0.3 x 0.3) |
| 4000.0 | 0.075 | Outdoor, Combination 1 | (12, 8, 2, 1, 1; 4, 8) | 192 | 64 | 1 | 1:3. | (0.5, 0.8) | (0.72 x 0.3) |
| 4000.0 | 0.075 | Outdoor, Combination 2 | (16, 8, 2, 1, 1; 4, 8) | 256 | 64 | 1 | 1:4. | (0.5, 0.8) | (0.959 x 0.3) |
| 4000.0 | 0.075 | Outdoor, Combination 3 | (16, 16, 2, 1, 1; 4, 16) | 512 | 128 | 1 | 1:4. | (0.5, 0.5) | (0.6 x 0.6) |
| 7000.0 | 0.043 | Indoor, Combination 1 | (4, 8, 2, 1, 1; 2, 8) | 64 | 32 | 1 | 1:2. | (0.5, 0.5) | (0.086 x 0.171) |
| 7000.0 | 0.043 | Indoor, Combination 2 | (16, 8, 2, 1, 1; 4, 8) | 256 | 64 | 1 | 1:4. | (0.5, 0.5) | (0.343 x 0.171) |
| 7000.0 | 0.043 | Indoor, Combination 3 | (16, 16, 2, 1, 1; 8, 8) | 512 | 128 | 1 | 1:2. | (0.5, 0.5) | (0.343 x 0.343) |
| 7000.0 | 0.043 | Outdoor, Combination 1 | TBD | 768 | 128 | 1 |  | (0.5, 0.8) |  |
| 7000.0 | 0.043 | Outdoor, Combination 2 | (32, 16, 2, 1, 1; 8, 16) | 1024 | 256 | 1 | 1:4. | (0.5, 0.8) | (1.096 x 0.343) |
| 7000.0 | 0.043 | Outdoor, Combination 3 | TBD | 1536 | 256 | 1 | 1:6. | (0.5, 0.8) |  |
| 7000.0 | 0.043 | Outdoor, Combination 4 | (32, 32, 2, 1, 1; 8, 16) | 2048 | 256 | 1 | 1:4. | (0.5, 0.5) | (0.685 x 0.685) |
| 7000.0 | 0.043 | Outdoor, Combination 5 | (64, 16, 2, 1, 1; 16, 16) | 2048 | 512 | 1 | 1:4. | (0.5, 0.5) | (1.37 x 0.343) |
| 15000.0 | 0.020 | Indoor, Combination 1 (no agreement) | (8, 8, 2, 1, 1; 2, 8) | 128 | 32 | 1 | 1:4. | (0.5, 0.5) | (0.08 x 0.08) |
| 15000.0 | 0.020 | Indoor, Combination 2 (no agreement) | (8, 16, 2, 1, 1; 8, 16) | 256 | 256 | 1 | 1:1. | (0.5, 0.5) | (0.08 x 0.16) |
| 15000.0 | 0.020 | Indoor, Combination 3 (no agreement) | (16, 16, 2, 1, 1; 8, 16) | 512 | 256 | 1 | 1:2. | (0.5, 0.5) | (0.16 x 0.16) |
| 15000.0 | 0.020 | Outdoor, Combination 1 (no agreement) | (48, 16, 2, 1, 1; 8, 16) | 1536 | 256 | 1 | 1:6. | (0.5, 0.8) | (0.767 x 0.16) |
| 15000.0 | 0.020 | Outdoor, Combination 2 (no agreement) | (64, 16, 2, 1, 1; 16, 16) | 2048 | 512 | 1 | 1:4. | (0.5, 0.8) | (1.023 x 0.16) |
| 30000.0 | 0.010 | Indoor, Combination 1 | (4, 4, 2, 2, 2; 1, 1) | 128 | 2 | 4 | 1:4. | (0.5, 0.5) | (0.04 x 0.04) |
| 30000.0 | 0.010 | Indoor, Combination 2 | (8, 8, 2, 2, 2; 1, 1) | 512 | 2 | 4 | 1:8. | (0.5, 0.5) | (0.08 x 0.08) |
| 30000.0 | 0.010 | Indoor, Combination 3 | (16, 8, 2, 2, 2; 1, 1) | 1024 | 2 | 4 | 1:16. | (0.5, 0.5) | (0.16 x 0.08) |
| 30000.0 | 0.010 | Indoor, Combination 4 | (24, 16, 2, 1, 1; 1, 1) | 768 | 2 | 1 | 1:24. | (0.5, 0.5) | (0.12 x 0.08) |
| 30000.0 | 0.010 | Outdoor, Combination 1 | (16, 8, 2, 4, 2; 1, 1) | 2048 | 2 | 8 | 1:16. | (0.5, 0.5) | (0.32 x 0.08) |
| 30000.0 | 0.010 | Outdoor, Combination 2 | (16, 8, 2, 4, 4; 1, 1) | 4096 | 2 | 16 | 1:16. | (0.5, 0.5) | (0.32 x 0.16) |
| 30000.0 | 0.010 | Outdoor, Combination 3 | (16, 16, 2, 2, 1; 1, 1) | 1024 | 2 | 2 | 1:16. | (0.5, 0.5) | (0.16 x 0.08) |

Firstly, the agreements on 700 MHz and 2GHz still have optional configurations that do not have fully defined parameters. From the discussions it is hard to trace whether these parameters were left open on purpose (i.e., to allow for some additional flexibility) or not. Therefore, if these configurations still need to be clarified we can consider the following options:

- For around 700.0 GHz, for the optional configuration, the number of TXRUs was not captured

  - It can be selected as 16, corresponding to (M, N, P, Mg, Ng;Mp, Np) = (8, 4, 2, 1, 1, 2, 4)

- For around 2 GHz, Outdoor, Combination 1(Optional) only the number of AEs = 32 and the number of TXRUs = 4 are defined; however, the configuration itself was not captured.

  - 32 AEs in total corresponds to 16 AEs per polarization. There are only two practical ways to distribute those into a rectangular array: either 4x4 or 8x2.

    - For the outdoor deployments, the rectangular shape of the antenna is more typical (more antennas in vertical direction).

      - The only possible configuration in this case will be (8, 2, 2, 1, 1; 1, 2)

      - However, this configuration has only two AE columns. It is not representative both due to the limited coverage in horizontal direction and narrow antenna dimensions (1x0.15) m.

    - If we consider squarer configuration (4, 4, 2, 1, 1; 1, 2), then the problem is that the number of AE columns is less than number of TXRUs.

    - Hence both options seems to be limiting.

  - One resolution could be to increase the number of TXRUs to 8 and the number of AEs to 64, i.e., to consider configuration (8, 4, 2, 1, 1; 1, 4). In this case, the antenna size will be (0.48 x 0.3) m.

Some of the BS antenna combinations at around 700 MHz and 2GHz have parameters that are not fully captured. Moreover, for around 2 GHz, Optional outdoor antenna Combination 1 with 32 AEs and only 4 TXRUs does not look very representative.

If found to be needed, RAN1 can clarify further optional antenna configurations at 700 MHz and 2GHz as follows:

  - 700 MHz, Optional configuration: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1, 2, 4), i.e., with 16 TXRUs

  - 2 GHz: Outdoor, Combination 1(Optional): consider increasing the number AEs from 32 to 64, and number of TXRUs from 4 to 8, and assume (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4).

For around 4 GHz carrier, out of the options that were listed in the agreement from RAN1#122bis meeting:

- For indoors, out of two combinations we see Indoor, Combination 2: (8, 8, 2, 1, 1; 2, 8) as a baseline. It is also aligned in dimensions (around 0.3 x 0.3 m) that matches well the agreed baseline at 2GHz.

- For outdoors, out of three options, our preference for the baseline is Combination 1: (12, 8, 2, 1, 1; 4, 8). Combination 2 is rather close to it, and hence looks like a reasonable assumption to us. However, Combination 3 is not preferred due to its non-typical shape (0.6 x 0.6 m) and also extreme number of AEs and TXRUs for this frequency.

For around 4 GHz carrier, we propose to consider the following BS antenna configurations as a baseline:

  - For Indoors, Configuration 2: (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 8), 128 AEs, 32 TXRUs.

  - For Outdoors, Configuration 1: (M, N, P, Mg, Ng; Mp, Np) = (12, 8, 2, 1, 1; 4, 8), 192 AEs, 64 TXRUs.

For around 7 GHz, even wider range of five options were listed at in the agreement from RAN1#122bis for outdoor deployments. Since 7 GHz is one of the key bands for 6G, careful study is required to ensure that the coverage and other KPIs can match or exceed the ones that are observed in 5G, e.g., around 4 GHz carrier. The larger panel may not guarantee better performance, especially for users at the cell edge, because such BSs may need to transmit with lower total Tx power due to limitations induced by the EIRP requirements. For that reason, we think that the panels sizes with 2048 antenna elements should not be considered at this frequency.

Too large BS antenna panels of 2048 antenna element, if considered at around 7 GHz frequency, may need to use reduced Tx power due to EIRP limits insufficient to provide good coverage for cell edge users.

Out of three other options at 7GHz, two options (Outdoor Combination 1 and Outdoor Combination 3) have their configurations as TBA. This was because the ratio of TXRUs to antenna elements for these configurations is 1:6 in vertical direction. A potential concern is that for such ratios, the beams are becoming too narrow in the vertical direction. The comparison of the half power beam width for 6 and 4 AEs in vertical direction is shown below in Figure . HPBW is ≈ 10.6° for 6 and ≈ 15.9° for 4 AEs per TXRU. Such narrow beamwidth may potentially cause some coverage issues at the cell edge, but on the other hand could limit interference in the other deployments.

Figure 1: Vertical cut of the boresight beam for 6-array (left) and 4-array (right) vertical sub-arrays @7 GHz carrier.

One way to avoid 1:6 ratio is to consider Outdoor Combination 2, 768 AEs with 256 TXRUs instead of 128. In this case the combination will be (24, 16, 2, 1, 1; 8, 16), and the vertical AE-TRXU ratio will be 1:3. Otherwise, we also support Outdoor combination 2 as another candidate for the default option.

For indoor deployments at 7 GHz, the most feasible BS antenna size, in our view, is Combination 3 with 512 antenna elements, that can be selected as default. This combination also matches the dimensions of the indoor BS antennas at the other frequencies.

For the BS antenna around 7GHz, to consider as baseline:

  - For outdoor, either modified Combination 1, 768 AEs with 256 TXRUs instead of 128 with configurations (M, N, P, Mg, Ng; Mp, Np) = (24, 16, 2, 1, 1; 8, 16) or already listed Configuration 2 (M, N, P, Mg, Ng; Mp, Np) = (32, 16, 2, 1, 1; 8, 16), 1024 AEs, 256 TXRUs.

  - For indoor deployments, Combination 3, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs.

15 GHz is another new 6G carrier frequency that also will require an accurate study. For this frequency, no agreement was achieved at RAN1#122bis meeting. Even though this carrier lies in between the 7 GHz where digital or hybrid beamforming (HBF) is expected to be used and 30 GHz FR2 bands with analogue beamforming (ABF), the ABF solutions at around 15 GHz could be usable for DL only. On the other hand, HBD or DBF at these frequencies is preferred from system point of view to support a larger number of users.

As of now, the EIRP limitation for 10-15 GHz is in the range of ~75…85dBm, so the products at around 15 GHz may not meet the existing macro site grid requirements. This would make such solutions usable only in hot-sport outdoor deployments which will limit the number of such deployments.

For indoor, deployments, we do not think that antenna size, less than for 7GHz would make sense, therefore a similar configuration with 512 AEs and 128 TXRUs can be taken as a baseline.

Finally, the proposals that were listed at RAN1#122bis assume only 16 antenna elements in horizonal direction, which will result in narrow BS antennas with the width around 16 cm. Thus, we think that 32 antenna elements in the horizontal dimension should be considred as an option.

As a result, we propose to consider the following configurations for 15 GHz for the further evaluations:

For the BS antenna around 15 GHz to consider:

  - For indoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs, as baseline.

  - For outdoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 1, 32), 3072 AEs, 64 TXRUs and (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 2, 32), 3072 AEs, 128 TXRUs.

### BS TX power

We can use several existing references as the starting point for the gNB TX power discussion:

- Table A.2.1-1: System level evaluation assumptions for Indoor hotspot, Dense urban, Rural, and Urban macro in TR 38.802 (Study on New Radio Access Technology Physical Layer Aspects (Release 14))

  - Simulation bandwidth: 20MHz per CC below 6GHz and 80 MHz per CC above 6GHz

  - Indoor hotspot:

    - Below 6GHz: 24dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm

    - Above 6GHz: 23 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 23dBm EIRP should not exceed 58 dBm(*)

      - (*) See Appendix in R1-164383 and R1-167533 for the derivation of maximum allowed EIRP. EIRP limit is only used for evaluation purpose in RAN1.

  - Dense urban:

    - Macro layer:

      - Below 6GHz: 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 44 dBm

      - Above 6GHz: 40 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 40 dBm

    - Micro layer:

      - 4 GHz: 33dBm for 20MHz system bandwidth

      - Above 6GHz: 33 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 33 dBm.

    - EIRP should not exceed 73 dBm and 68 dBm for the macro and micro layers respectively(*)

  - Rural:

    - 49 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm

  - Urban macro:

    - Below 6GHz: 49 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm

    - Above 6GHz: 43 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 43dBm

    - EIRP should not exceed 78 dBm (*)

- Typical Power Spectrum densities were agreed in TR 38.830, Table A.3: Link budget template:

  - Power Spectrum Density (dBm/MHz)

    - Note: For FR1 downlink, (3b) should satisfy the following:

      - For 4GHz frequency, 24 and 33

        - This translates to 37 dBm per 20 MHz / 44 dBm per 100 MHz and 46 dBm per 20 MHz / 53 dBm per 100 MHz, respectively

      - For 2.6 GHz frequency, 33

        - This translates to 46 dBm per 20 MHz/46 dBm per 100 MHz

      - For 700MH and 2GHz frequency, 36

        - This translates to 49 dBm per 20 MHz/56 dBm per 100 MHz

    - Note: For FR2 downlink, the following should be satisfied:

      - 40 dBm for 100 MHz Urban scenario,

      - 23 dBm for 100 MHz Indoor scenario.

As mentioned above, Appendix in R1-164383 can be checked for the derivation of maximum allowed EIRP. The values listed there are:

- 61 dBm, 79 dBm and 88 dBm for indoor hotspot, dense urban (micro layer) and urban macro scenario, respectively.

Additionally, maximum EIRP values are also provided by regulators, e.g., FCC in US [1], [2], [3].

The upper bound of EIRP for BS antenna can be estimated from the following equation:

EIRP (dBm) = Total transmit power (dBm) + Total antenna gain (dB) – Losses (dB)

In the assumption of ideal transmitter implementation (e.g., no correction factors to account for various non-idealities impacting the actual gain of antenna gain components),

Total antenna gain (dB) = 10*LOG10(#AEs) + Gain of AE (dBi),

where following gNB antenna model from 38.901 the Gain of AE = 8 dBi, and we also assume Losses to be 0 dB.

In Figure 3, we demonstrate how the EIRP defined for 100 MHz BW scaling with the number of BS antenna for a few Total transmit powers of 42, 46 and 49 dBm per 20 MHz BW. Horizontal lines also show the typical max EIRP values.

Figure 2: EIRP upper bound vs Number of BS antenna elements for several Tx power.

For larger antenna panel sizes, the EIRP requirements may be violated. There are several ways to address this issue:

- Define single total Tx power per 20 MHz (or PSD per MHz) and limit transmitting power, e.g., max value for 100 MHz BW (e.g., 50 dBm per 100 MHz at 7 GHz)

- Define single total Tx power per 20 MHz (or PSD per MHz) and add max EIRP limit per 100 MHz (e.g., 88 dBm).

  - This kind of approach was already used in the past.

- Define different total Tx powers in a more specific way, i.e., per band and deployment scenario considering max antenna panel size (e.g., 46 dBm at 7 GHz).

Even though the approach with a single value of total transmit power seems to be the most attractive from the parameter definition point of view, it will be hard to find a single value matching all panel sizes, frequencies, and scenarios.

For the indoor hotspots, where very large antenna panels are not planned to be used, the former TX power from TR 38.802 can be followed:

For the indoor deployments, for the total transmit power per BS, RAN1 to consider below 15 GHz: 24 dBm per 20 MHz, EIRP should not exceed 61 dBm, above 15 GHz: 23 dBm per 100 MHz, EIRP should not exceed 58 dBm.

For the outdoor scenarios, some differentiation can be made in between Dense urban, and Urban Macro/Rural/Sub-urban macro scenarios:

For the outdoor deployments, macro layer, for the total transmit power per BS, RAN1 to consider:

  - For around 700 and 2GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise.

  - For around 4 GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm.

  - For around 7 GHz and around 15 GHz: 44 dBm per 20 MHz for dense urban and 46 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm.

  - For around 30 GHz: 33 dBm per 20 MHz, EIRP should not exceed 75 dBm.

## UE-related parameters

### Hand-held UE modelling

One of the key new aspects that should provide more realistic UE modelling in 6G is the introduction of a new directional antenna model in Rel-19 version of TR 38.901. The new UE antenna model resolves the following long-standing unrealistic assumptions:

- Realistic device dimensions (15x7cm) instead of half-wavelength spacing in between UE antennas,

- Directive UE antenna radiation patterns instead of isotropic

- New UE antenna modelling instead of unrealistic assumption of cross-polarized UE antenna

- Optional possibility to model imbalance in between antennas

- Optional possibility to model antenna blockage by hand and head.

Figure 3 demonstrates the new UE antenna model candidate locations.

Figure 3: Handheld UT antenna ports and polarization directions (Figure 7.3-7 from TR 38.901).

Even though for above 1 GHz frequencies, any of 8 candidate antenna locations can be used, in the case of UE equipped with 4 antennas, candidate antennas locations (1,3,5,7) were used. In general, this assumption is not limiting, and antenna location (2,4,6,8) can be used as well. However, we prefer to use already calibrated locations (1,3,5,7) as default.

Additionally, in TR 38.901 Table 7.6.14.2-2, the following note was made on modelling Spatial Non-Stationarity (SNS, i.e., hand grip and/or head blockage): NOTE 1: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8. Again, this does not mean that other UE antenna candidate location cannot be simulated. However, if one needs to simulate two-antenna UE with SNS, (4, 8) antenna ports are preferred.

New Rel-19 UE antenna model from TR 38.901 considerably improves the realism of UE modelling. For UE with four antennas, the candidate locations (1,3,5,7) were calibrated, and for UE with two antennas, e.g., below 1 MHz, only ports (8, 4) have UE spatial non-stationarity model (blockage) defined.

RAN1 should use new directive handheld UE antenna model described in section 7.3 of Rel-19 version of TR 38.901 by default.

For handheld UE antenna modelling, RAN1 to select the following default configurations frequency:

  - For around 700 MHz: 1/2T2R directional antennas, (4, 8) as described in section 7.3 in TR 38.901,

  - For around 2 GHz, around 4GHz: 2T4R directional antennas, (1, 3, 5, 7) as described in section 7.3 in TR38.901,

  - For around 7 GHz and around 15 GHz: 4T4/8R directional antennas, (1, 3, 5, 7) for 4 and (1,2,3,4,5,6,7,8) for 8 as described in section 7.3 in TR38.901.

For the handheld device transmit power, in general, all existing power classes can be considered, i.e. PC1(23 dBm), PC2 (26 dBm) and PC1.5 (29 dBm). However, for the new bands 23 dBm Tx power may not be sufficient to achieve the same coverage as 5G deployment grids.

For handheld UE modelling, RAN1 to consider the following UE transmit power:

  - For below 7 GHz: 23, 26, 29 dBm

  - For 7 GHz and above: 26, 29 dBm

### CPE modelling

Fixed Wireless Access (FWA) has become a key component for delivering high speed internet without the need for wired infrastructure, making it a cost-effective solution in a variety of deployment scenarios including suburban, rural and urban areas. To ensure the long-term success of FWA deployments, it is essential that ongoing 3GPP standards development consider parameters and deployment scenarios that help all markets (as given by [1]). The fixed nature of FWA devices and its radio and traffic characteristics can be exploited to optimize the FWA air interface in 6G.

Potential FWA customers with CPE locations in poor coverage areas (especially at higher bands) can be part of a “pre-qualification” methodology (see [1] for further details) which is used in many markets today and should be modelled in system level simulation performance evaluations of FWA in 3GPP. That is, when considering service to poor coverage areas at higher frequency bands, the CPE may be (i) reassigned to use lower bands, or (ii) moved outdoor or its antenna array is moved outdoors (with better coverage and enjoying higher EIRP subject to regulatory limits), or (iii) reject FWA service at the proposed location.

Regarding CPE transmit power levels it is worth repeating from [1]:

“CPE power classes like PC1 or PC1.5 are common, and higher power may be possible, up to regulatory limits. Regardless, a given transmit power can only be used where it doesn’t exceed the regulatory max average EIRP limits (including FCC band limits and user exposure level limits) and where it meets out of band emissions.  Note: User exposure is measured differently for CPE (Mobiles) and Hand-held UE (Portables) due to differences in how those devices are used and typical distances from the user. International regulatory bodies as well as the FCC conclude that Portables are used within 20 cm of a user and Mobiles are used with a minimum gap of 20 cm from a user which permits higher mobile EIRPs compared to portables for meeting required exposure level regulations for each device type. Lower power classes can be used in deployments where high power is not needed.”

Proposed modeling of CPE antennas as discussed in [1] is summarized below in Table 2-2 for FR1 and ‘FR3’. Similar details for FR2 (besides proposing [45-51] dBm outdoor CPE EIRP) can be provided in future 3GPP FWA modeling discussions.

Table 3: CPE antenna configurations for FR1 and ‘FR3’

|  | Antenna Element Gain (dBi) per TXRU |  |  |  |
| --- | --- | --- | --- | --- |
| Carrier Freq | Indoor CPE |  | Outdoor CPE or CPE array |  |
| (GHz) | {360°, 22.5°} OMNI DIR | {90°,90°} DIR | {65°, 50°} DIR | {32°, 32°} DIR |
| 2 | 5 | 5 | 9 |  |
| 3.5 | 5 | 5 | - | 14 |
| 7 | 5 | 5 | - | 14 |
| DIR - CPE directs it antenna elements to point at the serving gNB |  |  | HPBW {horizontal, vertical} |  |
| OMNI Directional - CPE does not direct its antenna elements to point at the serving gNB |  |  |  |  |
| Coherent or non-coherent precoding may be applied which may result in additional directionality |  |  |  |  |
| Antenna arrays: e.g., 4TX, 6TX, 8TX, 16TX can be modeled w. proposed TXRU antenna element gains given above |  |  |  |  |
| PC_MAX is chosen to meet the regulatory max average band EIRP limit accounting for total array gain and TDD Duty Cycle |  |  |  |  |

Note Table 1 in [1]  gives the proposed BS antenna heights, inter-site distances, device deployment criteria, CPE antenna height, transmit power and antenna element patterns to be used for modeling FR1, FR2, and ‘FR3’ FWA deployments. For ease of comparison to previous modeling results it may be useful to modify the inter-site distance for the UMa scenario to cover both 500m and 700m although the pre-qualification will likely make 700m cell edge performance acceptable.

Additionally, we think that the CPE modelling will benefit from considering the realistic device dimensions and candidate antenna locations that were introduced in Rel-19 version of TR 38.901 (see Figure 4 below).

Figure 4: CPE UT antenna placement candidate locations relative to centre of device (side view) (Figure 7.3-6 from TR 38.901).

For CPE antenna modelling, RAN1 to use CPE antenna placement from section 7.3 in TR 38.901:

  - 2/4T4R with antenna locations (1, 3, 5, 7)

  - 6/8T6/8R with antenna locations (1, 2, 6, 7, 8, 9) for 6 and (1,2,3,4,5,6,7,8) for 8.

For CPE antenna modelling, RAN1 to select CPE antenna configurations per frequency and device type (outdoor/indoor CPE) as described in Table 3 above.

For the CPE transmit power, in general, all existing power classes can be considered, e.g., PC1(31 dBm), PC1.5 (29 dBm) and PC2 (26 dBm) -- as well as new or modified power classes defined in 6G -- and also up to the maximum average band EIRP set by the regulator (e.g. FCC in U.S.) and that meets required MPE and that accounts for TDD Duty cycle and any other maximum average EIRP limit mitigations (e.g., minimum compliance distances) for both indoor and outdoor CPEs.

For CPE modelling, RAN1 to support 23, 26, 29, 31 dBm power classes and up to the max conducted power that meets the maximum average band EIRP limit (subject to regulatory limitations and accounting for maximum average EIRP limit mitigations for both indoor and outdoor CPEs).

## Interference modelling in SLS

It is well-known from many link and system-level simulation studies that one of the performance-determining effects is the receiver’s ability to correctly estimate the experienced interference. As an example, a multi-antenna MMSE-IRC receiver ability to suppress interfering signals depends on how well the spatial interference covariance matrix is estimated. From communications theory (see e.g. Jack Winters Optimum Combining article from 1984) it is well-known that a receiver with M antenna has M-1 degrees of freedom that can be used either for suppressing interferers or for diversity to suppress fading variations. Thus, a UE Rx with 4 antennas may in principle be able to suppress up to four interfering signals, and a gNB Rx with tens to hundreds of antennas can suppress many more. However, this is all assuming perfect knowledge of spatial covariance interference matrix. For many 3GPP simulation studies, realistic interference estimation has therefore been default assumption as this is one of performance-determining effects. Going all the way back to LTE NAICS studies (see e.g. 3GPP TR 36.866) the importance of realistic spatial interference covariance estimation was therefore recognized for both demodulation (i.e. data channel reception) and CSI estimation, where the sample covariance by Wishart model was adopted. The Wishart model is well-proven and has been widely used since these times, both for cases with plain MMSE-IRC receivers, and also for cases with more advanced interference cancellation receivers such as e.g. NAICS and more.

6G simulations shall include realistic spatial interference covariance estimation for multi-antenna receivers. For system-level simulations, we suggest adopting the well-known Wishart model.

## Traffic models

At RAN1#122bis the following conclusion was reached regarding the reuse of the existing traffic models:

Conclusion The following existing traffic models could be used for 6GR performance evaluations, - Full buffer - FTP Model 1 (in TR 36.814) - FTP Model 3 (in TR 36.872) - XR Traffic models (in TR 38.838) - VoIP model (as in TR 36.814) - Instant message (as in TR 38.840) - Note that which model(s) will be used can be further decided when performing simulations in each individual topic.

On the fundamentals of the traffic models, it is important to keep in mind that such models shall not include modelling of any of the RAN mechanisms. The traffic model shall only include what the application traffic source generates in terms of data for transmission, potentially including Core Network (CN) effects such as the CN latency from the traffic source to the RAN (for DL traffic). And the equivalent of the latency from the RAN to the traffic sink for UL traffic. This also means that aspects such as PDU discarding, which is a PDCP protocol layer functionality for 5G NR (and likely also 6GR, subject to RAN2 decisions), are not part of the traffic model definition. For cases where effects such as TCP (including related flow control) are to be included, TCP is also not part of the traffic model itself, but can be modelled separately between the traffic source, the CN/RAN, and the traffic sink.

None of the RAN functionalities shall be embedded in the traffic model definition. The traffic model shall only include what the application traffic source generates in terms of data for transmission, and potential CN latencies.

Since RAN1 has already concluded that the set of widely adopted legacy traffic models can be used for the 6G evaluations, there is no rush to introduce new models that require sophisticated analysis in several 3GPP WGs. Considering that feedback is needed at least from SA4 on AIML service traffic model and extensions to the XR model, we think that at this stage (e.g., at the upcoming meeting), RAN1 should spend time on the model that can be discussed in RAN1 only. In particular, the focus should be on the extensions of FTP-1/FTP-3 models that are discussed further below.

RAN1 to prioritize reaching agreements on FTP traffic modes within RAN1, while waiting for the feedback from SA4 and other WGs on traffic models for AI, XR, Haptic feedback, etc.

We do, however, still see great value in the already-defined FTP3 model, and we suggest marking it as the default one among various FTP models as follows:

RAN1 to select FTP-3 as the default FTP mode using the following parametrization: default file size of 0.5 MB, variable file arrival rate resulting in Low, Medium, Large offered loads per cell (corresponding to average PRB utilization of approximately 10%, 30% and 60%).

### Extension of the FTP-1/3 traffic model

At RAN1#122bis it was agreed to study extensions for the FTP Model 1/FTP Model 3:

Agreement Study extensions to FTP Model 1/FTP Model 3 to incorporate the following: - Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time) - FFS number of packet sizes (e.g., 2 or 3). - FFS whether to have fixed or variable packet size and packet arrival rate for a given UE. - FFS applicability of multiple packet sizes to only one or both of FTP Model 1/FTP Model 3. - FFS packet size and arrival rate characteristics. - Packet delay budget (PDB) related parameters - FFS PDB applicability to packets (e.g., one PDB parameter for only one traffic flow or different PDB parameters for different traffic flows). - FFS how to consider the PDB, e.g., whether to drop packets when exceeding the budget, PDB aware metric. - Note consider the following for PDB: - Applicability to the extension to FTP Model 1/ FTP Model 3 with one packet size. - Applicability or not to the extension to FTP Model 1/ FTP Model 3 with multiple packet sizes.

The discussion on extensions of the FTP-3 model – hereafter denoted eFTP-3 – to include a mixture of different file sizes continues in RAN1. We suggest that the eFTP-3 model is a simple superposition of two FTP-3 standard models with different file sizes and average arrival rates. This means that the eFTP-3 model generates traffic with file sizes of B1 bits according to a Poisson process with an average arrival rate r1, as well as traffic with file sizes of B2 bits according to another Poisson process with an average arrival rate r2. The two Poisson processes for triggering new file size bursts are assumed to be independent. For the sake of simplicity, we suggest that B2>B1, and B2=K×B1 and r2=r1/K, where K is a natural number, e.g. K=4 such that the eFTP-3 model generates file sizes that vary with a factor of four. Given these assumptions, the generated average load (bit rate) per UE will equal B1×R1×2. Similarly, as for the (legacy) FTP-3 model, we propose to assume B1=0.5 MB as the default setting, K=4, while varying the file arrival rate to result in Low, Medium, Large offered loads per cell that corresponds to average PRB utilization of approximately 10%, 30% and 60%.

We suggest an extended FTP-3 (eFTP-3) model that consists of superposition of two FTP-3 legacy models with different file sizes (B1 and B2) and average arrival rates (r1 and r2). The characteristic of the model is as follows:

  - File sizes of B1 and B2 bits are generated according two independent Poisson process with average arrival rates of r1 and r2,

  - B1=0.5 MB as the default setting for FTP-3,

  - B2=K×B1 and r2=r1/K, where K is a natural number with default setting K=4,

  - The file arrival rate r1 is varied to result in Low, Medium, Large offered loads per cell that corresponds to average PRB utilization of approximately 10%, 30% and 60%.

  - Alternative parametrizations for B1, and K are also possible.

Our preference is to have a single traffic generator with a mixture of two packet sized and inter-arrival rates. In this respect, we do not see it possible to distinguish separate traffic or QoS flows at PHY. However, we think that can be possible to have different UE classes in a single simulation. In this case, eFTP-3 traffic model may have different parametrizations per that class. For example UEs from the 1st class can have only legacy FTP-3 traffic (K=0) and for the 2nd class K = 4.

eFTP-3 traffic model can optionally have different parameters for different groups/classes of UEs in a simulation, for example, K = 0 (only legacy FTP-3) for some UEs and K = 4 for the rest.

Related to the PDB aspects, we consider the PDB as a metric associated with traffic model but not being explicitly part of the traffic model itself. Mechanisms such as dropping of packets when exceeding a certain PDB is in 5G NR part of the PDCP protocol, referred to as packet discarding (based on timer-based mechanisms). Potential packet discarding functionality for 6GR is the responsibility of RAN2 to standardize. If RAN2 confirms such packet discarding functionality for 6GR, it can be included in the 6GR system-level simulations as part of the RAN functionalities, but not as part of the traffic model definition.

A PDB metric may be additionally associated with the traffic model.

Packet discarding shall not be part of the traffic model definition. Such mechanisms are up to RAN2 to decide and can be included in the RAN simulation assumptions (but not as part of the traffic model itself).

Finally, we do not see much value in introducing such extensions both for FTP-3 and FTP-1 models.

It is sufficient to consider extensions to FTP Model 3 only.

### Bi-directional traffic model

The models outlined so far (full-buffer, FTP-1, FTP-3 and XR) are open loop traffic models, where the traffic source keeps generating data independently on whether the traffic is served and correctly arrives at its destination (aka the traffic sink). The alternative is usage of closed-loop models, where the data generated by the traffic source is adapted in coherence with the RAN performance, and hence other effects start to become visible. This is, for instance, the case when considering TCP effects with build-in flow control mechanisms, and the potential inter-play between various RAN recovery loops such as HARQ, RLC, PDCH when various error cases are considered (e.g. NACK-to-ACK errors). Similarly, another relevant example is the use of adaptive codecs (e.g. rate adaptive Video codecs) where the source data rate of a video feed is adjusted depending on the data rate that the wireless connection can accommodate. Closed loop traffic models are sometimes also referred to as bi-directional models. Although closed-loop (bi-directional) traffic models are not needed for all 6GR simulation studies, we find that it is important to agree on at least one reference case with such models. It could, e.g., include the TCP CUBIC (i.e. including TCP flow control) and could be based on the existing XR traffic models to enforce dependency between DL video streaming and UL control feedback, etc. As mentioned, closed-loop models are important for having realistic studies of the various RAN recovery loops (e.g. HARQ, RLC, PDCP) and TCP effects.

RAN1 to include at least one closed-loop (bi-directional) traffic model into the 6GR study, e.g., to consider a scenario when the existing XR model is modelled with realistic TCP ACK feedback.

Default assumptions for simulation with TCP functionality shall be agreed for cases where this is enabled. Our suggestion is to rely on TCP CUBIC.

Additionally, the following was captured in the feature lead proposal, although not discussed online during the latest RAN1#122bis meeting:

New model 5-bidirectional traffic - (FL1) Question - Whether to define a bidirectional traffic flow modelling with simplified modelling as follows: - In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays - Component 2: SR+ UL grant+UL transmission delay. - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL.

As mentioned, we suggest maintaining a crystal-clear split between the traffic and 6GR RAN functionalities. In our view, the traffic model definition shall not embed L2 and L1 6GR functionalities; it shall only include modeling of traffic to be carried over the 6GR RAN. Regarding the introduction of the simplified model discussed above, we acknowledge that it can mitigate the simulation burden, but this may come with a loss of accuracy and sensibility of the model to multiple realistic effects, such as loss of UL packets with the feedback. Additionally, agreeing on the parameters of such analytical model may be a tedious task. Instead, we believe that explicit modelling of the UL RAN component can provide deeper insights. In this case, instead of traffic model parameters the agreement should be about the simulation parameters of the UL part.

Given this standpoint, we suggest the following understanding of the bi-directional model:

For simulations with TCP, we suggest that the transmission of the TCP-ACK over the RAN is simulated in UL explicitly, and possible CN latency is added to that.

  - For cases where CN delay plays a role, bi-direction model can assume a fixed one-way CN delay of e.g. 5 ms, representing the latency between the application (traffic source/sink) and the gNB.

  - Such parameters as L1/L2 processing latencies, HARQ processing, preparing and decoding of transmission, assumption on TTI size, etc. should not be a part of the traffic model definition, but simulation assumptions.

### AI/ML services

At RAN1#123, the LS [5] to SA4 was prepared based on the agreement:

Agreement For the study traffic model(s) for 6GR AI/ML services: - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870. Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services. Note: RAN1 is discussing the following options for the model: - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget. - Token is the minimum unit of data generated in the application layer. - How to associate Tokens to PHY layer packets. - How to reflect the variable importance of tokens. - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay. - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc. - Option-1c: reusing or extending the FTP-3/XR traffic model. - FFS other models/options need to be defined for other AI/ML services.

Our view is that we need to wait for SA4’s reply to the LS sent in RAN1#122. Once clear guidance is received, RAN1 can come back to this topic avoiding inaccurate assumptions such as considering models characterized only by PHY parameters (Option 1-b) as well as reusing or extending existing traffic models such as FTP3 and XR (Option 1-c). Otherwise, RAN1 approach will be premature and might result in neglecting important aspects of AI/ML services. For example, related to token usage, text prompts in UL and text response in DL may be tokenized. However, the presence of tokens is a lot less clear for other types of traffic. For instance, for ChatGPT and Gemini, the UE can send video over WebRTC, as URLs or files. Images can be sent with base64 encoding encapsulated in JSON and none of the non-text uplink data is tokenized [6][7]. Furthermore, aspects such as “importance of tokens”, “potential encapsulation of tokens with other traffic”, “scheduling priority of tokens” are all under the responsibility of RAN2, so RAN1 is not in a position to make decisions on such matters for the time being.

We also observe that the decision about the best way of transferring/transmitting data collected for other AI/ML services is under the responsibility of RAN2 and already on RAN2 agenda, so RAN1 is not in a position to make a decision for the time being on such models wither.

The related studies are planned in the related WGs, such as the New Study on Media Aspects for 6G System in SA4 and RAN2 New WID on XR (eXtended Reality) for NR Phase 4 [8].

Once clear guidance is received from SA4, RAN1 can come back to discuss modeling of AI traffic and potential extensions of XR traffic. Aspects related to e.g., “importance of tokens”, “potential encapsulation of tokens with other traffic”, “scheduling priority of tokens” are up to RAN2 to decide.

### Immersive communications

At RAN1#123, the LS [5] to SA4 was prepared based on the agreement:

Agreement Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services, - XR traffic models (in TR 38.838) are considered as starting point. - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc. - FFS how many models need to be defined and the corresponding representative use cases. - FFS how to incorporate haptics traffic (TR26.854). Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

We find the defined XR traffic case from 3GPP TR 38.838 with real-time video streaming most relevant, considering 30 or 45 Mbps for the DL and 10 Mbps for the UL, assuming 60 fps as default. 3GPP TR 38.838 also describes multi-stream XR models that cover slice-based and group of picture (GOP)-based models, including cases with video plus audio. However, those models were never used during the Release 17 to 19 XR studies. Hence, we do not suggest prioritizing such models for 6GR studies either. Note that ITU-R is also considering use of such XR traffic models from 3GPP TR 38.838. There an is ongoing discussion in ITU-R WP 5D on introduction of a new Composite Requirement for IMT-2030 targeting XR use case, and as proposed in the RAN LS in [9], the candidate DL and UL bit rates are 30 and 10 Mbps, respectively, with a latency bound of 40 ms (including both DL and UL).

RAN1 to adopt the single-stream packet-based XR traffic model for real-time video from TR 38.838 with default settings of 30 or 45 Mbps for DL and 10 Mbps for UL, assuming 60 fps.

3GPP TR 38.383 also includes relevant models for uplink control information for various feedback information related to XR downlink video streaming such as, e.g., (i) UE pose information update received from 3-6 degrees of freedom tracking and device sensors; and (ii) control information including user input data, auxiliary information, and/or (iii) commands from the client to the server. We consider such models relevant also for the study item phase of 6GR.

RAN1 to adopt for the 6GR study the model of XR UL control information feedback from 38.838 that needs to be transmitted together with DL traffic.

### Massive IoT

It is acknowledged that the study of massive IoT within the context of 6G is essential, and system design efforts should reflect this need. Appropriate traffic models must therefore be integrated into the overall study to ensure the characteristics of massive IoT are accurately represented.

To avoid unnecessary complexity stemming from an overly broad set of traffic models, it is recommended to investigate how existing traffic models might be utilized to emulate massive IoT scenarios. This approach will help streamline the evaluation process and maintain clarity in the analysis.

RAN1 to consider adoption of the following models for massive IoT:

  - Instant messaging traffic for IoT applications can be modeled using FTP model 3 with an assumed packet size of 0.1 MB, following the proposal in TR 38.840.

  - Software updates in massive IoT can be effectively represented by the FTP model 1, accommodating the transfer of larger files as typically required for such updates.

## Link budget evaluations

At RAN1#122bis the discussion of the approaches for link budget evaluations resulted in two Candidate options listed below:

Agreement For link budget template, consider the following candidates: - Candidate 1: Reusing the link budget template from TR38.830, i.e., the following table with notes as follows: - The values of the parameters are TBD. - MCL in row (22bis) is TBD. - FFS: whether/how/why to update System configuration. Channel for evaluation. Scenarios and Carrier frequency (GHz). BS antenna heights (m). UT antenna heights (m). Cell area reliability (%). Lognormal shadow fading std deviation (dB). Tx Diversity. Number of SSB. Transmitter. (1) Number of transmit antenna elements. (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink. (2a) Number of transmit chains modelled in LLS. (3) Total transmit power (dBm) Note: total transmit power for system bandwidth. (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz). (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink. (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c). (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ((3c) /1000000) (dBm). (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB). (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ((1) / (2)) (dB) for downlink, and = (4c) + 10 log ((1) / (2a)) (dB) for uplink. (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB). (4c) Gain of antenna element (dBi). (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink. (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log((2)/(2a)) (dB) Note: zero for uplink. (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink. (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink). (9) EIRP = (3bis) + (4) + (5) – (8) dBm. Receiver. (10) Number of receive antenna elements. (10a) Number of receive TxRUs Note: this row is void (empty) for downlink. (10b) Number of receive chains modelled in LLS. (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB). (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ((10)/(10a)) (dB) for uplink = (11c) + 10 log ((10)/(10b)) (dB) for downlink. (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB). (11c) Gain of antenna element (dBi). (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink. (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log((10a)/(10b)) (dB) Note: zero for downlink. (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink. (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink). (13) Receiver noise figure (dB). (14) Thermal noise density (dBm/Hz). (15) Receiver interference density (dBm/Hz). (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz). (18) Effective noise power = (16) + 10 log ((3c)) (dBm). (19) Required SNR (dB). (20) Receiver implementation margin (dB). (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS. (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm). (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB). (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12). Calculation of available pathloss. (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB). (26) BS selection/macro-diversity gain (dB). (27) Penetration margin (dB). (28) Other gains (dB) (if any please specify). (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB). Range/coverage efficiency calculation. FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m). - Candidate 2: Template as Table 7.10.1-1 from TR38.913. - FFS: whether/how/why to update. Item; Value. Transmitter. (1) Tx power (dBm). Receiver. (2) Thermal noise density (dBm/Hz). (3) Receiver noise figure (dB). (4) Interference margin (dB). (5) Occupied channel bandwidth (Hz). (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm). (7) Required SINR (dB). (8) Receiver sensitivity = (6) + (7) (dBm). (9) MaxCL = (1) - (8) (dB).

The original design of 5G NR has not included enough analysis of all coverage aspects in the first release. Therefore, a significant effort was spent starting from Release-17 on coverage enhancements. Since coverage improvements were introduced only in later releases, their widespread adoption was limited. Hence, we believe that it is crucial to perform accurate evaluation of the 6G coverage and address potential coverage issues especially for the new frequency bands from the beginning of the new generation.

TR 38.830 provides an exhaustive background for link budget evaluation, especially in Section 4. The basic evaluation methodology is based on link-level simulation and is articulated in 2 steps:

-	Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

-	Step 2: Obtain the baseline performance based on required SINR and link budget template.

The evaluation methodology based on system-level simulation was considred but left optional.

Section 4.2 of TR 38.830 defines the following metrics that are also considered for the 6G link budget evaluation:

Definition of MCL (Max Coupling Loss): - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2). Definition of MIL (Maximum Isotropic Loss aka Hardware link budget): - MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain. Definition of MPL (Maximum Path Loss): - MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains. For LLS based methodology, coverage bottleneck(s) identification is performed using at least MIL or MCL (assuming the set of simulation assumptions). MCL values can also be used to identify the coverage bottleneck(s). MPL can be used as supplemental information for coverage bottleneck(s) identification.

The meaning of the metrics above can be interpreted in the following way:

- MCL indicates how much loss the air-interface can tolerate before the receiver falls below sensitivity, if one considers only the gNB antenna gain component 2

  - Note that antenna gain component 2 (i.e., “digital antenna gain”) is present only in gNB antenna gain modelling Option 1 from 38.901, i.e., when the number of RF chains (k) simulated in the LLS to derive Required SINR is less than the number of TxRUs (N).

- MIL indicates how much loss can be tolerated after accounting for the actual RF hardware on both ends (gains and losses):

  - MIL = MCL – Tx loss – Rx loss + gNB antenna gain components (3 + 4) + UE antenna gain.

- MPL indicates how much propagation loss (including margines) can the link withstand.

  - MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains

Therefore, the three metrics are not interchangeable and picking one over the other has impact on how further discussions can go. First, we should start by deciding what we mean by comparable coverage between 5G mid-band and 6GHz at around 7 GHz:

- If it is comparable MCL, then we are checking channel structures essentially (plus “digital antenna gain”)

  - This view is good to ensure that 6G design won’t deliver worse link performance than 5G in terms of channel structures.

    - Technically, this is the best way to ensure acceptable performance for the service-specific requirements, e.g., for VoIP.

    - However, this does not guarantee that the same ISD can be maintained for deployments, since it will depend on array structures at gNB.

- If it is comparable MIL, then the “ E2E” antenna array gain is included:

  - This is good to identify bottlenecks when the “E2E” chain is considered. For instance, this can help understanding what the coverage gap between a broadcast and a unicast channel is in practice, e.g., for PDCCH. Such difference is not visible just by using MCL. However, such approach

    - Does not ensure service-specific requirements, e.g., for VoIP.

    - Still does not ensure that the same ISD can be maintained for deployments, since it will depend on array structures at gNB.

- If it is comparable MPL, then the deployment specific factors/gains are included, and it provides the real “link budget” assessment in terms of raw coverage

  - This ensures that the same ISD can be maintained for deployments, since deployment specific aspects, which may depend on the carrier frequency and impairments, would also be factored in. However,

    - This complicates the reading of the situation in terms of channel structures and service-specific requirements.

    - It is more challenging for the companies to agree on the values to consider for carrier frequency specific aspects and impairments.

At RAN1#122bis, the main concern about the methodology Candidate 1 was its complexity and that antenna gain and assumptions depends on many aspects that are hard to agree on. However, the main difference between Candidate 1 and Candidate 2 is just in accounting for Antenna gain component 2 Figure 5 below demonstrate two open for antenna gain modelling in TR 38.830. In Option 2, fully digital beamforming, Antenna gain component 2 is not present, because it will be included into Required SINR value, that is provided by LLSs.

Figure 5: gNB antenna gain modelling Option 1 (left) and Option 2 (right) from 38.830.

The methodology formulated in 38.830 is a result of the profound study. The main difference between Candidate 1 and Candidate 2 for link budget evaluation in respect to MCL computation is in the way how gNB antenna gain component 2 is treated. However, the methodology from 38.830 also covers the gNB antenna gain modelling Option 2 where the Antenna gain component 2 is not considered in the link budget template.

RAN1 to use methodology formulated in TR 38.830 as framework and as a baseline. Simplifications can be applied, e.g., before parameters for LLS with 6G parameters are available.

Only MPL metric can be used to ensure that the same ISD can be maintained for different deployments. However, it is the most challenging metric due to a need to agree on the values for carrier frequency specific aspects and impairments.

RAN1 should consider all metrics: MCL, MIL, and MPL because they are not interchangeable and provide evaluation of different aspects of coverage.

## Assumptions for NTN

At the previous RAN1#122bis meeting the following NTN-related agreements were achieved:

Agreement The attached templates for NTN in R1-2507956 are endorsed in principle. Agreement At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations: - L-band (i.e., 1.5GHz) - S-band (i.e. 2 GHz) - Ku-band (FFS detailed frequency range) - Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

As a starting point we should rely on the past 3GPP studies on NTN and the corresponding technical reports 38.811 and 38.821, when defining the parameter values. For example, the UE and BS antenna gain, G/T and transmit power are already defined in TR 38.821. The starting point should according to our preference be Set-1 as defined in Table 6.1.1.1-1 in TR 38.821 combined with Table 6.1.1.1-3 for the UE characteristic parameters at least for a handheld device, given that highest commonality between TN and NTN device types is expected. If one or more parameters are needed, but not available in the TRs or if the value (range) of the parameter has significantly evolved, RAN1 shall discuss an updated value.

In our view, the deployment parameters shall cover the LEO-MEO-GEO scenarios already envisioned in NR and in addition the VLEO (e.g. 350 km orbital altitude). Furthermore, both transparent and regenerative architectures are in scope in combination with Earth-moving and (quasi) Earth-fixed cells. Finally, the specification shall not restrict whether one or multiple satellite beams are used per cell, nor shall it restrict the type of frequency reuse. Therefore, the simulation parameters shall include the same deployment parameters as in the NR technical reports. According to our understanding, legacy parameters can also be used for a study of multi-orbit deployment by combining the various satellite parameters.

RAN1 shall reuse NTN parameters from TR 38.811 and TR 38.821 as much as possible. The VLEO deployment scenario may be added to the Set-1 definitions from Table 6.1.1.1-1 of TR 38.821.

# Conclusion

In this contribution we discuss different aspects of Evaluation assumptions for the upcoming 6G networks. The following observations and proposals are made:

On BS-related parameters:

Observation 1: Some of the BS antenna combinations at around 700 MHz and 2GHz have parameters that are not fully captured. Moreover, for around 2 GHz, Optional outdoor antenna Combination 1 with 32 AEs and only 4 TXRUs does not look very representative.

Proposal 1: If found to be needed, RAN1 can clarify further optional antenna configurations at 700 MHz and 2GHz as follows:

- 700 MHz, Optional configuration: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1, 2, 4), i.e., with 16 TXRUs

- 2 GHz: Outdoor, Combination 1(Optional): consider increasing the number AEs from 32 to 64, and number of TXRUs from 4 to 8, and assume (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4).

Proposal 2: For around 4 GHz carrier, we propose to consider the following BS antenna configurations as a baseline:

- For Indoors, Configuration 2: (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 8), 128 AEs, 32 TXRUs.

- For Outdoors, Configuration 1: (M, N, P, Mg, Ng; Mp, Np) = (12, 8, 2, 1, 1; 4, 8), 192 AEs, 64 TXRUs.

Observation 2: Too large BS antenna panels of 2048 antenna element, if considered at around 7 GHz frequency, may need to use reduced Tx power due to EIRP limits insufficient to provide good coverage for cell edge users.

Proposal 3: For the BS antenna around 7GHz, to consider as baseline:

- For outdoor, either modified Combination 1, 768 AEs with 256 TXRUs instead of 128 with configurations (M, N, P, Mg, Ng; Mp, Np) = (24, 16, 2, 1, 1; 8, 16) or already listed Configuration 2 (M, N, P, Mg, Ng; Mp, Np) = (32, 16, 2, 1, 1; 8, 16), 1024 AEs, 256 TXRUs.

- For indoor deployments, Combination 3, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs.

Proposal 4: For the BS antenna around 15 GHz to consider:

- For indoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs, as baseline.

- For outdoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 1, 32), 3072 AEs, 64 TXRUs and (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 2, 32), 3072 AEs, 128 TXRUs.

Proposal 5: For the indoor deployments, for the total transmit power per BS, RAN1 to consider below 15 GHz: 24 dBm per 20 MHz, EIRP should not exceed 61 dBm, above 15 GHz: 23 dBm per 100 MHz, EIRP should not exceed 58 dBm.

Proposal 6: For the outdoor deployments, macro layer, for the total transmit power per BS, RAN1 to consider:

- For around 700 and 2GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise.

- For around 4 GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm.

- For around 7 GHz and around 15 GHz: 44 dBm per 20 MHz for dense urban and 46 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm.

- For around 30 GHz: 33 dBm per 20 MHz, EIRP should not exceed 75 dBm.

On UE-related parameters:

Observation 3: New Rel-19 UE antenna model from TR 38.901 considerably improves the realism of UE modelling. For UE with four antennas, the candidate locations (1,3,5,7) were calibrated, and for UE with two antennas, e.g., below 1 MHz, only ports (8, 4) have UE spatial non-stationarity model (blockage) defined.

Proposal 7: RAN1 should use new directive handheld UE antenna model described in section 7.3 of Rel-19 version of TR 38.901 by default.

Proposal 8: For handheld UE antenna modelling, RAN1 to select the following default configurations frequency:

- For around 700 MHz: 1/2T2R directional antennas, (4, 8) as described in section 7.3 in TR 38.901,

- For around 2 GHz, around 4GHz: 2T4R directional antennas, (1, 3, 5, 7) as described in section 7.3 in TR38.901,

- For around 7 GHz and around 15 GHz: 4T4/8R directional antennas, (1, 3, 5, 7) for 4 and (1,2,3,4,5,6,7,8) for 8 as described in section 7.3 in TR38.901.

Proposal 9: For handheld UE modelling, RAN1 to consider the following UE transmit power:

- For below 7 GHz: 23, 26, 29 dBm

- For 7 GHz and above: 26, 29 dBm

Proposal 10: For CPE antenna modelling, RAN1 to use CPE antenna placement from section 7.3 in TR 38.901:

- 2/4T4R with antenna locations (1, 3, 5, 7)

- 6/8T6/8R with antenna locations (1, 2, 6, 7, 8, 9) for 6 and (1,2,3,4,5,6,7,8) for 8.

Table 3: CPE antenna configurations for FR1 and ‘FR3’

|  | Antenna Element Gain (dBi) per TXRU |  |  |  |
| --- | --- | --- | --- | --- |
| Carrier Freq | Indoor CPE |  | Outdoor CPE or CPE array |  |
| (GHz) | {360°, 22.5°} OMNI DIR | {90°,90°} DIR | {65°, 50°} DIR | {32°, 32°} DIR |
| 2 | 5 | 5 | 9 |  |
| 3.5 | 5 | 5 | - | 14 |
| 7 | 5 | 5 | - | 14 |
| DIR - CPE directs it antenna elements to point at the serving gNB |  |  | HPBW {horizontal, vertical} |  |
| OMNI Directional - CPE does not direct its antenna elements to point at the serving gNB |  |  |  |  |
| Coherent or non-coherent precoding may be applied which may result in additional directionality |  |  |  |  |
| Antenna arrays: e.g., 4TX, 6TX, 8TX, 16TX can be modeled w. proposed TXRU antenna element gains given above |  |  |  |  |
| PC_MAX is chosen to meet the regulatory max average band EIRP limit accounting for total array gain and TDD Duty Cycle |  |  |  |  |

Proposal 11: For CPE antenna modelling, RAN1 to select CPE antenna configurations per frequency and device type (outdoor/indoor CPE) as described in Table 3 above.

Proposal 12: For CPE modelling, RAN1 to support 23, 26, 29, 31 dBm power classes and up to the max conducted power that meets the maximum average band EIRP limit (subject to regulatory limitations and accounting for maximum average EIRP limit mitigations for both indoor and outdoor CPEs).

On interference modelling in SLS

Proposal 13: 6G simulations shall include realistic spatial interference covariance estimation for multi-antenna receivers. For system-level simulations, we suggest adopting the well-known Wishart model.

On traffic models:

Proposal 14: None of the RAN functionalities shall be embedded in the traffic model definition. The traffic model shall only include what the application traffic source generates in terms of data for transmission, and potential CN latencies.

Proposal 15: RAN1 to prioritize reaching agreements on FTP traffic modes within RAN1, while waiting for the feedback from SA4 and other WGs on traffic models for AI, XR, Haptic feedback, etc.

Proposal 16: RAN1 to select FTP-3 as the default FTP mode using the following parametrization: default file size of 0.5 MB, variable file arrival rate resulting in Low, Medium, Large offered loads per cell (corresponding to average PRB utilization of approximately 10%, 30% and 60%).

Proposal 17: We suggest an extended FTP-3 (eFTP-3) model that consists of superposition of two FTP-3 legacy models with different file sizes (B1 and B2) and average arrival rates (r1 and r2). The characteristic of the model is as follows:

- File sizes of B1 and B2 bits are generated according two independent Poisson process with average arrival rates of r1 and r2,

- B1=0.5 MB as the default setting for FTP-3,

- B2=K*B1 and r2=r1/K, where K is a natural number with default setting K=4,

- The file arrival rate r1 is varied to result in Low, Medium, Large offered loads per cell that corresponds to average PRB utilization of approximately 10%, 30% and 60%.

- Alternative parametrizations for B1, and K are also possible.

Proposal 18: eFTP-3 traffic model can optionally have different parameters for different groups/classes of UEs in a simulation, for example, K = 0 (only legacy FTP-3) for some UEs and K = 4 for the rest.

Proposal 19: A PDB metric may be additionally associated with the traffic model.

Proposal 20: Packet discarding shall not be part of the traffic model definition. Such mechanisms are up to RAN2 to decide and can be included in the RAN simulation assumptions (but not as part of the traffic model itself).

Proposal 21: It is sufficient to consider extensions to FTP Model 3 only.

Proposal 22: RAN1 to include at least one closed-loop (bi-directional) traffic model into the 6GR study, e.g., to consider a scenario when the existing XR model is modelled with realistic TCP ACK feedback.

Proposal 23: Default assumptions for simulation with TCP functionality shall be agreed for cases where this is enabled. Our suggestion is to rely on TCP CUBIC.

Proposal 24: For simulations with TCP, we suggest that the transmission of the TCP-ACK over the RAN is simulated in UL explicitly, and possible CN latency is added to that.

- For cases where CN delay plays a role, bi-direction model can assume a fixed one-way CN delay of e.g. 5 ms, representing the latency between the application (traffic source/sink) and the gNB.

- Such parameters as L1/L2 processing latencies, HARQ processing, preparing and decoding of transmission, assumption on TTI size, etc. should not be a part of the traffic model definition, but simulation assumptions.

Proposal 25: Once clear guidance is received from SA4, RAN1 can come back to discuss modeling of AI traffic and potential extensions of XR traffic. Aspects related to e.g., “importance of tokens”, “potential encapsulation of tokens with other traffic”, “scheduling priority of tokens” are up to RAN2 to decide.

Proposal 26: RAN1 to adopt the single-stream packet-based XR traffic model for real-time video from TR 38.838 with default settings of 30 or 45 Mbps for DL and 10 Mbps for UL, assuming 60 fps.

Proposal 27: RAN1 to adopt for the 6GR study the model of XR UL control information feedback from 38.838 that needs to be transmitted together with DL traffic.

Proposal 28: RAN1 to consider adoption of the following models for massive IoT:

- Instant messaging traffic for IoT applications can be modeled using FTP model 3 with an assumed packet size of 0.1 MB, following the proposal in TR 38.840.

- Software updates in massive IoT can be effectively represented by the FTP model 1, accommodating the transfer of larger files as typically required for such updates.

On link budget evaluations:

Observation 4: The methodology formulated in 38.830 is a result of the profound study. The main difference between Candidate 1 and Candidate 2 for link budget evaluation in respect to MCL computation is in the way how gNB antenna gain component 2 is treated. However, the methodology from 38.830 also covers the gNB antenna gain modelling Option 2 where the Antenna gain component 2 is not considered in the link budget template.

Proposal 29: RAN1 to use methodology formulated in TR 38.830 as framework and as a baseline. Simplifications can be applied, e.g., before parameters for LLS with 6G parameters are available.

Observation 5: Only MPL metric can be used to ensure that the same ISD can be maintained for different deployments. However, it is the most challenging metric due to a need to agree on the values for carrier frequency specific aspects and impairments.

Proposal 30: RAN1 should consider all metrics: MCL, MIL, and MPL because they are not interchangeable and provide evaluation of different aspects of coverage.

On NTN:

Proposal 31: RAN1 shall reuse NTN parameters from TR 38.811 and TR 38.821 as much as possible. The VLEO deployment scenario may be added to the Set-1 definitions from Table 6.1.1.1-1 of TR 38.821.

# References

- R1-2509079, Fixed Wireless Access Scenarios, AI 11.2, RAN WG1#123, Dallas Tx, Nov 17-21, 2025.

- FCC 47 CFR § Part 27.5(h), (1)(ii) BS and (2) UE max average EIRPs for 2.5GHz (n41) band. https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-27/subpart-C/section-27.50

- FCC 47 CFR § Part 27.5(k), (1) BS rural, (2) BS non-rural and (3) UE max average EIRPs for 3.45GHz (n77) band. FCC's specific implementation and guidance for this band (KDB 662911 D02) allows for a per-polarization EIRP measurement for compliance.

- FCC OET, KDB 662911 D02, “MIMO with Cross-Polarized Antenna”, 10.25.2011 https://apps.fcc.gov/kdb/GetAttachment.html?id=i%2BFRza%2B2Hh0pf9nHJHJGHw%3D%3D&desc=662911%20D02%20MIMO%20with%20Cross%20Polarized%20Antenna%20v01&tracking_number=49466

- R1-2508184, LS on traffic model study in RAN1, 3GPP TSG RAN WG1 Meeting #122bis                                       Prague, Czech Republic, 13th – 17th October 2025.

- OpenAI API

- Gemini API

- RP-251866, New WID on XR (eXtended Reality) for NR Phase 4, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

- RP-252946, Draft Reply LS to ITU-R_WP5D_TEMP_167 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), 3GPP TSG RAN Meeting #109 Beijing, China, September 15-18, 2025.