3GPP TSG RAN WG1 #123                                                                  R1- 2508366

Dallas, Texas, USA, November 17th – 21st, 2025

Source:	Kyocera

Title:	AI/ML in 6GR interface

Agenda item:	11.6

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss AI/ML for 6GR and radio access network in the physical layer, focusing on the objectives agreed on in 3GPP TSG RAN meeting #108 [1] and are outlined as follows:

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

According to the above objectives, the initiative should focus on integrating AI/ML into the design of 6G RAN, building on the existing 5G NR AI/ML framework [2]. New (or existing) use cases should be identified considering their commercial demand (performance) and practicality (computational complexity). Moreover, the AI/ML framework should aim to develop extensible enablers tailored to the new (or existing) use cases, covering LCM procedures and data collection/management. Importantly, the 6G system must also be capable of operating without AI/ML to ensure flexibility and resilience in network design.

# Observation updates and analysis on proposed use cases

In Section 4.1 of the FL concluding notes [3] of RAN1#122bis meeting, the FL suggested the following:

Suggest to only leave the new/updated (sub-)use cases, and delete the one already captured in observations this meeting. Provide same level of proposals (including the table, with change mark if it is a revise of existing table) is appreciated.

We suggest leaving the use cases that were proposed in RAN1#122bis meeting. However, to have manageable workload and high-quality output, we propose to limit the study to up to 4 use cases in the 6GR study. From our perspective, we prioritize the study of DM-RS overhead reduction with neural receivers, Low overhead CSI-RS with AI/ML, low overhead SRS and inter-cell beam management uses cases.

The location of AI/ML model deployment has implications on specification requirements and deployment complexity. NW-sided models typically present minimal specification impact, as they can be implemented independently of UE capabilities and do not necessitate extensive UE-specific support. In contrast, UE-sided models are contingent on device implementation and capabilities, introducing additional layers of complexity. Two-sided models further compound deployment challenges by requiring tight coordination and interaction between the NW and the UE. Additionally, the exploration of two-sided models may intersect with ongoing studies in Release 20 AI/ML for the NR air interface. To avoid redundancy, it may be wise to defer the study of two-sided models for the 6GR study.

Hence, based on the previous discussion, we have the following proposal:

- RAN1 should consider the following aspects during the 6GR study:

    - Down select the 6GR study to up to 4 new use cases.

    - From the AI/ML model type perspective, one-sided AI/ML model use cases should be high priority.

In the next parts of the contribution, we provide analysis of these use cases according to the chair guidance in RAN1 reflector "As communicated by the FL via email on November 3, 2025, at 7:01 AM PST", which is stated as follows:

For 6GR, I will keep the agenda and related notes under each agenda as it is, however, I would like to share some thoughts here for clarifying some of your questions and facilitating your preparation of your contributions since a lot of companies are wondering how to proceed with the following discussion of so many use cases after we have preliminary categorization of them, whether RAN1 will try to do some down selection, how to proceed with these use case since February meeting. To be clear, I'm not planning any down selection exercises based on counting the supporters in RAN1, but I need to plan when/where/how to discuss each use case in the whole release if we still have some cases till next year. Here are some key points that I'm thinking 1. It is good to provide detailed information about the label that you are using in your use case, e.g., the detailed assumption of the labels, how to collect, e.g., whether the collection should be explicitly supported, prerequisite for collecting label, ...... 2. It is good to provide detailed assessment about whether the use case can be supported without/minor standard effort, and what's the benefit of introducing claimed standard effort, including what kind of standard effort, ...... 3. It is good to provide detailed analysis about why the use case is better to be studied at the very beginning (e.g., the beginning of the study of the corresponding non AI/ML based features) rather than later on, and what conditions the use case should be contingent on...... 4. Not limit your views to your favorite use cases only.

## DM-RS overhead reduction with neural receivers

Label Definition and Collection. In this use-case, the label corresponds to the coded bit sequence for one transport block transmitted in a TTI and generated by the channel encoder, which is known at both the gNB and UE. Labels are collected by logging the encoder output during controlled transmissions across multiple TTIs with varying different parameters including SNR values and DMRS configurations for UL and DL. Explicit support is required to store these sequences for supervised training. Assumptions include error-free knowledge of coded bits and consistent mapping per 3GPP-defined modulation schemes. Prerequisites for label collection include access to encoder outputs, synchronization between transmitted and received data, and configuration awareness (e.g., DMRS and SNR settings).

Assessment of Standardization Effort and Benefits. From our perspective, this use case requires minimal additional standardization effort beyond what is already introduced in the AI/ML framework initiated in Rel-18/19. The primary dependencies relate to label creation and logging, including assumptions on encoder type, coding rate, modulation, and ensuring diversity for AI/ML model generalization, as well as associated identifiers for UE-side neural receivers. The expected standardization work mainly involves signalling to configure neural receiver parameters (e.g., DM-RS-to-symbol power ratio for superimposed DM-RS) and defining performance monitoring KPIs and fallback procedures in case of model failures. The benefit of this effort is significant: as demonstrated by multiple contributions in RAN1#122bis, neural receivers can substantially reduce DM-RS overhead, freeing more resource elements for PDSCH/PUSCH data and thereby improving throughput. Given the alignment with the existing AI/ML framework and the potential gains, DM-RS overhead reduction is an important use case for Rel-20.

Justification for Early Study and Contingent Conditions. Studying DM-RS overhead reduction with neural receivers at the beginning of Rel-20 is important because it directly impacts the design of DM-RS configurations and signalling for PDSCH/PUSCH. If neural receiver considerations are introduced later, existing non-AI/ML-based DM-RS designs may constrain achievable gains or require disruptive changes. Early integration ensures that any enhancements to DM-RS patterns, power ratios, or signalling can accommodate AI/ML-based decoding without backward compatibility issues. This use case is contingent on several conditions: availability of the AI/ML framework defined in Rel-18/19 (LCM, model management, signalling), clear guidelines for label generation and diversity to ensure model generalization, agreement on performance monitoring and fallback procedures for neural receivers, and feasibility of training and inference within practical complexity and latency limits. Addressing these conditions early will align neural receiver benefits with ongoing physical layer design, avoiding costly rework later.

Based on the above discussion, we have the following observation:

- Studying DM-RS overhead reduction at the beginning of Rel-20 may directly impact the design of DM-RS configurations and signalling for PDSCH/PUSCH.

## SRS-assisted CSI-RS overhead reduction

In RAN1#122bis meeting, we presented the SRS-assisted CSI-RS overhead reduction use case shown in Figure 1, where on one hand, uplink SRS measurements can capture channel information across all gNB ports, but only over a subset of the bandwidth. Hence, it might be able to provide the gNB with partial spatial (and frequency) channel information to determine the sparse CSI-RS configuration to be sounded next. On the other hand, the gNB may transmit CSI-RS, based on the determined sparse CSI-RS configuration, over a subset of its antenna ports across the entire bandwidth, with corresponding feedback provided based on this configuration. The gNB can then aggregate these two sources of information—CSI-RS reports and SRS measurements—and input them into an AI/ML model. This model captures information from both the report of the CSI-RS transmission on only a subset of its ports and UL SRS sounding on the subset of the bandwidth and aims to predict the CSI for a future DL slot transmission.

Figure 1 CSI-RS overhead reduction with SRS assistance

Depending on the design of the “Combine” unit, which aggregates the CSI-RS reports and SRS measurements information, this use case can be considered as a general use case for both low overhead CSI-RS with AI/ML and low overhead SRS use cases, where in the former, the “Combine” unit may exclude the SRS information and only consider the CSI-RS reports. Whereas, in the later, the “Combine” unit may exclude the CSI-RS reports and only consider the SRS information.

For low overhead CSI-RS with AI/ML use case, we request to add the following to Table A (highlighted in yellow):

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML |
| --- | --- |
| Reported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12 |
| Model input (for decoder of 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 1b. Received SRS info as assistance information for NW-sided model6 2. Reported CSI for NW-sided model3,4,5 |
| . . . | . . Rest of the table is omitted . |

This will in turn allow using the approach where the AI/ML model benefits from both CSI-RS and SRS transmissions.

Label Definition and Collection. In SRS-assisted CSI-RS overhead reduction use case, the label used for training the AI/ML model is defined as a high-fidelity channel estimate, ideally obtained under high SNR conditions. This label reflects the true downlink channel state and serves as a ground truth for prediction. We acknowledge that such high SNR conditions may not always be present in practical deployments. To ensure practicality, a hybrid label collection strategy can be used. When high SNR conditions are available, such as when the UE is close to the gNB, the label can be derived directly from full-port, full-band CSI-RS feedback. In scenarios where high SNR is not achievable, on one hand, filtering and averaging techniques may be applied over multiple time slots to enhance the reliability of the channel estimate. Additionally, uplink SRS measurements, though bandwidth-limited, can complement CSI-RS data and contribute to label generation under reciprocity assumptions. On the other hand, confidence-weighted training, where labels are assigned reliability scores based on SNR or estimation quality, allowing the AI/ML model to learn effectively even from imperfect data. This approach ensures that label collection is adaptable to real-world deployment constraints, while maintaining the integrity of the learning process.

Based on the above discussion, we have the following proposal:

- Proponents for low overhead CSI-RS with AI/ML should consider practical conditions for collecting the AI/ML model labels.

Assessment of Standardization Effort and Benefits. The proposed SRS-assisted CSI-RS overhead reduction use case is expected to not require a significant standardization effort, as it aligns well with the existing 5G NR AI/ML framework. In Figure 1, a gNB side AI/ML model is presented, though it can be extended to the UE side with similar assumptions. In either case, the specification impact is limited to leveraging already supported mechanisms such as CSI-RS configuration for supported functionality, UE AI/ML capability reporting, and performance monitoring. The anticipated standardization effort would mainly involve signalling enhancements—for example, enabling the gNB to configure the UE with sparse CSI-RS patterns and SRS configurations tailored for AI/ML-assisted operation, and allowing the UE to report its AI/ML processing capabilities. These enhancements fall within the scope of existing NR procedures and do not require fundamental changes to the physical layer or protocol stack. The key benefit of introducing this AI/ML-assisted approach is the potential reduction in CSI-RS or SRS overhead, achieved by intelligently inferring full channel state information from partial measurements. This not only improves spectral efficiency but also enables more adaptive and resource-efficient channel state acquisition, especially in scenarios with dense antenna configurations or limited feedback capacity.

Justification for Early Study and Contingent Conditions. Studying the SRS-assisted CSI-RS overhead reduction use case at the early stages of standardization is advantageous because it allows AI/ML considerations to be natively integrated into the design of CSI acquisition and feedback mechanisms, rather than being added later. This is particularly important as the use case leverages the interplay between CSI-RS and SRS configurations—features that are foundational to channel state acquisition in NR. Early inclusion ensures that any signalling support (e.g., sparse CSI-RS pattern configuration, AI/ML capability reporting, or performance monitoring) can be aligned with the current NR AI/ML framework, minimizing the need for future rework or backward compatibility constraints. Moreover, early study enables a more holistic evaluation of trade-offs between measurement overhead and prediction accuracy, guiding the design of efficient and scalable solutions for massive MIMO and high-mobility scenarios. The use case is contingent on the availability of flexible CSI-RS and SRS configuration options, support for AI/ML model execution at the gNB or UE, and mechanisms for exchanging assistance information and capability indicators. By addressing these aspects upfront, the standard can better accommodate AI/ML-driven enhancements while maintaining interoperability and implementation flexibility.

Based on the above discussion, we have the following observation:

- Studying CSI-RS overhead reduction (with assistance information) at the beginning of Rel-20 may directly impact the design of CSI acquisition and feedback mechanisms. Early inclusion ensures that any signalling support can be aligned with the current NR AI/ML framework, minimizing the need for future rework or backward compatibility constraints.

## Inter-cell beam management

Label Definition and Collection. In the context of AI/ML-based inter-cell beam management, labels represent the ground truth beams that the model aims to predict and can vary based on the prediction scope. For serving-cell-only predictions, the label is the Top-K beams from Set A for the serving cell, while for multi-cell predictions, it includes the Top-K beams for the serving cell and each neighboring cell. For regression models, labels may also include L1-RSRP measurements of these beams to comply with Rel-18/19 discussions. These labels are typically derived from exhaustive search-based legacy beam management, where all target cells configure and transmit the full Set A of beams, and the UE measures all beams in Set A.

Label collection can follow two approaches: (1) NW-side inference, where the UE reports all measurements to the serving gNB, which then determines the Top-K beams for each target cell, or (2) UE-side inference, where the UE processes measurements internally to identify the Top-K beams. A prerequisite for accurate label collection is that each target cell must configure distinct CSI-RS resource sets for Set A beam sweeping; overlapping resource sets may lead to ambiguity in beam identification across cells since one or more measurements may belong to more than one cell.

Assessment of Standardization Effort and Benefits. Inter-cell beam management builds upon the AI/ML-based beam management framework already established in 3GPP Rel-18/19, which includes foundational support for LCM procedures such as resource set configuration for inference and performance monitoring. Given this existing groundwork, the use case can be supported with minor standardization effort focused on extending current mechanisms/protocols to enable coordination across cells. The benefit of introducing this incremental effort lies in enabling seamless handover processes, where the optimal serving beam from each target cell is pre-determined, reducing latency and improving reliability. Furthermore, the inter-cell AI/ML framework can be leveraged in distributed MIMO scenarios involving multi-TRP transmission, allowing for dynamic and efficient beam selection per TRP. This is expected to enhance the overall system throughput and user experience, especially in dense deployments. The standard effort required would primarily involve enhancements to signalling procedures and performance metrics to support inter-cell coordination and inference sharing.

Justification for Early Study and Contingent Conditions. Early integration of inter-cell beam management allows the AI/ML framework to be designed in harmony with the baseline 5G NR MIMO procedures, ensuring compatibility and avoiding later-stage architectural constraints. This approach lays the foundation for scalable evolution towards 6G-ready MIMO systems, where AI-based beam management can be natively embedded into more advanced multi-TRP and distributed MIMO configurations. The study should be contingent on deployment scenarios that involve dense cell layouts, high mobility, and multi-TRP transmission—conditions where inter-cell coordination and predictive beam selection are critical for performance optimization.

Based on the above discussion, we have the following observation:

- Initiating the study of inter-cell beam management early enables the AI/ML framework to be seamlessly integrated with baseline 5G NR MIMO procedures. This ensures architectural compatibility from the outset and avoids the need for complex retrofitting later in the standardization process.

# General aspects

In the FL notes [3] from RAN1#122bis meeting, the FL had the following proposal

Proposal 3.1 For study/evaluation of AI/ML use cases in 6GR, consider - Performance-related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead), etc - including the overhead associated with inferencing, performance monitoring, online fine-tuning (if any) - Note: conclusions shall be drawn based on final performance metric (e.g., throughput) instead of intermediate KPIs - AI/ML Model related metrics, including model/computation complexity/power consumption, inference latency, training latency (for online training/fine-tuning, when if applicable), scalability, etc - FFS on whether/how to measure power consumption - Inter-vendor collaboration complexity and potential performance impact, when applicable - Generalization performance under a wide range of conditions - FFS on whether and how to consider realistic deployment scenarios Note: Which types of KPI (intermediate, link-level, system-level) will be adopted and detailed metrics to be discussed per use case. Note: RAN4 interoperability testing feasibility and complexity is up to RAN 4

Deep learning inference on UE is constrained by limited computing, memory, and power. Consequently, UE-side AI/ML models should favor compact architectures that preserve task performance while minimizing latency and energy. Proponents are encouraged to provide efficiency metrics without revealing proprietary details. These metrics may include parameter count, memory footprint, FLOPs per inference for the target input, measured inference time on representative UE hardware, and energy per inference (and, where helpful, energy per FLOP) with clear disclosure of measurement conditions.

To enable fair cost-benefit assessment, report link-level results and system-level throughput under the proposed deployment profile alongside these efficiency metrics and compare against meaningful baselines. Results should be contextualized by fixing workload and operating conditions—such as input sizes, precision or quantization, batching, and device power limits.

One of the primary challenges in deploying AI/ML-based solutions is the requirement for real-time inference. Inference latency is influenced by several factors, including the underlying hardware platform, software stack, and the level of code optimization. These latency constraints impose limitations on the design and complexity of neural network architectures. Given that network configurations can change within milliseconds, it is essential to utilize models capable of delivering inference within comparable timeframes. This requirement significantly restricts the architectural flexibility of neural networks and may consequently impact their overall performance and adaptability.

Hence, based on this discussion, we have the following proposal:

- UE-side AI/ML solutions should prioritize compact architectures that preserve task performance while minimizing latency and energy.

  - Recommendation: proponents of UE-side AI/ML use cases should provide model efficiency metrics without disclosing proprietary details. This should include core complexity and runtime metrics to substantiate the claimed performance gains under realistic UE conditions.

  - Suggested metrics for model complexity and energy consumption calculation:

    - Model size: parameter count and memory footprint.

    - Computational cost: FLOPs per inference for the target input configuration.

    - Runtime behavior: measured inference time on representative UE hardware and software stacks.

    - Energy efficiency: energy per inference and, where useful, energy per FLOP; report measurement setup and conditions.

    - System impact: link-level or system-level throughput under the proposed deployment profile to quantify cost-benefit relative to baseline and light weight AI/ML methods.

# Conclusion

In the contribution, we made the following observations:

- Studying DM-RS overhead reduction at the beginning of Rel-20 may directly impact the design of DM-RS configurations and signalling for PDSCH/PUSCH.

- Studying CSI-RS overhead reduction (with assistance information) at the beginning of Rel-20 may directly impact the design of CSI acquisition and feedback mechanisms. Early inclusion ensures that any signalling support can be aligned with the current NR AI/ML framework, minimizing the need for future rework or backward compatibility constraints.

- Initiating the study of inter-cell beam management early enables the AI/ML framework to be seamlessly integrated with baseline 5G NR MIMO procedures. This ensures architectural compatibility from the outset and avoids the need for complex retrofitting later in the standardization process.

And the following proposals:

- RAN1 should consider the following aspects during the 6GR study:

    - Down select the 6GR study to up to 4 new use cases.

    - From the AI/ML model type perspective, one-sided AI/ML model use cases should be high priority.

- Proponents for low overhead CSI-RS with AI/ML should consider practical conditions for collecting the AI/ML model labels.

- UE-side AI/ML solutions should prioritize compact architectures that preserve task performance while minimizing latency and energy.

  - Recommendation: proponents of UE-side AI/ML use cases should provide model efficiency metrics without disclosing proprietary details. This should include core complexity and runtime metrics to substantiate the claimed performance gains under realistic UE conditions.

  - Suggested metrics for model complexity and energy consumption calculation:

    - Model size: parameter count and memory footprint.

    - Computational cost: FLOPs per inference for the target input configuration.

    - Runtime behavior: measured inference time on representative UE hardware and software stacks.

    - Energy efficiency: energy per inference and, where useful, energy per FLOP; report measurement setup and conditions.

    - System impact: link-level or system-level throughput under the proposed deployment profile to quantify cost-benefit relative to baseline and light weight AI/ML methods.

# References

| [1] | NTT DOCOMO (Moderator), “RP-251881, New SID: Study on 6G Radio,” 3GPP TSG RAN Meeting #108, Prague, Czech Republic, 2025. |
| --- | --- |
| [2] | TR 38.843 v2.0.1, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (Release 18),” Third Generation Partnership Porject (3GPP), 2023. |
| [3] | Samsung (Moderator), “R1-2508118, Moderator Summary #5 on AI/ML for 6GR,” 3GPP TSG RAN WG1 #122bis, Prague, Czech, 2025. |