3GPP TSG RAN WG1 #123			R1-2508367

Dallas, USA, November 17th - 21st , 2025

## Agenda item: 11.3.1

Source: Kyocera

Title: Considerations for additional 6GR DL waveforms

Document for:	Discussion

# Introduction

According to the 3GPP 6G SID in [1], the following objectives related to 6G Waveform are given in Section 4.1:

(2)	Physical Layer structure for 6GR,

a)	Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

b)	Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

The following latest agreement relevant to 6GR DL waveform was made in RAN1#122 [2]:

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

In accordance with guidance from the Chair [2], proponents were encouraged to present detailed technical analyses at RAN1#123 meeting. While some companies have proposed DFT-s-OFDM for NTN and FR2 scenarios, our prior contribution [3] concluded that DFT-s-OFDM does not offer significant advantages for 6GR downlink. Furthermore, in section 3 of this contribution, we demonstrate that enhancements to CP-OFDM could provide comparable reduced PAPR performance.

Although performance evaluations of enhanced CP-OFDM and alternative waveforms have been presented in the previous RAN1 meetings, we believe these assessments should be analyzed within realistic and representative 3GPP deployment situations. Furthermore, the waveform proposals should be scrutinized not only for performance gains but also for their implications on MRSS, scheduling flexibility, and UE receiver complexity—critical factors for successful 6GR deployment. MIMO considerations, while important, are excluded from this document due to the clear implementation advantage that CP-OFDM has over new waveforms, owing to the maturity of the 5G NR-based MIMO framework and its efficient support for per-subcarrier precoding and channel estimation.

# Considerations for the new waveforms for 6GR Downlink

## MRSS

To enable seamless 5G–6G coexistence, 6GR PHY must maintain compatibility with the 5G NR frame structure. New waveforms such as Zak-OTFS [4] and AFDM [5] introduce distinct signal processing paradigms that impact MRSS performance and coexistence.

MRSS facilitates concurrent operation of 5G NR and 6GR within shared carriers and frequency bands, enabling dynamic resource allocation based on traffic and service requirements (see Figure 1). However, adjacent spectrum sharing between CP-OFDM and non-orthogonal waveforms like Zak-OTFS introduces significant hardware complexity due to different interference mitigation and channel estimation schemes.

Figure 1: An example of 5G NR – 6GR MRSS operation

For example, Zak-OTFS’s Delay-Doppler domain processing and overlapping resource allocation compromise the orthogonality inherent to OFDM systems, necessitating additional guard subcarriers to mitigate spectral leakage and adjacent-channel interference. This results in increased overhead and system complexity.

Efficient cross-waveform coexistence demands advanced interference management, adaptive channel estimation, and coordinated resource scheduling to maintain spectral efficiency across both 5G NR-6GR air interfaces.

Proposal 1: A rigorous evaluation of cross-waveform interference and CP-OFDM coexistence with non-orthogonal waveforms in MRSS operations is required when considering the adoption of an additional non-CP-OFDM waveform for 6GR.

## Scheduler Flexibility

CP-OFDM offers superior scheduler flexibility in 6GR due to its structure based on orthogonal subcarriers with consistent symbol duration. This enables straightforward time-frequency resource allocation in integer multiples of the basic resource element (one OFDM symbol-one subcarrier). Thus, CP-OFDM provides superior scheduling granularity due to its orthogonal subcarrier structure and uniform symbol duration.

In contrast, Zak-OTFS and AFDM operate in transformed domains—Delay-Doppler and Chirp-segment, respectively—where resource mapping spans the entire time-frequency plane. This broad mapping enhances resilience in scenarios with high mobility and significant delay spreads. However, it also limits scheduling flexibility for users with independently varying channel conditions, due to overlapping resource allocations.

Potential challenges may arise in mapping OTFS and AFDM symbols onto 5G NR/6GR resource grid due to inherent differences in waveform structure relative to OFDM. These differences can lead to increased complexity in symbol/frame alignment and resource allocation procedures such as rate-matching, etc., and may necessitate the introduction of additional numerologies to support various transmission formats. Such requirements are inconsistent with one of the SID objectives, which seeks to avoid excessive configurations and multiple options for the same functionality [1].

While standalone 6GR gNBs may be capable of handling the increased processing complexity, MRSS deployments introduce additional challenges. These include the need for coordination between CP-OFDM and non-CP-OFDM schedulers, involving pilot signal alignment and guard resource management.

Proposal 2: RAN1 should evaluate the new waveform’s performance by considering practical scheduler implementations to ensure reliable performance results.

## UE receiver complexity

Despite advancements in low-complexity OFDM receiver architectures, Zak-OTFS and AFDM receivers entail higher computational demands due to waveform-specific transformations and domain-specific equalization. This results in elevated power consumption, posing challenges for battery-constrained UE.

Even with advanced techniques such as deep learning-based joint channel estimation and detection, the complexity for the new waveforms remains significantly higher than for CP-OFDM. Consequently, these waveforms impose stricter requirements on UE processing capabilities, thermal design, and battery life. Moreover, the dual waveform-based UE architecture is expected to significantly increase testing efforts, thereby extending the development and deployment timeline.

Observation: Non-orthogonal waveforms may necessitate advanced UE receiver architectures, which potentially impact the 6GR deployment timeline.

# Enhancements to CP-OFDM

During the last meeting, several companies proposed enhancements to CP-OFDM aimed at reducing PAPR and improving coverage for NTN and IoT applications. In addition, OFDM-based techniques such as windowing/filtering PAPR reduction can be studied. Traditional PAPR reduction methods include Selective Mapping (SLM), Partial Transmit Sequences (PTS), Tone Reservation (TR), and Active Constellation Extension (ACE). Most of these approaches—except ACE—require side information, introduce distortion, or degrade BER performance. ACE stands out for several reasons:

- It avoids the need for side information, reducing signalling overhead.

- It adjusts only the outer constellation points, preserving the minimum Euclidean distance between symbols and maintaining BER performance.

- It is more computationally efficient than SLM and PTS, which involve multiple IFFT operations or complex optimization.

Figure 2: Performance of ACE based PAPR reduction compared to conventional CP-OFDM and DFT-S-OFDM

Figure 2 presents preliminary results comparing the PAPR of conventional CP-OFDM and DFT-S-OFDM with that of CP-OFDM employing a simple Smart Gradient Project (SGP) ACE algorithm, with evaluation assumptions provided in the Appendix section. The SGP method iteratively adjusts the constellation points of the OFDM signal to reduce PAPR while preserving signal integrity. The ACE algorithm can additionally be integrated with geometric shape modulation schemes that are being studied in the 6GR study. While DFT-S-OFDM inherently exhibits lower PAPR due to its single-carrier characteristics, ACE approach allows OFDM to achieve comparable PAPR levels without compromising flexibility or spectral efficiency.

Proposal 3: Enhancements to CP-OFDM, including PAPR reduction techniques such as ACE, should be evaluated in the 6GR study.

# Conclusions

We have briefly examined some of the challenges associated with introducing additional non-OFDM-based waveforms in the 6GR. Specifically, challenges related to MRSS compatibility, limitations in scheduling flexibility, and increased complexity in UE receiver and MIMO processing have been identified. In our view, these factors warrant careful examination and resolution before initiating any formal study on additional downlink waveform designs. As such, this contribution presents the following proposals and observations regarding the new waveforms and further studies on CP-OFDM enhancements:

Proposal 1: A rigorous evaluation of cross-waveform interference and CP-OFDM coexistence with non-orthogonal waveforms in MRSS operations is required when considering the adoption of an additional waveform for 6GR.

Proposal 2: RAN1 should evaluate the new waveform’s performance by considering practical scheduler implementations to ensure reliable performance results.

Observation: Non-orthogonal waveforms may necessitate advanced UE receiver architectures, which potentially impact the 6GR deployment timeline.

Proposal 3: Enhancements to CP-OFDM, including PAPR reduction techniques such as ACE, should be evaluated in the 6GR study.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[2] Chair notes RAN1#122 eom, Bengaluru, IN, Aug. 2025

[3] R1-2506952, Consideration for 6GR DL Waveform, Kyocera, RAN1#122-bis, Prague, CZ, Oct. 2025.

[4] R1-2507368, Waveform for 6GR Air Interface, Cohere Technologies, RAN1#122-bis, Prague, CZ, Oct. 2025.

[5] R1-2507507, Discussion on 6GR waveform, ETRI and Univ. of Surrey, RAN1#122-bis, Prague, CZ, Oct. 2025.

# Appendix

Table A below shows the evaluation assumptions for Figure 2 PAPR results in section 3.

Table A Simulation Assumptions

| Parameter | Value |
| --- | --- |
| SNR | -5 dB |
| Number of frames | 20 |
| Grid size | 52 PRBs |
| SCS | 30 kHz |
| Traffic direction | UL |
| Number of layers | 1 |
| Number of DM-RS symbols/slot | 2 |
| PUSCH transmission scheme | Non codebook |
| Number of Tx (UE) antennas | 2 |
| Number of Rx (gNB) antennas | 8 |
| Code rate | 193/1024 |
| SGP related parameters | Maximum number of iterations = 10 Maximum magnitude (A) = 4.86 dB Oversampling factor = 100 |