3GPP TSG RAN WG1 #123                                                                                   R1-2508570

Dallas, USA, November 17th – 21st, 2025

Source:	CATT

Title:	Remaining issues on NR MIMO Phase 5

Agenda Item:	8.2

Document for:	Discussion and Decision

Introduction

This contribution provides our views on remaining issues on enhancements for UEIBM and CSI for NR MIMO phase 5.

Discussion

Enhancements for UE-initiated/event-driven beam management

Multiplexing issue

For UEIBR, it has been agreed that 1-bit indication of first PUCCH is always multiplexed in the PUSCH when the 1-bit first PUCCH is collided/overlapped with a PUSCH. An issue to be solved is how to multiplex the L (L>=1) first PUCCH(s) with UEIRIs with a PUSCH when L first PUCCH(s) collide/overlap with a PUSCH. In RAN1 #122 meeting, this issue was discussed with the following candidate options provided:

- Option-1: A field of bit sequence with a length of L bit is piggyback into the PUSCH.

  - Each of bits in the bit sequence corresponds to respective first PUCCH(s) by an ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

- Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI

  - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR.

  - An all-zero value for the log2L+1 bits represents a UEIRI value across all L UEIRIs.

In TS38.213, it is specified that when a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation,

- first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6 in TS 38.213, if any

- second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 in TS 38.213

- third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 in TS 38.213

- fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause.

The rule should also be applied to UE-initiated/event-driven beam reporting. Regarding the multiplexing a number of L (L>=1) PUCCH format 0/1 with UEIRIs are collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, a field of log2L+1 bits of representing at most one of positive UEIRI is adopted. Therefore, for the multiplex the L (L>=1) first PUCCH(s) with UEIRIs with a PUSCH when L first PUCCH(s) collide/overlap with a PUSCH, i.e., Option-2 is supported.

Proposal 1: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, support option-2:

- Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI

  - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR.

  - An all-zero value for the log2L+1 bits represents a UEIRI value across all L UEIRIs.

Reference time unit for determining current beam

In RAN1#122 meeting, the definition of the time for deriving the current beam RS in a report instance, while considering the dynamical update of indicated TCI state was discussed [1]. If the indicated TCI state is updated, the indicated TCI state with the RS for the current beam that used to trigger the UEIBR may not be the current indicated TCI state. In RAN1 #122bis meeting, two cases are identified for further consideration [2]. One is the TCI state update happens between event-triggering and first PUCCH. The other is the TCI state update is between first PUCCH and second PUSCH. In the first case, if the TCI state is updated before the first PUCCH, the counter would be reset and the first PUCCH would not be transmitted. No enhancements are needed in this case. The second case can be avoided by gNB’s implementation. However, this may lead to performance loss as the scheduling flexibility from gNB is restricted. In fact, if the second case occurs, although the reported beam quality is not with the RS in the current indicated TCI state, gNB can still know the beams that have larger RSRPs. These pieces of information are beneficial for gNB to configure better beam to UE in the future. Thus, no enhancements are needed in this case.

Observation 1: Regarding UE-initiated/event-driven beam reporting, when the TCI state update happens between the event-triggering and the first PUCCH, the first PUCCH would be dropped.

Observation 2: Regarding UEIBR for UE-initiated/event-driven beam reporting, the reporting on the second PUSCH is beneficial to further indicate/configure beams for UE when the TCI state update happens between first PUCCH and second PUSCH carrying UEIBR.

Collision handling between SRS carrier switching and PUSCH carrying UEI CSI report for Mode A

In RAN1#121 meeting, it was agreed that the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A [3]. In RAN1 #122bis meeting, the agreement was captured in TS 38.214 for UE procedure for reporting channel state information and in TS 38.213 for UE procedure for reporting control information. The agreement can also be applied to clause 6.2.1.3 in TS 38.214 for collision handling between SRS carrier switching and PUSCH carrying UEI CSI report for Mode A in a similar way. The corresponding TP is provided bellow:

Reason for change

In RAN1#121 meeting, it was agreed that the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. The agreement should be captured in TS38.214 section 6.2.1.3 for collision handling between SRS carrier switching and PUSCH carrying UEI CSI report for Mode A.

Summary of change

In TS38.214 section 6.2.1.3, clarify that the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for PUSCH carrying UEI CSI report for Mode A.

Consequences if not approved

The UE behavior for collision handling between SRS carrier switching and PUSCH carrying UEI CSI report for Mode A is not clear.

Proposed TP for TS38.214:

#### 6.2.1.3 UE sounding procedure between component carriers <Irrelevant part is omitted> For the remaining of this clause, unless otherwise stated, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated report when reportTransmissionMode is configured as ‘ModeA’ in the CSI report configuration (as defined in Section 5.2.1.5.4.1). The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCPon a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. <Irrelevant part is omitted>

Measurement restriction (from in-coming LS from RAN4 (R4-2508391))

In RAN1#122 meeting, one LS on event triggered L1-RSRP reporting (R1-2506520/R4-2511656) was received. The details of the CR are provided in the following:

- 1 Overall description RAN4 is discussing measurement requirements for event-triggered L1-RSRP reporting. It is RAN4 common understanding that NW can configure “timeRestrictionForChannelMeasurement” for event-triggered L1-RSRP reporting. For Event-2, RAN4 agreed to define separate requirements for measurement period when the UE is configured or not configured with“timeRestrictionForChannelMeasurement”, similar to existing measurement period requirements. Specifically, for SSB-based and periodic and semi-persistent CSI-RS based L1-RSRP measurement, if timeRestrictionForChannelMeasurement is configured, the measurement period is based on M = 1, and event evaluation should be based on the most recent result from both new beam and current beam; If timeRestrictionForChannelMeasurement is not configured, the measurement period for current and new beam is based on M = 3. For aperiodic CSI-RS based L1-RSRP, M is always equal to 1. - 2 Actions To RAN1 ACTION: RAN4 respectfully asks RAN1 to take above information into account and provide feedback if there is any issue with the above RAN4 agreement.

During the discussions in RAN1# 122 meeting and RAN1 #122bis meeting, RAN1 did not identify any issues on the above RAN4 agreement of introducing “timeRestrictionForChannelMeasurement”. From RAN1’s aspect, one open question is whether “timeRestrictionForChannelMeasurement” can be configured for Event-1 and Event-7. As this is introduced for the measurement period for the UE, it is our view that it can be applied to all the trigger events for UEIBR.

In TS 38.214 section 5.2.1.4.3, it has already clearly stated that UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS associated with the CSI resource setting. In this case, no updates are needed in the current spec.

Proposal 2: Besides Event-2, timeRestrictionForChannelMeasurement can be configured to both Event-1 and Event-7.

CSI enhancements

The subband size indication of CJTC-P report

Reason for change:

In current TS 38.214, if the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-P', the granularity of phase offset report is given by the higher layer parameter subbandSize. However, the higher layer parameter subbandSize is used to configure the subband size of PMI report. The granularity of phase offset report should be given by the higher layer parameter subbandSizeCJTC in current TS 38.331, which is a different parameter from the subband size of PMI report.

Summary for change:

Change the parameter subbandSize in clause 5.2.1.4, 5.2.1.4.2 and 5.2.4.2.11 in TS 38.214 into subbandSizeCJTC to align with parameter in TS 38.331.

Consequences if not approved

Incorrect subband size of phase offset report is configured. UE cannot identify whether the phase offset should be reported in the form of wideband or subband.

Proposed TP for TS38.214:

- 5.2.1.4 Reporting configurations <Unrelated parts are omitted> - reportQuantity is set to 'cjtc-P' and subbandSizeCJTC is set to 'wideband', or <Unrelated parts are omitted> - 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-P', - the UE selects and reports a reference CSI-RS resource, nref, from the NTRP configured CSI-RS resources for channel measurement. - If the UE is configured with the higher layer parameter subbandSizeCJTC set to 'wideband', the UE reports the value of the phase offset, Φn, associated with each configured CSI-RS resource n=0,1,…,NTRP-1, n≠nref, and for the entire configured reporting band, relative to the reference CSI-RS resource, nref, as described in Clause 5.2.1.4.11. For the reference CSI-RS resource, nref, the value of Φnref is assumed zero and it is not reported, - otherwise, the UE is configured with subband size NPRBSB-P∈{1,2,4,8,16} given by the higher layer parameter subbandSizeCJTC. The UE reports the values of the phase offset, {Φn,s}, with s=0,1,…,NSB-P-1 for the NSB-P≤16 sub-bands indicated by the higher layer parameter numberOfSubbandsPO out of all sub-bands in the configured reporting band, and associated with each configured CSI-RS resource, n=0,1,…,NTRP-1, n≠nref, relative to the reference CSI-RS resource, nref. The first subband size is given by NPRBSB-P-NBWP,istartmodNPRBSB-P and the last subband size given by NBWP,istart+NBWP,isizemodNPRBSB-P if NBWP,istart+NBWP,isizemodNPRBSB-P≠0 and NPRBSB-Pif NBWP,istart+NBWP,isizemodNPRBSB-P=0. The values of {Φn,s} are obtained as described in Clause 5.2.1.4.11. For the reference CSI-RS resource, nref, the values of {Φnref,s} are assumed zero and not reported. <Unrelated parts are omitted> - 5.2.1.4.11 CJTC-P reporting of phase offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-P', if subbandSizeCJTC is set to 'wideband', the phase offset value, Φn , for CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by <Unrelated parts are omitted>

Restrictions on the configuration of CJTC-P report for NES

Reason for change:

In current TS 38.214, if the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList, the report quantities that UE does not expect to be configured introduced are specified. In Rel-19, some new report quantities are introduced. Specifically, the following new report quantities are introduced in Rel-19 for CJT calibration: 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P'. Since there is no agreement that the report quantities of CJT calibration can be configured when the UE is configured with NES, the report quantities that UE does not expect to be configured for NES should cover the new report quantities introduced for CJT calibration in Rel-19.

Summary for change:

Include the report quantities introduced for CJT calibration in Rel-19 to the list that UE does not expect to be configured when UE is configured with sub-configurations in clause 5.2.1.4.2 in TS 38.214.

Consequences if not approved

Report quantities 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P' and sub-configurations can be configured simultaneously. When it happens, UE does not know which information should be reported to gNB. Then UE’s reporting behavior would be unclear for gNB.

Proposed TP for TS38.214:

- 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList: <Unrelated parts are omitted> - The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', 'tdcp', 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P'. <Unrelated parts are omitted>

RRC parameter alignment for multi-CRI report

Reason for change:

In current TS 38.214, higher layer parameter mrSelectedResources is used to indicate MR<M CSI-RS resources to be selected for reporting, which is not aligned with the parameter mr-SelectedResources in TS 38.331.

Summary for change:

Change the parameter mrSelectedResources in clause 5.2.1.4.2, 5.2.3 and 5.2.4 in TS 38.214 into mr-SelectedResources to align with parameter in TS 38.331.

Consequences if not approved

Parameter name is not aligned between TS 38.214 and TS 38.331.

Proposed TP for TS38.214:

- 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> - the CSI report contains M CRIs with the exception that, for aperiodic reporting and subject to UE capability, if CSI-ReportConfig is configured with higher layer parameter mr-SelectedResources indicating MR<M CSI-RS resources to be selected for reporting, M-MR CRIs are reported. MR∈{1,2} if codebookType is set to 'typeI-SinglePanel' and MR∈{1}, if codebookType is set to 'typeII-r16'. <Unrelated parts are omitted> - 5.2.3 CSI reporting using PUSCH <Unrelated parts are omitted> - For Type I and refined Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4. For a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s). For a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for Type I and refined Type I single panel CSI feedback for one or more of the sub-configurations, Part 1 for a sub-configuration contains corresponding RI (if reported), CRI (if reported) and CQI for the first codeword (if reported). Part 2 for a sub-configuration contains the corresponding CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI (if reported). For a CSI-ReportConfig configured with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel', Part 1 contains M CRI(s), or M-MR CRI(s) if the higher layer parameter mr-SelectedResources is configured, M RI(s), M CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains M sets of: the CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI. - For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,1 are reported in the increasing order of their indices, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded. - For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10), Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), refined eType II (see Clause 5.2.2.2.5a), refined FeType II Port Selection (see Clause 5.2.2.2.9a) and refined eType II for predicted PMI with N4=1 (see Clause 5.2.2.2.11a), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1, Further Enhanced Type II Port Selection for predicted PMI, refined eType II, refined FeType II Port Selection or refined eType II for predicted PMI with N4=1. For a CSI-ReportConfig with higher layer parameter valueOfM, and codebookType set to 'typeII-r16', Part 1 contains M CRI(s), or M-MR CRI(s) if the higher layer parameter mr-SelectedResources is configured, M RI(s), M sets of CQI and is zero padded to a fixed payload size (if needed). Part 2 contains M PMI(s). Part 1 and 2 are separately encoded. <Unrelated parts are omitted> - 5.2.4 CSI reporting using PUCCH <Unrelated parts are omitted> When the PUCCH carry Type I or refined Type I CSI with wideband frequency granularity, the CSI payload carried by the PUCCH format 2 and PUCCH formats 3, or 4 are identical and the same irrespective of RI(s) (if reported), CRI(s) (if reported). A CSI-ReportConfig with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs can be configured with wideband frequency granularity only with csi-ReportMode set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 set to X=0. For Type I and refined Type I CSI sub-band reporting on PUCCH formats 3, or 4, the payload is split into two parts. The first part contains RI (if reported), CRI (if reported), CQI for the first codeword. The second part contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI > 4. For a CSI-ReportConfig configured with subband reporting, codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s). For a CSI-ReportConfig containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, and configured with subband reporting, for Type I or refined Type I single panel CSI for one or more of the sub-configurations, Part 1 for a sub-configuration contains corresponding RI (if reported), CRI (if reported) and CQI for the first codeword (if reported). Part 2 for a sub-configuration contains the corresponding CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI (if reported). For a CSI-ReportConfig configured with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', codebookType set to 'typeI-SinglePanel', and configured with subband reporting, Part 1 contains M CRI(s) or M-MR CRI(s), if the higher layer parameter mr-SelectedResources is configured, M RI(s), M CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains M sets of: the CQI for the second codeword (if reported), LI (if reported) and PMI. <Unrelated parts are omitted>

Conclusion

In this contribution, we provide our views on remaining issues of NR MIMO phase 5, with the following observations, proposals and TPs provided:

UE-initiated/event-driven beam management

Observation 1: Regarding UE-initiated/event-driven beam reporting, when the TCI state update happens between the event-triggering and the first PUCCH, the first PUCCH would be dropped.

Observation 2: Regarding UEIBR for UE-initiated/event-driven beam reporting, the reporting on the second PUSCH is beneficial to further indicate/configure beams for UE when the TCI state update happens between first PUCCH and second PUSCH carrying UEIBR.

Proposal 1: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, support option-2:

- Option-2: A field of log2L+1 bits of representing at most one of positive UEIRI

  - The codepoints other than all-zero value in the field are ordered based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs to represent a positive UEIBR.

  - An all-zero value for the log2L+1 bits represents a UEIRI value across all L UEIRIs.

Proposal 2: Besides Event-2, timeRestrictionForChannelMeasurement can be configured to both Event-1 and Event-7.

TP1: Proposed TP for TS38.214 on collision handling between SRS carrier switching and PUSCH carrying UEI CSI report for Mode A：

#### 6.2.1.3 UE sounding procedure between component carriers <Irrelevant part is omitted> For the remaining of this clause, unless otherwise stated, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated report when reportTransmissionMode is configured as ‘ModeA’ in the CSI report configuration (as defined in Section 5.2.1.5.4.1). The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCPon a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. <Irrelevant part is omitted>

CSI enhancements

TP 2: Proposed TP for TS38.214 on the subband size indication of CJTC-P report

- 5.2.1.4 Reporting configurations <Unrelated parts are omitted> - reportQuantity is set to 'cjtc-P' and subbandSizeCJTC is set to 'wideband', or <Unrelated parts are omitted> - 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-P', - the UE selects and reports a reference CSI-RS resource, nref, from the NTRP configured CSI-RS resources for channel measurement. - If the UE is configured with the higher layer parameter subbandSizeCJTC set to 'wideband', the UE reports the value of the phase offset, Φn, associated with each configured CSI-RS resource n=0,1,…,NTRP-1, n≠nref, and for the entire configured reporting band, relative to the reference CSI-RS resource, nref, as described in Clause 5.2.1.4.11. For the reference CSI-RS resource, nref, the value of Φnref is assumed zero and it is not reported, - otherwise, the UE is configured with subband size NPRBSB-P∈{1,2,4,8,16} given by the higher layer parameter subbandSizeCJTC. The UE reports the values of the phase offset, {Φn,s}, with s=0,1,…,NSB-P-1 for the NSB-P≤16 sub-bands indicated by the higher layer parameter numberOfSubbandsPO out of all sub-bands in the configured reporting band, and associated with each configured CSI-RS resource, n=0,1,…,NTRP-1, n≠nref, relative to the reference CSI-RS resource, nref. The first subband size is given by NPRBSB-P-NBWP,istartmodNPRBSB-P and the last subband size given by NBWP,istart+NBWP,isizemodNPRBSB-P if NBWP,istart+NBWP,isizemodNPRBSB-P≠0 and NPRBSB-Pif NBWP,istart+NBWP,isizemodNPRBSB-P=0. The values of {Φn,s} are obtained as described in Clause 5.2.1.4.11. For the reference CSI-RS resource, nref, the values of {Φnref,s} are assumed zero and not reported. <Unrelated parts are omitted> - 5.2.1.4.11 CJTC-P reporting of phase offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-P', if subbandSizeCJTC is set to 'wideband', the phase offset value, Φn , for CSI-RS resource n=0,…,NTRP-1, n≠nref, is indicated by <Unrelated parts are omitted>

TP 3: Proposed TP for TS38.214 on the restrictions on the configuration of CJTC-P report for NES

- 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList: <Unrelated parts are omitted> - The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', 'tdcp', 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P'. <Unrelated parts are omitted>

TP 4: Proposed TP for TS38.214 on the RRC parameter alignment for multi-CRI report

- 5.2.1.4.2 Report quantity configurations <Unrelated parts are omitted> - the CSI report contains M CRIs with the exception that, for aperiodic reporting and subject to UE capability, if CSI-ReportConfig is configured with higher layer parameter mr-SelectedResources indicating MR<M CSI-RS resources to be selected for reporting, M-MR CRIs are reported. MR∈{1,2} if codebookType is set to 'typeI-SinglePanel' and MR∈{1}, if codebookType is set to 'typeII-r16'. <Unrelated parts are omitted> - 5.2.3 CSI reporting using PUSCH <Unrelated parts are omitted> - For Type I and refined Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4. For a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s). For a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for Type I and refined Type I single panel CSI feedback for one or more of the sub-configurations, Part 1 for a sub-configuration contains corresponding RI (if reported), CRI (if reported) and CQI for the first codeword (if reported). Part 2 for a sub-configuration contains the corresponding CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI (if reported). For a CSI-ReportConfig configured with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel', Part 1 contains M CRI(s), or M-MR CRI(s) if the higher layer parameter mr-SelectedResources is configured, M RI(s), M CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains M sets of: the CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI. - For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,1 are reported in the increasing order of their indices, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded. - For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10), Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), refined eType II (see Clause 5.2.2.2.5a), refined FeType II Port Selection (see Clause 5.2.2.2.9a) and refined eType II for predicted PMI with N4=1 (see Clause 5.2.2.2.11a), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1, Further Enhanced Type II Port Selection for predicted PMI, refined eType II, refined FeType II Port Selection or refined eType II for predicted PMI with N4=1. For a CSI-ReportConfig with higher layer parameter valueOfM, and codebookType set to 'typeII-r16', Part 1 contains M CRI(s), or M-MR CRI(s) if the higher layer parameter mr-SelectedResources is configured, M RI(s), M sets of CQI and is zero padded to a fixed payload size (if needed). Part 2 contains M PMI(s). Part 1 and 2 are separately encoded. <Unrelated parts are omitted> - 5.2.4 CSI reporting using PUCCH <Unrelated parts are omitted> When the PUCCH carry Type I or refined Type I CSI with wideband frequency granularity, the CSI payload carried by the PUCCH format 2 and PUCCH formats 3, or 4 are identical and the same irrespective of RI(s) (if reported), CRI(s) (if reported). A CSI-ReportConfig with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs can be configured with wideband frequency granularity only with csi-ReportMode set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 set to X=0. For Type I and refined Type I CSI sub-band reporting on PUCCH formats 3, or 4, the payload is split into two parts. The first part contains RI (if reported), CRI (if reported), CQI for the first codeword. The second part contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI > 4. For a CSI-ReportConfig configured with subband reporting, codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s). For a CSI-ReportConfig containing a list of sub-configurations provided by csi-ReportSubConfigToAddModList, and configured with subband reporting, for Type I or refined Type I single panel CSI for one or more of the sub-configurations, Part 1 for a sub-configuration contains corresponding RI (if reported), CRI (if reported) and CQI for the first codeword (if reported). Part 2 for a sub-configuration contains the corresponding CQI for the second codeword (if reported) when RI is larger than 4, LI (if reported) and PMI (if reported). For a CSI-ReportConfig configured with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', codebookType set to 'typeI-SinglePanel', and configured with subband reporting, Part 1 contains M CRI(s) or M-MR CRI(s), if the higher layer parameter mr-SelectedResources is configured, M RI(s), M CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains M sets of: the CQI for the second codeword (if reported), LI (if reported) and PMI. <Unrelated parts are omitted>

Reference

- Chairman’s Notes, 3GPP RAN1 #122, Bangalure, India, August, 25th – 29th, 2025.

- Chairman’s Notes, 3GPP RAN1 #122bis, Prague, Czech, October 13th – 17th, 2025.

- Chairman’s Notes, 3GPP RAN1 #121, St Julian’s, Malta, May 19th – 23rd, 2025.