3GPP TSG RAN WG1 #123                                                                                 R1-2508716

Dallas, USA, Nov 17th – 21st, 2025

Source:	OPPO

Title:	Discussion on enhancement of SRS capacity and coverage

Agenda Item:	10.2.1

Document for:	Discussion and Decision

Introduction

In RAN1#122bis meeting, SRS enhancement for MIMO Phase 6 was discussed with the following agreements:

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop and K=R (i.e., each subgroup includes R/K=1 symbol (or 1 pair of consecutive symbols for 8-Tx SRS with ports8tdm))), the following configuration combinations and basic starting position patterns are supported

- when PF=K=2, support pattern {0,1}

- when PF=4 and K=2, select one of the following patterns:

  - Alt 1: {0,2}

  - Alt 2: {0,1}

- when PF=K=4, select one of the following patterns:

  - Alt 1: {0,2,1,3}

  - Alt 2: {0,1,2,3}

Note: exact pattern(s) are deduced from the basic patterns. FFS details.

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-1 from the agreement in RAN1#122bis as the following:

- when PF=4 and K=2, select one of the following patterns:

  - Alt 1: {0,2}

- when PF=K=4, select one of the following patterns:

  - Alt 1: {0,2,1,3}

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop and K<R (i.e., each subgroup includes RK>1 symbol (or >1 pair of consecutive symbols for 8-Tx SRS with ports8tdm)), support the following configuration combinations and basic starting position patterns

- when PF=K=2, select at least one of the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,1,…,1}

  - Alt 2 (non-consecutive mapping): {0,1,,…,0,1}

- when PF=4 and K=2, down select from the following basic patterns:

  - Alt 1-1 (consecutive mapping):{0,…,0,2,…,2}

  - Alt 1-2 (consecutive mapping):{0,...,0,1,…,1}

  - Alt 2-1 (non-consecutive mapping):{0,2,…,0,2}

  - Alt 2-2 (non-consecutive mapping): {0,1,…,0,1}

- when PF=K=4, down select from the following basic patterns:

  - Alt 1-1 (consecutive mapping): {0,…,0,2,…,2,1,…,1,3,…,3}

  - Alt 1-2 (consecutive mapping): {0,…,0,1,…,1,2,…2,3,…,3}

  - Alt 2-1 (non-consecutive mapping): {0,2,1,3,…,0,2,1,3}

  - Alt 2-2 (non-consecutive mapping): {0,1,2,3,…,0,1,2,3}

Note: exact patterns are deduced from the basic patterns. FFS details.

Working Assumption:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-2:

- when PF=K=2, select at least one of the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,1,…,1}

  - Alt 2 (non-consecutive mapping): {0,1,,…,0,1}

- when PF=4 and K=2, down select from the following basic patterns:

  - Alt 1 (consecutive mapping):{0,…,0,2,…,2}

  - Alt 2 (non-consecutive mapping): {0,2,…,0,2}

- when PF=K=4, down select from the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,2,…,2,1,…,1,3,…,3}

  - Alt 2 (non-consecutive mapping): {0,2,1,3,…,0,2,1,3}

Agreement

To determine the time-domain location of a cross-slot SRS resource,

- ‘l0’ is interpreted as the starting symbol in the starting slot for an SRS resource.

- ‘l'+l0’ is interpreted as the symbol index in a slot or two consecutive S and U slots.

- The restriction ‘loffset≥NsymbSRS-1’ is removed.

Agreement

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency, support periodic, semi-persistent and aperiodic SRS.

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support all SRS usages (i.e., codebook, nonCodebook, antennaSwitching and beamManagement)

Agreement:

- Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

- Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

Agreement:

- For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set.

- Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately.

- Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately.

- Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

In this contribution, we further discuss the issues on SRS capacity and coverage for Rel-20 MIMO.

Discussion

  - Multiple frequency-domain starting positions for RPFH

In the past meetings, it was agreed that the repetition symbols for RPFH are equally divided into K subgroups and different starting positions can be applied across the K subgroups. It was also discussed whether to allocate concecutive or non-consecutive symbols within each subgroup and non-consecutive mapping was agreed as working assumpiton. In our understanding, there is not much difference between concecutive or non-consecutive symbols mapping.

- From the aspect of UE multiplexing, non-consecutive symbols mapping is beneficial for multiplexing among Rel-20 UE with different values of R, while consecutive symbols mapping could support multiplexing with legacy UEs configured with SRS repetition.

- From the aspect of performance, non-consecutive symbols mapping is beneficial for interpolation in frequency domain, while consecutive symbols mapping could support combinaiton among consecutive SRS symbol for bettter channel estimation.

- For the issues of phase continutiy, we don’t think it would inhibit frequency interpolation within a hop. Even for DMRS bundling for coverage enhancement, the interpolation within large PUSCH bandwidth among consecutive symbols can be supported. The phase continuty within bandwidth of one SRS hop should not be an issue considering the same RF would be applied. If there is, then multiple frequency-domain starting positions for RPFH would be meaningless.

In our understanding, the two opitions are similar and it is unnecessary to support both options. Cconsidering compatibility among different repetition numbers (the same pattern for different R with truncation), non-consecutive mapping is slightly preferred.

Fig.1: Example of cyclic mapping and sequential mapping

Proposal 1: For K subgroups in R repetition symbols within each SRS frequency hop, confirm the working assumption on Alt-2.

In current 38.211, the following frequency offset is introduced within each hop for RPFS:

noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

-	kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0;

-	khop is given by Table 6.4.1.4.3-3 with values of {0, 2} or {0, 2, 1, 3}

To support multiple frequency-domain starting positions, the formula can be updated with additional frequency offset, e.g.

noffsetRPFS=NscRBmSRS,BSRSkF+khop+krepmod PFPF,

where krep is determined by r, and r∈0,1,…,R-1 is the repetition symbol index within a hop for SRS frequency hopping. The value of krep is depended on the pattern for the strating position, e.g. cyclic mapping or sequencial hopping.

Proposal 2: Modify the frequency offset for RPFS (e.g. noffsetRPFS) to support starting positions hopping within a hop across repetition symbols.

In Rel-17, the starting RB index for RPFS could hop across different periods. If multiple starting positions within one SRS frequency hop are configured, the channel of the whole hop can be estimated by interpolation. In this case, different starting position across different periods could not bring additional benefit since the position is not important. Hence, these two features are not needed to be configured simultaneously.

Proposal 3: It seems unnecessary to support both of legacy RPFS start RB index hopping across multiple periods and intra-repetition hopping within each SRS frequency hop simultaneously.

  - Cross-slot SRS between one U slot and one adjacent S slot

Agreement: Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. - Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. Agreement: - For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set. - Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately. - Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately. - Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

To support one aperiodic SRS resource (set) to be configured across a S slot and U slot, some modificaiton to the SRS resource configuration should be introduced. At least the following two scenarios were discussed in RAN1#122bis  meeting:

- Scenario 1:  Cross-slot SRS resource

  - Case 1-1: Allow a SRS resource with starting symbol in a configured S slot for resource set and SRS symbols across the S slot and next U slot (Fig.2a). For this case, current paramter configuration can be fully reused. For the SRS resource across the slot boundary, 1≤loffset<NsymbSRS-1. For the other SRS resources with in one slot, loffset≥NsymbSRS-1. The SRS rsource set is still configured in single slot (Slot S).

  - Case 1-2: Allow a SRS resource across the S slot and U slot and at least one another resource in the U slot (Fig.2b). For this case, some standardizatoin effort on the SRS parameters is needed to indicate the starting symbol of one resource is in previous slot or current slot (as legacy). For example, one additional bit in one of the SRS resources to indicate the starting symbol index is corresponding to the previous S slot. In this caes, the SRS rsource set can still be configured in single slot (slot offset corresponds to Slot U). This case in scenario 1 has not been confirmed by RAN1 based on the Note.

Fig.2: Example of cross-slot SRS transmission for a SRS resource set (single slot configuration)

- Scenario 2: Cross-slot SRS resource set

  - By introducing per resource slot configuation (e.g. slot offset refer to the legacy slot for SRS resource set with value of 0 or 1), a SRS resource set can occupy symbols in adjacent S+U slots. As shown in Fig.3a and 3b, one SRS resource in a resource set can be allocated in the S slot, U slot or across S and U slot.

  - Though there is cross-slot SRS resource in 3a, it is not included in scenario 1 since it is combination of scenario 1 and scenario 2. The prerequisite of this combination is support of both scenario 1 and scenario 2 first.

Fig.3: Example of cross-slot SRS transmission for a SRS resource set (multi-slot configuration)

In the past meetings, Case 1-1 in scenario 1 was agreed and FFS for Case 1-2 and scenario 2.

- Case 1-2 is more common than Case 1-1 since there are more UL symbols in U slot which can be used for SRS. There is no reason to preclude Case 1-2. For Case 1-2, the slot offset of the SRS resource set can refer to the second of the two slots, which is used for both normal SRS resource and cross-slot SRS resuorce.

- The main difference between the two scenarios is whether to allow multiple slots for one SRS resource set. By now, we haven’t found the use case for scenario 2. Even for antenna switching SRS, one slot is sufficient for each resource set.  Considering additional standardization effort and backward compatibility issue to support scenario 2, we think scenario 1 is sufficient.

Proposal 4: For a aperiodic SRS resource set including cross-slot SRS resource, all the SRS resources should be allcoated in the same slot (U or S slot) as legacy except the cross-slot SRS resource.

- Case 1-2 (interpretation 2) is supported in scenario 1, and one bit signaling can be introduced for Case 1-2 (interpretation 2) to indicate the starting symbol of the resource is in previous slot.

- Scenario 2 is not supported.

Proposal 2-9: ‌If a slot contains only one resource from one resource set for usage of antenna switching, and this resource spans consecutive S and U slots, then another resource set for usage of antenna switching is allowed to be configured/triggered in that slot.‌

For cross-slot SRS, whether to support configuring/triggering multiple aperiodic SRS resource sets in a slot for usage of antenna switching was discussed without conclusions. In our understanding, at least the four cases in Fig.4 should be considered.

- Case 1 and 2 are the target scenarios of cross-slot SRS and could be supported. However, we are not sure whether the two resource sets can be assumed to be in the same slot since actually different slot offsets are configured for the resource sets.

- Case 3 should not be considered in Rel-20, which it is inconsistent with the WID. This case would also impact the legacy SRS transmission even without cross-slot SRS.

- Case 4 is based on scenario 2 above, and will not be supported before scenario 2 is agreed.

Fig4: Different cases for multiple aperiodic SRS resource sets in a slot

To support Case 1 and Case 2 above, we have the following proposal:

Proposal 5: Support configuring/triggering a SRS resource set for antenna switching in a slot including only one resource from another SRS resource set for antenna switching if the resource is cross-slot SRS resource.

Agreement: When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell.

In Rel-16,  PUSCH with priority index 0 and corresponding DMRS should be transmitted before SRS for prioritized URLLC scheduling. With cross-slot SRS, to avoid scheduling restriction for PUSCH with priority index 0, the cross-slot SRS resource is allowed to be transmitted before PUCCH with priority index 0. However, for normal SRS resource without crossing slot, the legacy rule introduced by URLLC should not be broken. That is, PUSCH with priority index 0 and corresponding DMRS should be still transmitted before a non-cross-slot SRS resource.

Proposal 6: Transmitting PUSCH with a priority index 0 and corresponding DMRS after non-cross-slot SRS resource is not supported.

Agreement: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, study the following alternatives and down-select one to determine “available slot” in the next meeting: - Alt-0 (per SRS resource set): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-1 (per SRS resource): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-2 (per slot): An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same slot offset in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

For the defintion of available slot, three alternatives were proposed for further study.

- Alt-0 can support aperiodic cross-slot SRS resource with current agreement of scenario 1.

- Alt-1 fully breaks the legacy restriction to available slot. It is out of scope since it contains many cases for available slot which are not related to S+U SRS transmission. For example, there are UL or flexible symbol(s) for only one of the SRS resources in the resource set (this resource may not be cross-slot SRS), while all the other resources are invalid..

- Alt-2 has similar issue as Alt-1. For example, there is a slot satisfying there are UL or flexible symbol(s) for one of SRS resource(s) with same slot offset in the SRS resource set (this resource may not be cross-slot SRS), while all the other resources are invalid.

We think Alt-1 and Alt-2 go far away from original definition of available slot, and Alt-0 should be agreed.

Proposal 7: For the extended definition of available slot for cross-slot SRS, support Alt-0.

For aggregated symbols across U+S slots, “intra-slot” frequency hopping can also be considered. The essential issue for frequency hopping is how to derive the frequency hopping number n_SRS. If only Scenario 1 is supported, for a SRS resource with symbols across slots, the definition of slot index, SRS symbol number, repetition number and symbol index with a SRS resource remains unchanged. Then the current formula to derive n_SRS can be fully reused. If Scenario 2 above is agreed, different resources within a resource set may be configured with different slot offsets within a set. However, as long as the slot offset for a SRS resource can be well defined by per-resource slot offset indication, the formula to derive n_SRS can still be reused.

Fig.4: Example for frequency hopping across (U+S) slots

Proposal 8: For frequency hopping only or combination of frequency hopping and repetition across (U+S) slots, current formula to determine n_SRS can be fully reused via appropriate definition of slot offset.

In Rel-20 MIMO, multiple frequency-domain starting positions for RPFH are also discussed as in section 2.1. For a UE supporting both multiple frequency-domain starting positions for RPFH and SRS repetition across U+S slots, these two features may be simultaneously configured. If the FD starting positions are only determined by the repetition symbol index within a hop for SRS frequency hopping, it is expected that no additional specificiation impact is needed to support multiple frequency-domain starting positions for RPFH in multiple sysbols across (U+S) slots for a SRS resource.

Proposal 9: It seems no additional specificiation impact to support multiple frequency-domain starting positions for RPFH in multiple sysbols across (U+S) slots.

Conclusion

In this contribution, we discuss potential enhancement on SRS for NR MIMO phase 6 with the following proposals:

Proposal 1: For K subgroups in R repetition symbols within each SRS frequency hop, confirm the working assumption on Alt-2.

Proposal 2: Modify the frequency offset for RPFS (e.g. noffsetRPFS) to support starting positions hopping within a hop across repetition symbols.

Proposal 3: It seems unnecessary to support both of legacy RPFS start RB index hopping across multiple periods and intra-repetition hopping within each SRS frequency hop simultaneously.

Proposal 4: For a aperiodic SRS resource set including cross-slot SRS resource, all the SRS resources should be allcoated in the same slot (U or S slot) as legacy except the cross-slot SRS resource.

- Case 1-2 (interpretation 2) is supported in scenario 1, and one bit signaling can be introduced for Case 1-2 (interpretation 2) to indicate the starting symbol of the resource is in previous slot.

- Scenario 2 is not supported.

Proposal 5: Support configuring/triggering a SRS resource set for antenna switching in a slot including only one resource from another SRS resource set for antenna switching if the resource is cross-slot SRS resource.

Proposal 6: Transmitting PUSCH with a priority index 0 and corresponding DMRS after non-cross-slot SRS resource is not supported.

Proposal 7: For the extended definition of available slot for cross-slot SRS, support Alt-0.

Proposal 8: For frequency hopping only or combination of frequency hopping and repetition across (U+S) slots, current formula to determine n_SRS can be fully reused via appropriate definition of slot offset.

Proposal 9: It seems no additional specificiation impact to support multiple frequency-domain starting positions for RPFH in multiple sysbols across (U+S) slots.