3GPP TSG RAN WG1 #123                             	     R1-2508746

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	   11.4.2

Source: 	   Tejas Networks Ltd.

Title:            Discussion on modulation for 6GR

Document for:   Discussion and decision

# Introduction

In the RAN #108 meeting, a new Study Item (SI) was approved to initiate the exploration of a next-generation radio access technology (RAT), referred to as 6G Radio (6GR) [1]. One of the critical areas requiring in-depth investigation is modulation. The selection of modulation schemes is a crucial fundamental aspect of wireless communication systems. As a core element of physical layer design, modulation schemes significantly influence both the achievable communication capacity and the robustness of wireless signals. The upper bound on the channel capacity is stated by Shannon in his seminal work. The highest capacity for an AWGN channel is obtained when the transmitted signal presents a Gaussian distribution. However, most of the previous generation radio access technologies developed by 3GPP used uniformly distributed constellation symbols. This results in a loss of capacity. This loss can be recovered by either geometric shaping or probabilistic shaping. During RAN1 meeting #122 and meeting #123 following agreements are achieved, which enable further study of modulation techniques for 6G Radio air interface.

Agreements from RAN1 meeting #122

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Agreements from RAN1 meeting #123

Agreement For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following: - BLER performance under AWGN channel (at least for performance calibration) - 1st transmission (baseline) and with HARQ re-transmission - BLER performance under fading channel with fixed MCS - 1st transmission (baseline) and with HARQ re-transmission - Throughput performance with link adaptation (adaptive MCS and rank) under fading channel - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions) - Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements, - Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR - Expected spec impact - FFS detailed assumption of constellation shaping and improved MCS table - System level evaluation can be done after link level evaluation.

Agreement For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated. - channel configuration, including Channel profiles,Tx/RX antenna settings - For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available. - Precoder assumption - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based) - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions, - or genie beamforming - Open loop MIMO - Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL - LLR demapper: Max-log (baseline) or Log-MAP - Channel estimation: Realistic (baseline) or ideal - Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage Note: For MIMO, SIMO, MISO and SISO are included when possible

Agreement For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR

Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

In this contribution, we provide analysis on geometric shaping, particularly non-uniform constellation (NUC).

# Non-Uniform Constellation (NUC)

## Overview

Radio Access Technologies (RAT) developed by 3GPP, such as 5G-NR and LTE, have adopted uniform constellations (UC) as their modulation scheme due to their simplicity in mapping and de-mapping processes, despite the associated capacity loss. A uniform constellation features points that are equally spaced along both the In-phase (I) and Quadrature-phase (Q) axes. A uniform constellation with a square shape further offers low complexity de-mapping at the receiver, as both I and Q components can be separated using two simple pulse amplitude modulation (PAM) de-mappers. An example of a uniform constellation is illustrated in Figure 1.

A non-uniform constellation (NUC), as the name implies, features constellation points that are unevenly distributed in the IQ plane. The arrangement of these points is optimized based on the underlying bit-interleaved coded modulation (BICM) chain, operating SINR, channel frequency selectivity etc. NUCs have been adopted in the ATSC 3.0 next-generation terrestrial broadcast standard [2], which is being deployed worldwide. Non-uniform constellations can be categorized into 1-Dimensional Non-Uniform Constellation (1D-NUC) and 2-Dimensional Non-Uniform Constellation (2D-NUC).

1- Dimensional Non-Uniform Constellation (1D-NUC)

In 1D-NUC, constellation points are non-uniformly placed in the I and Q planes independently. This allows for the independent de-mapping of I and Q components, which reduces demodulation complexity and enables parallelization. An example 1D-NUC is illustrated in Figure 2.

2- Dimensional Non-Uniform Constellation (2D-NUC)

In 2D Non-Uniform Constellation (2D-NUC) schemes, constellation points are jointly and non-uniformly distributed across the I and Q planes. Unlike 1D-NUC or uniform constellations, this joint placement introduces an additional degree of freedom, which enhances shaping gain. However, it also increases the complexity of the de-mapping process, as the I and Q components cannot be treated independently. An example 2D-NUC is illustrated in Figure 2.

Figure 1: Example Uniform Constellation

Figure 2: Example 1D and 2D Non-Uniform Constellation

## NUC Design

NUC can be designed using various methods; one such design method, based on optimizing BICM capacity, is provided in [3]. The final shape of the designed constellation depends upon the SNR, channel fading, and the BICM chain adapted. Figure 3 and Figure 4 illustrate the designed 1D-NUC and 2D-NUC for MCS 7, 13, and 20 of the MCS Index Table 2 specified in 38.214.

Figure 3: 1D-NUC Constellation MCS: [7 13 20]

Figure 4: 2D-NUC Constellation MCS: [7 13 20]

## NUC De-mapping complexity

The de-mapping process in a wireless receiver involves computing the soft decisions for each bit of the received symbols. These soft decisions can be calculated using the log-likelihood ratio (LLR). In an AWGN channel with received symbol y=x+n  the bit-wise log-likelihood ratio (LLR) for ith bit bi is given by the exact expression.

LLRbiy=lnc∈C0ie-y-c2σ2c∈C1ie-y-c2σ2                                           1

where C0i and C1i are the sub-constellations, whose ith bit is 0 and 1, respectively, σ2 is the noise variance, x  is the transmitted symbol and n is the noise. While equation (1) is optimal, evaluating the two log-sum-exp terms over many constellation points is computationally expensive. Consequently, practical receivers commonly adopt the max-log approximation, which replaces each log-sum-exp by its dominant term, yielding

LLRbiy≈ 1σ2 minc∈C1iy-c2-minc∈C0iy-c2                      (2)

Equation (2) eliminates exponentials and logarithms and reduces the computation to a small number of Euclidean distance evaluations and comparisons, delivering near-optimal performance at moderate-to-high SNR with a fraction of the complexity of equation (1).

LLR Computation of 1D-NUC

In the case of 1D-NUC, the in-phase (I) and quadrature (Q) components can be processed independently, which further reduces computational complexity. For the in-phase component, equation (2) simplifies to:

LLRbiyr≈ 1σ2 mincr∈Cr,1iyr-cr2-mincr∈Cr,0iyr-cr2                      3

Where yr denotes the real part of the received symbol y, subset Cr,1i and Cr,0i contains the real part of the C0i and C1i respectively. A similar expression applies to the quadrature component. Notably, LLR computation for a uniform constellation also uses equation (3); therefore, the complexity of LLR computation for uniform constellations and 1D-NUC is essentially the same. Further reduction in computational complexity can be achieved through piecewise linear interpolation, with the interpolation coefficients stored in a look-up table (LUT).

LLR Computation of 2D-NUC

In the case of 2D-NUC, the in-phase (I) and quadrature (Q) components need to be processed jointly. Thus, equation (2) cannot be simplified further. While 2D-NUC offers enhanced shaping gain, it incurs a higher computational load for LLR calculations.

A closer analysis of equations (2) and (3) indicates that the dominant contributor to computational complexity in LLR calculation is the evaluation of Euclidean distances between the received symbol and all candidate constellation points. To quantify this complexity, it is useful to compare the number of arithmetic operations (additions and multiplications) required for distance computations in 1D-NUC and 2D-NUC schemes. Table 1 summarizes the total computational requirements for various QAM constellations under both 1D-NUC and 2D-NUC mappings.

Table 1: Euclidean distance calculation complexity of 1D-NUC and 2D-NUC

| QAM | 1D-NUC and UC |  | 2D-NUC |  |
| --- | --- | --- | --- | --- |
|  | Additions | Multiplication | Additions | Multiplication |
| 16 | 8 | 8 | 48 | 32 |
| 64 | 16 | 16 | 128 | 192 |
| 256 | 32 | 32 | 512 | 768 |
| 1024 | 64 | 64 | 2048 | 3072 |
| 4096 | 128 | 128 | 8192 | 12288 |

An analysis of the computations summarized in Table 1 clearly indicates that 2D-NUC requires substantially more operations compared to 1D-NUC, resulting in significantly higher computational complexity. However, by leveraging the inherent symmetry in the constellation design, it is possible to reduce the computational burden.

Observation 1 :- The de-mapping complexity of 1D-NUC is comparable to that of a uniform constellation, as both require a similar number of operations for LLR computation.

Observation 2 :- The de-mapping complexity of 2D-NUC is significantly higher than that of 1D-NUC due to the need for joint processing of in-phase and quadrature components.

## NUC Performance Evaluation

Performance evaluations under AWGN channel and fading channel are conducted to assess the shaping gain of 1D-NUC and 2D-NUC for 16,64 and 256QAM corresponding to MCS index 5 to 27 of MCS Table 2 specified in 38.214. NUC Constellation values and bit labelling used in the simulation are provided in our companion contribution [4]. In AWGN simulations, only the PDSCH BICM chain is modeled, excluding the remaining processing steps involved in PDSCH waveform generation. In contrast, fading channel simulations incorporate both the BICM chain and the complete PUSCH waveform generation process, along with the corresponding receiver-side processing steps. These simulations also account for HARQ retransmission mechanisms.

### AWGN channel

Link-level simulations (LLS) under an AWGN channel are conducted to evaluate shaping gain and facilitate calibration and alignment among contributing companies.

LLS parameters used under AWGN channel condition are summarized in Table 2 while the shaping gain obtained from the LLS simulations are summarized in Table 3 and Figure 5.

Table 2: LLS Simulation Parameters for AWGN channel

| Parameter | Value |
| --- | --- |
| BICM Chain | 5G-NR PDSCH BICM chain |
| Channel | AWGN |
| LDPC Algorithm | BP, Number of Iteration = 50 |
| Number of Code Blocks simulated | 1000 |
| Transport Block Size | 8400 |
| Antenna Configuration | 1T1R |
| De-mapper | ML De-mapper |
| Target BLER | 1% |
| Channel Estimation | Perfect |

Table 3:  Shaping gain of 1D-NUC and 2D-NUC for AWGN Channel

| MCS Index | Spectral efficiency | Required SNR (dB) @ 1% BLER |  |  | Shaping Gain dB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UC | 1D-NUC | 2D-NUC | 1D-NUC | 2D-NUC |
| 5 | 1.4766 | 3.90 | 3.90 | 3.80 | 0.00 | 0.10 |
| 6 | 1.6953 | 4.90 | 4.90 | 4.80 | 0.00 | 0.10 |
| 7 | 1.9141 | 5.80 | 5.70 | 5.60 | 0.10 | 0.20 |
| 8 | 2.1602 | 6.80 | 6.60 | 6.60 | 0.20 | 0.20 |
| 9 | 2.4063 | 7.70 | 7.70 | 7.60 | 0.00 | 0.10 |
| 10 | 2.5703 | 8.40 | 8.30 | 8.30 | 0.10 | 0.10 |
| 11 | 2.7305 | 9.50 | 9.10 | 9.00 | 0.40 | 0.50 |
| 12 | 3.0293 | 10.40 | 10.20 | 9.90 | 0.20 | 0.50 |
| 13 | 3.3223 | 11.40 | 11.20 | 10.80 | 0.20 | 0.60 |
| 14 | 3.6094 | 12.30 | 12.00 | 11.80 | 0.30 | 0.50 |
| 15 | 3.9023 | 13.30 | 13.00 | 12.80 | 0.30 | 0.50 |
| 16 | 4.2129 | 14.30 | 14.00 | 13.80 | 0.30 | 0.50 |
| 17 | 4.5234 | 15.30 | 15.10 | 15.00 | 0.20 | 0.30 |
| 18 | 4.8164 | 16.30 | 16.10 | 16.00 | 0.20 | 0.30 |
| 19 | 5.1152 | 17.40 | 17.20 | 17.10 | 0.20 | 0.30 |
| 20 | 5.3320 | 18.20 | 17.50 | 17.40 | 0.70 | 0.80 |
| 21 | 5.5547 | 18.80 | 18.30 | 18.20 | 0.50 | 0.60 |
| 22 | 5.8906 | 19.80 | 19.40 | 19.20 | 0.40 | 0.60 |
| 23 | 6.2266 | 20.90 | 20.50 | 20.40 | 0.40 | 0.50 |
| 24 | 6.5703 | 21.90 | 21.50 | 21.40 | 0.40 | 0.50 |
| 25 | 6.9141 | 23.10 | 22.80 | 22.70 | 0.30 | 0.40 |
| 26 | 7.1602 | 23.90 | 23.70 | 23.60 | 0.20 | 0.30 |
| 27 | 7.4063 | 25.05 | 24.85 | 24.80 | 0.20 | 0.25 |

Figure 5: Shaping gain of 1D-NUC and 2D-NUC for AWGN Channel

Observation 3 :- Initial investigation using 5G-NR PDSCH BICM chain for a target BLER of 1% without retransmission using MCS table 2 under AWGN channel shows shaping gains of up to 0.7dB for 1D-NUC and up to 0.8dB for 2D-NUC.

### Fading channel

Performance evaluations under the TDL-A fading channel are conducted to assess the shaping gain of both 1D-NUC and 2D-NUC schemes. These simulations utilize the NR PUSCH chain and employ MCS Table 2 as specified in 3GPP TS 38.214. The link-level simulation parameters used for fading channel evaluations are provided in Table 4. For simulations with retransmission, HARQ configurations detailed in Table 5 are applied. BLER is used as the performance metric for simulations without retransmissions, whereas throughput percentage (w.r.t maximum achievable throughput) is employed as the performance metric for simulations when HARQ is enabled. NUC values and bit labeling optimized for the AWGN channel and target SNR are used, without incorporating any channel-specific or fading-specific optimizations

Table 4: LLS Simulation Parameters for fading channel

| Parameter | Value |
| --- | --- |
| Link | Uplink using 5G-NR PUSCH chain |
| Fading Channel | TDL-A Delay Spread = 30nS, Doppler = 10Hz |
| Bandwidth | 20MHz (51 PRB) |
| SCS | 30KHz |
| Time Domain Allocation | 14 Symbols |
| DMRS Configuration | DMRS Configuration Type:1 DMRS Additional position: pos1 Number of CDM Groups without Data: 2 DMRS Duration: Single symbol |
| LDPC Algorithm | Layered NMSA, Number of Iteration = 10 |
| No. of Transport Blocks simulated | 5000 |
| Channel Estimation | Perfect |
| HARQ | Disabled (single Transmission using RV 0) |
| Antenna Configuration | 1Tx, 2Rx |
| De-mapper | ML De-mapper |
| Target BLER | 1% |

Table 5: HARQ Configuration

| No. of HARQ Process | 1 |
| --- | --- |
| Redundancy Version sequence | [0, 2, 3 ,1] |
| Maximum no. of HARQ transmission | 4 |

#### Fading channel performance without retransmission.

Shaping gain obtained from LLS without retransmission are summarized in Table 6 and Figure 6. Simulation configuration details are provided in section 2.4.2.

Table 6: Shaping gain of 1D-NUC and 2D-NUC for TDL-A Channel @ 1% BLER

| MCS Index | Spectral efficiency | Required SNR (dB) @ 1% BLER |  |  | Shaping Gain(dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UC | 1D-NUC | 2D-NUC | 1D-NUC | 2D-NUC |
| 5 | 1.4766 | 11.20 | 11.10 | 11.10 | 0.10 | 0.10 |
| 6 | 1.6953 | 12.30 | 12.30 | 12.20 | 0.00 | 0.10 |
| 7 | 1.9141 | 13.20 | 13.20 | 13.20 | 0.00 | 0.00 |
| 8 | 2.1602 | 14.30 | 14.30 | 14.20 | 0.00 | 0.10 |
| 9 | 2.4063 | 15.30 | 15.30 | 15.30 | 0.00 | 0.00 |
| 10 | 2.5703 | 16.20 | 16.20 | 16.20 | 0.00 | 0.00 |
| 11 | 2.7305 | 16.80 | 16.30 | 16.30 | 0.50 | 0.50 |
| 12 | 3.0293 | 17.70 | 17.40 | 17.20 | 0.30 | 0.50 |
| 13 | 3.3223 | 18.90 | 18.60 | 18.50 | 0.30 | 0.40 |
| 14 | 3.6094 | 19.80 | 19.70 | 19.50 | 0.10 | 0.30 |
| 15 | 3.9023 | 20.80 | 20.60 | 20.50 | 0.20 | 0.30 |
| 16 | 4.2129 | 21.80 | 21.60 | 21.40 | 0.20 | 0.40 |
| 17 | 4.5234 | 22.70 | 22.60 | 22.50 | 0.10 | 0.20 |
| 18 | 4.8164 | 24.70 | 24.60 | 24.50 | 0.10 | 0.20 |
| 19 | 5.1152 | 25.90 | 25.80 | 25.80 | 0.10 | 0.10 |
| 20 | 5.3320 | 25.80 | 25.20 | 25.20 | 0.60 | 0.60 |
| 21 | 5.5547 | 26.60 | 26.10 | 26.00 | 0.50 | 0.60 |
| 22 | 5.8906 | 27.30 | 26.90 | 26.80 | 0.40 | 0.50 |
| 23 | 6.2266 | 29.60 | 29.30 | 29.20 | 0.30 | 0.40 |
| 24 | 6.5703 | 30.80 | 30.50 | 30.40 | 0.30 | 0.40 |

Note: MCS 25,26 & 27 are not simulated as the required SNR for 1% BLER target is >31dB.

Figure 6: Shaping gain of 1D-NUC and 2D-NUC for TDL-A Channel without Retransmission

Observation 4 :- Initial investigation using 5G-NR PUSCH chain for a target BLER of 1% using MCS table 2 under TDL-A channel without retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 2D-NUC.

#### Fading channel performance with retransmission.

Link level simulations for TDL-A channel are performed to assess the gain from NUC under channel fading. Simulation configuration used are described in section 2.4.2. These simulations use throughput percentage as the performance metric, as it is more appropriate than BLER when retransmissions are enabled. Performance gain can be quantified in two distinct ways. One approach is to measure the SNR margin required to achieve 70% throughput for non-uniform constellation (NUC) compared to uniform constellation (UC). Alternatively, the gain can be expressed as the percentage throughput improvement of NUC over UC at a fixed target SNR. In this case, the target SNR is defined as the SNR at which the UC scheme achieves 70% throughput.

The shaping gains (in terms of SNR margin) observed in the simulations are summarized in Table 7 and illustrated in Figure 7. The corresponding throughput gains are presented in Table 8 and Figure 8.

Table 7: Shaping gain of 1D-NUC and 2D-NUC for TDL-A Channel @ 70% Throughput

| MCS Index | Spectral efficiency | Required SNR (dB) @ 70% Throughput |  |  | Shaping Gain dB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UC | 1D-NUC | 2D-NUC | 1D-NUC | 2D-NUC |
| 5 | 1.4766 | 5.20 | 5.20 | 5.20 | 0.00 | 0.00 |
| 6 | 1.6953 | 6.10 | 6.20 | 6.20 | -0.10 | -0.10 |
| 7 | 1.9141 | 6.90 | 7.00 | 7.00 | -0.10 | -0.10 |
| 8 | 2.1602 | 7.80 | 7.90 | 7.80 | -0.10 | 0.00 |
| 9 | 2.4063 | 8.50 | 8.60 | 8.50 | -0.10 | 0.00 |
| 10 | 2.5703 | 9.30 | 9.40 | 9.30 | -0.10 | 0.00 |
| 11 | 2.7305 | 10.20 | 9.90 | 9.80 | 0.30 | 0.40 |
| 12 | 3.0293 | 11.10 | 10.80 | 10.70 | 0.30 | 0.40 |
| 13 | 3.3223 | 12.20 | 12.00 | 11.80 | 0.20 | 0.40 |
| 14 | 3.6094 | 13.00 | 12.80 | 12.60 | 0.20 | 0.40 |
| 15 | 3.9023 | 13.90 | 13.80 | 13.60 | 0.10 | 0.30 |
| 16 | 4.2129 | 14.80 | 14.60 | 14.40 | 0.20 | 0.40 |
| 17 | 4.5234 | 15.60 | 15.50 | 15.40 | 0.10 | 0.20 |
| 18 | 4.8164 | 17.30 | 17.20 | 17.10 | 0.10 | 0.20 |
| 19 | 5.1152 | 18.30 | 18.10 | 18.00 | 0.20 | 0.30 |
| 20 | 5.3320 | 18.80 | 18.30 | 18.30 | 0.50 | 0.50 |
| 21 | 5.5547 | 19.60 | 19.00 | 18.90 | 0.60 | 0.70 |
| 22 | 5.8906 | 20.30 | 19.80 | 19.70 | 0.50 | 0.60 |
| 23 | 6.2266 | 21.50 | 21.20 | 21.10 | 0.30 | 0.40 |
| 24 | 6.5703 | 22.40 | 22.10 | 22.00 | 0.30 | 0.40 |
| 25 | 6.9141 | 23.90 | 23.70 | 23.60 | 0.20 | 0.30 |
| 26 | 7.1602 | 24.50 | 24.20 | 24.20 | 0.30 | 0.30 |
| 27 | 7.4063 | 25.80 | 25.60 | 25.60 | 0.20 | 0.20 |

Figure 7:  Shaping gain of 1D-NUC and 2D-NUC for TDL-A Channel @ 70% Throughput

Table 8 : Throughput Gain (%) of 1D-NUC and 2D-NUC for TDL-A channel with retransmission

| MCS Index | Spectral efficiency | Required SNR (dB) @ 70% Throughput (TP) for UC | Throughput (%) |  | Throughput Gain (%) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1D-NUC | 2D-NUC | 1D-NUC | 2D-NUC |
| 5 | 1.4766 | 5.20 | 71.45 | 70.00 | 1.49 | -0.57 |
| 6 | 1.6953 | 6.10 | 70.00 | 69.95 | -1.55 | -1.62 |
| 7 | 1.9141 | 6.90 | 69.20 | 69.95 | -1.70 | -0.64 |
| 8 | 2.1602 | 7.80 | 69.75 | 71.40 | -1.97 | 0.35 |
| 9 | 2.4063 | 8.50 | 69.50 | 70.95 | -2.39 | -0.35 |
| 10 | 2.5703 | 9.30 | 69.90 | 70.30 | -1.62 | -1.06 |
| 11 | 2.7305 | 10.20 | 74.55 | 74.85 | 5.97 | 6.40 |
| 12 | 3.0293 | 11.10 | 73.95 | 75.00 | 5.57 | 7.07 |
| 13 | 3.3223 | 12.20 | 73.80 | 75.15 | 4.38 | 6.29 |
| 14 | 3.6094 | 13.00 | 72.85 | 75.00 | 3.70 | 6.76 |
| 15 | 3.9023 | 13.90 | 72.40 | 74.70 | 3.13 | 6.41 |
| 16 | 4.2129 | 14.80 | 72.85 | 75.10 | 2.53 | 5.70 |
| 17 | 4.5234 | 15.60 | 72.30 | 73.10 | 2.84 | 3.98 |
| 18 | 4.8164 | 17.30 | 72.25 | 73.05 | 2.26 | 3.40 |
| 19 | 5.1152 | 18.30 | 72.05 | 72.95 | 1.55 | 2.82 |
| 20 | 5.3320 | 18.80 | 77.35 | 77.65 | 9.56 | 9.99 |
| 21 | 5.5547 | 19.60 | 76.65 | 77.05 | 8.42 | 8.98 |
| 22 | 5.8906 | 20.30 | 75.75 | 76.15 | 6.54 | 7.10 |
| 23 | 6.2266 | 21.50 | 74.00 | 75.55 | 5.41 | 7.62 |
| 24 | 6.5703 | 22.40 | 74.10 | 75.45 | 4.00 | 5.89 |
| 25 | 6.9141 | 23.90 | 72.95 | 73.75 | 4.14 | 5.28 |
| 26 | 7.1602 | 24.50 | 73.20 | 73.90 | 3.46 | 4.45 |
| 27 | 7.4063 | 25.80 | 71.75 | 72.30 | 2.21 | 2.99 |

Figure 8: Throughput Gain (%) of 1D-NUC and 2D-NUC for TDL-A channel with retransmission

Observation 5 :- Initial investigation using 5G-NR PUSCH chain for a 70% throughput using MCS table 2 under TDL-A channel with retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 0.7dB for 2D-NUC.

Observation 6 :- Initial study using 5G-NR PUSCH chain using MCS table 2 under TDL-A channel with retransmission shows up to 10% throughput gain for 1D-NUC and 2D-NUC.

Observation 7 :- NUC Performance evaluation indicates reduction in throughput (up to 2.39%) for MCS index corresponding to 16QAM compared to UC (under TDL-A channel using 5G-NR PUSCH chain using MCS table 2).

Observation 8 :- NUC optimized for AWGN channel and a target SNR also provides throughput gain under fading channel (TDL-A).

Observation 9 :- Higher-order modulation schemes achieve greater shaping gain under both AWGN and fading (TDL-A) channel.

Proposal 1 :- RAN1 to Study the benefits and applicability of Non-Uniform Constellation (1D-NUC and 2D-NUC) for 6G Radio Air Interface.

# Conclusions

This document discussed non-uniform constellation design, de-mapping complexity and the expected shaping gain under AWGN and fading channel. The proposals and observations based on the above discussion are summarized as follows,

Observation 1 :- The de-mapping complexity of 1D-NUC is comparable to that of a uniform constellation, as both require a similar number of operations for LLR computation.

Observation 2 :- The de-mapping complexity of 2D-NUC is significantly higher than that of 1D-NUC due to the need for joint processing of in-phase and quadrature components.

Observation 3 :- Initial investigation using 5G-NR PDSCH BICM chain for a target BLER of 1% without retransmission using MCS table 2 under AWGN channel shows shaping gains of up to 0.7dB for 1D-NUC and up to 0.8dB for 2D-NUC.

Observation 4 :- Initial investigation using 5G-NR PUSCH chain for a target BLER of 1% using MCS table 2 under TDL-A channel without retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 2D-NUC.

Observation 5 :- Initial investigation using 5G-NR PUSCH chain for a 70% throughput using MCS table 2 under TDL-A channel with retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 0.7dB for 2D-NUC.

Observation 6 :- Initial study using 5G-NR PUSCH chain using MCS table 2 under TDL-A channel with retransmission shows up to 10% throughput gain for 1D-NUC and 2D-NUC.

Observation 7 :- NUC Performance evaluation indicates reduction in throughput (up to 2.39%) for MCS index corresponding to 16QAM compared to UC (under TDL-A channel using 5G-NR PUSCH chain using MCS table 2).

Observation 8 :- NUC optimized for AWGN channel and a target SNR also provides throughput gain under fading channel (TDL-A).

Observation 9 :- Higher-order modulation schemes achieve greater shaping gain under both AWGN and fading (TDL-A) channel.

Proposal 1 :- RAN1 to Study the benefits and applicability of Non-Uniform Constellation (1D-NUC and 2D-NUC) for 6G Radio Air Interface.

# References

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June. 2025.

- N. S. Loghin et al., “Non-uniform constellations for ATSC 3.0,” IEEE Trans. Broadcast., vol. 62, no. 1, pp. 197–203, Mar. 2016.

- J. Barrueco et al., "Constellation Design for Bit-Interleaved Coded Modulation (BICM) Systems in Advanced Broadcast Standards," in IEEE Transactions on Broadcasting, vol. 63, no. 4, pp. 603-614, Dec. 2017.

- R1-2508746 NUC Constellation Values and Bit label.xls