3GPP TSG RAN WG1 Meeting #123	R1-2508760

Dallas, United States ,November  17 – 21, 2025

Agenda Item:	10.6.1

Source:	Tejas Networks

Title:	Views on GNSS Resilient NR-NTN Operation

Document for:	Discussion and Decision

# Introduction

In RAN WG1 meeting #122_bis  the following agreements were made with respect to the GNSS resilient NR-NTN Operation

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration) - 2 Te  Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

    - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Other values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least:

- Impact on signalling overhead.

Observation:

For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis:

- Solution 1: To improve robustness:

  - Solutions based on modified random access procedures, using existing PRACH formats:

    - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

    - Solution 1B: New PRACH restricted sets.

    - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

    - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command)

    - Solution 1E: Selection of a set of existing PRACH root

    - Solution 1F: Using TA margin and RO masking.

    - Other solutions are not precluded

- Solution 2: Solutions based on reducing time-frequency uncertainty

  - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

  - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Solution 2C: Solutions based on broadcasting DL timestamp(s)

  - Solution 2D: UE side time/frequency pre-compensation based on reference location.

  - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

  - Other solutions are not precluded

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded

In this contribution we present our views and analysis on the following aspects on GNSS Resilient NR-NTN Operation

- Evaluation of Differential Delay and Differential Doppler

- Analytical Characterization  of  Existing PRACH formats with estimated differential delay and doppler

- Proposed Dual Format PRACH details – Transmission , Configuration and detector aspects

- Timing Advance

- Signalling overhead for Connected Mode Aspects

# Discussion

## Evaluation of Differential Delay and Differential Doppler

As agreed in RAN1#122_bis, assumptions for evaluating differential delay and differential Doppler were defined for Scenario 1 and Scenario 2. The differential delay is calculated as the difference between the UE-to-satellite distance at the nearest point and the farthest point within the uncertainty area. For uplink performance evaluation, twice the differential one-way delay should be considered. Similarly, the differential one-way Doppler/frequency offset is determined as the difference between the largest and smallest UE-to-satellite Doppler/frequency offsets within the uncertainty area. For uplink performance evaluation, a scaled one-way differential Doppler/frequency offset should be applied to estimate the total frequency offset at the gNB receiver.

Assuming the oscillator frequency offsets of the UE and BS are f₁ ppm and f₂ ppm respectively, and the Doppler-induced frequency offset due to relative motion is f₃ ppm, the UE aligns its local oscillator to the received downlink (DL) signal. The uplink (UL) transmission frequency is then shifted by the duplex spacing. According to 3GPP TS 38.101-1, the UE's transmitted carrier frequency must be accurate within ±0.1 ppm. As a result, the frequency offset observed at the BS receiver is approximately (2·f₃ + 0.1) ppm, which is roughly twice the residual Doppler shift

Proposal 1 : Aligned to the frequency of DL signal, the UL frequency misalignment at the BS side is double the residual Doppler shift. The scaling factor for one-way differential Doppler /frequency offset  for UL performance evaluation is 2 .

### Calculation Methodology for differential delay and doppler

For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered. Geometry model and equations could be considered. For the variables in the equations, refer to Figure below.

For non-nadir beam

dfar=RE+hue2sin2α+hs-hue2+2RE+huehs-hue-RE+huesinα

φ1=φ-2RcRE

φ=sin-1dfarRE+hscosα

dnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1

β=π2-(α+φ)

γ=cos-1RE+hsdnearsinφ1

β1=π2-(α+φ1)

fdfar=vssinβ+vuecosαcfc

fdnear=vssinβ1+vuecosγcfc

∆t=dfar-dnearc

∆fd=fdfar-fdnear

For nadir beam

dnear=hs-hue

φ=RcRE

dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ

γ=cos-1RE+hsdfarsinφ

β=π2-(α+φ)

fdfar=vssinβ+vuecosαcfc

∆t=dfar-dnearc

∆fd=2fdfar

Figure 1:Figure for calculation of Differential delay and doppler

## Estimation of Differential delay and doppler – Scenario 1

To estimate differential delay and Doppler, a large number of evaluation cases are defined and there is a need to assess various elevation angles. These characteristics are determined by the elevation angle at the nearest point from the satellite and elevation angle at the farthest point from the satellite for delay and Doppler estimation as per agreement in the introduction section. Detailed evaluation tables are provided in the excel sheet attached with the TDoc. A summary of the case counts is shown below for two configurations: Option 1 uses fixed beam diameters across all elevation angles, while Option 2 employs elongated beams with decreasing elevation angles.

S band -fixed diameters Frequency Band; Satellite Orbit; Beam Diameter (fixed); Max Differential one-way Delay(ms); Max Residual one-way Frequency Offset(khz). S band (2 GHz); LEO 600; 50 (Set1); 0.143; 4.2. 4.22. 90 (Set 2); 0.26; 7.54. 7.58. LEO 1200; 90(Set 1); 0.257; 3.78. 3.79. 190(Set 2); 0.535; 7.95. 7.96. GEO; 250(Set 1); 0.81; 0.000257. 0.0010. 450(Set 2); 1.5; 0.000462. 0.00185. Observation 1: The maximum one-way differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.26 ms,0.535 ms and 1.5 ms respectively for Set 2 scenarios in S band and the maximum one-way residual frequency offset are 7.58 khz , 7.96 khz and 0.00185 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively. Ka band -fixed diameters Frequency Band; Satellite Orbit; Beam Diameter(fixed); UE Speed (kmph); Max Differential one-way Delay(ms); Max Residual Frequency Offset(khz). Ka band (30 Ghz); LEO 600km; 20(Set 1); 120; 0.0575; 25.34. 1500; 26.74. LEO 1200km; 40(Set 1); 120; 0.115; 25.23. 1500; 27.13. GEO; 110(Set 1); 120; 0.357; 0.00678. 1500; 0.848.

Observation 2: The maximum one-way differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.0575 ms,0.115 ms and 0.357 ms respectively for Set 2 scenarios in Ka band and the maximum one-way residual frequency offset are 26.74 khz , 27.13 khz and 0.848 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively. S band – Beam elongates with reducing elevation angle In the previous section we considered fixed beam diameters for all elevation angles. The beam diameters which are defined for nadir beams is assumed for the farther beams too. Practically the beam diameters elongate as the elevation angle decreases .The differential delay and residual doppler depends on the assumed beam diameters for different elevation angles hence we need to look at the elongation of beam diameters to get the worst-case scenario . The elevation angle for cell edge UEs in the farther beams where the beam centre angles are given is adopted from the NB-IoT NTN TR 36.763 .The differential delay for Scenario 1 with beam diameters elongating with reducing beam diameters for S band is given below Orbit; Beam Diameter at Nadir; Elevation Angle at Beam Centre; Elevation Angle at Cell Edge; Maximum Differential one-way Distance(km); Maximum Differential one-way Delay(ms). GSO; 250 km (Set 1); 90; 88.67; 1.45; 0.0048. 12.5; 15; 529; 1.76. 450 km (Set 2); 90; 88; 3.29; 0.0109. 12.5; 18; 1039; 3.46. LEO 600 km; 50 km (Set 1); 90; 87.38; 0.574; 0.00198. 30; 33.02; 158.4; 0.52. 90 km (Set 2); 90; 85.35; 1.81; 0.00605. 30; 23.8; 353.3; 1.18. LEO 1200 km; 90 km (Set 1); 90; 87.44; 1.01; 0.0036. 30; 26.27; 298.75; 0.995. 190 km (Set 2); 90; 84.61; 4.48; 0.0145. 30; 22.16; 684; 2.24. In the case of elongated beam diameters , the maximum residual doppler for each scenario and evaluation case will be same as fixed beam diameter case as the maximum residual doppler is defined by the nadir beam and the beam diameter for Nadir beams is same for fixed and elongated beam diameter case

Observation 3: The maximum one-way  differential delay for LEO 600km,LEO 1200km and GEO scenarios are 1.18 ms,2.24 ms and 3.46 ms respectively for Set 2 scenarios in S band considering the beam size increases with reducing elevation angle

### Estimation of Differential Delay and Doppler – Scenario 2

As per the agreements in RAN1#122, Scenario 2 addresses the condition where the User Equipment (UE) relies on previously acquired GNSS location information that has not been updated for a time duration T, resulting in degraded GNSS accuracy as shown in the figure. In this scenario, the UE uses the outdated GNSS data—valid as of T time units ago—for computing the UE-specific Timing Advance (TA) and for Doppler pre-compensation.

Figure 2:GNSS Unavailability scenario 2

We evaluate the differential doppler and delay within the uncertainty area X km for different satellite scenarios and the values are provided in the excel sheet attached with this document for the uncertainty area radius of 1, 5, 10 and 25 km for different LEO scenarios. The maximum differential delay values for minimum elevation angles(30 and 12.5 for LEO and GEO Orbits respectively) are summarized in the following table

Maximum one-way differential delay for minimum elevation angle in microseconds

| Band | Orbit | Max one way differential delay(us) for Uncertainty Area Radius X km |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 km | 10 km | 25 km |
| S band | LEO 600 | 5.78 | 57.7 | 143 |
|  | LEO 1200 | 5.77 | 57.6 | 143.57 |
|  | GEO | 6.51 | 65.1 | 163.6 |
| Ka band | LEO 600 | 5.78 | 57.6 | 143.29 |
|  | LEO 1200 | 5.78 | 57.8 | 143.8 |
|  | GEO | 6.52 | 65.2 | 163 |

The maximum one-way differential doppler values for nadir beams are summarized in the following table in khz

| Band | Orbit | Max one way residual doppler(khz) for Uncertainty Area Radius X km |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 km | 10 km | 25 km |
| S band | LEO 600 | 0.169 | 1.69 | 4.22 |
|  | LEO 1200 | 0.085 | 0.855 | 2.11 |
|  | GEO | 0.00008 | 0.0008 | 0.004 |
| Ka band | LEO 600 | 2.67 | 26.7 | 66.8 |
|  | LEO 1200 | 1.34 | 13.43 | 33.58 |
|  | GEO | 0.001 | 0.0103 | 0.257 |

Considering the LEO 600km case , the 25 km uncertainty area is equal to the beam diameter of Set 1 satellite configuration . In this case ,which is the worst-case scenario, the differential delay and residual doppler observed is equivalent to that of scenario 1. For other cases, LEO 1200 km and GEO, the same can be observed when the GNSS uncertainty area equals the beam diameter.

Observation 4 : In the worst-case scenario, the maximum differential delay and differential doppler observed for farther beams in Scenario 2 is equivalent to that in Scenario 1, provided the GNSS displacement error reaches the beam diameter.

Proposal 2 : The worst-case differential delay and residual Doppler across all elevation angles and beam configurations in Scenario 2 are equivalent to those in Scenario 1, assuming the maximum GNSS displacement error spans the beam diameter. Consequently, any mitigation or compensation strategy developed for Scenario 1 is directly applicable to Scenario 2 as well.

## PRACH Analysis for GNSS unavailable NTN Scenarios

### PRACH Capability to support delay and doppler

PRACH long formats are designed for large cell deployments and high coverage scenarios in Frequency Range 1 (FR1) , i.e., below 6 GHz. These formats use a sequence length of 839 and subcarrier spacings of 1.25 kHz or 5 kHz . PRACH short formats are designed for rapid initial access and synchronization, especially in small to medium cell deployments. These formats are based on a short sequence length of 139 subcarriers and support various subcarrier spacings depending on the frequency range. The summary of maximum supported two-way differential delay and supported doppler  both long and short formats can be found in the following table. The detailed table for the same calculation can be found in Appendix A section A.2.The analytical calculation follows the agreements in the RAN1#122_bis as mentioned in section 1.

Note : We consider GP lengths for formats where the GP lengths are non zero as there can be consecutive Random Occasions

Long Formats

|  |  |  | Support of Doppler (khz) |  |  |
| --- | --- | --- | --- | --- | --- |
| Format | SCS | Limiting differential two-way delays (us) | Unrestricted | ResTypeA | ResTypeB |
| 0 | 1.25 | 96.88 | Upto 625 | Upto 1.75 | Upto 3.125 |
| 1 | 1.25 | 684.38 | Upto 625 | Upto 1.75 | Upto 3.125 |
| 2 | 1.25 | 152.6 | Upto 625 | Upto 1.75 | Upto 3.125 |
| 3 | 5 | 96.88 | Upto 2.5 | Upto 7.5 | Upto 12.5 |

Short Formats

| Format | SCS(khz) | Limiting differential two way delays (us) | Max Supported Doppler(khz) |
| --- | --- | --- | --- |
| A1 | 15/30/60/120 | 9.36/4.68/2.34/1.17 | SCS/2 |
| A2 | 15/30/60/120 | 18.72/9.36/4.68/2.34 |  |
| A3 | 15/30/60/120 | 28.08/14.04/7.02/3.51 |  |
| B1 | 15/30/60/120 | 2.34/1.17/0.585/0.286 |  |
| B2 | 15/30/60/120 | 7.02/3.51/1.76/0.88 |  |
| B3 | 15/30/60/120 | 11.7/5.85/2.93/1.47 |  |
| B4 | 15/30/60/120 | 25.54/12.76/6.38/3.19 |  |
| C0 | 15/30/60/120 | 35.62/17.81/8.91/4.46 |  |
| C2 | 15/30/60/120 | 66.56/33.28/16.64/8.32 |  |

Observation 5 : The maximum supported two-way differential delay for PRACH is 684.38 us for long format 1 but only with a maximum doppler support of 3.125 khz with restricted type B. For long formats, maximum supported doppler is 12.5 khz with restricted type B for long format 3.

Observation 6 : For short formats , the doppler support is typically half the SCS and limiting differential two-way delay is upto 66.56 us (format C2,15 khz SCS)

### PRACH Analytical characterization with Delay Doppler values

Knowing the PRACH capabilities for supported delay and doppler along with the differential delay and doppler values for different evaluation cases. We can analytically characterize the PRACH performance for each evaluation cases. Summary of the analysis can be found in the given table. In the summary for each evaluation case and UE speeds we have identified the maximum possible  two-way differential delay (farther beams ) and  maximum frequency offset ( two times residual doppler at nadir beams) .The scaling factor for the calculation of residual doppler offset is 2 . Pre-compensation of delay and doppler within the uncertainty area is not considered and the differential delay and doppler values are based on the agreements made in RAN1#122_bis

S band Fixed Diameters

| Band | Satellite Orbit | Beam Diameter (fixed) | UE Speed (kmph) | Max Differential Delay(ms) (RTD) | Max Residual Frequency Offset(khz) (Two-way) | PRACH formats Supported (Delay) | PRACH formats Supported (Doppler) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S band (2 GHz) | LEO 600 | 50 km (Set 1) | 3 | 0.286 | 16.80 | Only Long format1 | Short formats for very large elevation angle |
|  |  |  | 120 |  | 16.88 |  |  |
|  |  | 90 km (Set 2) | 3 | 0.52 | 15.08 | Only Long format1 | Short formats for very large elevation angle |
|  |  |  | 120 |  | 15.16 |  |  |
|  | LEO 1200 | 90 km (Set 1) | 3 | 0.514 | 7.56 | Only Long format1 | Long format 3 with restricted sets Short formats for very large elevation angle |
|  |  |  | 120 |  | 7.58 |  |  |
|  |  | 190 km (Set 2) | 3 | 1.07 | 15.9 | No format | Short formats for very large elevation angle |
|  |  |  | 120 |  | 15.92 |  |  |
|  | GEO | 250 km (Set 1) | 3 | 1.62 | 0.002 | No format | All formats All formats |
|  |  |  | 120 |  | 0.004 |  |  |
|  |  | 450 km (Set 2) | 3 | 3 | 0.002 | No format | All formats All formats |
|  |  |  | 120 |  | 0.004 |  |  |

For S-band GEO scenarios, both Set 1 and Set 2 satellite configurations exhibit maximum round-trip differential delays of 1.62 ms and 3 ms respectively. These values exceed the maximum differential two-way delay supported by PRACH Format 1, which is limited to 0.684 ms—the highest among all formats. For S-band LEO scenarios at 600 km altitude, the maximum differential delay for both Set 1 and Set 2 configurations is within the supportable range of PRACH Long Format 1. However, this is valid only for residual Doppler values up to 3.125 kHz.The required doppler support for LEO 600km nadir scenarios is more than 16 khz which cannot be supported by long format 1.For higher Doppler conditions, Long Format 3 (with restricted preamble sets) and all short formats offer sufficient Doppler tolerance. For S-band LEO scenarios at 1200 km altitude, the maximum differential delay for Set 1 configuration is within the supportable range of PRACH Long Format 1. The required doppler support for LEO 600km nadir scenarios is more than 16 khz which cannot be supported by long format 1. However, for Set 2 configuration, the maximum round-trip differential delay exceeds the delay support capabilities of all PRACH formats, including both long and short formats. While the maximum residual Doppler can be accommodated by Long Format 3 (with restricted preamble sets) and all short formats, the limited differential delay tolerance of short formats makes them unsuitable for beams other than nadir beams . Across all evaluated scenarios, no existing PRACH format can simultaneously meet the maximum differential delay and maximum residual Doppler requirements.

Observation 7: For S band GEO scenarios and LEO 1200km Set 2 scenario , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats.

Observation 8: For S band LEO 600km(Set 1 and Set 2 ) and LEO 1200km (Set 1) scenarios , the maximum differential delay scenarios can only be handled by the PRACH long format 1 but only with a doppler support of maximum 3.125 khz which is insufficient for nadir scenarios.

Observation 9: For S band LEO scenarios , the maximum residual doppler for cases can be supported using short formats(suitable SCS) and Long Format 3 with restricted Sets . But the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams.

Ka band Fixed diameters

| Frequency Band | Satellite Orbit | Beam Diameter (fixed) | UE Speed (kmph) | Max Differential Delay(ms) (RTD) | Max Residual Frequency Offset (khz) | PRACH formats Supported (Delay) | PRACH formats Supported (Doppler) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ka band (30 Ghz) | LEO 600(km) | 20 km (Set 1) | 120 | 0.115 | 50.68 | Only Long format1 | Short format C0/C2 with SCS 120khz or higher |
|  |  |  | 1500 |  | 53.48 |  | Short format C0/C2 with SCS 120khz |
|  | LEO 1200(km) | 40 km (Set 1) | 120 | 0.230 | 50.46 | Only Long format1 | Short format C0/C2 with SCS 120khz or higher |
|  |  |  | 1500 |  | 54.26 |  | Short format C0/C2 with SCS 120khz |
|  | GEO | 110 km (Set 1) | 120 | 0.714 | 0.0136 | No format | All formats |
|  |  |  | 1500 |  | 1.696 |  | Long format 3 , all short |

Observation 10: For Ka band LEO 600 km, LEO 1200 scenarios , the maximum differential delay scenarios can be handled by the PRACH long format 1 for Set 1 satellite configuration. The maximum residual doppler is supported by  short formats C0/C2 with SCS 60khz or higher  but the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams.

Considerations on scenario 2

In section 2.1.3, the differential delay and residual doppler values were captured for uncertainty area radius of  1km , 5km , 10km and 25 km. The short formats support a maximum two-way differential delay of 66.6 microseconds(C2) , which does not support the scenario uncertainty area beyond approximately   6 km  based on the differential delay numbers. For long formats the differential delay values till 684 micro seconds can be supported by Format 1 which corresponds to approximately but supports only upto 0.625 khz without restricted sets and upto 3.125 with restricted type B. From the tables in section 2.1.3 ,  for LEO 600km scenario, the differential round trip doppler beyond 10km is not supported by long format 1.

Observation 11 : The maximum two-way differential delay values supported by short formats is 66.6 microseconds (C2) which can support only a GNSS uncertainty area upto 5-6 km approximately even with good resilience to doppler shifts

Considerations on Elongated beam diameters with decreasing elevation angle

Referring to table for elongated beam diameters in section 2.2.2 it is clear that for S band , for Set 1 satellite configurations the maximum differential delay for all satellite orbits exceeds the two-way limiting differential delay for all PRACH formats.

Observation 12:  For S band scenarios with elongating beam diameters , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats.

The existing PRACH formats are inadequate to support the maximum differential delay and dopplers for GNSS unavailable scenarios for many evaluation cases. For some cases, even though the doppler can be supported using the short formats, there also exist some differential delay which the short formats cannot support due to the very low limiting two-way distances.

Observation 13: Although several PRACH formats can support delay and doppler separately for some of the cases but they do not support large values of delay and doppler simultaneously which is the case for many NTN scenarios. Consequently, the detection capabilities of the existing PRACH formats are inadequate for NTN scenarios that operate without GNSS assistance.

Proposal 3: Enhancements are required  in the existing PRACH and NR Initial Access procedures to support the large residual dopplers and differential delays in various NTN scenarios without GNSS availability.

## Pre-compensation of Common Delay and doppler using a Single Reference Point

The differential delay and doppler values calculated in Section 2 does not specify any reference point for the pre-compensation of differential delay and doppler within the uncertainty area. For scenario 1 differential delay and doppler within the beam can be reduced by a reference point indicated by the network.

To support accurate uplink timing in NR NTN systems, 3GPP TR 38.821[4] specifies that the beam specific reference point used for common Timing Advance (TA) calculation can be defined by the network such that it represents the location within the beam contour closest to the satellite. This ensures that all UE-specific propagation delays relative to the reference point are positive. The rationale behind this requirement is that the PRACH mechanism is unable to detect negative TA values, and the gNB is not capable of signalling negative TA to the UE. The Common TA parameters can be signalled through the SIB-19.

The UE pre-compensates the common doppler with respect to the  reference point  signalled by the network . This compensation is applied relative to the beam centre, where the Doppler shift is reduced to zero. As a result, the residual Doppler experienced by UEs across the beam footprint is minimized(exactly into half for non-nadir beams). The total residual frequency offset is equal to 2 times the differential doppler calculated with respect to the beam centre

The differential delay and residual doppler for scenario 2 are defined by the location uncertainty area depending on the  GNSS outage duration T

Proposal 4 :The beam-specific reference point used for common Timing Advance (TA) calculation in Scenario 1 should be  defined by the network such that it represents the location within the beam contour closest to the satellite

Proposal 5: The UE should pre-compensate the common doppler with respect to the beam centre in scenario 1 which is the reference point signalled by the network to the UE so that the maximum differential residual doppler within the uncertainty area reduces to half the value.

## PRACH Enhancements to support GNSS Resilient Operation

### Dual Format PRACH Transmission with existing Formats

As discussed above, existing PRACH formats are inadequate for ensuring reliable initial access in NTN scenarios where GNSS support is unavailable. This limitation highlights the need for enhancements to both PRACH configurations and NR initial access procedures. The objective of this Study Item (SI) is to assess the impact of GNSS unavailability on initial access performance, while aiming to minimize changes to existing physical layer signalling and procedures. Thus, we define a new dual format based PRACH transmission methods to improve the PRACH capability under large differential delay and residual doppler in NTN scenarios as defined below .

- Dual-Format-PRACH:

Format1: with SCS 1.25/5 kHz, 839 LRA - Long formats  of 5GNR

Format2: with SCS options as 15/30/60kHz, 139 LRA Short formats of 5GNR

The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. The duration of the RO is influenced by several factors, including the satellite orbit type, beam footprint diameter, and the elevation angle at the beam centre. These parameters determine the extent of differential delay across the beam, and the maximum supported values are outlined in the corresponding reference table in section 2.1.2.

In the following illustration, the RACH Occasion (RO) duration is defined as 10 ms, which approximately corresponds to the maximum differential delay within a cell for GEO satellite scenarios. Format 1 of the dual-format configuration refers to the long format F0, utilizing a 1.25 kHz subcarrier spacing (SCS) and a sequence length of 839. Format 2 corresponds to the short format C0, with a 15 kHz SCS and a sequence length of 139. Although the actual signal duration for each PRACH transmission is approximately 1 ms, the overall RO duration is determined by the maximum differential delay expected in the satellite scenario, ensuring reliable detection across the beam footprint. The increased duration of the defined Random Occasion leads to resource overhead due to the large propagation delays associated with the NTN scenarios and its impact needs to be further studied.

Figure 3:Dual Format PRACH separated by Frequency within a RACH Occasion

There can be multiple ways to configure the proposed Dual-Format PRACH  where the two formats are separated by either frequency or time. In the below illustration the two formats are separated in both frequency and time as illustrated below.

Figure 4: Dual Format PRACH separated by both Time & Frequency within a RACH Occasion

Proposal 6: The proposed Dual Format PRACH uses the existing formats as that of the legacy NR PRACH and can be configured such that two formats are separated in either time or frequency with minimal procedural changes to the existing Initial Access setup.

Observation 14: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. The impact of increased resource overhead due to the extended RACH Occasion duration needs to be further studied.

Proposal 7: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. Although the actual signal duration for each PRACH transmission is approximately 1 ms, the overall RO duration is determined by the maximum differential delay.

Multi-Window Hypothesis based PRACH detector

This detection algorithm operates in two main stages using a multi-hypothesis approach, followed by a decision algorithm to determine the final Timing Advance (TA) from multiple hypotheses across both formats. Two types of hypothesis windows are defined based on the Dual-Format input configuration, a large window for detection of the Coarse TA using the Format1 of Dual Format and a short window for finer detection of TA using the Format2 of  Dual Format along with a  decision Algorithm for detection of the final TA.The proposed scheme introduces minimal additional complexity at the UE side, primarily limited to enhancements in the RRC mechanisms to support Dual Format PRACH transmission. However, on the gNB receiver side, the complexity increases due to the broader range of delay variations. This necessitates finer-granularity timing estimation, which is achieved through a multi-window hypothesis detection mechanism, thereby increasing receiver processing requirements and complexity.

Observation 15 : The proposed scheme introduces minimal additional complexity at the UE side but increases the complexity due to the multi-window hypothesis detection mechanism used to support the  broader range of delay variations in NTN scenarios without GNSS availability. The impact of increase in complexity on the Initial access latency should be further studied.

The NTN Cell can contain both GNSS available UEs and GNSS unavailable UEs as GNSS unavailability can be UE specific. The GNSS less UEs should use enhanced PRACH mechanisms to tackle the large differential delays and doppler using PRACH configurational updates and network needs to support both GNSS available and GNSS less UEs in the same beam. Thus separate PRACH formats and separate PRACH occasions for GNSS capable and GNSS less UEs should be configured.     The unavailability of GNSS needs to be detected at the UE or at the network and signalled so that Network and UE is aware of the GNSS resilient operation is active and UE can be configured accordingly.RAN1 should study Network assisted or UE assisted  methods to detect the GNSS unavailability in a UE and  the corresponding signalling aspects

Observation 16 :The NTN Cell can contain both GNSS available UEs and GNSS unavailable UEs and separate PRACH formats and separate PRACH occasions may be configured.

Proposal 8: RAN1 should study Network assisted or UE assisted  methods to detect the GNSS unavailability in a UE and  the corresponding signalling aspects

### Simulation Results for Dual Format PRACH under GNSS resilient NTN Operation

We evaluated the performance of the proposed dual Format PRACH with the simulation parameters tabulated in Appendix A.
For evaluation of large differential delay in case of minimal residual doppler Case 3 was evaluated for GEO scenarios for beam set 2 configurations for S band where the differential delays are up to 3ms.The detection rate Results for Dual Format is based on the detection rates of both the formats. The delay range considered are [0 , Max Differential Delay]. For the simulations, the proposed dual format PRACH transmission , the first sequence is chosen from the 1.25 khz SCS i.e., Format 0 and the second sequence is chosen from the higher SCS , Format C0. The detection rate and timing error CDF graphs are attached.

Figure 5 :Detection Probability of  Dual Format PRACH GEO- Beam Set2

Figure 6:CDF of  Timing Error of  Dual Format PRACH GEO-Beam Set2

For evaluation of minimal differential delay in case of large residual doppler was evaluated for  LEO-600km scenarios for beam set 1 configuration for S band where the residual doppler is up to 4 khz for nadir beams . The detection rate Results for Dual Format is based on the detection rates of both the formats. The detection rate and timing error CDF graphs are attached below.

Figure 7:Detection Probability of  Dual Format PRACH LEO 600km Beam Set1

Figure 8:CDF of  Timing Error of  Dual Format PRACH LEO 600km-Beam Set1

Observation 17: The proposed Dual-Format PRACH demonstrates strong performance with minimal timing errors(<1 us in most cases)  in Set 2 GEO and LEO scenarios within the S-band, especially under GNSS-denied conditions. It effectively handles large differential delays and residual doppler beyond the capabilities of existing PRACH formats

We consider the LEO 600km scenario with the defined maximum  Round Trip differential delay of  1 ms and with a maximum residual doppler of approximately 8.5 khz(set2) for the performance evaluation of a worst-case delay scenario with maximum doppler offset. The delay and doppler are modelled as in the above scenario with same Dual Format Configurations. The detection rate and timing error CDF graphs are attached below.

Figure 9:Detection Probability of Dual Format PRACH - LEO 600km Set 2

Figure 10:CDF of Timing Error of Dual Format PRACH -LEO 600km Set 2

The simulation results for the timing error graph shows that in all of the simulated cases the 99 percentile of the  timing error estimation error is less than 1 us for all the SNR points. The timing error is estimated for the cases where the preamble was successfully detected.

Observation 18 : The proposed Dual-Format PRACH demonstrates strong performance with minimal timing errors(99 percentile< 1 us)  in the defined maximum differential delay case within the S-band, especially under GNSS-denied conditions. It effectively handles large differential delays and large residual offsets which are beyond the capabilities of existing PRACH formats

Observation 19: The required SNR for Dual Format PRACH can be further reduced by  using repetition of preamble formats and corresponding receiver optimizations. Currently format C0 is used as long format which can be changed to format C2 to get better performance.

Proposal 9: Robust performance for preamble detection can be achieved by the proposed Dual Format PRACH in the worst-case scenarios of  differential delay and residual doppler values in NTN Scenarios where GNSS is unavailable.

## Timing and Frequency Adjustments

### Timing Advance and Frequency Adjustment during Initial Access

Timing Advance Command(TAC) in RAR
 	Timing Advance Command (TAC) Initial TAC via Random Access Response (RAR) with 12 bits and Value range: 0–3846.
Using the Initial TAC, the maximum compensable TA is given below

Table 9: Maximum Compensable TA via TAC in  RAR

| SCS | Max Timing Advance(ms) | Max Compensated Distance(km) |
| --- | --- | --- |
| 15 khz | 2 ms | 300 km |
| 30 khz | 1 ms | 150 km |
| 60 khz | 0.5 ms | 75 km |
| 120 khz | 0.25 ms | 37.5 km |
| 240 khz | 0.125 ms | 18.75 km |

The maximum required TA to be signalled for Cell Edge UEs in NTN scenarios is equal to twice the differential delay defined for the Satellite scenario depending upon the defined Beam Footprint. The maximum differential delay and corresponding max TA for Cell Edge UEs. The maximum differential delay within a satellite beam depends upon the elevation angle of beam centre, satellite altitude, defined beam diameter.

The two way differential delay values for GEO scenarios for fixed beam size in S band  exceeds the TAC capability. Also considering elongating beams with reducing elevation angle , the existing TAC capability is not sufficient for most of the satellite orbits for Set 2 satellite configuration in S band

Observation 20 : For GEO scenarios in the S-band with a fixed beam size, the two-way differential delay values exceed the current TAC capability. Additionally, as beam elongation increases with decreasing elevation angles, the existing TAC capability becomes insufficient for most satellite orbits under the Set 2 configuration in the S-band.

There are multiple ways to enhance the TA signalling capability of TAC in RAR as given below

- Increasing the bit width of TAC Command from 12 to 13 using same number of octets in RAR message to double the TA signalling capability.

- Introducing a scaling factor in Timing Advance formula with reduced resolution and increasing the range

- Indication of Negative TAC in RAR

Proposal 10: For GNSS less NTN Scenarios , existing TA signalling capability is insufficient and study enhancements to increase the capability including increasing the bit width of TAC, introducing a scaling factor in TA calculation and indication of negative TAC in RAR

Frequency Adjustments in RAR

The analysis in section 2.1.2 shows that the differential roundtrip Doppler shift in LEO cell can be up to 7.96 khz for LEO 1200 km S band Set 2 scenarios. After the pre compensation of the doppler shift with the centre of the beam as reference point, the residual  doppler shift of a cell edge UE reduces to  3.98 khz .When a preamble transmitted by an NTN UE without GNSS-based pre-compensation is received by gNB, its frequency offset will be much larger than that in legacy NTN. The frequency offset needs to be adjusted before subsequent PUSCH transmissions (e.g., Msg3).

Proposal 11: FFS on  indication of  frequency adjustment commands in Random Access Response

## Timing and frequency adjustments in Connected Mode

In Connected Mode the timing adjustment command TAC via MAC Control Element (MAC-CE) with 6 bits and Value range: 0–63 is used to update the current value to a new value .The maximum timing advance compensated via MAC CE is -16.6 us to 16.6 us for 15 khz SCS

Because of UE position uncertainty when GNSS is unavailable and the high velocity of satellites, the difference between the UE’s estimated Timing Advance (TA) and the actual Round Trip Time (RTT) can drift significantly. If the UE applies TA pre-compensation for Msg1 using its reference location and the initial TA error is corrected by a TA command in Msg2, the TA will be reasonably accurate when the UE enters the RRC_CONNECTED state. However, the TA may still experience considerable drift afterward. According to TR 38.821, Table 7.1-1 [3], the maximum Round Trip Delay variation can reach ±93.0 µs/sec for LEO satellites at 600 km altitude and ±47.6 µs/sec for 1200 km altitude. When GNSS is unavailable, the network must frequently transmit TAC MAC Control Elements (CEs) to the UE. To maintain TA within the RAN4 requirement limits, TA commands need to be sent at least 2–3 times per second.

RAN1 to study enhancements to MAC CE TA signalling capability in GNSS resilient scenarios. To reduce the frequency of TA commands in the RRC_CONNECTED state, it is recommended that RAN1 investigate the inclusion of TA drift-related parameters (e.g., drift rate and drift variation rate) within the TA command.

Observation 21: Due to UE position uncertainty when GNSS is unavailable and the high velocity of LEO satellites, the network must transmit TAC MAC CEs frequently to help the UE maintain synchronization for LEO orbits at 600 km and 1200 km when GNSS is not available

Proposal 12: RAN1 to study enhancements to MAC CE TA signalling capability and investigate the inclusion of TA drift-related parameters (e.g., drift rate and drift variation rate) within the TA command.

For frequency alignment, the UE pre-compensates Doppler based on the beam centre. However, due to the high satellite velocity (approximately 7.8 km/s), there can be a discrepancy between the Doppler shift estimated by the UE and the actual Doppler shift. As specified in TR 38.821, the maximum Doppler rate for LEO satellites is 0.27 ppm/s at 600 km altitude and 0.09 ppm/s at 1200 km altitude. It is recommended that RAN1 study the need for frequency adjustment commands, including the potential inclusion of frequency drift parameters (e.g., drift rate and drift variation rate).

Proposal 13: It is recommended that RAN1 study the need for frequency adjustment commands, including the potential inclusion of frequency drift parameters (e.g., drift rate and drift variation rate).

# Conclusion

In this document, the following observations and proposals are presented:

Proposal 1 : Aligned to the frequency of DL signal, the UL frequency misalignment at the BS side is double the residual Doppler shift. The scaling factor for one-way differential Doppler/frequency offset  for UL performance evaluation is 2 .

Observation 1: The maximum one-way  differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.26 ms,0.535 ms and 1.5 ms respectively for Set 2 scenarios in S band and the maximum one-way residual frequency offset are 7.58 khz , 7.96 khz and 0.00185 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively.

Observation 2: The maximum one-way  differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.0575 ms,0.115 ms and 0.357 ms respectively for Set 2 scenarios in Ka band and the maximum one-way residual frequency offset are 26.74 khz , 27.13 khz and 0.848 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively.

Observation 3: The maximum one-way  differential delay for LEO 600km,LEO 1200km and GEO scenarios are 1.18 ms,2.24 ms and 3.46 ms respectively for Set 2 scenarios in S band considering the beam size increases with reducing elevation angle

Observation 4 : In the worst-case scenario, the maximum differential delay and differential doppler observed for farther beams in Scenario 2 is equivalent to that in Scenario 1, provided the GNSS displacement error reaches the beam diameter.

Proposal 2 : The worst-case differential delay and residual Doppler across all elevation angles and beam configurations in Scenario 2 are equivalent to those in Scenario 1, assuming the maximum GNSS displacement error spans the beam diameter. Consequently, any mitigation or compensation strategy developed for Scenario 1 is directly applicable to Scenario 2 as well.

Observation 5 : The maximum supported two-way differential delay for PRACH is 684.38 us for long format 1 but only with a maximum doppler support of 3.125 khz with restricted type B. For long formats, maximum supported doppler is 12.5 khz with restricted type B for long format 3.

Observation 6 : For short formats , the doppler support is typically half the SCS and limiting differential two-way delay is upto 66.56 us (format C2,15 khz SCS)

Observation 7: For S band GEO scenarios and LEO 1200km Set 2 scenario , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats.

Observation 8: For S band LEO 600km(Set 1 and Set 2 ) and LEO 1200km (Set 1) scenarios , the maximum differential delay scenarios can only be handled by the PRACH long format 1 but only with a doppler support of maximum 3.125 khz which is insufficient for nadir scenarios.

Observation 9: For S band LEO scenarios , the maximum residual doppler for cases can be supported using short formats(suitable SCS) and Long Format 3 with restricted Sets . But the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams.

Observation 10: For Ka band LEO 600 km, LEO 1200 scenarios , the maximum differential delay scenarios can be handled by the PRACH long format 1 for Set 1 satellite configuration. The maximum residual doppler is supported by  short formats C0/C2 with SCS 60khz or higher  but the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams.

Observation 11 : The maximum two-way differential delay values supported by short formats is 66.6 microseconds (C2) which can support only a GNSS uncertainty area upto 5-6 km approximately even with good resilience to doppler shifts

Observation 12:  For S band scenarios with elongating beam diameters , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats.

Observation 13: Although several PRACH formats can support delay and doppler separately for some of the cases but they do not support large values of delay and doppler simultaneously which is the case for many NTN scenarios. Consequently, the detection capabilities of the existing PRACH formats are inadequate for NTN scenarios that operate without GNSS assistance.

Proposal 3: Enhancements are required  in the existing PRACH and NR Initial Access procedures to support the large residual dopplers and differential delays in various NTN scenarios without GNSS availability.

Proposal 4 :The beam-specific reference point used for common Timing Advance (TA) calculation in Scenario 1 should be  defined by the network such that it represents the location within the beam contour closest to the satellite

Proposal 5: The UE should pre-compensate the common doppler with respect to the beam centre in scenario 1 which is the reference point signalled by the network to the UE so that the maximum differential residual doppler within the uncertainty area reduces to half the value.

Proposal 6: The proposed Dual Format PRACH uses the existing formats as that of the legacy NR PRACH and can be configured such that two formats are separated in either time or frequency with minimal procedural changes to the existing Initial Access setup.

Observation 14: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. The impact of increased resource overhead due to the extended RACH Occasion duration needs to be further studied.

Proposal 7: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. Although the actual signal duration for each PRACH transmission is approximately 1 ms, the overall RO duration is determined by the maximum differential delay.

Observation 15 : The proposed scheme introduces minimal additional complexity at the UE side but increases the complexity due to the multi-window hypothesis detection mechanism used to support the  broader range of delay variations in NTN scenarios without GNSS availability. The impact of increase in complexity on the Initial access latency should be further studied.

Observation 16 :The NTN Cell can contain both GNSS available UEs and GNSS unavailable UEs and separate PRACH formats and separate PRACH occasions may be configured.

Proposal 8: RAN1 should study Network assisted or UE assisted  methods to detect the GNSS unavailability in a UE and  the corresponding signalling aspects

Observation 17: The proposed Dual-Format PRACH demonstrates strong performance with minimal timing errors(<1 us in most cases)  in Set 2 GEO and LEO scenarios within the S-band, especially under GNSS-denied conditions. It effectively handles large differential delays and residual doppler beyond the capabilities of existing PRACH formats

Observation 18 : The proposed Dual-Format PRACH demonstrates strong performance with minimal timing errors(99 percentile< 1 us)  in the defined maximum differential delay case within the S-band, especially under GNSS-denied conditions. It effectively handles large differential delays and large residual offsets which are beyond the capabilities of existing PRACH formats

Observation 19: The required SNR for Dual Format PRACH can be further reduced by  using repetition of preamble formats and corresponding receiver optimizations. Currently format C0 is used as long format which can be changed to format C2 to get better performance.

Proposal 9: Robust performance for preamble detection can be achieved by the proposed Dual Format PRACH in the worst-case scenarios of  differential delay and residual doppler values in NTN Scenarios where GNSS is unavailable.

Observation 20 : For GEO scenarios in the S-band with a fixed beam size, the two-way differential delay values exceed the current TAC capability. Additionally, as beam elongation increases with decreasing elevation angles, the existing TAC capability becomes insufficient for most satellite orbits under the Set 2 configuration in the S-band.

Proposal 10: For GNSS less NTN Scenarios , existing TA signalling capability is insufficient and study enhancements to increase the capability including increasing the bit width of TAC, introducing a scaling factor in TA calculation and indication of negative TAC in RAR

Proposal 11 : FFS on  indication of  frequency adjustment commands in Random Access Response

Observation 21: Due to UE position uncertainty when GNSS is unavailable and the high velocity of LEO satellites, the network must transmit TAC MAC CEs frequently to help the UE maintain synchronization for LEO orbits at 600 km and 1200 km when GNSS is not available.

Proposal 12: RAN1 to study enhancements to MAC CE TA signalling capability and investigate the inclusion of TA drift-related parameters (e.g., drift rate and drift variation rate) within the TA command.

Proposal 13: It is recommended that RAN1 study the need for frequency adjustment commands, including the potential inclusion of frequency drift parameters (e.g., drift rate and drift variation rate).

References

- RP-251863 “Study on GNSS resilient NR-NTN operation”, Thales , RAN#108 Prague, Czech Republic, June 9-13th, 2025.

- Chair_notes,RAN1# 122_b , RAN#122_bis Prague,Czech Republic, October 13th-17th ,2025

- R1-2505632 “ Discussion on GNSS resilient NR-NTN operation ”, Tejas Networks , RAN#122_bis Bangalore , India , August 25-29th,2025

- R1-2507202 “ Views on GNSS resilient NR-NTN operation for 5GA”, Tejas Networks , RAN#122_bis, Prague,Czech Republic, October 13th-17th ,2025

- 3GPP TR 38.811 V15.0.0, “Study on New Radio (NR) to support non-terrestrial networks”.

- 3GPP TR 38.821 V16.2.0, “Solutions on NR to support non-terrestrial networks”.

- 3GPP TS 38.133 V16.4.0, “Requirements for the support of Radio Resource Management

- 3GPP TS 38.211  V18.4.0, “NR Physical Channels and Modulation”.

- 3GPP TS 38.212  v18.4.0, “NR Multiplexing and Channel Coding”.

- R1-1909479 “Summary of 7.2.5.3 on UL timing and PRACH for NTN” , ZTE , RAN1 #98 , Prague, CZ, August 26th – 30th, 2019.

- 3GPP TS 38.331  v18.5.0, “NR Radio Resource Control (Protocol Specification)”.

- 3GPP TS 38.331  v18.5.0, “Medium Access Control (Protocol Specification)”.

# Appendix A

LLS parameters for PRACH evaluation

| Configurations | S/L-band | Ka/Ku-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | For Ka: UL 30GHz, for Ku: UL 14 GHz |
| Channel Model | Baseline NTN TDL-C rural model in TR38.811 |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx | 1 Rx |
| Antenna Configuration at the UE | A single omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| PRACH format | To be reported by companies. | To be reported by companies |
| PRACH configuration | To be reported by companies. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing |  |
| Receiver | Companies to report the receiver for PRACH detection. |  |