3GPP TSG RAN WG1 #123			R1- 2508899

Dallas, USA, Nov 17th – Oct 21th, 2025

Agenda Item:	10.3.1

Source: 	LG Electronics

Title: 	Evaluations for Rel-20 Ambient IoT

Document for:	Discussion and decision

- Introduction

The Re1-20 SI on solutions for Ambient IoT was approved in RAN#108 meeting. The latest SI objective relevant to this agenda item is as follows [1].

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This SI is to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT. Note: This SI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc., as well as the features of 6G relevant to IoT. The study shall not aim to replace existing 3GPP LPWA technologies. The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI (FS_Ambient_IoT_Solutions) and WI (Ambient_IoT_Solutions) in RAN WGs are taken into this SI. A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. …

- Discussion

In this contribution, we present our views on Ambient IoT evaluation.

  - Link level simulation

As agreed in RAN1#122bis, we conducted preliminary link-level simulations based on the standardized simulation table for the Rel-20 Ambient IoT study item. The objective was to evaluate the performance of R2D transmission under various channel and system conditions.

In this evaluation, we assumed a data rate of ~8 kbps and reused the R-TAS design in Rel-19 corresponding to the LLS table agreed in RAN1#122bis. Specifically, the PRDCH starting point and the M value, which determines the number of OOK chips per OFDM symbol, were derived based on the Rel-19 R-TAS structure.

The details about evaluation assumptions for the R2D link are shown in Table 1.

Table 1. R2D LLS evaluation assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 900 MHz |
| Information bit length | 20 bits |
| Waveform | OOK-4 with 15 kHz SCS |
| Sampling rate | 3.84 MHz |
| Tx Bandwidth | 1 PRB |
| Line code | Manchester |
| Channel | TDL-C with 300ns delay spread UE velocity: 3 km/h |

As shown in Figure 1, the required SNR to achieve a BLER of 10%, which are the target BLER values agreed in RAN1#122bis, was observed to be 11.5 dB. These results provide a baseline understanding of the link-level performance for R2D transmission in Rel-20, and highlight the impact of M value and outdoor channel configuration on decoding reliability.

Figure 1. Link level simulation result in R2D transmission

Observation 1: Based on the LLS results, the required SNR to achieve BLER targets of 10% was observed to be 11.5 dB

  - Link budget

Based on the required SNR values derived from the BLER targets of 10% and 1% as presented in Clause 2.1, we performed a link budget analysis using the parameters agreed upon in RAN1#122bis. The results of this analysis are summarized in Table 2.

Table 2. R2D link budget calculation

| Target BLER | Total Tx power | Required SNR | Penetration margin | MPL | Distance |
| --- | --- | --- | --- | --- | --- |
| 10% | 43 dBm | 11.5 dB | 20 dB | 121.65 | 616 m |
| 10% | 38 dBm | 11.5 dB | 20 dB | 116.65 | 459 m |

The link budget calculation assumes a standalone deployment scenario and adopts the UMA NLOS path loss model to reflect realistic outdoor conditions. A penetration margin of 20 dB is included to account for signal attenuation in scenarios where the device is located inside a car with metallized windows, which may be relevant for supporting outdoor Ambient IoT devices.

While the exact coverage distances are subject to further simulation and analysis, preliminary results indicate that the effective R2D coverage—after accounting for penetration loss—is expected to be less than 500 meters. This suggests that the current Rel-19 R2D signal and channel design may not fully meet the SID target coverage range of 500 meters in all deployment scenarios, particularly for outdoor or obstructed environments.

To address this gap, R2D coverage enhancement techniques such as repetition or FEC schemes may be necessary to ensure reliable communication within the intended coverage range.

Observation 2: Under the UMA NLOS scenario with penetration loss considered, the estimated R2D coverage may not fulfil the SID target coverage range for some deployment scenarios

Observation 3: R2D coverage enhancement techniques (e.g., repetition, FEC) may be needed to meet the SID target coverage using the current Rel-19 R2D signal/channel design

  - Energy harvesting/storage

Unlike Device 1 in Rel-19, which relies on externally generated signals, Device 2b and Device C introduced in Rel-20 are expected to generate signals internally. This architectural change is likely to result in higher power consumption, making energy efficiency a more critical concern.

Given this increased power demand, relying solely on RF energy harvesting may not be sufficient. Alternative energy sources—such as solar energy—are expected to play a more significant role in enabling sustainable operation for these devices.

As highlighted in previous contributions, there appears to be a lack of consensus among companies regarding the assumptions for energy storage. Some suggest that it is sufficient to store only the energy required for a single channel transmission (e.g., PRDCH/PDRCH), while others mention that the device should store enough energy to support an entire inventory round or random-access procedure.

From our perspective, considering the limited and intermittent nature of energy harvesting—especially in the case of solar-powered devices—it is essential that the energy storage be dimensioned to support at least one full inventory round or random-access procedure. This ensures that the device can maintain availability even when energy harvesting opportunities are constrained, such as during night-time or in shaded environments.

Proposal 1: Energy storage should be considered to support at least one full inventory round or random-access procedure

- Conclusion

In this contribution, we shared our views on Ambient IoT evaluation.

Observation 1: Based on the LLS results, the required SNR to achieve BLER targets of 10% was observed to be 11.5 dB

Observation 2: Under the UMA NLOS scenario with penetration loss considered, the estimated R2D coverage may not fulfil the SID target coverage range for some deployment scenarios

Observation 3: R2D coverage enhancement techniques (e.g., repetition, FEC) may be needed to meet the SID target coverage using the current Rel-19 R2D signal/channel design

Proposal 1: Energy storage should be considered to support at least one full inventory round or random-access procedure

- References

RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices