# 3GPP TSG RAN WG1 #123									 R1-2509044

# Dallas, USA, Nov 17th – 21st, 2025

# Source:	Hanbat National University

# Title:		Discussion on 6GR modulation schemes

# Agenda item:	11.4.2

# Document for:	Discussion/Decision

Introduction

The study item on “Study on 6G Radio” was approved in RAN#108 meeting and it contains several physical layer structure design aspects in its scope [1]. as follows:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. RAN4 can be involved, if necessary, based on the LS from RAN1

In this regard, as a starting point of the 6GR modulation schemes, the following agreements on the evaluation assumptions were made in the 3GPP RAN1#122 meeting [2]:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

In addition, in 3GPP RAN1#122-bis meeting, the following agreements were made [3]:

Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation. Agreement For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR Agreement For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following, - Probabilistic shaping for CP-OFDM and DFT-s-OFDM (Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE) - Geometric shaping for CP-OFDM and DFT-s-OFDM (Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE)

In this contribution, we discuss some considerations for identifying the further details of 6GR modulation schemes.

Discussions

Legacy uniform QAM constellation

From the discussions, most companies agreed to use 5G-NR uniform QAM constellations - QPSK, 16QAM, 64QAM, 256QAM, and 1024QAM as baseline for 6GR downlink (DL) and uplink (UL) channels. Several companies support studying uniform 4096 and 1024 QAM for DL and UL respectively for high SNR scenarios while other companies argue there's no immediate need for higher modulation orders especially in UL due to power limitations.

The highlight of the discussion was on the scope of evaluation for the performance of the modulation schemes. Which includes performance, complexity, EVM requirements, implementation losses, PAPR/MPR and realistic feasibility. Also, for applicable scenarios, participants highlighted that 6G is expected to focus on high SNR use cases like hotspots and Fixed Wireless Access (FWA). Concerning technical challenges, companies like Ericsson suggested that EVM, PAPR, and MPR requirements that affect transmitter efficiency must be addressed.

During the discussion, some companies suggested the adoption of π/2-BPSK DFT-s-OFDM as the baseline uplink modulation scheme (MPR-0 baseline) for 6G NR, making it the default option for coverage-critical transmissions, while QPSK would remain available for capacity-oriented use cases.

Constellation shaping was also discussed in detail and in further discussions, RAN4 was expected to be involved early for RF and other hardware validation.

Proposal 1:  RAN1 to study the use of π/2-BPSK DFT-s-OFDM as a supporting modulation scheme in UL data transmission for extremely low SINR cases as well as initial steps for UL control channel.

Shaped constellation

The discussion on constellation shaping focused on the use of Geometric Shaping (GS) and Probabilistic Shaping (PS) as a method to achieve improved modulation efficiency in 6G New Radio (NR). The core objective of the contributions was to define a common baseline for consistent evaluation of these techniques across all participating companies.

The evaluation framework ensures a fair and standardized comparison of shaped constellations against existing non-shaped NR MCS tables for both CP-OFDM and DFT-s-OFDM waveforms. It defines key performance evaluations focusing on BLER and throughput across AWGN and fading channels, while also assessing feasibility metrics such as complexity, latency, PAPR, EVM, and MPR. The study further emphasizes optimizing MCS tables to fully leverage shaping gains and mandates transparency by requiring clear documentation of shaping algorithms, parameters, rate loss, and switching criteria between geometric and probabilistic shaping. AI/ML-based shaping approaches are allowed, but only the resulting constellation points are to be shared for standardization purposes.

The participants agreed on a rigorous evaluation framework to benchmark geometric and probabilistic shaping (GS/PS) against non-shaped NR MCS using standardized link-level assumptions, including TDL/CDL channels, SU-MIMO (with MU-MIMO for future study), realistic CSI/SRS, LMMSE receivers, and NR-LDPC coding (24 RBs, 30 kHz SCS). They also agreed to evaluate the performance and complexity of high-order uniform modulations such as 4096QAM (DL) and 1024QAM (UL), acknowledging constellation shaping’s potential to enhance spectral efficiency pending proof under realistic conditions.

Proposal 2: RAN1 to study on the detailed assumptions and the proposed improved MCS table to be used as a common baseline for the subsequent system-level evaluation of constellation shaping for 6G NR.

Proposal 3: RAN1 is encouraged to continue studying Geometric Constellation Shaping (GCS) and Probabilistic Constellation Shaping (PCS) as additional constellation techniques to enhance spectral efficiency for 6G New Radio (NR).

Proposal 4: Postpone MU-MIMO shaping while continuing studies on shaping for 1024QAM and 4096QAM

Conclusion

In this contribution, we discussed 6GR modulation schemes. Our proposals are reproduced below:

Proposal 1:  RAN 1 to study the use of π/2-BPSK DFT-s-OFDM as a supporting modulation scheme in UL data transmission for extremely low SINR cases as well as initial steps for UL control channel.

Proposal 2: RAN1 to study on the detailed assumptions and the proposed improved MCS table to be used as a common baseline for the subsequent system-level evaluation of constellation shaping for 6G NR.

Proposal 3: RAN1 is encouraged to continue studying Geometric Constellation Shaping (GCS) and Probabilistic Constellation Shaping (PCS) as additional constellation techniques to enhance spectral efficiency for 6G New Radio (NR).

Proposal 4: Postpone MU-MIMO shaping while continuing studies on shaping for 1024QAM and 4096QAM

References

- RP-251881 “New SID: Study on 6G Radio,” 3GPP RAN#108, Jun. 2025.

- 3GPP RAN1 Chair’s notes RAN1#122, Aug. 2025.

- 3GPP RAN1 Chair’s notes RAN1#122-bis, Oct. 2025.