3GPP TSG RAN WG1 #123	R1-25009154

Dallas, US, Nov 17th – 21st, 2025

Source: 	MediaTek Inc.

Title:	R2D aspects for Ambient IoT

Agenda item:	10.3.2.1

Document for:	Discussion/Decision

- Introduction

In RAN #109, the revised SID on enhancements for solutions for Ambient IoT in NR outdoor for active devices was described in the [1]:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

In this contribution, we discuss the necessary and feasible changes to the Rel-19 A-IoT specifications on R2D signals, channels, waveform and procedures to support A-IoT active devices in outdoor scenarios.

- R2D synchronization signal

In the last R122bis meeting [2], the following agreements on L1 control information are supported:

Agreement Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking. - Alt1: periodic synchronization signal - Alt2: aperiodic synchronization signal

In the case of DO-A transmission in R20, a device is expected to send the first DO-A D2R message without waiting for a triggering R2D message. Therefore, the device must perform frequency and timing synchronization before transmitting the initial DO-A D2R message. Otherwise, the reader would need to continuously monitor for D2R messages and allocate a large guard bandwidth for D2R transmission. To enable stable synchronization or tracking, it is proposed that a periodic synchronization signal be supported.

Proposal 1: A periodic synchronization signal can be supported

For the case of the R2D synchronization signal has the functionality of timing synchronization/tracking, the R2D synchronization signal can not be a single tone signal. The R2D synchronization signal should be sequence-based, using known deterministic sequences with desirable correlation properties.

Proposal 2: A sequence-based synchronization signal can be supported

- R2D CFO calibration

In the last R122bis meeting, the following agreements on L1 control information are supported:

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Based on discussions in previous meetings, there are two main types of CFO calibration signals under consideration: sequence-based signals (such as 0/1 patterns) and single-tone signals. From the perspective of device CFO calibration complexity, the single-tone signal—which occupies only one RE—is advantageous for analog circuit tracking and RF frequency locking. On the other hand, from the standpoint of minimizing resource overhead, a sequence-based signal can efficiently support both CFO calibration and timing synchronization through correlation and channel estimation. Therefore, we believe that both option a and option c are effective solutions.

Proposal 3: The CFO calibration signal can be selected from one of the following options:

- Option a: unmodulated sinusoid single tone

- Option c: modulated multiple tones

Given the varying characteristics of D2R transmissions, such as whether the transmissions are periodic or aperiodic, it is important to consider different types of CFO calibration signals to best suit each scenario. For instance, in the case of periodic D2R transmissions, the reader can transmit periodic CFO calibration signals. These periodic signals can be aligned with existing synchronization signals or broadcast signals, thereby providing regular opportunities for devices to perform frequency calibration and maintain accurate synchronization with the reader.

On the other hand, for aperiodic D2R transmissions, where the timing of transmissions is irregular or event-driven, it may be more appropriate for the reader to transmit aperiodic CFO calibration signals. These aperiodic signals can be associated with specific channels, such as the PRDCH, and can be triggered as needed to support devices that require calibration at unpredictable intervals. By supporting both periodic and aperiodic CFO calibration signals, the system can flexibly accommodate a wide range of D2R transmission patterns and improve the calibration accuracy.

Proposal 4: Both periodic and aperiodic CFO calibration signal can be supported

In scenarios where a modulated multiple-tone CFO calibration signal is supported, it is proposed that the CFO calibration signal be designed to be identical to the synchronization signal. Aligning the CFO calibration signal with the synchronization signal enables devices to perform both frequency offset calibration and timing synchronization using a single signal, thereby improving resource efficiency and minimizing overhead. Furthermore, utilizing a unified signal for both purposes can enhance system robustness and simplify standardization efforts. Therefore, we recommend that, when modulated multiple-tone CFO calibration is employed, the CFO calibration signal should be the same as the synchronization signal.

Proposal 5: For the case of a modulated multiple tones CFO calibration signal is supported, the CFO calibration signal is the same as the synchronization signal.

- R2D L1 control information

In the last R122bis meeting, the following agreements on L1 control information are supported:

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted Agreement For the functionality of indicating the end of the PRDCH, study the following option: - Option 1: Use the L1 R2D control information to indicate the TBS. Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration Agreement For the functionality of chip duration determination of the data payload in the PRDCH, study the following options: - Option 1: same as the chip duration of the corresponding L1 R2D control information - Option 2: indicated by the corresponding L1 R2D control information

The main functionality of the L1 control information is to scheduling the PRDCH, e.g., the chip duration of PRDCH, the TBS of PRDCH, the enable or disable of the CFO calibration signal which is transmitted with PRDCH, etc.. Therefore, it is important to distinguish L1 control information from the PRDCH payload. The current PRDCH design already provides adequate support for transmitting L1 control information, allowing it to be efficiently carried within a PRDCH.

Proposal 6: The L1 R2D control information is in a PRDCH which is different from the PRDCH that carries the payload.

Proposal 7: The chip duration of the L1 R2D control information can be selected from the following options:

- Option 1: Fixed chip duration.

- Option 3: Use CAP

- R2D broadcast information

In the last R122bis meeting, the following agreement is supported:

Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

The broadcast information plays a crucial role in the overall system operation and should, at a minimum, include details about the resources allocated for the first DO-A message. This ensures that all devices are aware of when and where the initial DO-A message will be transmitted, enabling them to properly synchronize and prepare for communication.

The existing PRDCH is already designed to efficiently support the transmission of such broadcast information. Its current structure and capabilities are sufficient to carry the necessary broadcast details without requiring any additional modifications or the introduction of new channels. Therefore, it is both practical and efficient to transmit the broadcast information within the PRDCH.

Furthermore, to maximize the effectiveness of the broadcast information, it should be transmitted in association with the synchronization signal. By aligning the broadcast information with the synchronization signal, devices can reliably receive critical system information at predictable intervals, which helps maintain system-wide synchronization and coordination. Additionally, making the broadcast information transmission periodic ensures that devices joining the network at different times can still access the necessary information in a timely manner.

Proposal 8: The broadcast information is carried by PRDCH

Proposal 9: The broadcast information can be transmitted periodically and right after the synchronization signal

- Conclusion

In this contribution, the following observation and proposals are made:

Proposal 1: A periodic synchronization signal can be supported

Proposal 2: A sequence-based synchronization signal can be supported

Proposal 3: The CFO calibration signal can be selected from one of the following options:

- Option a: unmodulated sinusoid single tone

  - Option c: modulated multiple tones

Proposal 4: Both periodic and aperiodic CFO calibration signal can be supported

Proposal 5: For the case of a modulated multiple tones CFO calibration signal is supported, the CFO calibration signal is the same as the synchronization signal.

Proposal 6: The L1 R2D control information is in a PRDCH which is different from the PRDCH that carries the payload.

Proposal 7: The chip duration of the L1 R2D control information can be selected from the following options:

- Option 1: Fixed chip duration.

  - Option 3: Use CAP

Proposal 8: The broadcast information is carried by PRDCH

Proposal 9: The broadcast information can be transmitted periodically and right after the synchronization signal

- Reference

- RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, LG Electronics, RAN #109, September 15-18, 2025.

- R1-2508175, FL summary #9 for 10.3.2 “Study of air interface for Device 2b/C”, Moderator (LG Electronics), RAN1#122bis, Oct 13th – 17th, 2025