3GPP TSG RAN WG1 #123                                                                                           R1-2509244

Dallas, USA, Nov 17th – 21st, 2025

Source:	ZTE Corporation, Sanechips

Title:	Maintenance on Low band carrier aggregation via switching

Agenda item:	8.8

Document for:	Discussion and Decision

Introduction

After RAN1#121 meeting, the WI “Low band carrier aggregation via switching” has been completed from RAN1 perspective.

In this contribution, we provide our analysis and potential TPs on the remaining issues on Low band carrier aggregation via switching based on the latest RAN1 spec [1].

HARQ-ACK information bits for SPS PDSCH receptions

In the current specification, there is a semi-static Type-1 HARQ-ACK codebook and a dynamic Type-2 HARQ-ACK codebook. Meanwhile, HARQ-ACK information bits for SPS PDSCH receptions can be independently generated as Type-1 HARQ-ACK codebook or can be included in Type-1 HARQ-ACK codebook, or can be independently generated and appended to Type-2 HARQ-ACK codebook, as highlighted with yellow below. Furthermore, the HARQ-ACK information bits for SPS PDSCH receptions are generated only for the activated SPS PDSCH receptions, and are not required for the configured but not activated SPS PDSCH receptions or the deactivated SPS PDSCH receptions, as highlighted with cyan below.

Therefore, HARQ-ACK information bits for SPS PDSCH receptions can be applied for both Type-1 HARQ-ACK codebook and Type-2 HARQ-ACK codebook. It is generated not only depended on semi-static RRC configuration, but also based on DCI activation.

9.1.2 Type-1 HARQ-ACK codebook determination <Unchanged parts are omitted> If a UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast or multicast HARQ-ACK information, the UE reports HARQ-ACK information in a PUCCH only for one of - a SPS PDSCH release indicated by DCI format 1_0 or by DCI format 4_1, with counter DAI field value of 1, or - a PDSCH reception providing a transport block having enabled HARQ-ACK information report and scheduled by DCI format 1_0 or by DCI format 4_1, with counter DAI field value of 1 on the PCell, or - SPS PDSCH receptions with transport blocks having enabled HARQ-ACK information reports, in the MA,c occasions for candidate PDSCH receptions as determined in clause 9.1.2.1, in which case the UE generates a HARQ-ACK information only for the SPS PDSCH release, or only for the PDSCH reception, or only for SPS PDSCH receptions according to corresponding MA,c occasions on respective serving cells, where the value of counter DAI in DCI format 1_0 or in DCI format 4_1 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in clause 9.1.2.1 and clause 9.1.2.2 for a HARQ-ACK codebook determination apply. In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD−NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321] where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel <Unchanged parts are omitted> If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI and excluding the ones that provide only transport blocks with disabled HARQ-ACK information report, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits. <Unchanged parts are omitted>

In RAN1#122 meeting, the following agreement was achieved to guarantee that the unavailable SPS PDSCH overlapped with switching pattern is not involved in handling of overlapping among SPS PDSCHs within a slot.

Agreement

- Following TP to Section5.1, TS38.214 is endorsed in principle

5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for PDSCH without a corresponding PDCCH transmission receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>

Since the SPS PDSCH receptions overlapping with a set of symbols determined as non-active period of cell DTX shall be excluded from the generation of HARQ-ACK information bits for SPS PDSCH receptions, SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern shall be excluded from the generation of HARQ-ACK information bits for SPS PDSCH receptions based on same principle. Otherwise, different UE behaviors shall be required to generate HARQ-ACK information bits for SPS PDSCH receptions for the following two cases.

- Case 1: There is one PDSCH on a serving cell without a corresponding PDCCH transmission in a slot, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern will be included.

- Case 2: There are more than one PDSCH on a serving cell each without a corresponding PDCCH transmission in a slot, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern will be excluded.

if { a UE is configured to receive SPS PDSCHs from slot nD−NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321] }

Proposal 1: SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern shall be excluded from the generation of HARQ-ACK information bits for SPS PDSCH receptions. The following TP for clause 9.1.2 in TS38.213 should be adopted.

Reason for change: HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) based on the switching pattern will be excluded when there is more than one SPS PDSCH on a serving cell in a slot, but will be included when there is only one SPS PDSCH on a serving cell in a slot. Therefore, different UE behaviors on HARQ-ACK information bits generation for SPS PDSCH receptions are required. Summary of change: Align the UE behaviour on HARQ-ACK information bits generation for SPS PDSCH receptions. That is, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) based on the switching pattern will be excluded regardless there is more than one or only one SPS PDSCH on a serving cell in a slot. Consequences if not approved: For low NR band carrier aggregation via switching, different UE behaviors on HARQ-ACK information bits generation for SPS PDSCH receptions are required. 9.1.2 Type-1 HARQ-ACK codebook determination <Unchanged parts are omitted> In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD−NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321], and/or - not available for PDSCH without a corresponding PDCCH transmission receptions as described in clause 24 where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while <Unchanged parts are omitted>

Operation on repetition transmission and reception

When the UE is provided LBCA-SwitchingPattern, RRC configured signals/channels shall be not transmitted/received if the set of symbols for the signals/channels includes at least one symbol that is unavailable based on the switching pattern, as shown below.

The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’.

While, based on the agreement, collision handling of semi-static DL/UL channels/signals is per-slot, based on the same principle of the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum.

Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

Therefore, the collision handling for higher-layer configured transmission and reception is per repetition(slot) in case repetition is configured. For example, SPS PDSCH repetition shall be omitted per slot, not per multi-slot PDSCH.

Proposal 2: For Rel-19 low NR band carrier aggregation via switching, the collision handling for higher-layer configured transmission and reception is per repetition if overlapping with at least one unavailable symbol based on the switching pattern. The following TP for clause 24 in TS38.213 should be adopted.

Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that collision handling for higher-layer configured transmission and reception is per repetition(slot) or per channel(all slots), if overlapping with at least one unavailable symbol of a slot based on the switching pattern. Summary of change: The collision handling for higher-layer configured transmission and reception is per repetition if overlapping with at least one unavailable symbol based on the switching pattern. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear the collision handling for higher-layer configured transmission and reception is per repetition or per channel. 24 Downlink carrier aggregation via switching <Unchanged parts are omitted> When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers overlaps, even partially, within a set of symbols of a slot when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers overlaps, even partially, within a set of symbols of a slot on the PCell when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers overlaps, even partially, within a set of symbols of a slot on the SCell when the set of symbols of the slot includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. <Unchanged parts are omitted>

RRC parameters alignment

The RRC parameters in TS38.213 and TS38.331 are not aligned. In order to align the parameters in TS38.213 with that in TS38.331, LBCA-SwitchingPattern, LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-SCelltoPCell shall be changed to switchingPattern, gapDurationPCelltoSCell and gapDurationSCelltoPCell, respectively.

Proposal 3: For Rel-19 low NR band carrier aggregation via switching, RRC parameters in TS38.213 shall be aligned with that in TS38.331. The following TP for clause 24 in TS38.213 should be adopted.

Reason for change: For Rel-19 low NR band carrier aggregation via switching, RRC parameters in TS38.213 and TS38.331 are not aligned. Summary of change: Change LBCA-SwitchingPattern, LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-SCelltoPCell to switchingPattern, gapDurationPCelltoSCell and gapDurationSCelltoPCell, respectively. Consequences if not approved: RRC parameters in TS38.213 and TS38.331 are not aligned. 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPatternswitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPatternswitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCellgapDurationPCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCellgapDurationSCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. <Unchanged parts are omitted>

Conclusion

In this contribution, the proposals are summarized as follows:

Proposal 1: SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern shall be excluded from the generation of HARQ-ACK information bits for SPS PDSCH receptions. The following TP for clause 9.1.2 in TS38.213 should be adopted.

Reason for change: HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) based on the switching pattern will be excluded when there is more than one SPS PDSCH on a serving cell in a slot, but will be included when there is only one SPS PDSCH on a serving cell in a slot. Therefore, different UE behaviors on HARQ-ACK information bits generation for SPS PDSCH receptions are required. Summary of change: Align the UE behaviour on HARQ-ACK information bits generation for SPS PDSCH receptions. That is, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) based on the switching pattern will be excluded regardless there is more than one or only one SPS PDSCH on a serving cell in a slot. Consequences if not approved: For low NR band carrier aggregation via switching, different UE behaviors on HARQ-ACK information bits generation for SPS PDSCH receptions are required. 9.1.2 Type-1 HARQ-ACK codebook determination <Unchanged parts are omitted> In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD−NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or - determined as non-active period of cell DTX [11, TS 38.321], and/or - not available for PDSCH without a corresponding PDCCH transmission receptions as described in clause 24 where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while <Unchanged parts are omitted>

Proposal 2: For Rel-19 low NR band carrier aggregation via switching, the collision handling for higher-layer configured transmission and reception is per repetition if overlapping with at least one unavailable symbol based on the switching pattern. The following TP for clause 24 in TS38.213 should be adopted.

Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that collision handling for higher-layer configured transmission and reception is per repetition(slot) or per channel(all slots), if overlapping with at least one unavailable symbol of a slot based on the switching pattern. Summary of change: The collision handling for higher-layer configured transmission and reception is per repetition if overlapping with at least one unavailable symbol based on the switching pattern. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear the collision handling for higher-layer configured transmission and reception is per repetition or per channel. 24 Downlink carrier aggregation via switching <Unchanged parts are omitted> When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers overlaps, even partially, within a set of symbols of a slot when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers overlaps, even partially, within a set of symbols of a slot on the PCell when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers overlaps, even partially, within a set of symbols of a slot on the SCell when the set of symbols of the slot includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. <Unchanged parts are omitted>

Proposal 3: For Rel-19 low NR band carrier aggregation via switching, RRC parameters in TS38.213 shall be aligned with that in TS38.331. The following TP for clause 24 in TS38.213 should be adopted.

Reason for change: For Rel-19 low NR band carrier aggregation via switching, RRC parameters in TS38.213 and TS38.331 are not aligned. Summary of change: Change LBCA-SwitchingPattern, LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-SCelltoPCell to switchingPattern, gapDurationPCelltoSCell and gapDurationSCelltoPCell, respectively. Consequences if not approved: Parameters in TS38.213 and TS38.331 are not aligned. 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPatternswitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPatternswitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCellgapDurationPCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCellgapDurationSCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. <Unchanged parts are omitted>

Reference

- TS38.213-j10, Physical layer procedures for control.