3GPP TSG RAN WG1 #123	R1-2509362

Dallas, 17 – 21 November 2025

Agenda item:		10.2.1

Source:	Rakuten

Title:	Discussion on Improvement of SRS Capacity and Coverage

Document for:		Discussion and Decision

# Introduction

The WI of Study on NR MIMO Phase 6 in RAN #108 includes the following objectives [1]:

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

    - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

    - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this report, we share our views and studies on multiple frequency-domain starting positions within each SRS hop and SRS transmission across S slot and adjacent U slot.

# Discussion

## Multiple frequency-domain starting positions within each SRS hop

### Patterns for intra-hopping starting positions for the case R/K>1

While the patterns intra-hopping starting positions for the case R/K=1 has been agreed in RAN1 #122bis meeting, the meeting has not yet reached a consensus on the patterns for the case R/K>1, leaving two different approaches, Alternative 1 for consecutive mapping, where the frequency domain starting position of SRS transmission retains in R/K consecutive symbols, and Alternative 2 for non-consecutive mapping, where the starting position keeps being hopped from one symbol to another within every cycle of K symbols.

We view that both of them are effective: none of them will perform worse than the other for interpolating the channel estimates and none of them will complicate the spec than the other. However, non-consecutive mapping can benefit some use cases. For example, when the SRS transmission is cut short in time to give resource to other channels such as PUSCH, the gNB will still receive SRS at multiple positions in frequency domain if non-consecutive mapping is facilitated, hence performing better in channel interpolation. This exemplified scenario would happen when gNB wants to prioritize resource for PUSCH in short term when the coverage extension for SRS is not an issue for a period of time. Therefore, we express our preference on non-consecutive mapping in the following proposal:

Proposal 1: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop when R/K>1, support non-consecutive mapping (i.e. Alt-2) for the following cases:

- PF=K=2

- PF=4 and K=2

- PF=K=4.

## SRS transmission across S slot and adjacent U slot

### Additional SRS resource in the SRS resource set including at least a cross-slot SRS resource

At RAN1 #122bis [2], it has been agreed to interpret the resource set level slot offset for aperiodic cross-slot SRS resource set in scenario 1 as the first of two slots as follows:

Agreement: - For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set. - Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately. - Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately. - Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

Scenario 1 pertains to the configuration of an SRS resource set, which can be periodic, semi-persistent, or aperiodic, that includes at least one SRS resource with a time-domain allocation spanning two consecutive slots, where an S slot precedes a U slot according to an agreement in RAN1 #122 meeting [3]. Consistent with Notes 1 in the agreement above, the outstanding issue is whether an aperiodic SRS resource set may also include an additional SRS resource whose time-domain allocation is entirely contained within the second slot.

In our opinion, the configuration of an SRS resource set comprising two resources, one spanning both slots and the other fully within the second slot, is considered essential to enable antenna switching. Conversely, configurations where neither resource is fully contained within the second slot remain feasible and should not be excluded from the enhanced feature. The former is referred to as Interpretation 1 of Scenario 1 while the latter is referred to as Interpretation 2 of Scenario 1 in the offline discussion. We however do view them as use cases of SRS resource set configuration in supporting cross-slot SRS rather than interpretations. Therefore, our position is reflected in the following proposal:

Proposal 2: For Scenario 1, where a periodic, semi-persistent, or aperiodic SRS resource set includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, it is proposed to support configurations in which the SRS resource set may include an additional SRS resource whose time-domain transmission is either:

- Entirely within the first S slot; or

- Entirely within the second U slot.

The second use case in this proposal corresponds to Scenario 2, which was already agreed to be studied as referenced in [3]. Consequently, further study of Scenario 2 is no longer required.

In a cross-slot SRS resource set configuration as described in the scenario of Proposal 1, the time-domain transmission of an SRS resource with loffset≥NsymbSRS-1 can be fully placed within either the first slot or second slot. To determine which use case is applied, we agree the opinion presented in the offline discussion in RAN1122bis meeting that propose an slot-level indication to indicate the use case. Therefore, we propose the following:

Proposal 3: In a periodic, semi-persistent, or aperiodic SRS resource set that includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, for any other SRS resource whose configuration parameters satisfy loffset≥NsymbSRS-1, introduce an optional flag to indicate the second U slot to contain the entire time-domain SRS transmission

- If the flag is absent, the SRS is transmitted in the first S slot.

We need to clearly indicate whether the first or second slot is used for the SRS resource that meets the condition loffset≥NsymbSRS-1. The UE assumes there is no conflict between SRS resources in time-domain in terms of UE’s capability on SRS switching timing and will not adjust slots unless the flag introduced in Proposal 2 explicitly signals it. We note that the configuration with enabled flag does not conflict with an agreement in RAN1 #122 meeting quoted bellow:

Agreement To determine the time-domain location of cross-slot SRS, - One set of time-domain resource allocation related parameters (i.e., startPosition, nrofSymbols, and repetitionFactor) is configured for the SRS resource without restriction on “within a slot”. - For the index of each SRS symbol , it is the same as legacy spec., i.e., l’ = 0, 1, …, NsymbSRS-1. - The offset loffset∈{ 0, 1, …, 13} counts symbols backwards from the end of the starting slot of the resource

where the starting slot of the resource is considered as the U slot when the flag is enabled.

### Additional symbol offset for cross-slot SRS resource

The agreement referenced above specifies the starting symbol for any SRS resource, regardless of whether it spans multiple slots. Furthermore, the agreement reached at the RAN1 #122bis meeting [2] clarifies how the time-domain allocation is determined for a cross-slot SRS resource:

Agreement To determine the time-domain location of a cross-slot SRS resource, - ‘l0’ is interpreted as the starting symbol in the starting slot for an SRS resource. - ‘l'+l0’ is interpreted as the symbol index in a slot or two consecutive S and U slots. - The restriction ‘loffset≥NsymbSRS-1’ is removed.

We interpret that if NsymbSRS>loffset+1, loffset+1 SRS symbols reside in the first S slot and the remaining NsymbSRS-loffset-1 SRS symbols will be contained in the second U slot, starting from OFDM symbol 0 to OFDM symbol NsymbSRS-loffset-2. However, we also see that instead of starting from OFDM symbol 0, it is advantageous to configure the starting symbol in the second U slot to allocate them towards the end of the slot. Certain use cases may benefit from reserving some initial symbols in this slot for PUSCH and excluding them from SRS transmission. Based on this, we propose the following:

Proposal 4: For Scenario 1, where a periodic, semi-persistent, or aperiodic SRS resource set includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, support the time-domain transmission of the cross-slot SRS to start from a symbol later than the first symbol of the second slot.

To enable this functionality, we propose introducing a parameter at the SRS resource level as outlined below:

Proposal 5: Introduce an optional parameter ‘startPosition2’ at the SRS resource level to indicate the starting position in the time domain of the second slot when SRS transmission across S slot and adjacent U slot is enabled.

- This slot-level parameter is only applicable if the first of NsymbSRS SRS symbol occurs in an S slot

- If ‘startPosition2’ is not specified, the time-domain transmission of the cross-slot SRS resource in the second slot starts from the first symbol of that slot.

We denote the configured value for parameter ‘startPosition2’ as loffset,2. In an S slot, the number of symbols available for uplink transmission, denoted by Nsymbslotn, may be smaller than the configured number of symbols NsymbSRS for SRS transmission. In such case, if SRS transmission across S slot and adjacent U slot is enabled, the allocation of symbols for SRS transmission is proposed as follows:

Proposal 6: When SRS is transmitted across from an S slot of index n, which contains Nsymbslotn symbols (Nsymbslotn<Nsymbslot=14) to an adjacent U slot,

- it occupies NsymbSRS'=minNsymbSRS,loffset'+1 consecutive symbols in the S slot, starting from OFDM symbol l0=Nsymbslot-1-loffset' in this slot,

  - loffset'=minloffset,Nsymbslotn-1,

- If NsymbSRS-NsymbSRS'>0, the remaining NsymbSRS-NsymbSRS' symbols are allocated in the U slot, starting from l0,2=Nsymbslot-1-loffset,2,

  - loffset,2 is configured as slot-level parameter ‘startPosition2’

    - If the configuration for loffset,2 is absent, loffset,2=Nsymbslot-1.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:.

Proposal 1: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop when R/K>1, support non-consecutive mapping (i.e. Alt-2) for the following cases:

- PF=K=2

- PF=4 and K=2

- PF=K=4.

Proposal 2: For Scenario 1, where a periodic, semi-persistent, or aperiodic SRS resource set includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, it is proposed to support configurations in which the SRS resource set may include an additional SRS resource whose time-domain transmission is either:

- Entirely within the first S slot; or

- Entirely within the second U slot.

Proposal 3: In a periodic, semi-persistent, or aperiodic SRS resource set that includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, for any other SRS resource whose configuration parameters satisfy loffset≥NsymbSRS-1, introduce an optional flag to indicate the second U slot to contain the entire time-domain SRS transmission

- If the flag is absent, the SRS is transmitted in the first S slot.

Proposal 4: For Scenario 1, where a periodic, semi-persistent, or aperiodic SRS resource set includes at least one SRS resource with a time-domain allocation spanning a first S slot and a consecutive U slot, support the time-domain transmission of the cross-slot SRS to start from a symbol later than the first symbol of the second slot.

Proposal 5: Introduce an optional parameter ‘startPosition2’ at the SRS resource level to indicate the starting position in the time domain of the second slot when SRS transmission across S slot and adjacent U slot is enabled.

- This slot-level parameter is only applicable if the first of NsymbSRS SRS symbol occurs in an S slot

- If ‘startPosition2’ is not specified, the time-domain transmission of the cross-slot SRS resource in the second slot starts from the first symbol of that slot.

Proposal 6: When SRS is transmitted across from an S slot of index n, which contains Nsymbslotn symbols (Nsymbslotn<Nsymbslot=14) to an adjacent U slot,

- it occupies NsymbSRS'=minNsymbSRS,loffset'+1 consecutive symbols in the S slot, starting from OFDM symbol l0=Nsymbslot-1-loffset' in this slot,

  - loffset'=minloffset,Nsymbslotn-1,

- If NsymbSRS-NsymbSRS'>0, the remaining NsymbSRS-NsymbSRS' symbols are allocated in the U slot, starting from l0,2=Nsymbslot-1-loffset,2,

  - loffset,2 is configured as slot-level parameter ‘startPosition2’

    - If the configuration for loffset,2 is absent, loffset,2=Nsymbslot-1.

# References

| [1] | RP-251856, “WI: NR MIMO Phase 6,” Samsung. |
| --- | --- |
| [2] | RAN1 #122bis, “RAN1 Chair's Notes,” October 2025. |
| [3] | RAN1 #122, “RAN1 Chair's Notes,” August 2025. |