3GPP TSG RAN WG1 #123			R1-2509447

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.4

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 8.4

Document for:	Endorsement

  - Maintenance on Solutions for Ambient IoT (Internet of Things) in NR

Note: Maximum one contribution.

[123-R19-A-IoT] Email discussion on A-IoT – Jingwen (CMCC)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509447	Session Notes of AI 8.4	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2509485	Summary #1 for maintenance on solutions for A-IoT in NR	Moderator (CMCC)

Agreement

RAN1 adopts the following text proposal

| Reasons for change | Capture the RAN1 agreement that a device is not required to monitor R2D during TR→D for preparing D2R transmission. |
| --- | --- |
| Summary of change | Add to Clause 7.2.3 that a device is not required to monitor R2D for the time between chip χR2D= χendR2D and chip χD2R=0 as determined in Clause 7.1.2. |
| Consequences if not approved | The RAN1 agreement is not captured in the specification. |
| Text proposals | <Unchanged parts omitted> ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - for a duration between the end of chip χR2D= χendR2D and the start of chip χD2R=0 as determined in Clause 7.1.2, and when performing a D2R transmission, - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. <Unchanged parts omitted> |

Agreement

  - Device assumes that there is a transition edge at the end of R2D.

  - FFS: TP to adapt above

Agreement

Update the agreement as follows.

Agreement

  - Device assumes that there is a transition edge at the end of R2D.

  - TP to capture the agreement is “Device assumes a transition edge at the end of chip χR2D= χendR2D.”

  - FFS: TP to adapt above

R1-2509486	Summary #2 for maintenance on solutions for A-IoT in NR	Moderator (CMCC)

R1-2508407	Discussion on potential overlapping between TDMed Msg1 transmissions	vivo

R1-2508442	Maintenance on Rel-19 Ambient IoT	CMCC

R1-2508503	Maintenance on Rel-19 Ambient IoT	Huawei, HiSilicon

R1-2508541	Maintenance on Solutions for Ambient IoT in NR	NEC

R1-2508572	Remaining issues on Rel-19 A-IoT	CATT

R1-2508662	Discussion on remaining issues for Ambient IoT in NR	Xiaomi

R1-2508709	Maintenance on Rel-19 Ambient IoT	OPPO

R1-2508778	Maintenance issues on Rel-19 A-IoT	Samsung

R1-2508813	Discussion on remaining issues of Rel-19 Ambient IoT	ZTE Corporation, Sanechips

R1-2508893	Maintenance on Rel-19 Ambient IoT	LG Electronics

R1-2508996	Rel-19 AIoT (Ambient_IoT_Solutions) maintenance	Nokia

R1-2509024	Maintenance of AIoT	Ofinno

R1-2509118	Maintenance of Rel-19 Ambient IoT	InterDigital, Inc.

R1-2509202	Maintenance on Ambient IoT	Qualcomm Incorporated

R1-2509259	Maintenance on solutions for Ambient IoT in NR	NTT DOCOMO, INC.