3GPP TSG RAN WG1 #123	                                   	R1-2509436

Dallas, USA, Nov 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.1: UE features Batch A (NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, TEI19 with [Common_PDCCH_rep_TN])

Agenda Item:		9.1

Document for: 	Endorsement

  - UE features Batch A

Note: Maximum one contribution per company/organization/university for NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, and TEI19 with [Common_PDCCH_rep_TN])

Note: For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- NTN_Ph3

- IoT_NTN_Ph3

- IoT_NTN_TDD

- TEI19 with [Common_PDCCH_rep_TN])

NR-NTN, TEI19 with [Common_PDCCH_rep_TN]

Proposal 4.1.2-3 [ASN1]:

Regarding FG 65-1-2 and 67-9,

- Put “YES” in “Need for the gNB to know if the feature is supported” column

- Confirm “Optional with capability signalling” in mandatory/optional column

Proposal 4.1.2-1 [ASN1]:

Regarding FG 65-1-2/3 and 67-9/10:

- Remove “FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition” from FG 65-1-2

- Remove “FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS”

- Adopt Alt 1 below

  - Alt 1: Confirm 65-1-3 and 67-10 (i.e., separate FG for Type 0A/0B/1/1A/2/2A PDCCH CSS)

    - For FG 65-1-3, update the consequence column to “PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported”

      - Add “conditionally mandatory with capability signalling for UE supporting FG 65-1-2”

      - Add FG 65-1-2 as prerequisite FG

    - For FG 67-10, update the consequence column to “PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN is not supported”

      - Add “conditionally mandatory with capability signalling for UE supporting FG 67-9”

      - Add FG 67-9 as prerequisite FG

Proposal 4.1.3-2 [ASN1]:

Confirm “optional with capability signaling” in FG 65-1-5

Proposal 4.1.2-2 (if FG 65-1-3 and 67-10 are confirmed):

Regarding FG 65-1-3 and 67-10,

- Remove FFS from Prerequisite column

Question 4.1.3-1:

Adopt Alt-1 for prerequisite FG in 65-1-5

- Alt-1: One of {5-17a and 16-2b-5}

- Alt-2: None

R1-2509436	Session Notes of AI 9.1: UE features Batch A (NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, TEI19 with [Common_PDCCH_rep_TN])	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2508353	UE features Batch A (NTN and TEI19)	Nokia

R1-2508415	Remaining issues of UE features for NR_NTN_Ph3 and TEI (Common_PDCCH_rep_TN)			vivo

R1-2508482	Remaining issues for Rel-19 WIs in UE features Batch A	Huawei, HiSilicon

R1-2508576	Discussion on UE features for NTN	CATT

R1-2508650	UE features Batch A (NR-NTN Phase 3 and TEI19)	Ericsson

R1-2508667	UE features for NTN for NR Phase 3	Xiaomi

R1-2508784	UE features for batch A	Samsung

R1-2508852	Discussion on the UE features Batch A	ZTE Corporation, Sanechips

R1-2508961	Discussion on UE features for NR NTN Phase 3	ETRI

R1-2509090	Views on UE features Batch A	Apple

R1-2509162	Discussions on UE Features for NR NTN Ph3	MediaTek Inc.

R1-2509192	UE features for Batch A	OPPO (chongqing) Intelligence

R1-2509210	UE features for NTN	Qualcomm Incorporated

R1-2509263	Discussion on UE features Batch A	NTT DOCOMO, INC.