3GPP TSG RAN WG1 #123	                                   	R1-2509436

Dallas, USA, Nov 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.1: UE features Batch A (NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, TEI19 with [Common_PDCCH_rep_TN])

Agenda Item:		9.1

Document for: 	Endorsement

  - UE features Batch A

Note: Maximum one contribution per company/organization/university for NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, and TEI19 with [Common_PDCCH_rep_TN])

Note: For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- NTN_Ph3

- IoT_NTN_Ph3

- IoT_NTN_TDD

- TEI19 with [Common_PDCCH_rep_TN])

NR-NTN, TEI19 with [Common_PDCCH_rep_TN]

Agreement:

Regarding FG 65-1-2 and 67-9,

- Put “YES” in “Need for the gNB to know if the feature is supported” column

- Confirm “Optional with capability signalling” in mandatory/optional column

Agreement:

Regarding FG 65-1-2/3 and 67-9/10:

- Remove “FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition” from FG 65-1-2

- Remove “FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS”

- Adopt Alt 1 below

  - Alt 1: Confirm 65-1-3 and 67-10 (i.e., separate FG for Type 0A/0B/1/1A/2/2A PDCCH CSS)

    - For FG 65-1-3, update the consequence column to “PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported”

      - Add “conditionally mandatory with capability signalling for UE supporting FG 65-1-2”

      - Add FG 65-1-2 as prerequisite FG

    - For FG 67-10, update the consequence column to “PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN is not supported”

      - Add “conditionally mandatory with capability signalling for UE supporting FG 67-9”

      - Add FG 67-9 as prerequisite FG

Agreement:

Confirm “optional with capability signaling” in FG 65-1-5

| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | see Note | N/A | UE does not support an additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional with capability signaling Note: The capability signalling is introduced to allow the NW to collect the statistics about the percentage of UEs that support this feature FFS: A UE that supports Rel-19 NR-NTN must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-1-2 | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 2. Support reception of SIB1 PDSCH repetition within 20 ms duration |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration are not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| 65-1-3 | PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS 2. Required number of BDs for the two PDCCH candidates in RRC CONNECTED mode | 65-1-2 | Yes | N/A | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Candidate values for component 2: {1, 2} | Conditionally mandatory with capability signaling for UE supporting FG 65-1-2 |
| 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition 2. Support of repetition factors 2 and 4 | [FFS] | YES | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |

| 67-9 | PDCCH repetition for Type0 PDCCH CSS in TN [Common_PDCCH_rep_TN] | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 in TN |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS in TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 67-10 | PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS for TN | 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS for TN 2. Required number of BDs for the two PDCCH candidates in RRC CONNECTED mode | 67-9 | Yes | N/A | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN Candidate values for component 2: {1, 2} | Conditionally mandatory with capability signaling for UE supporting FG 67-9 |

Proposal for FG 65-1-3/67- completion:

Confirm [0A/0B/1/1A/2/2A] for FG 65-1-3 and 67-10 (both FG name and component)

Proposal 4.1.3-1:

Adopt Alt-1 for prerequisite FG in 65-1-5

- Alt-1: One of {5-17a and 16-2b-5}

- Alt-2: None

Proposal 4.1.1-1:

Regarding “FFS: A UE that supports Rel-19 NR-NTN must support this FG” in “Mandatory/optional” column in FG 65-1-1, adopt Alt-1:

- Alt-1: Remove “FFS: A UE that supports Rel-19 NR-NTN must support this FG” in “Mandatory/optional” column

  - Note: This alternative implies that FG 65-1 is optional even for UE supporting Rel-19 NR-NTN feature(s)

- Alt-2: Confirm “A UE that supports Rel-19 NR-NTN must support this FG”

  - Note: This alternative implies that FG 65-1 shall be supported for UE supporting Rel-19 NR-NTN feature(s)

- Alt-3: FG 65-1-1 is defined to be a prerequisite of FGs 65-1-2/65-1-3 (if defined)/65-1-5.

R1-2509436	Session Notes of AI 9.1: UE features Batch A (NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, TEI19 with [Common_PDCCH_rep_TN])	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2508353	UE features Batch A (NTN and TEI19)	Nokia

R1-2508415	Remaining issues of UE features for NR_NTN_Ph3 and TEI (Common_PDCCH_rep_TN)			vivo

R1-2508482	Remaining issues for Rel-19 WIs in UE features Batch A	Huawei, HiSilicon

R1-2508576	Discussion on UE features for NTN	CATT

R1-2508650	UE features Batch A (NR-NTN Phase 3 and TEI19)	Ericsson

R1-2508667	UE features for NTN for NR Phase 3	Xiaomi

R1-2508784	UE features for batch A	Samsung

R1-2508852	Discussion on the UE features Batch A	ZTE Corporation, Sanechips

R1-2508961	Discussion on UE features for NR NTN Phase 3	ETRI

R1-2509090	Views on UE features Batch A	Apple

R1-2509162	Discussions on UE Features for NR NTN Ph3	MediaTek Inc.

R1-2509192	UE features for Batch A	OPPO (chongqing) Intelligence

R1-2509210	UE features for NTN	Qualcomm Incorporated

R1-2509263	Discussion on UE features Batch A	NTT DOCOMO, INC.