3GPP TSG RAN WG1 #123	                                   	R1-2509437

Dallas, USA, Nov 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.2: UE features Batch B (NR_duplex_evo, NR_LPWUS, NR_XR_Ph3, NR_MC_enh2, NR_LBCA_Sw)

Agenda Item:		9.2

Document for: 	Endorsement

  - UE features Batch B

Note: Maximum one contribution per company/organization/university for NR_duplex_evo, NR_LPWUS, XR phase 3, NR_MC_enh2, and NR_LBCA

Note: For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- UE f NR_duplex_evo

- NR_LPWUS

- XR phase 3

- NR_MC_enh2

- NR_LBCA

R1-2509437	Session Notes of AI 9.2: UE features Batch B (NR_duplex_evo, NR_LPWUS, NR_XR_Ph3, NR_MC_enh2, NR_LBCA_Sw)	Ad-Hoc Chair (NTT DOCOMO, INC.)

NR_duplex_evo

Proposed conclusion 2.1-1:

For “support of rate matching for PDSCH crossing boundaries of DL usable PRBs” in SBFD operation,

- Add a new component in FG 60-1: “Support of rate matching for PDSCH crossing boundaries of DL usable PRBs”

- Note: New FG is not defined.

Proposal 2.3-1 [ASN1]:

For Msg3 repetition in SBFD symbols,

- Define the following new FG

| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameter | 60-3 or 60-4, 30-6 | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Inform RAN2 of the issue of practical configurability of Msg3 repetition for SBFD-aware UE, and ask RAN2 whether it is possible to define feature combination(s) for Msg3 repetition in SBFD symbols

Potential way forwards for Msg3 repetition in SBFD symbol

- Alt-1: A new FG for Msg.3 repetition in SBFD symbols is introduced as follows:

| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameter | 60-3 or 60-4, 30-6 | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt-2: Adopt the following updates in FG 60-3 and 60-4

  - Add a new component: “X. Support of Msg.3 repetition in SBFD symbols”

  - Add in Note column: “Component X applies only when the UE reports FG 30-6 in the same band”

- Alt-3: Neither new component nor FG is added for support of Msg.3 repetition in SBFD symbols

Proposal 2.2-1:

- Confirm FG 54-1 as prerequisite FG for FG 60-5a

Agreement:

For “FFS: Whether/how to capture maximum number of UL muting symbols per slot” in FG 60-7b/7c, adopt the following:

- Remove FFS

- Adopt Alt-1.2 below

  - Alt-1.2: Add a component “Maximum number of UL muting symbols per slot”, with candidate values {2, 4}

Agreement:

- Confirm the following FFS for FG 60-8 component 4 candidate values

  - [Candidate values for component 4 are {8, 16, 32, 64}]

- Confirm the following FFS in FG 60-9 component 2 candidate values

  - [Candidate values for component 2 are {4, 8, 16, 32}]

Agreement:

Support the following new components in FG 60-8

- Maximum number of aperiodic CSI report setting per BWP for L1-CLI-RSSI report

  - Candidate values: {1, 2, 3, 4}

- Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Candidate values: All integers from 1 to 32

- Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC

  - Candidate values: All integers from 1 to 16

Proposal 2.5-2-4:

Add a new component in FG 60-8: “Support of active L1-CLI-RSSI resource counting for a single L1-CLI-RSSI resource across two DL subbands”

- Candidate values: {“with factor-of-two relaxation for active L1-CLI-RSSI resource counting”, “no relaxation”}

Proposal 2.5-2-4 (alternative approach):

For active resource counting for L1 CLI-RSSI measurement across two DL subbands, add the following note for FG 60-1:

- Note for component 11: If the UE reporting component 11 as “with factor-of-two relaxation for active CSI-RS resource and port counting” also reports FG 60-7 in the same band, factor-of-two relaxation for active CSI-RS resource and port counting also applies to L1 CLI-RSSI measurement resource

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Number of partial PRGs supported 11. Supported CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands 12. Maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s) 13. Determination of TBS of PDSCH/PUSCH in SBFD symbols |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement Component 10 candidate values: {2, 4} Component 11 candidate values: {“with factor-of-two relaxation”, “with factor-of-two relaxation for active CSI-RS resource and port counting”, “no relaxation”} Note for component 11: If the UE reporting component 11 as “with factor-of-two relaxation for active CSI-RS resource and port counting” also reports FG 60-7 in the same band, factor-of-two relaxation for active CSI-RS resource and port counting also applies to L1 CLI-RSSI measurement resource | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Question 2.6-2 [ASN1]:

Companies are encouraged to share view on whether to add new component for the following in FG 60-9:

- Q2.6-2-1: Maximum number of configured L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Candidate values; {???}

- Q2.6-2-2: Maximum number of simultaneous L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Candidate values; {???}

- Q2.6-2-3: Maximum number of aperiodic CSI report setting per BWP for L1-SRS-RSRP report

  - Candidate values: {1, 2, 3, 4}

- Q2.6-2-4: Maximum number of SRS-RSRP measurement resources across all CCs within a slot

  - Candidate values: {2, 4, 8}

- Q2.6-2-5: Maximum number of aperiodic SRS-RSRP measurement resources across all CCs

  - Candidate values: {1, 2, 3, 4}

NR_LPWUS

Proposed conclusion 2.6-2-2

There is no consensus to adopt the following in Rel-19

- For FG 62-2/2a, add a new component “Nominal MO duration = actual LP-WUS duration”

- Define new FG for supporting nominal MO duration > actual LP-WUS duration for LP-WUS operation in CONNECTED mode

NR_MC_enh2

Proposal: (reply to RAN2)

- From RAN1 perspective, there is no issue identified on the following approach:

  - “define new Rel-19 capability parameters for Rel-18 FG 49-4a/4b/4c/4d/5a/5b/6/7/8/9/10/12/12a/13/14, and FG66-1/66-2 will be defined as prerequisite for the new Rel-19 capability parameters”

- Regarding “whether this new Rel-19 capability parameters can only have one prerequisite of FG66-1/66-2 (i.e., prerequisite of the corresponding Rel-18 FGs are not prerequisite of new Rel-19 capability parameters)”, from RAN1 perspective, down-select one of the following:

  - Alt-1: New Rel-19 capability parameters can have prerequisite of the corresponding Rel-18 FGs as well

  - Alt-2: New Rel-19 capability parameters can have prerequisite of the corresponding Rel-18 FGs as well, but RAN1 doesn’t see the need to have prerequisite of the corresponding Rel-18 FGs

  - Alt-3: New Rel-19 capability parameters cannot have prerequisite of the corresponding Rel-18 FGs as well

- Capture this agreement in LS for the updated NR UE feature list up to RAN1#123 Wednesday

Proposal: (advanced UE capability for larger number of unicast DL/UL DCI, similar to Rel-18 49-3x/3y)

- Remove [FFS support of more than one unicast DL DCI for the case when the UE indicates support of more than one unicast DCI per slot based on Rel-16 capability] in component for FG 66-1/2

- Introduce the following two FGs:

| 66-5x | Advanced UE capability for larger number of unicast DL DCI with different SCS and/or different carrier type | Processing up to X unicast DCI scheduling PDSCH per scheduled cell in a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3: - Up to Y DCI formats 1_3 for the set of cells, and - Up to X unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells in the set of cells - For each cell in the set of cells, no more than X DCIs scheduling PDSCH for the cell - X is based on pair of (scheduling CC SCS, scheduled CC SCS), Y is based on pair of (SCS1, SCS3): - SCS1is the SCS for scheduling CC, SCS2 is the SCS of a cell in the set of cells, and SCS3 is the largest SCS among all cells in the cell set - Candidate value(s) of X, Y - X={2,4, 8} for (15,120), (15,60), (30,120) and X ={2, 4} for (15,30), (30,60), X={1} for (30,30) - Y={2,4,8} for (15,120), (15,60), (30,120) and Y={2,4} for (15,30), (30,60), Y={1} for (30,30) - X and Y apply per slot of scheduling CC | 66-1 | Yes |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-5y | Advanced UE capability for larger number of unicast UL DCI with different SCS and/or different carrier type | Processing up to X unicast DCI scheduling PUSCH per scheduled cell in a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 - Up to Y DCI formats 0_3 for the set of cells, and - Up to X unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells in the set of cells - For a cell in the set of cells, no more than X DCIs scheduling PUSCH for the cell - X is based on pair of (scheduling CC SCS, scheduled CC SCS), Y is based on pair of (SCS1, SCS3): - SCS1is the SCS for scheduling CC, SCS2 is the SCS of a cell in the set of cells, and SCS3 is the largest SCS among all cells in the cell set - Candidate value(s) of X, Y - X={2,4, 8} for (15,120), (15,60), (30,120) and X ={2, 4} for (15,30), (30,60), X={1,2} for (30,30) - Y={2,4,8} for (15,120), (15,60), (30,120) and Y={2,4} for (15,30), (30,60), Y={1} for (30,30) - X and Y apply per slot of scheduling CC | 66-2 | Yes |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |

Proposal: (similar to Rel-18 49-4a/4b/4c/4d)

Introduce the following FG 66-6a/6b/6c/6d:

| 66-6a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 with different SCS and/or different carrier type | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 66-1 | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-6b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 with different SCS and/or different carrier type | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 66-2 | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 66-6c | Configurable Type-1A fields for DCI format 0_3/1_3 with different SCS and/or different carrier type | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {66-1, 66-2} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 66-6d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 with different SCS and/or different carrier type | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {66-1, 66-2} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |

Proposal: (similar to Rel-18 49-5a/5b)

Introduce the following FG 66-7a/7b

| 66-7a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 with different SCS and/or different carrier type | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value | 66-1 | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-7b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 with different SCS and/or different carrier type | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | 66-1 | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. For component 2 and 5, same values as for FG25-6 are reported (if the UE also report FG25-6) | Optional with capability signaling |

Proposal: (similar to Rel-18 49-6/6a/6b)

Introduce the following FG 66-8/8a/8b:

| 66-8 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 with different SCS and/or different carrier type | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and 'codeBlockGroupTransmission" for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot - Candidate values for the component 6 of this FG is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration 7. Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK | 66-1 | Yes |  |  | Per FS | N/A | N/A | N/A | If a UE reports both 11-3 and this FG, it can support two slot-based HARQ-ACK codebooks, and one slot-based and one-sub-slot-based HARQ-ACK codebooks. If a UE reports this FG but not 11-3, it can only support two slot-based HARQ-ACK codebooks. The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs Component 6 is applied to the sub-slot HARQ-ACK codebook. It is assumed that only 1 actual PUCCH transmission for HARQ-ACK within a slot for slot-based HARQ-ACK codebook. - Component 6 is reported for 2-symbol*7 sub-slot configuration. For 7-symbol*2 sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases. For component 6, maximum of 1 actual PUCCH transmission for HARQ-ACK within a slot for slot-based HARQ-ACK codebook. Thus value reported for component 6 has no meaning for "slot-based + slot based". For component 6, same values as for FG11-4 are reported (if the UE also report FG11-4) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-8a | Two HARQ-ACK codebooks with two sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 with different SCS and/or different carrier type | 1. Supports two subslot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured in USS per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and "codeBlockGroupTransmission" for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. - Candidate values for the component 6 of this FG is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration. | 11-3, 66-8 | Yes |  |  | Per FS | N/A | N/A | N/A | The number of PUCCHs for CSI reporting per slot is not impacted compared with Rel-15 by introducing the new HARQ-ACK CBs Component 6 is applied to the two sub-slot HARQ-ACK codebooks, respectively. Component 6 is reported for 2-symbol*7 sub-slot configuration. For 7-symbol*2 sub-slot configuration, the value of component 6 is {2} for both NCP and ECP cases. For component 6, same values as for FG11-4a are reported (if the UE also report FG11-4a) | Optional with capability signaling |
| 66-8b | DL priority indication in DCI with mixed DCI formats including DCI format 1_3 with different SCS and/or different carrier type | Support of priority indicator field configured in DCI formats 1_3 and (1_1 or 1_2) in a BWP when configured to monitor both DCI formats 1_3 and (1_1 or 1_2) in the BWP | 66-8 |  |  |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Proposal: (similar to Rel-18 49-7/7a)

Introduce the following FG 66-9/9a:

| 66-9 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 1_3/0_3 with different SCS and/or different carrier type | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) for DCI format 1_3/0_3 1) Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 0_3 2) Multiplexing/prioritization between UL channels/signals with the same PHY priority level 3) Prioritization between UL channels/signals with different PHY priority levels 4) Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. 5) Additional number of symbols (d2) of the preparation time needed for the high priority UL transmission that cancels a low priority UL transmission | At least one of {66-1, 66-2} |  |  |  | Per FS | N/A | N/A | N/A | Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} For component 4 and 5, same values as for FG12-1 are reported (if the UE also report FG12-1) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-9a | UL priority indication in DCI with mixed DCI formats including DCI format 0_3 with different SCS and/or different carrier type | Support of priority indicator field configured in DCI formats 0_3 and (0_1 or 0_2) in a BWP when configured to monitor both DCI formats 0_3 and (0_1 or 0_2) in the BWP | 66-9 |  |  |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Proposal: (similar to Rel-18 49-8/9/10/11)

Introduce the following FG 66-10/11/12/13:

| 66-10 | Triggered HARQ-ACK codebook re-transmission for DCI format 1_3 with different SCS and/or different carrier type | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset | 66-1 |  |  |  | Per band | N/A | N/A | N/A | Candidate values for component 3 is: M = {-7, -5, …, 1} Candidate values for component 4 is: N= {4, 6, …, 24} Note: The minimum requirement for Component 3 and Component 4 of this FG is valid for HARQ CBs consisted of HARQ Processes with a single HARQ bit per HARQ Process ID For component 3 and 4, same values as for FG25-7 are reported (if the UE also report FG25-7) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-11 | SCell dormancy indication within active time in DCI format 0_3/1_3 with different SCS and/or different carrier type | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_3/1_3 | 6-5, at least one of {66-1, 66-2} |  |  |  | Per BC | N/A | N/A | N/A | One dormant BWP and one non-dormant BWP is supported per carrier More than one non-dormant BWP per carrier is supported only if UE feature 6-3/6-4 is also supported One dormant BWP and one non-dormant BWP are UE specific BWPs even for UEs not supporting 6-2 or 6-3 | Optional with capability signaling |
| 66-12 | Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3/1_3 with different SCS and/or different carrier type | 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 and 1_3 2) minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset 3) minimumSchedulingOffset K2 configuration for PUSCH 4) Support of extended value range for aperiodic CSI-RS triggering offset | At least one of {66-1, 66-2} |  |  |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 66-13 | PHY priority indication for one-shot HARQ-ACK feedback triggered by DCI format 1_3 with different SCS and/or different carrier type | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI format 1_3 | 66-7a and 66-8 |  |  |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Proposal: (similar to Rel-18 49-12/12a)

Introduce the following FG 66-14/14a:

| 66-14 | Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra-cell and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC with different SCS and/or different carrier type | 1: TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2: The minimum beam application time in Y symbols per SCS 3: The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1, 66-1 |  |  |  | Per band | N/A | N/A | N/A | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2 Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Note: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in 23-1-1, component 5 Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH Note: For component 2 and 3, same values as for FG23-1-1b are reported (if the UE also report FG23-1-1b) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-14a | Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC with different SCS and/or different carrier type | 1. TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 23-10-1, 66-1 |  |  |  | Per band | N/A | N/A | N/A | If a UE supports FG 23-10-1m, the signalled component values also apply to inter-cell beam management Candidate values of component 2: {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336} Candidate values of component 3: {2, …, 8} Candidate values of component 4: {2, …, 8} Note: For component 2, 3 and 4, same values as for FG23-10-1b are reported (if the UE also report FG23-10-1b) | Optional with capability signaling |

Proposal: (similar to Rel-18 49-13/14)

Introduce the following FG 66-15/16:

| 66-15 | Default QCL assumption for multi-cell scheduling by DCI format 1_3 with different SCS and/or different carrier type | Indicates whether the UE can be configured with enabledDefaultBeamFormultiCellScheduling for default QCL assumption for multi-cell scheduling by DCI format 1_3 for same/different numerologies - Candidate values are {different only, both} - When "both" is reported, the UE supports this feature for same SCS and for different SCS combination(s) (low-to-high, high-to-low or both) reported for 49-1b | 66-1 |  |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-16 | Support of BWP switch indication by DCI format 0_3/1_3 with different SCS and/or different carrier type | Support of BWP switch indication by DCI format 0_3/1_3 | At least one of {66-1, 66-2} for the BC At least one of {6-2, 6-3, 6-4} for at least one band of the BC |  |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |

NR_LBCA_Sw

Proposed Conclusion:

Conclude the following:

- No RAN1 FG is defined for Rel-19 NR low-band CA.

R1-2508354	UE features Batch B (SBFD and MCE)	Nokia

R1-2508416	Remaining issues of UE features for NR_duplex_evo and NR_MC_enh2	vivo

R1-2508483	Remaining issues for Rel-19 WIs in UE features Batch B	Huawei, HiSilicon

R1-2508509	UE features Batch B	Ericsson

R1-2508577	UE features for SBFD	CATT

R1-2508668	UE features for Rel-19 SBFD operation, multi-carrier enhancements and low band CA	Xiaomi

R1-2508700	UE features for Batch B	OPPO

R1-2508785	UE features for SBFD and MCE Phase 3	Samsung

R1-2508786	UE features for MCE Phase 3	Samsung

(Withdrawn)

R1-2508816	Discussion on UE feature Batch B	ZTE Corporation, Sanechips

R1-2508924	UE features for Msg.3 repetition in SBFD operation	Fujitsu

R1-2509032	Discussion on UE features for evolution of NR duplex operation: SBFD	Ofinno

R1-2509211	UE features Batch B	Qualcomm Incorporated

R1-2509264	Discussion on UE features Batch B	NTT DOCOMO, INC.