3GPP TSG RAN WG1 #123			R1-2509443

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.5

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 8.5

Document for:	Discussion, Decision

## Maintenance on Enhancements of network energy savings for NR

Note: Maximum one contribution per company/organization/university. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- On-demand SSB SCell operation

- On-demand SIB1 for idle/inactive mode UEs

- Adaptation of common signal/channel transmissions

[123-R19-NES] Email discussion on NES– Ajit (Ericsson)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509443	Session Notes of AI 8.5	Ad-Hoc Chair (Ericsson)

R1-2508408	Maintenance on enhancements of network energy savings for NR	vivo

R1-2508493	Maintenance on Rel-19 Network Energy Savings	Huawei, HiSilicon

R1-2508522	Maintenance on Enhancements of network energy savings	Nokia, Nokia Shanghai Bell

R1-2508573	Maintenance on enhancements of network energy savings for NR	CATT

R1-2508701	Maintenance on enhancements of network energy savings for NR	OPPO

R1-2508779	Maintenance on Enhancements of network energy savings for NR	Samsung

R1-2508814	Discussion on remaining issues of Rel-19 NES	ZTE Corporation, Sanechips

R1-2508894	Remaining issues on enhancements of NES for NR	LG Electronics

R1-2508941	Maintenance for Network Energy Saving	Google

R1-2509025	Maintenance of Enhancements of NES	Ofinno

R1-2509082	Remaining issues on Rel-19 NES	Apple

R1-2509156	Maintenance on enhancements of network energy savings for NR	MediaTek Inc.

R1-2509172	Maintenance on enhancements of network energy savings for NR	Sharp

R1-2509203	Maintenance on network energy savings for NR	Qualcomm Incorporated

R1-2509260	Maintenance on enhancements of network energy savings for NR	NTT DOCOMO, INC.

R1-2509311	Remaining issues on OD-SSB	ASUSTeK

R1-2509416	Maintenance for Rel-19 NES	Ericsson

R1-2509168	List of RAN1 agreements for Rel-19 NES WI	Rapporteur(Ericsson)

### On-demand SSB SCell operation

R1-2508895	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2508896	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

#### LS on OD-SSB	R1-2508305 (RAN2, Apple)

### On-demand SIB1 for idle/inactive mode UEs

R1-2509472	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2509473	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2509474	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2509475	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

### Adaptation of common signal/channel transmissions

R1-2509165	Summary#1 of R19 maintenance for adaptation of common signalling	Moderator(Ericsson)

R1-2509166	Summary#2 of R19 maintenance for adaptation of common signalling	Moderator(Ericsson)

R1-2509167	Final summary of R19 maintenance for adaptation of common signalling Moderator(Ericsson)