3GPP TSG RAN WG1 #123			R1-2509445

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.2

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 10.2

Document for:	Discussion, Decision

## NR MIMO Phase 6

Please refer to RP-252936 for detailed scope of the WI.

[123-R20-MIMO] Email discussion on Rel-20 MIMO – Darcy (MTK)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509445	Session Notes of AI 10.2	Ad-Hoc Chair (Ericsson)

### Improvement of SRS capacity and coverage

Including a) Multiple frequency-domain starting positions for SRS repetition, and b) Cross-slot SRS between one U slot and one adjacent S slot.

R1-2508347	NR MIMO Phase 6: SRS Enhancement	InterDigital, Inc.

R1-2508368	Improvement of SRS capacity and coverage	MediaTek Inc.

R1-2508371	Improvement SRS capacity and coverage	TCL

R1-2508377	Discussion on improvement of SRS capacity and coverage	Spreadtrum, UNISOC

R1-2508421	Discussion on improvement of SRS capacity and coverage	vivo

R1-2508491	Improvement of SRS capacity and coverage	Huawei, HiSilicon

R1-2508525	Discussion on improvement of SRS capacity and coverage	ZTE Corporation, Sanechips

R1-2508553	Discussion on improvement of SRS capacity and coverage	NEC

R1-2508586	Improvement of SRS capacity and coverage	CATT

R1-2508603	Discussion on SRS capacity and coverage improvement	China Telecom

R1-2508673	Discussion on the improvement of SRS capacity and coverage	Xiaomi

R1-2508716	Discussion on enhancement of SRS capacity and coverage for MIMO phase 6	OPPO

R1-2508756	Improvement of SRS capacity and coverage	Tejas Network Limited

R1-2508791	Views on improvement of SRS capacity and coverage	Samsung

R1-2508919	Discussion on improvement of SRS capacity and coverage	Panasonic

R1-2508928	Discussion on improvement of SRS capacity and coverage	Fujitsu

R1-2508956	Improvement of SRS capacity and coverage	Lenovo

R1-2508965	Discussion on improvement of SRS capacity and coverage for NR MIMO Phase 6	ETRI

R1-2508988	Discussion on improvement of SRS capacity and coverage	HONOR

R1-2509011	Improvement of SRS Capacity and Coverage	Nokia

R1-2509014	On Rel-20 improvement of SRS capacity and coverage	Ericsson

R1-2509068	Discussion on improvements of SRS capacity and coverage	Sony

R1-2509099	On Rel-20 MIMO SRS capacity and coverage improvement	Apple

R1-2509220	SRS enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2509250	Improvement of SRS capacity and coverage	Transsion Holdings

R1-2509269	Discussion on Improvement of SRS capacity and coverage	NTT DOCOMO, INC.

R1-2509297	Views on enhancements for Improvement of SRS capacity and coverage	KDDI Corporation

R1-2509320	Improvement of SRS capacity and coverage	Sharp

R1-2509358	Discussion on improvement of SRS capacity and coverage	Google

R1-2509362	Discussion on Improvement of SRS Capacity and Coverage	Rakuten Mobile, Inc

R1-2509393	Discussion on improvement of SRS capacity and coverage	NICT

R1-2508829	Moderator summary on improvement of SRS capacity and coverage: Round1	Moderator (CATT)

R1-2508830	Moderator summary on improvement of SRS capacity and coverage: Round2	Moderator (CATT)

R1-2508831	Moderator summary on improvement of SRS capacity and coverage: Round3	Moderator (CATT)

R1-2508832	Moderator summary on improvement of SRS capacity and coverage: Round4	Moderator (CATT)

#### Multiple FD starting positions for SRS repetition symbols within each SRS FH for RPFS

Conclusion:

There is no consensus to support the intra-repetition hopping for SRS repetition symbols within each SRS frequency hop when R>K.

The Working assumption is not confirmed.

#### Cross-slot SRS between one U slot and one adjacent S slot

### Enhancing DL CSI acquisition

Including a) Early SRS/CSI/CSI-RS triggering, and b) CSI-RS density reduction for 48, 64, and 128 CSI-RS ports.

R1-2508328	Enhancing DL CSI acquisition	FUTUREWEI

R1-2508342	On DL CSI Acquisition Enhancements for FR1	Nokia

R1-2508348	NR MIMO Phase 6: DL CSI Enhancement	InterDigital, Inc.

R1-2508363	Enhanced DL CSI acquisition for MIMO Phase 6	Ericsson

R1-2508369	Enhancing DL CSI acquisition	MediaTek Inc.

R1-2508370	Discussion on enhancing DL CSI acquisition	TCL

R1-2508378	Discussion on enhancing DL CSI acquisition	Spreadtrum, UNISOC

R1-2508422	Discussion on enhancing DL CSI acquisition	vivo

R1-2508492	DL CSI acquisition enhancement	Huawei, HiSilicon

R1-2508510	Discussion on enhancing DL CSI acquisition 	Quectel

R1-2508526	Discussion on enhancing DL CSI acquisition	ZTE Corporation, Sanechips

R1-2508548	Discussion on Enhancing DL CSI acquisition	NEC

R1-2508587	Enhancing DL CSI acquisition	CATT

R1-2508636	DL CSI Enhancements for NR Rel-20	AT&T

R1-2508674	Discussion on enhancing DL CSI acquisition	Xiaomi

R1-2508717	Discussions on Enhancing DL CSI Acquisition	OPPO

R1-2508752	Discussion on enhancing DL CSI acquisition	LG Electronics

R1-2508757	Enhancing DL CSI acquisition	Tejas Network Limited

R1-2508792	Views on enhancing DL CSI acquisition	Samsung

R1-2508840	Discussions on enhancing DL CSI acquisition	China Telecom

R1-2508882	Discussion on early DL CSI acquisition design	Fainity Innovation

R1-2508929	Discussion on enhancing DL CSI acquisition	Fujitsu

R1-2508966	Discussion on enhancing DL CSI acquisition for NR MIMO Phase 6	ETRI

R1-2508989	Discussion on enhancing DL CSI acquisition	HONOR

R1-2509033	Discussion on enhancing DL CSI acquisition	Ofinno

R1-2509048	DL CSI acquisition enhancements for Rel. 20 MIMO	Fraunhofer IIS, Fraunhofer HHI

R1-2509069	Discussion on DL CSI acquisition enhancements	Sony

R1-2509100	On Rel-20 MIMO CSI enhancement	Apple

R1-2509164	Discussion on Early DL CSI Acquisition Enhancements	Panasonic

R1-2509221	DL CSI acquisition enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2509270	Discussion on Enhancing DL CSI acquisition	NTT DOCOMO, INC.

R1-2509307	Discussion on DL CSI acquisition	ITRI, Acer Incorporated

R1-2509321	Enhancing DL CSI acquisition	Sharp

R1-2509325	Discussion on Enhancing DL CSI acquisition	IIT Kanpur

R1-2509342	Views on DL Channel acquisition enhancements	CEWiT

R1-2509359	Discussion on enhancing DL CSI acquisition	Google

R1-2509363	Discussion on Enhancement of CSI DL Acquisition	Rakuten Mobile, Inc

R1-2509394	Discussion on enhancing DL CSI acquisition	NICT

R1-2508517	Moderator summary on enhancing DL CSI acquisition (Round 0)	MediaTek Inc.

R1-2508518	Moderator summary on enhancing DL CSI acquisition (Round 1)	MediaTek Inc.

R1-2508519	Moderator summary on enhancing DL CSI acquisition (Round 2)	MediaTek Inc.

R1-2508520	Moderator summary on enhancing DL CSI acquisition (Round 3)	MediaTek Inc.

#### Early SRS/CSI/CSI-RS triggering

Conclusion

For UE transition from IDLE/INACTIVE to CONNECTED mode, there is no RAN1 consensus to support the following cases triggered by MSG4 of 4-Step RACH:

- Periodic and semi-persistent SRS-AS

- Periodic and semi-persistent CSI-RS for CSI associated with aperiodic CSI reporting

Conclusion

For SCell transition from deactivation to activation, there is no RAN1 consensus to support the following cases triggered based on legacy SCell activation command:

- Periodic and semi-persistent SRS

- Periodic and semi-persistent CSI-RS for CSI associated with aperiodic CSI reporting

#### CSI-RS frequency-domain density reduction

Conclusion

No enhancement to the following legacy specification restriction on frequency density of CSI-RS port in a CSI subband for the frequency-domain densities ρ = 1/4 and 1/8, i.e., the following from Clause 5.2.1.4 in TS 38.214 shall be applied:

- A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource.

FFS: Whether to apply the legacy specification restriction to the frequency-domain densities ρ = 1/3 and 1/6

Agreement

Support frequency-domain density ρ = 1/8 for K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation with the following configurations, for wideband CSI reporting based on Rel-19 Type-1 codebooks:

- K=4 16-port NZP CSI-RS resources in a resource set aggregating 64 CSI-RS ports

- K=4 32-port NZP CSI-RS resources in a resource set aggregating 128 CSI-RS ports

- Working assumption:

  - K=2 24-port NZP CSI-RS resources in a resource set aggregating 48 CSI-RS ports

- FFS: For subband CSI reporting and Type-2 codebook

Support of frequency-domain density ρ = 1/8 is a separate UE capability.

Note: Different frequency-domain densities configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation are not precluded