3GPP TSG-RAN WG1 #123	<No TDoc Number>

Dallas, USA, November 17 – 21, 2025

Source:	Moderator (Nokia)

Title:	Feature Lead summary #1 on 6G waveform

Document for:	Discussion

Agenda item:	11.3.1

Work Item:	FS_6G_Radio

# 1.	Introduction

44 Tdocs were submitted (including one moved from AI 11.1 and one submitted to AI 11.3) with a total of 567 pages, of which consist of 287 observations and 285 proposals spanning 41 pages. This contribution acts as a platform to facilitate discussions on various topics raised in these conributions.

Schedule for waveform sessions:

- Monday 14:30-15:30 offline

- Monday 17:00-17:50 online

- Tuesday 11:00-12:00 offline

- Tuesday 12:10-13:00 online

- Tuesday 18:40-19:30 offline

- Wednesday 9:10-9:50 online

- Wednesday 17:00-18:30 offline

All proposals:

# 2.	References

|  | Tdoc# | Title | Source |
| --- | --- | --- | --- |
| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| [2] | R1-2508367 | Considerations for additional 6GR DL waveform | Kyocera Corporation |
| [3] | R1-2508387 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
| [7] | R1-2508471 | Discussion on waveform for 6GR air interface | THALES |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
| [9] | R1-2508625 | Waveform for 6GR Air Interface | Shanghai Jiao Tong University. |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
| [13] | R1-2508684 | Discusson on 6GR Waveform | Xiaomi |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
| [17] | R1-2508833 | Waveform for 6G Radio Air Interface | IMU |
| [18] | R1-2508844 | Waveform for 6GR air interface | BJTU |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
| [21] | R1-2508863 | Support for Diverse Waveforms on the 6GR Air Interface | National Spectrum Consortium |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
| [23] | R1-2508890 | Waveform considerations for 6G Uplink | Wisig, IITH |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
| [26] | R1-2508973 | Discussion on 6GR waveform | ETRI, University of Surrey |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
| [28] | R1-2509049 | Discussion on waveform for 6GR air interface | Ruijie Networks Co. Ltd |
| [29] | R1-2509059 | Waveform for 6G NR | Cohere Technologies, IIT Delhi |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
| [33] | R1-2509143 | Waveform for 6GR air interface | MediaTek Inc. |
| [34] | R1-2509231 | Waveforms for 6GR | Qualcomm Incorporated |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
| [36] | R1-2509282 | Discussion on Waveform | NTT DOCOMO, INC. |
| [37] | R1-2509303 | Discussion on DL DFT-s-OFDM for 6GR | LG Electronics, CATT, Thales, Lenovo, IITH, WiSig |
| [38] | R1-2509322 | Study on waveform for 6GR | Sharp |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
| [40] | R1-2509368 | Discussion on Waveforms of 6GR Air Interface | Rakuten Mobile, Inc |
| [41] | R1-2509372 R1-2509461 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
| [43] | R1-2509410 | Waveform design for 6GR air interface | Tejas Network Limited |
| [44] | R1-2509413 | OSDM for 6GR | University of Sheffield |

# 3.	DL PAPR reduction

## 3.1	DL PAPR evaluation methodology

| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
| --- | --- | --- | --- |
|  | - Proposal 7: For downlink low-PAPR proposals the primary evaluation criterion is - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 8: For downlink low-PAPR proposals the primary evaluation criterion may use the following criterion: - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal. - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 3: For studying the additional waveform for 6GR, evaluate waveform proposals using agreed 6GR waveform (i.e., CP-OFDM for DL and CP-OFDM/DFT-s-OFDM for UL) as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. Proposal 6: For downlink low-PAPR proposals the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 7: For DL DFT-s-OFDM on-demand synch. signal/LP-WUS, the performance evaluation criterion from waveform perspective is net gain - Net Gain [dB] = Tx power gain relative to the reference – Required SNR degradation relative to the reference - For on-demand synch. signal and sequence based LP-WUS, the required SNR is for detection rate below 1% and false alarm rate below [1%] assuming same resource overhead - The requirements of RSRP accuracy based on on-demand synch. signal should be met, e.g. as in TS 38.133 Proposal 8: Take Table 9 as a start point for DL DFT-s-OFDM waveform evaluation for on-demand synch. signal/LP-WUS. Table 9 On-demand synch. signal/LP-WUS waveform evaluation assumptions Parameter; On-demand synch. signal; LP-WUS. Carrier frequency and scenario; 4 GHz or 7GHz. SCS; 30KHz. Channel Model; TDL-C 300ns, optional CDL-C 300ns. UE Speed; 3km/h, 120km/h. BS antenna configurations; TDL channel: 4 CDL channel: 64, 128, 256 for 7GHz; 32, 64 for 4GHz. UE antenna configurations; TDL Channel: 2 or 4 CDL channel: up to 16 for 7GHz; up to 4 for 4GHz;. PA model; UE PA model for LPR. OFDM Symbol #; 2; 4. Payload size; 3∙336 IDs; 5 bits as a start point. Proposal 9: Study DL DFT-s-OFDM for on-demand synch. signal/DL-WUS for coverage enhancement, network energy saving and UE energy saving under related agendas, e.g., initial access, network energy saving, UE energy saving agenda. |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 3: For downlink low-PAPR proposals, the Net Gain can be used for evaluation with following updates: - Net Gain [dB] = PAPR gain relative to the reference – SNR degradation relative to the reference. - Note:For data and control channel, the SNR is associated with 10% BLER. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 1: Study adopting Downlink DFT-s-OFDM for low data rate MBB in the terrestrial network as part of network energy savings and coverage. - Target channel(s): Two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH). - Evaluation methodology: - LLS to start with focusing on key metrics such as: Net Gain and NES - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @PHY channel target BLER - Realistic PA model for gNB - NES gain (%) – baseline against corresponding CP-OFDM - Further discuss details on how to do without SLS |  |  |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 2: Transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Complementary metrics—such as SNR degradation, effective occupied bandwidth, and PA efficiency can also be analysed to provide a comprehensive assessment of waveform linearity and spectral behaviour. Proposal 4: Downlink transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Evaluation is based on the Net Gain metric (Tx power gain – link loss at 10% BLER) using a realistic base station PA model, complemented by secondary metrics such as PA efficiency, ACLR, EVM, network energy savings, and occupied bandwidth. |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 4: Use the Net Gain (dB) = Tx power gain – Link Loss to the reference @10% BLER as a primary downlink waveform evaluation metric |  |  |
| [37] | R1-2509303 | Discussion on DL DFT-s-OFDM for 6GR | LG Electronics, CATT, Thales, Lenovo, IITH, WiSig |
|  | Proposal 1: For downlink low-PAPR proposals the primary evaluation criterion is one of - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - Network energy saving gain relative to baseline for BS - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding |  |  |

## 3.2	DL DFT-s-OFDM

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | Proposal 3: RAN1 to deprioritize DFT-s-OFDM study for DL for the following reasons: - PAPR of DFT-s-OFDM in DL is comparable to DL CP-OFDM with transparent PAPR reduction techniques. - No meaningful pure DFT-s-OFDM NES gain or DL coverage gain in current FRs of interest compared to DL CP-OFDM especially where DFT-s-OFDM system limitations may be tolerated (e.g., low load, etc.). - DL EIRP could be already at the maximum with CP-OFDM in these DFT-s-OFDM potential scenarios, and no DL coverage extension can be foreseen. - DL DFT-s-OFDM may rely more on time domain multiplexing reducing time domain NES gain potential. - Base station and UE baseband complexity with DL DFT-s-OFDM is higher (e.g., DFT, Rx processing, transparent PAPR techniques need to be maintained). - DFT-s-OFDM can have link performance degradation compared to CP-OFDM in different conditions in DL (e.g., low complexity UE Rx, impractical R-ML per RE with DFT-s-OFDM, limited number of UE Rx antennas, etc.), - DFT-s-OFDM consideration in DL may need to impose limitations for minimizing PAPR impact with DFT-s-OFDM to the following: - UE frequency domain resource allocation (e.g., contiguous) - UE frequency domain multiplexing - Multiplexing of different DL physical channels/signals and efficient spectrum use (e.g., no FDM of physical channels using CP-OFDM with channel/signal using DFT-s-OFDM, or no/limited number of FDMed channels using DFT-s-OFDM) - SU/MU-MIMO precoding (e.g., limited number of layers for all UEs per port, wideband/subband precoding) - Efficient multi-RAT/MRSS support in FR1 (e.g., avoid frequency multiplexing of 5G DL CP-OFDM with some 6G DL in same RU) - Efficient RU use and carrier configuration flexibility (e.g., number and possible active carriers, potential of non-contiguous intra-band, inter-band spectrum aggregation, fragmented DL support per RU, restriction for all carriers in RU BS RF BW to use same DL waveform at least in overlapping DL transmission period) |  |  |
| [3] | R1-2508387 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
|  | Proposal 1：Support DL DFT-s-OFDM in 6GR for the following use cases and channel: - Target use case: LPWA, NTN - Target channel: PDSCH only |  |  |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
|  | - Proposal 5: RAN1 captures followings as a conclusion for DL DFT-s-OFDM - MRSS compatibility - DL DFT-s-OFDM can be TDMed and FDMed with NR DL CP-OFDM as if UL DFT-s-OFDM can coexist with UL CP-OFDM in NR. - Target modulations, and impact to other modulation, if applicable - DL DFT-s-OFDM can be independently operated with any modulation. - MIMO (SU and MU-MIMO) compatibility - In NTN scenario, since LOS paths are predominantly employed, the spatial multiplexing gain to be exploited for MIMO are limited. - In IoT scenario, considering the target data rate and UE complexity, the MIMO operation is typically not used. - Multi-user multiplexing/scheduling flexibility - DL DFT-s-OFDM can support TDM and FDM for multi-user multiplexing. - If DFT transform precoding is separately applied to each FDMed DL channel/signal, the PAPR can be reduced compared to the reference. - If DFT transform precoding is jointly applied to multiple FDMed DL channels/signals, the significant PAPR reduction can be achieved at the expense of the increased UE complexity. - Multiplexing/coexistence with baseline waveform - DL channel(s)/signal(s) with DL DFT-s-OFDM can be TDMed and FDMed with DL channel(s)/signal(s) with DL CP-OFDM. Even in this case, non-negligible PAPR reduction can be achieved. - Transmitter/receiver complexity and impact power consumption - Transmitter complexity for DL DFT-s-OFDM is marginal since the DFT transform precoding can be implemented by IDFT for receiving UL DFT-s-OFDM and the conjugate function. - Receiver for DL DFT-s-OFDM may need the equalization process while the channel estimation complexity would be similar compared to the reference. - Proposal 6: DFT-s-OFDM waveform is supported as the additional basis for 6GR in downlink. - DFT transform precoding for DL is available at least for a single UE-dedicated PDSCH. - FFS: Whether or how the DFT transform precoding is applied to the common DL channels (e.g., PDSCH containing common signaling, SS/PBCH). - FFS: Whether or how the DFT transform precoding is applied to multiple PDSCHs for multiple UEs. - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE - FFS: Whether or how to support multiplexing with DMRS - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | Vivo |
|  | - Proposal 9 Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. |  |  |
| [7] | R1-2508471 | Discussion on waveform for 6GR air interface | THALES |
|  | Proposal 2: Identify the set of NTN scenarios/use cases for which is beneficial to use DFT-s-OFDM in DL. Proposal 3: RAN1 to study the performance of DFT-s-OFDM in the downlink for non-terrestrial network (NTN)-based 6G radio access. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 7: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA. Proposal 9: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement. Proposal 10: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied. |  |  |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
|  | Proposal 6: Study the support of low PAPR waveforms like DFT-s-OFDM for 6G downlink transmissions. Proposal 7: Study a simplified framework for DL waveform support, where a default waveform is used for initial access, and UE-specific configuration for DFT-s-OFDM is performed semi-statically via RRC signaling. Proposal 8: Study the waveform design for PDCCH in deployments supporting DL DFT-s-OFDM, evaluating two approaches: - The use of CP-OFDM for PDCCH to ensure implementation simplicity and compatibility. - The feasibility of using DFT-s-OFDM for PDCCH to improve performance, including a detailed analysis of the required structural redesign, challenges in supporting multiple users, and the overall system impact. Proposal 9: Study multi-user scheduling techniques for downlink DFT-s-OFDM, including group-based or sub-band DFT, to balance multi-user throughput with low-PAPR properties. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms |  |  |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
|  | Proposal 1: RAN1 to deprioritize the study of DFT-s-OFDM for downlink due to lack of any significant advantage compared to CP-OFDM even for the potential use cases such as NES, NTN ISAC, and FR3, and for the transmission of common channels/signals as well as to keep the 6GR system design to a reasonable complexity. |  |  |
| [13] | R1-2508684 | Discusson on 6GR Waveform | Xiaomi |
|  | Proposal 1: To support the coverage performance for NTN DL, low-PAPR waveform such as DFT-S-OFDM can be considered. Proposal 2: To support the coverage performance for NTN DL, low-PAPR waveform can be applied to the following channels: - PDCCH at least for CSS (except for type-3) - PDSCH with Msg 4 - PDSCH with SIB1 |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 7: DFT-s-OFDM is not supported as additional DL baseline waveform for 6GR, due to limited performance gain, restriction on multiuser scheduling and extra complexity on UE side. - DL DFT-s-OFDM for NTN can be further studied in NTN agenda. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 9: Study DL DFT-s-OFDM for on-demand synch. signal/DL-WUS for coverage enhancement, network energy saving and UE energy saving under related agendas, e.g., initial access, network energy saving, UE energy saving agenda. |  |  |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
|  | Proposal 4: Discontinue the study for the potential support of DFT-s-OFDM waveform for 6GR downlink - No evidence of any potential benefit in DL coverage over CP-OFDM in all relevant use cases such as NTN, IoT, FR3, energy efficiency, common signals/channels. In fact, in most cases, DL DFT-s-OFDM would likely result in spectral efficiency loss, increased energy consumption at base stations and UEs, and substantial specification/testing efforts . |  |  |
| [17] | R1-2508833 | Waveform for 6G Radio Air Interface | IMU |
|  | Proposal 7: CP-OFDM remain the sole downlink waveform for 6G radio to ensure simplicity, backward compatibility, and unified evolution of the physical layer. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 1: Study adopting Downlink DFT-s-OFDM for low data rate MBB in the terrestrial network as part of network energy savings and coverage. - Target channel(s): Two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH). - Evaluation methodology: - LLS to start with focusing on key metrics such as: Net Gain and NES - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @PHY channel target BLER - Realistic PA model for gNB - NES gain (%) – baseline against corresponding CP-OFDM - Further discuss details on how to do without SLS Proposal 2: Target Downlink Channels to evaluate DFT-s-OFDM can be narrowed down to two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH). Proposal 3: Downlink DFT transform precoding can be applied in following two ways: - Per UE DFT: DFT transform precoding can be applied to each downlink channel of a UE in a time slot where each DFT size is same as the number of occupied subcarriers of the downlink signal of a UE - Sub-band DFT: A single DFT transform precoding can be applied to downlink channel of a group of UEs where DFT size can be larger than the number of assigned subcarriers of each UE. |  |  |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 9: DL DFT-s-OFDM for individual signal / channel (such as LP-WUS / LP-WUR signal) are not required to be concluded for now. The important point it whether to support DFT spreading to overall channel’s method. Proposal 10: To support DFT spreading to overall DL channel’s method is excluded. |  |  |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission. Proposal 3: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM. |  |  |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
|  | Proposal 1: Support DFT-s-OFDM waveform as the DL waveform with regard to the following aspects: - To support the same coverage for FR1 and FR3 - To provide a good coverage for NTN - Compared to other coverage enhancement techniques, e.g., to increase the number of antennas or to transmit the DL signals by multiple repetitions, using DFT-s-OFDM waveform does not require additional complexity for complicated CSI calculation, does not require large delay for beam measurement or multi-repetitions-based DL signal reception, and does not require large overhead for complicated CSI report and more DL-RSs for beam measurement. |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 3: Study downlink waveform enhancements including the use of DFT-s-OFDM with FDSS and SLM. |  |  |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
|  | Proposal 4: RAN1 should study multiplexing of CP-OFDM reference signals and other signals on the same component carrier with DFT-s-OFDM physical channels. Proposal 5: RAN1 should study multi-layer transmission with DFT-s-OFDM for both UL and DL. |  |  |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
|  | Proposal 7: Do not support DFT-S-OFDM in the DL |  |  |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
|  | Proposal 10: Consider DFT-s-OFDM as potential additional waveform for downlink. Proposal 11: Consider at least /2-BPSK and QPSK as modulation schemes for DFT-s-OFDM waveform in downlink. Proposal 12: Target channels/signals for DFT-s-OFDM can be unicast PDSCH, UE specific PDCCH, and relevant reference signals (e.g., DMRS, CSI-RS). |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | - Proposal 8: It is recommended to adopt DFT-s-OFDM as a complementary waveform to CP-OFDM in the 6G downlink. |  |  |
| [37] | R1-2509303 | Discussion on DL DFT-s-OFDM for 6GR | LG Electronics, CATT, Thales, Lenovo, IITH, WiSig |
|  | Proposal 2: For the study of the DFT-s-OFDM as the potential additional waveform for DL, RAN1 consider followings: - Potential Target channels/signals - PDCCH, e.g., CSS PDCCH - PDSCH, e.g., unicast PDSCH (including Msg4), common PDSCH (including SIB1 PDSCH) - SSB - Wake-up signal Note: Which channels will be evaluated is up to company - How to apply the DFT transform precoding in case of multiplexing UEs or channels/signals into one symbol - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE - Other options are not precluded. - How to multiplex DMRS (for PDCCH and/or PDSCH) - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level - Other options are not precluded. |  |  |
| [38] | R1-2509322 | Study on waveform for 6GR | Sharp |
|  | Proposal 1: For 6GR waveform, RAN1 should NOT study DL DFT-s-OFDM. |  |  |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
|  | Proposal 1: Support usage of DFT-s-OFDM in DL. |  |  |
| [41] | R1-2509372 R1-2509461 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
|  | Proposal 1: 3GPP should consider the use of DFT-s-OFDM in DL/UL at least for NTN and FR2 use-cases. |  |  |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
|  | Proposal 2: For DFT-s-OFDM as 6GR waveform - DFT-s-OFDM for downlink may be studied as an optional mode for specific use cases. - Low-PAPR enhancement techniques (e.g., FDSS, FDSS-SE, frequency-selective mapping) may be studied as optional uplink features within the OFDM-based baseline. |  |  |
| [43] | R1-2509410 | Waveform design for 6GR air interface | Tejas Network Limited |
|  | Proposal 4: DFT-s-OFDM is a power-efficient choice for NTN downlink transmission which offers stronger signal coverage for energy-constrained satellites. |  |  |

## 3.3	DL CP-OFDM PAPR reduction

| [2] | R1-2508367 | Considerations for additional 6GR DL waveform | Kyocera Corporation |
| --- | --- | --- | --- |
|  | Proposal 3: Enhancements to CP-OFDM, including PAPR reduction techniques such as ACE, should be evaluated in the 6GR study. |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | Vivo |
|  | - Proposal 9 Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, scheme on reducing the overhead and indicating complexity of side information shall be studied. Proposal 3: It is proposed to study Tone Reservation (TR) for CP-OFDM waveform in 6GR. Proposal 4: If SLM/TR are applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH. Proposal 7: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA. Proposal 9: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement. Proposal 10: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 4: To improve coverage, tone reservation should be considered in 6G waveform design as a low-complexity scheme to achieve Net Gain along with the compatibility with both UL and DL waveforms. Proposal 5: To improve coverage, Selected Mapping(SLM) should be considered in 6G waveform design as a low-complexity scheme to achieve Net Gain along with the compatibility with both UL and DL waveforms. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 4: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 3: Study downlink waveform enhancements including the use of DFT-s-OFDM with FDSS and SLM. |  |  |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
|  | Proposal 1: RAN1 should study PAPR reduction for CP-OFDM that can be applied to the NTN DL. Proposal 4: RAN1 should study multiplexing of CP-OFDM reference signals and other signals on the same component carrier with DFT-s-OFDM physical channels. Proposal 5: RAN1 should study multi-layer transmission with DFT-s-OFDM for both UL and DL. |  |  |

# 4.	UL PAPR reduction

## 4.1	UL PAPR evaluation methodology

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | Proposal 5: Proposed low PAPR methods are compared using net gains and assuming similar spectral efficiency and bandwidth for each compared method and used baseline. Proposal 6: RAN1 to discuss the methodology for comparison of low PAPR methods. |  |  |
| [3] | R1-2508387 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
|  | Proposal 2: Regarding around 7GHz PA model(s), before RAN 4 feedback, the following four options can be considered for RAN1 initial evaluation: MP model, GMP model, GMP model with LASSO and WMP model |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
|  | - Proposal 1: Study waveform adjustments to achieve high UE power efficiency and UL coverage. - Proposal 2: Normalized memoryless PA model defined in RAN4 can be used for MPR evaluation for 6GR. PA model [2] Model; Parameter. yn= k∈Kpakx(n)x(n)2k; ak; k∈[0,1,…,7]: [-0.618347-0.785905i; 2.0831-1.69506i; -14.7229+16.8335i; 61.6423-76.9171i; -145.139+184.765i; 190.61-239.371i; -130.184+158.957i; 36.0047-42.5192i]. - Proposal 7: RAN1 studies the enhancement of OOB emission performance in OFDM systems through phase continuity between OFDM symbols. - Proposal 8: In addition to the current RAN4 requirements for MPR calculation, RAN1 can further consider the net gain assessed with performance comparisons conducted under the condition of consistent OOB emission. |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 2. The study focuses on PAPR reduction for DFT-s-OFDM waveform. Proposal 3. The proposed design of FDSS and tone reservation should clarify the difference and improvement comparing to the corresponding work already been done for NR. Proposal 4. The MCSs used in the link-level evaluations of PAPR reduction techniques should satisfied the required 5th percentile user spectral efficiency in IMT-2030 requirements. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 13: Besides the link level simulation, following aspects should be analyzed: - Implementation complexity in receiver. - Spec impact, e.g., reference signals design, codebook design. |  |  |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
|  | Proposal 4: The evaluation of FDSS enhancements must incorporate realistic PA models and be validated against RF conformance requirements, including ACLR and EVM. |  |  |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
|  | Proposal 2: For uplink low-PAPR proposals, throughput should be evaluated in addition to Net Gain to select and compare the best combination of frequency resources (BW) and spectral efficiency (MCS). Proposal 3: To obtain broader insights with respect to the evaluations of different uplink low-PAPR proposal for 6GR, RAN1 to consider MPR-based system-level simulations in addition to link-level simulations.. |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 8: For uplink low-PAPR waveform proposals the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. - Net gain including MPR can be evaluated by RAN4. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 1: Take Table 1~4 as the start point for low PAPR waveform enhancement evaluations. Table 1 General evaluation assumptions 3GPP Rel-18 NR; 3GPP 6GR. Carrier frequency and scenario; 4GHz (Urban), 28GHz (Urban) 700MHz (Rural),; 4GHz, 7GHz. Channel BW; 100MHz for Urban, 20MHz for Rural,; Up to 400MHz for Urban (7GHz), Up to 200MHz for Urban (4GHz),. SCS; 30 kHz (4GHz), 120 kHz (28GHz) 15 kHz (700 MHz),; 30 kHz (4GHz), 30KHz (7GHz). Channel model; TDL-C 300ns for FR1 Urban (4GHz), TDL-A 30ns for FR2 Urban (28GHz), TDL-D 30ns for Rural; TDL-C 300ns, Optional CDL-C 300ns. UE speed; 3km/h; 3km/h. Number of Tx antennas for TDL channel; 1, Optional: 2; 1, Optional: 2. Number of Rx antennas for TDL channel; 4 for FR1 Urban, 2 or 4 for FR1 Rural,; 4. Number of Tx antennas for CDL channel; NA; 1, or any other value up to 16 for 7GHz, and up to 4 for 4GHz. Number of Rx antennas for CDL channel; NA; 4GHz: 32,64,128 7GHz: 32,64,128, 256, 512. Rank per UE; 1; 1. Number of DMRS symbols; 2; 2. Number of PUSCH data symbols; 12; 12. HARQ configuration; No retransmissions; No retransmissions. Frequency hopping; Disabled; Disabled. Table 2 Single user evaluation assumption for MCS and PRB# No Spectrum Extension; With Spectrum Extension. MCS; #RBs; #RB before extension (A); #RB after extension (B); Spectrum extension factor α=(B-A)/B. NR MCS; B; (1-α)B; B; α∈14,13,38,25,716,12. 30; 20; 30; 1/3. 30; 18; 30; 2/5. 120; 90; 120; 1/4. 120; 80; 120; 1/3. 120; 72; 120; 2/5. 120; 641; 120; 7/161. 120; 60; 120; 1/2. 240/480/960; 180/360/720; 240/480/960; 1/4. 240/480/960; 160/320/640; 240/480/960; 1/3. 240/480/960; 150/300/600; 240/480/960; 3/8. 240/480/960; 144/288/576; 240/480/960; 2/5. 240/480/960; 1281/270/540; 240/480/960; 7/161. 240/480/960; 120/240/480; 240/480/960; 1/2. 32; 24; 32; 1/4. 32; 20; 32; 3/8. 32; 18; 32; 7/16. 32; 16; 32; 1/2. 128; 96; 128; 1/4. 128; 90; 128; 1/31. 128; 80; 128; 3/8. 128; 72; 128; 7/16. 128; 64; 128; 1/2. 256/512/1024; 192/384/768; 256/512/1024; 1/4. 256/512/1024; 1801/3601/7201; 256/512/1024; 1/31. 256/512/1024; 160/320/640; 256/512/1024; 3/8. 256/512/1024; 1501/3001/6001; 256/512/1024; 2/51. 256/512/1024; 144/288/576; 256/512/1024; 7/16. 256/512/1024; 128/256/512; 256/512/1024; 1/2. Note1: RB# is rounding to the nearest effective RB# (Integer powers of 2, 3, 5), where the actual spectral spreading factor is only approximately equal.. Table 3 link level multi user evaluation assumption Parameters; Values. User #; 2, or 3. Per user configuration; With spectrum extension provided in Table 2. Multi-user overlap RBs; Reported by companies. Baseline scheme; No spectrum extension. Per UE MCS; Chosen to align with the spectral efficiency. BS Rx Port Virtualization; Reduce to 1 port, e.g., by DFT vector. Table 4 Tx power evaluation assumption Parameters; Values. PA model; Reported by companies, e.g., PA model in R4-164542 [3]. RF requirements; EVM, IBE, SEM, ACLR [2]. RB locations; Edge, Outer, Inner [2]. |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 1: For uplink low-PAPR proposals, the existing sub-6 GHz PA model from RAN4 (R4-163314) can be reused as baseline. Proposal 2: For uplink low-PAPR proposals, adopt the following method for calculating Tx power gain during evaluation of low-PAPR candidate waveforms: Step 1: Compute average power of each waveform to be used as PA input; Step 2: Obtain PA output according to PA input and PA model; Step 3: Tx power gain is the PA output of low-PAPR waveform minus that of conventional OFDM/DFT-s-OFDM waveform. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 6: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 7: RAN1 to establish a standardized evaluation methodology that integrates PA-model-based simulation and link-level validation using Net Gain as the primary metric, with complementary indicators (MPR, ACLR, EVM, and complexity) where applicable. Proposal 8: RAN1 to study the performance of DFT-s-OFDM and potential enhancement candidates under PA saturation and Doppler conditions representative of NTN and high-mobility links. |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 1: It is recommended that the 6G waveform evaluation framework incorporate: - Net Gain shall serve as the performance evaluation criterion for low-PAPR uplink waveforms, with the requirement that it must be greater than 0. - A consistent set of supporting metrics (e.g. PAPR, BLER, OOBE) for comprehensive characterization. Proposal 2: Waveform design for 6GR should account for Inter-Symbol-and-Carrier Interference (ISCI) in high-mobility scenarios to maintain reliable communication and sensing performance. |  |  |

## 4.2	UL PAPR reduction

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | - Proposal 7: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio. Proposal 8: Transparent filtering approach (receiver does not need to be aware of the used filter parameters) is assumed for FDSS and FDSS-SE in 6G Radio. |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
|  | - Proposal 3: Support to study CFR-SE based low-PAPR waveform for 6GR UL. |  |  |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 2. The study focuses on PAPR reduction for DFT-s-OFDM waveform. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 5: Both symmetric and asymmetric FDSS-SE can be supported in 6GR‌. Proposal 6: It is proposed to study 32-QAM schemes for PAPR reduction for DFT-S-OFDM in 6GR. Proposal 11: For improving spectrum efficiency, multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied. |  |  |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
|  | Proposal 3: Study the use of Frequency Domain Spectrum Shaping (FDSS) for DFT-s-OFDM in the 6GR uplink to enhance coverage and power efficiency. Proposal 5: Study a non-transparent FDSS operation for 6GR, including the signaling of the applied shaping filter, to enable advanced receiver equalization and unlock greater performance benefits. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 2: CP-OFDM and DFT-s-OFDM are the only waveforms for uplink. Study enhancements for PAPR reduction for DFT-s-OFDM. Proposal 9: Study FDSS with spectrum extension as a potential candidate for uplink PAPR reduction for DFT-s-OFDM to support coverage enhancement for 6G |  |  |
| [13] | R1-2508684 | Discusson on 6GR Waveform | Xiaomi |
|  | Proposal 3: The UL coverage enhancement mechanism in Rel-18 including power domain solution and waveform switch related solution can be taken as a starting point for UL coverage performance guarantee for both TN and NTN. |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 9: Prioritize the implementation-based schemes without specification impacts. - Study additional gain from schemes with specification impacts. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 2: Study how to utilize the signal property of frequency redundancy in the spectrum extension based schemes to enhance multi-user experience. For example, scheduled PRBs for multiple UEs are overlapped partially. Proposal 3: Study lower PAPR waveform enhancement providing net gain vs NR DFT-s-OFDM waveform. FFS the exact supported lower PAPR waveform enhancement. Proposal 4: Study spectrum extension/truncation plus FDSS lower PAPR waveform enhancement, e.g., I/Q-offset DFT-s-OFDM, DFT-s-OFDM FDSS-SE. Proposal 5: Study pruning QAM under DFT-s-OFDM type waveform(s) for coverage enhancement. |  |  |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
|  | Proposal 1: Study non-transparent frequency domain spectrum shaping (FDSS) on DFT-s-OFDM to reduce PAPR for 6GR UL coverage enhancement. |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 6: To improve coverage, FDSS can be considered in 6G waveform design. - FDSS w/o SE is considered as the baseline. Proposal 7: I-modulation (I-π/2-BPSK/QPSK/QAM) scheme should be considered to reduce PAPR for DFT-s-OFDM. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 5: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc. |  |  |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 6: RAN1 should assess the need to introduce MPR / PAPR reduction techniques, e.g., FDSS-SE / FDSS-CE targeting coverage enhancement for UL. Proposal 7: The gain is obtained in which RB allocation should also be investigated. |  |  |
| [23] | R1-2508890 | Waveform considerations for 6G Uplink | Wisig, IITH |
|  | Proposal-1. Adopt π/2-BPSK DFT-s-OFDM as the 0-dB MPR reference for 6G uplink. Proposal-2. Carry forward existing DFT-s-OFDM modulation schemes as primary uplink options for 6G NR. |  |  |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 5: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM. Proposal 6: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques. |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 7: RAN1 to establish a standardized evaluation methodology that integrates PA-model-based simulation and link-level validation using Net Gain as the primary metric, with complementary indicators (MPR, ACLR, EVM, and complexity) where applicable. Proposal 8: RAN1 to study the performance of DFT-s-OFDM and potential enhancement candidates under PA saturation and Doppler conditions representative of NTN and high-mobility links. |  |  |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
|  | Proposal 2: RAN1 should study PAPR reduction for DFT-s-OFDM especially when used with higher-order modulation. Proposal 3: RAN1 should study constellation shaping for low PAPR for DFT-s-OFDM with higher-order modulation. Proposal 6: For DFT-s-OFDM spectral extension, RAN1 should consider a mode in which the transmitter may freely decide, for each DFT-s-OFDM symbol if SE should be applied or not. No indication of said decision needs to be indicated to the receiver. |  |  |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
|  | Proposal 1: The following table details the motivations of Low PAPR waveforms for 6G Motivation/Information; Comments. Targeted link direction; UL. Motivation; Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity. MRSS compatibility; Yes. Target channels/signals; PUSCH. MIMO (SU and MU-MIMO) compatibility; N/A. Target modulations; pi/2-BPSK, QPSK. Multi-user multiplexing/scheduling flexibility; same as baseline DFT-S-OFDM. Multiplexing/coexistence with baseline waveforms; Yes. Impact on synchronization and initial access; N/A. Expected specification impact; RAN1 Specification RAN4 Requirements. Receiver Complexity; Transparent Non-transparent. Impact to power consumption; Improved PA efficiency. Proposal 2: To evaluate MPR or power gain, PA model and RF requirement should be revisited to support power boost feature in 6G. Proposal 3: Study UL π/2-BPSK DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters to reach near constant envelope waveform using transparent and non-transparent schemes. Proposal 4: Consider near constant envelope waveform and potential impact to RAN4 for example in terms of amount of power boost and associated RF requirement. Proposal 5: Study Transparent/non-Transparent scheme with the GMSK approximation filter to approximate constant envelope waveform with π/2-BPSK and DFT-S-OFDM for uplink coverage enhancement |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
|  | Proposal 2: Consider frequency domain spectrum shaping (FDSS) as a candidate scheme for PAPR reduction for DFT-s-OFDM with at least /2-BPSK and QPSK. Proposal 3: Consider FDSS with spectrum extension (FDSS-SE) as a candidate scheme for PAPR reduction for DFT-s-OFDM with at least /2-BPSK and QPSK. Proposal 4: Consider both FDSS and FDSS-SE for at least data channel (e.g., PUSCH). |  |  |
| [33] | R1-2509143 | Waveform for 6GR air interface | MediaTek Inc. |
|  | Proposal 1: As O-QPSK modulated DFT-s-OFDM offers a significant net gain of 2dB with respect to π/2-BPSK modulated DFT-s-OFDM, support O-QPSK modulated DFT-s-OFDM for 6G UL coverage enhancement. Proposal 2: Support O-QPSK modulated DFT-s-OFDM with subcarrier truncation for coverage enhancement at spectral efficiency larger than 1 bit/s/Hz. |  |  |
| [34] | R1-2509231 | Waveforms for 6GR | Qualcomm Incorporated |
|  | Proposal 1: For 6G Radio waveform study, limit initial focus to waveform design for communication use cases. Waveforms and specific waveform enhancements for other use cases such as sensing to be discussed separately. Proposal 2: Design considerations for 6G waveform study for communication purposes to include: - new spectrum bands and associated requirements, e.g large BW - needs for new deployment scenarios, e.g. suburban macro, FWA, etc. - duplex operation, e.g., subband full duplex - enhancing coverage, e.g. design of low PAPR waveforms - Support for high power transmissions in uplink, e.g., higher power classes, MPR optimizations - integration with use cases such as sensing and positioning - Co-channel and adjacent channel requirements - Support for spatial multiplexing, beamforming, multiple access - Transceiver complexity associated with synthesis and reception; processing latency - Energy/power efficiency - Considerations on backward compatibility and coexistence with 5G - Scheduling flexibility and agility Proposal 3: In 6GR study on waveforms, focus on enhancements to the DFT-S-OFDM family of waveforms. Proposal 4: For 6GR, study the family of low PAPR waveforms obtained using DFT-S-OFDM with Pi/2 BPSK and truncated mapping. Proposal 7: For 6GR waveform study, for DFT-S-OFDM waveforms, decouple the size of allocation from the DFT size. Define any DFT size that is a product of powers of 2, 3 and 5 as a valid DFT size. Proposal 8: For 6GR waveform study, when considering DFT-S-OFDM waveforms, consider flexible frequency-domain mapping of the DFT output to the spectrum allocation, e.g., frequency-domain multiplexing of DMRS and data, non-contiguous mapping, etc. Proposal 9: For 6GR waveform study, consider multi-tx enhancements for DFT-S-OFDM where different transmit ports transmit over different frequency domain allocations. Proposal 10: For 6GR waveform study, consider feasibility to enhance spectrum utilization for small channel bandwidths using spectrum confinement techniques (e.g. WOLA) of reasonable complexity. |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 4: It is recommended that Frequency Domain Spectrum Shaping (FDSS) be considered a foundational component for the 6G uplink waveform design. Proposal 5: It is recommended to investigate enhanced FDSS techniques, with a focus on FDSS with Spectrum Extension (SE), for potential inclusion in the 6G specification. The study should also encompass other spectral processing methods, such as spectral truncation. Proposal 6: It is proposed that the evaluation of FDSS techniques for 6G shall encompass a wide range of resource allocation scenarios. This must include extreme configurations, such as very narrowband (e.g., ≤ 5 RBs) and very wideband (e.g., ≥ 100 RBs) allocations, to ensure a comprehensive performance characterization. |  |  |
| [41] | R1-2509372 R1-2509461 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
|  | Proposal 3: 3GPP should study the option of enabling mechanisms for PAPR reduction techniques both in CP-OFDM and DFT-s-OFDM. For 6GR, we propose to revisit such techniques that enables PAPR reduction for uplink coverage enhancements. Proposal 4: PAPR reduction techniques such as FDSS, FDSS-SE, with non-transparent frequency domain filtering technique along with their specification impacts should be studied in 6GR. Proposal 5: PAPR reduction techniques such as FDSS and FDSS-SE with transparent frequency domain filtering (receiver is not aware of the transmitter’s filter coefficients/parameters) should also be considered for study in 6GR. |  |  |

# 5.	Other waveforms

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | Proposal 4: RAN1 to deprioritize studying Zak-OTFS for the following reasons: - CP-OFDM outperforms Zak-OTFS with realistic simulation assumptions and realistic channel estimation - Zak-OTFS is claimed to provide benefit mainly in propagation conditions that are not typical in real deployments - Zak-OTFS would be a major change for the current systems even if it may be able to be implemented on top of CP-OFDM waveform - Zak-OTFS would introduce additional complexity to both network and UE side |  |  |
| [2] | R1-2508367 | Considerations for additional 6GR DL waveform | Kyocera Corporation |
|  | Proposal 1: A rigorous evaluation of cross-waveform interference and CP-OFDM coexistence with non-orthogonal waveforms in MRSS operations is required when considering the adoption of an additional waveform for 6GR. Proposal 2: RAN1 should evaluate the new waveform’s performance by considering practical scheduler implementations to ensure reliable performance results. |  |  |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
|  | - Proposal 4: Study the benefits of Spread OFDM compared to CP-OFDM and DFT-s-OFDM. |  |  |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 5. The proponents of non-OFDM based waveform are recommended to provide comprehensive air-interface design related to the proposed waveform, with proper evaluations according to the key requirements of 6GR. |  |  |
| [7] | R1-2508471 | Discussion on waveform for 6GR air interface | THALES |
|  | Proposal 1: RAN1 to investigate the performance of candidate waveforms under varying carrier frequency and time offsets inherent to satellite links, specifically considering scenarios with significant uncertainty in the UE’s position. This study will focus on conditions relevant to GNSS-free physical layer operation, aiming to identify robust waveform solutions suitable for environments with no GNSS availability. |  |  |
| [9] | R1-2508625 | Waveform for 6GR Air Interface | Shanghai Jiao Tong University. |
|  | Proposal 1: RAN1 to study additional waveform candidate (e.g., AFDM) for 6G that stays structurally compatible with OFDM, enabling reuse of 5GNR/6GR ecosystem components, while targeting enhanced performance in sensing, high mobility, and NTN scenarios. Proposal 2: RAN1 to investigate AFDM as a candidate waveform for 6G radio, with a specific focus on evaluating its robustness in high-mobility scenarios, high-efficiency sensing capabilities, low PAPR characteristics, and integration into an 6GR compatible system architecture. Proposal 3: RAN1 to study and specify the design of AFDM parameters (e.g., c1 and c2), low-complexity receiver algorithms for communication, sensing, and PAPR reduction, and their integration into a 6G-compatible system architecture, to enhance robustness against doubly-selective channels while targeting superior performance in sensing, high-mobility, and NTN scenarios. Proposal 4: RAN1 to investigate the implications of AFDM on MIMO channel estimation, receiver signal processing, etc., and to study low-complexity techniques to ensure the efficient integration of AFDM with existing multi-antenna systems. |  |  |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
|  | Proposal 5: Discontinue the study for the potential support of “other waveforms” in 6GR - Deviating from “single technology framework” goal of 6GR study - Increased risk of RAT divergence for different 6GR use cases - Not conducive to the agreed support for NR-6GR migration via MRSS |  |  |
| [17] | R1-2508833 | Waveform for 6G Radio Air Interface | IMU |
|  | Proposal 1: Redesign The CP to actively utilize its redundancy rather than being discarded. Proposal 2: The CP duration should become a function of the channel characteristics as well as of the numerology. Proposal 3: Redesign 5G NR OFDM frame and symbol structure to achieve the goals of multi-numerology operation (via different methods) within a single unified framework. Proposal 4: The 6GR frame structure and OFDM symbol should be designed as a uniform waveform framework capable of meeting all major 6G requirements, technologies, and channel conditions instead of multiple waveform types. Proposal 5: The uniform waveform framework design should natively support: - Low-PAPR, energy-efficient transmission, - Native integration of sensing (ISAC), - AI-native and adaptive operation, - URLLC and massive connectivity, - High spectral efficiency with backward compatibility, - Coexistence with low power applications such as OOK generation for WuS and Ambient IoT, - Unified TN and NTN operation, - Extreme flexibility and forward compatibility, - Improved uplink coverage and coexistence. Proposal 6: Redesign the CP-OFDM-based 6GR frame structure to enable native integration of heterogeneous services within a single, reconfigurable architecture. Proposal 8: RAN1 should study U-OFDM as downlink waveform for 6G Proposal 9: standardize a small set of subcarrier spacings (SCS)s families for U-OFDM to simplify implementation while enabling flexible alignment with NR resource structures, guard/circularity timing, and high-SCS operation, as in Table 1. Table 1: Proposed U-OFDM Subcarrier Spacing (SCS) Families ID; SCS Formula Δf; μ Range; Design Principle / Alignment. A; 2μ×15kHz; μ∈{4,5,6,…}; NR-consistent binary scaling of SCS (legacy-friendly), extended to very large Δf for high carrier frequencies.. B; 2μ×12×15kHz; μ∈{0,1,2,…}; SCS equals an integer multiple of the NR resource block (RB) bandwidth (12 subcarriers), i.e., Δf aligns to 12×15kHz= 180 kHz and its powers of two.. C; 2μ×14×15kHz; μ∈{0,1,2,…}; Subsymbol duration aligns with CP/UW duration: for μ=0CP=1 subsymbol, for μ=1CP=2 subsymbols, etc. (circularity-friendly timing lattice).. Proposal 10: Support flexible and scalable time-frequency resource allocation in 6G, considering the following generalized definitions of the Resource Block (RB): - Option 1: RB defined by a single subcarrier-subsymbol pair (m,n); - Option 2: RB defined by a single subcarrier m and a group of subsymbols in time; - Option 3: RB defined by a single subsymbols n and a group of subcarriers in frequency; - Option 3: RB defined by a group of subcarriers in frequency and a group of subsymbols in time. Proposal 11: RAN1 should study both CP-U-OFDM and UW-U-OFDM for 6GR. Proposal 12: The CP itself should be generalized to consist of one or more OFDM subsymbols. Proposal 13: Consider incorporating both CP-based and UW-based U-OFDM guard structures in 6G waveform discussions. Proposal 14: Study the use of cut-off redundant subsymbols in U-OFDM as a means to enable mini-block segmentation of a symbol. Proposal 15: The intrinsic low-PAPR feature of U-OFDM be leveraged for energy-efficient transmission and considered in future PAPR-aware configuration studies. Proposal 16: Study U-OFDM as a natively URLLC-capable waveform. Proposal 17: Evaluate U-OFDM as a baseline ISAC-compatible waveform. Proposal 18: Study the channel-robust properties of U-OFDM under high-mobility and long delay scenarios, focusing on adaptive cut-off placement and subsymbol duration. Proposal 19: Study adaptive cut-off placement strategies for high-mobility users to minimize ICI while maintaining spectral efficiency and low receiver complexity. Proposal 20: Evaluate guard design methods where the number of redundant subsymbols is determined by measured or estimated delay spread rather than fixed numerology parameters. Proposal 21: Consider U-OFDM as a baseline design for channels with simultaneous delay and Doppler dispersion, given its flexible and adaptive parameterization. Proposal 22: Evaluate U-OFDM as a unified service-multiplexing waveform for 6G, capable of concurrently serving heterogeneous users and applications under a single numerology-free configuration. Proposal 23: Consider studying the optimizing cut-off placement, subsymbol scheduling, and guard design to dynamically adapt to mixed service and channel profiles. Proposal 24: Study U-OFDM as an evolutionary extension of CP-OFDM, offering enhanced flexibility and functionality while maintaining interoperability with existing NR systems. Proposal 25: Evaluate U-OFDM as a unifying waveform framework capable of reproducing standard CP-OFDM for legacy compatibility while simultaneously supporting interference-free mixed-numerology-like performance. Proposal 26: Evaluate U-OFDM as an OOK-native waveform for energy-limited technologies such as Ambient IoT and wake-up radios. Proposal 27: Study U-OFDM unified waveform framework for advanced MIMO and beamforming operations in future systems. Proposal 28: Explore the subsymbol-level processing for adaptive precoding, aging compensation, and sensing-aided beam tracking under high mobility and wideband conditions. Proposal 29: Evaluate U-OFDM as a time-domain full-duplex solution achieving SBFD performance objectives without additional spectrum or hardware overhead. Proposal 30: Study scheduling algorithms and control signaling for dynamic UL/DL interleaving at the subsymbol level to optimize latency and utilization. Proposal 31: Study the micro-grant and repetition-based combining schemes as optional U-OFDM configurations for power-limited or coverage-critical uplink devices. Proposal 32: Consider U-OFDM as a complimentary solution to SBFD, delivering comparable coverage and latency benefits with reduced implementation cost and system complexity. Proposal 33: Evaluate U-OFDM under standardized SE metrics to quantify the combined gain from adaptive guard allocation, guard reuse for signaling, and pilot reduction, particularly in multi-service and ISAC-enabled scenarios. Proposal 34: Study U-OFDM as the baseline structure for AI-adaptive PHY design Proposal 35: Evaluate U-OFDM as the reference waveform for sub-millisecond and microsecond latency applications, including semantic and cooperative AI communications. Proposal 36: Study U-OFDM as an enabling waveform for AI-native traffic models with symmetric UL/DL requirements. Proposal 37: Study UW-U-OFDM for CFO tracking and frequency synchronization. Proposal 38: Evaluate UW-U-OFDM as a waveform candidate for integrated PN tracking. Proposal 39: Study UW-based synchronization schemes in U-OFDM systems. Proposal 40: Explore and study the UW-based per-beam signaling for multi-beam synchronization and reciprocity calibration in large-array TDD deployments. Proposal 41: Adopt UW-U-OFDM as a baseline ISAC-compatible waveform, where UW design jointly optimizes synchronization, channel estimation, and sensing performance without additional resource consumption |
| [18] | R1-2508844 | Waveform for 6GR air interface | BJTU |
|  | Proposal 1: RAN1 should study receiver-side DD-domain signal processing techniques that maintain full compatibility with the CP-OFDM transceiver, for high-mobility and ISAC enhancements of CP-OFDM. Proposal 2: RAN1 should study the design criteria for TF-domain pilot patterns that enable accurate DD-domain channel estimation, ensuring the pilot spacing is sufficient to avoid aliasing for the target delay and Doppler spreads of 6GR deployment scenarios. Proposal 3: RAN1 should evaluate the DD-a-OFDM scheme as a candidate technology for performance enhancement of CP-OFDM in high-mobility and ISAC use cases, with evaluation metrics to include BER under high Doppler, channel estimation accuracy and sensing precision (MSE against CRLB), and pilot overhead compared to baseline, i.e., the classical CP-OFDM. |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 10: DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level (i.e., eDFT-s-OFDM waveform) can be considered as a candidate waveform technology for 6G waveform design to improve the performance at least for high-speed scenario. Proposal 11: GFB-OFDM should be considered in 6G waveform study as a scheme to improve the spectrum utilization. |  |  |
| [21] | R1-2508863 | Support for Diverse Waveforms on the 6GR Air Interface | National Spectrum Consortium |
|  | Proposal P1. Study suitability of different waveforms for different services and device types and identify key elements of a framework that enables the 6G protocol stack to support different physical layer waveforms. |  |  |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 1: 6GR should allow certain time / frequency resources can be different waveform for forward compatibility perspective and to support MRSS. Proposal 2: For 6GR waveform design, time/frequency grid should be allowed to be aligned and orthogonal with NR boundary. Proposal 3: OFDM-based waveform should be supported for 6GR. - The definition of “OFDM-based” is to have subcarrier mapping and IFFT to generate time-domain signal. Proposal 4: Striving for OFDM-based waveforms across all the identified use cases can be sufficient at least for 6G Day 1. Proposal 5: Any enhancements to CP-OFDM or DFT-s-OFDM and/or any newly introduced waveform must demonstrate clear and justified advantages over 5G waveform. Proposal 8: The need to introduce flexible DMRS and data techniques (e.g., TDM between data and DMRS before DFT precoding such as OTFDM) could be investigated. |  |  |
| [26] | R1-2508973 | Discussion on 6GR waveform | ETRI, University of Surrey |
|  | Proposal 1. RAN1 to explicitly capture pros and cons of potential new or enhanced waveform candidates as summarized in the FLS. - Simple re-use of NR CP-OFDM and DFT-s-OFDM without any official RAN1 observation is NOT recommended, given that a number of contributions have been submitted during three WG meetings with in-depth assessments on the waveform candidates. Proposal 2. RAN1 to capture the following summary on AFDM waveform to the TR on 6GR: Description. Name of the proposal; AFDM. Applicable link direction; UL / DL. Enhancement to CP-OFDM?; Yes. Enhancement to DFT-s-OFDM?; Yes. Additional OFDM-compatible waveform?; No. Target channel(s); PDSCH, PUSCH, PRACH (for NTN). Target modulation; No restriction (applicable for all existing NR modulations). Motivation / use case; NTN, ISAC, high-speed mobility. Key Metric / KPI; BLER, sensing capability, possible NetGain in low-PAPR and delay-Doppler robustness. Key spec impact foreseen; RAN1 waveform definition, time-frequency resource mapping, pilot design, scheduling extensions. |
| [29] | R1-2509059 | Waveform for 6G NR | Cohere Technologies, IIT Delhi |
|  | Proposal 1: Zak-OTFS and its special variance Zak-OTFS-over-OFDM are included in the waveform study for 6G Proposal 2: The study of ADFM will be included under the Zak-OTFS study for the 6G waveform |  |  |
| [36] | R1-2509282 | Discussion on Waveform | NTT DOCOMO, INC. |
|  | Proposal 1: For 6GR study on waveform, - Only OFDM-based waveform(s) should be considered (as described in the SID) - Any new waveform(s), even for OFDM-based, should be justified by clear gain - Unified design across scenarios/use cases is strongly preferred - Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g., - Potential better coverage by better PAPR performance for uplink - Spectrum efficiency improvement |  |  |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
|  | Proposal 2: Support for OFDM-OOK kind of waveforms for low end devices. |  |  |
| [40] | R1-2509368 | Discussion on Waveforms of 6GR Air Interface | Rakuten Mobile, Inc |
|  | Proposal 1: In extending NR waveforms and finding alternative candidates for 6GR waveform, - Identifying the target areas for relevant factors based on typical 6G scenarios - Consider at least the following areas: - Compatibility to NR waveforms for MRSS - Complexity and power efficiency. Proposal 2: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following scenarios and channel conditions: - High-mobility DL - including assessment on the resilience to Doppler shifts and inter-carrier interference - Fragmented Spectrum and Sparse Access - including assessment on the adaptability for spectrum allocation in non-contiguous or opportunistic bands. - Low-SNR and cell-edge - including investigation on the performance in coverage-limited areas accounting for robustness and power efficiency. - JSAC and multi-service integration - including exploration of structures supporting simultaneous sensing and communication. Proposal 3: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following aspects for the impacts on spectrum sharing and compatibility with NR: - Waveform coexistence and guard band design - including analysis on the coexistence of CP-OFDM and candidate waveforms within the same band - Numerology and timing alignment - including investigate timing and subcarrier spacing alignment across waveforms - Control and data channel multiplexing - including assessment on control channel decoding, synchronization signal design, and cross-carrier scheduling impacted by waveform diversity. Proposal 4: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following aspect for the impacts on complexity and power consumption from the PHYr perspective: - PA efficiency and PAPR - including analysis of PA linearity and energy drain. - Baseband processing load - including assessment of computational complexity of channel estimation, equalization, and demodulation for candidate waveforms. - RF front-end duty cycles - including analysis of RF activity patterns. - RAN4 may be involved in - Thermal and energy budget constraints - including analysis of power impacted of waveform processing under typical traffic loads. |  |  |
| [41] | R1-2509372 R1-2509461 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
|  | Proposal 2: Study the use of single carrier TDMA bursts in the current frame structure for NTN. |  |  |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
|  | Proposal 4: For 6GR waveform with consideration of MRSS Compatibility - MRSS compatibility shall be a mandatory design principle for 6GR waveform development, ensuring seamless coexistence and migration with NR-based systems across shared-spectrum deployments. - CP-OFDM shall be maintained as the baseline waveform for downlink, and CP-OFDM and DFT-s-OFDM shall remain aligned with the NR grid structure on uplink to preserve consistent time–frequency behavior essential for MRSS operation. - Any optional waveform enhancements shall be introduced only if they remain fully transparent to MRSS, without altering grid alignment, coexistence characteristics, or cross-RAT operational behavior. - New waveform families shall be considered only when they provide clear and well-justified performance gains, and only if their introduction does not compromise MRSS coexistence, synchronization, or spectrum-sharing performance. Proposal 5: For unified or multiple waveforms for 6GR - An OFDM-based waveform shall be adopted as the unified baseline for 6GR, maintaining consistency with NR and avoiding unnecessary divergence in device and network implementation. - Multiple baseline waveforms shall not be introduced, as doing so would create fragmentation, increase complexity, and undermine MRSS alignment and coherent multi-band operation. - Optional, OFDM-compatible waveform features may be studied for specific use cases, provided they operate transparently within the unified grid structure and are considered only when they offer clear and well-justified benefits without altering the baseline framework. |  |  |
| [44] | R1-2509413 | OSDM for 6GR | University of Sheffield |
|  | Proposal: To further advance the study of CP-OSDM in 6GR compared to CP-OFDM. |  |  |

# 6.	Rank > 1 for UL DFT-s-OFDM

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | UL Multi-layer DFT-s-OFDM Proposal 12: Low PAPR waveform study, if any, for multi-layers UL transmission in 6G needs to consider at least fully coherent and partially coherent UEs. Proposal 13: For study of UL multi-layer MIMO enhancement with low PAPR DFT-s-OFDM waveform, the power-limited UEs and power non-limited UEs should be considered, and the net gain should be evaluated as follows: Table 4: MIMO net gain metric for power limited and power non-limited UEs Power limited UE; Power non-limited UEs. MIMO Net gain [dB]; Tx power difference [dB] – Rx SINR difference for 10% BLER [dB]; Rx SINR difference for 10% BLER [dB]. Proposal 14: Support CP-OFDM for all number of layers in uplink. Proposal 15: The potential of UL DFT-s-OFDM for more than 1 layer in 6G needs further justification. Proposal 16: For multi-layer DFT-s-OFDM potential study and comparison with CP-OFDM, RAN1 to consider the following metric evaluations: - Cell average/edge SE gain or loss relative to the 5G NR baseline reference - UL multi-layer net gain for power limited UEs and power non-limited UEs according to Table 4 Proposal 17: RAN1 to consider the following baseline reference for multi-layer waveforms study: - 5G NR UL MIMO codebooks - CP-OFDM for all number of layers & DFT-s-OFDM for single layer - Fully coherent UE coherence capability - DWS rel-18 enabled [disabled optional] Proposal 18: RAN1 to consider the following system level configuration for multi-layer UL waveforms study: Parameters; Values. System configuration; Uma, 21 cells, 10 UEs/ cell and up to 50 UEs/cell. Traffic model; Full buffer, [FTP model TBD in AI 11.2]. Carrier frequency; 3.5GHz, 7GHz. Channel bandwidth/Subcarrier spacing; Up to 100 MHz, 30KHz. Frame structure; TDD (DDDDU). UE ports; Up to 8. BS antenna configuration/ports; Baseline configuration in AI 11.2. UE antenna model; Isotropic. ISD; 500. Maximum Number of layers (maxRank) per UE; Adaptive rank, maxRank 2 SU-MIMO. Receiver; LMMSE. Waveform and MIMO configuration; 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies. Power class and power mode; Power class 2, Full power mode. UL Maximum power; - Static assumption per waveform for all modulations and number of layers with average power difference between waveforms of 1 or 2 dB - [optional] MPR based Pcmax according to modulation/FDRA/waveform/coherency/etc.. Proposal 19: RAN1 to consider the following link level configuration for multi-layer UL waveforms study: Parameters; Values. Carrier frequency​​; 3.5GHz, 7GHz​​. Subcarrier spacing​​; 30kHz​​. UE/BS antenna ports; 2 or 4 / 32. FDRA; 64, and 256 RB or 270 RB​​. Waveform​​s; CP-OFDM DFT-s-OFDM​​. MCS/modulation​​; [4,9,10,16,17,23] QPSK/16QAM/64QAM​​ MCS Table 1. Number of layers​; 2 layers​​. Channel model​​; CDL-A30​​. UE speed​​; 3km/h​​. Channel est.​​; Practical​. Precoding matrix​; Adaptive based on channel. HARQ retransmission​; disabled. DMRS configuration; Configuration type 1 2 DMRS symbols per slot. Number of PUSCH data; 12 symbols. Waveform and MIMO configuration; 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies. BLER target; 10%. |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
|  | - Proposal 1: For evaluation of DFT-s-OFDM for uplink high-rank transmission in 6GR, the following aspects can be considered: - UL Precoder Design: Evaluate the impact of precoder design on PAPR and link-level performance, considering both single-layer and multi-layer mapping to antenna ports. - MIMO Receiver Type: Compare waveform performance using linear receivers (e.g., MMSE), while noting the limitations of non-linear receivers such as ML for DFT-s-OFDM. - Coverage Performance Metrics: Use metrics such as MCL, MPL, and MIL to assess coverage performance under different waveform and rank configurations. |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
|  | - Proposal 5: Perform a Link-Level simulation for a comprehensive evaluation of the net gain achieved by two-layer DFT-s-OFDM transmission over two-layer CP-OFDM. - Proposal 6: Support rank 2 for DFT-s-OFDM waveform at least for non-coherent precoders. |  |  |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 1. The target use cases are recommended to be clarified and distinguished between multi-layer CP-OFDM and multi-layer DFT-s-OFDM before starting detailed design for multi-layer DFT-s-OFDM. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 12: For evaluating DFT-s-OFDM for UL with number of layers > 1, link level simulation is considered: - Net gain = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - The reference is the link level performance of CP-OFDM with corresponding number of layers. - A realistic PA model should be used. - Both low (e.g., 20MHz) and high (e.g., 100MHz) bandwidth. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 5: Evaluation for multi-layer DFT-s-OFDM for UL should include gNB advanced MIMO receiver complexity (e.g., ML or MMSE-SIC). Proposal 6: Baseline method for comparison of multi-layer DFT-s-OFDM for UL should be multi-layer CP-OFDM with frequency selective precoding Proposal 7: MPR for multi-layer DFT-s-OFDM should be determined using link level simulation Proposal 8: Co-polarization instead of cross-polarization should be assumed for UE antenna polarization when multi-layer DFT-s-OFDM is evaluated |  |  |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
|  | Proposal 4: RAN1 to consider system-level evaluations for studying the multi-layer DFT-s-OFDM in the uplink by accounting for MPR based on RF power class and RF power scaling models with system-level parameter settings in Table 4 as a starting point. Table 4: System-level simulation parameters for multi-layer DFT-s-OFDM and multi-layer CP-OFDM. Carrier frequency; 4 GHz. Subcarrier spacing; 30 kHz. Bandwidth (#PRBs); 20 MHz (51 PRBs). Channel model; 38.901. Deployment scenario; UMa scenario with 7 cell sites with 500m inter-site distance and 200 UEs in total. gNB antenna configuration; (M, N, P, Mg, Ng; Mp, Np) = (12, 8 ,2, 1, 1; 4, 8) with (dH, dV) = (0.5, 0.8)λ and electrical downtilt of 10°. UE antenna configuration; 2 Tx: Two cross polarized antenna elements. MIMO precoder; Non-coherent codebook for UL 2Tx in 3GPP NR. gNB antenna height; 25 m. gNB noise figure; 5 dB. UE antenna height; According to 36.873. UE transmit power; MPR is selected based on the RB allocation (e.g., inner/outer/edge), RB size and modulation order, subject to RAN4 requirements.. Power control parameters; α=0.8, P0 = -80 dBm. gNB noise figure; 5 dB. Modulation and coding scheme settings; Table 6.1.4.1-1 with q=1 in TS 38.214 (DFT-s-OFDM) Table 5.1.3.1-1 in TS 38.214 (CP-OFDM). Traffic model; FTP Model 1. Scheduling; Frequency domain scheduling. Uplink receiver; MMSE-IRC. Metric; Mean, median, and cell-edge user throughputs. Proposal 5: RAN1 to consider at least UE power class 2 with Rel-15 power scaling model and power class 3 with Rel-16 MODE0 scaling for 2TX antenna configurations at the UEs in the system-level evaluations of multi-layer DFT-s-OFDM in the uplink. Proposal 6 : Support multi-layer DFT-s-OFDM and multi-layer CP-OFDM for uplink transmissions in 6GR. |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 10: For DFT-s-OFDM for UL with multiple layers, the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 10: Take Table 11 as the start point for UL multi-layer DFT-s-OFDM link level net gain evaluations. Table 11 Evaluation assumptions for UL multi-layer link-level performance evaluation Parameters; Values. Carrier frequency and scenario; 4 GHz, 7GHz. Channel BW; Up to 400MHz (7GHz), Up to 200MHz (4GHz),. SCS; 30 kHz. Channel model; CDL-C 300ns. UE speed; 3km/h. Number of TX antennas; 2 or other value up to 16 for 7GHz and up to 4 for 4GHz. Number of RX antennas; 4GHz: 32,64,128 7GHz: 32,64,128, 256, 512. MCS; Reported by companies. Rank per UE; ≥ 2. Number of DMRS symbols; 2. Number of PUSCH data symbols; 12. HARQ configuration; No retransmissions. Frequency hopping; Disabled. BLER; 10%. Proposal 11: At least 2-layers uplink DFT-s-OFDM (including enhancement) waveform with observed coverage net gains should be supported in 6GR. The maximum number of layers should be further studied in 6GR MIMO agenda, e.g., codebook design. |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
|  | UL DFT-s-OFDM rank>1 Proposal 2: To assess whether multi-rank DFT-s-OFDM can offer significant spectral efficiency gain, focus the study on rank-2 (two-layer UL transmission on PUSCH) Proposal 3: To assess whether rank-2 DFT-s-OFDM can offer significant UL spectral efficiency gain, further investigate its performance in deployment scenarios with primarily line-of -sight channels |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 8: The link-level simulation to evaluate the PAPA/Tx power gain compared with the OFDM is considered to justify the performance of DFT-s-OFDM for UL with number of layers > 1. Proposal 9: DFT-s-OFDM with rank 2 for uplink transmission can be considered in 6G waveform study. |  |  |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 7: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks. |  |  |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
|  | Proposal 2: Support the DFT-s-OFDM waveform for multiple layers for UL transmission. |  |  |
| [28] | R1-2509049 | Discussion on waveform for 6GR air interface | Ruijie Networks Co. Ltd |
|  | Proposal 1: - If support for DFT-s-OFDM with rank > 1 is defined, RAN1 will not limit the applicable bands - DFT-s-OFDM with Rank > 1 study is - Limited to max rank = 2 |  |  |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
|  | Proposal 8: Use the Net Gain as Evaluation Metric for Multi-Layer DFT-S-OFDM - Baseline scheme is 2-layer DFT-S-OFDM with non-coherent precoding |  |  |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
|  | Proposal 8: Consider UE power output capability (e.g., MPR) as a criterion for evaluating DFT-s-OFDM for multilayer uplink transmission. |  |  |
| [34] | R1-2509231 | Waveforms for 6GR | Qualcomm Incorporated |
|  | Proposal 5: For 6G Radio, support DFT-S-OFDM in addition to CP-OFDM for multi-layer transmissions in uplink. Proposal 6: Study feasibility of using non-identity precoders for multi-layer DFT-S-OFDM transmissions with focus on the PAPR-precoding gain trade-off of using such precoders. |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 7: The support of at least Rank 2 DFT-s-OFDM for uplink transmission is proposed for inclusion in the 6G waveform study. |  |  |
| [38] | R1-2509322 | Study on waveform for 6GR | Sharp |
|  | Proposal 2: For the evaluation of multi-layer UL DFT-s-OFDM, Net Gain should be used and the assumed precoder should be reported. |  |  |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
|  | Proposal 3: For DFT-s-OFDM with Rank > 1 for UL as 6GR waveform - Alt 1a: DFT-s-OFDM is used for lower ranks (e.g. 1 and 2) only, CP-OFDM is used for all ranks. - Alt 2a: applicable for all bands |  |  |

# 7.	Uncategorized proposals

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | Baseline communication waveform - Proposal 1: CP-OFDM waveform as defined in 5G NR is supported for communications in 6G downlink. - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink - Proposal 2: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for communications in 6G uplink. - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded as potential additions. Low PAPR for coverage extension - Proposal 9: High power class should be the baseline for 6G due to significant enhancement in coverage. - Proposal 10: Power boosting features such as the ones specified in NR should be part of the baseline for 6G. Proposal 11: Dynamic waveform switching using DCI is introduced to 6G in the first release |  |  |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
|  | #### 1. Continuity and differentiation from 5GNR system - Proposal 2: Discuss how the UE should be instructed to use a particular waveform for 6GR in uplink. - Option 1: Waveform selection based on transmission rank. - Option 2: Explicit waveform indication via cell-specific configuration, channel-specific, or BWP-specific configuration, including dynamic switching. - Option 3: Waveform selection based on frequency band or usage scenario. - Proposal 3: Discuss whether reference signal design should consider commonality across CP-OFDM and DFT-s-OFDM in both uplink and downlink. #### 4. New Waveform for sensing - Proposal 8: A new waveform such as FMCW is studied for sensing as well as OFDM. |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
|  | - Proposal 4: Support to study AI/ML-based waveform enhancements together with non-AI waveforms. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR: - Larger FFT size (e.g. from 4096 to 8192 or 16384) - Larger transmission channel bandwidth - Increased downlink free space path loss in NTN due to large propagation distance. Proposal 14: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW): - OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication - OFDM-based CW with same SCS as communication Proposal 15: For enhance the sensing, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing). |  |  |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
|  | Proposal 1: CP-OFDM and DFT-s-OFDM in NR are baseline as 6GR uplink waveform. 6GR could study to support dynamic waveform switching during initial access. Proposal 2: 6GR strives for a unified waveform baseband generation and upconversion for all channels and signals including PRACH. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 3: Support dynamic waveform switching for the uplink for 6GR. Proposal 4: Waveform for sensing is not covered in Agenda Item 11.3.1 and shall be studied separately in Agenda Item 11.14. |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 1: A unified 6GR baseline waveform is studied to fulfil the requirements of eMBB and 6G IoT. - The baseline waveform is used for 6G HRLLC. Proposal 2: Study waveforms to fulfil the requirement of 6G Sensing and 6G NTN (Ubiquitous Connectivity). - Strive for reusing the 6GR baseline waveform for 6G Sensing and 6G NTN. - An additional waveform can be considered if significant gain over the baseline waveform can be justified for a specific vertical scenario, but only supported by the vertical BS/UE. Proposal 4: For studying the 6GR baseline waveform, support up to 2 waveforms in DL and up to 2 waveforms in UL, e.g., one optimized for spectrum efficiency, one optimized for coverage. - At least 1 waveform in DL and 1 waveform in UL are mandatorily supported for all device types, e.g., CP-OFDM in DL and DFT-s-OFDM in UL. Proposal 5: Only one DL waveform is supported for 6GR initial access procedure. Proposal 11: Study multiple access (MA), targeting a single MA scheme for each waveform, to fulfil the requirement of all 6G usage scenarios using this waveform. Proposal 12: Orthogonal multiple access (OMA) is the baseline for 6GR. Evaluate OMA proposals using 5G NR as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 6: Study pruning QAM under CP-OFDM waveform for ISAC. |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 12: Study pulse RS design using OFDM-based waveform for large sensing coverage. - The pulse is applicable for both mono-static and bi-static sensing. - Study the application for communication, e.g., RSRP measurement, time/frequency tracking. |  |  |
| [23] | R1-2508890 | Waveform considerations for 6G Uplink | Wisig, IITH |
|  | Proposal-3: DFT-s-OFDM-based waveforms should be considered the primary uplink waveforms; CP-OFDM may be considered secondary. |  |  |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
|  | Proposal 3: Postpone the dynamic waveform switching related discussion until most of the details for each waveform are finalized. |  |  |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 2: Evaluate dynamic waveform switching between CP-OFDM and DFT-s-OFDM modes under MRSS-aligned 6GR configurations using the Net Gain criterion (@ 10% BLER) for both single- and multi-layer uplink transmissions. Proposal 5: The "Net Gain" evaluation framework shall be used to evaluate AI/ML-based PAPR reduction schemes. Proposal 6: In addition to "Net Gain," AI/ML-based schemes should be evaluated on their computational complexity and feasibility. The following metrics can be considered: - Model Complexity (e.g., number of parameters, FLOPs/symbol). - Signaling Overhead (e.g., bits for side information, if any). - Complexity Type: A clear distinction between offline training requirements and the real-time, on-device inference complexity. |  |  |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
|  | Proposal 6: Send LS to RAN4, to inform RAN4 that near constant envelope waveform shall be considered for 6G, and request RAN4 to evaluate the PA model, RF requirements, maximum power boost, and Tx power gain of a near constant envelope (sub-1dB) waveforms. |  |  |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
|  | Proposal 1: Confirm that CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for 6GR for uplink. Proposal 5: Consider high UE power class (e.g., 26 dBm) as a mandatory feature or at least mandatory for higher bands (e.g., above 2 GHz) in 6GR from Day 1. Proposal 6: Study the possibility of reducing MPR in 6GR. Proposal 7: Support dynamic switching between DFT-s-OFDM and CP-OFDM in 6GR from Day 1. Proposal 8: Consider UE power output capability (e.g., MPR) as a criterion for evaluating DFT-s-OFDM for multilayer uplink transmission. |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 3: we propose considering a unified waveform design framework to simultaneously support communication and sensing functionalities (ranging/velocity estimation/imaging) in 6G systems. Proposal 9: It is proposed to study efficient waveform multiplexing and switching mechanism for 6GR. |  |  |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
|  | Proposal 3: Support for waveform selection for different time/frequency/physical channels/physical signals. |  |  |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
|  | Proposal 1: For baseline waveform candidates for 6GR - CP-OFDM should be maintained as the baseline waveform for 6G downlink, preserving continuity with NR, ensuring backward compatibility, and supporting consistent numerology and grid alignment across RATs. - Both CP-OFDM and DFT-s-OFDM should be retained as baseline waveforms for 6G uplink, maintaining alignment with NR and supporting a unified uplink waveform framework across deployment scenarios. |  |  |

# 8.	Monday offline

## 8.1	DL PAPR reduction

~10 companies propose to deprioritize/discontinue the DL DFT-s-OFDM discussions under the waveform agenda item and take up the proposal under the specific agenda item that justifies the use case (NTN, energy efficiency, etc.)

Question 1: Discuss the following as a possible way forward

- Discontinue discussion on DL DFT-s-OFDM for DL PDCCH/PDSCH for communication for TN under waveform agenda item. DL DFT-s-OFDM may be further studied for

- PDCCH/PDSCH for NTN under the NTN agenda item

- [on-demand] Sync/SSB under the initial access or energy efficiency agenda item

- Wake-up signal under initial access or other physical layer signals, channels and procedures agenda item

Assuming that the work is not deprioritized/discontinued, characterize what the proposed solution alternatives are.

Question 2: Discuss the characterization of the solution alternatives:

- Target channels/signals for transform precoding

- Common PDCCH

- Common PDSCH

- UE-specific PDCCH and unicast PDSCH (including Msg4)

- [On-demand] Sync/SSB

- Wake-up signal

- Application of the transform precoding

- Alt1: Before multiplexing with other DL signals/channels, i.e.:

  - Transform precoding is applied to each DL signal/channel among the set of signals/channels to be multiplexed for transmission in the same set of OFDM symbols

  - DFT size is the same as the number of assigned subcarriers for the DL signal/channel

- Alt2: After multiplexing with other DL signals/channels, i.e.:

  - Transform precoding is applied to a set of DL signals/channels multiplexed on a set of subcarriers prior to the transform precoding

  - DFT size is the sum of assigned subcarriers for the DL signals/channels being transmitted

- Alt3: After multiplexing with a subset of other DL signals/channels. I.e. more than one set of signals/channels multiplexed as in Alt2 can be frequency-multiplexed.

- Multiplexing of DMRS and PDCCH/PDSCH

- Alt1: transform precoding is applied to the PDCCH/PDSCH REs excluding the DMRS REs

- Alt2: transform precoding is applied a contiguous set of PDCCH/PDSCH REs, DMRS and PDCCH/PDSCH are time-multiplexed on different symbols

### 8.1.1	Company comments

Provide your views on question 1 above

| Company | Comment |
| --- | --- |
| CMCC | Agree. A certain specific downlink physical channel or signal may apply DFT-s-OFDM waveform for some dedicated use case (e.g., LP-WUS). Such specific design can be studied in the corresponding agenda item (e.g., energy saving). |
| vivo | We are generally fine with this arrangement. It is helpful to clarify the scope of this DL DFT-s-OFDM in 11.3.1 discussion first. |
| OPPO | Support. |
| Xiaomi | We are Ok to agree to the proposed wayforward if unfortunately we could not reach conseuns on any proposal in question 2. |
| InterDigital | We support the proposal. If there are companies who believe DL DFT-s-OFDM could be useful for NTN, the need/justification shall be discussed in the NTN agenda. The last two bullets can be discussed in 11.5, “Energy Efficiency”. |
| NEC | We do not support this proposal to discontinue the discussion in this ageda item. Key 6G requirements like a strong focus on Energy Savings and coverage enhancement from Day-1 create an immediate need for a downlink waveform with better coverage and power efficiency. These requirements should not be discussed within the individual agenda items like NTN but are fundamental to the goals of 6G for all deployments. |
| Sony | We are not fine with the proposed way forward. This agenda item is meant to discuss aspects of waveforms that can be used in both UL and DL for all use cases. We prefer to discuss the issues of DFT-s-OFDM for DL and PAPR reduction for both CP-OFDM and DFT-s-OFDM here. This reduces the work on all waveform delegates and eases scheduling issues during the study. |
| QC | Agree in principle that we need to downscope. How to handle future discussions may need more deliberation. |
| LGE | On the order of discussion, we prefer to discuss Question 2 first. Then, we are OK with the direction of Question 1 as mentioned by Xiaomi. On the Question 2, we also need to have KPI for the DL DFT-s-OFDM. |
| Ericsson | Agree in general. Based on the signal-specific enhancments cannot be used as a motivation to introduce DFT-s-OFDM as such enhancements can be discussed in the signal design discussions. |
| Apple | Okay with the way forward. |
| ETRI | We think that the discussion can be resumed after we first finalize the evaluation assumptions (e.g., Questions 6 to 8 below).” |
| Samsung | Agree. In our contribution, we have provided a lengthy analysis demonstrating that there is no evidence of any potential benefit of DL DFT-s-OFDM over CP-OFDM in use cases such as NTN, IoT, FR3, energy efficiency, common signals/channels. If needed, new waveform for sensing purpose can be discussed under the 6GR ISAC study. While we much prefer to discontinue the study for all use cases (except for ISAC), we may be able to accept the feature lead proposal in Question 1 as a compromise, i.e. there is no need to continue the discussion on DL DFT-s-OFDM for TN under waveform agenda item. Regarding the comment from CMCC, we are also fine to study WUS signal design under the WUS or Control Channel agenda item. |
| Panasonic | Support the way forward. |
| Lenovo | Although we think 11.3.1 is the right agenda item to discuss all related communication waveforms for 6GR, we are open to discuss it under other AIs. |
| Spreadtrum | From a discussion organization perspective, the 6G waveform should establish a clear framework. For DL DFT-s-OFDM, a definite conclusion is needed—either support or not—instead of using “may be further studied.” We should avoid repeated discussion in later related agenda. It is recommended that we can study and collect the simulation results for DFT-s-OFDM in target use cases as a reference. Detailed design can be discussed in related agendas. |
| Lekha | Agree with Spreadtrum on this point. We need to reach a conclusion for DFT-s-OFDM for specific use-cases on whether to support it or not. |

Provide your views on question 2 above

| Company | Comment |
| --- | --- |
| CMCC | If the study of DL DFT-s-OFDM waveform is deprioritized, there is no need to discuss about this question. |
| vivo | Regarding alternatives for application of the transform precoding For Alt 1: if we follow this operation, the whole waveform from BS side is not DFT-s-OFDM, the claimed PAPR benfit is gone. Then why do we need to have this DFT-s-OFDM for DL? For Alt 2: we need to note this means different UEs can only be multiplexed in time domain, which limits the capacity of the whole network. Otherwise, UE has to receive signals intended to be transmitted to other UEs, as each UE needs to receive the signal of the whole BW. Such complexity is not acceptable at UE side. For Alt 3: same as Alt 1, the claimed PAPR benefit is gone. |
| OPPO | We are fine to study the potential solutions for DL DFT-s-OFDM. But 1) they should be proposed by at least proponent. 2) They should be clarified. - Application of the transform precoding - Alt1: It is not single-carrier in DL. We do not think this Alt should be considered. - Alt2: Is it a transform precoding for all DL signals/channels on this symbol? If so, this is the solution we can understand. But the Alt should be modified to: - Transform precoding is applied to a set ofall DL signals/channels multiplexed on a set of subcarriers prior to the transform precoding - Alt3: We understand this the FDM version of Alt2. But similar to Alt.1, it is not single-carrier in DL. We do not think this Alt should be considered. - For Application of the transform precoding - Alt1: It works. We suggest to focus on this Alt. - Alt2: We want to clarify: On the DMRS symbol, are there also data REs? Is the DMRS symbol applied with transform precoding? We guess the DMRS symbol is DMRS only, and applied with transform procoding, similar to LTE UL. Is our understanding correct? |
| Xiaomi | We are supporting of the MUX proposal. For the target channel part, we believe the common PDSCH or unicast PDSCH is the most relevant target channels. We are supportive of the MUX part in general by revising Alt 2 since if the multiplexing is before DFT, actually all the channels will be allocated resources within a DFT-s-OFDM symbol. It’s not correct to say subcarriers. For Alt 3, does it mean there will be multiple DFT transform precoders each of which can multiplex multiple DL transmissions prior to DFT operation? - Application of the transform precoding - Alt1: Before multiplexing with other DL signals/channels, i.e.: - Transform precoding is applied to each DL signal/channel among the set of signals/channels to be multiplexed for transmission in the same set of OFDM symbols - DFT size is the same as the number of assigned subcarriers for the DL signal/channel - Alt2: After multiplexing with other DL signals/channels, i.e.: - Transform precoding is applied to a set of DL signals/channels multiplexed on a set of resources subcarriers prior to the transform precoding - DFT size is the sum of assigned resources subcarriers for the DL signals/channels being transmitted - Alt3: After multiplexing with a subset of other DL signals/channels. I.e. more than one set of signals/channels multiplexed as in Alt2 can be frequency-multiplexed. |
| InterDigital | Assuming we continue discussion of necessity of DL DFT-s-OFDM in the waveform agenda, use cases for the first two bullets shall be treated first. The third bullet may depend on the progress of initial access discussion. The fourth and fifth bullet depend on the progress of Enery efficiency discussion in 11.5. |
| NEC | We support the proposal and need to align on these mentioned aspects |
| Sony | - Target channels/signals for transform precoding - We agree with the list – except for [on-demand]Sync. We prefer multiplexing with CP-OFDM SSBs for example. - Application of the transform precoding - Our preference is Alt1 – this will ease specification & UE complexity. In Alt2 and Alt3, the UE will need to deal with a bigger IDFT size than is necessary. - Multiplexing of DMRS and PDCCH/PDSCH - We think there are Alt3 that allow FDMA of PDCCH/PDSCH and DMRS: - Alt3a: transform precoding is applied to the PDCCH/PDSCH REs including the DMRS Res - Alt3b: transform precoding is applied to the PDCCH/PDSCH REs excluding the DMRS Res. The DMRS are FDMA multiplexed to designaled REs of the same OFDM symbol prior to IFFT. |
| QC | We also would like to know if a UE is expected to simultaneously receive/process two different waveform types (CP-OFDM and DFT-S-OFDM, say). We would also like to know if this will be limited to single-layer transmission. |
| LGE | In this meeting, a number of companies showed that the clear PAPR reduction even though Alt 1 is considered. Regarding the proposal, it would be good to have the proposal for the KPI as follows: - For downlink low-PAPR proposals the primary evaluation criterion is - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - Network energy saving gain relative to baseline for BS - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding |
| LGE | In this meeting, a number of companies showed that the clear PAPR reduction even though Alt 1 is considered. Regarding the proposal, it would be good to have the proposal for the KPI as follows: - For downlink low-PAPR proposals the primary evaluation criterion is - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - Network energy saving gain relative to baseline for BS - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding |
| Ericsson | If we agree to deprioritize DFT-s-OFDM for downlink now, then we do not need to discussion the topics mentioned in Question 2 for now. |
| Apple | There is a need to understand the motivation before selecting the target channels. |
| Samsung | If the study of DL DFT-s-OFDM waveform is deprioritized, there is no need to discuss about this question. |
| Lenovo | We are fine with the proposal |
| Spreadtrum | For application of transform precoding: in order to avoid introducing high complexity at the UE side: we prefer Alt1 or conditionally support Alt2/3 under limited bandwidth conditions For multiplexing of DMRS: it is ok to study both. The benefites and impacts of Alt 2 should be clarified. |
| Lekha | Considering, we continue the DFT-s-OFDM study, the first two bullets among the target channel options can be handled first. |

## 8.2	UL PAPR reduction

Agreement of RAN1#122bis - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding

PA models before receiving further feedback from RAN4:

- MP model, GMP model, GMP model with LASSO and WMP model [Spreadtrum]

- Normalized memoryless PA model defined in RAN4 [vivo]

- PA model in R1-166004/R4-164542 [Huawei]

- PA model in R4-163314 [ZTE] (FL note: this the same model as in the LS R1-166004)

- PA model [TR 38.803 V14.4.0] [vivo]

| Model | Parameter |
| --- | --- |
| yn= k∈Kpakx(n)x(n)2k | ak; k∈[0,1,…,7]: [-0.618347-0.785905i; 2.0831-1.69506i; -14.7229+16.8335i; 61.6423-76.9171i; -145.139+184.765i; 190.61-239.371i; -130.184+158.957i; 36.0047-42.5192i] |

Question 3: Discuss whether there is a need to agree on a single PA model before receiving further guidance from RAN4, and if so, agree on the model(s) to use

Evaluation metrics

- Net gain assuming similar spectral efficiency and bandwidth for each compared method [Nokia]

- PAPR reduction and SINR degradation @10% BLER, Net gain including MPR can be evaluated by RAN4. [OPPO]

- Implementation complexity in receiver. [CATT]

- ACLR, EVM, IBE, SEM [NEC, Huawei]

- CM characteristic [Lenovo]

Question 4: Discuss the evaluation metrics

Evaluation assumptions

Question 5: Discuss the evaluation assumptions, taking the Huawei’s proposed set in tables 1-3 below as the basis of discussion

Table 1 General evaluation assumptions [Huawei]

|  | 3GPP Rel-18 NR For reference only | 3GPP 6GR |
| --- | --- | --- |
| Carrier frequency and scenario | 4GHz (Urban), 28GHz (Urban) 700MHz (Rural), | 4GHz, 7GHz |
| Channel BW | 100MHz for Urban, 20MHz for Rural, | Up to 400MHz for Urban (7GHz), Up to 200MHz for Urban (4GHz), |
| SCS | 30 kHz (4GHz), 120 kHz (28GHz) 15 kHz (700 MHz), | 30 kHz (4GHz), 30KHz (7GHz) |
| Channel model | TDL-C 300ns for FR1 Urban (4GHz), TDL-A 30ns for FR2 Urban (28GHz), TDL-D 30ns for Rural | TDL-C 300ns, Optional CDL-C 300ns |
| UE speed | 3km/h | 3km/h |
| Number of Tx antennas for TDL channel | 1, Optional: 2 | 1, Optional: 2 |
| Number of Rx antennas for TDL channel | 4 for FR1 Urban, 2 or 4 for FR1 Rural, | 4 |
| Number of Tx antennas for CDL channel | NA | 1, or any other value up to 16 for 7GHz, and up to 4 for 4GHz |
| Number of Rx antennas for CDL channel | NA | 4GHz: 32,64,128 7GHz: 32,64,128, 256, 512 |
| Rank per UE | 1 | 1 |
| Number of DMRS symbols | 2 | 2 |
| Number of PUSCH data symbols | 12 | 12 |
| HARQ configuration | No retransmissions | No retransmissions |
| Frequency hopping | Disabled | Disabled |

Table 2 Single user evaluation assumption for MCS and PRB# [Huawei]

| No Spectrum Extension |  | With Spectrum Extension |  |  |
| --- | --- | --- | --- | --- |
| MCS | #RBs | #RB before extension (A) | #RB after extension (B) | Spectrum extension factor α=(B-A)/B |
| NR MCS | B | (1-α)B | B | α∈14,13,38,25,716,12 |

Table 3 link level multi user evaluation assumption [Huawei]

| Parameters | Values |
| --- | --- |
| User # | 2 or 3 |
| Per user configuration | With spectrum extension provided in Table 2 |
| Multi-user overlap RBs | Reported by companies |
| Baseline scheme | No spectrum extension |
| Per UE MCS | Chosen to align with the spectral efficiency |
| BS Rx Port Virtualization | Reduce to 1 port, e.g., by DFT vector |

### 8.2.1	Company comments

Provide your views on question 3 above

| Company | Comment |
| --- | --- |
| CMCC | For clearer comparison, a single PA model is recommended for the evaluation of the UL PAPR reduction techniques. |
| vivo | If we can agree on a single PA, that is good. But even if we cannot, we can simply list the options and let companies to report. In RAN4 study of previous releases, they don’t mandate to use a single PA either. In RAN4, any PA model is valid as long as it can output a certain power at a certain modulation order. |
| OPPO | It is desired to have a single PA model. |
| Xiaomi | Given the LS sent in previous meeting, we think for PA part we need to hold and wait for RAN4 reply to avoid duplicated evaluation. If in the end we need to use RAN4 PA, it seems unnecessary to perform evaluation based on RAN1 assumed PA firstly? |
| InterDigital | It will be desirable to agree on the common PA model. If it is not agreeable, companies can describe the PA model implemented in their simulation. |
| NEC | We think that agreeing on any PA assumption before refceiving RAN4 guidance can result in conflicts in future. For now it may be preferable to allow companies to select their PA assumptions and state them clearly in their TDocs as well. |
| Sony | We think we need a single PA model to be used by all companies in evlauting UL PAPR schemes. |
| QC | Companies in RAN1 can proceed with their own realistic PA models. No need to prematurely enforce a PA model before we receive input from RAN4. Accuracy of polynomial models in the deep saturation region is a concern. |
| LGE | We think Companies can provide any PA model for their evaluation before agreeing PA model for evaluation. |
| Ericsson | There exist different PA models depending on the frequency range in TR 38.803, e.g., separate memory polynomial models are mentioned for below 6 GHz and above 6 GHz. If we agree in Question 5 different frequencies for evaluations, then we may need to consider this aspect as well. Having said that, we do not need to agree one PA model at this point, rather companies can report the PA model used in their evaluations. |
| Apple | Agreeing on a single PA model will be difficult and will depend on the goal of the waveform. As an example, in our contribution, we use the Saleh model that allows for modeling of high efficiency PA which is appropriate given that it enables power boosting with constant envelope waveform approximation. A.A.M. Saleh, “Frequency-independent and frequency-dependent nonlinear models of TWT amplifiers,” IEEE Trans. Communications, vol. COM-29, pp.1715-1720, November 1981 |
| Samsung | Before receiving guidance from RAN4, performance evaluation may be conducted using either (1) a PA model derived from TR 38.803, or (2) an in-house PA model. If so, the proponents should provide as much information as possible regarding PA model used. At this stage, there is no need to spend excessive effort on agreeing on a single PA model. |
| Lenovo | Agree with Xiaomi. To avoid duplicating discussion effort, it is better to wait for RAN4 reply on LS |
| NTT DOCOMO | At least for sub-6GHz, a single PA model could be agreed. • The PA model in the LS R1-166004 could be used for sub-6GHz |
| Lekha | A single PA model will be beneficial. In case no consensus is reached, we can agree on a list or wait for RAN4 reply on LS. |

Provide your views on question 4 above

| Company | Comment |
| --- | --- |
| CMCC | The related detailed evaluation(s) will anyway be done in RAN4 for the conclusion. In RAN1, we can focus on the metric of “net gain”, and try to down-select a few candidates for the following RAN4 evaluations. |
| vivo | We have agreed net gain last meeting. It seems sufficient to use it. |
| OPPO | If RAN1 will evaluate net gain including MPR, RAN1 needs to clarify the evaluation metholodgy for MPR, e.g., the combination of starting PRB and number of PRB. That will be a “simplified” MPR evaluation. We do not think it is feasible to do the same MPR evaluation as RAN4 does. But how to simplifying it needs a RAN1 agreement. |
| Xiaomi | For the evaluation metric agreed in the previous meeting, our understanding is RAN1 will perform the overall net gain assessment taking into account RAN4 PA and RF characteristics. We can also accept OPPO’s clarification given RAN4 expertise on MPR and RF related calculations. |
| InterDigital | Same view as vivo. We don’t need to discuss the details of the metric further as powe difference will be based on RAN4 requirements. PAPR or CM should not be used to evaluate the net gain. The first bullet can be used a note when performing the net gain evaluation. |
| NEC | From our perspective, we think that along with Net gain, the companies can report any impact on ACLR, EVM characteristics, which will be crucial to provide an overall understanding of the evaluation scheme. |
| Sony | For this study, we support OPPO’s proposal for the RAN1 study to only evaluate: PAPR reduction and SINR degradation @10% BLER and leave evaluation of Net gain including MPR to RAN4. |
| QC | Same view as Nokia. Use net gain assuming similar spec eff. and bandwidth for each method of interest while also complying with RAN4 constraints on ACLR, EVM, IBE, and SEM. We also need to be mindful of complexity on the transmitter and the receiver. Discourage using PAPR/cubic metric. More importantly proponents need to present a Spec Eff vs SNR+OBO curve to assess the benefit of any scheme across a range of spectral efficiencies of interest. |
| LGE | We think Companies can provide Net Gain based on ‘PAPR reduction and SINR degradation @1% BLER’. |
| Ericsson | We think it is more natural to refine the evaluation methodology agree in the last meeting by clarifying how different listed metrics come to into picture. For instance, the following updates can be considered. Agreement of RAN1#122bis - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency and same bandwidth as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used for MPR or power backoff computation - When calculating the Tx power gain, the RAN4 metrics such as ACLR, EVM, IBE, SEM on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics Additionally, PAPR can’t be used as a metric given that NetGain is already accurate than that as mentioned also by QC. |
| Apple | We think that the Net Gain agreed on in the last meeting should be used as a baseline. Our results show that using the PAPR may result in an entirely different set of conclusions. The use of RAN4 metrics such as ACLR should be taken into account as mentioned by Ericsson. |
| Samsung | Net gain was agreed as the link level performance evaluation metric at the last meeting. When calculating net gain, non-ideal channel estimation and equalization, and PA model are inherently taken into account. In addition, performance evaluation already considers RAN4 requirements such as ACLR and IBE, as well as Tx power gain. Therefore, there is little need to further diversify the performance evaluation metrics. |
| NTT DOCOMO | Agree on the view that net gain should be evaluated based on the assumption of similar spectral efficiency and bandwidth for each compared method The use of RAN4 metrics such as ACLR, EVM, and OBW should be considered for MPR evaluation |
| Lekha | Along with Net Gain, some primary analysis on EVM and ACLR will be good before RAN4 takes over. |

Provide your views on question 5 above

| Company | Comment |
| --- | --- |
| CMCC | Regarding the assumption on “Channel BW”, “400 MHz” can be an optional candidate, with “100 MHz” being added as mandatory. |
| OPPO | As comments to Question 4, we need to define how MPR is evaluated in RAN1. Same as RAN4 assumption? Or a simplified one. |
| Xiaomi | We prefer focusing on single user case firstly, i.e. table 1 and table 2. For the UE Tx Antenna in Table 1, for CDL Channel, we don’t understand why “up to 16 antennas are used”. Does this mean antenna element or antenna ports? From our understanding, 1T or 2T can be considered for evaluation. Number of Tx antennas for CDL channel 1, or any other value up to 16 for 7GHz, and up to 4 for 4GHz |
| vivo | For number of Tx antennas, we should be more careful. Considering commercial UEs and future UE implementation, either we do not go beyond 2Tx or we make 1Tx and 2Tx as baseline, for both 4GHz and 7GHz. |
| QC | On Table 1: Number of rx antennas at the gNB for TDL and CDL need some reconciliation. The current numbers are very different. On Table 2: For QPSK + spectrum extension, please also add 1/6 & 2/7. --- Truncation factors for pi/2 BPSK are missing. We will need to add an extra column or a table. If adding a separate table: Can we add truncation factors of 10%, 20%, 30%, and 40%. Truncation factor here is defined as: Tones before truncation = A Tones after truncation = B Truncation factor = (A-B)/A. We prefer to use tones rather than RB to not preclude other enhancements that may make spectrum extension and truncation techniques more broadly accessible to a scheduler. |
| Ericsson | It would be good to agree on the set of values for B in combination with α in Table 2 so that the results can be comparable. Also, in Table 3, baseline scheme shall be when there is no FDSS and no spectral extension. |
| Apple | Additional assumptions such as the following should be taken into account: - DMRS symbol type - ACLR requirements - Resource allocation 24 RBs: inner RB allocation, edge RB allocation |
| Samsung | For efficient performance evaluation, it is necessary to avoid uncesarily expanding the evaluation environment. Introducing new conditions beyond those used in NR, i.e., CDL channel models, should be considered only when a clear necessity is demonstrated. |
| Lenovo | BW of 400MHz @30kHz would require large FFT size, we think it should be an optional candidate. |
| NTT DOCOMO | Except for 4GHz and 7GHz, the carrier frequency with 15GHz for FR3 and 28GHz for FR2 should also be evaluated. • The evaluation assumption in 3GPP Rel-18 for 28GHz could be reused • The evaluation assumption for 15GHz based on agreements in agenda 11.2 should be used as the reference The evaluation for a single user should be considered firstly. |
| Lekha | We suggest adding ACLR requirements. |

## 8.3	Other waveforms

Evaluation assumptions

Question 6: Discuss the evaluation assumptions

- Select simulation scenarios from the scenarios agreed in 11.2

- 700MHz Rural, 4 GHz and 7 GHz Urban Macro, 30 GHz Dense Urban

Reference comparison

Question 7: Discuss the reference against which to compare the proposal

- Select simulation scenarios from the scenarios agreed in 11.2

- MIMO up to 4 layers in DL, up to 2 layers in UL

- …

Proposal characterization

Question 8: Agree on a template that can be used to capture each proposal, potentially for inclusion to the TR, template below as the starting point

|  | Description |
| --- | --- |
| Name of the proposal |  |
| Applicable link direction | DL/UL/both |
| Enhancement to CP-OFDM? | No/Yes |
| Enhancement to DFT-s-OFDM? | No/Yes |
| Additional OFDM-compatible waveform? | No/Yes |
| Target channel(s) | PDCCH/PDSCH/PUCCH/PUSCH/xxx |
| Target modulation |  |
| Motivation / use case | Improved spectral efficiency, … |
| Key Metric / KPI | Spectral efficiency, … |
| Key spec impact foreseen |  |
| MRSS compatibility | Please explain |
| Multiplexing/coexistence with other waveforms | Please explain |
| Multi-user multiplexing | Please explain |
| MIMO compatibility | Please explain |

### 8.3.1	Company comments

Provide your views on question 6 above

| Company | Comment |
| --- | --- |
| CMCC | The evaluation assumptions for the study of UL PAPR reduction and multi-layer DFT-s-OFDM can be reused or referred to for the evaluations of non-OFDM waveform. |
| OPPO | We are open to discuss the evaluation assumptions. But before that, we need to check whether a proposal has provided the self-evaluation following what we ageed in RAN1#122. Only the proposal provided with self-evaluation following ageements in RAN1#122 can be studied by other companies. |
| IMU | We are flexible and open to any discussion regarding the evaluation assumptions. However, for 6GR, we believe evaluation assumptions should go beyond conventional parameters. Since 6GR serves diverse technologies and requirements, the template should explicitly include: (i) target application (e.g., ISAC, NTN, AI, WuS/amb-IoTss), (ii) target requirement (e.g., latency, efficiency, reliability), and (iii) target channel condition (e.g., delay spread, mobility, RF impairments). This provides a fairer and more use-case-oriented evaluation. |
| ETRI | The potential outcomes from Question 5 can be applied to other waveforms. Other than PAPR reduction, we think that spectral efficiency and coverage enhancements can be evaluated based on the outcomes of Section 11.2 (e.g., using the link-budget template) for all NR and other waveforms. |
| Samsung | Deviating from “single technology framework” for 6GR study, the study for the potential support of “other waveforms” in 6GR should be deprioritized. If the study of other waveforms is deprioritized, there is no need to discuss about this question. |
| Cohere | The evaluation should start with LLS and include TDL covering different channel conditions, multiple bands including 7 GHz, 15 GHz, 30 GHz, and UE speeds up to at least 500 km/h. For UL the evaluation should take into account the PAPR. |
| NTT DOCOMO | The same view as CMCC. |
| Lekha | Same view as OPPO |

Provide your views on question 7 above

| Company | Comment |
| --- | --- |
| CMCC | Similar to our comment to question 6, the evaluation assumptions for the study of UL PAPR reduction and multi-layer DFT-s-OFDM can be reused or referred to for the evaluations of non-OFDM waveform. |
| OPPO | Open to study. But proponent should first clarify the use case they focus on. |
| IMU | We are open to discussion taking into consideration the use cases mentioned in our comment to question 6. |
| ETRI | The potential outcomes from Question 5 can be applied to other waveforms. Other than PAPR reduction, we think that spectral efficiency and coverage enhancements can be evaluated based on the outcomes of Section 11.2 (e.g., using the link-budget template) for all NR and other waveforms. |
| Cohere | The reference should be CP-OFDM under the same agreed channel conditions. We are OK with MIMO of up to 4 layers in DL and up to 2 layers in UL |
| NTT DOCOMO | The same view as CMCC. |
| Lekha | Same view as OPPO |

Provide your views on question 8 above

| Company | Comment |
| --- | --- |
| CMCC | This direction can help companies understand each proposed design for the following detailed study. |
| OPPO | This a good template helping us to understand the proposals, and determine what proposals are qualified for cross-evaluation. |
| IMU | Since 6GR is expected to support a wide range of technologies and applications, each with distinct requirements and operating environments, we suggest expanding the evaluation template to explicitly capture these dimensions. For example, the following categories could be included: - Target technology/application (e.g., sensing, TN-NTN, AI-native operation, WuS/Ambient IoT, etc.) - Target requirements/capabilities (e.g., latency, spectral efficiency, energy efficiency, connectivity, reliability, etc.) - Target channel conditions (e.g., RMS delay spread, mobility profile, bandwidth, RF impairments, etc.) This would allow a more accurate and use-case-oriented assessment of potential 6GR waveform designs. |
| ETRI | Agreed. This template can serve as a valuable reference for other companies to better understand and further investigate the proposed waveform. |
| Cohere | The proposed list is a good start. The motivation/use case should include ISAC, TN/NTN, AI-native waveform) |
| NICT | We agree with including “Multiplexing/coexistence with other waveforms” especially with those for PAPR reduction since it attracts the biggest concern of the waveform agenda. Our contribution R1-2509396, which is not captured eventually in this FL summary, is about a waveform that can coexist with FDSS-SE, etc. |
| NTT DOCOMO | Agree. The template looks good. |
| Lekha | Agree with the template. |

## 8.4	Rank > 1 for UL DFT-s-OFDM

Evaluation assumptions

Question 9: Discuss the evaluation assumptions

- Select simulation scenarios from the scenarios agreed in 11.2

- 4 GHz, 7 GHz, 28 GHz Urban.

Evaluation methodology

Question 10: Discuss the evaluation methodology

- Net gain:

  - For power limited UEs:  Tx power difference [dB] – Rx SINR difference for 10% BLER [dB]

  - For power non-limited UEs: Rx SINR difference for 10% BLER [dB]

- MPR for multi-layer DFT-s-OFDM to be determined with link level simulations

- Performance benefit to be evaluated using system level simulation

Proposal 18: RAN1 to consider the following system level configuration for multi-layer UL waveforms study:

| Parameters | Values |
| --- | --- |
| System configuration | Uma, 21 cells, 10 UEs/ cell and up to 50 UEs/cell |
| Traffic model | Full buffer, [FTP model TBD in AI 11.2] |
| Carrier frequency | 3.5GHz, 7GHz |
| Channel bandwidth/Subcarrier spacing | Up to 100 MHz, 30KHz |
| Frame structure | TDD (DDDDU) |
| UE ports | Up to 8 |
| BS antenna configuration/ports | Baseline configuration in AI 11.2 |
| UE antenna model | Isotropic |
| ISD | 500 |
| Maximum Number of layers (maxRank) per UE | Adaptive rank, maxRank 2 SU-MIMO |
| Receiver | LMMSE |
| Waveform and MIMO configuration | 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies |
| Power class and power mode | Power class 2, Full power mode |
| UL Maximum power | - Static assumption per waveform for all modulations and number of layers with average power difference between waveforms of 1 or 2 dB - [optional] MPR based Pcmax according to modulation/FDRA/waveform/coherency/etc. |

Proposal 19: RAN1 to consider the following link level configuration for multi-layer UL waveforms study:

| Parameters | Values |
| --- | --- |
| Carrier frequency​​ | 3.5GHz, 7GHz​​ |
| Subcarrier spacing​​ | 30kHz​​ |
| UE/BS antenna ports | 2 or 4 / 32 |
| FDRA | 64, and 256 RB or 270 RB​​ |
| Waveform​​s | CP-OFDM DFT-s-OFDM​​ |
| MCS/modulation​​ | [4,9,10,16,17,23] QPSK/16QAM/64QAM​​ MCS Table 1 |
| Number of layers​ | 2 layers​​ |
| Channel model​​ | CDL-A30​​ |
| UE speed​​ | 3km/h​​ |
| Channel est.​​ | Practical​ |
| Precoding matrix​ | Adaptive based on channel |
| HARQ retransmission​ | disabled |
| DMRS configuration | Configuration type 1 2 DMRS symbols per slot |
| Number of PUSCH data | 12 symbols |
| Waveform and MIMO configuration | 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies |
| BLER target | 10% |

Reference comparison

Question 11: Discuss the reference against which to compare the proposal

- 5G NR UL MIMO codebooks, frequency selective precoding

- CP-OFDM for all number of layers & DFT-s-OFDM for single layer

- Fully coherent UE coherence capability

- DWS rel-18 enabled [disabled optional]

### 8.4.1	Company comments

Provide your views on question 9 above

| Company | Comment |
| --- | --- |
| CMCC | Can you clarify the relationship between question 9 and 10? |
| OPPO | We think the evaluation assumption should be similar to that in Section 4. |
| Samsung | If required, the 4 GHz Urban should be prioritized. |
| CATT | We are open for this. |
| NTT DOCOMO | 15GHz should also be considered. |
| Lekha | We are open to this. |

Provide your views on question 10 above

| Company | Comment |
| --- | --- |
| CMCC | The direction looks OK. |
| vivo | We can still use net gain as the metric here. We don’t need to distinguish power limited UEs or non-power limited UEs. We can generally use net gain. Our understanding of having UL DFT-s-OFDM for rank > 1 is to improve the data rate for power limited UEs, otherwise we can simply use CP-OFDM. Hence non-power limited UEs are not the target here. Even we include the evaluation for these non-power limited UEs, we can still use net gain. It is just the gain of MPR reduction is gone, and the final net gain is smaller for these UEs. |
| OPPO | We think the evaluation assumption should be similar to that in Section 4. |
| LGE | Bandwidth We think the evalution assumption needs to be aligned with Table 1 General evaluation assumptions [Huawei] in Question 5. Traffic model We think Full buffer model is not realistic. FTP model should be assumed for the evaluation. |
| Ericsson | For Power class and power mode: for better clarity and to avoid confusions, we would like to propose to explicitly mention the following for power class and power scaling in evaluation assumptions for 2TX configuration as this is where the optimal benefits and the best use case for multi-layer DFT-s-OFDM: - PC2 and 23 dBm PA with Rel-15 power scaling - Total power is limited up to 26 dBm, each PA supports up to 23 dBm Additionally, one could consider the following as well: - PC3 and 23 dBm PA with Rel-16 MODE0 power scaling - Total power is limited up to 23 dBm, each PA supports up to 23 dBm Also, for UL max power: if we use “Static assumption per waveform for all modulations and number of layers with average power difference between waveforms of 1 or 2 dB”, then the results may be skewed due to too much simplification, depending on the value of the power difference applied blanketely for all UEs. For instance, in the case of 20 MHz full band allocation, the power difference between CP- and DFT-s-OFDM waveforms for QPSK, 16-QAM and 64-QAM are in the range of 2.4 dB, 1.9 dB, and 1.7 dB, which can be considered as a starting point. It would be more appropriate to use a subset of values as function of modulation and/or inner/outer RB allocation, in order to reflect the practical settings, rather than a single value. For waveform and MIMO configuration: For fully and partial coherent codebooks, PAPR depends on the selected precoder (i.e., TPMI). Thus, we should clarify how to determine the PA backoff depending on the precoder selection. For Number of layers: it is also premature to limit the settings to only rank-2 in the evaluations and hence we prefer to remove that constraint at present. Since 4 Tx power class 1.5 UEs may have 1/4th of the total power per Tx chain, there is strong potential for >2 layers, and it is not appropriate at this stage of the study to limit DFT-S-OFDM to 2 layers. |
| Samsung | For the system level configuration, FTP model 1, which is a realistic traffic model, should be considered. In addition, in the table from proposal 18, the Full Buffer traffic model should be removed. Link level performance evaluation could be considered as optional. |
| Panasonic | What PA assumption should be discussed. In order to support MIMO of CP-OFDM with the total of 23 dBm, one way is 20 dB + 20 dB amplifier for rank 2 and SFN gain of 23 dB in rank 1. The other is two 23 dB amplifiers and MIMO is reduced to 20 dB for each. In the second implementation, DFT-S-OFDM with multi-layer would have the gain as unused power of rank 1 is used. |
| CATT | Apart from net gain, the real implementation complexity in receiver on gNB side, e.g., complex joint FD equalization and MIMO decoder should be considered. When DFT-s-OFDM with number of layers > 1 is implemented, the complexity of the receiver will increase dramatically. Such complexity should be avoided |
| InterDigital | We have several comments regarding SLS and LLS evaluation assumptions. - Link level evaluation (e.g., net gain in the first bullet) should be deprioritized as system performance is more relevant KPI for multi-layer UL DFT-s-OFDM. Link level evaluating performance (e.g., BLER) of multi-layer UL DFT-s-OFDM does not show how power limited UEs affect to the system performance. - For the number of ports at UE, can we follow the decision made in Agenda 11.2. UE antenna modeling is currently being discussed in Agenda 11.2 - For UE antenna modeling, co-polarized antenna model shall be included in the evaluatinon assumption. In addition, Rel-19 handheld UE model described in 7.3.2 in TR 38.901 should be taken into account. - Regarding UL Maximum power, it should be determined dynamically based on at least number of allocated resource blocks, pathloss measurement, modulation, inner/outer allocation. According to our understanding, maximum UE transmission power is determined based on MPR (which depends on modulation and waveform), power control parameters and number of scheduled resource blocks. - Companies shall disclose transmission power for UEs in SLS which is associated with allocated number of resource blocks, waveform (OFDM or DFT-s-OFDM) and rank. We would like to know how many UEs were power limited in SLS conducted by companies. This will indicate how transmission power is determined in system level simulations. |
| NTT DOCOMO | Agree. The direction looks OK. |
| QC | We would like to get clarification on the second bullet on determination of MPR based on link-level simulations, suggest removing it. For system level simulation configurations to evaluate multi-layer DFT-s-OFDM - For UE ports, we suggest up to 4 for handheld devices and up to 8 for CPE - For max number of layers per UE, we agree with Ericsson that limiting max rank to 2 is premature at this stage. As pointed out, a 4 Tx PC1.5 (29dBm) device powered by four quarter power 23 dBm PAs may stand to benefit from DFT-s-OFDM with rank > 2. - For waveform and MIMO configuration, since non-coherent precoding is widely prevalent and offers less complexity and reduced implementation demands on UEs, this should be considered as baseline configuration. - For power class and power mode, assumptions on the power scaling and per PA power should be specified. We agree with Ericsson that for 2Tx UE, it is reasonable to assume 2 half-power PAs as this reflects common implementation in practice. For link level simulation configurations to evaluate multi-layer DFT-s-OFDM - For number of BS antenna ports, 64 and 128 should be considered. - For FDRA, 8 RBs, 16 RBs and 32 RBs should also be added to reflect allocations to cell-edge UEs. - For MCS/modulation, should be left open to any valid MCS in 5G NR, both 64QAM and 256QAM tables should be considered. - For number of layers, both 2 layers and 4 layers should be studied. - For channel model, TDL-C 300ns DS should be added. For waveform and MIMO configuration, since non-coherent precoding is widely prevalent and offers less complexity and reduced implementation demands on UEs, this should be considered as baseline configuration. |

Provide your views on question 11 above

| Company | Comment |
| --- | --- |
| CMCC | As part of the evaluation assumptions, the 2nd and 3rd bullet can be discussed in question 10. The 4th bullet may also be suitable to be discussed in question 10, if it means that Rel-18 DWS is assumed to be disabled in the system level evaluations. |
| vivo | The correct baseline should be the second, i.e., CP-OFDM for all number of layers & DFT-s-OFDM for single layer. For the first one, this is not a technique in the current specification. Hence it is not a baseline in our understanding. Whether to support this should be a separate discussion. The third is a configuration or a condition in this study. We can study whether UL DFT-s-OFDM is only applicable to non-coherent CB, or it should be applicable to coherent CB as well. But whether to use non-coherent or coherent CB shouldn’t be the baseline scheme. We are not clear what the fourth is. |
| OPPO | We can use the second baseline as the starting point. |
| Samsung | The second option, aligned with 5G NR operation, should be used as the reference for performance evaluation. |
| CATT | The DFT-s-OFDM=1 and CP-OFDM >1 can be switched by a dynamic indication for coverage enhancement. Thus, to study whether to support DFT-s-OFDM for UL with number of layers > 1, the potential benefits of DFT-s-OFDM with number of layers > 1 should be compared with CP-OFDM with number of layers > 1 based on the link level simulation |
| InterDigital | At least the second bullet shall be used as the baseline. |
| NTT DOCOMO | The second bullet should be used as the baseline. |
| QC | We agree with vivo, OPPO, Samsung, Interdigital and NTT Docomo that the 2nd bullet should be used as a baseline to compare against i.e. CP-OFDM for all number of layers & DFT-s-OFDM for single layer as specified in 5G. Non-coherent uplink precoding should be considered as baseline since this is widely prevalent, offering less complexity and reduced implementation demands on UEs. It is important to note that schemes that offer better performance via additional uplink transmit power are more robust than those that offer equivalent performance while relying on beamforming gains as beamforming gains may not be consistently observed and can be highly dependent on aspects such as SRS configuration / periodicity, UE antenna architectures, etc. |

# 9.	Monday on-line

TBA