3GPP TSG-RAN WG1 #123	<No TDoc Number>

Dallas, USA, November 17 – 21, 2025

Source:	Moderator (Nokia)

Title:	Collection of waveform observations and proposals

Document for:	Discussion

Agenda item:	11.3.1

Work Item:	FS_6G_Radio

# 1.	Introduction

This document collects all the observations and proposals of the RAN1#123 Tdocs submitted to the agenda item 11.3.1 Waveform for 6G. 44 Tdocs were submitted (including one moved from AI 11.1 and one submitted to AI 11.3) with a total of 530 pages, of which consist of 287 observations and 285 proposals spanning 41 pages.

# 2.	Contributions

|  | Tdoc# | Title | Source |
| --- | --- | --- | --- |
| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| [2] | R1-2508367 | Considerations for additional 6GR DL waveform | Kyocera Corporation |
| [3] | R1-2508387 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
| [7] | R1-2508471 | Discussion on waveform for 6GR air interface | THALES |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
| [9] | R1-2508625 | Waveform for 6GR Air Interface | Shanghai Jiao Tong University. |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
| [13] | R1-2508684 | Discusson on 6GR Waveform | Xiaomi |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
| [17] | R1-2508833 | Waveform for 6G Radio Air Interface | IMU |
| [18] | R1-2508844 | Waveform for 6GR air interface | BJTU |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
| [21] | R1-2508863 | Support for Diverse Waveforms on the 6GR Air Interface | National Spectrum Consortium |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
| [23] | R1-2508890 | Waveform considerations for 6G Uplink | Wisig, IITH |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
| [26] | R1-2508973 | Discussion on 6GR waveform | ETRI, University of Surrey |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
| [28] | R1-2509049 | Discussion on waveform for 6GR air interface | Ruijie Networks Co. Ltd |
| [29] | R1-2509059 | Waveform for 6G NR | Cohere Technologies, IIT Delhi |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
| [33] | R1-2509143 | Waveform for 6GR air interface | MediaTek Inc. |
| [34] | R1-2509231 | Waveforms for 6GR | Qualcomm Incorporated |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
| [36] | R1-2509282 | Discussion on Waveform | NTT DOCOMO, INC. |
| [37] | R1-2509303 | Discussion on DL DFT-s-OFDM for 6GR | LG Electronics, CATT, Thales, Lenovo, IITH, WiSig |
| [38] | R1-2509322 | Study on waveform for 6GR | Sharp |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
| [40] | R1-2509368 | Discussion on Waveforms of 6GR Air Interface | Rakuten Mobile, Inc |
| [41] | R1-2509372 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
| [43] | R1-2509410 | Waveform design for 6GR air interface | Tejas Network Limited |
| [44] | R1-2509413 | OSDM for 6GR | University of Sheffield |

# 3.	Observations and proposals

| [1] | R1-2508336 | Waveform for 6G Radio Air Interface | Nokia |
| --- | --- | --- | --- |
|  | Baseline communication waveform - Proposal 1: CP-OFDM waveform as defined in 5G NR is supported for communications in 6G downlink. - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink - Proposal 2: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for communications in 6G uplink. - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded as potential additions. On additional waveforms proposed Observation 1: DFT-s-OFDM PAPR advantage (i.e., DL coverage and energy saving potential)) cannot be fully maintained in DL without system performance impact and significant limitations compared to CP-OFDM at least due to: possible frequency domain user allocation and UE multiplexing, channel and signal multiplexing, (non-)contiguous intra/inter band spectrum aggregation techniques, SU/MU-MIMO precoding, and multi-RAT/MRSS support in FR1 sharing same RU. Observation 2: NES gain by DL DFT-s-OFDM alone and DL coverage enhancement for common, control and/or data channels is at best questionable especially in current 6G FRs of interest when compared to CP-OFDM with transparent PAPR reduction techniques, which are already present in deployed base stations. DL DFT-s-OFDM may also compromise NES gain achieved by time domain techniques due to reduced sleep time opportunity due to its higher need for time domain multiplexing. Observation 3: PDSCH throughput and coverage (i.e., net gain) can be also significantly impacted when DFT-s-OFDM is considered due to design limitations (e.g., MIMO precoding, Rx processing, etc.), DFT-s-OFDM inherent performance losses in several channel conditions that are more pronounced with smaller number of UE Rx antennas. Proposal 3: RAN1 to deprioritize DFT-s-OFDM study for DL for the following reasons: - PAPR of DFT-s-OFDM in DL is comparable to DL CP-OFDM with transparent PAPR reduction techniques. - No meaningful pure DFT-s-OFDM NES gain or DL coverage gain in current FRs of interest compared to DL CP-OFDM especially where DFT-s-OFDM system limitations may be tolerated (e.g., low load, etc.). - DL EIRP could be already at the maximum with CP-OFDM in these DFT-s-OFDM potential scenarios, and no DL coverage extension can be foreseen. - DL DFT-s-OFDM may rely more on time domain multiplexing reducing time domain NES gain potential. - Base station and UE baseband complexity with DL DFT-s-OFDM is higher (e.g., DFT, Rx processing, transparent PAPR techniques need to be maintained). - DFT-s-OFDM can have link performance degradation compared to CP-OFDM in different conditions in DL (e.g., low complexity UE Rx, impractical R-ML per RE with DFT-s-OFDM, limited number of UE Rx antennas, etc.), - DFT-s-OFDM consideration in DL may need to impose limitations for minimizing PAPR impact with DFT-s-OFDM to the following: - UE frequency domain resource allocation (e.g., contiguous) - UE frequency domain multiplexing - Multiplexing of different DL physical channels/signals and efficient spectrum use (e.g., no FDM of physical channels using CP-OFDM with channel/signal using DFT-s-OFDM, or no/limited number of FDMed channels using DFT-s-OFDM) - SU/MU-MIMO precoding (e.g., limited number of layers for all UEs per port, wideband/subband precoding) - Efficient multi-RAT/MRSS support in FR1 (e.g., avoid frequency multiplexing of 5G DL CP-OFDM with some 6G DL in same RU) - Efficient RU use and carrier configuration flexibility (e.g., number and possible active carriers, potential of non-contiguous intra-band, inter-band spectrum aggregation, fragmented DL support per RU, restriction for all carriers in RU BS RF BW to use same DL waveform at least in overlapping DL transmission period) Observation 4: Zak-OTFS would be a major change for the current systems even if it may be able to be implemented on top of CP-OFDM waveform, while introducing additional complexity to both network and UE side. Observation 5: Simulation assumptions need to be considered carefully so that performance of any potential proposal is evaluated against practical configurations of the baseline CP-OFDM waveform, and in deployment scenarios identified in study of 6G requirements ongoing in TSG RAN. Observation 6: Zak-OTFS is claimed to provide benefit mainly in propagation conditions that are not typical in real deployments, but in fact CP-OFDM outperforms Zak-OTFS with realistic simulation assumptions and realistic channel estimation. Proposal 4: RAN1 to deprioritize studying Zak-OTFS for the following reasons: - CP-OFDM outperforms Zak-OTFS with realistic simulation assumptions and realistic channel estimation - Zak-OTFS is claimed to provide benefit mainly in propagation conditions that are not typical in real deployments - Zak-OTFS would be a major change for the current systems even if it may be able to be implemented on top of CP-OFDM waveform - Zak-OTFS would introduce additional complexity to both network and UE side Low PAPR for coverage extension Observation 7: Having both extension and truncation based schemes makes it very difficult to find existing MCSs to be able to do fair comparisons between the proposals. Proposal 5: Proposed low PAPR methods are compared using net gains and assuming similar spectral efficiency and bandwidth for each compared method and used baseline. Proposal 6: RAN1 to discuss the methodology for comparison of low PAPR methods. - Proposal 7: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio. Proposal 8: Transparent filtering approach (receiver does not need to be aware of the used filter parameters) is assumed for FDSS and FDSS-SE in 6G Radio. - Proposal 9: High power class should be the baseline for 6G due to significant enhancement in coverage. - Proposal 10: Power boosting features such as the ones specified in NR should be part of the baseline for 6G. Proposal 11: Dynamic waveform switching using DCI is introduced to 6G in the first release UL Multi-layer DFT-s-OFDM Observation 8: MIMO precoding can combine multiple layers and leads to higher PAPR for DFT-s-OFDM especially when considering high maximum rank transmissions and modulation orders. Observation 9: Partially and fully coherent UEs are expected to be more common in 6G period. Proposal 12: Low PAPR waveform study, if any, for multi-layers UL transmission in 6G needs to consider at least fully coherent and partially coherent UEs. Proposal 13: For study of UL multi-layer MIMO enhancement with low PAPR DFT-s-OFDM waveform, the power-limited UEs and power non-limited UEs should be considered, and the net gain should be evaluated as follows: Table 4: MIMO net gain metric for power limited and power non-limited UEs Power limited UE; Power non-limited UEs. MIMO Net gain [dB]; Tx power difference [dB] – Rx SINR difference for 10% BLER [dB]; Rx SINR difference for 10% BLER [dB]. Observation 10: In case of single UL waveform assumption, multi-layer uplink DFT-s-OFDM may lead to system level performance loss in average relative to using CP-OFDM. Observation 11: Multi-layer DFT-s-OFDM may significantly degrades the UL performance of power non-limited UEs compared to using CP-OFDM. Proposal 14: Support CP-OFDM for all number of layers in uplink. Proposal 15: The potential of UL DFT-s-OFDM for more than 1 layer in 6G needs further justification. Proposal 16: For multi-layer DFT-s-OFDM potential study and comparison with CP-OFDM, RAN1 to consider the following metric evaluations: - Cell average/edge SE gain or loss relative to the 5G NR baseline reference - UL multi-layer net gain for power limited UEs and power non-limited UEs according to Table 4 Proposal 17: RAN1 to consider the following baseline reference for multi-layer waveforms study: - 5G NR UL MIMO codebooks - CP-OFDM for all number of layers & DFT-s-OFDM for single layer - Fully coherent UE coherence capability - DWS rel-18 enabled [disabled optional] Proposal 18: RAN1 to consider the following system level configuration for multi-layer UL waveforms study: Parameters; Values. System configuration; Uma, 21 cells, 10 UEs/ cell and up to 50 UEs/cell. Traffic model; Full buffer, [FTP model TBD in AI 11.2]. Carrier frequency; 3.5GHz, 7GHz. Channel bandwidth/Subcarrier spacing; Up to 100 MHz, 30KHz. Frame structure; TDD (DDDDU). UE ports; Up to 8. BS antenna configuration/ports; Baseline configuration in AI 11.2. UE antenna model; Isotropic. ISD; 500. Maximum Number of layers (maxRank) per UE; Adaptive rank, maxRank 2 SU-MIMO. Receiver; LMMSE. Waveform and MIMO configuration; 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies. Power class and power mode; Power class 2, Full power mode. UL Maximum power; - Static assumption per waveform for all modulations and number of layers with average power difference between waveforms of 1 or 2 dB - [optional] MPR based Pcmax according to modulation/FDRA/waveform/coherency/etc.. Proposal 19: RAN1 to consider the following link level configuration for multi-layer UL waveforms study: Parameters; Values. Carrier frequency​​; 3.5GHz, 7GHz​​. Subcarrier spacing​​; 30kHz​​. UE/BS antenna ports; 2 or 4 / 32. FDRA; 64, and 256 RB or 270 RB​​. Waveform​​s; CP-OFDM DFT-s-OFDM​​. MCS/modulation​​; [4,9,10,16,17,23] QPSK/16QAM/64QAM​​ MCS Table 1. Number of layers​; 2 layers​​. Channel model​​; CDL-A30​​. UE speed​​; 3km/h​​. Channel est.​​; Practical​. Precoding matrix​; Adaptive based on channel. HARQ retransmission​; disabled. DMRS configuration; Configuration type 1 2 DMRS symbols per slot. Number of PUSCH data; 12 symbols. Waveform and MIMO configuration; 5G NR Codebook UL MIMO: - fullyAndPartialAndNonCoherent codebook for CP-OFDM (i.e., all TPMIs as defined in NR) - DFT-s-OFDM CB reported by companies. BLER target; 10%. |
| [2] | R1-2508367 | Considerations for additional 6GR DL waveform | Kyocera Corporation |
|  | Proposal 1: A rigorous evaluation of cross-waveform interference and CP-OFDM coexistence with non-orthogonal waveforms in MRSS operations is required when considering the adoption of an additional waveform for 6GR. Proposal 2: RAN1 should evaluate the new waveform’s performance by considering practical scheduler implementations to ensure reliable performance results. Observation: Non-orthogonal waveforms may necessitate advanced UE receiver architectures, which potentially impact the 6GR deployment timeline. Proposal 3: Enhancements to CP-OFDM, including PAPR reduction techniques such as ACE, should be evaluated in the 6GR study. |  |  |
| [3] | R1-2508387 | Discussion on waveform for 6GR | Spreadtrum, UNISOC |
|  | Observation 1: DL DFT-s-OFDM can achieve a PAPR reduction of 2-3dB compared to CP-OFDM waveform. Observation 2: DFT-s-OFDM demonstrates good compatibility with existing CP-OFDM waveform structure. Proposal 1：Support DL DFT-s-OFDM in 6GR for the following use cases and channel: - Target use case: LPWA, NTN - Target channel: PDSCH only Observation 3: The slight loss in spectral efficiency of DFT-s-OFDM is acceptable for its target use cases. Observation 4: The negative impact of DFT-s-OFDM on scheduling flexibility and multi-user multiplexing is manageable for the target use cases. Observation 5: The complexity introduced by DFT-s-OFDM for both gNB and UE is acceptable when balanced against the coverage gains. Observation 6: If DL DFT-OFDM is applied to PDSCH only for NTN and LPWA, the impact on eMBB UE is negligible Proposal 2: Regarding around 7GHz PA model(s), before RAN 4 feedback, the following four options can be considered for RAN1 initial evaluation: MP model, GMP model, GMP model with LASSO and WMP model |  |  |
| [4] | R1-2508394 | Discussion on waveform for 6GR | LG Electronics |
|  | #### 1. Continuity and differentiation from 5GNR system - Proposal 1: For evaluation of DFT-s-OFDM for uplink high-rank transmission in 6GR, the following aspects can be considered: - UL Precoder Design: Evaluate the impact of precoder design on PAPR and link-level performance, considering both single-layer and multi-layer mapping to antenna ports. - MIMO Receiver Type: Compare waveform performance using linear receivers (e.g., MMSE), while noting the limitations of non-linear receivers such as ML for DFT-s-OFDM. - Coverage Performance Metrics: Use metrics such as MCL, MPL, and MIL to assess coverage performance under different waveform and rank configurations. - Proposal 2: Discuss how the UE should be instructed to use a particular waveform for 6GR in uplink. - Option 1: Waveform selection based on transmission rank. - Option 2: Explicit waveform indication via cell-specific configuration, channel-specific, or BWP-specific configuration, including dynamic switching. - Option 3: Waveform selection based on frequency band or usage scenario. - Proposal 3: Discuss whether reference signal design should consider commonality across CP-OFDM and DFT-s-OFDM in both uplink and downlink. #### 2. Enhanced Waveform Techniques for 6GR - Observation 1: Spread OFDM is similar to CP-OFDM in terms of overall structure; however, it introduces an additional operation of DFT spreading. Specifically, modulated symbols mapped to data REs in the frequency domain are first multiplexed in a virtual domain, then DFT-spread, and finally mapped to data REs in the frequency domain. This additional step, similar to DFT precoding in DFT-s-OFDM, introduces extra complexity. - Observation 2: Compared to CP-OFDM transmission over a narrow bandwidth, Spread OFDM achieves higher frequency diversity gain by applying DFT spreading over a wider bandwidth. - Common channels such as PDSCH for System Information, RAR, or Paging, or UE-specific channels such as SPS, Msg3 PUSCH, or Configured Grant. These channels may benefit from frequency diversity gain through spreading techniques. - Proposal 4: Study the benefits of Spread OFDM compared to CP-OFDM and DFT-s-OFDM. - Observation 3: Since NR NTN is introduced in the middle of NR phase, it was not possible to introduce new waveform. - Observation 4: In NR NTN, for coverage enhancement, repetition of physical channels/signals are introduced, but it will require the increased active beam ratio due to the increased occupied resources. In other hands, with the limited active beam ratio or active time of a beam, the usage of the repetition for coverage would be restricted. - Observation 5: Applying DFT-s-OFDM for DL transmissions can be beneficial in the NTN scenario where the LOS is dominant and the total DL transmit power is limited. The reduced PAPR can enhance the EIRP, and then the number of repetitions or retransmissions for coverage enhancement could be efficiently reduced. As a result, the coverage ratio of the satellite will be improved. - Observation 6: Applying DFT-s-OFDM for DL transmissions can be beneficial in the IoT scenario where the narrowband is used and the non-MIMO operation is used in general. The reduced PARP can reduce the number of repetitions or retransmissions for coverage enhancement. - Observation 7: DFT or IDFT can be implemented by IDFT or DFT together with the conjugate function, respectively. - Observation 8: Even though CP-OFDM is used for PBCH while DFT-s-OFDM is used for PDSCH which is FDMed with the PBCH, the overall PARP can be reduced by 2 dB compared to the case where the CP-OFDM is used for both PBCH and PDSCH. - Observation 9: For 3 PDSCHs which are FDMed with some RB gaps, applying DFT transform precoding to each PDSCH separately can reduce the PAPR by 2 dB while applying DFT transform precoding to all the PDSCHs jointly can reduce the PAPR by 3 dB compared to the case where the DFT is not applied. - Proposal 5: RAN1 captures followings as a conclusion for DL DFT-s-OFDM - MRSS compatibility - DL DFT-s-OFDM can be TDMed and FDMed with NR DL CP-OFDM as if UL DFT-s-OFDM can coexist with UL CP-OFDM in NR. - Target modulations, and impact to other modulation, if applicable - DL DFT-s-OFDM can be independently operated with any modulation. - MIMO (SU and MU-MIMO) compatibility - In NTN scenario, since LOS paths are predominantly employed, the spatial multiplexing gain to be exploited for MIMO are limited. - In IoT scenario, considering the target data rate and UE complexity, the MIMO operation is typically not used. - Multi-user multiplexing/scheduling flexibility - DL DFT-s-OFDM can support TDM and FDM for multi-user multiplexing. - If DFT transform precoding is separately applied to each FDMed DL channel/signal, the PAPR can be reduced compared to the reference. - If DFT transform precoding is jointly applied to multiple FDMed DL channels/signals, the significant PAPR reduction can be achieved at the expense of the increased UE complexity. - Multiplexing/coexistence with baseline waveform - DL channel(s)/signal(s) with DL DFT-s-OFDM can be TDMed and FDMed with DL channel(s)/signal(s) with DL CP-OFDM. Even in this case, non-negligible PAPR reduction can be achieved. - Transmitter/receiver complexity and impact power consumption - Transmitter complexity for DL DFT-s-OFDM is marginal since the DFT transform precoding can be implemented by IDFT for receiving UL DFT-s-OFDM and the conjugate function. - Receiver for DL DFT-s-OFDM may need the equalization process while the channel estimation complexity would be similar compared to the reference. - Proposal 6: DFT-s-OFDM waveform is supported as the additional basis for 6GR in downlink. - DFT transform precoding for DL is available at least for a single UE-dedicated PDSCH. - FFS: Whether or how the DFT transform precoding is applied to the common DL channels (e.g., PDSCH containing common signaling, SS/PBCH). - FFS: Whether or how the DFT transform precoding is applied to multiple PDSCHs for multiple UEs. - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE - FFS: Whether or how to support multiplexing with DMRS - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level #### 3. Link Level Performance Evaluation Criterion - Proposal 7: For downlink low-PAPR proposals the primary evaluation criterion is - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding #### 4. New Waveform for sensing - Proposal 8: A new waveform such as FMCW is studied for sensing as well as OFDM. |  |  |
| [5] | R1-2508432 | Discussion on Waveform for 6GR air interface | vivo |
|  | - Considerable net gain can be achieved for CFR-SE: for QPSK, > 1.5dB compared with DFT-s-OFDM, ~1dB gain compared with transparent CFR; for π/2-BPSK, ~1.5 dB gain compared with DFT-s-OFDM, ~0.5 dB gain compared with transparent CFR. With same spectrum efficiency, CFR-SE achieves slightly better performance than truncated π/2 BPSK. Compared with DFT-s-OFDM, 1.7 dB, 1 dB and 0.8 dB net gain can be achieved by utilizing UE-sided AI/ML-based waveform for modulation_order=2,4,6, and, 2.4 dB, 1.3 dB and 1.3 dB net gain can be achieved by utilizing two-sided AI/ML-based waveform for modulation_order=2,4,6. Assuming PC2, DFT-s-OFDM achieves MPR gain compared with CP-OFDM at rank 2 for both coherent and non-coherent precoders. DFT-s-OFDM achieves negligible BLER loss compared with CP-OFDM at rank 2 for non-coherent precoders when BS Rx antenna is 32 or more. OFDM system with subcarrier-controlled symbol-phase continuity not only achieves a significant OOB emission reduction gain, but also enhances the sharpness of the main-lobe spectrum. OFDM system with subcarrier-block-controlled symbol-phase continuity achieves a significant OOB emission reduction gain without compromising any main-lobe decay. The gain in OOB emission containment without compromising main-lobe decay can be interpreted as a gain in SNR. Given the nonlinear relationship between SNR gain and OOB emission gain, simulation-based analysis remains one of the most effective methods for validation. - Study waveform adjustments to achieve high UE power efficiency and UL coverage. Normalized memoryless PA model defined in RAN4 can be used for MPR evaluation for 6GR. Support to study CFR-SE based low-PAPR waveform for 6GR UL. Support to study AI/ML-based waveform enhancements together with non-AI waveforms. Perform a Link-Level simulation for a comprehensive evaluation of the net gain achieved by two-layer DFT-s-OFDM transmission over two-layer CP-OFDM. Support rank 2 for DFT-s-OFDM waveform at least for non-coherent precoders. RAN1 studies the enhancement of OOB emission performance in OFDM systems through phase continuity between OFDM symbols. In addition to the current RAN4 requirements for MPR calculation, RAN1 can further consider the net gain assessed with performance comparisons conducted under the condition of consistent OOB emission. Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. |  |  |
| [6] | R1-2508455 | Discussion on the waveform design for 6G radio | CMCC |
|  | Observation 1. The purpose of introducing DFT-s-OFDM in downlink is to improve the coverage performance by boosting the transmit power. Observation 2. DFT-s-OFDM in downlink mainly works for NTN, while the feasibility is unclear for TN. Observation 3. DFT-s-OFDM in downlink may lead to non-negligible additional complexity to the receiver implementation. Proposal 1. The target use cases are recommended to be clarified and distinguished between multi-layer CP-OFDM and multi-layer DFT-s-OFDM before starting detailed design for multi-layer DFT-s-OFDM. Proposal 2. The study focuses on PAPR reduction for DFT-s-OFDM waveform. Proposal 3. The proposed design of FDSS and tone reservation should clarify the difference and improvement comparing to the corresponding work already been done for NR. Proposal 4. The MCSs used in the link-level evaluations of PAPR reduction techniques should satisfied the required 5th percentile user spectral efficiency in IMT-2030 requirements. Proposal 5. The proponents of non-OFDM based waveform are recommended to provide comprehensive air-interface design related to the proposed waveform, with proper evaluations according to the key requirements of 6GR. |  |  |
| [7] | R1-2508471 | Discussion on waveform for 6GR air interface | THALES |
|  | Observation 1: Based on the simulation results, for downlink transmission it is observed that DFT-s-OFDM achieves a reduction of approximately 2.5 to 3 dB across the waveform distribution when compared to the baseline CP-OFDM in terms of PAPR. In contrast, the other evaluated waveforms demonstrate comparable PAPR values to each other, with variations of less than 1 dB among them. Observation 2: The simulation results show that before amplification, UFMC and BF-OFDM achieve significantly lower out-of-band emissions (about 40 dB below CP-OFDM) while WOLA and F-OFDM perform even better due to their filter designs. OTFS, however, causes interference in adjacent bands, highlighting the need for pulse shaping. After amplification with the HPA model, amplifier-induced distortion limits the out-of-band emission improvements across all waveforms. Observation 3: Simulation results show that DFT-s-OFDM consistently delivers superior ACLR performance, attributed to its lower PAPR and reduced susceptibility to PA-induced non-linear distortion. The out-of-band emission advantages of filtered waveforms are mostly lost after amplification, with OBO improvements over CP-OFDM rarely exceeding 0.5 dB. OTFS uplink achieves ACLR above 30 dB at just 6 dB backoff, while its downlink shows higher total degradation, indicating greater in-band distortion. The resemblance between TD and ACLR trends further highlights DFT-s-OFDM’s robustness to PA distortion. Observation 5: It is observed that under the TDL channel model with HPA non-linearities, OTFS outperforms all other waveforms in BLER, benefiting from the sparse channel and effective MPA receiver operation. DFT-s-OFDM also shows relatively strong resilience due to its lower PAPR, resulting in less in-band distortion. In contrast, UFMC is more vulnerable to ISI and ICI from channel dispersion and HPA effects due to the lack of a cyclic prefix. The remaining waveforms exhibit similar performance, behaving mainly as filtered variants of CP-OFDM designed for out-of-band emission reduction. Observation 6: Simulation results show that both OFDM waveform variants and OTFS exhibit comparable uncoded BER performance under normalized frequency synchronization errors. Notably, UFMC demonstrates increased sensitivity, with higher BER attributable to inter-symbol interference from filter dispersion beyond its guard interval, compounded by inherent noise enhancement effects. Additionally, DFT-s-OFDM does not outperform other waveforms, as its lower PAPR offers no advantage in the absence of power amplifier non-linearities. Observation 7: it is observed based on simulations that - WOLA, UFMC, and BF-OFDM waveforms display pronounced tapering at the edges of the CP protection window, resulting from their respective filtering and windowing operations. - F-OFDM, due to its short filter length, maintains BER performance similar to CP-OFDM without notable degradation. - UFMC, which uses a guard interval instead of a CP, exhibits both a shifted protection window and increased BER degradation, attributable to ISI from filtering and inherent noise enhancement. - OTFS achieves a stable BER across time offsets, as its longer, slot-based CP ensures robust protection, provided offsets remain within the CP duration; this design maintains SNR without additional overhead compared to symbol-wise CP use in OFDM. Observation 8: It is observed based on simulation that DFT-s-OFDM achieves a 3 dB PAPR reduction compared to CP-OFDM for a single beam. However, this advantage diminishes to less than 1 dB when more than five beams are active, as the superposition increasingly resembles a multicarrier waveform with Gaussian-like characteristics. Observation 9: The PAPR for DFT-s-OFDM increases as the number of frequency-domain multiplexed users and the modulation order rise. Observation 10: Simulation results show that CP-OFDM yields consistently high PAPR regardless of the multiple access technique, while DFT-s-OFDM achieves lower PAPR as FDM’ed users and/or number of simultaneous beams increase. Observation 11: PTS-enhanced DFT-s-OFDM scheme manages to maintain a roughly constant PAPR with the number of users in the frequency domain for a single beam but fails to sustain it over more than one beam. Proposals: Proposal 1: RAN1 to investigate the performance of candidate waveforms under varying carrier frequency and time offsets inherent to satellite links, specifically considering scenarios with significant uncertainty in the UE’s position. This study will focus on conditions relevant to GNSS-free physical layer operation, aiming to identify robust waveform solutions suitable for environments with no GNSS availability. Proposal 2: Identify the set of NTN scenarios/use cases for which is beneficial to use DFT-s-OFDM in DL. Proposal 3: RAN1 to study the performance of DFT-s-OFDM in the downlink for non-terrestrial network (NTN)-based 6G radio access. |  |  |
| [8] | R1-2508595 | Discussions on waveform for 6GR | CATT |
|  | Observation 1: The PAPR is increasing with the increasing of DFT points e.g. 4096 to 8192 or 16384. Observation 2: To maintain the same coverage (i.e. same power spectral density), transmission power will be doubled with the doubled channel bandwidth. Observation 3: Since downlink free space path loss in NTN is large, higher transmission power at BS is required. Observation 4: The corner constellation points in odd-order quadrature amplitude modulation (e.g., 16-QAM) have higher power, resulting in a higher peak-to-average power ratio (PAPR). This increases power amplifier linearity requirements and reduces power efficiency. Observation 5: 32-QAM outperforms 16-QAM in in terms of PAPR, BER and throughput. Observation 6: The FSPL in NTN is higher than that in TN. Due to limited energy supply, higher efficient waveform is necessary for satellite communication. Observation 7: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet EVM requirements. Observation 8: An additional 2 dB OBO is needed for CP-OFDM waveforms compared to DFT-s-OFDM waveform for the same modulation scheme to meet equivalent demodulation performance. Observation 9: DFT-s-OFDM waveform can be used across all channels/signals. Observation 10: The issue of multiplexing/coexistence with baseline waveforms does not exist as only one type of waveform is chosen in a single cell in one certain scenario. Observation 11: For NTN scenario, rank 1 is the baseline assumption. Observation 12: When DFT-s-OFDM waveform is used in DL, multi-user resource mapping before DFT precoding can be implemented. Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR: - Larger FFT size (e.g. from 4096 to 8192 or 16384) - Larger transmission channel bandwidth - Increased downlink free space path loss in NTN due to large propagation distance. Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, scheme on reducing the overhead and indicating complexity of side information shall be studied. Proposal 3: It is proposed to study Tone Reservation (TR) for CP-OFDM waveform in 6GR. Proposal 4: If SLM/TR are applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH. Proposal 5: Both symmetric and asymmetric FDSS-SE can be supported in 6GR‌. Proposal 6: It is proposed to study 32-QAM schemes for PAPR reduction for DFT-S-OFDM in 6GR. Proposal 7: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA. Proposal 8: For downlink low-PAPR proposals the primary evaluation criterion may use the following criterion: - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal. - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Proposal 9: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement. Proposal 10: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied. Proposal 11: For improving spectrum efficiency, multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied. Proposal 12: For evaluating DFT-s-OFDM for UL with number of layers > 1, link level simulation is considered: - Net gain = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - The reference is the link level performance of CP-OFDM with corresponding number of layers. - A realistic PA model should be used. - Both low (e.g., 20MHz) and high (e.g., 100MHz) bandwidth. Proposal 13: Besides the link level simulation, following aspects should be analyzed: - Implementation complexity in receiver. - Spec impact, e.g., reference signals design, codebook design. Proposal 14: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW): - OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication - OFDM-based CW with same SCS as communication Proposal 15: For enhance the sensing, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing). |  |  |
| [9] | R1-2508625 | Waveform for 6GR Air Interface | Shanghai Jiao Tong University. |
|  | Observation 1: The structural compatibility of AFDM with OFDM-based systems, which arises from its implementation using existing IFFT/FFT blocks, combined with the inherent wideband nature of its individual chirp subcarriers, makes it a compelling candidate for ISAC. It offers the potential for high-resolution sensing with minimal hardware modification to the NR framework. Observation 2: In TDL-D channels, AFDM demonstrates a gain of nearly 2 dB over OFDM at 300 km/h and more than 4 dB at 500 km/h for a BER of 1e-3, with an even more pronounced advantage at the higher velocity of 1000 km/h. Observation 3: Compared with OFDM, AFDM can achieve a larger maximum unambiguous range with lower pilot overhead, thereby extending sensing range, improving target resolution capability and spectral efficiency. Observation 4: The flexibility in selecting the pre-chirp parameter c2 in AFDM can be leveraged to significantly reduce PAPR by optimizing the phase structure of the transmitted signal. Proposal 1: RAN1 to study additional waveform candidate (e.g., AFDM) for 6G that stays structurally compatible with OFDM, enabling reuse of 5GNR/6GR ecosystem components, while targeting enhanced performance in sensing, high mobility, and NTN scenarios. Proposal 2: RAN1 to investigate AFDM as a candidate waveform for 6G radio, with a specific focus on evaluating its robustness in high-mobility scenarios, high-efficiency sensing capabilities, low PAPR characteristics, and integration into an 6GR compatible system architecture. Proposal 3: RAN1 to study and specify the design of AFDM parameters (e.g., c1 and c2), low-complexity receiver algorithms for communication, sensing, and PAPR reduction, and their integration into a 6G-compatible system architecture, to enhance robustness against doubly-selective channels while targeting superior performance in sensing, high-mobility, and NTN scenarios. Proposal 4: RAN1 to investigate the implications of AFDM on MIMO channel estimation, receiver signal processing, etc., and to study low-complexity techniques to ensure the efficient integration of AFDM with existing multi-antenna systems. |  |  |
| [10] | R1-2508628 | Discussion on 6G Waveform | NEC |
|  | Proposal 1: CP-OFDM and DFT-s-OFDM in NR are baseline as 6GR uplink waveform. 6GR could study to support dynamic waveform switching during initial access. Proposal 2: 6GR strives for a unified waveform baseband generation and upconversion for all channels and signals including PRACH. Proposal 3: Study the use of Frequency Domain Spectrum Shaping (FDSS) for DFT-s-OFDM in the 6GR uplink to enhance coverage and power efficiency. Proposal 4: The evaluation of FDSS enhancements must incorporate realistic PA models and be validated against RF conformance requirements, including ACLR and EVM. Proposal 5: Study a non-transparent FDSS operation for 6GR, including the signaling of the applied shaping filter, to enable advanced receiver equalization and unlock greater performance benefits. Observation 1: Key 6G requirements, like support for NTN and a strong focus on Network Energy Savings from Day-1, create an immediate need for a downlink waveform that offers better coverage and power efficiency than the 5G baseline. Proposal 6: Study the support of low PAPR waveforms like DFT-s-OFDM for 6G downlink transmissions. Proposal 7: Study a simplified framework for DL waveform support, where a default waveform is used for initial access, and UE-specific configuration for DFT-s-OFDM is performed semi-statically via RRC signaling. Proposal 8: Study the waveform design for PDCCH in deployments supporting DL DFT-s-OFDM, evaluating two approaches: - The use of CP-OFDM for PDCCH to ensure implementation simplicity and compatibility. - The feasibility of using DFT-s-OFDM for PDCCH to improve performance, including a detailed analysis of the required structural redesign, challenges in supporting multiple users, and the overall system impact. Proposal 9: Study multi-user scheduling techniques for downlink DFT-s-OFDM, including group-based or sub-band DFT, to balance multi-user throughput with low-PAPR properties. |  |  |
| [11] | R1-2508631 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | 6GR DL and UL baseline waveforms Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms Proposal 2: CP-OFDM and DFT-s-OFDM are the only waveforms for uplink. Study enhancements for PAPR reduction for DFT-s-OFDM. Proposal 3: Support dynamic waveform switching for the uplink for 6GR. Proposal 4: Waveform for sensing is not covered in Agenda Item 11.3.1 and shall be studied separately in Agenda Item 11.14. Multi-layer DFT-s-OFDM Proposal 5: Evaluation for multi-layer DFT-s-OFDM for UL should include gNB advanced MIMO receiver complexity (e.g., ML or MMSE-SIC). Proposal 6: Baseline method for comparison of multi-layer DFT-s-OFDM for UL should be multi-layer CP-OFDM with frequency selective precoding Proposal 7: MPR for multi-layer DFT-s-OFDM should be determined using link level simulation Proposal 8: Co-polarization instead of cross-polarization should be assumed for UE antenna polarization when multi-layer DFT-s-OFDM is evaluated Net gain evaluation results for PAPR reduction techniques for UL DFT-s-OFDM Observation 1: For inner allocation, UL DFT-s-OFDM with FDSS-SE yields from 0.7 dB to 1.2 dB gain over UL DFT-s-OFDM with FDSS Observation 2: For outer allocation, UL DFT-s-OFDM with FDSS-SE yields from 0.8 dB to 1.4 dB gain over UL DFT-s-OFDM with FDSS Proposal 9: Study FDSS with spectrum extension as a potential candidate for uplink PAPR reduction for DFT-s-OFDM to support coverage enhancement for 6G |  |  |
| [12] | R1-2508647 | On 6G waveforms | Ericsson |
|  | Observation 1 Even if there exist both CP-OFDM and DFT-s-OFDM waveforms in the downlink, if vendors choose to implement only one of the two waveforms, e.g., CP-OFDM is selected due to its advantage of facilitating faster and smoother migration towards 6GR by leveraging the implementation maturity from 5G NR, then DFT-s-OFDM in the downlink is less-likely/not deployed in practice. Observation 2 DFT-s-OFDM as a waveform for downlink introduces several new challenges in 6GR system design such as separate handling of physical channels and signals compared to CP-OFDM, reduced scheduling flexibility (i.e., one user and physical channel/signal per symbol), limitations in implementation of enhanced MIMO schemes beyond wideband precoding and diagonal precoding, carrier aggregation over non-contiguous frequency portions of the band. Observation 3 DFT-s-OFDM increases the time-resource span of physical channels and signals due to its limited multiplexing capability, which is rather counterproductive from the vantage of network energy savings. Observation 4 DFT-s-OFDM would limit the system-level coverage of NTN due to ineffective utilization of time and frequency resources that are constrained in NTN deployments. Observation 5 A short pulse for short-pulsed based sensing can be realized using CP-OFDM waveform, and it does not require DFT-s-OFDM. Observation 6 DFT-s-OFDM as a waveform for the downlink offers very limited advantages that are applicable to certain limited use cases while introducing significant complexity in overall 6GR system design. Observation 7 DFT-s-OFDM is not suitable for downlink common channels since it complicates multiplexing common channel with other OFDM-based transmissions, e.g., data channel, inefficient resource mapping, and increasing implementation complexity. Observation 8 PA output backoff needs to be determined using RF simulations and according to RAN4 requirements for error vector magnitude, in band emissions, spectrum flatness, spectrum emission mask, and adjacent channel leakage, spurious emissions while accounting for counter-IM3. Observation 9 FDSS allows to apply power boosting for inner RB allocations only, while FDSS-SE allows to apply power boosting for all RB allocations. Observation 10 If boosting is used for low code rate QPSK and low code rate pi/2-BPSK, FDSS-SE can provide more than 1 dB gain over FDSS for sufficiently wide bandwidth allocations. Observation 11 Benefits of FDSS-SE over FDSS decrease with increasing code rate and can result in net losses for QPSK. Observation 12 If the combination of number of PRBs and MCS is optimized subject to SNR, then there is little or no performance advantage due to FDSS and FDSS-SE compared to the baseline. Observation 13 At least for 2TX antenna configurations at the UE, rank-2 selected in the majority of instances with either of CP-OFDM or DFT-s-OFDM waveform across different combinations of UE power classes and power scaling models. Observation 14 For UE power class 3 with Rel-16 MODE0 scaling, the probability of rank-2 is slightly larger for DFT-s-OFDM compared to that of CP-OFDM due to higher transmission power for DFT-s-OFDM (i.e., lower MPR is applied). Observation 15 Multi-layer DFT-s-OFDM achieves significant cell-edge throughput gains (i.e., up to ~125% for 2 Tx) as well as substantial gains with respect to median user throughput (i.e., up to ~20% for 2 Tx) at the system-level compared to that of CP-OFDM. Observation 16 Since 4 Tx power class 1.5 UEs may have 1/4th of the total power per Tx chain, there is strong potential for >2 layers, and it is not appropriate at this stage of the study to limit DFT-S-OFDM to 2 layers. Observation 17 Multi-layer CP-OFDM and DFT-s-OFDM are complementary in that multi-layer DFT-s-OFDM can enable better exploit power headroom with simpler UE RF capabilities, while CP-OFDM can maximize precoding gain in UEs with larger arrays with coherence capabilities. Observation 18 Fast CP-OFDM / DFT-s-OFDM switching for MIMO requires additional mechanisms that add complexity to scheduling and UE PHR determination and so far have not been specified in NR. Based on the discussion in the previous sections we propose the following: Proposal 1 RAN1 to deprioritize the study of DFT-s-OFDM for downlink due to lack of any significant advantage compared to CP-OFDM even for the potential use cases such as NES, NTN ISAC, and FR3, and for the transmission of common channels/signals as well as to keep the 6GR system design to a reasonable complexity. Proposal 2 For uplink low-PAPR proposals, throughput should be evaluated in addition to Net Gain to select and compare the best combination of frequency resources (BW) and spectral efficiency (MCS). Proposal 3 To obtain broader insights with respect to the evaluations of different uplink low-PAPR proposal for 6GR, RAN1 to consider MPR-based system-level simulations in addition to link-level simulations. Proposal 4 RAN1 to consider system-level evaluations for studying the multi-layer DFT-s-OFDM in the uplink by accounting for MPR based on RF power class and RF power scaling models with system-level parameter settings in Table 4 as a starting point. Proposal 5 RAN1 to consider at least UE power class 2 with Rel-15 power scaling model and power class 3 with Rel-16 MODE0 scaling for 2TX antenna configurations at the UEs in the system-level evaluations of multi-layer DFT-s-OFDM in the uplink. Proposal 6 Support multi-layer DFT-s-OFDM and multi-layer CP-OFDM for uplink transmissions in 6GR. |  |  |
| [13] | R1-2508684 | Discusson on 6GR Waveform | Xiaomi |
|  | Observation 1: For DL DFT-s-OFDM waveform, apart from TDM, FDM can be used for multi-user multiplexing at the cost of certain PAPR increase. Observation 2: For DL DFT-s-OFDM waveform, apart from TDM, FDM can be used for multi-channel multiplexing at the cost of increased number of RF chains. Proposal 1: To support the coverage performance for NTN DL, low-PAPR waveform such as DFT-S-OFDM can be considered. Proposal 2: To support the coverage performance for NTN DL, low-PAPR waveform can be applied to the following channels: - PDCCH at least for CSS (except for type-3) - PDSCH with Msg 4 - PDSCH with SIB1 Proposal 3: The UL coverage enhancement mechanism in Rel-18 including power domain solution and waveform switch related solution can be taken as a starting point for UL coverage performance guarantee for both TN and NTN. |  |  |
| [14] | R1-2508727 | Discussion on waveform and multiple access for 6G Radio | OPPO |
|  | Proposal 1: A unified 6GR baseline waveform is studied to fulfil the requirements of eMBB and 6G IoT. - The baseline waveform is used for 6G HRLLC. Proposal 2: Study waveforms to fulfil the requirement of 6G Sensing and 6G NTN (Ubiquitous Connectivity). - Strive for reusing the 6GR baseline waveform for 6G Sensing and 6G NTN. - An additional waveform can be considered if significant gain over the baseline waveform can be justified for a specific vertical scenario, but only supported by the vertical BS/UE. Proposal 3: For studying the additional waveform for 6GR, evaluate waveform proposals using agreed 6GR waveform (i.e., CP-OFDM for DL and CP-OFDM/DFT-s-OFDM for UL) as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. Proposal 4: For studying the 6GR baseline waveform, support up to 2 waveforms in DL and up to 2 waveforms in UL, e.g., one optimized for spectrum efficiency, one optimized for coverage. At least 1 waveform in DL and 1 waveform in UL are mandatorily supported for all device types, e.g., CP-OFDM in DL and DFT-s-OFDM in UL. Proposal 5: Only one DL waveform is supported for 6GR initial access procedure. Proposal 6: For downlink low-PAPR proposals the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. Proposal 7: DFT-s-OFDM is not supported as additional DL baseline waveform for 6GR, due to limited performance gain, restriction on multiuser scheduling and extra complexity on UE side. - DL DFT-s-OFDM for NTN can be further studied in NTN agenda. Proposal 8: For uplink low-PAPR waveform proposals the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. - Net gain including MPR can be evaluated by RAN4. Proposal 9: Prioritize the implementation-based schemes without specification impacts. - Study additional gain from schemes with specification impacts. Proposal 10: For DFT-s-OFDM for UL with multiple layers, the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. Observation 1: Based on the preliminary evaluation results, DFT-s-OFDM for UL with 2 layers suffers 1dB ~ 6dB SINR loss @10% BLER with different MCS levels, although it brings about 1.6dB PAPR reduction at 10% probability over 2-layer CP-OFDM. Proposal 11: Study multiple access (MA), targeting a single MA scheme for each waveform, to fulfil the requirement of all 6G usage scenarios using this waveform. Proposal 12: Orthogonal multiple access (OMA) is the baseline for 6GR. Evaluate OMA proposals using 5G NR as the benchmark, with the consideration of following: - Spectrum efficiency. - Coverage. - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. |  |  |
| [15] | R1-2508735 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 1: Take Table 1~4 as the start point for low PAPR waveform enhancement evaluations. Observation 1: I/Q-offset DFT-s-OFDM has signal structure in frequency domain, where the values of spectrum extension part are conjugate symmetric values of the in-band part. Observation 2: Multi-user frequency overlap transmission could be exploited to mitigate the spectral efficiency loss effect caused by the spectrum extension, and improve UE experience for more UEs. Proposal 2: Study how to utilize the signal property of frequency redundancy in the spectrum extension based schemes to enhance multi-user experience. For example, scheduled PRBs for multiple UEs are overlapped partially. Observation 3: With larger spectrum extension and/or lower modulation order, I/Q-offset DFT-s-OFDM can achieve a lower PAPR. Different FDSS filters also impact the PAPR under same modulation order and spectrum extension factor, which could be optimized in real deployment. Observation 4: I/Q-offset DFT-s-OFDM can provide net gain compared with NR DFT-s-OFDM waveforms - For no frequency overlap scheduling - 0.5b/s/Hz and outer/inner RB locations, provide ~0.5dB gain - 0.8b/s/Hz and outer RB locations, provide >1dB gain - 0.8b/s/Hz and inner RB locations, provide ~0.2dB gain - For frequency overlap scheduling, - 0.5b/s/Hz and outer/inner RB locations, provide >1.5dB gain - 0.8b/s/Hz and outer RB locations, provide 2dB gain - 0.8b/s/Hz and inner RB locations, provide >1dB gain - 1.9b/s/Hz and outer RB locations, provide >1dB gain - 1.9b/s/Hz and inner RB locations, provide ~0.5dB gain Observation 5: Multi-user frequency overlap scheduling could provide higher net gain than no frequency overlap scheduling for I/Q-offset DFT-s-OFDM. Observation 6: Low PAPR I/Q-offset DFT-s-OFDM can provide net coverage gain for users to meet the diverse data rate requirements. Observation 7: Low PAPR waveform I/Q-offset DFT-s-OFDM can provide net gain for 20%+ users scheduled by the whole system bandwidth. Proposal 3: Study lower PAPR waveform enhancement providing net gain vs NR DFT-s-OFDM waveform. FFS the exact supported lower PAPR waveform enhancement. Observation 8: I/Q-offset DFT-s-OFDM is the generalization of some offset QAM (separate I and Q path input to DFT-s-OFDM) based proposals, e.g., enhancement on pi/2 BPSK and O-QAM modulated DFT-s-OFDM. Observation 9: Tone reservation highly relies on the implementation algorithm, and high UE implementation complexity is identified in R18. Proponents should provide baseline algorithm for fair comparison with other clear transmission schemes, e.g., FDSS. Observation 10: I/Q-offset DFT-s-OFDM with larger SE factor (e.g., 3/8, 7/16) could provide higher net gain than lower SE factor (e.g., 1/4, 1/3). Observation 11: I/Q-offset DFT-s-OFDM is better than other spectrum extension plus FDSS waveforms especially at large SE factors. Other spectrum extension plus FDSS waveforms have similar net gain performance. Proposal 4: Study spectrum extension/truncation plus FDSS lower PAPR waveform enhancement, e.g., I/Q-offset DFT-s-OFDM, DFT-s-OFDM FDSS-SE. Observation 12: Pruning QAM under basis DFT-s-OFDM waveform could provide net gain comparing with NR QPSK and 16QAM modulation under certain spectral efficiency range (e.g., 1.6~2.0 bit/s/Hz). Proposal 5: Study pruning QAM under DFT-s-OFDM type waveform(s) for coverage enhancement. Observation 13: 8-Pruning QAM under CP-OFDM could provide higher spectral efficiency compared with QPSK with same sensing performance, and provide better sensing performance compared with 16QAM especially when FDSS is applied. Proposal 6: Study pruning QAM under CP-OFDM waveform for ISAC. Observation 14: DL DFT-s-OFDM for on-demand synch. Signal/LP-WUS in LPR can provide potential network energy saving and UE energy saving gain, without the impact on other signals on the MR. Proposal 7: For DL DFT-s-OFDM on-demand synch. signal/LP-WUS, the performance evaluation criterion from waveform perspective is net gain - Net Gain [dB] = Tx power gain relative to the reference – Required SNR degradation relative to the reference - For on-demand synch. signal and sequence based LP-WUS, the required SNR is for detection rate below 1% and false alarm rate below [1%] assuming same resource overhead - The requirements of RSRP accuracy based on on-demand synch. signal should be met, e.g. as in TS 38.133 Proposal 8: Take Table 9 as a start point for DL DFT-s-OFDM waveform evaluation for on-demand synch. signal/LP-WUS. Observation 15: Utilizing sequence based DFT-s-OFDM for on-demand synch. signal and LP-WUS transmission yields a network gain of around 3 dB compared to CP-OFDM. Proposal 9: Study DL DFT-s-OFDM for on-demand synch. signal/DL-WUS for coverage enhancement, network energy saving and UE energy saving under related agendas, e.g., initial access, network energy saving, UE energy saving agenda. Proposal 10: Take Table 11 as the start point for UL multi-layer DFT-s-OFDM link level net gain evaluations. Observation 16: As the number of RX number increases, SNR degradation of multi-layer DFT-s-OFDM compare to CP-OFDM is mitigated, when RX number greater or equal than 64, there is no difference between these two waveforms on BLER performance. Observation 17：Considering the MPR gain of DFT-s-OFDM, UL multi-layer DFT-s-OFDM can provide a net gain compare to UL multi-layer CP-OFDM Observation 18: Throughput gain is observed in Urban-Marco 10% RU scenario based on UL multi-layer DFT-s-OFDM compare to CP-OFDM. Proposal 11: At least 2-layers uplink DFT-s-OFDM (including enhancement) waveform with observed coverage net gains should be supported in 6GR. The maximum number of layers should be further studied in 6GR MIMO agenda, e.g., codebook design. |  |  |
| [16] | R1-2508802 | Discussion on waveform for 6GR | Samsung |
|  | UL DFT-s-OFDM PAPR reduction with FDSS Observation 1: Frequency domain spectrum shaping with the half-sine pulse filter for π/2-BPSK transmission achieves very low PAPR of below 1 dB. Observation 2: Half-sine pulse filter can satisfy 3GPP in-band emission requirement, so the effective occupied bandwidth does not increase. Observation 3: PAPR gain translates into effective coverage gain when frequency domain spectrum shaping is applied with matching the filters at the transmitter and receiver sides. Proposal 1: Study non-transparent frequency domain spectrum shaping (FDSS) on DFT-s-OFDM to reduce PAPR for 6GR UL coverage enhancement. UL DFT-s-OFDM rank>1 Observation 4: If the introduction of multi-rank DFT-s-OFDM could offer “significant” spectral efficiency gain (over what’s already supported in Rel-18 NR), it would be primarily for rank-2 Observation 5: At 3.5 GHz frequency in UMa and RMa assuming 2TX handheld UEs, the acclaimed spectral efficiency gain of two-layer DFT-s-OFDM over two-layer CP-OFDM (due to lower PAPR) is not observed in SLS - Advanced non-linear receiver for two-layer DFT-s-OFDM can be used to improve its performance at the expense of sNB/network cost and energy consumption Proposal 2: To assess whether multi-rank DFT-s-OFDM can offer significant spectral efficiency gain, focus the study on rank-2 (two-layer UL transmission on PUSCH) Proposal 3: To assess whether rank-2 DFT-s-OFDM can offer significant UL spectral efficiency gain, further investigate its performance in deployment scenarios with primarily line-of -sight channels DL DFT-s-OFDM Observation 6: For downlink, the PAPR reduction of DFT-s-OFDM over CP-OFDM does not translate to significant DL coverage gain at least because of the following factors: - F1) Non-contiguous DL FDRA and its BLER advantage - F2) Common use of carrier aggregation (CA) - F3) Higher-rating PA resulting in minimal/zero PA backoff for CP-OFDM (including multi-carrier PA that makes the SC property of DFT-s-OFDM inapplicable) - F4) Active antenna arrays employing beamforming - F5) Multi-layer MIMO transmission In addition, it suffers from the following drawbacks: - D1) Multiplexing efficiency loss leading to SE and ESG loss - D2) Real-time multi-waveform processing at the UE receiver - D3) Overly complex and/or sub-optimal MIMO receiver leading to SE loss and higher UE power consumption Proposal 4: Discontinue the study for the potential support of DFT-s-OFDM waveform for 6GR downlink - No evidence of any potential benefit in DL coverage over CP-OFDM in all relevant use cases such as NTN, IoT, FR3, energy efficiency, common signals/channels. In fact, in most cases, DL DFT-s-OFDM would likely result in spectral efficiency loss, increased energy consumption at base stations and UEs, and substantial specification/testing efforts Other waveforms Proposal 5: Discontinue the study for the potential support of “other waveforms” in 6GR - Deviating from “single technology framework” goal of 6GR study - Increased risk of RAT divergence for different 6GR use cases - Not conducive to the agreed support for NR-6GR migration via MRSS |  |  |
| [17] | R1-2508833 | Waveform for 6G Radio Air Interface | IMU |
|  | Observation 1: The current CP design ensures robustness but leaves part of the transmitted energy unused. With rising interest in energy-efficient and sensing-capable waveforms, treating the CP purely as a guard interval is suboptimal. Proposal 1: Redesign The CP to actively utilize its redundancy rather than being discarded. Observation 2: The fixed TCP/Tsym ratio limits waveform adaptability. Optimal CP length should reflect the actual channel delay spread, not the numerology configuration. Proposal 2: The CP duration should become a function of the channel characteristics as well as of the numerology. Observation 3: The current multi-numerology design achieves service diversity but sacrifices flexibility. Restricting one numerology per carrier limits the ability to provide differentiated services within a common physical layer framework. Proposal 3: Redesign 5G NR OFDM frame and symbol structure to achieve the goals of multi-numerology operation (via different methods) within a single unified framework. Observation 4: Existing waveform structures, including CP-OFDM, were primarily designed for broadband data transmission and are not optimized for the heterogeneous demands of 6G. Proposal 4: The 6GR frame structure and OFDM symbol should be designed as a uniform waveform framework capable of meeting all major 6G requirements, technologies, and channel conditions instead of multiple waveform types. Proposal 5: The uniform waveform framework design should natively support: - Low-PAPR, energy-efficient transmission, - Native integration of sensing (ISAC), - AI-native and adaptive operation, - URLLC and massive connectivity, - High spectral efficiency with backward compatibility, - Coexistence with low power applications such as OOK generation for WuS and Ambient IoT, - Unified TN and NTN operation, - Extreme flexibility and forward compatibility, - Improved uplink coverage and coexistence. Observation 5: While CP-OFDM has proven robustness and backward compatibility, it cannot simultaneously meet the diverse requirements of 6G technologies and channel conditions in its current form. Proposal 6: Redesign the CP-OFDM-based 6GR frame structure to enable native integration of heterogeneous services within a single, reconfigurable architecture. Observation 6: Introducing multiple new waveforms would further fragment the ecosystem and increase complexity. Proposal 7: CP-OFDM remain the sole downlink waveform for 6G radio to ensure simplicity, backward compatibility, and unified evolution of the physical layer. Proposal 8: RAN1 should study U-OFDM as downlink waveform for 6G Proposal 9: standardize a small set of subcarrier spacings (SCS)s families for U-OFDM to simplify implementation while enabling flexible alignment with NR resource structures, guard/circularity timing, and high-SCS operation, as in Table 1. Table 1: Proposed U-OFDM Subcarrier Spacing (SCS) Families ID; SCS Formula Δf; μ Range; Design Principle / Alignment. A; 2μ×15kHz; μ∈{4,5,6,…}; NR-consistent binary scaling of SCS (legacy-friendly), extended to very large Δf for high carrier frequencies.. B; 2μ×12×15kHz; μ∈{0,1,2,…}; SCS equals an integer multiple of the NR resource block (RB) bandwidth (12 subcarriers), i.e., Δf aligns to 12×15kHz= 180 kHz and its powers of two.. C; 2μ×14×15kHz; μ∈{0,1,2,…}; Subsymbol duration aligns with CP/UW duration: for μ=0CP=1 subsymbol, for μ=1CP=2 subsymbols, etc. (circularity-friendly timing lattice).. Proposal 10: Support flexible and scalable time-frequency resource allocation in 6G, considering the following generalized definitions of the Resource Block (RB): - Option 1: RB defined by a single subcarrier-subsymbol pair (m,n); - Option 2: RB defined by a single subcarrier m and a group of subsymbols in time; - Option 3: RB defined by a single subsymbols n and a group of subcarriers in frequency; - Option 3: RB defined by a group of subcarriers in frequency and a group of subsymbols in time. Observation 7: The guard duration in U-OFDM can be flexibly adjusted to match the channel's delay spread, unlike fixed CP lengths in CP-OFDM. This enables efficient resource usage across diverse propagation environments, as seen in Figure 5. Observation 8: U-OFDM provides temporal continuity, allowing joint processing of subsymbols across adjacent symbols. This opens opportunities for advanced equalization, prediction, and multi-symbol detection. Observation 9: The guard region in U-OFDM is also configurable: it may consist of regular OFDM subsymbols or be replaced by a unique word (UW), a structured sequence optimized for synchronization, channel sounding, PAPR reduction, or localization. Proposal 11: RAN1 should study both CP-U-OFDM and UW-U-OFDM for 6GR. Proposal 12: The CP itself should be generalized to consist of one or more OFDM subsymbols. Proposal 13: Consider incorporating both CP-based and UW-based U-OFDM guard structures in 6G waveform discussions. Observation 10: The use of cut-off redundant subsymbols enables segmentation of a U-OFDM symbol into multiple mini-blocks, each preserving local circularity. This allows the receiver to apply FFT-based demodulation on shorter blocks rather than waiting for the entire symbol duration, significantly reducing latency and enabling pipelined or parallel processing. Observation 11: Cut-off regions function as localized guard intervals, either cyclic (CP) or structured (UW), that reproduce a portion of the symbol's earlier content. These boundaries improve robustness to long delay spreads and allow fine-grained control over demodulation granularity without disrupting the global symbol structure. Observation 12: Cut-off subsymbols are optional for the design and unlock the full flexibility of the OFDM design. Proposal 14: Study the use of cut-off redundant subsymbols in U-OFDM as a means to enable mini-block segmentation of a symbol. Observation 13: Reducing the effective NFFT per subsymbol lowers PAPR and relaxes amplifier back-off, improving transmitter efficiency, especially in power-limited scenarios such as FR2/FR3 and IoT. Proposal 15: The intrinsic low-PAPR feature of U-OFDM be leveraged for energy-efficient transmission and considered in future PAPR-aware configuration studies. Observation 14: U-OFDM transforms the Tx/Rx chain into a continuous streaming pipeline. The transmitter minimizes buffering, and the receiver enables parallel and early processing across symbol segments, achieving faster feedback and lower overall latency. Proposal 16: Study U-OFDM as a natively URLLC-capable waveform. Observation 15: U-OFDM offers an inherent ISAC capability by reusing deterministic guards as sensing references. This eliminates the need for additional pilots or dedicated sensing waveforms, enabling efficient joint operation in a unified frame structure. Proposal 17: Evaluate U-OFDM as a baseline ISAC-compatible waveform. Observation 16: By combining subsymbol-level segmentation with adjustable time-frequency parameters, U-OFDM can simultaneously mitigate delay and Doppler impairments. This dual robustness is difficult to achieve with conventional CP-OFDM or DFT-s-OFDM, where waveform parameters are fixed for the entire symbol. Proposal 18: Study the channel-robust properties of U-OFDM under high-mobility and long delay scenarios, focusing on adaptive cut-off placement and subsymbol duration. Observation 17: The cut-off segmentation in U-OFDM provides a Doppler-aware control knob: smaller sub-block durations reduce ICI without changing the global numerology, improving performance for high-speed users. Proposal 19: Study adaptive cut-off placement strategies for high-mobility users to minimize ICI while maintaining spectral efficiency and low receiver complexity. Observation 18: By decoupling the guard design from symbol duration, U-OFDM guarantees circularity under diverse channel conditions and provides improved robustness against ISI. Proposal 20: Evaluate guard design methods where the number of redundant subsymbols is determined by measured or estimated delay spread rather than fixed numerology parameters. Observation 19: U-OFDM eliminates the rigid Doppler-delay trade-off of CP-OFDM by independently tuning sub-block duration and guard configuration. Proposal 21: Consider U-OFDM as a baseline design for channels with simultaneous delay and Doppler dispersion, given its flexible and adaptive parameterization. Observation 20: U-OFDM provides a unified, service-adaptive waveform structure in which latency, mobility, and delay spread can be jointly optimized at the subsymbol level. This allows URLLC, eMBB, and mMTC traffic to coexist in the same time-frequency frame with minimal signaling and without the guard overheads of multi-numerology CP-OFDM. Observation 21: 5 G NR CP-OFDM cannot achieve this degree of service multiplexing without substantial guard allocations and signaling complexity. In contrast, U-OFDM internalizes redundancy and circularity at the subsymbol level, effectively harmonizing delay, Doppler, and latency requirements in a single transmission frame. Proposal 22: Evaluate U-OFDM as a unified service-multiplexing waveform for 6G, capable of concurrently serving heterogeneous users and applications under a single numerology-free configuration. Proposal 23: Consider studying the optimizing cut-off placement, subsymbol scheduling, and guard design to dynamically adapt to mixed service and channel profiles. Observation 22: The structural framework of U-OFDM inherently includes CP-OFDM as a limiting case. This ensures that legacy receivers can process U-OFDM signals configured with standard CP parameters without requiring hardware or architectural modification. Proposal 24: Study U-OFDM as an evolutionary extension of CP-OFDM, offering enhanced flexibility and functionality while maintaining interoperability with existing NR systems. Observation 23: CP-OFDM is directly embedded within the U-OFDM framework, confirming that U-OFDM does not disrupt legacy signal formats but expands their configurability. Observation 24: U-OFDM achieve zero-interference mixed-numerologies-like performance by assigning cut-off subsymbols to process mini-blocks in time, thus U-OFDM removes the need for frequency-domain guard bands and simplifies multi-service coexistence. Proposal 25: Evaluate U-OFDM as a unifying waveform framework capable of reproducing standard CP-OFDM for legacy compatibility while simultaneously supporting interference-free mixed-numerology-like performance. Observation 25: The subsymbol-based structure of U-OFDM allows direct mapping of OOK pulses without overlapping CP tails or inter-symbol leakage. This enables efficient wake-up signaling and ambient backscatter operation using the same waveform, with minimal energy overhead and without dedicated OOK hardware paths. Observation 26: By decoupling CP/UW from data, U-OFDM provides clean time-domain transitions, enabling fully orthogonal OOK signaling within the same OFDM resource grid. Proposal 26: Evaluate U-OFDM as an OOK-native waveform for energy-limited technologies such as Ambient IoT and wake-up radios. Observation 27: U-OFDM's subsymbol lattice naturally defines a short time scale for fast precoder and beam updates, while its flexible structure supports hierarchical scheduling of beams and MIMO layers. The same framework supports hybrid precoding, MU-MIMO transmission, and joint communication-sensing beam management without waveform modification. Proposal 27: Study U-OFDM unified waveform framework for advanced MIMO and beamforming operations in future systems. Proposal 28: Explore the subsymbol-level processing for adaptive precoding, aging compensation, and sensing-aided beam tracking under high mobility and wideband conditions. Observation 28: U-OFDM enables per-subsymbol time multiplexing of UL and DL without guard bands or analog duplexers. This architecture achieves SBFD-like simultaneous availability while remaining entirely digital and self-contained. Proposal 29: Evaluate U-OFDM as a time-domain full-duplex solution achieving SBFD performance objectives without additional spectrum or hardware overhead. Observation 29: This structure enables uplink availability even during downlink transmission, ideal for 6G use cases with near-symmetric traffic loads (e.g., XR, AI, or digital-twin applications). Proposal 30: Study scheduling algorithms and control signaling for dynamic UL/DL interleaving at the subsymbol level to optimize latency and utilization. Observation 30: Micro-grant scheduling enables low-latency UL access and coverage extension through coherent accumulation, while maintaining continuous DL service. Proposal 31: Study the micro-grant and repetition-based combining schemes as optional U-OFDM configurations for power-limited or coverage-critical uplink devices. Observation 31: U-OFDM provides the benefits of SBFD, simultaneous UL and DL availability, through digital time multiplexing on a single carrier, avoiding analog complexity and spectral inefficiency. Proposal 32: Consider U-OFDM as a complimentary solution to SBFD, delivering comparable coverage and latency benefits with reduced implementation cost and system complexity. Observation 32: U-OFDM replaces static and redundant guard structures with adaptive, information-rich components. This enables flexible trade-offs between throughput across varying propagation conditions. Proposal 33: Evaluate U-OFDM under standardized SE metrics to quantify the combined gain from adaptive guard allocation, guard reuse for signaling, and pilot reduction, particularly in multi-service and ISAC-enabled scenarios. Observation 33: U-OFDM's subsymbol granularity provides the necessary degrees of freedom for AI-based optimization, enabling real-time adaptation of physical-layer parameters that are fixed in conventional OFDM systems. Proposal 34: Study U-OFDM as the baseline structure for AI-adaptive PHY design Observation 34: U-OFDM's short subsymbol structure allows physical-layer latency reduction by orders of magnitude compared to CP-OFDM, aligning with the stringent timing requirements of AI-native networks. Proposal 35: Evaluate U-OFDM as the reference waveform for sub-millisecond and microsecond latency applications, including semantic and cooperative AI communications. Observation 35: U-OFDM enables native UL/DL coexistence within one symbol, aligning with the symmetric traffic characteristics of AI workloads. Proposal 36: Study U-OFDM as an enabling waveform for AI-native traffic models with symmetric UL/DL requirements. Observation 36: U-OFDM transforms the physical layer into an intelligent, reconfigurable substrate that can be co-optimized by AI agents for maximum spectral and energy efficiency. Observation 37: UW-U-OFDM maintains full compatibility with standard OFDM processing chains while transforming the guard interval into a flexible, multi-purpose signal. It supports precise synchronization, efficient control signaling, and native sensing without additional bandwidth or computational cost. Observation 38: Full-band UWs embedded within U-OFDM provide robust and scalable frequency references, eliminating the need for dedicated frequency pilots or narrowband synchronization bursts. Proposal 37: Study UW-U-OFDM for CFO tracking and frequency synchronization. Observation 39: The UW lattice provides continuous, wideband phase references that facilitate low-complexity PN tracking and first-order ICI correction directly in the frequency domain. Proposal 38: Evaluate UW-U-OFDM as a waveform candidate for integrated PN tracking. Observation 40: Multiple embedded UWs enhance time resolution and synchronization reliability while maintaining spectral efficiency by reusing guard energy. Proposal 39: Study UW-based synchronization schemes in U-OFDM systems. Observation 41: UW-U-OFDM unifies synchronization, channel sounding, and calibration within a common OFDM framework, scaling efficiently with antenna count and beam density. Proposal 40: Explore and study the UW-based per-beam signaling for multi-beam synchronization and reciprocity calibration in large-array TDD deployments. Observation 42: UWs provide native sensing capability within the same time-frequency structure used for communication, achieving high SNR and joint optimization potential. Proposal 41: Adopt UW-U-OFDM as a baseline ISAC-compatible waveform, where UW design jointly optimizes synchronization, channel estimation, and sensing performance without additional resource consumption |
| [18] | R1-2508844 | Waveform for 6GR air interface | BJTU |
|  | Observation 1: By introducing DD domain channel estimation, DD-a-OFDM significantly outperforms classical OFDM in high-mobility conditions, effectively eliminating the error floor and approaching the performance of ideal channel knowledge. Observation 2: The DD-domain signal processing, i.e., DD domain channel estimation, in DD-a-OFDM not only mitigates ICI but also converts it into a form that enables highly accurate channel parameter estimation, which is crucial for ISAC. Proposal 1: RAN1 should study receiver-side DD-domain signal processing techniques that maintain full compatibility with the CP-OFDM transceiver, for high-mobility and ISAC enhancements of CP-OFDM. Proposal 2: RAN1 should study the design criteria for TF-domain pilot patterns that enable accurate DD-domain channel estimation, ensuring the pilot spacing is sufficient to avoid aliasing for the target delay and Doppler spreads of 6GR deployment scenarios. Proposal 3: RAN1 should evaluate the DD-a-OFDM scheme as a candidate technology for performance enhancement of CP-OFDM in high-mobility and ISAC use cases, with evaluation metrics to include BER under high Doppler, channel estimation accuracy and sensing precision (MSE against CRLB), and pilot overhead compared to baseline, i.e., the classical CP-OFDM. |  |  |
| [19] | R1-2508856 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Observation 1: TR-based OFDM achieves consistent Tx power gain of approximately 2 dB across QPSK, and 64QAM modulations, demonstrating its robustness and modulation-agnostic effectiveness. Observation 2: TR-based OFDM achieves consistent Tx power gain of approximately 2 dB across different bandwidth, demonstrating its robustness and bandwidth-agnostic effectiveness. Observation 3: TR-based DFT-s-OFDM achieves Tx power gain of approximately 2 dB for QPSK modulations, demonstrating its robustness effectiveness. Observation 4: With a properly chosen number of reserved tones, TR achieves effective Tx power gain while preserving spectral efficiency for both CP-OFDM and DFT-s-OFDM. Observation 5: The impacts of reserved tone has negligible effect on BLER performance across all modulation coding schemes under different bandwidths for CP-OFDM when the number of reserved tones is properly chosen. Observation 6: The impacts of reserved tone has negligible effect on BLER performance for DFT-s-CP-OFDM when the number of reserved tones is properly chosen. Observation 7: A proper configuration of number of reserved tones can achieve the trade-off between Tx power gain and demodulation performance. Observation 8: SLM with different configurations have demonstrated noticeable Tx power gain , e.g., around 1.4dB ~2.7dB, across all modulation orders (QPSK, 64QAM), which indicates that the SLM scheme is robust and modulation-agnostic. Observation 9: SLM with different configurations have demonstrated noticeable Tx power gain, e.g., around 1.2dB ~2.7dB, across all bandwidths (11RB, 24RB), which indicates that the SLM scheme is robust and bandwidth-agnostic. Observation 10: SLM scheme has no effect on BLER performance across all modulation coding schemes under different bandwidths. Observation 11: A properly configuration of SLM scheme can achieve the trade-off among Tx power gain,BLER performance and computational complexity. Observation 12: For pi/2-BPSK, FDSS w/o SE can achieve 3.9dB Tx power gain. Observation 13: For QPSK, FDSS w/o SE can achieve 2dB Tx power gain, and on top of this, FDSS with symmetric SE (i.e., solution-1) can provide additional Tx power gain gain, and the performance gain can be further increased as the extension ratio increases. Observation 14 :The impacts of FDSS w/ and w/o SE on BLER performance is slightly decreased compared with the case without FDSS across all typical modulation coding schemes, while the impacts of spectrum extension factor has negligible effect on BLER performance. Observation 15: For pi/2 BPSK, similar performance (i.e., the Net Gain) is achieved for both FDSS w/ symmetric SE and FDSS w/o SE, but better performance can be achieved FDSS w/ symmetric SE for QPSK. Observation 16: If the bandwidth of I-modulation is larger than that of the original π/2-BPSK/QPSK/QAM, it corresponds to spectrum expansion; if the bandwidth of I-modulation is smaller than that of the original, it corresponds to spectrum truncation. Observation 17: The I-modulation scheme can be achieved using either a time-domain approach or a frequency-domain approach. Observation 18: The I-modulation scheme is applicable to a wide range of modulation schemes, from low-order π/2-BPSK to high-order QAM. Observation 19: The I-modulation scheme achieves significant PAPR reduction compared to conventional π/2-BPSK, QPSK, and 16QAM with and without transparent FDSS. Observation 20: The I-modulation scheme provides better performance at 10% BLER compared to conventional π/2-BPSK, QPSK with transparent FDSS. Observation 21: For π/2-BPSK, the I-modulation scheme achieves approximately 3 dB Net Gain compared to conventional π/2-BPSK without transparent FDSS, and approximately 1 dB Net Gain compared to conventional π/2-BPSK with transparent FDSS. Observation 22: For QPSK, the I-modulation scheme achieves approximately 2.1 dB Net Gain compared to conventional QPSK without transparent FDSS, and approximately 1.8 dB Net Gain compared to conventional QPSK with transparent FDSS. Observation 23: DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level (i.e., eDFT-s-OFDM waveform) achieves superior performance to DFT-s-OFDM with less reference signal overhead in high-speed scenario. Observation 24: Compared to 5G, the usage of the GFB-OFDM can enable higher spectrum utilization ratio, e.g., 86.4% for 5MHz with lower OOBE. Observation 25: The necessity for OTFS in 6GR requires further justification given its unfavorable complexity-performance trade-off. Observation 26: For sensing, a coverage of larger than 1 km is needed for one Tx/Rx pair, and hence a large transmission power, e.g., 58 dBm, is needed. Proposal 1: For uplink low-PAPR proposals, the existing sub-6 GHz PA model from RAN4 (R4-163314) can be reused as baseline. Proposal 2: For uplink low-PAPR proposals, adopt the following method for calculating Tx power gain during evaluation of low-PAPR candidate waveforms: Step 1: Compute average power of each waveform to be used as PA input; Step 2: Obtain PA output according to PA input and PA model; Step 3: Tx power gain is the PA output of low-PAPR waveform minus that of conventional OFDM/DFT-s-OFDM waveform. Proposal 3: For downlink low-PAPR proposals, the Net Gain can be used for evaluation with following updates: - Net Gain [dB] = PAPR gain relative to the reference – SNR degradation relative to the reference. - Note:For data and control channel, the SNR is associated with 10% BLER. Proposal 4: To improve coverage, tone reservation should be considered in 6G waveform design as a low-complexity scheme to achieve Net Gain along with the compatibility with both UL and DL waveforms. Proposal 5: To improve coverage, Selected Mapping(SLM) should be considered in 6G waveform design as a low-complexity scheme to achieve Net Gain along with the compatibility with both UL and DL waveforms. Proposal 6: To improve coverage, FDSS can be considered in 6G waveform design. - FDSS w/o SE is considered as the baseline. Proposal 7: I-modulation (I-π/2-BPSK/QPSK/QAM) scheme should be considered to reduce PAPR for DFT-s-OFDM. Proposal 8: The link-level simulation to evaluate the PAPA/Tx power gain compared with the OFDM is considered to justify the performance of DFT-s-OFDM for UL with number of layers > 1. Proposal 9: DFT-s-OFDM with rank 2 for uplink transmission can be considered in 6G waveform study. Proposal 10: DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level (i.e., eDFT-s-OFDM waveform) can be considered as a candidate waveform technology for 6G waveform design to improve the performance at least for high-speed scenario. Proposal 11: GFB-OFDM should be considered in 6G waveform study as a scheme to improve the spectrum utilization. Proposal 12: Study pulse RS design using OFDM-based waveform for large sensing coverage. - The pulse is applicable for both mono-static and bi-static sensing. - Study the application for communication, e.g., RSRP measurement, time/frequency tracking. |  |  |
| [20] | R1-2508621 | Discussion on 6GR Waveform | Lenovo |
|  | Proposal 1: Study adopting Downlink DFT-s-OFDM for low data rate MBB in the terrestrial network as part of network energy savings and coverage. - Target channel(s): Two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH). - Evaluation methodology: - LLS to start with focusing on key metrics such as: Net Gain and NES - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @PHY channel target BLER - Realistic PA model for gNB - NES gain (%) – baseline against corresponding CP-OFDM - Further discuss details on how to do without SLS Proposal 2: Target Downlink Channels to evaluate DFT-s-OFDM can be narrowed down to two physical channels e.g., one common channel and one UE specific downlink channel (PDSCH). Observation 1: DFT-s-OFDM maintains MRSS compatibility with 5G. Observation 2: DFT-s-OFDM maintains same scheduling flexibility compared with 5G. Proposal 3: Downlink DFT transform precoding can be applied in following two ways: - Per UE DFT: DFT transform precoding can be applied to each downlink channel of a UE in a time slot where each DFT size is same as the number of occupied subcarriers of the downlink signal of a UE - Sub-band DFT: A single DFT transform precoding can be applied to downlink channel of a group of UEs where DFT size can be larger than the number of assigned subcarriers of each UE. Observation 3: Sub-band DFT transform precoding scheme gains from frequency diversity of the channel, SNR at UE receiver due to frequency spreading is enhanced due to applying single DFT transform precoding over a larger bandwidth (5MHz BW) than UE allocated frequency resources, - 4dB SNR gain can be obtained compared to CP-OFDM and per UE DFT-s-OFDM for UEs each with 1 RB allocation 1.5 dB gain can be obtained for UEs each with 6RBs allocation Observation 4: Sub-band DFT for DL can enhance the throughput of low data rate MBB traffic at cell edge. At same coverage 40% to 60% gain in the throughput can be achieved compared to CP-OFDM Observation 5: PHY channels can be multiplexed in the frequency domain using the different waveform. - PAPR/CM of the signal with multiplexed waveforms varies depending on the allocation ratio between DFT-s-OFDM-based channel and CP-OFDM-based channel and other factors such as modulation etc., - For e.g., with 50:50 ratio between SB-DFT and CP-OFDM more than 1dB PAPR gain is observed Observation 6: PHY channels can be multiplexed in the frequency domain using the same DFT-s-OFDM waveform. PAPR/CM reduction gain depends on the number of sub-bands, sub-band sizes, other factors such as modulation etc., - For e.g., Multiplexing 2xSB-OFDM waveform in a slot still provides 3.5dB PAPR gain compared to CP-OFDM Observation 7: BS transmitter needs module update to apply DFT transform precoding Observation 8: UE receiver complexity depends on DMRS REs multiplexing and whether frequency domain or time domain receiver processing is applied. Observation 9: Even in worst case scenario of applying random scheduling of UEs with different waveforms, the back-off can vary from 5dB down to 2 dB. Over 3000 slots, more than 1.3 dB average gain can be obtained in the Back-off compared to CP-OFDM and the corresponding average network energy saving obtained from the different instants of dynamic Back-off over 3000 slots can reach 16%. Observation 10: Specification impact of DFT-s-OFDM for the common channel/signal is higher compared to the unicast data channel (PDSCH). Observation 11: Sub-band/group based DFT scheme achieves approximately a 2.75 dB cubic metric (CM) gain compared to the CP-OFDM waveform, and a 1.31–2.68 dB gain over per UE DFT-S-OFDM which translates into reductions of more than 30% in the number of repetitions required to achieve a 10% BLER. Observation 12: With selected mapping scheme, the number of used sequences is important for PAPR/CM reduction, e.g., a gap of 0.5dB can be seen between 8 and 4 sequences. Proposal 4: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement, and compare to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. Observation 13: FDSS using conventional filter methods (e., root-raised cosine, Hamming, Hanning, etc.) can give a good reduction in PAPR, however, the reduction of CM can be minor in some cases. Proposal 5: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc. Proposal 6: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform |  |  |
| [21] | R1-2508863 | Support for Diverse Waveforms on the 6GR Air Interface | National Spectrum Consortium |
|  | Observation O1. 6G aims to support diverse services and device types. Different waveforms are suitable for different purposes or applications. Even if OFDM is chosen as the 6GR waveform, it is desirable to incorporate flexibility of supporting different waveforms to make 6G futureproof or forward-compatible. Proposal P1. Study suitability of different waveforms for different services and device types and identify key elements of a framework that enables the 6G protocol stack to support different physical layer waveforms. |  |  |
| [22] | R1-2508887 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 1: 6GR should allow certain time / frequency resources can be different waveform for forward compatibility perspective and to support MRSS. Proposal 2: For 6GR waveform design, time/frequency grid should be allowed to be aligned and orthogonal with NR boundary. Proposal 3: OFDM-based waveform should be supported for 6GR. - The definition of “OFDM-based” is to have subcarrier mapping and IFFT to generate time-domain signal. Proposal 4: Striving for OFDM-based waveforms across all the identified use cases can be sufficient at least for 6G Day 1. Proposal 5: Any enhancements to CP-OFDM or DFT-s-OFDM and/or any newly introduced waveform must demonstrate clear and justified advantages over 5G waveform. Proposal 6: RAN1 should assess the need to introduce MPR / PAPR reduction techniques, e.g., FDSS-SE / FDSS-CE targeting coverage enhancement for UL. Proposal 7: The gain is obtained in which RB allocation should also be investigated. Proposal 8: The need to introduce flexible DMRS and data techniques (e.g., TDM between data and DMRS before DFT precoding such as OTFDM) could be investigated. Observation 1: From system perspective, there are many challenges to support DFT-s-OFDM in DL, for example to multiplex SSB and other channel jointly, and the, the motivation of low PAPR waveform in DL is unclear. Proposal 9: DL DFT-s-OFDM for individual signal / channel (such as LP-WUS / LP-WUR signal) are not required to be concluded for now. The important point it whether to support DFT spreading to overall channel’s method. Proposal 10: To support DFT spreading to overall DL channel’s method is excluded. |  |  |
| [23] | R1-2508890 | Waveform considerations for 6G Uplink | Wisig, IITH |
|  | Observation-1. The MPR gain of π/2-BPSK DFT-s-OFDM relative to QPSK CP-OFDM is ~5.5 dB under full-bandwidth allocation. Proposal-1. Adopt π/2-BPSK DFT-s-OFDM as the 0-dB MPR reference for 6G uplink. Proposal-2. Carry forward existing DFT-s-OFDM modulation schemes as primary uplink options for 6G NR. Proposal-3: DFT-s-OFDM-based waveforms should be considered the primary uplink waveforms; CP-OFDM may be considered secondary. |  |  |
| [24] | R1-2508917 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission. Proposal 2: Transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Complementary metrics—such as SNR degradation, effective occupied bandwidth, and PA efficiency can also be analysed to provide a comprehensive assessment of waveform linearity and spectral behaviour. Proposal 3: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM. Proposal 4: Downlink transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Evaluation is based on the Net Gain metric (Tx power gain – link loss at 10% BLER) using a realistic base station PA model, complemented by secondary metrics such as PA efficiency, ACLR, EVM, network energy savings, and occupied bandwidth. Proposal 5: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM. Proposal 6: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques. Proposal 7: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks. |  |  |
| [25] | R1-2508946 | Waveform for 6GR Air Interface | Google |
|  | Proposal 1: Support DFT-s-OFDM waveform as the DL waveform with regard to the following aspects: - To support the same coverage for FR1 and FR3 - To provide a good coverage for NTN - Compared to other coverage enhancement techniques, e.g., to increase the number of antennas or to transmit the DL signals by multiple repetitions, using DFT-s-OFDM waveform does not require additional complexity for complicated CSI calculation, does not require large delay for beam measurement or multi-repetitions-based DL signal reception, and does not require large overhead for complicated CSI report and more DL-RSs for beam measurement. Proposal 2: Support the DFT-s-OFDM waveform for multiple layers for UL transmission. Proposal 3: Postpone the dynamic waveform switching related discussion until most of the details for each waveform are finalized. |  |  |
| [26] | R1-2508973 | Discussion on 6GR waveform | ETRI, University of Surrey |
|  | Proposal 1. RAN1 to explicitly capture pros and cons of potential new or enhanced waveform candidates as summarized in the FLS. - Simple re-use of NR CP-OFDM and DFT-s-OFDM without any official RAN1 observation is NOT recommended, given that a number of contributions have been submitted during three WG meetings with in-depth assessments on the waveform candidates. Observation 1. AFDM, with its favorable properties and CP-OFDM compatibility, is a promising candidate for further 6G waveform evaluation as enhancement of CP-OFDM waveform or alternative of DFT-s-OFDM. Observation 2. AFDM has the following properties (compared with NR CP-OFDM): - NR CP-OFDM can be generated by AFDM with proper parameter tuning (i.e., AFDM is a superset of CP-OFDM) - Lower PAPR (see Section 2.2. for detailed analysis) - Backward compatible with NR CP-OFDM (by either of parameter tuning or scheduling) - Higher PA efficiency (further analysis will be provided in future meeting) Observation 3. AFDM has the following properties (compared with NR DFT-s-OFDM): - Higher Doppler tolerance - Higher delay tolerance Observation 4. From target use case perspectives, AFDM is particularly beneficial for NTN and ISAC use cases. Observation 5. From target link direction perspectives, AFDM is applicable for both uplink and downlink. Observation 6. It is expected that AFDM may provide the following benefits, at least: - PAPR reduction: 3 dB (by turning AFDM modulation parameter) - BLER: 0.5 dB gain @ SNR= 5dB with Doppler frequency of 3000 Hz; 1.5 dB gain @ SNR= 5dB with Doppler frequency of 6000 Hz - Compatibility with FMCW Radar Observation 7. AFDM may coexist with CP-OFDM-based RATs in MRSS, enabling resource partitioning for NTN, high-mobility, and ISAC slices while supporting legacy UEs. Observation 8. AFDM is best applied to data channels (PDSCH, PUSCH) and optional NTN access signals, while CP-OFDM remains baseline for synchronization and broadcast. Observation 9. AFDM’s MIMO compatibility needs further study, though single-layer or diversity modes may suffice in LoS-dominant NTN and ISAC scenarios. Observation 10. AFDM can support existing NR modulations transparently, sustaining high-order QAM under Doppler while coexisting with CP-OFDM. Observation 11. AFDM may enable flexible multi-user multiplexing via chirp-domain resource partitioning. Observation 12. AFDM may coexist with CP-OFDM/DFT-s-OFDM baseline waveforms through FDM, TDM, or hybrid partitioning, ensuring backward compatibility for legacy UEs while enabling robust performance for AFDM-capable devices. Observation 13. AFDM can work alongside CP-OFDM for synchronization and access, while optional AFDM-based RACH may enhance robustness in NTN and high-mobility scenarios. Observation 14. AFDM may introduce specification impacts mainly in waveform definition, resource mapping, pilot and RACH design, and scheduling/signaling extensions, while remaining structurally compatible with the existing NR framework to ensure smooth coexistence with CP-OFDM. Observation 15. AFDM can reuse existing FFT/IFFT hardware with minimal added complexity, while lower PAPR and Doppler robustness improve PA efficiency and power savings. Proposal 2. RAN1 to capture the following summary on AFDM waveform to the TR on 6GR: Description. Name of the proposal; AFDM. Applicable link direction; UL / DL. Enhancement to CP-OFDM?; Yes. Enhancement to DFT-s-OFDM?; Yes. Additional OFDM-compatible waveform?; No. Target channel(s); PDSCH, PUSCH, PRACH (for NTN). Target modulation; No restriction (applicable for all existing NR modulations). Motivation / use case; NTN, ISAC, high-speed mobility. Key Metric / KPI; BLER, sensing capability, possible NetGain in low-PAPR and delay-Doppler robustness. Key spec impact foreseen; RAN1 waveform definition, time-frequency resource mapping, pilot design, scheduling extensions. |
| [27] | R1-2509042 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 1: Study adaptive low-PAPR enhancement schemes for DFT-s-OFDM uplink, including FDSS-SE and frequency-selective mapping. Proposal 2: Evaluate dynamic waveform switching between CP-OFDM and DFT-s-OFDM modes under MRSS-aligned 6GR configurations using the Net Gain criterion (@ 10% BLER) for both single- and multi-layer uplink transmissions. Proposal 3: Study downlink waveform enhancements including the use of DFT-s-OFDM with FDSS and SLM. Proposal 4: Use the Net Gain (dB) = Tx power gain – Link Loss to the reference @10% BLER as a primary downlink waveform evaluation metric Proposal 5: The "Net Gain" evaluation framework shall be used to evaluate AI/ML-based PAPR reduction schemes. Proposal 6: In addition to "Net Gain," AI/ML-based schemes should be evaluated on their computational complexity and feasibility. The following metrics can be considered: - Model Complexity (e.g., number of parameters, FLOPs/symbol). - Signaling Overhead (e.g., bits for side information, if any). - Complexity Type: A clear distinction between offline training requirements and the real-time, on-device inference complexity. Proposal 7: RAN1 to establish a standardized evaluation methodology that integrates PA-model-based simulation and link-level validation using Net Gain as the primary metric, with complementary indicators (MPR, ACLR, EVM, and complexity) where applicable. Proposal 8: RAN1 to study the performance of DFT-s-OFDM and potential enhancement candidates under PA saturation and Doppler conditions representative of NTN and high-mobility links. |  |  |
| [28] | R1-2509049 | Discussion on waveform for 6GR air interface | Ruijie Networks Co. Ltd |
|  | Proposal 1: - If support for DFT-s-OFDM with rank > 1 is defined, RAN1 will not limit the applicable bands - DFT-s-OFDM with Rank > 1 study is - Limited to max rank = 2 |  |  |
| [29] | R1-2509059 | Waveform for 6G NR | Cohere Technologies, IIT Delhi |
|  | Observation 1: CP-OFDM with a single SCS doesn’t deliver the best performance of CP-OFDM over all channel conditions Observation 2: CP-OFDM fails when the channels have high delay and Doppler spreads Observation 3: Both Zak-OFTFS and Zak-OTFS-over-OFDM outperform CP-OFDM when channels have normal to high delay and Doppler spreads Observation 4: There is almost not degradation in Zak-OTFS performance with increased delay and Doppler spreads Observation 5: There is very little degradation in Zak-OTFS-over-OFDM performance with increased delay and Doppler spreads Observation 6: Zak-OTFS provides flexibility in waveform selection with benefits to 6G in multiple aspects, meets the MRSS, and has little expected impact on the specification Based on the above observations we propose the following: Proposal 1: Zak-OTFS and its special variance Zak-OTFS-over-OFDM are included in the waveform study for 6G Observation 7: AFDM waveform delivers attractive performance only when its parameter c1 is an integer multiple of 1/(MN) which makes it equivalent to Zak-OTFS waveform spread using GDAFT Observation 8: Performance of AFDM is the equivalent to that of Zak-OTFS when the c1 parameter of AFDM is an integer multiple of 1/(MN) Observation 9: Since the performance of AFDM with the right choice of parameters is equivalent to that of Zak-OTFS, they can be treated as one waveform Based on these observations we propose the following: Proposal 2: The study of ADFM will be included under the Zak-OTFS study for the 6G waveform |  |  |
| [30] | R1-2509074 | Considerations for 6GR waveform | Sony |
|  | Observation 1: PAPR reduction for CP-OFDM is less effective when MIMO is used. Observation 2: Spatial multiplexing gains to be exploited for MIMO are limited on the NTN DL. Proposal 1: RAN1 should study PAPR reduction for CP-OFDM that can be applied to the NTN DL. Observation 3: DFT-s-OFDM with high-order modulation exhibits significant PAPR. Proposal 2: RAN1 should study PAPR reduction for DFT-s-OFDM especially when used with higher-order modulation. Proposal 3: RAN1 should study constellation shaping for low PAPR for DFT-s-OFDM with higher-order modulation. Proposal 4: RAN1 should study multiplexing of CP-OFDM reference signals and other signals on the same component carrier with DFT-s-OFDM physical channels. Proposal 5: RAN1 should study multi-layer transmission with DFT-s-OFDM for both UL and DL. Observation 4: For DFT-s-OFDM, most transmit OFDM symbols already have satisfactory PAPR and so do not need SE. Observation 5: When SE is applied at will at the transmitter, and no indication is sent to the receiver, optimal demodulation is almost of identical complexity compared with that of a standard DFT-s-OFDM receiver. Observation 6: When SE is applied at will at the transmitter and no indication is sent to the receiver, optimal demodulation at the receiver requires knowledge of the probability with which SE is applied. Observation 7: Flexible SE provides superior spectral efficiency and improved PAPR reduction compared with deterministic SE Proposal 6: For DFT-s-OFDM spectral extension, RAN1 should consider a mode in which the transmitter may freely decide, for each DFT-s-OFDM symbol if SE should be applied or not. No indication of said decision needs to be indicated to the receiver. |  |  |
| [31] | R1-2509110 | Waveforms for 6GR air interface | Apple |
|  | Observation 1: Low PAPR Waveform provides benefit in coverage (allows a higher output power without waveform saturation) and energy efficiency (depending on the PA type and operating mode). Proposal 1: The following table details the motivations of Low PAPR waveforms for 6G Motivation/Information; Comments. Targeted link direction; UL. Motivation; Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity. MRSS compatibility; Yes. Target channels/signals; PUSCH. MIMO (SU and MU-MIMO) compatibility; N/A. Target modulations; pi/2-BPSK, QPSK. Multi-user multiplexing/scheduling flexibility; same as baseline DFT-S-OFDM. Multiplexing/coexistence with baseline waveforms; Yes. Impact on synchronization and initial access; N/A. Expected specification impact; RAN1 Specification RAN4 Requirements. Receiver Complexity; Transparent Non-transparent. Impact to power consumption; Improved PA efficiency. Observation 2: The existing PA model from, e.g. R4-164542 may be insufficient for evaluating true high-power performance. A new realistic PA model to accurately assess the benefits of near constant envelop waveforms may be required to study near constant envelop waveform especially in a power-boost mode. Observation 3: Enabling a high-power boost feature necessitates a re-evaluation of RF requirements to ensure spectral compliance and manage potential interference Proposal 2: To evaluate MPR or power gain, PA model and RF requirement should be revisited to support power boost feature in 6G. Proposal 3: Study UL π/2-BPSK DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters to reach near constant envelope waveform using transparent and non-transparent schemes. Observation 4: FDSS Filtering - For ACLR 31 and ACLR 37, waveforms with π/2-BPSK, DFT-S-OFDM and the GMSK approximation filter show consistently better Net Gains compared to those of legacy filtering and no filtering Observation 5: Transparency vs Non-Transparency - For ACLR 31 and ACLR 37, the non-Transparent scheme shows improvement in Net Gain for the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filters - For ACLR 31 and ACLR 37, the non-Transparent scheme does not show any Net Gain for the legacy 3-tap filters Observation 6: Symmetric Bandwidth Extension - For ACLR 31, symmetric BW extension shows Net Gains for π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter only with BT = 0.5. For other BT values we do not observe improvement. - For ACLR 37, symmetric BW extension shows Net Gains for all the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter (i.e., 1 pulse or 2 pulse, with different BT values) - For ACLR 31 and ACLR 37, symmetric BW extension does not show any Net Gains for the legacy 3-tap filters Proposal 4: Consider near constant envelope waveform and potential impact to RAN4 for example in terms of amount of power boost and associated RF requirement. Proposal 5: Study Transparent/non-Transparent scheme with the GMSK approximation filter to approximate constant envelope waveform with π/2-BPSK and DFT-S-OFDM for uplink coverage enhancement Proposal 6: Send LS to RAN4, to inform RAN4 that near constant envelope waveform shall be considered for 6G, and request RAN4 to evaluate the PA model, RF requirements, maximum power boost, and Tx power gain of a near constant envelope (sub-1dB) waveforms. Proposal 7: Do not support DFT-S-OFDM in the DL Proposal 8: Use the Net Gain as Evaluation Metric for Multi-Layer DFT-S-OFDM - Baseline scheme is 2-layer DFT-S-OFDM with non-coherent precoding |
| [32] | R1-2509133 | Discussion on waveform for 6GR air interface | Ofinno |
|  | Enhancements of uplink waveforms: Proposal 1: Confirm that CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for 6GR for uplink. Observation 1: The PAPR reduction technique is needed to enhance uplink coverage, which is mainly limited due to UE maximum output power. Observation 2: Due to limited number of transmit antennas in the UE, PAPR reduction becomes critically important in achieving uplink coverage around 7 GHz (in 6G) comparable to 3.5 GHz (in 5G). Proposal 2: Consider frequency domain spectrum shaping (FDSS) as a candidate scheme for PAPR reduction for DFT-s-OFDM with at least /2-BPSK and QPSK. Observation 3: PAPR reduction technique(s) beyond FDSS will be needed to achieve target uplink coverage considering wide range of scenarios and radio environments envisaged in 6G. Proposal 3: Consider FDSS with spectrum extension (FDSS-SE) as a candidate scheme for PAPR reduction for DFT-s-OFDM with at least /2-BPSK and QPSK. Proposal 4: Consider both FDSS and FDSS-SE for at least data channel (e.g., PUSCH). Observation 4: In 5G, large number of frequency bands and several carrier aggregation band combinations support UE power class 2 (PC2) (i.e., 26 dBm). Proposal 5: Consider high UE power class (e.g., 26 dBm) as a mandatory feature or at least mandatory for higher bands (e.g., above 2 GHz) in 6GR from Day 1. Observation 5: In 5G, maximum power reduction (MPR) can be very large (e.g., up to several dBs) for several combination of waveform and modulation scheme in both single carrier and carrier aggregation operations. Proposal 6: Study the possibility of reducing MPR in 6GR. Observation 6: The dynamic waveform switching allows the network to quickly switch the UE from CP-OFDM to DFT-s-OFDM to lower MPR and increase coverage. Observation 7: The dynamic waveform switching allows the network to quickly switch the UE from DFT-s-OFDM to CP-OFDM to DFT-s-OFDM to increase data rate, e.g., under high CQI report. Proposal 7: Support dynamic switching between DFT-s-OFDM and CP-OFDM in 6GR from Day 1. Observation 8: Limiting the DFT-s-OFDM for single layer uplink transmission (like in 5G) will result in low throughput/bit rate for users far from the base station. Proposal 8: Consider UE power output capability (e.g., MPR) as a criterion for evaluating DFT-s-OFDM for multilayer uplink transmission. Observation 9: CP-OFDM and DFT-s-OFDM being specified for 5G uplink, will facilitate multi-RAT spectrum sharing (MRSS) in uplink between the 5G and 6G systems. Enhancements of downlink waveforms: Proposal 9: Confirm that CP-OFDM waveform as defined in 5G NR is supported for 6GR for downlink. Observation 10: Base station typically employs PA linearization techniques (e.g., digital pre-distortion, power backoff, etc.), which is up to the base station implementation. Observation 11: A base station employing large antenna array size (e.g., around 7 GHz) to increase coverage will require high energy efficiency. Observation 12: From downlink coverage perspective and network energy saving standpoint, waveform with relatively smaller PAPR will be beneficial in both TN and NTN deployment scenarios. Proposal 10: Consider DFT-s-OFDM as potential additional waveform for downlink. Proposal 11: Consider at least /2-BPSK and QPSK as modulation schemes for DFT-s-OFDM waveform in downlink. Proposal 12: Target channels/signals for DFT-s-OFDM can be unicast PDSCH, UE specific PDCCH, and relevant reference signals (e.g., DMRS, CSI-RS). Observation 13: Impact of any additional waveform (e.g., DFT-s-OFDM) for downlink in 6GR needs to be investigated for multi-RAT spectrum sharing (MRSS) between the 5G and 6G systems. Waveforms for large Doppler and delay spread environments: Observation 14: CP-OFDM and DFT-s-OFDM are suitable for most practical radio environments in which 6G will be deployed. Observation 15: The 5G system supports speed up to 500 km/h for high-speed scenarios using the existing waveforms (CP-OFDM and DFT-s-OFDM). Observation 16: CP-OFDM and DFT-s-OFDM are likely to be feasible for most practical communication related use cases in 6GR. Observation 17: Waveform for sensing use cases, which may involve extreme radio environment (with higher speed and/or large delay spread), will be separately studied under agenda 11.14. |  |  |
| [33] | R1-2509143 | Waveform for 6GR air interface | MediaTek Inc. |
|  | Proposal 1: As O-QPSK modulated DFT-s-OFDM offers a significant net gain of 2dB with respect to π/2-BPSK modulated DFT-s-OFDM, support O-QPSK modulated DFT-s-OFDM for 6G UL coverage enhancement. Proposal 2: Support O-QPSK modulated DFT-s-OFDM with subcarrier truncation for coverage enhancement at spectral efficiency larger than 1 bit/s/Hz. |  |  |
| [34] | R1-2509231 | Waveforms for 6GR | Qualcomm Incorporated |
|  | Proposal 1: For 6G Radio waveform study, limit initial focus to waveform design for communication use cases. Waveforms and specific waveform enhancements for other use cases such as sensing to be discussed separately. Observation 1: Potential areas of focus for 6G waveforms include: - Waveforms for cell-edge UEs - Waveforms for multi-layer transmissions in uplink - Better spectrum utilization - Enabling higher power uplink transmission - More flexibility in scheduling, waveform selection and spectrum usage in uplink Proposal 2: Design considerations for 6G waveform study for communication purposes to include: - new spectrum bands and associated requirements, e.g large BW - needs for new deployment scenarios, e.g. suburban macro, FWA, etc. - duplex operation, e.g., subband full duplex - enhancing coverage, e.g. design of low PAPR waveforms - Support for high power transmissions in uplink, e.g., higher power classes, MPR optimizations - integration with use cases such as sensing and positioning - Co-channel and adjacent channel requirements - Support for spatial multiplexing, beamforming, multiple access - Transceiver complexity associated with synthesis and reception; processing latency - Energy/power efficiency - Considerations on backward compatibility and coexistence with 5G - Scheduling flexibility and agility Proposal 3: In 6GR study on waveforms, focus on enhancements to the DFT-S-OFDM family of waveforms. Observation 2: Low PAPR DFT-S-OFDM waveforms with Pi/2 BPSK that are generated using upsampling and spreading are unlikely to exhibit transmit-power gains relative to legacy DFT-S-OFDM waveforms with pi/2 BPSK and FDSS. Observation 3: The family of low PAPR DFT-S-OFDM waveforms with Pi/2 BPSK generated using upsampling and spreading can be shown to be equivalent to waveforms generated using spectral extension and FDSS. Observation 4: Determining the value of the family of low PAPR DFT-S-OFDM waveforms with Pi/2 BPSK that are generated using upsampling and spreading (or equivalently, spectral extension with FDSS) needs a close examination of the trade-off between spectral efficiency and PAPR/transmit power gains. Observation 5: Applicability of legacy DFT-S-OFDM waveforms with pi/2 BPSK is limited to spectral efficiencies below 1 bit/s/Hz. Observation 6: DFT-S-OFDM waveforms with Pi/2 BPSK modulation satisfy the following frequency domain property: Sk = SL2-k* , where Sk are the frequency domain samples and L is the DFT size. Proposal 4: For 6GR, study the family of low PAPR waveforms obtained using DFT-S-OFDM with Pi/2 BPSK and truncated mapping. Observation 7: It may be feasible to allow partial spectrum sharing between users utilizing DFT-s-OFDM Pi/2 BPSK with FDSS to improve uplink spectral efficiency. Proposal 5: For 6G Radio, support DFT-S-OFDM in addition to CP-OFDM for multi-layer transmissions in uplink. Proposal 6: Study feasibility of using non-identity precoders for multi-layer DFT-S-OFDM transmissions with focus on the PAPR-precoding gain trade-off of using such precoders. Proposal 7: For 6GR waveform study, for DFT-S-OFDM waveforms, decouple the size of allocation from the DFT size. Define any DFT size that is a product of powers of 2, 3 and 5 as a valid DFT size. Proposal 8: For 6GR waveform study, when considering DFT-S-OFDM waveforms, consider flexible frequency-domain mapping of the DFT output to the spectrum allocation, e.g., frequency-domain multiplexing of DMRS and data, non-contiguous mapping, etc. Proposal 9: For 6GR waveform study, consider multi-tx enhancements for DFT-S-OFDM where different transmit ports transmit over different frequency domain allocations. Proposal 10: For 6GR waveform study, consider feasibility to enhance spectrum utilization for small channel bandwidths using spectrum confinement techniques (e.g. WOLA) of reasonable complexity. |  |  |
| [35] | R1-2509254 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 1: It is recommended that the 6G waveform evaluation framework incorporate: - Net Gain shall serve as the performance evaluation criterion for low-PAPR uplink waveforms, with the requirement that it must be greater than 0. - A consistent set of supporting metrics (e.g. PAPR, BLER, OOBE) for comprehensive characterization. Proposal 2: Waveform design for 6GR should account for Inter-Symbol-and-Carrier Interference (ISCI) in high-mobility scenarios to maintain reliable communication and sensing performance. Proposal 3: we propose considering a unified waveform design framework to simultaneously support communication and sensing functionalities (ranging/velocity estimation/imaging) in 6G systems. Proposal 4: It is recommended that Frequency Domain Spectrum Shaping (FDSS) be considered a foundational component for the 6G uplink waveform design. Proposal 5: It is recommended to investigate enhanced FDSS techniques, with a focus on FDSS with Spectrum Extension (SE), for potential inclusion in the 6G specification. The study should also encompass other spectral processing methods, such as spectral truncation. Proposal 6: It is proposed that the evaluation of FDSS techniques for 6G shall encompass a wide range of resource allocation scenarios. This must include extreme configurations, such as very narrowband (e.g., ≤ 5 RBs) and very wideband (e.g., ≥ 100 RBs) allocations, to ensure a comprehensive performance characterization. Proposal 7: The support of at least Rank 2 DFT-s-OFDM for uplink transmission is proposed for inclusion in the 6G waveform study. - Proposal 8: It is recommended to adopt DFT-s-OFDM as a complementary waveform to CP-OFDM in the 6G downlink. Proposal 9: It is proposed to study efficient waveform multiplexing and switching mechanism for 6GR. |  |  |
| [36] | R1-2509282 | Discussion on Waveform | NTT DOCOMO, INC. |
|  | Observation 1: The MPR gain under UE RF requirements, such as EVM, ACLR, and OBW, should be considered as the transmit power gain when evaluating low-PAPR proposals Observation 2: Wider coverage may be needed for 6G - For 6GR waveform, PAPR performance improvement for better coverage, especially for the uplink may be necessary for 6GR - It should be noted that several other aspects should be considered together with PAPR performance, such as achievable net gains, link budget, and system-wise performance when it is deployed Observation 3: DFT-s-OFDM enhancement with SE, including FDSS-SE and FDSS-CE, can reduce PAPR performance to achieve better coverage with a small impact, which can be considered the waveform candidates for 6G uplink Observation 4: FDSS-CE has no requirement for the number of RBs used for SE, and requires lower implementation complexity than FDSS-SE based on the serial implementation method for asymmetric SE Observation 5: QPSK rotation can reduce the PAPR of the DFT-s-OFDM waveform while achieving a higher data rate than pi/2-BPSK. Observation 6: QPSK rotation can be combined with DFT-s-OFDM, DFT-s-OFDM with FDSS and/or SE enhancement for further PAPR reduction. Observation 7: DFT-s-OFDM with frequency domain truncation (FDT) can achieve a tunable trade-off between PAPR and spectrum efficiency Observation 8: For FR1, DFT-s-OFDM with FDSS can’t achieve a net gain with any given SE factor and/or any given bandwidth due to the non-severe PA nonlinearity in FR1 and large SNR degradation introduced by ISI Observation 9: For FR1, FDSS-SE, and FDSS-CE can achieve a similar net gain with any given SE factor and/or any given bandwidth. Observation 10: For FR1, the net gain introduced by QPSK rotation in FDSS-SE and FDSS-CE is almost negligible Observation 11: UL multi-layer DFT-s-OFDM (e.g., rank-2) could achieve higher spectrum efficiency, but the impact on PAPR needs to be carefully considered Observation 12: DL DFT-s-OFDM may have a significant impact on DL operation, so the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified Observation 13: OTFS-based waveforms cause additional complexity and cost due to low compatibility with OFDM-based waveforms Proposal 1: For 6GR study on waveform, - Only OFDM-based waveform(s) should be considered (as described in the SID) - Any new waveform(s), even for OFDM-based, should be justified by clear gain - Unified design across scenarios/use cases is strongly preferred - Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g., - Potential better coverage by better PAPR performance for uplink - Spectrum efficiency improvement |  |  |
| [37] | R1-2509303 | Discussion on DL DFT-s-OFDM for 6GR | LG Electronics, CATT, Thales, Lenovo, IITH, WiSig |
|  | Proposal 1: For downlink low-PAPR proposals the primary evaluation criterion is one of - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal - Network energy saving gain relative to baseline for BS - FFS: Other evaluation metrics - Note: - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For link loss relative to the reference, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - Companies to report how to calculate the Tx power gain, modulation and coding Proposal 2: For the study of the DFT-s-OFDM as the potential additional waveform for DL, RAN1 consider followings: - Potential Target channels/signals - PDCCH, e.g., CSS PDCCH - PDSCH, e.g., unicast PDSCH (including Msg4), common PDSCH (including SIB1 PDSCH) - SSB - Wake-up signal Note: Which channels will be evaluated is up to company - How to apply the DFT transform precoding in case of multiplexing UEs or channels/signals into one symbol - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE - Other options are not precluded. - How to multiplex DMRS (for PDCCH and/or PDSCH) - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level - Other options are not precluded. |  |  |
| [38] | R1-2509322 | Study on waveform for 6GR | Sharp |
|  | Proposal 1: For 6GR waveform, RAN1 should NOT study DL DFT-s-OFDM. Proposal 2: For the evaluation of multi-layer UL DFT-s-OFDM, Net Gain should be used and the assumed precoder should be reported. Observation 1: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.46 dB and Raw CM improvement at 1% CCDF is 0.70 dB for FDSS compared to conventional DFT-s-OFDM. Observation 2: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.78 dB and Raw CM improvement at 1% CCDF is 0.49 dB for FDSS-SE compared to conventional DFT-s-OFDM. Observation 3: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 1.01 dB and Raw CM improvement at 1% CCDF is 0.26 dB for FDSS compared to conventional DFT-s-OFDM. Observation 4: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 2.44 dB and Raw CM improvement at 1% CCDF is 1.22 dB for FDSS-SE compared to conventional DFT-s-OFDM. Observation 5: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.54 dB for FDSS compared to conventional DFT-s-OFDM. Observation 6: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.98 dB for FDSS-SE compared to conventional DFT-s-OFDM. Observation 7: For Case 2 (QPSK), MPR-based Net Gain is -0.61 dB for FDSS compared to conventional DFT-s-OFDM. Observation 8: For Case 2 (QPSK), MPR-based Net Gain is -1.12 dB for FDSS-SE compared to conventional DFT-s-OFDM. Observation 9: Interlace OFDM has the following advantages: - (1) Power boosting - (2) Mitigation of frequency offset and phase noise effects - (3) OFDM symbol repetition - (4) DMRS overhead reduction - (5) BWP-level multiplexing Observation 10: Interlace OFDM and Interlace DFT-s-OFDM have SNR gain compared to CP-OFDM and DFT-s-OFDM, and they are helpful for the coverage edge scenario. Observation 11: Interlace DFT-s-OFDM does not contributes significantly to the energy efficiency. Observation 12: Interlace OFDM and Interlace DFT-s-OFDM have lower spectral efficiency for a given number of PRBs allocated to a single UE compared to CP-OFDM and DFT-s-OFDM. Observation 13: Interlace OFDM and Interlace DFT-s-OFDM are more robust to high speed compared to CP-OFDM and DFT-s-OFDM. Observation 14: Interlace OFDM complicates scheduling but improves scheduling flexibility. Observation 15: In Interlace OFDM, one of the following ways can be selected depending on a use case: - Way 1: Multiplexing within one interlace group. - Pros: Power boosting, DMRS overhead reduction, and increasing the number of multiplexed UEs on different UEs can be achieved while keeping the same spectral utilization as the legacy CP-OFDM/DFT-s-OFDM. - Cons: Phase noise and frequency offset effects cannot be mitigated. - Way 2: Multiplexing between different interlace groups. - Pros: Phase noise and frequency offset effects can be mitigated. - Cons: Spectral utilization is reduced. |  |  |
| [39] | R1-2509349 | Views on 6GR waveforms | CEWiT |
|  | Observation: DFT-s-OFDM provides substantial PAPR gains, especially for higher order modulation schemes. Proposal 1: Support usage of DFT-s-OFDM in DL. Proposal 2: Support for OFDM-OOK kind of waveforms for low end devices. Proposal 3: Support for waveform selection for different time/frequency/physical channels/physical signals. |  |  |
| [40] | R1-2509368 | Discussion on Waveforms of 6GR Air Interface | Rakuten Mobile, Inc |
|  | Proposal 1: In extending NR waveforms and finding alternative candidates for 6GR waveform, - Identifying the target areas for relevant factors based on typical 6G scenarios - Consider at least the following areas: - Compatibility to NR waveforms for MRSS - Complexity and power efficiency. Proposal 2: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following scenarios and channel conditions: - High-mobility DL - including assessment on the resilience to Doppler shifts and inter-carrier interference - Fragmented Spectrum and Sparse Access - including assessment on the adaptability for spectrum allocation in non-contiguous or opportunistic bands. - Low-SNR and cell-edge - including investigation on the performance in coverage-limited areas accounting for robustness and power efficiency. - JSAC and multi-service integration - including exploration of structures supporting simultaneous sensing and communication. Proposal 3: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following aspects for the impacts on spectrum sharing and compatibility with NR: - Waveform coexistence and guard band design - including analysis on the coexistence of CP-OFDM and candidate waveforms within the same band - Numerology and timing alignment - including investigate timing and subcarrier spacing alignment across waveforms - Control and data channel multiplexing - including assessment on control channel decoding, synchronization signal design, and cross-carrier scheduling impacted by waveform diversity. Proposal 4: In extending NR waveforms and finding alternative candidates for 6GR waveform, consider the following aspect for the impacts on complexity and power consumption from the PHYr perspective: - PA efficiency and PAPR - including analysis of PA linearity and energy drain. - Baseband processing load - including assessment of computational complexity of channel estimation, equalization, and demodulation for candidate waveforms. - RF front-end duty cycles - including analysis of RF activity patterns. - RAN4 may be involved in - Thermal and energy budget constraints - including analysis of power impacted of waveform processing under typical traffic loads. |  |  |
| [41] | R1-2509372 R1-2509461 | Discussion on Waveform for 6GR Air Interface | Indian Institute of Tech (M) |
|  | Proposal 3: 3GPP should study the option of enabling mechanisms for PAPR reduction techniques both in CP-OFDM and DFT-s-OFDM. For 6GR, we propose to revisit such techniques that enables PAPR reduction for uplink coverage enhancements. |  |  |
| [42] | R1-2509377 | Considerations on waveform for 6GR air interface | ITL |
|  | Proposal 1: For baseline waveform candidates for 6GR - CP-OFDM should be maintained as the baseline waveform for 6G downlink, preserving continuity with NR, ensuring backward compatibility, and supporting consistent numerology and grid alignment across RATs. - Both CP-OFDM and DFT-s-OFDM should be retained as baseline waveforms for 6G uplink, maintaining alignment with NR and supporting a unified uplink waveform framework across deployment scenarios. Proposal 2: For DFT-s-OFDM as 6GR waveform - DFT-s-OFDM for downlink may be studied as an optional mode for specific use cases. - Low-PAPR enhancement techniques (e.g., FDSS, FDSS-SE, frequency-selective mapping) may be studied as optional uplink features within the OFDM-based baseline. Proposal 3: For DFT-s-OFDM with Rank > 1 for UL as 6GR waveform - Alt 1a: DFT-s-OFDM is used for lower ranks (e.g. 1 and 2) only, CP-OFDM is used for all ranks. - Alt 2a: applicable for all bands Proposal 4: For 6GR waveform with consideration of MRSS Compatibility - MRSS compatibility shall be a mandatory design principle for 6GR waveform development, ensuring seamless coexistence and migration with NR-based systems across shared-spectrum deployments. - CP-OFDM shall be maintained as the baseline waveform for downlink, and CP-OFDM and DFT-s-OFDM shall remain aligned with the NR grid structure on uplink to preserve consistent time–frequency behavior essential for MRSS operation. - Any optional waveform enhancements shall be introduced only if they remain fully transparent to MRSS, without altering grid alignment, coexistence characteristics, or cross-RAT operational behavior. - New waveform families shall be considered only when they provide clear and well-justified performance gains, and only if their introduction does not compromise MRSS coexistence, synchronization, or spectrum-sharing performance. Proposal 5: For unified or multiple waveforms for 6GR - An OFDM-based waveform shall be adopted as the unified baseline for 6GR, maintaining consistency with NR and avoiding unnecessary divergence in device and network implementation. - Multiple baseline waveforms shall not be introduced, as doing so would create fragmentation, increase complexity, and undermine MRSS alignment and coherent multi-band operation. - Optional, OFDM-compatible waveform features may be studied for specific use cases, provided they operate transparently within the unified grid structure and are considered only when they offer clear and well-justified benefits without altering the baseline framework. |  |  |
| [43] | R1-2509410 | Waveform design for 6GR air interface | Tejas Network Limited |
|  | Observation 1: When FDSS-SE is applied to DFT-s-OFDM, it effectively lowers the PAPR, allowing user devices to transmit more efficiently and maintain stronger uplink connections over wider coverage areas. Proposal 1: Support the integration of FDSS-SE with DFT-s-OFDM, as this combination provides an efficient uplink waveform for 6GR that improves power efficiency and extends coverage for power-constrained user devices. Observation 2: Increasing the number of layers causes DFT-s-OFDM to lose its low PAPR advantage due to precoding-induced signal superposition. Link-level simulations are required to determine the maximum layer count at which DFT-s-OFDM can operate effectively without compromising its intended performance. Proposal 2: Study on enabling the DFT-s-OFDM waveform for multi-layer transmission up to layer ‘N’, examining its feasibility, performance trade-offs, and uplink efficiency. FFS: Study on determining the value of ‘N’ through link-level simulations to ensure measurable performance improvement. Observation 3: Dynamic waveform switching enables the transmitter to effectively adapt the waveform to current network conditions, ensuring the balance between coverage, efficiency and throughput. Proposal 3: Dynamic waveform switching allows real-time adaptation of the waveform to meet varying uplink requirements. Observation 4: 20% improvement in downlink coverage allows the satellite to deliver a stronger and more reliable signal, extending connectivity across a wider area. Proposal 4: DFT-s-OFDM is a power-efficient choice for NTN downlink transmission which offers stronger signal coverage for energy-constrained satellites. |  |  |
| [44] | R1-2509413 | OSDM for 6GR | University of Sheffield |
|  | Observation 1: CP-OSDM supports 256QAM in an 8T8R MIMO configuration for 5G-NR at 35 dB SNR. Observation 2: CP-OSDM achieves a 95% throughput at 35 dB SNR in an 8T8R MIMO configuration for 5G-NR. Propo<sal: To further advance the study of CP-OSDM in 6GR compared to CP-OFDM. |  |  |