first3GPP TSG RAN WG1 Meeting #123	 R1-250xxxx
Dallas, USA
17th – 21st November 2025

Agenda item:	11.4.2

Title:	FL summary #2 on modulation, joint channel coding and modulation

Source:	Modulator (Qualcomm Incorporated)

Document for:	Discussion/Decision

# Introduction

From [1], for Physical Layer structure for 6GR, we have the following objective

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

The following email thread is assigned for the discussion

[123-R20-6GR-Modulation, joint channel coding and modulation] Email discussion on Rel-20 6GR-Modulation,joint channel coding and modulation –Jing (Qualcomm)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

In this contribution, we summarize the contributions submitted to agenda item 11.4.2 on modulation, joint channel coding and modulation, and discussion during the meeting.

# Discussion

## Discussion on legacy uniform QAM constellation

According to SID [1], 5G NR modulation should be considered for 6GR. There are various proposals to further enhance modulation. The following tries to capture the discussion on the topic.

| Company | Proposals |
| --- | --- |
| Lekha | Observation 3: Machine learning-aided modulation enables real-time adaptation of modulation schemes by leveraging channel conditions, mobility, and QoS requirements, leading to improved efficiency and robustness. Proposal 2: It is proposed to use machine learning-aided modulation for real-time adaptation of modulation schemes by leveraging channel conditions, user mobility, and QoS requirements to enhance efficiency and robustness. Observation 4: In 6G, modulation schemes are expected to be tailored for specific use cases to maximize performance based on diverse and evolving requirements. Proposal 3: In 6G, modulation schemes are expected to be tailored to specific use cases to meet diverse performance requirements. Higher-order QAM supports eMBB, ML-aided modulation enhances URLLC, and low-complexity schemes like OOK, BPSK, and QPSK are ideal for mMTC and VLC. Advanced applications such as holographic communication can use AI based adaptive modulation, RIS uses RIS modulated QAM, and phase shift keying schemes are preferred for satellite links. |
| ZTE | Observation 1: 5G NR MCS tables are thoroughly evaluated and carefully designed with considerations of multiple aspects, including e.g., AWGN channel and fading channel, EVM performance, spatial multiplexing and rank adaptation. Observation 2: MPR differences across modulation orders are observed for DFT-s-OFDM-based waveform in the uplink. In addition, MPR is dependent on factors such as UE power class, antenna configuration, and the location of RBs within the system bandwidth. Observation 3: Different scenarios adopting different CQI/MCS tables is beneficial to CQI feedback, MCS indication and OTA test effort reduction. |
| Panasonic | Proposal 1: Any enhancements must demonstrate clear and justified advantages over 5G NR modulation schemes. Proposal 7: Resource-specific modulation order assignment should be studied at least with carrier / BWP granularity.  FFS: Finer granularity such as PBG / RB is further considered. Proposal 8: For improved MCS table, the need to introduce flexible MCS table configurability should be studied. Proposal 9: For improved MCS table, the need to integrate repetition in the MCS table than separate field of the repetition number indication should be studied. |
| LGE | Observation 2: Mixed modulation can support various spectral efficiency more efficiently with improved reliability than the conventional NR modulation scheme. Proposal 3: RAN1 should study the use of mixed modulation for 6GR |
| DCM | Observation 5: Overlapped SE is supported in NR for very limited MCS table and MCS index. For 6G, the improved MCS table with overlapped SE should consider the tradeoff between performance and system complexity. Proposal 3: Performance comparison among different SE enhancement schemes (GCS/PCS/improved MCS table) is necessary. |
| Oppo | Observation 18: For the enhanced AMC supporting multiple combinations of modulation order and coding rate with the same spectral efficiency, the benefits of enhanced AMC should be further studied when integrating the UE CQI feedback procedure. |
| Nokia | Proposal 1: • For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for data channel • For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for CP-OFDM for data channel • For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported for DFT-s-OFDM for data channel • FFS: Enhancements and other modulation schemes Proposal 8: Study the possibility of allowing spectral efficiency operating points achievable with more than a single modulation order. |
| Ericsson | Observation 1 Uniform constellation based modulation schemes have been used from 3G to 5G with increased modulation order, including up to 1024-QAM for downlink and 256-QAM for uplink. It has technological merits to cover a wide variety of services under different channel conditions. Proposal 1 Support 5G NR uniform QAM constellation as the basis modulation scheme for 6G. Proposal 2 Support at least QPSK, 16 QAM, 64 QAM and 256 QAM for uplink. Proposal 3 Support at least QPSK, 16 QAM, 64 QAM, 256 QAM and 1024 QAM for downlink. Proposal 4 High level decisions on supporting 5G modulation schemes for 6G shall be made at the early phase of Release 20. Proposal 5 High level decisions on new modulation schemes beyond 5G technology shall be made as early as possible in Release 20 after extensive evaluation results are available. |
| Vivo | Observation 8: The current NR UL MCS table does not consider the MPR. The switching points of the modulation orders can be further optimized. Observation 9: The best design of switch points in UL MCS table is different from the case when MPR is not considered.  When MPR is not considered, the two MCS entries (QPSK with 0.66CR and 16QAM with 0.33 CR) specified in NR are better than other potential combinations of modulation and code rates with similar SE.  When MPR is considered, QPSK with 0.74 code rate (SE 1.48) and 0.85 code rate (SE 1.7) are 1dB and 0.2dB better than the current MCS 11 (16QAM with SE 1.48) and MCS 12 (16QAM with SE 1.7). Proposal 3: Study the impact of MPR on UL MCS table design for improved MCS table. |
| CMCC | Proposal 1: Adaptive MCS selection for each SE can be considered. |
| HW | Proposal 3: Adaptability of Modulation-Related Parameters with respect to the channel characteristic needs to be carefully studied and reported with respect to the feasibility and signalling overhead. Observation 1: For a given spectral efficiency (SE), the optimal MCS parameters (modulation and code rate) are different between AWGN and fading channels. • An MCS table that is optimized for AWGN channels may suffer performance loss in fading channels, while an alternative MCS parameters with different modulation and code rate (with the same SE) could offer significant performance gain (e.g., up to 1.5dB). Proposal 1: Study enhanced adaptive modulation and coding (E-AMC) schemes to select the optimal MCS based on channel characteristics for performance improvement. |
| Xiaomi | Proposal 1: • For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel • For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel • For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel. Observation 8: The MCS combinations in each NR MCS table is predefined and cannot be changed. Observation 9: The selection of MCS tables in NR has become increasingly complex, especially for PUSCH in later releases. Proposal 10: Study a more flexible mechanism for defining and selecting MCS tables for 6GR. |
| Hanbat | Proposal 1: RAN1 to study the use of π/2-BPSK DFT-s-OFDM as a supporting modulation scheme in UL data transmission for extremely low SINR cases as well as initial steps for UL control channel. |

A few proposals mentioned in the contributions include

Support NR uniform constellations: Xiaomi, Nokia, Ericsson (except pi/2-BPSK in UL)

Resource specific modulation order selection: Panasonic (subband specific down to PRG/RB level), LGE (spatial layer specific)

MCS table design:

  - More configurability: Panasonic

  - Semi-static selection between multiple MCS tables: ZTE, Xiaomi

Further study AMC (MCS table with overlapped SE): DCM, HW, Oppo, Nokia, CMCC

For UL, consider MPR in the MCS table design: vivo

### First round discussion

Discussion 2.1-1 (replaced and closed)

Companies are encouraged to evaluate the proposal to allow a single spectrum efficiency point to be supported by multiple MCS entries with different modulation orders with uniform QAM. When providing results, companies are recommended to provide the following information

- Details on the overlapping MCS table design

- Performance benefit under different channel and rank assumptions

- MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency.

- Impact to UE CSI feedback

| Company | Comments |
| --- | --- |
| Xiaomi | We are OK to study a single SE associated with multiple MCS entries. But we should also list other aspects as proposed by many companies. For 6GR, we should have a proper study on whether the MCS tables are predefined and fixed as NR, and if so how to select the MCS tables. Companies are encouraged to evaluate the following enhancements to MCS design - A single spectrum efficiency point to be supported by multiple MCS entries with different modulation orders with uniform QAM. When providing results, companies are recommended to provide the following information - Details on the overlapping MCS table design - Performance benefit under different channel and rank assumptions - MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency. - Impact to UE CSI feedback - Configurability of MCS entries of a MCS table. When providing results, companies are recommended to provide the following information - The necessity, benefits and challenges - Details on the MCS table design - Impact to UE CSI feedback if any - MCS table selection mechanism if multiple predefined MCS tables are supported for 6GR When providing results, companies are recommended to provide the following information - The necessity, benefits and challenges - Details on the MCS table selection design |
| OPPO | - Similar to the results collection campaign for constellation shaping, simulation results for overlapping MCS can also be captured in a unified table. The content of spreadsheet should be first discussed for the ease of future calibration. |
| CMCC | - Adaptive MCS selection for each SE can be studied. Before considering the MCS table design/MCS selection mechanism/impact to CSI feedback, the necessity of adaptive MCS selection should be verified by simulation. When providing simulation results, companies are recommended to provide: - For a given SE, BLER performance across multiple MCS under fading channel and AWGN channel. - Fading channel assumption should be clarified. |
| vivo | In our understanding, one particular issue we need to consider is the impact of MPR on UL MCS table design. We provide detailed analysis and results in our contribution R1-2508435. Specifically, for MCS points with same SE, whether we consider MPR or not will have different best MCS. Hence we suggest to revise the second bullet as following to reflect this study. - “Performance benefit under different channel, rank and UE Tx power assumptions” |
| Panasonic | We share the similar view to Xiaomi that in addition to study a single SE associated with multiple MCS entries, other aspects should also be studied. We support Xiaomi’s addition on configurability of MCS entries of a MCS table and MCS table selection mechanism. |
| DOCOMO | Fine with the proposal. Furthermore, as stated in our contribution, the trade-off between performance and complexity should be considered, e.g., by limiting the number of MCS entries with overlapped SE in one table. It is recommended to achieve some preliminary consensus among companies on this limitation before companies providing results. |
| Samsung | We are open to studying the concept of overlapping MCS tables where a single spectral-efficiency point can be supported by multiple modulation orders. However, we prefer not to limit the discussion to uniform QAM, as the outcome of the constellation shaping study (both probabilistic and geometric) may directly influence MCS design. It would therefore be more efficient to revisit MCS design and selection mechanisms after the shaping discussion is concluded, since addressing both topics in parallel could make the overall discussion unnecessarily complex and less focused. |
| Ericsson | Since most companies support uniform QAM or study improved MCS table based on uniform QAM, we’d like to add “Support uniform QAM as baseline for 6GR ” to the proposal. |
| Huawei, HiSilicon | - Firstly, we support the proposal from Xiaomi, Ericsson and Nokia to support uniform QAM as the basis for 6GR modulation. We should follow similar principle as the discussion for channel coding, the reuse of NR modulation scheme should be supported by 6GR. Support 5G NR uniform QAM constellation as the basis modulation scheme for 6G: - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for data channel - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for CP-OFDM for data channel - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for DFT-s-OFDM for data channel - In the last meeting, the improved MCS table is agreed to be evaluated with shaping solutions. If an improved MCS table is used for a shaping scheme, the performance of uniform QAM using the same MCS table should be also provided, so that the group can know how much gain comes from shaping and how much gain comes from AMC, rather than mixing the shaping gain and AMC gain together. Regarding the impact on UE CSI feedback, it is not clear on what would be the intention, but it should be a common issue for all modulation enhancements including enhanced AMC and shaping solutions. If needed, we should have a separate proposal applicable for all modulation enhancements. Based on the above reasons, some revisions highlighted in purple are proposed: Companies are encouraged to evaluate the proposal to allow a single spectrum efficiency point to be supported by multiple MCS entries with different modulation orders with uniform QAM. When providing results, companies are recommended to provide the following information - Details on the overlapping MCS table/entries design - Performance benefit under different channel and rank assumptions - MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency. - For evaluating shaping scheme, if an improved MCS table is used, the performance of uniform QAM using the same MCS table shall be provided. - Impact to UE CSI feedback |

### Second round discussion

Discussion 2.1-1A

Companies are encouraged to evaluate the proposal to allow a single spectrum efficiency point to be supported by multiple MCS entries (with different modulation orders with uniform QAM or with different shaping parameters for PS/GS). When providing results, companies are recommended to provide the following information

- Details on the overlapping MCS table design

- Performance benefit under different channel and rank assumptions

- MCS selection mechanism across multiple MCS corresponding to the same spectrum efficiency.

- Impact to UE CSI feedback if UE feedback is needed for gNB to select between multiple MCS entries corresponding to the same SE.

FL notes: There are proposals to study multiple MCS table selection and configurable MCS table. The FL believes these can be separate discussions and can be done when more details become available.

Please provide your view below:

| Company | Comments |
| --- | --- |

## Discussions on uniform constellation extension to 4K in DL and 1K in UL

We received the following proposals on high order uniform QAM.

| Company | Proposals |
| --- | --- |
| Lekha | Observation 1: The comparison of PAPR versus CCDF for 256-QAM, 1024-QAM, and 4096-QAM modulation schemes shows that DFT-s-OFDM consistently achieves a lower PAPR than OFDM. Specifically, at a CCDF of 10^(-3), DFT-s-OFDM exhibits a 2.75 dB reduction in PAPR compared to OFDM, demonstrating improved power efficiency across all modulation orders. Observation 2: Higher-order QAM schemes like 4096-QAM significantly improve spectral efficiency but are highly sensitive to channel impairments due to their increased SNR requirements. Proposal 1: To enable practical deployment of higher order modulation schemes in 6G, advanced adaptive signal processing techniques and intelligent link adaptation strategies should be developed to dynamically select modulation orders based on real-time channel conditions. |
| Spreadtrum | Proposal 1: It is suggested that further comprehensive simulations are needed to help quantify these potential SINR thresholds and evaluate the actual system-level spectral efficiency gains, rather than focusing only on the theoretical peak. |
| CATT | Observation 1: 4096-QAM offers significant spectral efficiency gains but suffers from high PAPR and increased sensitivity to frequency and phase noise, leading to greater implementation challenges. Proposal 1: 1024-QAM is recommended for downlink reuse, while uplink modulation should be limited to a maximum of 256-QAM due to UE transmit power constraints. |
| Samsung | Observation 1: As the modulation order increases, the equivalent EVM requirement tightens by roughly 6 dB for every ×4 increase in constellation size from 256QAM. Observation 2: The conservative EVM estimates decrease from 5.52 % for 256-QAM to 2.88 % for 1024-QAM and 1.52 % for 4096-QAM, indicating a consistent tightening of the EVM requirement as the modulation order increases. Proposal 1: For downlink, in order to increase spectral efficiency in indoor and high‑SNR regions, we propose supporting 4K‑QAM and initiating study to define EVM requirements in RAN4 and expand the usable SNR range for reliable 4K‑QAM operation. Proposal 2: For uplink, in order to increase efficiency spectral in indoor and high‑SNR regions, we propose supporting 1K‑QAM and initiating study to define EVM requirements in RAN4 and expand the usable SNR range for reliable 1K‑QAM operation. |
| ZTE | Proposal 1: 1024QAM can be considered for 6GR UL. |
| DCM | Observation 6: Uniform QAM with higher modulation order is challenging at practical systems due to the extremely high SNR requirements and non-robustness over RF impairments. |
| Oppo | Observation 1: 4096QAM requires approximately -38 dB EVM, reflecting a 6 dB reduction compared to the NR 1024QAM EVM requirement. This stringent EVM requirement for 4096QAM would cause deployment barriers and further increase hardware cost. Observation 2: In both UMa and UMi scenarios, only a very limited subset of UEs’ link quality is enough to support 4096QAM. It is expected that 4096QAM may only bring marginal gains on the system throughput and cell average spectral efficiency. Proposal 1: Up to 1024-ary constellation for DL and up to 256-ary constellation for UL are supported for 6GR modulation scheme. |
| Nokia | Proposal 2: Support uniform 1024QAM in Uplink. Observation 1: Uniform 4096QAM in DL drives computation complexity and the power consumption of R-ML MIMO detection at the UE side. |
| Ericsson | Observation 1 The PA backoff required for 1024 QAM is 10.22 dB for DFT-s-OFDM and 13.00 dB for CP-OFDM. Proposal 1 For study of 1024 QAM in UL, send an LS to RAN4 to request input on assumptions on the device side for power backoff requirement. |
| Vivo | Observation 10: Regarding 1024QAM for UL and 4096QAM for DL,  For 1024QAM, the operating SNR for 1024QAM is around 25 dB with 2%/2% Tx/Rx EVM, and approximately 36 dB with 3%/4% Tx/Rx EVM.  For 4096QAM, the operating SNR range for 4096QAM is around 27 dB to 33 dB in AWGN channel, and the operating SNR range for 4096QAM is around 30 dB to 37 dB in TDL-A channel.  For 4096QAM, the Tx EVM and Rx EVM should both be below 2% to yield the performance gains over 1024QAM, regardless of AWGN or TDL-A channels. |
| CMCC | Observation 1: For higher-order QAM, such as uniform 4096QAM for DL and uniform 1024QAM for UL, the required power back-off undermines the data rate advantage, and the extreme hardware demands make implementation impractical. |
| HW | Proposal 1: Observations about the performance for higher order modulation should be made together with other aspects especially including the applicable scenarios, associated restrictions, and challenges, rather than observations solely on performance benefit. Full study should be performed before making decision on whether to introduce a higher modulation order for 6G. |
| Qualcomm | Observation 1: Fixed wireless access could be one potential use case for higher order QAM extension. |
| Xiaomi | Proposal 4: For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference  Net Gain [dB] = SNR degradation/gain relative to the reference @10% BLER - PAPR degradation  A realistic PA model should be used  RAN4 early inputs are needed, such as the time/freq synchronization assumption, phase noise assumption and quantization loss assumption etc.  FFS other evaluation assumptions. Proposal 5: For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, at least the following associated restrictions should be studied.  The applicable frequency ranges  The applicable device type(s) if any  The number of MIMO layers  The scaling factor to the maximum data rate  FFS other potential associated restrictions Proposal 6: For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, RAN4 early involvement is required. |
| Apple | Observation 1: Higher order QAM in DL and UL are sensitive to RF impairment. Practical RF impairment modeling including phase noise model, PA model and IQ imbalance model, are needed for the evaluation. Proposal 1: To avoid duplicate effort, the evaluation of 4KQAM for DL and 1KQAM for UL should depend on RAN4 progress and request. |
| MTK | Observation 1: Based on the possible SNR in the field and the LLS evaluations for the current modulation schemes, 4K-QAM is not feasible for DL. Observation 2: Due to lower SNR (compared to DL SNR) in UL and the limitation at the UE to achieve low EVM, it is expected that 1024QAM is not feasible in UL as well. |

Summary of views on 4K QAM DL and 1K QAM UL:

Support: Lekha, Samsung, Qualcomm

Support UL 1K QAM only: ZTE, Nokia

Needs careful study: Spreadtrum, Apple, Ericsson, vivo, HW, Xiaomi

No need or not practical: CATT, Oppo, CMCC, MTK, DCM

### First round discussion

Discussion 2.2-1 (replaced and closed)

For high order QAM link level performance study, recommend to use the following format for performance reporting.

| Spectrum efficiency | Assumed EVM | AWGN channel 10% BLER SNR | AWGN channel 1% BLER SNR | Fading channel 1 10% BLER SNR | Fading channel 1 1% BLER SNR |
| --- | --- | --- | --- | --- | --- |
| SE1 |  |  |  |  |  |
| SE2 |  |  |  |  |  |
| SE3 |  |  |  |  |  |
| Other parameters | Channel estimation assumption (genie or realistic) Fading channel assumptionAssumed residual freq offset range Number of allocated RBs |  |  |  |  |

| Company | Comments |
| --- | --- |
| Xiaomi | Based on RAN4 LS in R1-2508314, RAN4 will model and evaluate the performance and the implementation complexity of higher-order modulations. Considering the evaluation is highly related to Tx/Rx EVM assumptions and PA back-off, RAN4 inputs may be required, and we should avoid duplicated work in different WGs. So, it’s better to first clarify which work should be done in RAN1 and which should be left to RAN4. |
| OPPO | Support in principle. Based on our understanding, the EVM metrics should be further clarified, i.e. Tx EVM and/or Rx EVM. Besides, before collecting performance results, the responsible working group for performing EVM evaluations should be first discussed to avoid duplicate work between RAN1 and RAN4. |
| CMCC | From our understanding, higher-order modulation may have more stringent EVM requirements and higher PAPR and thus lead to MPR/A-MPR increase, which can partially offset the data rate gains achieved by the higher-order modulation. So, it is essential to have RAN4 involution early to define reasonable requirements and evaluate the feasibility for higher-order modulation. |
| vivo | We are not sure why we need to look at 1% BLER. Further, in Rel-17 study on DL 1KQAM, the metric we used is throughput. Specifically, we look at the switch SNR point for throughput compared with lower modulation. Hence we suggest to add one new column “switch SNR point for throughput compared with lower modulation”. |
| Lenovo | We are fine with the proposal |
| CATT | We need to include the impact of the PAPR for higher order modulation. |
| Tejas | Support in principle, for fading channel 1% BLER may not be suitable as some of the higher SE MCS point will require very high SNR or may not meet 1% BLER target. |
| ZTE, Sanechips | Thanks to FL for providing the template of results collection for higher order modulation. We have several suggestions on the format for performance reporting: - It is necessary to clarify the spectral efficiency under which modulation order in one table, since we have 4k QAM for downlink and 1k QAM for uplink to evaluate. And we are also fine if we separate results for 4k QAM for downlink and results for 1k QAM for uplink in two tables. - Add 'Receiver type' to other parameters, since we notice that different companies provides different simulation assumptions for receivers. - It is necessary to provide the throughput performance for downlink 4k QAM and for uplink 1k QAM. Besides, for evaluating the applicable scenario of downlink 4k QAM and for uplink 1k QAM, SNR geometry should be also considered. |
| IDC | Support the proposed results-collection format. For completeness, receiver type and realistic EVM/PAPR assumptions should be captured, and coordination with RAN4 on EVM requirements is recommended to ensure consistent evaluation of 1024QAM and 4096QAM. |
| Samsung | We agree with the proposed performance reporting metrics and format. In particular, EVM should be studied rigorously, as it is a key parameter that directly determines the feasibility of high-order QAM in practical systems. Providing fading-channel performance results can be useful for reference and qualitative comparison. However, such results should be interpreted carefully, since fading-channel simulations are often difficult to calibrate and reproduce consistently across different companies. |
| Huawei, HiSilicon | The applicable scenarios, applicable frequency, and target device type are still not clear and these aspects should be proposed/clarified by the proponents and studied by the group first. Without these inputs and RAN4 inputs on the EVM, phase noise etc., the link level simulation for performance gain cannot bring useful information regarding whether benefit/gains could be obtained in deployment/field. |

Discussion 2.2-2

For applicability of high order QAM for DL 4K QAM and UL 1K QAM, please provide the target deployment scenario (hotspot, FWA in macro) follow evaluation methodology system evaluation layout with possible modification, and the relative gain when the high order QAM is used.

| Company | Comments |
| --- | --- |
| Xiaomi | Similar comment as above. |
| OPPO | Support in principle, but the target deployment scenario and evaluation methodology are unclear for us. First, per agreements of AI 11.2, whether FWA is supported in Urban macro/Sub-urban macro deployment scenario has not been determined. Additionally, the UE assumptions for hotspot are also unknown. This proposal should be postponed until clear deployment scenarios have been made. Second, the baseline to derive “relative gain” is also unclear. It should be made clear whether legacy MCS table or all possible combinations below DL 1024QAM/UL 256QAM serve as the baseline. In addition, the evaluation metrics used in the system-level simulation should also be clarified. |
| CMCC | From our understanding, higher-order modulation may have more stringent EVM requirements and higher PAPR and thus lead to MPR/A-MPR increase, which can partially offset the data rate gains achieved by the higher-order modulation. So, it is essential to have RAN4 involution early to define reasonable requirements and evaluate the feasibility for higher-order modulation. |
| Samsung | We understand that 1K-QAM for UL and 4K-QAM for DL may be applicable to certain specific scenarios or applications (e.g., hotspot or FWA). However, in the RAN1 study, we believe it is not necessary to target specific applications. The modulation study should aim to establish general and representative performance characteristics, rather than being limited to experimental setups tailored to particular deployment scenarios. This approach ensures that the study remains broadly applicable and not biased toward specific use cases. |
| Huawei, HiSilicon | Currently we don’t see benefit. But we are open for companies to further clarify the applicable scenarios, applicable frequency, and target device type. |

### Second round discussion

Discussion 2.2-1A

For high order QAM link level performance study, recommend to use the following format for performance reporting.

| (modOrder, coding rate, SE) | Assumed TX/RX EVM | Channel 1 | Channel 2 | Channel 3 |
| --- | --- | --- | --- | --- |
|  |  | SNR to achieve target BLER | SNR to achieve target BLER | SNR to achieve target BLER |
| (10, 900.5, 8.7939) for DL 4K QAM evaluation (8, 916.5, 7.1602) for UL 1K QAM evaluation | Legacy EVM for 4K QAM for DL and 1K QAM for UL respectively |  |  |  |
| (10, 948, 9.2578) for DL 4K QAM evaluation (8, 948, 7.4063) for UL 1K QAM evaluation | Legacy EVM for 4K QAM for DL and 1K QAM for UL respectively |  |  |  |
| SE1 |  |  |  |  |
| SE2 |  |  |  |  |
| SE3 |  |  |  |  |
| Other parameters |  |  |  |  |

Notes:

- For assumed TX/RX EVM, before we receive any concrete numbers from RAN4, companies can provide their assumptions. One example can be 6dB tighter than the EVM of 1K QAM for DL and 256QAM for UL.

- Other parameters include: Channel estimation assumption (genie or realistic), channel configurations (AWGN, SISO, SIMO, MIMO and TX/RX antenna configurations, channel types, number of spatial layers,), assumed residual freq offset range, number of allocated RBs, etc

- Two highest MCS points in DL 1K QAM and UL 256QAM in NR added in the table for reference.

Please provide your view below:

| Company | Comments |
| --- | --- |
| vivo | As we commented previously, the R17 study on the feasibility of 1K QAM looks at the switching point of throughput from lower modulation to higher modulation. Thus, we need to look at the crossover SNR achieved by the high order modulation over the low order modulation. We suggest to capture the crossover SNR in the table. Moderator: This is a good point. We do need a few baseline SE points with 1K QAM in DL and 256QAM in UL as reference. |

## Discussions on shaped constellations

For constellation shaping, we received the following proposals

| Company | Proposals |
| --- | --- |
| Lekha | Observation 7: Joint probabilistic and geometric shaping improves communication efficiency by lowering average and peak symbol powers, which reduces PAPR and enhances power amplifier performance. By jointly optimizing symbol probabilities and constellation geometry, it enables near-capacity transmission with higher achievable rates and better BER performance at the same SNR. Observation 8: Constellation shaping enhances communication performance by optimizing either the geometry or probability distribution of modulation symbols, with recent machine learning methods enabling joint geometric and probabilistic shaping to maximize mutual information (I(X; Y)). This approach performs near channel capacity on AWGN channels, outperforms existing methods on various channels, and requires no prior knowledge or assumptions about the channel, making it suitable for unknown or non-differentiable channel models. Proposal 6: An adaptive scheme that selects between uniform modulation, probabilistic shaping, geometric shaping, or joint probabilistic and geometric shaping based on different SNR ranges can be implemented to achieve optimal performance. |
| Spreadtrum | Observation 2: PS and GS both introduce substantial complexity and latency at the receiver. Observation 3: PS and GS both introduce new CSI processing overhead and increase UE storage. Proposal 2: Standardization of constellation shaping (PS/GS) should proceed only after comprehensive simulation and consideration verify that its performance benefits justify the full implementation costs, including complexity, latency, PAPR impact, and computational overhead for CSI processing and link adaptation. |
| CATT | Proposal 2: It is recommended that constellation shaping is considered when the spectral efficiency is above 3 bits per two dimensions symbol, or the constellation size is not less than 64. |
| Samsung | Observation 7: Systematic PS does not incur any changes in coding and modulation chain in 5G NR. Observation 8: If the probability of the 2nd MSB-level bit being 0 is distribution-matched to be greater than 0.5, it is possible to obtain a probability distribution of the constellation that is close to Gaussian while maintaining the 5G system. Proposal 3: Study 1D-NUC as a 6GR constellation shaping scheme. Proposal 4: Study systematic PS as a 6GR constellation shaping scheme. |
| ZTE | Proposal 2: Consider performance and complexity trade-off in the study of shaping modulation for 6GR. Observation 1: The following three aspects need to be considered for CCDM algorithms:  Precision mismatch between transmitter and receiver  Serial process for encoding and decoding  Performance loss in small TBSs Observation 2: IR-HARQ performance may degrade when PAS is used. Proposal 1: The following issues for PAS can be studied and evaluated: • Impact on channel coding chain • Serial process and storage caused by DM algorithms • Initial and retransmission performance |
| Panasonic | Proposal 5: RAN1 can assess the need to introduce non-uniform constellation without increasing implementation complexity for MPR / PAPR reduction and/or spectral efficiency improvement. Proposal 6: AI/ML-based constellation shaping should be firstly discussed in AI/ML related agenda. |
| LGE | Proposal 1: RAN1 should consider NUC as a candidate modulation scheme for 6GR. Proposal 2: Study the applicability of the constellation shaping for modulation orders supported in 5G NR (e.g., from 64 QAM up to 1024 QAM). |
| ETRI | Observation 1: For geometric shaping (GS), the following observations can be made: - For 2D-GS, the performance gains are generally higher than those of 1D-GS; however, the implementation complexity considerably increases as modulation order becomes higher (≥256QAM). - For 1D-GS, the performance gains are acceptable in higher modulation orders (256~4096QAM), and the implementation complexity remains manageable even for higher modulation orders. - GS schemes can be easily applied to the existing 5G NR BICM chain and MCS table in a backward compatible manner. For probabilistic shaping (PS), the following observations can be made: - PS would inevitably require fundamental modifications to the 5G NR BICM chain, including the redesign of key components (e.g., bit interleaver, scrambler, systematic bits reordering, etc.). - PS would require large memory resources (ESS) and introduce higher latency (CCDM), which will be challengeable in practice. Proposal 1: RAN1 to consider the following proposals for 6G modulation schemes: - Low-order modulations (QPSK–16QAM): Consider to maintain uniform constellations, as shaping methods do not provide meaningful gains for low-order modulations. - 64QAM: Consider 2D geometric shaping (2D-GS), since 2D-GS offers better performance gains compared to 1D-GS, while maintaining manageable complexity for 64QAM. - 256QAM–4096QAM: Consider 1D geometric shaping (1D-GS), as 1D-GS achieves acceptable shaping gains for higher-order modulations with low complexity. - Consider to reuse 5G NR MCS tables. Given uniform constellations defined in 5G NR as the baseline, new shaping modulations can be signaled as optional modes. To support higher throughput, an additional MCS table for higher modulations (e.g., 4096QAM) can be defined. Proposal 2: RAN1 to consider the following for further evaluations of 6G modulation candidates: - BLER performance evaluation at low error rates (e.g., 10-4 ~ 10-5) - Details of fading channel evaluations (e.g., Rayleigh, etc.) - Performance evaluations depending on variable code block sizes (including short/long length, referring to the channel coding agenda) |
| IMU | Observation 1: Both GCS and PCS can significantly improve spectral efficiency and BLER-vs-SNR over uniform constellations across practical channels, with achievable gains depending on modulation order, distribution-matcher design and blocklength, receiver complexity, and channel conditions. Observation 2: GCS (and GCS+PCS) may incur significantly higher demodulation complexity because LLR evaluation cannot be decoupled into independent I/Q searches. Low-complexity receiver support (via near-separable mappings, structured labeling, bounded-complexity 2-D demappers, or efficient approximations) is therefore critical for shaped constellations to be viable across device classes. Proposal 1: Study PCS/GCS for enhancing communication performance, prioritizing minimal-intrusion designs that reuse NR LDPC, interleaving, scrambling, bit-collection, and legacy QAM mapping, and are implementation-friendly in terms of complexity, latency, and storage. Report BLER/throughput under AWGN and 3GPP fading (SISO, MIMO rank=1 and >1), plus TX/RX complexity. Proposal 2: RAN1 to study receiver architectures for GCS and GCS+PCS that maintain low complexity and latency, with priority on shaped constellations that are demappable with minimal complexity while reusing the existing NR RX chain as much as possible. |
| DCM | Observation 1: By limiting the constellation points and bit labelling as a subset of high-order QAM constellation points, QAM-based constellation shaping (QAM-CS) could provide lower PAPR than QAM and lower implementation complexity than Geometric Constellation Shaping (GCS) and could be considered as a candidate of 6G modulation schemes. Proposal 1: Study QAM-based Constellation Shaping (QAM-CS) for PAPR reduction. Performance gain over UE/gNB complexity should be assessed. Proposal 2: Low-PAPR modulation based on constellation shaping should be discussed in Modulation agenda. |
| Oppo | Proposal 2: Support geometrical shaping for 6GR modulation scheme. Proposal 3: For geometrical shaping, further study 1D-NUC and/or 2D-NUC for 6GR modulation scheme. |
| Lenovo | Observation 2: The non-paired constellation framework retains legacy QAM demapping at the receiver, while only the transmitter uses NUC, specifically designed for this non-paired framework, i.e., NP-NUC, for mapping. Thus, receiver updates are not necessary to realize geometric shaping gains. Proposal 2: Study a non-paired constellation approach for geometric shaping, where the transmitter employs NP-NUC, while the receiver uses the legacy QAM for demapping. Observation 5: Both NUC and NP-NUC approaches for constellation shaping need to modify the MCS tables to specify an NUC for each MCS index. Also, the combination of modulation order and coding rate corresponding to each MCS index in the uniform QAM MCS tables might need to be modified. Proposal 7: The trade-off between the potential gain of PCS techniques and their corresponding impact on the processing chain w.r.t. latency, complexity, PAPR, (sub-)optimality for retransmission should be considered. |
| Nokia | Proposal 3: Geometrically shaped constellations shall be a down-selection (down-sampling) of a higher order uniform QAM. Sign symmetry per I/Q shall be maintained. Proposal 4: Evaluation of geometrically shaped constellations shall take the quantization into account. The quantization can be considered by assuming a maximum number of bits, e.g. 6 or 8 bits per I/Q.Proposal 5: Geometrically shaped constellations shall be quantized based on a higher order uniform QAM constellation. To improve the quantization error, the even amplitudes between two legible odd amplitudes are considered as legible quantization result. Proposal 6: Strive to reduce the number of different geometrically shaped constellations of the same order. Observation 2: The gains observed over the AWGN channel thanks to the PS do not carry over to other channels, e.g., fading channels. Even under the assumption of perfect CSI at the receiver, the performance may become worse than the uniform QAM, which provides very robust performance across different channel conditions. |
| IDC | Observation 2: PCS can be easily integrated with NR shared channel processing, reusing LDPC encoding and decoding schemes Proposal 1: Study PCS as potential candidates for 6G joint coding and modulation with uniform QAM of NR as the baseline. |
| AT&T | Proposal 1 The 6GR study on performance/complexity tradeoff of non-uniform constellations based on geometric shaping and/or probabilistic shaping is focused on high-order modulation values. Proposal 1 For 6GR proposals on non-uniform constellation and probabilistic shaping, prioritize designs that leverage the existing NR modulation architecture at both the transmit and receive chains. Proposal 2 For 6GR proposals on non-uniform constellation and probabilistic shaping, evaluation is needed for scenarios including SU-MIMO. Proposal 3 For 6GR proposals on non-uniform constellation, further emphasis is needed on the scalability of the constellation to include different values of constellation points, in addition to the underlying MCS selection procedure needed at the receiver side. |
| DeepSig | Observation 1: AI/ML based methods of constellation shaping are a form of probabilistic shaping that operates through higher order mappings. Observation 2: AI/ML-native methods can constitute alternative MCS tables which span a comparable or extended range of information rates compared to existing MCS and modulation/constellation choices. Proposal 1: Ensure study scope allows for a range of approaches which fit nicely into the constellation shaping exploration, including AI/ML based methods for optimizing transmit modulation and symbol choice. (At the end of the day, these are also optimizing for probabilistic shaping based on the cross-entropy loss of the recoverable LLRs). Proposal 2: Consider the impact of multi-RE constellation shaping methods (e.g. MRE-CS and MRE-PF-CS) in addition to purely single RE constellation shaping methods under the same evaluation scenarios and KPIs. Proposal 3: Consider the joint impact of DMRS allocation choices and overhead reduction – for example by supporting a zero-DMRS mode for comparison as well, as constellation shaping impacts the need for reference signals. Proposal 4: Consider evaluation scenarios which include site-specific deployments and reference geometries such as ray-tracing environments in addition to statistically whitened CDL channel models. (Where more impactful results might be obtained to show how shaping might benefit specific scenarios more or less) |
| Vivo | Observation 1: AI optimized constellations selected from 4096QAM can achieve similar performance gain compared to AI optimized constellations without constraint. Proposal 1: Support to study AI/ML-based/optimized NUC modulation, including constellation design and corresponding low complexity demodulation scheme. Proposal 2: To minimize the potential signalling overhead and facilitate EVM requirements definition for transmitting/updating constellation, a low-order AI optimized constellation can be selected from the high order QAM constellation points, e.g., from 4096QAM. Observation 2: Probabilistic shaping may have great impacts on the coding chain, such as the channel coding module, and some new building blocks are also needed. Large spec efforts could be needed. Further, scrambling after coding will defeat the potential gain of PAS as the adding of scrambling bits will change the probability of modulation symbols. Observation 3: Compared to legacy QAM modulation, both PAS with different DM structures and non-direct mapping-based PS have significantly higher computational complexity, and/or storage complexity, and/or processing latency. |
| CMCC | Observation 2: For geometric shaping, the constellation design should ensure universal performance across diverse channel conditions. Observation 3: Geometric shaping can reuse the 5G procedures for symbol mapping at the transmitter and symbol demapping at the receiver. Observation 4: For geometrically-shaped constellations, if the constellation map relies on a pre-defined lookup table, the memory and access overhead at the transmitter and the demapping complexity at the receiver should be considered, especially for 1024 and higher-order schemes. Observation 5: Probabilistic shaping can reuse the 5G constellation map. Observation 6: Probabilistic shaping requires a redesign of the channel coding and modulation chain. Observation 7: The core innovation of probabilistic shaping is the novel bit generation process, while the final step of mapping bits to a constellation point remains identical to legacy QAM. |
| HW | Proposal 2: Apply the following rules for collecting results and for summarizing observations in the study of 6G modulation: - The performance results should be reported together with other aspects in the agreement, including at least transmitter and receiver complexity, latency impact and parallelism implementation, and storage requirements;  The template for collecting results should include columns of performance gains, transmitter and receiver complexity, storage overhead and latency impact;  For fair comparison and consensus building, the performance gains concurrently with a transparent analysis of the corresponding transmitter and receiver complexity, storage overhead and latency impact shall be considered to draw observations; - Observations are made for performance gains together with other aspects in the agreement, including at least transmitter and receiver complexity, latency impact and parallelism implementation, and storage requirements. Observation 6: According to theoretical analysis, optimized modulation order and code rate combinations offers superior benefits (e.g., up to 1.5dB) compared to pure constellation shaping techniques (e.g., less than 0.2dB) under fading conditions. |
| Qualcomm | Observation 2: A typical AC encoding implementation of CCDM is serial in the output sequence length, with encoding operations including addition, subtraction, comparison, multiplication, and division arithmetic. Observation 3: A typical ESS encoding implementation requires fixed storage, and is serial in the output sequence length, with encoding operations including addition, subtraction, comparison arithmetic, and table lookups. Proposal 1: Study DM enhancements toward lower complexity and higher throughput. Observation 4: ESS generally has smaller rate loss than CCDM for a given output sequence length. Proposal 2: RAN1 to study DM schemes to achieve optimized performance/rateloss and complexity/latency trade-off. Observation 7: Probabilistic shaping can re-use systematic bits priority mapping for unshaped bit levels in bit collection and interleaving. Observation 8: Scrambling of unshaped bits, including parity bits, is compatible with probabilistic shaping. Proposal 3: PS shall may back to uniform QAM transmission for HARQ retransmission. Proposal 4: Study HARQ retransmission design for PS in 6GR. Observation 9: For geometric shaping, the bit labeling order in bit-to-symbol mapping needs to be carefully designed in order to work well with the 5G NR LDPC code and SBPM mapping. Observation 10: In geometric shaping, intersections in bit-level capacity arise due to joint optimization across the I/Q dimensions. Observation 11: The transition from modulation order m-1 to m occurs at lower SNR values for PS and GS than for uniform QAM. |
| Xiaomi | Observation 1: The net gain of 1D-NUC and 2D-NUC depends on many factors, including the modulation order, shaping dimensions, channel conditions, and the extent of PAPR loss. Observation 2: 1D-NUC has demodulation complexity comparable to that of NR uniform modulation, but theoretically it yields a smaller performance gain compared to 2D-NUC. Observation 3: 2D-NUC can theoretically achieve a higher BLER-SNR gain, but at the cost of much higher demodulation complexity. Proposal 7: For evaluation purpose, ATSC 3.0 scheme is used as the candidate GS scheme. Observation 4: There are many candidate distribution matcher algorithms available, and evaluation comparison can only be conducted until more details are disclosed from the proponents. |
| Hanbat | Proposal 2: RAN1 to study on the detailed assumptions and the proposed improved MCS table to be used as a common baseline for the subsequent system-level evaluation of constellation shaping for 6G NR. Proposal 3: RAN1 is encouraged to continue studying Geometric Constellation Shaping (GCS) and Probabilistic Constellation Shaping (PCS) as additional constellation techniques to enhance spectral efficiency for 6G New Radio (NR). Proposal 4: Postpone MU-MIMO shaping while continuing studies on shaping for 1024QAM and 4096QAM |
| MTK | Observation 3: ESS has smaller rate loss than CCDM which make it more suitable for small to medium block lengths. Observation 4: ESS is a good candidate for PAS due to its good performance and small rate loss for any shaper length. Observation 5: ESS storage can be further reduced, subject to acceptable performance loss. Proposal 2: RAN1 to support probabilistic constellation shaping for higher-order modulation. |
| Google | Observation 1: Depends on the code block size, PS-based modulation may introduce a higher computation complexity than 1D and 2D NUC (i.e., GS-based modulation). PS-based modulation provides a better performance than 1D and 2D NUC, in terms of spectral efficiency and BLER. GS-based modulation can reuse 5GNR BICM framework without much modification. Proposal 1: For simulation that considering realistic CSI report with improved MCS table, companies are encouraged to report the corresponding CQI tables. Observation 2: To compare BLER between shaping modulations, shaping rate of PS-based modulation should be included in calculating spectral efficiency. Proposal 2: For assessing performance of PS-based modulation with 5G MCS table and improved MCS table, shaping rate should be considered in determining spectral efficiency. |

On PAPR of PS/GS:

| Company | Proposals |
| --- | --- |
| Spreadtrum | Observation 1: PS and GS both degrade PAPR performance relative to uniform QAM, especially for the UL DFT-s-OFDM waveform. |
| Samsung | Observation 5: 1D-NUC does not bring significant PAPR increase compared to uniform QAM. Observation 10: Systematic PS exhibits greater robustness against PAPR degradation compared to fully-shaped PS. Observation 11: In DFT-s-OFDM, as the symbol probability distribution becomes more Gaussian-like, it induces more PAPR degradation. |
| IMU | Observation 3: DFT-s-OFDM already reduces PAPR versus CP-OFDM through single-carrier precoding. Constellation shaping (PCS/GCS) can further suppress peaks by reshaping symbol statistics (e.g., lowering the probability of high-amplitude points) while retaining the NR mapper and LDPC chain. The combination DFT-s-OFDM + shaping therefore targets additional PAPR reduction without introducing a new waveform, and can be evaluated alongside interpolated π/2-BPSK, OQPSK, rotated-QPSK. |
| Oppo | Observation 16: PS-based modulation exhibits ~1 dB PAPR increase with DFT-s-OFDM waveform compared with NR uniform QAM scheme. |
| Lenovo | Observation 4: In CP-OFDM systems, NUC and NP-NUC do not increase PAPR. In DFT-s-OFDM, NUC and NP-NUC increase PAPR. The PAPR increment caused by NP-NUC is much smaller than that caused by NUC. Observation 8: For link adaptation in PS systems, jointly optimizing modulation order, FEC code rate, and the shaping parameter to satisfy a target BLER for a given channel quality (e.g., SNR) is a nontrivial and computationally intensive task. Proposal 3: Study optimizing link adaptation for PS systems for maximizing the data rate subject to a BLER constraint at a given channel quality (e.g., SNR). Evaluate different candidate solutions to maximize the probabilistic shaping gain. |
| Ericsson | Observation 11 For CP-OFDM, similar range and statistic distribution are observed for PAPR for NUC and QAM with different modulation orders and code rates. |
| HW | Observation 16: For DFT-s-OFDM waveform, both geometric shaping and probabilistic shaping increases the PAPR, counteracting the goal of using DFT-s-OFDM. Such an increase is particularly pronounced in probabilistic shaping. Observation 17: The final gain of shaping schemes should consider both the shaping gains and also the PAPR loss or MPR considering realistic PA. Proposal 5: For shaping schemes, the factors including at least phase noise, EVM requirement, MPR or A-MPR increase under realistic PA model require RAN4 confirmation or inputs. |
| Qualcomm | Observation 24: Both PS and GS have similar PAPR as uniform QAM when CP-OFDM waveform is used. Observation 25: For DFT-S-OFDM waveform, PS with properly selected probability distributions (different from Maxwell-Boltzmann distribution) may provide similar PAPR to uniform QAM, while still providing meaningful shaping gain from both BICM capacity analysis as well as link level evaluations. Proposal 5: Study probabilistic shaping design for DFT-S-OFDM waveform to achieve a good trade-off between PAPR and link level performance gain. |
| Sony | Observation 4: For DFT-s-OFDM, the PAPR behaviour may be improved by constellation shaping Observation 5: For DFT-s-OFDM, the PAPR behaviour may be improved by coded modulation Proposal 4: RAN1 should study constellation shaping for PAPR reduction of DFT-s OFDM Proposal 5: RAN1 should study coded modulation schemes for PAPR reduction of DFT-s OFDM |

On spec impact of PS/GS:

| Company | Proposals |
| --- | --- |
| CATT | Observation 2: Probabilistic shaping has a significant impact on the coding chain, including the addition of a distribution matcher module, modifications of bit interleaving and bit scrambling, etc. |
| ZTE | Observation 1: Introducing NUC for 6GR does not affect the LDPC coding chain in 5G. The LDPC coding chain in 5G can be reused in 6GR. Observation 12: PAS modulation has impacts on coding chain in terms of:  Additional procedures before or after channel coding  Bit interleaving |
| Oppo | Observation 8: PS-based modulation could have significant impact on Tx/Rx chain design, including code rate design, interleaving, scrambling, and redundancy version (RV) design, etc. |
| Lenovo | Observation 7: The PS framework requires modifying the MCS tables to include the PMF of symbols or a corresponding shaping parameter for each MCS index. Additionally, the combinations of modulation order and LDPC coding rate in the uniform QAM MCS tables need to be updated to reflect the rate of the distribution matcher. |
| HW | Observation 3: Supporting probabilistic shaping requires significant changes for the legacy NR coding and modulation chain and introduces extra complexity, necessitating a substantial redesign of the NR coding and modulation chain. |

On complexity and storage requirements of PS/GS

| Company | Proposals |
| --- | --- |
| CATT | Observation 3: For probabilistic shaping implemented by enumerative sphere shaping, the number of operations required by the decoding progress of the distribution matcher is about 5% of that required by demapping. Observation 4: For probabilistic shaping implemented by enumerative sphere shaping, the distribution matcher requires storages to implement encoding and decoding. The storage sizes shown by Table 3 are up to 1.388 Mbits. |
| Tejas | Observation 1 :- The de-mapping complexity of 1D-NUC is comparable to that of a uniform constellation, as both require a similar number of operations for LLR computation. Observation 2 :- The de-mapping complexity of 2D-NUC is significantly higher than that of 1D-NUC due to the need for joint processing of in-phase and quadrature components. Proposal 1 :- RAN1 to Study the benefits and applicability of Non-Uniform Constellation (1D-NUC and 2D-NUC) for 6G Radio Air Interface. |
| Samsung | Observation 6: 1D-NUC and uniform-QAM exhibit similar complexity for the sphere decoding based ML receiver within the SNR range of interest, while 1D-NUC outperforms uniform-QAM consistently. |
| ZTE | Observation 2: NUC modulation/demodulation is a full parallel process, and it is easier to achieve ultra-high throughput for 6GR. Observation 9: Transmitter complexity for NUC modulation is mainly arises from storage of constellation points and the impact of transmitter complexity for NUC is negligible. Observation 10: For MCS level 18 in SISO scenarios, computational complexities of 2D-NUC, 1D-NUC and QAM are as below.  Computational complexity of 2D-NUC is ~2M operations  Computational complexity of 1D-NUC is ~831K operations  Computational complexity of uniform QAM is ~823K operations Observation 11: For MCS level 18 in MIMO (4T4R) scenarios, computational complexities of 2D-NUC, 1D-NUC and QAM are as below.  Computational complexity of 2D-NUC is ~11M operations  Computational complexity of 1D-NUC is ~4.76M operations  Computational complexity of uniform QAM is ~4.73M operations Observation 16: PAS schemes with (A)ESS/shell mapping have an ultra-high memory requirement for encoding/decoding. Observation 1: For MCS level 19 in SISO scenarios, computational complexities of CCDM-based PAS and QAM are as below  Computational complexity of PAS is ~891K operations  Computational complexity of uniform QAM is ~762K operations Observation 2: For MCS level 19 in MIMO (4T4R) scenarios, computational complexities of PAS and QAM for LMMSE receiver are as below.  Computational complexity of PAS is ~5M operations  Computational complexity of uniform QAM is ~4.5M operations |
| DCM | Observation 3: GCS optimized for various channel condition requires storage at both transmitter and receiver side. The performance gain of GCS over QAM increase with the modulation order, at the cost of increasing computation complexity at receiver side due to the complex-valued LLR computation. Observation 4: Probabilistic Constellation Shaping (PCS) will increase the implementation complexity over QAM and have large specification impacts. |
| Oppo | Observation 6: Uniform QAM and 1D-NUC require similar demapping complexity. Observation 7: Compared with 2D-NUC, lower demodulation complexity could be achieved with 1D-NUC with slight performance degradation. Observation 15: For PS-based modulation, the introduction of DM may lead to serial processing latency and/or storage overhead. |
| Ericsson | Observation 2 The computational (and storage, if applicable) complexity of PSCM depends on the number of shaped bits per amplitude for each modulation symbol. Observation 3 1D-NUC has a modest increase (~ ≤1.2x) in computational complexity compared to uniform QAM but the complexity increase in 2D-NUC compared to uniform QAM is significant (~15x-1000x). |
| HW | Observation 2: In geometric shaping, 2D-NUC may require a prohibitive demodulation complexity in high modulation order. • The demodulation complexity of 2D-NUC is approximately 14.4x for 1024QAM and 4.2x for 256QAM over the baseline LDPC decoding complexity. • Demodulation complexity of 1D-NUC is significantly lower than that of 2D-NUC. Observation 4: In probabilistic shaping, the DM complexity grows with the modulation order, and different DM schemes incur different complexities: • Complexity of CCDM is approximately 2x LDPC decoding for 1024QAM. • Complexity of ESS is roughly 1/6 of LDPC decoding for 1024QAM. However, the storage of ESS is very high. • Complexity of energy-based AC can reach 28.7x that of LDPC decoding for 1024QAM. Observation 5: The throughput of typical probabilistic shaping schemes is low: • CCDM incurs ~10x processing delay over LDPC decoding due to its inherent serial arithmetic nature, creating a throughput bottleneck. • Even when the shaping block length is reduced (e.g., to 128), the processing delay still remains significant, approximately equivalent to 5 iterations of LDPC decoding. Observation 7: In the worst-case scenario, the computational complexity of sphere decoding (SD) is prohibitive (e.g., 3×108x over LDPC decoding). Observation 9: With probabilistic shaping, the computational complexity of reduced ML detection (i.e., using QRM-MLD) is 1~10x of that for LDPC decoding. Observation 10: With 2D-NUC, the computational complexity of reduced ML detection (i.e., using QRM-MLD) is 10~100 times of that for LDPC decoding. |
| Qualcomm | Observation 5: CCDM comprises 1/6.6 of those of the LDPC decoder (with NR LDPC code at 10 iterations and offset min-sum decoding), and 1/30 of the area of LDPC decoder to reach the same data rate. Observation 6: The storage overhead of ESS is manageable in practical setups to achieve sufficient shaping gain. Observation 26: PS incurs limited complexity on the demodulation for both linear and non-linear demodulator. Observation 27: The demodulation complexity for (unstructured) GS can be prohibitive due to irregular constellation point locations, especially for large modulation order and high rank MIMO. |

Multiple companies provided evaluation results for constellation shaping as well:

| Company | Proposals |
| --- | --- |
| CATT | PS for AWGN channel fixed MCS: Observation 5: • For 16-QAM, the performance gain of probabilistic shaping compared to uniform BICM at 2.5 bits/2D symbol is about 0.4 dB; • For 64-QAM, the performance gain of probabilistic shaping compared to uniform BICM at 3.0~4.5 bits/2D symbol is about 0.8~1.0 dB; • For 256-QAM, the performance gain of probabilistic shaping compared to uniform BICM at 5.0~6.5 bits/2D symbol is about 0.8~1.4 dB; • For 1024-QAM, the performance gain of probabilistic shaping compared to uniform BICM at 7.0~8.5 bits/2D symbol is about 0.8~1.7 dB. GS for AWGN channel fixed MCS: Observation 6: • The performance gain of 16-NU-QAM compared to 16-QAM is very small. • The performance gain of 64-NU-QAM compared to 64-QAM is about 0.2 dB. • The performance gain of 256-NU-QAM compared to 256-QAM is 0.4~0.6 dB at 4.0~5.0 bits/2D symbol. For the spectral efficiency of 6.5 bits/2D, there is no gain. The performance gain of 1024-NU-QAM compared to 1024-QAM is 0.6~0.8 dB at 5.0~7.0 bits/2D symbol. For the spectral efficiency of 8.0 bits/2D symbol, there is no gain. PS/GS for SISO fading channel: Observation 7: • In the TDL-C 300 ns static UE SISO channel scenario, GS and PAS achieve better performance than uniform QAM at 1% BLER for 64QAM, 256QAM, and 1024QAM, and PAS shows performs better than GS; • In the TDL-C 300 ns static UE SISO channel scenario, the performance gain of PAS compared to uniform QAM at 1% BLER is about 0.18~0.52 dB; |
| Tejas | GS for AWGN channel, fixed MCS: Observation 3 :- Initial investigation using 5G-NR PDSCH BICM chain for a target BLER of 1% without retransmission using MCS table 2 under AWGN channel shows shaping gains of up to 0.7dB for 1D-NUC and up to 0.8dB for 2D-NUC. GS for fading channel, fixed MCS: Observation 4 :- Initial investigation using 5G-NR PUSCH chain for a target BLER of 1% using MCS table 2 under TDL-A channel without retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 2D-NUC. Observation 5 :- Initial investigation using 5G-NR PUSCH chain for a 70% throughput using MCS table 2 under TDL-A channel with retransmission shows shaping gains of up to 0.6dB for 1D-NUC and 0.7dB for 2D-NUC. Observation 6 :- Initial study using 5G-NR PUSCH chain using MCS table 2 under TDL-A channel with retransmission shows up to 10% throughput gain for 1D-NUC and 2D-NUC. Observation 7 :- NUC Performance evaluation indicates reduction in throughput (up to 2.39%) for MCS index corresponding to 16QAM compared to UC (under TDL-A channel using 5G-NR PUSCH chain using MCS table 2). Observation 8 :- NUC optimized for AWGN channel and a target SNR also provides throughput gain under fading channel (TDL-A). Observation 9 :- Higher-order modulation schemes achieve greater shaping gain under both AWGN and fading (TDL-A) channel. |
| Samsung | GS for AWGN and fading: Observation 3: 1-D NUC outperform uniform QAM over a wide range of operating Es/No points, channel model parameters, and antenna configurations. Observation 4: The performance gain of 1-D NUC over uniform QAM increases with the modulation order. PS for AWGN and fading Observation 9: For the AWGN channel, systematic PS achieves an SNR gain of 0.4 to 1.2 dB to achieve the same BLER, compared to uniform QAM. |
| ZTE | GS for fading channel: Observation 7: According to our simulation results, 2D-NUC can have following performance.  Shaping gain from 0.17dB to 0.85 dB where shaping gain of NUC increases as the modulation order increases  ~0.27dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.  ~0.5 dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel.  ~0.6 dB shaping gain for MCS level 23 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.  ~0.4 dB shaping gain for MCS level 23 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel. Observation 8: According to our simulation results, 1D-NUC can have the following performance.  ~0.18dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the AWGN channel.  ~0.35 dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel.  ~0.42dB shaping gain for MCS level 23 in NR 256QAM table at BLER = 0.1 in the AWGN channel.  ~0.42dB shaping gain for MCS level 23 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel. PS for AWGN and fading channels: Observation 15: According to our simulation results, PAS can have the following performance.  ~0.27dB shaping gain for MCS level 19 in NR 256QAM table at BLER = 0.1 in the AWGN channel.  ~0.38dB shaping gain for MCS level 19 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.  ~0.53dB shaping gain for MCS level 24 in NR 256QAM table at BLER = 0.1 in the AWGN channel.  ~0.71dB shaping gain for MCS level 24 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.  ~2.1 dB performance loss for MCS level 19 in NR 256QAM table at BLER = 0.1 in the TDL-A 4T4R channel under LMMSE receiver.  ~3.0 dB performance loss for MCS level 24 in NR 256QAM table at BLER = 0.1 in the TDL-A 4T4R channel under LMMSE receiver. |
| LGE | GS for AWGN and fading channel: Observation 1: NUC outperforms NR Uniform QAM in AWGN channel and multi-path channels with various delay spreads and UE speeds. |
| DCM | GS with fixed MCS Observation 2: Proposed 16QAM-CS could achieve lower PAPR and provide BLER and throughput performance gain compared with 16QAM under multiple DFT-s-OFDM-based waveforms. |
| Oppo | GS for AWGN and fading: Observation 3: In AWGN and Rayleigh fading channels, 2D-NUC could achieve 0.7 dB and 0.4 dB gain @10% BLER compared with uniform QAM, respectively. Observation 4: In AWGN and Rayleigh fading channels, 1D-NUC has less than 0.2 dB performance degradation compared with 2D-NUC. Observation 5: GS-based modulation could retain stable performance gain over different channel types with well-designed constellation patterns. PS for AWGN and fading: Observation 9: Ideal-DM-based PS could achieve about 1.2 dB performance gain compared with uniform QAM. But the rate loss issue of practical DMs may degrade the performance gain of PS. Observation 10: The performance gain of PS reduces as the code block length decreases. With very small code block length, the performance of CCDM-based PS can be similar to uniform QAM. Observation 11: In i.i.d. Rayleigh fading channel, the performance gain of PS significantly declines. Ideal-DM-based PS and CCDM-based PS can have 0.4 dB and 0.7 dB performance loss compared with uniform QAM @ 10% BLER. Observation 12: In i.i.d. Rayleigh fading channel, the performance gain of PS declines as the number of transmission resources reduces, and CCDM-based PS has 2.4 dB and 3.1 dB performance loss compared with uniform QAM with 100 REs and 50 REs, respectively. Observation 13: Compared with uniform QAM, PS could exhibit performance gain in 1-layer transmissions but exhibits performance loss for 2-layer and 4-layer transmissions for both ideal and realistic channel estimation methods. Observation 14: Compared with uniform QAM, PS is more sensitive to the code block size. The BLER performance shows significant variation between with and without CSI-RS overheads. |
| Lenovo | GS for AWGN and fading channels: Observation 3: The SNR gains of NUC range from 0.15 to 0.5 dB over AWGN channels, from 0.1 to 0.6 dB over TDL channels in SISO systems, and from 0.1 to 0.5 dB over CDL channels in MIMO systems. The SNR gains of NP-NUC range from 0.04 to 0.15 dB over AWGN channels, from 0 to 0.22 dB over TDL channels in SISO systems, and from 0 to 0.2 dB over CDL channels in MIMO systems. The gains are obtained through simulations for the MCS indices listed in the MCS table. PS for AWGN: Observation 6: The SNR gains of PS range from 0.15 to 1.15 dB in AWGN channels. The gains are obtained under the configurations of the MCS indices listed in the MCS table. |
| IDC | PS for AWGN: Observation 3: Initial simulation results for PCS show performance gains, e.g., 0.5-0.6 dB for 64 QAM, 0.5-0.8 dB for 256 QAM and 0.8 dB for 1024 QAM, over uniform QAM. Observation 4: Initial simulation results for PCS with log-MAP LLR show performance gains over ML around 0.7 ~ 1 dB for 256 QAM in AWGN. Observation 5: Direct method based distribution matchers like CCDM can also enable LLR with log-MAP at the Rx for further enhanced performance. Observation 6: Simulation results for PCS show potential 0.5% – 0.7% EVM requirements relaxation over uniform QAM. |
| Ericsson | Observation 4 For 1T1R and AWGN channel, PSCM has about 0.2 dB higher gain than ATSC 3.0 for our evaluated code rates and spectral efficiencies. Observation 1 In TDL-A fading channel, PSCM shows loss with 1T1R and regains its good performance with increased number of transceivers, with 1T1R the range of performance is from -2.5 dB loss to 0.1 dB gain, with 4T4R the range of gain is from 0.2 dB to 0.7dB. Observation 2 In TDL-A fading channel, ATSC 3.0 (2D-NUC with modulation order 8) performance is slightly impacted by fading channel, with 1T1R the shaping gain is from 0.2 dB to 0.9 dB, with 4T4R the shaping gain is from 0.5 dB to 0.9 dB. Observation 3 For 32T4R with rank=4 and MMSE receiver, PSCM shows performance loss ranging from -0.7 dB to -0.1 dB. Observation 4 For 32T4R with rank=4 and MMSE receiver, ATSC 3.0 (2D-NUC) with modulation order 8 shows performance gain between 0.4 dB to 0.7 dB, similar to the performance gain with rank =1. Observation 5 ATSC 3.0 (2D-NUC) with modulation order 8 shows a non-negative gain ranging from 0.2 dB to 0.9 dB across various scenarios in both AWGN and fading channels. Observation 6 PSCM shows gain in AWGN channel and loss in fading channel with a shaping factor optimized for AWGN channel. PSCM seems to be sensitive to fading channels, close loop MIMO implementation and the number of transceivers, and shows varied gain/loss ranging from -2.5 dB to 1 dB. Proposal 6 MIMO settings with close loop MIMO shall be calibrated for comparing the baseline performance of NR 256 QAM. Proposal 7 The PSCM shaping factor optimized for AWGN channel shall be calibrated across different companies with regard to a particular spectrum efficiency and RB allocations. Proposal 8 Performance of PSCM in AWGN channel for 1T1R, TDL-A n30 channel for 1T1R and 4T4R, and CDL-B channel for 32T4R, based on optimized shaping factor for AWGN channel shall be calibrated across different companies. Proposal 9 Results based on ATSC 3.0 with 2D-NUC and 256 QAM for SU-MIMO shall be calibrated across different companies. |
| Vivo | Observation 4: Compared to legacy 256QAM assuming existing MCS table, geometric shaping has  0.34~0.96 dB BLER performance gain in AWGN channel for MCS 20~MCS27  0.5~0.76 dB BLER performance gain in TDL-A channel for MCS 20~MCS27 Observation 5: Compared to legacy 256QAM assuming existing MCS table, probabilistic shaping has  0.65~1.35 dB BLER performance gain in AWGN channel for MCS 20~MCS24  0.56~1.05 dB BLER performance gain in TDL-A channel for MCS 20~MCS24 with 6RB  0.32~0.6 dB BLER performance loss in TDL-A channel for MCS 20~MCS24 with 24RB  Note: PS is not applicable to MCS 25~27 of the existing NR MCS table Observation 6: Compared to legacy QAM assuming existing MCS table, GS can provide up to 5% and 3% throughput gain with 6 RBs and 24 RBs respectively. Observation 7: Compared to legacy QAM assuming existing MCS table, PS can provide 1% throughput gain with 6 RBs and no throughput gain with 24 RBs. |
| HW | Observation 8: In precoded MIMO system equipped with sphere decoding, probabilistic shaping results in significant performance loss (~3dB) over uniform QAM. Observation 12: We have the following observations regarding the BLER performance comparison between CCDM, 1D-NUC, 2D-NUC and uniform QAM (with and without AMC): • In AWGN channels, CCDM, 1D-NUC and 2D-NUC achieve shaping gains of 1.0 dB, 0.6 dB and 0.8dB over uniform QAM, respectively. • Conversely, in fading channels when using the common configurations as AWGN (i.e., the same constellations or probabilistic distributions), such shaping gains experience a substantial reduction, and shaping losses are observed. Observation 13: Although the loss of shaping gain in fading channel could be partially restored when HARQ retransmission is combined with initial transmission, this limited shaping gain increment may not contribute much on the realistic throughput gains considering probability of HARQ retransmission is usually controlled to be a lower value, e.g., 10% iBLER. Observation 14: Under link adaptation and HARQ retransmission, CCDM shows similar or throughput performance loss over both uniform QAM, 1D-NUC and 2D-NUC (based on ATSC). Meanwhile, the throughputs provided by uniform QAM, 1D-NUC and 2D-NUC are comparable with only marginal throughput gain of 1D-NUC and 2D-NUC over uniform QAM in high SNRs. Observation 15: System-level simulations (SLS) using an AWGN-based physical layer abstraction model show that CCDM/1D-NUC/2D-NUC provides 0.23%/-0.02%/4.77% gain of cell-average SE for Dense Urban scenario with carrier frequency 700MHz, and 1.61%/1.28%/3.66% gain of cell-average SE for Dense Urban scenario with carrier frequency 7GHz, under the full-buffer traffic model. |
| Qualcomm | Observation 12: PS achieves significant gain over uniform QAM baseline in AWGN channel. Observation 13: PS achieves 04~0.7 dB gain over GS in AWGN channel. Observation 14: With maximum 512 block length for PS with CCDM, the performance loss is less than 0.1dB. Observation 15: Both PS and GS have robust HARQ performance on AWGN channel. Figure 17 Performance comparison between PS and GS and uniform QAM, 1Tx32Rx SIMO with realistic channel and noise/interference estimation, TDL-A 30ns, 22Hz doppler Observation 16: For SIMO scenarios with 8 and 32 Rx antennas, both PS and GS provide substantial gains over uniform QAM, where PS maintaining a stable gain over GS, similar to the AWGN channel. Observation 17: With accurate beamforming at the transmitter, similar shaping gains as in AWGN channel are observed over MIMO fading channels, with LMMSE receiver. Observation 18: PS may provide more than 1.53 dB performance gain in MIMO scenarios due to interference shaping, for both open-loop precoding, as well as closed-loop precoding with realistic SRS sounding periodicity. Observation 19: With ideal SRS chest, very short SRS periodicity, per-tone precoding, zero SRS processing delay, and number of Tx antennas equal number of Rx antennas and full rank transmission, SVD precoding may lead to severe layer imbalance, which yield performance degradation for both uniform QAM and PS, especially when the coding rate is high. Observation 20: With layer-balancing precoding, PS shows 1~1.5 dB performance gain over uniform QAM at the examined MCS values. Observation 21: With realistic SRS chest, moderate SRS periodicity, and per-PRG precoding, PS still achieves good performance gain for high MCS values with SVD precoding. Observation 22: With closed-loop precoding and outer-loop link adaptation, PS may provide 5~16% throughput gains across wide range of SNR values and mobility conditions; realistic DMRS channel and noise/interference estimation Observation 23: With MU-MIMO, the interference shaping gain from probabilistic shaping can be observed even with rank 2 due to the presence of rank-2 inter-user interference. |
| Xiaomi | Observation 5: For 256QAM in AWGN channel, 2D-NUC scheme in ATSC 3.0 can provide about 0.42~0.77 dB gain in dB with respect to NR baseline at BLER 10%. Observation 6: For 256QAM in TDL-A channel, 2D-NUC scheme in ATSC 3.0 can provide about 0.49~0.8 dB gain in dB with respect to NR baseline at BLER 10%, when the number of PRBs is 6. Observation 7: For 256QAM in TDL-A channel, 2D-NUC scheme in ATSC 3.0 can provide very marginal gain (almost no gain) with respect to NR baseline at BLER 10%, when the number of PRBs is 48. Proposal 8: Before collecting simulation results and providing corresponding observations, RAN1 should first establish a clearer and aligned understanding of the proposed constellation shaping schemes including their design principles, advantages, limitations, and potential specification impacts. |
| Apple | Observation 2: For one-layer 1x2 SIMO transmission over TDL-A channel, -0.7 to 0.4dB loss/gain is observed for different MCSs. Observation 3: For two-layer MIMO transmission over TDL-A channel, -1.4dB to 0.5dB loss/gain is observed for different MCSs for 2x2 MIMO, while -1dB to 0.7dB gain was observed for 2x4 MIMO. Observation 4: For four-layer MIMO transmission over TDL-A channel, -0.9dB to 2.2dB loss/gain is observed for different MCSs for 4x4 MIMO, while -1dB to 1.4dB gain was observed for 4x6 MIMO. |
| MTK | Observation 6: 1–2 dB gain can be observed in T-put performance with dynamic rank and MCS adaptation. |

### First round discussion

Discussion 2.3-1 (closed)

For PS/GS AWGN calibration, a spreadsheet is provided in the appendix (draft folder). The FL attempts to capture all the SNR gains for different NR reference SE points if numbers are provided in the contributions. Please check for accuracy. If only BLER curves are provided in the contribution, please provide SNR gain/loss numbers.

| Company | Comments if any |
| --- | --- |
| OPPO | We support to calibrate the simulation results from different resources, but the spreadsheet format should be discussed first for the ease of future results collection. In addition to BLER performance, we think the required computational complexity and modified Tx/Rx chain modules should also be added in the spreadsheet to draw a complete picture of different methods. |
| Spreadtrum | Support |
| Lenovo | Fine. |
| CATT | Support |
| Tejas | we share similar view as OPPO. |
| ZTE, Sanechips | In our contribution, we provide some evaluation results without figure. We summarize assumptions and the evaluation results, which will be updated in the spreadsheet later. NR MCS reference (Mod order, coding rate, SE); CCDM-PAS. Parameters; Gain in dB. (8, 682.5, 5.3320); CCDM with MB parameter v=0.0192; -3.5 dB for TBS of 24, -2.2 dB for TBS of 48, -1.6 dB for TBS of 104, -0.25 dB for TBS of 240.. Assumptions; TBS=24, 48, 104, 240, Layered BP decoding with 25 iterations, MAP demodulation, AWGN channels, target BLER=0.01. |
| Samsung | We appreciate the FL’s effort in compiling the spreadsheet that aggregates SNR gain results from each contribution. We have updated the spreadsheet to include data extracted from Samsung’s contributions that only provided graphical results. However, since the submission formats vary across companies, we believe it is important to discuss and align on a common reporting format to ensure consistent interpretation and fair comparison of the results. |
| Ericsson | In the spread sheet we shall include also the evaluation results of the baseline in addition to the gain and loss. For example, the SNR for reaching 10% BLER based on NR MCS/SE need to be provided to see the differences on baseline among different companies. This is a general comment from us on all the evaluations, only capturing a gain/loss without its reference value may not give a comprehensive understanding of the evaluations. |
| Huawei, HiSilicon | - It has already been agreed in RAN1#122bis, the evaluation and comparison of constellation shaping and improved MCS table should consider at least BLER performance, throughput performance, transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements etc. Therefore, feature lead should not only collect the BLER performance, the corresponding complexity, storage overhead, latency impact/parallelism implementation that we have agreed should be also collected. - Actually, the rate loss impacting the BLER performance highly depends on the length of the DM of shaping solutions with different complexity and storage requirement. The through-put of shaping solutions shall be also highly impacted by the parallelism of DM schemes. It doesn’t make sense to collect BLER/throughput performance in isolation with the corresponding complexity/storage/parallelism. Therefore, before we jump to any observations, we should discuss how to collect the results and complexity/overhead analysis for the candidate solutions in a reasonable way. A template for collecting results should be firstly discussed to include at least includes columns of BLER performance, throughput impact, the corresponding transmitter complexity, corresponding receiver compleixy, storage impact. - It is useful to do some calibration among companies for the following two purposes: Calibrate the performance under AWGN of the evaluated solutions to make sure companies correctly implement the same solution proposed by proponents. Before this calibration, it is required that the proponents should report clearly the details including the detailed constellations for GS, the detailed distributions, algorithm, DM length of PS. Without such details, it is difficult to reproduce the results of other proponents and difficult to do the calibration. Furthermore, if a company considers different Modulation-Related Parameters, e.g., constellations or distributions, in fading channels from that of AWGN, it should be reported when they provide the results; Calibrate the baseline. It is also good among companies to calibrate the performance of baseline including the AWGN and fading channels. This is essential for comparison and analyse the gains in case there are divergent gains are reported. E.g., some results gains may correspond to the mixed gain from shaping and enhanced AMC and some other companies may report the gain only considering shaping. The baseline performance should be also reported and calibrated, rather than only a relative gain are reported. |

Discussion 2.3-2 (closed and continue discussion in the next meeting)

For AWGN channel fixed MCS simulation, a few tentative observations as follows

- PS/GS has SNR gain over a wide range of MCS/SE points, subject to proper parameter choices

- SNR gain wise, PS>2D-NUC>1D-NUC

FL notes: It might be too early to form official observations, if more companies plan to participate in the AWGN fixed MCS simulation campaign. Please provide your comments below on the tentative observations, or you would like to delay the observations pending more simulation results in the future meetings. Please also mention if you plan to bring in AWGN simulation results in the next meeting, so we know what we are waiting for.

| Company | Comments if any |
| --- | --- |
| Xiaomi | Our understanding is AWGN is used for performance calibration as agreed in RAN1#122bis, not for drawing any meaningful observations since AWGN is far from realistic channels. |
| OPPO | We agree with FL that the observations are a little bit early. AWGN channel is a good choice for data calibration but both AWGN channel and fading channel are important for evaluating shaping methods. Therefore, the observations over AWGN and fading channel should be provided together to reflect the performance comprehensively. |
| vivo | We suggest to collect companies’ results at least for one extra meeting cycle first, and then to drive observations, so that companies can have more time to cross check, and the observations can provide more reliable information for future work. For the details, it is important to mention what the detailed range of MCS that PS or GS can provide benefit is. For example, PS is not applicable for higher code rates, as it may exceeds the maximum supported LDPC code rate 0.925. |
| Spreadtrum | We agree with FL that the observations are too early. We should encourage more companies to provide sufficient simulation results before giving observations. |
| Lenovo | Suggest to evaluate various candidate solutions (for GS, modulation order and coding rate, and for PS modulation order, coding rate, and shaping parameter) corresponding to a specific SE and find the best solutions to maximize the gain of GS and PS over different block lengths. |
| ZTE, Sanechips | Thanks to FL for the very comprehensive collection of all companies’ results on performance. We have several suggestions on the tentative observations: - For the first bullet - Suggestion 1-1: For the MCS/SE points, we think it should be further clarified. We made an agreement in last meeting that it should use the list of spectrum efficiencies in NR MCS table as starting point, but different companies may have different combinations of modulation order and SE. To this end, when saying PS/GS has SNR gain over a wide range of MCS/SE points, it would better to clarify whether modulation order is same as 5G NR or not. - Suggestion 1-2: we can take some examples to better understand what the proper parameter choices means, such as block length for PS, parameter for target distribution for PS, MCS level, etc. - For the second bullet - Suggestion 2-1: For the second bullet, we think it should be the sub-bullet of the first bullet. The reason is that for SNR gain comparison, we observe that the performance relationship among different shaping schemes is also subject to parameter choices. - Suggestion 2-2: The comparison among different shaping modulation schemes should be at the same MCS/SE points. - Suggestion 2-3: Add other observations on PAS when parameter choices are changed. In order to have a comprehensive analysis, we think some scenarios which reveals other performance relationships should be captured in observation. In summary, we suggest the template of the tentative observations should be: For AWGN channel fixed MCS simulation, observations are as follows - PS/GS has SNR gain over uniform QAM in a wide range of MCS/SE points with modulation order as 5G NR, subject to proper parameter choices (e.g., block length for PS, parameter for target distribution for PS, and MCS level) - SNR gain wise, PS > 2D-NUC > 1D-NUC at same MCS/SE points, subject to parameter choices - SNR gain wise, 2D-NUC>1D-NUC>PS at same MCS/SE points, subject to parameter choices - PS/GS has SNR gain over uniform QAM in a wide range of MCS/SE points with different modulation order from 5G NR, subject to proper parameter choices (e.g., block length for PS, parameter for target distribution for PS, and MCS level) - SNR gain wise, PS > 2D-NUC > 1D-NUC at same MCS/SE points, subject to parameter choices - SNR gain wise, 2D-NUC>1D-NUC>PS at same MCS/SE points, subject to parameter choices |
| DOCOMO | It is too early to make observations, especially the observation with unclear conditions. |
| Samsung | Based on the currently available results, the tentative observations appear reasonable. However, since many companies have not yet fully disclosed their shaping methods and simulation assumptions, we believe it is too early to adopt these observations as official conclusions. In our own results, we observed that when the MCS table is redesigned, 1D-NUC occasionally outperforms PS at higher spectral efficiencies. This suggests that the relative performance between PS and NUC can depend on MCS design and optimization details. Therefore, we agree with the FL’s note that these observations should be treated as preliminary and that further validation with more simulation results from other companies is needed before forming any official conclusion. |
| Ericsson | It is still premature to draw such conclusion. Our initial simulation on AWGN is mainly for calibrating results among companies. We will bring in more results on next meeting including the throughput results. |
| Huawei, HiSilicon | See our comments for Discussion 2.3-1. Before the calibration of baseline and solutions, and without the collection of corresponding complexity/storage/parallelism, it is difficult to draw observations. Companies even do not know the details of the proposals from proponents, and the results have not been reproduced and collected properly. It is not a time point for observations. We can start from the template used for results collection and calibration first in this meeting. |
| ETRI | We prefer to keep the observation pending until the next meetings. The initial observation should equally consider both AWGN and fading channels. For this meeting (#123), the group can discuss details of fading channel configurations to be simulated. Also, the group needs to discuss which code lengths (including short/long length) to be simulated. |

Discussion 2.3-3 (closed)

For PS/GS fading channel calibration, a spreadsheet is provided in the appendix (draft folder). Since the numbers are all over the place in this round of contributions, the FL does not believe there is any chance of drawing any conclusions. However, it is still good to collect the parameters and assumptions, so more accurate cross check is possible. Please check for accuracy. If only BLER curves are provided in the contribution, please provide SNR gain/loss numbers.

| Company | Comments if any |
| --- | --- |
| OPPO | Similar comments as in Discussion 2.3-1 and 2.3-2. We support to calibrate the simulation results from different resources, but the spreadsheet format should be discussed first for the ease of future results collection and the complexity and modified modules should be captured. As both AWGN channel and fading channel are important for evaluating shaping methods, the observations over AWGN and fading channels should be provided together to reflect the performance comprehensively. |
| Spreadtrum | Support |
| CATT | OK |
| ZTE, Sanechips | Suggest the similar observation template as we provided in the comment of Discussion 2.3-2 for fading channel. |
| DOCOMO | Agree that observations under fading channel could be drew later. After reading companies’ results, we find that there are many factors affecting the performance of PS/GS (especially PS), e.g., MIMO configuration, rank or layer number, receiver type, etc. Therefore, it is recommended to firstly list all the candidate affecting factors as well as their impacts on performance gain for PS and GS separately, before making any observations. |
| Samsung | We appreciate the FL’s continued effort in compiling and organizing the fading-channel calibration spreadsheet. We also observed that results under fading channels differ significantly from those under the AWGN channel, with noticeable variations across companies. Given these inconsistencies, we believe the fading-channel results should be treated as reference only, mainly to help cross-check assumptions and parameters, rather than to draw any quantitative conclusions at this stage. |
| Ericsson | In the spread sheet we shall include also the evaluation results of the baseline in addition to the gain and loss. For example, the SNR for reaching 10% BLER based on NR MCS/SE need to be provided to see the differences on baseline among different companies. This is a general comment from us on all the evaluations, only capturing a gain/loss without its reference value may not give a comprehensive understanding of the evaluations. |
| Huawei, HiSilion | See our comments for Discussion 2.3-1 and Discussion 2.3-2. The BLER performance and throughput impact are highly related with the transmitter/receiver complexity and parameters related with parallelism. It is more reasonable to discuss the template for results collection first. |
| ETRI | During this meeting, the group can discuss further details of fading channel configurations to be simulated, so for the next meeting, we can make better conclusion. |

Discussion 2.3-4 (closed and replaced by 2.3-4A)

- For a given scenario (channel type, receive assumption, etc), the performance of PS/GS may depend on variety of factors including:

- For a given spectrum efficiency (from NR MCS table), proper choice of constellation size, coding rate, and constellation shape for GS and target distribution for PS

- Block length and algorithm of DM for PS (especially when block length is small)

- FL notes: For fading channel fixed MCS simulations, there are less consistency across different company results. This discussion tries to come up with a list on potential areas for optimizations that may affect performance of shaping schemes

| Company | Comments if any |
| --- | --- |
| Xiaomi | Ok with the intention. Prefer to formulate in the following way: - For a given scenario (channel type, receive assumption, etc), tThe performance gain/loss of PS/GS may depend on variety of factors including at least: - Channel type, e.g., TDL or CDL - System setting, e.g., number of allocated PRBs, number of re-transmissions - Receive assumption, e.g., channel estimation assumption - For a given spectrum efficiency (from NR MCS table), proper choice of constellation size, coding rate, and constellation shape for GS and target distribution for PS - Block length and algorithm of DM for PS (especially when block length is small) |
| OPPO | Support |
| Lenovo | The gains of GS and PS also depend on other simulation setup, such as the choice of the number of RBs and the number of MIMO layers. The simulation parameters should be calibrated across the companies to align with the results by different companies. |
| ZTE, Sanechips | Fine with FL’s proposal. Besides, we think performance of PS/GS may depend on VRB type (e.g., DVRB or LVRB) and CSI feedback. |
| Samsung | We agree that the performance of PS/GS can vary significantly across scenarios, depending on factors such as constellation size, coding rate, shaping parameters, DM block length, and algorithmic details. What we need to verify is whether PS/GS can consistently provide gain over uniform QAM under each defined scenario, rather than focusing on absolute performance numbers. Therefore, as the FL note mentions, it is crucial to establish common assumptions and a unified simulation environment across companies to ensure fair and comparable evaluation results. |
| Ericsson | RAN1 need to select a proper baseline for the comparison including also the uniform QAM. |
| Huawei, HiSilicon | This verifies the necessity that the impacting factors need to be reported with performance rather than isolate them. |
| ETRI | OK in general. For block lengths, the group can discuss which block lengths (including short and long lengths) will be simulated for PS/GS comparisons. |

Discussion 2.3-5 (closed and replaced by 2.3-5A)

Multiple companies provided PAPR analysis for PS and GS compared with uniform QAM, especially for DFT-s-OFDM waveform. A few observations below

For CP-OFDM, PS/GS does not further degrade PAPR, and does not improve PAPR either

For DFT-s-OFDM,

  - PS/GS further degrades PAPR if PAPR is not also considered in PS/GS design

    - [Spreadtrum, Samsung, Oppo, Lenovo, HW]

  - Potentially can improve (?), maintain, or limit the increase of PAPR if PS/GS is carefully designed with PAPR reduction as a design goal

    - [IMU, Qualcomm (uses distribution different from MB), Sony, DCM]

| Company | Comments if any |
| --- | --- |
| Xiaomi | For the proposed design to maintain, or limit the increase of PAPR for DFT-s-OFDM, we believe it is at the cost of reducing the shaping gain, which should be reflected in the proposal. In addition, for GS used for improving PAPR for DFT-s-OFDM, our understanding is only for low SE. It is different with other GS schemes and should be clarified in the proposal. |
| OPPO | Due to the different design methods, the performance of PAPR can be captured in observations with the detailed shaping methods. For instance, with fully-shaped/partial-shaped bits, the PAPR performance is… Also, the observations on PAPR can also be provided later until more results are provided. |
| Spreadtrum | Based on current observations, we think the PAPR performance degradation of DFT-s-OFDM cannot be ignored and must be designed with the goal of not increasing PAPR |
| Lenovo | Along with PAPR, we should also be studying cubic metric (CM) to consider realistic constraints. |
| DOCOMO | Constellation shaping can improve PAPR as stated in our contribution. For the system design, net gain taking into account both SNR gain and PAPR gain are important. For higher modulation order, e.g., higher than 64QAM, it is possible that improving SNR gain is more important than improving PAPR gain, due to the considerable shaping gain. However, for lower modulation order, e.g., 16QAM, the shaping gain is small. In this case, improving PAPR gain is more beneficial for obtaining the net gain. |
| Samsung | Across all company observations, it was consistently confirmed that PS/GS has no meaningful impact on PAPR in CP-OFDM, which is expected because CP-OFDM inherently exhibits high PAPR due to its multi-carrier nature and the coherent addition of many subcarriers. As a result, constellation shaping neither improves nor further degrades PAPR for CP-OFDM. For UL DFT-s-OFDM, however, PAPR remains an important issue even with high-order modulation, since DFT-s-OFDM is used specifically to achieve lower PAPR for uplink transmissions. In this context, shaping can either increase or, if carefully designed, help limit PAPR depending on the chosen PS/GS method. Therefore, we believe PAPR studies should primarily focus on UL DFT-s-OFDM, where PAPR effects are most relevant to practical system operation. |
| Ericsson | We provided our initial results on PAPR without considering RF impairment, more results including the impact of RF impairment are required to draw any conclusion. |
| Huawei, HiSilicon | For the DFT-s-OFDM, we can agree that the Net Gain [dB] = SNR degradation/gain relative to the reference – MPR/PAPR degradation should be used for evaluation. The definition and methodology used for evaluating net gain can reuse the discussions waveform AI. |

Discussion 2.3-6 (closed and replaced by 2.3-6A)

Multiple companies provided complexity and storage analysis for PS and GS compared with uniform QAM. It might be too early to conclude anything, but a few observations are noted below

For PS

  - The demapper complexity is similar to corresponding uniform QAM demapper complexity

  - The DM complexity and/or storage requirement depends on DM algorithm used (ESS, CCDM, etc), block length, and the number of bit levels shaped per symbol

  - The DM processing delay depends on block length

  - Parallelism inversely proportional to block length

For GS,

  - The demapper complexity should be compared with uniform QAM demapper complexity

    - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity

      - As a function of 1D-NUC or 2D-NUC

      - As a function of # of spatial layers

      - Depends on LMMSE receiver or rML receiver

    - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed

| Company | Comments if any |
| --- | --- |
| Xiaomi | Ok with the intention. For PS, companies should also report the overall computational complexity not only including DM but also other impacted aspects. |
| OPPO | We agree with FL that it might be too early for conclusions and more quantitative analysis is needed. For both PS and GS, a unified baseline is needed for comparing complexity and storage. For instance, the complexity/storage requirements with legacy QAM demapper (either max-log or log-MAP) can serve as a baseline. The additional complexity/storage for PS and GS should be compared with the unified baseline, which may help to quantify the related metrics. For GS, it is also should be noted that no additional complexity/storage is introduced except in the demapper module, so that we can further focus on the evaluations on this module. |
| Spreadtrum | Similar comment as in discussion 2.3-2 |
| Lenovo | We suggest adding a sub-bullet for GS on storage requirements. Further, we note that NP-NUC (R1-2508623) works with legacy demapper being used for uniform QAM and does not incur any additional complexity or storage at the receiver. |
| ZTE, Sanechips | Thanks to FL for the very comprehensive collection of all companies’ results on complexity. We have several suggestions on the tentative observations: - Transmitter complexity and receiver complexity can be separately listed in order for clear comparison. - For transmitter complexity - For storage requirement for PS, we think the computation precision in bits for encoding/decoding should also be one of factors. - For transmitter complexity for PS, the throughput impact should be also considered. - For storage requirement for GS, it is depends on the assumption on complexity counting. - For receiver complexity - We suggest to use the same version as GS for complexity observation for PS, since we think it is too early to conclude that the PS demapper complexity is similar to corresponding uniform QAM demapper complexity. For PS demapper complexity, we think there are also some factors would affect the complexity of PS demapper. (e.g., the number of treated nodes, the number of spatial layers, probability distribution, and DM algorithm) - We think the number of operations is one of the angle to have complexity comparison between shaping modulation and uniform QAM. - We need to consider LMMSE receiver or rML receiver in receiver complexity for PS, since we agreed to have two options for downlink MIMO receiver. In summary, we suggest the template of the tentative observations should be: For PS - Transmitter complexity - DM complexity and/or storage requirement depends on DM algorithm used (ESS, CCDM, etc), block length, computation precision in bits for encoding/decoding, and the number of bit levels shaped per symbol - The DM processing delay/throughput depends on block length - Parallelism inversely proportional to block length - Receiver complexity - The demapper complexity is similar to corresponding should be compared with uniform QAM demapper complexity - Can report the ratio of PS demapper complexity over uniform QAM demapper complexity - As a function of treated nodes - As a function of # of spatial layers - As a function of # of operations - Depends on the probability distribution used (MB distribution, etc) - Depends on the DM algorithm used (ESS, CCDM, etc). - Depends on LMMSE receiver or rML receiver - The DM complexity and/or storage requirement depends on DM algorithm used (ESS, CCDM, etc), block length, computation precision in bits for encoding/decoding, and the number of bit levels shaped per symbol - The DM processing delay/throughput depends on block length - Parallelism inversely proportional to block length For GS, - Transmitter complexity - Storage requirement depends on the assumption on complexity counting, e.g, fixed point assumed or floating point assumed. - Receiver complexity - The demapper complexity should be compared with uniform QAM demapper complexity - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity - As a function of 1D-NUC or 2D-NUC - As a function of # of spatial layers - As a function of # of operations - Depends on LMMSE receiver or rML receiver |
| DOCOMO | For GS demapper complexity, we agree that uniform QAM should be considered as the baseline of demapper complexity comparison. However, the modulation order of baseline uniform QAM should be further discussed. There are two possible baseline definition. The first is to define the uniform QAM with same modulation order with GS as the baseline. For example, the demapper complexity of 256-point GS should be compared with the demapper complexity of 256QAM. The second way is to define the uniform QAM with maximum supported modulation order. For example, if maximum 1024QAM is supported in DL, it should be used as the baseline for GS with any modulation orders, e.g., 64-point or 256-point GS. From our perspective, we think the second definition is reasonable because the uniform QAM with maximum supported modulation order represents upper bound of UE capability. As long as the demapper complexity of GS is comparable with the UE capability, it is acceptable. |
| IDC | We support collecting quantitative data before conclusions. Transmitter and receiver complexity should be listed separately, referencing NR uniform QAM as a common baseline. Factors such as DM algorithm, block length, and receiver type (LMMSE or r-ML) should be clearly stated. |
| Samsung | We agree with the observations regarding the complexity and storage implications of PS and GS. It is important to account for the additional transmitter and receiver processing introduced by shaping, including DM-related complexity, latency, delay, and short-block rate loss for PS and demapper complexity for GS. Furthermore, whether a shaping solution remains compatible with the 5G BICM chain should be treated as a key decision criterion, as incompatibilities may lead to significant implementation overhead or architectural changes. |
| Huawei, HiSilicon | These very high-level observations cannot help us to make conclusions, especially without the connection with the performance benefit (if in a separate observation). For example, block length shall impact the rate loss, i.e., performance of PS, and also the complexity and throughput. Therefore, it is better to discuss first how to collect all the aspects we have agreed in RAN1#122bis and the results. |

### Second round discussion

Follow the discussion 2.3-1, we may need to first have a common understanding to the format of the simulation reporting.

Discussion 2.3-7 (agreed with modifications and closed)

For PS/GS fixed MCS performance reporting for 10% BLER (other target x% BLER can also be reported), adopt the following format for simulation result reporting.

| NR reference |  | Scheme A (e.g, PS, 1D-NUC, 2D-NUC etc) |  |  |
| --- | --- | --- | --- | --- |
| SE | (Mod order, coding rate) | SE point specific parameters | Baseline (uniform QAM) SNR to reach target BLER | Gain/loss in dB wrt NR baseline at target BLER x% |
| SE x | (modOrder, coding rate) | … |  |  |
| SE y | (modOrder, coding rate) | … |  |  |
| SE z | (modOrder, coding rate) | … |  |  |
| SE point independent assumptions |  | Common assumptions for the scheme simulated, including channel type (AWGN, SISO, SIMO, MIMO) and antenna configuration, number of spatial layers, number of RB allocated, TB size, shaping algorithm used (including block length), freq domain interleaver applied or not, receiver assumption, precoding assumption, realistic channel estimation, etc |  |  |

Note: For NR MCS reference, since NR has multiple MCS tables, it is not enough to provide the MCS index. Instead, need to provide the (modulation order, coding rate) pair for the simulated SE

Note: For SE point specific parameters:

For GS, this can be a pointer to the constellation used for this SE point

For PS, this can be a constellation size, coding rate and shaping parameter used for this SE point

- Note: Other metrics (at least complexity) comparison will be separately discussed, and if possible, we can merge the result in the same table

- Note: For AMC study, if possible, we can use the same table format

- FL notes: There are proposals to include complexity and other issues we agreed to consider in the same table. However, this discussion is about performance. We do have a separate discussion for complexity. The FL believes that it will be cleaner to separately discuss, and this performance discussion will not exclude other factors to be considered in the final decision.

Discussion 2.3-4A

- For fixed MCS performance study, for a given scenario (channel type, receiver assumption, number of RBs allocated, number of spatial layers, etc), the performance of PS/GS may depend on variety of factors including:

- For a given spectrum efficiency (from NR MCS table), proper choice of constellation size, coding rate, and constellation shape for GS and target distribution for PS

- Block length and algorithm of DM for PS (especially when block length is small)

- Frequency domain interleaver usage

- FL notes: For this discussion, the goal is to find out, for a given scenario where loss is observed for a given scheme, what are the ways to change the design to improve the performance. Eventually this can become the list of areas that we may need to optimize for. The FL left the link adaptation and HARQ out of this scope of this discussion, pending more simulation with link adaptation to identify issues

Please provide your view below:

| Company | Comments |
| --- | --- |
| Vivo | In our understanding, we should also look at the performance with link adaptation and/or HARQ turned on. For link adaptation, one important aspect which impacts the performance of PS/GS is the applicable code rate range of PS. Because PS cannot be used for high code rates because the final code rate may exceed the maximum code rate 0.925 for LDPC. Hence shaping gain may not be harvested on these high code rates. For HARQ, one important aspect is the IR HARQ gain which can be achieved by GS or PS. In general, IR gain compared with CC is higher than shaping gain in our observation. Hence whether IR gain can be fully achieved has large impact on the final performance of GS and PS. These aspects need to be taken into account when we draw observations or perform further study. Therefore, we suggest to add the following two bullets - For a given scenario (channel type, receiver assumption, number of RBs allocated, number of spatial layers, etc), the performance of PS/GS may depend on variety of factors including: - For a given spectrum efficiency (from NR MCS table), proper choice of constellation size, coding rate, and constellation shape for GS and target distribution for PS - Block length and algorithm of DM for PS (especially when block length is small) - Frequency domain interleaver usage - Applicable code rate range of PS/GS - Applicable HARQ scheme of PS/GS |
| Tejas | In our view, aspects of link adaptation and HARQ needs to be considered while optimizing the performance of GS/PS |
| OPPO | Support in principle. But the intension for this proposal should be first clarified. The factors such as block length, frequency domain interleaving, HARQ listed above have been captured in previous agreements on simulation assumptions. An extensional observation on the influence factors of performance may be too early before collecting enough performance results. |

Discussion 2.3-5A

For PAPR of PS and GS compared with uniform QAM,

For CP-OFDM, PS/GS does not further degrade PAPR, and does not improve PAPR either

For DFT-s-OFDM,

  - PS/GS further degrades PAPR if PS/GS design is only optimized for link performance gain

  - The PAPR degradation can be reduced if PS/GS is carefully designed with PAPR also as an optimization criterion. In this case, the Net Gain as agreed in waveform agenda item can be used as metric for the study

Please provide your view below:

| Company | Comments |
| --- | --- |
| OPPO | Same comments as in Discussion 2.3-5. The observations on PAPR should be provided later until more aligned results are provided. |
| IMU | We have same views as DOCOMO’s in Discussion 2.3-5 general. The trade-off between PAPR and SNR gain of shaped constellations should be carefully studied. |

Discussion 2.3-6A (replaced by 2.3-6B)

On how to evaluate complexity and storage requirement for PS and GS compared with uniform QAM.

For PS

  - The demapper complexity is similar to corresponding uniform QAM demapper complexity

    - Can report the ratio of demapper complexity with PS over the demapper complexity of NR MCS with the same spectrum efficiency

  - The DM complexity and/or storage requirement depends on DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol

    - Can normalize the complexity by number of information bits or numbers of Res

    - For storage requirement, need to report the storage needed for supporting all MCS in the table

  - The DM processing delay depends on block length

  - Parallelism inversely proportional to block length

For GS,

  - The demapper complexity should be compared with uniform QAM demapper complexity

    - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity

      - As a function of 1D-NUC or 2D-NUC

      - As a function of # of spatial layers

      - Depends on LMMSE receiver or rML receiver

    - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed

  - The storage requirement for storing all the constellations in the MCS table

Please provide your view below:

| Company | Comments |
| --- | --- |
| Spreadtrum | We have a confusion about complexity of PS, does the demapper complexity include the complexity of DDM (Distribution De-matcher)? Because in the second bullet for PS, only the complexity of DM is mentioned, and we are not sure where the complexity of DDM is reflected. In our view, the complexity of DDM should not be ignored. We have also a confusion about processing delay in the fourth bullet for PS, we are not sure where the processing delay of DDM is reflected. |
| vivo | Regarding PS, the processing delay and parallelism depend on the DM design and the block length, and they should be the evaluation metrics of interest. Suggest to change to the wording to: - The DM processing delay as a function of DM design and block length - Parallelism/Serialism as a function of DM design and block length |
| Tejas | In our view for PS complexity of Distribution de-matcher (DDM) should be considered in calculating the overall complexity of PS. Similarly, latency aspects of DDM should be considered. |
| OPPO | Similar views as vivo. Based on our understanding, this proposal aims to collect the information on how to collect the results on complexity and storage requirement. So the observation-like sentences should be avoided in the proposal, such as “Parallelism inversely proportional to block length”. We think GS and PS can be compared with a unified framework including complexity, storage, delay, and parallelism/serialism, providing a comparable starting point. Besides, we think the complexity does not need to be normalized, and the complexity for both GS and PS can be compared based on the same code block settings instead. Suggested wording as: On how to evaluate complexity, storage, delay and parallelism/serialism requirement for PS and GS compared with uniform QAM. For PS - The demapper complexity of demmaper and DM should be compared with is similar to corresponding uniform QAM demapper complexity - Can report the ratio of demapper complexity with PS over the demapper complexity of NR MCS with the same spectrum efficiency - As a function of # of spatial layers - With given receiver type (LMMSE, rML receiver) - The DM/invDM complexity as a function of DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol - The DM/invDM storage requirement as a function of DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol - For storage requirement, need to report the storage needed for given MCS and/or all MCS in the table - The DM/invDM processing delay as a function of DM design and block length - Parallelism/Serialism of DM/invDM as a function of DM design and block length For GS, - The demapper complexity should be compared with uniform QAM demapper complexity - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity - As a function of 1D-NUC or 2D-NUC - As a function of # of spatial layers - With given receiver type (LMMSE, rML receiver) - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed - For storage requirement, need to report the storage needed for given MCS and/or all MCS in the table - Processing delay, if applicable - Parallelism/Serialism, if applicable |
| IMU | We agree with the companies’ observations. |

Discussion 2.3-6B

On how to evaluate complexity, storage requirement, delay and parallelism/serialism for PS and GS compared with uniform QAM.

For PS

  - The demapper complexity is compared with uniform QAM demapper complexity

    - Can report the demapper complexity with PS and the demapper complexity of NR MCS with the same spectrum efficiency, and the ratio of the complexities

      - Also report the number of spatial layers and the

  - The Distribution Matcher (DM)/Distribution De-Matcher (DDM) complexity and/or storage requirement depends on DM algorithm used (ESS, CCDM, etc), precision of fixed point implementation, block length, and the number of bit levels shaped per symbol

    - For complexity, can report the complexity normalized by the number of information bits or numbers of REs

    - For storage requirement, can report the overall storage needed for supporting all MCS in the table

  - The DM/DDM processing delay as a function of DM design and block length

  - Parallelism/serialism as a function of DM design and block length

For GS,

  - The demapper complexity should be compared with uniform QAM demapper complexity

    - Can report the ratio of GS demapper complexity over the uniform QAM demapper complexity

      - As a function of 1D-NUC or 2D-NUC

      - As a function of # of spatial layers

      - With given receiver type (LMMSE or rML)

    - Also need to report the assumption on complexity counting, e.g, fixed point assumed or floating point assumed

  - The storage requirement for storing all the constellations in the MCS table

  - Processing delay, if applicable

  - Parallelism/Serialism, if applicable

- Please provide your view below:

| Company | Comments |
| --- | --- |

## Discussions on new modulations for PAPR reduction

| Company | Proposals |
| --- | --- |
| Lekha | Observation 5: SOQPSK is a constant-envelope modulation scheme designed for power-efficient communication, particularly in bandwidth-constrained environments. It minimizes spectral sidelobes and offers excellent spectral efficiency. Proposal 4: Shaped Offset Quadrature Phase Shift Keying based Orthogonal Frequency Division Multiplexing (SOQPSK-OFDM) is proposed for 6G to provide bandwidth efficiency, reduced spectral leakage, and Bit Error Rate (BER). |
| ZTE | Observation 1: If the bandwidth of I-modulation is larger than that of the original π/2-BPSK/QPSK/QAM, it corresponds to spectrum expansion; if the bandwidth of I-modulation is smaller than that of the original, it corresponds to spectrum truncation. Observation 2: The I-modulation scheme can be achieved using either a time-domain approach or a frequency-domain approach. Observation 3: The I-modulation scheme is applicable to a wide range of modulation schemes, from low-order π/2-BPSK to high-order QAM. Observation 22: The I-modulation scheme achieves significant PAPR reduction compared to conventional π/2-BPSK, QPSK, and 16QAM with and without transparent FDSS. Observation 23: The I-modulation scheme provides better performance at 10% BLER compared to conventional π/2-BPSK, QPSK with transparent FDSS. Observation 24: For π/2-BPSK, the I-modulation scheme achieves approximately 3 dB net gain compared to conventional π/2-BPSK without transparent FDSS, and approximately 1 dB net gain compared to conventional π/2-BPSK with transparent FDSS. Observation 25: For QPSK, the I-modulation scheme achieves approximately 2.1 dB net gain compared to conventional QPSK without transparent FDSS, and approximately 1.8 dB net gain compared to conventional QPSK with transparent FDSS. Proposal 4: I-modulation (I-π/2-BPSK/QPSK/QAM) scheme should be considered to reduce PAPR for DFT-s-OFDM. |
| Panasonic | Proposal 2: In addition to 5G NR pi/2-BPSK modulation, RAN1 can assess the need to introduce MPR / PAPR reduction technique such as phase rotated QPSK targeting coverage enhancement, especially for UL. Proposal 3: For MPR / PAPR analysis, how relationship with ACLR, SEM, EVM, spurious emissions, and occupied bandwidth progress will require discussion in RAN1 as PAPR only does not determine the amount of UE Tx power. Proposal 4: RAN1 can assess the need to introduce sequence-based DMRS-less transmission for small information block length such as PUCCH with UCI bits up to 11 bits. Observation 1: From system perspective, there are many challenges to support DFT-s-OFDM in DL, for example to multiplex SSB and other channel jointly, and the, the motivation of low PAPR waveform and modulation in DL is unclear. |
| IMU | Proposal 3: RAN1 to study constellation shaping with DFT-s-OFDM for PAPR reduction and PA efficiency gains, using uniform QAM as the baseline and minimal TX/RX changes. Evaluate PCS/GCS with DFT-s-OFDM for PAPR/CCDF reduction and PA behavior, alongside BLER/throughput. Include effects of DM blocklength, PA models, and conformance (EVM, ACLR, MPR/A-MPR). |
| Lenovo | Observation 1: Utilizing π/2-BPSK for DL narrowband transmissions could offer significant reductions in repetitions (e.g., ~27%-38% for 6RBs-12RBs) needed to achieve desirable target BLER (e.g., 10%) under extended coverage. Proposal 1: Study and evaluate benefits (e.g., BLER, EE gains) vs. caveats (e.g., SE losses) of utilizing π/2-BPSK modulation in 6GR for DL narrowband extended coverage applications. |
| Xiaomi | Proposal 3: For enhanced modulation schemes for PAPR reduction based on uniform modulation, RAN1 clarifies under which agenda item the discussion should proceed. |
| MTK | Proposal 1: For DFT-s-OFDM, support O-QPSK for coverage enhancement. |

A summary of proposals for PAPR enhancements with legacy uniform QAM DFT-s-OFDM:

Offset-QPSK category: Lekha (SOQPSK), MTK( O-QPSK)

Interpolation modulation: ZTE

Phase rotated QPSK: Panasonic

- Sequence based DMRS-less UL transmission with small payload: Panasonic

- DL DFT-s-OFDM waveform:

Support to study: Lenovo

Motivation/benefit not clear: Panasonic

### First round discussion

Discussion 2.4-1

- For legacy constellation enhancement targeting PAPR reduction for DFT-s-OFDM waveform, there is active related discussion in waveform agenda item. The FL recommends to continue discussion in the waveform agenda item.

- Please provide your view if you think otherwise.

| Company | View |
| --- | --- |
| Xiaomi | Support FL’s recommendation. |
| OPPO | As the low-PAPR modulation was discussed in modulation agenda in 5G NR, it seems further coordination between modulation and waveform agenda is needed. |
| CMCC | Support. |
| Spreadtrum | We agree with FL's recommendation. Duplicate efforts should be avoided. |
| Lenovo | Support |
| CATT | Support |
| Panasonic | We support OPPO’s view. |
| Tejas | Support FL’s recommendation |
| DOCOMO | We agree with OPPO. |
| IDC | OK with continuing detailed PAPR and low-PAPR modulation discussions under the waveform AI, while keeping coordination with the modulation AI to avoid duplication. |
| Samsung | We agree with the FL’s recommendation. Low-PAPR–related topics should continue to be discussed in the waveform agenda item, as they are more closely tied to waveform design than to constellation shaping or modulation study. |
| Huawei, HiSilicon | We are fine to follow the FL’s recommendation. |
| ETRI | Support |
| IMU | Support |

Discussion 2.4-2

- For sequence based DMRS-less UCI transmission, the FL recommends to discuss in PUCCH agenda item, when it starts.

- Please provide your view if you have other suggestions.

| Company | View |
| --- | --- |
| Xiaomi | Support FL’s recommendation. |
| OPPO | Support. |
| CMCC | We think the sequence based DMRS-less UCI transmission is not relative to the modulation agenda. |
| Spreadtrum | Same comments as in discussion 2.4-1 |
| Lenovo | Agree |
| CATT | Support |
| Panasonic | OK |
| Tejas | Support FL’s recommendation |
| ZTE, Sanechips | Fine with FL’s recommendation |
| DOCOMO | Support. |
| IDC | OK |
| Samsung | Support. |
| Huawei, HiSilicon | We are fine to follow the FL’s recommendation. |
| ETRI | Support |

Discussion 2.4-3

- For DFT-s-OFDM waveform for DL, there is active related discussion in waveform agenda item. The FL recommends to continue discussion in the waveform agenda item.

- Please provide your view if you have other suggestions.

| Company | View |
| --- | --- |
| Xiaomi | Support FL’s recommendation. |
| OPPO | Similar views as in discussion 2.4-1. |
| CMCC | Support. |
| Spreadtrum | Same comments as in discussion 2.4-1 |
| Lenovo | Support |
| Panasonic | Support |
| Tejas | Support FL’s recommendation |
| ZTE, Sanechips | Fine with FL’s recommendation. |
| DOCOMO | Support. |
| IDC | OK |
| Samsung | Support. |
| ETRI | Support |
| IMU | Support |

## Discussions on joint channel coding and modulation

| Company | Summaries of evaluations provided |
| --- | --- |
| Lekha | Observation 6: Joint channel coding and modulation can optimize the overall communication system. For instance, it can adapt the modulation scheme based on the channel conditions and the coding strategy, leading to better performance in varying environments. Proposal 5: Learning based joint channel coding and modulation are crucial for 6G systems, where low latency and high data rates are essential. Several applications including semantic communications are found to benefit from the joint modulation and coding scheme. |
| Spreadtrum | Proposal 3: Don’t support joint channel coding and modulation in 6GR. - Note: it is not precluded to discuss “joint channel coding and modulation” use case in 6G AI. |
| DCM | Proposal 4: Interleaver-related enhancement design is not closely related to the modulation design and should be discussed in Channel coding agenda. |
| Oppo | Observation 17: To address the issue of unbalanced bit-level reliability for QAM constellation, the interleaver may have to be customized for every possible individual combination of code block length and modulation order. That would lead to additional implementation difficulties. |
| IDC | Observation 1: The baseline for evaluation of benefits of enhanced joint channel coding and modulation schemes can be NR modulation |
| AT&T | Proposal 5 LCM procedures for AI/ML-based joint source coding, channel coding and modulation is deprioritized in the 6GR study. • FFS: whether the joint source coding, channel coding and/or modulation is signaled/specified explicitly without an underlying LCM procedure. |
| Vivo | Proposal 4: The imbalance between different bits in QAM symbols should be considered in the mapping between coded bits and modulated symbols, for example, in the form of a better bit interleaver design. Observation 11: Compared with NR design, the cross-codeblock interleaver design, e.g., MGCM can better leverage the unequal sub-channel capacity of QAM modulated symbols, by jointly modulating multiple different (e.g., coding rates/schemes) coded blocks. Observation 12: In the scenario of UCI multiplexed with UL-SCH, the cross-codeblock interleaver design, e.g., MGCM, can greatly simplify the RE allocation pattern, and meanwhile achieve greater throughput (larger than 10% in some simulated scenarios) for UL-SCH or deliver more UCI payload compared to NR baseline. Proposal 5: Further study the cross-codeblock interleaver design, e.g., MGCM, as a solution for joint coding and modulation, considering at least the following two use cases:  Two SCH data blocks coded by LDPC using different coding rates  Multiplexed UCI and UL-SCH data blocks, respectively coded by Polar and LDPC |
| Xiaomi | Proposal 9: Reuse the 5G NR BICM framework in 6GR for coding-modulation concatenation. |
| Sony | Observation 1: Receiver tests confirm that DBICM is superior to NR LDPC Observation 2: DBICM requires many code blocks which is undesired Observation 3: TB-DBICM and CBI-BICM do not require many coded blocks Proposal 1: RAN1 should study bit-mappings for DBICM to be used in combination with DBICM, its variants, and/or BICM-ID. Proposal 2: RAN1 should study the benefits of DBICM and CBI-BICM over BICM Proposal 3: RAN1 should establish benchmark receiver architectures for CBI-BICM and TB-DBICM |
| Rakuten | Proposal 1: Study joint design of channel coding and modulation with turbo-like iterative decoding, enabling information exchange between the demodulator and channel decoder, alongside optimization of modulation labelling rules for 6GR. |

Summary of proposals under this topic:

(AI based) modulation adaptation:

  - Study: Lekha

  - Deprioritize: AT&T

Joint channel coding and modulation:

  - No need or no benefit to further study: Spreadtrum

  - Study: Rakuten

BICM related interleaver enhancements:

  - Should discuss under coding: DCM

  - Study: vivo (MGCM), DBICM (Sony)

  - No need for enhancements: Xiaomi

### First round discussion

Discussion 2.5-1

Companies are encouraged to evaluate the proposed schemes, including DBICM and its enhancements, MLC, MGCM with cross codebook interleaving, QC-block interleaving, and also encouraged to propose other schemes. However, given limited online/offline time and limited information provided, the moderator does not plan to discuss these topics before more information becomes available.

If you have other suggestions, please provide below.

| Company | Suggestions |
| --- | --- |
| OPPO | Support. |
| CMCC | As there is some similarity in the framework and principle (e.g., to optimize the bit-mapping) between probabilistic constellation shaping and joint channel coding and modulation, we are wondering whether they can be discussed together and share very similar evaluation assumptions. |
| CATT | OK with FL’s proposal |
| ZTE, Sanechips | We noticed that FL’s proposal is duplicated with the proposal in the channel coding agenda item. |
| Samsung | We believe that QC-block interleaving is a promising approach for jointly optimizing LDPC coding and modulation. However, because its behavior depends strongly on the LDPC code structure, the constellation, and the bit-to-symbol mapping, we think this topic should be discussed in detail only after those elements are finalized. Once the underlying coding and modulation choices converge, we can engage in a more meaningful and intensive discussion on QC-block interleaving and related schemes. |

# Agreements so far

R1-122:

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

R1-122bis:

Agreement

For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following:

- BLER performance under AWGN channel (at least for performance calibration)

  - 1st transmission (baseline) and with HARQ re-transmission

- BLER performance under fading channel with fixed MCS

  - 1st transmission (baseline) and with HARQ re-transmission

- Throughput performance with link adaptation (adaptive MCS and rank) under fading channel

  - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions)

- Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements,

- Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR

- Expected spec impact

- FFS detailed assumption of constellation shaping and improved MCS table

- System level evaluation can be done after link level evaluation.

Agreement

For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following

- Probabilistic shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE

    - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them

  - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission

  - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss)

- Geometric shaping for CP-OFDM and DFT-s-OFDM

  - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE

    - If multiple constellation shapes are provided for each SE, how to switch between them

  - GS mapping details, such as bit to constellation point mapping and shape

  - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission

Agreement

For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated.

- channel configuration, including Channel profiles,Tx/RX antenna settings

- For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available.

- Precoder assumption

  - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based)

    - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions,

    - or genie beamforming

  - Open loop MIMO

- Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL

- LLR demapper: Max-log (baseline) or Log-MAP

- Channel estimation: Realistic (baseline) or ideal

- Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage

- Note: For MIMO, SIMO, MISO and SISO are included when possible

Agreement

For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following:

- PAPR/CM of the resulting waveform

- EVM, MPR/A-MPR

Agreement

For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.

# References

RP-251881, New SID: Study on 6G Radio

R1-2508089, FL summary #3 on modulation, joint channel coding and modulation, Qualcomm

R1-2508339, On Modulation for 6G Radio Air Interface, Nokia

R1-2508356, Modulation, joint channel coding and modulation for 6GR Interface, Lekha Wireless Solutions

R1-2508359, Modulation for 6GR air interface, Ericsson

R1-2508390, Discussion on modulation, joint channel coding and modulation for 6GR, Spreadtrum, UNISOC

R1-2508435, Discussion on Modulation for 6GR air interface, vivo

R1-2508458, Discussion on modulation schemes for 6GR interface, CMCC

R1-2508598, Modulation for 6G network, CATT

R1-2508623, Discussion on 6GR modulation, Lenovo

R1-2508641, Views on Modulation for 6GR, AT&T

R1-2508687, Discussion on modulation for 6GR air interface, Xiaomi

R1-2508730, Discussion on modulation, joint channel coding and modulation for 6GR, OPPO

R1-2508738, Modulation for 6GR air interface, Huawei, HiSilicon

R1-2508746, Discussion on modulation for 6GR, Tejas Network Limited

R1-2508805, Disscussion on modulation for 6GR, Samsung

R1-2508822, Discussion on modulation for 6GR, ZTE Corporation, Sanechips

R1-2508871, Modulation, joint channel coding and modulation for 6GR air interface, InterDigital, Inc.

R1-2508888, Discussion on modulation for 6GR air interface, Panasonic

R1-2508911, Discussion on modulation for 6GR, LG Electronics

R1-2508976, Discussion on 6GR modulation, ETRI

R1-2509020, Discussion on Modulation for 6GR Air Interface, IMU

R1-2509044, Discussion on 6GR modulation schemes, Hanbat National University

R1-2509076, Discussion on joint coding and modulation with DBICM, Sony

R1-2509113, On modulation for 6G air interface, Apple

R1-2509146, Modulation for 6GR interface, MediaTek Inc.

R1-2509234, Modulation, joint channel coding and modulation, Qualcomm Incorporated

R1-2509285, Discussion on Modulation, NTT DOCOMO, INC.

R1-2509295, Efficient AIML Constellation Shaping, DeepSig Inc

R1-2509300, Discussion on modulation for 6GR air interface, Google Korea LLC

R1-2509352, Modulation, joint channel coding and modulation, CEWiT

R1-2509371, Discussion on Modulation and JCCM for 6GR Air Interface, Rakuten Mobile, Inc