3GPP TSG RAN WG1 #124	R1-260xxxx

# Gothenburg, Sweden, Feb 9th – 13st, 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUCCH power control in SBFD operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2026-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the CONNECTED mode, the UE can also perform CBRA. In this case, if PRACH is transmitted on the first PRACH occasions, the gNB may not know the UE capability of SBFD when scheduling Msg4 PUCCH, e.g. if the prior Msg3 does not convey C-RNTI of the UE. The issue of Msg4 PUCCH in the CONNECTED mode is similar to that in the initial access. Therefore, regardless of the RRC state, same principle for Msg4 PUCCH in the SBFD symbols should be followed, i.e., p0-nominal-sbfd is used only if the associated PRACH is transmitted in the second PRACH occasions. For PUCCH other than Msg4 PUCCH, if it is in SBFD symbols, p0-nominal-sbfd is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct that p0-nominal-sbfd is used when PUCCH is in SBFD symbols and associated with PRACH in the second PRACH occasions, or when PUCCH is in SBFD symbols and not associated with PRACH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for the CR |  |  |  |  |  |  |  |  |

### 7.2.1	UE behaviour

<Unchanged parts omitted>

-	PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal or, if p0-nominal-sbfd is provided and, the PUCCH transmission is in SBFD symbols and associated with a PRACH in the second PRACH occasions, or the PUCCH transmission is in SBFD symbols and not associated with a PRACH transmission, by p0-nominal-sbfd, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_UE_PUCCH(qu)=0, 0≤qu<Qu