3GPP TSG-RAN WG1 Meeting #124	R1-2601665

Gothenberg, Sweden, Feb 9th-13th, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.291 | CR | 0003 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on D2R transmission time determination in TS 38.291 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (CMCC), Qualcomm, OPPO, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current specification, the timing offset to transmit Msg3 or first D2R for CFA is specified assuming TBS up to 128 bits. However, SA2 agreed to extend the maximum size of device permanent ID beyond 128 bits which is carried in Msg3 or first D2R for CFA. Therefore, the timing offset should be revisited considering extended TBS of Msg3 and first D2R for CFA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the TBS is larger than 256-bit, use Table 7.1.2-2 to determine the processing time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When TBS of Msg3 or first D2R for CFA is larger than 256 bits, device may not be able to prepare the D2R transmission within a specified timing offset between corresponding R2D reception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.1.2	Device procedure for transmission time determination

<Unchanged parts omitted>

If the D2R transmission is for an Access Random ID message

-	if after chip χR2D= χendR2D  there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission

-	the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R

-	otherwise

-	the device shall set TR→D=Toffset

else if the D2R transmission with NTBSD2R≤32 corresponds to a R2D Random ID Response message or to a contention-free access procedure

-	the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-1 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used

Table 7.1.2-1

| Tchip' [μs] | Δcode [μs] |
| --- | --- |
| τ | 0 |
| τ/2 |  |
| τ/4 |  |
| τ/8 |  |
| τ/16 |  |
| τ/32 | τ |
| τ/64 |  |
| τ/128 |  |
| τ/192 |  |

otherwise

-	the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-2 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used

Table 7.1.2-2

| D2R block size [bytes] | Δcode [μs] |
| --- | --- |
| NTBSD2R≤32 | 2τ |
| 32<NTBSD2R≤64 | 4τ |
| 64<NTBSD2R≤125 | 8τ |

<Unchanged parts omitted>