3GPP TSG-RAN WG1 #75     	                                R1-135289

November 11-15, 2013

San Francisco, USA

Agenda item:	6.2.1.1.3

Source: 	Qualcomm Incorporated

Title: 	Remaining details of backhaul signaling for eIMTA

Document for:	Discussion and Decision

# Introduction

In RAN#74b [1], the followings have been agreed regarding backhaul signalling for interference mitigation (IM) in TDD eIMTA:

- The OI over X2 is subframe-set dependent (up to 2 sets)

  - For subframe-set dependent OI, the association of the subframe-set dependent OI with each subframe is determined by X2 message(s)

    - Details up to RAN3

- No consensus to introduce subframe-set dependent HII and RNTP for eIMTA

- No consensus to introduce information about a set of >1 UL-DL configurations over X2 for eIMTA

And the following working assumption was also agreed:

- No interference type and/or interference source for subframe or subframe-set OI for eIMTA

  - Companies are still encouraged to check whether or not there are significant benefits of introducing interference type and/or interference source

In this contribution, we discuss remaining details of backhaul signalling, especially whether to exchange interference type and/or interference source. The eNB-eNB interference measurement also is introduced.

# Exchange of interference type and/or source

In eIMTA, eNB-eNB interference can be much higher than conventional UE-eNB interference. Interference in flexible UL subframes may be different from that in fixed UL subframes. Distinguishing eNB-eNB interference and UE-eNB interference is important. One potential enhancement is to add OI capturing interference type, e.g. eNB to eNB interference. In this way, aggressor eNBs could respond to OI more appropriately. But since the scheduled UEs may be quite different in each subframe and dual-loop UL power control [2] may further impact the accuracy, the subframe set specific OI with interference type needs further study.

The above potential issues of OI with interference type can be solved by introducing interference source info through eNB-eNB measurement. Firstly, the eNB-eNB interference can be accurately estimated with eNB-eNB measurement. Secondly, victim eNB can measure the interference generated by individual eNBs, and thereby it can notify only those eNBs which cause strong interference.

System simulations have been performed to compare different OI type schemes in a multiple pico cells deployment scenario. The following four cases are studied:

- SD-OI: Subframe Dependent OI (SD-OI) is served as baseline.

- SD-OI-IT: Subframe Dependent OI with Interference Type (SD-OI-IT).

- SD-OI-IS: Subframe Dependent OI with Interference Source (SD-OI-IS).

- TA: Traffic Adaption (TA) is the baseline with no backhaul signaling.

From the simulation results, it can be observed that SD-OI-IT achieves marginal gain over SD-OI under different loading. SD-OI-IS can achieve similar packet throughput gain in UL as SD-OI-IT, with noticeable DL packet throughput gain over SD-OI-IT. Thus, subframe-dependent OI with interference source can provide more performance benefits than subframe-dependent OI with interference type.

Detailed simulation parameters are listed in Table 1 of the Appendix.

Figure 1 Comparison of OI-based IM schemes

Proposal 1: Interference source information should be supported in additional to subframe set dependent OI. .

# eNB-eNB interference measurement

Considering eNB-eNB measurement for the subframe-set dependent OI, the victim eNB should measure the interference from surrounding eNBs, and can coordinate only with those aggressor eNBs that cause strong interference. The individual eNB-eNB measurement assisting subframe-set dependent OI is necessary for interference coordination. By using this information, the aggressor eNBs can be notified accurately to conduct the interference mitigation. Current eNB measurements (Received Interference Power and Thermal noise power) may not be sufficient because they cannot distinguish the interference received from neighbor UEs and neighboring eNBs. To facilitate eNB-eNB interference measurement method, a variety of approaches can be applied, e.g.

- One approach is to use blank UL sub-frame for eNB-eNB interference measurement. The measuring cell can be configured with a relative UL-heavy configuration and the measured cell can be configured with a relative DL-heavy configuration. The measuring cell can avoid UL scheduling in some of the flexible UL sub-frames and measure CRS of the measured cells.

- Another approach is to use special sub-frame for eNB-eNB interference measurement. The measuring cell can select a short DwPTS configuration (e.g., cfg.0/5), and the measured cell can select a long DwPTS configuration (e.g., cfg.1/2/3/4/6/7/8). The measuring cell can measure CRS from the DwPTS of measured cell during GP.

Proposal 2: To support interference source information, consider exchanging subframe and special subframe configuration over X2 for inter-eNB coordination.

# Conclusions

In this contribution, we discussed the remaining details of backhaul signaling. Through analysis and simulation, we find that interference source information exchange in additional to subframe set dependent OI brings significant performance benefits. Thus, we propose:

Proposal 1: Interference source information should be supported in additional to subframe set dependent OI.

Proposal 2: To support interference source information, consider exchanging subframe and special subframe configuration over X2 for inter-eNB coordination.

# References

- Draft Report of 3GPP TSG RAN WG1 #74b.

- R1-133579, “Evaluation of interference mitigation schemes for LTE TDD eIMTA”, Qualcomm Inc.

# Appendix – System Level Simulation Assumptions

Table 1. System level simulation assumptions for Pico-Pico scenario.

| Simulation Scenario | Co-channel outdoor Pico-outdoor Pico cells |
| --- | --- |
| System bandwidth | 10 MHz |
| Carrier frequency | 2 GHz |
| Inter-site distance | 500 m; [case1 in 36.942] |
| Macro deployment | The typical 19-cell and 3-sectored hexagon system layout [36.942]. Note that macro cells are deployed but not activated |
| Outdoor Pico deployment | 40m radius, random deployment; [36.814] |
| Number of Pico cells per sector | 4 |
| Min. distance between outdoor Pico cells | 40m; [36.814] |
| Min. distance between UE and outdoor Pico | 10m; [36.814] |
| Outdoor Pico antenna pattern | 2D, Omni-directional; [36.814] |
| Outdoor Pico antenna gain | 5 dBi; [36.814] |
| UE antenna gain | 0 dBi; [36.942] |
| Outdoor Pico noise figure | 13 dB; [36.104] |
| UE noise figure | 9 dB; [36.814] |
| Outdoor Pico max transmission power | 24 dBm as in [36.104] |
| UE power class | 23 dBm (200 mW); [36.814] |
| Number of UEs per Pico cell | 10 UEs uniformly dropped around each of the Pico cells within a radius of 40m |
| Shadowing standard deviation between outdoor Pico cells | 6dB; [36.814] |
| Shadowing correlation between UEs | 0 |
| Shadowing correlation between outdoor Picos | 0.5; [36.814] |
| Pathloss model |  |
| Outdoor Pico to outdoor Pico | LOS: if R<2/3 km, PL(R)=98.4+20log10(R) [ free space loss] else PL(R)=101.9+40log10(R), R in km [ Dual slop model TR25942 section5.1.4.3] NLOS: PL= 40log10(R)+169.36, R in km [25.942:section 7.4.1.2.1.4 TR 101 112(ETSI):Annex B1.8.1.2] Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 the probability of Relay-UE case1] |
| Outdoor Pico to UE | PL LOS(R)=103.8+20.9log10(R) PL NLOS(R)=145.4+37.5log10(R) For 2GHz, R in km Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 Pico-UE] |
| Penetration loss | 0 dB (Not modeled) |
| UE to UE | If R<=50m, PL=98.45+20*log10(R), R in km If R>50m, PL=55.78 +40*log10(R), R in m (Xia model) [Section 7.4.1.2.1.4 of TS25942, Annex B1.8.1.2 of TR 101 112(ETSI), ETSI STC SMG2 UMTS L1#9 Tdoc 679/98] |
| Evaluation metrics | DL and UL metrics collected separately, following metrics can be used - Packet throughput - defined as the packet size over the packet transmission time, including the packet waiting time in the buffer |
| Time scale for reconfiguration | 10ms |
| Simulation methodology | DL and UL shall be evaluated in an integrated simulator |
| Scheduler | FIFO |
| Pico antenna configuration | 1Tx, 1Rx |
| UE antenna configuration | 1Tx, 1Rx |
| Adaptation method of UL-DL reconfiguration | The standard set of seven LTE UL-DL configurations are used for adaptation. The traffic adaptation algorithm was based on the estimation of the required number of the DL and UL subframes by taking into account the amount of data in DL/UL user queues. |
| Link adaptation | MCS selection with 10% BLER If the highest MCS is selected, the BLER may be less than 10% |
| UE UL Power control | Open Loop Power Control P0 = -76 dBm, α = 0.8 on regular subframes |
| Set of TDD UL-DL configurations | All seven TDD UL-DL configurations |
| Small scaling fading channel | ITU UMa |
| CP length | Normal CP in both downlink and uplink. |
| Special subframe configuration | Special subframe configuration #8 |
| Packet drop time | The packet drop time is modeled according to 36.814 |
| Receiver type | MMSE receiver |
| UL modulation order | All modulations {QPSK, 16QAM, 64QAM} can be used as the UL modulation order |
| Shadowing standard deviation between outdoor Pico and UE | 3dB for LOS and 4dB for NLOS; [ ITU-R M.2135 UMi] |
| Traffic model | Same traffic generation methodology and arriving rate as agreed in isolated cell case [R1-120080], independent traffic generation per cell. Same arriving rate for all the cells |
| Reference TDD configuration | TDD UL-DL # 1 |
| Backhaul latency | ideal backhaul |