3GPP TSG RAN WG1 Meeting #76,	R1-140754

Prague, Czech Republic, 10th – 14th February 2014

Source:	Ericsson

Title:	MCS table design for downlink 256QAM

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

# Introduction

In RAN plenary meeting #62, a new WI on small cell enhancements – physical layer aspects was approved and it was decided to introduce 256QAM in the downlink for small cells in Rel-12 according to below agreement quoted from [1]:

- Spectrum efficiency enhancement with introduction of higher order modulation, i.e., 256QAM, in the downlink transmission, while keeping existing size of CQI feedback field and MCS indication.

The standard impacts of introducing downlink 256QAM have been discussed and the physical layer aspects in RAN1 mainly include CQI/MCS/TBS table design. In this contribution, we provide our view on MCS table design to support downlink 256QAM. The CQI table design and TBS table design is discussed in our companion contribution [2] and [3].

# Discussion

According to the WID [1], we keep the number of MCS indication bits to minimize the standard impacts, i.e., MCS table is designed with the same size as Rel-8 MCS table (32 entries).

As discussed in [2], the design objectives for Rel-12 CQI/MCS/TBS tables supporting 256QAM should strive to minimize specification and UE implementation/cost impacts. A good design should aim at covering 256QAM efficiently and meanwhile, not compromising link adaptation performance for existing modulation schemes or RRC/VoIP robustness as in Rel-8.

Observation:

- The design objectives for Rel-12 CQI/MCS/TBS tables supporting 256QAM should strive to minimize specification and UE implementation/cost impacts. The tables should cover 256QAM efficiently and not compromise link adaptation performance or RRC/VoIP robustness as in Rel-8.

Herein we provide the MCS table design taking into consideration of above design objectives. The proposed MCS table for 256QAM support is given in Table 1 in comparison with Rel-8 MCS table. The design principles/criteria are explained below for MCS entry design and reserved entry design separately.

Table 1: Rel-8 MCS table & Proposed MCS table for 256QAM support

| (a) Rel-8 PDSCH MCS Table |  |  |  |  | (b) Proposed PDSCH MCS Table for 256QAM support |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS | Mod | TBS | Target r×1024 |  | MCS | Mod | TBS | Target r×1024 |
| 0 | 2 | 0 | 120 |  | 0 | 2 | 0 | 120 |
| 1 | 2 | 1 | 157 |  |  |  |  |  |
| 2 | 2 | 2 | 193 |  | 1 | 2 | 2 | 193 |
| 3 | 2 | 3 | 251 |  |  |  |  |  |
| 4 | 2 | 4 | 308 |  | 2 | 2 | 4 | 308 |
| 5 | 2 | 5 | 379 |  |  |  |  |  |
| 6 | 2 | 6 | 449 |  | 3 | 2 | 6 | 449 |
| 7 | 2 | 7 | 526 |  |  |  |  |  |
| 8 | 2 | 8 | 602 |  | 4 | 2 | 8 | 602 |
| 9 | 2 | 9 | 679 |  |  |  |  |  |
| 10 | 4 | 9 | 340 |  | 5 | 4 | 9 | 340 |
| 11 | 4 | 10 | 378 |  | 6 | 4 | 10 | 378 |
| 12 | 4 | 11 | 434 |  | 7 | 4 | 11 | 434 |
| 13 | 4 | 12 | 490 |  | 8 | 4 | 12 | 490 |
| 14 | 4 | 13 | 553 |  | 9 | 4 | 13 | 553 |
| 15 | 4 | 14 | 616 |  | 10 | 4 | 14 | 616 |
| 16 | 4 | 15 | 658 |  | 11 | 4 | 15 | 658 |
| 17 | 6 | 15 | 438 |  | 12 | 6 | 15 | 438 |
| 18 | 6 | 16 | 466 |  | 13 | 6 | 16 | 466 |
| 19 | 6 | 17 | 517 |  | 14 | 6 | 17 | 517 |
| 20 | 6 | 18 | 567 |  | 15 | 6 | 18 | 567 |
| 21 | 6 | 19 | 616 |  | 16 | 6 | 19 | 616 |
| 22 | 6 | 20 | 666 |  | 17 | 6 | 20 | 666 |
| 23 | 6 | 21 | 719 |  | 18 | 6 | 21 | 719 |
| 24 | 6 | 22 | 772 |  | 19 | 6 | 22 | 772 |
| 25 | 6 | 23 | 822 |  | 20 | 6 | 23 | 822 |
| 26 | 6 | 24 | 873 |  | 21 | 8 | 23 | 617 |
| 27 | 6 | 25 | 910 |  | 22 | 8 | 24 | 655 |
| 28 | 6 | 26 | 1054 |  | 23 | 8 | 27 | 711 |
|  |  |  |  |  | 24 | 8 | 28 | 765 |
|  |  |  |  |  | 25 | 8 | 29 | 817 |
|  |  |  |  |  | 26 | 8 | 30 | 865 |
|  |  |  |  |  | 27 | 8 | 31 | 911 |
|  |  |  |  |  | 28 | 8 | 32 | 950 |
| 29 | 2 | reserved |  |  | 29 | mod↑ | reserved |  |
| 30 | 4 |  |  |  | 30 | same |  |  |
| 31 | 6 |  |  |  | 31 | mod↓ |  |  |

## MCS entry design

For MCS entries in the new table, the low SINR region is down-sampled to create spare MCS entries for 256QAM. In the QPSK region, MCS 0, 3, 6 and 9 are reserved, equivalent to 2dB sampling space. It should be noted that TBS 0, i.e. the first entry in Rel-8 MCS table, is kept to maintain the same RRC/VoIP robustness as Rel-8, as well as matching CQI table design discussed in [2].

The entries for medium SNR region from Rel-8 MCS table are kept for good resolution in the new MCS table. All the 16QAM entries and the majority part of 64QAM entries are kept, i.e., MCS 10-25 are unchanged in the new table.

In the high SNR region, 8 entries for 256QAM are obtained by sampling with equal spacing. It should be noted that MCS 25 rate is available as both 64QAM and 256QAM and MCS 26 is modified to be 256QAM with same spectral efficiency and extra 7 entries for 256QAM are added, of which six new entries contain six new TBS indices 27-32 in the MCS table. The new TBS indices correspond to six new TBS rows with new TBS values (across all possible bandwidths) that are required to attach to the current TBS table. The detailed design on TBS table is further discussed in another contribution [3]. The spectral efficiencies for 28 MCS entries in the new MCS table are shown in Figure 1 and the sample spacing is marked at the right side.

It can be seen that the majority entries of Rel-8 MCS table are kept. A larger SNR region is covered with good resolution especially in medium and high SNR region. With enough MCS in the low SNR region, the VoIP and RRC robustness is guaranteed. This MCS table is also very well matched with CQI table proposed in [2].

Figure 1 Spectral efficiency for MCS entries in new MCS table

## Reserved entry design

As in the Rel-8 MCS table, the new MCS table has 28 MCS entries and 3 reserved entries for re-transmission. With 256QAM introduced, the retransmission procedure may use one of the four modulation schemes. As shown in Table 1, an offset mechanism is introduced to adjust the modulation scheme adopted in retransmissions. There can be 2 alternatives that can be considered for the reserved entries:

Alternative 1:

- Index 29 indicates that retransmission should use the next higher modulation than original transmission. For example, 16QAM should be used if QPSK was used in the original transmission.

- Index 30 indicates that retransmission should use the same modulation scheme as in original transmission.

- Index 31 indicates that retransmission should use the next lower modulation scheme than original transmission. For example, QPSK should be used if 16QAM was used in the original transmission.

Alternative 2:

- If QPSK or 16QAM, or 64QAM is the original modulation order, the original Rel-8 design is applied, i.e., index 29, 30, 31 means QPSK, 16QAM and 64QAM to be used respectively as the modulation order for retransmission; if 256QAM is the original modulation order, index 29, 30, 31 means 16QAM, 64QAM and 256QAM, respectively.

## VoIP support

LTE is a wireless packet data communication system. To support voice applications, voice over internet protocol (VoIP) technology is deployed. The voice encoded packets have a range of transport sizes that have been specifically designed and supported in LTE. These transport block size are shown in Table 2 in bold face and red color.

Table 2: Rel-8 VoIP TBS

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 | 328 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 | 504 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 | 776 | 872 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | 936 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 | 1096 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 | 1256 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 | 1416 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1224 | 1384 | 1544 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1384 | 1608 | 1800 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1608 | 1800 | 2024 | 2280 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 | 1800 | 2024 | 2280 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 1992 | 2280 | 2600 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2152 | 2472 | 2728 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2280 | 2600 | 2984 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2536 | 2856 | 3240 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 2792 | 3112 | 3624 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 2984 | 3496 | 3880 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3240 | 3752 | 4136 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 3496 | 4008 | 4584 | 4968 |
| 22 | 520 | 1064 | 1608 | 2152 | 2664 | 3240 | 3752 | 4264 | 4776 | 5352 |
| 23 | 552 | 1128 | 1736 | 2280 | 2856 | 3496 | 4008 | 4584 | 5160 | 5736 |
| 24 | 584 | 1192 | 1800 | 2408 | 2984 | 3624 | 4264 | 4968 | 5544 | 5992 |
| 25 | 616 | 1256 | 1864 | 2536 | 3112 | 3752 | 4392 | 5160 | 5736 | 6200 |
| 26 | 712 | 1480 | 2216 | 2984 | 3752 | 4392 | 5160 | 5992 | 6712 | 7480 |

As for MCS table design, it is important that the new table can support many of the special TBS. With the proposed MCS table in Table 1 (b), it can be observed that all the special VoIP sizes can still be supported (i.e., the remaining rows without grey shading). Therefore, the proposed MCS table has a good support for VoIP TBS as well.

## Simulation results

Link level simulation has been performed using the new CQI table proposed in [2] and MCS table in this contribution. The achievable spectral efficiency for different MCS with 10% BLER target is shown in Figure 2 for both AWGN channel and EVA channel. It can be seen that the equidistant sampling is adopted well for each modulation order. The new MCS samples provide good resolution in medium and high SNR region, while at the same time keeping the robustness for low SNR region. It can be observed that the cross-overs between modulation orders are well designed to support both flat channels and dispersive channels.

Figure 2 Link level simulations using new CQI/MCS table (BLER 0.1)

Based on above discussions and link level results, we believe Table 1 (b) is a good candidate of MCS table for DL 256QAM support in Rel-12. Meanwhile, we make below proposals.

Proposal:

- Keep 28 MCS entries and 3 reserved entries for re-transmission with an offset mechanism to adjust the modulation scheme adopted in retransmissions.

- Retain TBS 0 in Rel-8 MCS table for RRC/VoIP robustness.

- Down-sample the lower SINR region to create spare MCS entries for the 256QAM region.

- New MCS table should provide good support and robust for VoIP TBS.

# Conclusions

In this contribution, we discussed MCS table design for supporting 256QAM in small cell scenarios, and made the following observations and proposals:

Observations:

- The design objectives for Rel-12 CQI/MCS/TBS tables supporting 256QAM should strive to minimize specification and UE implementation/cost impacts. The tables should cover 256QAM efficiently and not compromise link adaptation performance or RRC/VoIP robustness as in Rel-8.

Proposals:

- Keep 28 MCS entries and 3 reserved entries for re-transmission with an offset mechanism to adjust the modulation scheme adopted in retransmissions.

- Retain TBS 0 in Rel-8 MCS table for RRC/VoIP robustness.

- Down-sample the lower SINR region to create spare MCS entries for the 256QAM region.

- New MCS table should provide good and robust support for VoIP TBS.

# References

RP-132073, “WID Small cell enhancements-PHY”, Huawei

R1-140753, “CQI table design for 256QAM”, Ericsson

R1-140755, “TBS table design for 256QAM”, Ericsson