3GPP TSG-RAN WG1 #79	R1-144670

San Francisco, USA, 17th – 21st Nov 2014

Source: 	Intel Corporation

Title:                     	Potential Performance of FD-MIMO Systems with 8 TXRUs

Agenda item:	    6.3.3.3

Document for:	Discussion and Decision

# Introduction

In RAN1 #78bis meeting, companies have agreed the simulation parameters for phase one simulation [1]. The major goal of phase one simulation is to evaluate Rel-12 DL MIMO performance using the latest 3D channel model. In [2], the terminologies are clarified with regard to defining one 2D antenna array. Although the parameters for defining a 2D antenna array for phase one simulation is relatively stable, the antenna configurations for the phase two study is still widely open. Those details include the array size in the phase two study, antenna virtualization techniques to map antenna elements to TXRU, etc. In [3], we have recommended to study antenna arrays with more than four columns of antenna elements. In this contribution, we would like to address the other important aspect of “TXRU virtualization” which would have significant impact on the performance of FD-MIMO systems. In [4], we gave some initial simulation results on the throughput improvement of FD-MIMO using sub-space based TXRU virtualization technique and full buffer traffic model. In this contribution we would like to provide more system level results based on FTP traffic model one which is considered to be more realistic than full buffer traffic model by 3GPP.

# 2D TXRU Virtualization Technique

We consider a 2D antenna array defined in [2]. It is illustrated in Figure 1 for convenience.

Figure 1: 2D antenna array defined in [2].

Mathematically, the received signal in an FD-MIMO system can be described by:

y=HPx+n                                                                               (1)

where y is the Nr×1 received vector; H is the Nr×Nt channel matrix; P is the Nt×Np precoding matrix; x is the  Np×1 data symbol vector; n is the Nr×1 noise vector; Nr is the number of receiving antennas; Nt is the number of antenna elements, and Np is the number of layers. If the employed antenna array is a 2D antenna array as shown in Figure 1, Nt=2NM and Nt is usually much greater than 8. For example, when N = 4 and M = 8, Nt=64. Please be noted that if M=1, the 2D antenna array is reduced to one dimension antenna array.

Without adding a larger number of TXRUs, one way to implement the system is to virtualize the total Nt antenna elements into Nc∈1, 2, 4, 8 TXRUs and use one to one mapping from TXRU to CSI-RS antenna port. This allows implementing FD-MIMO technology with minimum cost. By doing so, (1) can be written as:

y=HPcPdx+n= HeffPdx+n                                                            (2)

where Pc is an Nt×Nc matrix; Pd is an Nc×Np matrix; Heff=HPc is the effective channel matrix with dimensions Nr×Nc, and Nt=NcK.

The proposed antenna virtualization method is described by the following steps:

Step 1: The eNB estimates each UE uplink channel matrix Hs  from uplink sounding.

Step 2: The eNB calculates the composite channel covariance matrix Rave:

Rave = 1Ss=1S(Hs) H(Hs)                                                                 (3)

where S denotes number of UEs in the cell served by the eNB.

Step 3: The eNB performs singular value decomposition (SVD) of the composite channel covariance matrix

Rave = V∆VH                                                                            (4)

where V is the matrix of eigenvectors of Rave  and ∆ is a diagonal matrix of eigenvalues of Rave.

Step 4: The eNB uses the Nc strongest eigenvectors to construct the antenna virtualization matrix according to:

Pc=v1⋯vNc                                                                        (5)

where vn, n=1…Nc, denotes the n-th eigenvector of V.

Step 5: The UE measures the CSI from the CSI-RS of the Nc ports, quantizes the amplitude and phase of its preferred precoder using scalar quantization and feeds back the channel quality indicator (CQI) together with the quantized precoder to the eNB.

Due to the existence of spatial reciprocity in FDD systems [7][8][9][10], the proposed 2D TXRU virtualization technique is applicable for both TDD and FDD systems.

# System Level Performance of 2D TXRU Virtualization

In this section, we present the system level throughput gain of 2D TXRU virtualization compared to the phase one baseline results we provided in [5] using FTP traffic model one. For FD-MIMO systems, we use a 2D virtualization technique to map 64 (N=4, M=8) antenna elements to 8 TXRUs and UE uses wideband beamforming based feedback. In the simulation, we update the TXRU virtualization matrix each time one new packet arrives according to section 2 and the same TXRU virtualization matrix is used throughout the packet life cycle. When the traffic load is high, the update of TXRU virtualization matrix can be made even slower, e.g. not once every time a new packet arrives. This is due to the fact that the channel sub-space for one given cell can be eventually stable after refining it using enough channel realizations. So at certain load adding more users into the cell does not change the sub-space further. Those 8 TXRUs are mapped to 8 CSI-RS ports using one to one mapping to cover the three-dimensional cell. A greedy scheduler tries to maximize the throughput per RBG by testing MIMO hypotheses from SU-MIMO to MU-MIMO including all active UEs. For the phase one results, 8 vertical antenna elements are mapped to one CRS port using DFT based narrow vertical beam. In the case of the FD-MIMO, single antenna element gain pattern is applied for one CRS port in order to maximize its throughput [6].

Table 1: System Throughput Gain of FD-MIMO VS. Phase One, UMa with 500m ISD, 2GHz

Table 2: System Throughput Gain of FD-MIMO VS. Phase One, UMa with 200m ISD, 2GHz

Table 3: System Throughput Gain of FD-MIMO VS. Phase One, UMa with 200m ISD, 3.5GHz

Table 4: System Throughput Gain of FD-MIMO VS. Phase One, UMi with 200m ISD, 2GHz

Table 5: System Throughput Gain of FD-MIMO VS. Phase One, UMi with 200m ISD, 3.5GHz

From the results we presented from Table 1 to Table 5, we can observe significant user throughput gain in all the tested FTP loads. Depending on the scenario and tested FTP load, the cell edge user throughput gain varies from the lowest possible gain 16% (UMa with 500m ISD at 2GHz and FTP load 1.5) to the highest possible gain 202.9% (UMa with 200m ISD at 3.5GHz and FTP load 4). Similarly the 50%-ile user throughput gain varies from 21% to 93% and the mean user throughput gain varies from 15% to 88% respectively. As such, we have below observation and proposal:

Observation: Significant user throughput gain can be obtained using sub-space based 2D TXRU virtualization technique to virtualize 64 antenna elements to 8 TXRUs.

Proposal: Capture the sub-space based 2D TXRU virtualization technique in this paper into the FD-MIMO TR.

# System Level Performance of 1D TXRU Virtualization

As described in [3], 1D TXRU virtualization can be one alternative to 2D TXRU virtualization. One of such 1D TXRU virtualization can be simply defined by using one length K DFT vector to map K consecutive antenna elements in the vertical dimension into one TXRU. For example, we can use K=4 DFT vector to map the antenna elements of one 2D array with (N=2, M=8, P=2) into 8 TXRUs. And we use the antenna down-tilt values defined for phase one simulation for the 1D virtualization vector.  In this section we compare the performance of 1D TXRU virtualization with 2D TXRU virtualization technique we provided in section 2 for the same antenna array and number of TXRUs. For 1D TXRU virtualization, we used non-quantized Eigen beams with rank adaptation. On the other hand for 2D TXRU virtualization, we use quantized Eigen beams with rank adaptation. Thus the 2D TXRU virtualization is put to disadvantage in the comparison. Table 6 presents the user throughput of 2D virtualization and the relative gain over 1D virtualization. It can been seen that only in one data point when the system load is extremely low, 1D virtualization has slightly better performance than 2D virtualization. And this can be due to the ideal CSI quantization of 1D virtualization. As the FTP load becomes higher, 2D virtualization significantly outperforms 1D virtualization. And the highest cell edge user throughput gain is over 100% in FTP load 4. And we did not list the user throughput results for the scenarios as they show the same trend.

Table 6: System Throughput Gain of 2D VS. 1D TXRU Virtualization for (N=2, M=8, K=4), UMa with 200m ISD, 3.5GHz

Observation 2: 2D TXRU virtualization significantly outperforms 1D TXRU virtualization.

# Conclusion

In this contribution, we presented one way to virtualize 64 antenna elements into 8 TXRUs using sub-space based 2D virtualization. The proposed virtualization technique only relies on long term spatial reciprocity not short term channel reciprocity, thus it is applicable to both TDD and FDD systems. The update rate of the TXRU virtualization matrix can be as slow as the packet arrival rate or even slower. We also compared the user throughput gain over Rel-12 baseline MIMO system defined in the phase one simulation. Our results show that significant user throughput gain can be achieved in all tested FTP loads and all scenarios. As such, we propose to capture the 2D TXRU virtualization technique in this paper into the FD-MIMO TR. Below are the recapped observation and proposal:

Observation 1: Significant user throughput gain can be obtained using sub-space based 2D TXRU virtualization technique to virtualize 64 antenna elements to 8 TXRUs.

Observation 2: 2D TXRU virtualization significantly outperforms 1D TXRU virtualization.

Proposal: Capture the sub-space based 2D TXRU virtualization technique in this paper into the FD-MIMO TR.

# References

- R1-144444, WF on simulation assumptions, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, WF on Antenna Modelling, Samsung et al.

- R1-144668, Antenna Array Modelling for Phase Two Evaluation, Intel

- R1-143780, Initial Evaluation of Elevation Beamforming and Full Dimension MIMO, Intel

- R1-144669, System Evaluation of Rel-12 Systems with 3D Channel Model, Intel

- R1-144096, On Cell Association in FD-MIMO Systems, Intel

- IEEE 802.20 Mobile Broadband Wireless Access Working Group, “Channel Models for IEEE 802.20 MBWA System Simulations”, Version 9, Revision 1, July 2005.

- K. Hugl, K. Kaliola, and J. Laurila, "Spatial reciprocity of uplink and downlink radio channels in FDD systems," in Proc. COST 273 Technical Document TD(02)066, p. 7, Espoo, Finland, May 2002.

- C. Brunner, M. Joham, W. Utschick, M. Haardt, J.A. Nossek,  "Downlink Beamforming for WCDMA Based on Uplink Channel Parameters," Proc. EPMCC 1999, pp. 375-380, Mar. 1999.

- R1-100853, Ericsson, ST-Ericsson, Channel reciprocity in FDD systems including systems with large duplex distance” TSG-RAN WG1 #60, San Francisco, USA, 22 — 26 February 2010.