3GPP TSG RAN WG1 Meeting #80		R1-150684

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.2.2

Source:	InterDigital

Title:	Baseline performance for Elevation Beamforming/FD-MIMO

Document for:	Discussion

# 1	Introduction

In RAN1 #79, it is agreed that for a given antenna array configuration, an enhancement proposal should be evaluated against a baseline case, where the enhancement case requires specifications change and the baseline case is considered to have no specification impact to Rel-12 [1].  In this contribution, we provide a baseline case assumption and the baseline performance which is believed to provide the best performance achievable using Rel-12 specifications.

- For a given antenna array configuration, an enhancement proposal that requires specifications change should at least be provided with the following:

  - A baseline case

    - A baseline is considered to have no specification impact to Rel-12 and providing the best performance achievable using Rel-12 specifications

  - An enhancement case

    - An enhancement is considered to have specification impact to Rel-12

  - The enhancement case should at least be evaluated against the  baseline case(s)

  - Antenna array configuration is given by the parameters {M,N,P,Q}

  - Baseline and Enhancement cases assume the same values for M, N, P, Q

    - 1D TXRU virtualization: The total number of associated TXRUs:  Q= MTXRU * N * P according to TXRU model-1 (as defined in RAN1#78bis)

    - 2D TXRU virtualization: The total number of TXRUs Q should be described by the proponent

In addition, below requirements are made for the companies to present their baseline cases.

- Companies are encouraged to share details of the baseline case

  - TXRU virtualization weights (for 1D or 2D virtualization)

  - CSI-RS to TXRU virtualization

  - CSI-RS port indexing

  - Cell association weights and method

  - SRS configurations including the number of SRS ports, periodicity, SRS bandwidth

Therefore, in this contribution, we provide our baseline scheme for EBF/FD-MIMO and its performance in 3D-UMa and 3D-UMi.

# 2	Baseline assumption

In [2], it is shown that the adaptive downtilt scheme which optimize downtilt angle in each cell provides reasonable amount of performance gains by exploiting elevation beamforming gain compared to the fixed downtilt case. The elevation beamforming gain could be further increased by adapting the beams in a UE-specific manner with the assumption that eNB has a full capability to change the elevation beam per UE at the same time (i.e. the number of TXRU is the same as the number of antenna elements). In this case, a UE may report a preferred elevation beam (such as the one giving the highest effective SINR or wideband SINR)  by using multiple CSI processes, thus up to 4 elevation beam may be used in a standard transparent manner.

The table 1 shows the assumption used for UE-specific elevation beamforming with multiple CSI processes as a baseline scheme.

Table 1: Baseline assumption

| Parameter | values | values |
| --- | --- | --- |
| Scenario | fixed downtilt | UE specific BF |
| Antenna array configuration (M,N,P,Q) | (8,4,2,8) | (8,4,2,8) supporting UE specific downtilt BF via multiple CSI processes |
| TXRU virtualization weights. | 1 TXRU per column per polarization, DFT weighted, all TXRU using the same fixed downtilt | 1 TXRU per column per polarization, DFT weighted, all TXRU using the same fixed downtilt selected from a downtilt set for UE specific BF |
| CSI-RS to TXRU virtualization | one TXRU mapped to one CRI-RS port | one TXRU mapped to one CRI-RS port |
| CSI-RS port indexing | As in Rel.10, port 15-18 mapped to one polarization (+45 deg) and port 19-22 mapped to the other polarization (-45 deg) | For each CSI process, port 15-18 mapped to one polarization (+45 deg) and port 19-22 mapped to the other polarization (-45 deg), i.e., 1) port 15-22 for downtilt angle 1 in CSI process -1. 2) port 15-22 for downtilt angle 2 in CSI process -2. 3) port 15-22 for downtilt angle 3 in CSI process -3. 4) port 15-22 for downtilt angle 4 in CSI process -4 |
| Cell association weights | CRS port 0 mapped same as CSI-RS port 15, i.e., [1 0 0 0 0 0 0 0] | CRS port 0 mapped same as CSI-RS port 15, i.e., [1 0 0 0 0 0 0 0] |
| Cell-association method | RSRP on CRS port 0 fixed at 100 deg downtilt | RSRP on CRS port 0 fixed at 100 deg downtilt |
| DFT weighted downtilt (set) | [100] for UMa and UMi | four downtilt angles (i.e. four elevation beams), each applied to one CSI process [98, 100, 102, 104] for UMa; [80, 85, 100, 105] for UMi. |

# 3	Baseline Performance

The baseline (UE specific downtilt) performance is evaluated in this section based on the above assumption.   The UE specific downtilt performance is compared with fixed downtilt for non-full buffer case with different traffic load. Further simulation assumption details are listed in Table A and Table B, which are based on the agreement in [3] and [4].

Table 2 shows the performance of UE specific downtilt and fixed downtilt in 3D-UMa channel.  As seen in the table, UE specific downtilt achieves more than 15% mean throughput gain at 40% RU and more than 20% mean throughput gain at 70% RU compared to fixed downtilt (100 deg) case.

Table 3 shows the performance of UE specific downtilt and fixed downtilt in 3D-UMi channel.  Up to 27% mean throughput gain and 25% mean throughput gain are observed respectively at ~30% RU and ~60% RU.

Table 2: Non-full buffer results (Mbps) – 3D-UMa

| Scenario, ISD, CF, downtilt | Medium RU (~40%) |  |  | High RU (~70%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 50% | 5% | Mean | 50% | 5% |
| 3D-UMa, 500m, 2 GHz, UE specific downtilt via multiple CSI process | 22.84 | 19.19 | 5.92 | 17.23 | 13.36 | 2.35 |
| 3D-UMa, 500m, 2 GHz, 100 deg | 19.93 | 15.17 | 4.56 | 13.94 | 10.64 | 2.37 |
| 3D-UMa, 500m, 2 GHz, 102 deg | 18.03 | 15.81 | 3.58 | 13.44 | 10.63 | 2.55 |
| 3D-UMa, 500m, 2 GHz, 104 deg | 17.75 | 15.28 | 3.9 | 12.43 | 10.21 | 1.96 |

Table 3: Non-full buffer results (Mbps) – 3D-UMi

| Scenario, ISD, CF, downtilt | Low RU (~30%) |  |  | High RU (~60%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 50% | 5% | Mean | 50% | 5% |
| 3D-UMi, 200m, 2 GHz, UE specific downtilt via multiple CSI process | 27.76 | 25.64 | 5.35 | 18.81 | 15.27 | 4.10 |
| 3D-UMi, 200m, 2 GHz, 100 deg | 21.74 | 18.34 | 5.30 | 15.07 | 12.59 | 3.64 |
| 3D-UMi, 200m, 2 GHz, 85 deg | 19.93 | 16.95 | 6.19 | 13.44 | 12.00 | 2.82 |
| 3D-UMi, 200m, 2 GHz, 80 deg | 20.29 | 16.25 | 6.94 | 15.00 | 12.44 | 3.00 |

# 4	Summary

In this contribution, we provided baseline scheme and performance for EBF/FD-MIMO.

# References

- RAN1 #79 Chairman’s note

- R1-144210, “Initial Evaluation Results for 2D antenna array”, InterDigital

- R1-144444, “WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, “WF on Antenna modeling”, Samsung, Nokia Networks, Nokia Corporation, CMCC, Huawei, HiSilicon, Interdigital, LGE, Motorola Mobility, Qualcomm, CHTTL

# Annex

The 2D Antenna array used in the simulation is configured as below.

There are M elements in a column and 2xN elements in a row. All the elements in one column per polarization are weighted (DFT-based weights) to form the desired downtilt angle θtilt.

Table A. System Level Simulation Assumptions for Adaptive Downtilt and Vertical Sectorization

| Parameter | Assumption |
| --- | --- |
| Network layout | 7-site 21-cell wraparound |
| Channel model | 3D Urban Macro (3D-UMa) 3D Urban Micro (3D-UMi) |
| eNB antenna configuration | (M,N,P,Q)=(8,4,2,8), 0.5 λ spacing in H, 0.8 λ spacing in V, cross-polarization (X-pol) [4] |
| UE antenna configuration | nR=2, cross-polarization, 0o/90o |
| UE attachment | RSRP on CRS port 0 [4] |
| Electrical Downtilt | θtilt = 100o for fixed downtilt case. [98 o 100 o 102 o 104 o ] for UMa; [80 o 85 o 100 o 105 o] for UMi. |
| Number of UEs per cell | 10 |
| UE distribution | uniformly dropped according to [3] |
| Traffic model | full buffer non-full buffer FTP model 1, packet size 0.5M bytes |
| Scheduler | proportional fair (PF) |
| Transmission scheme | SU/MU-MIMO dynamic switching with SU-MIMO feedback, Non-transparent MU-MIMO |
| Codebook | Rel.10 8Tx codebook |
| Link adaptation | AMC with OLLA, 10% BLER target |
| Receiver | MMSE-IRC, ideal channel estimation, ideal interference modelling |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered per 5ms |
| Receiver | feedback delay is 5 ms |
| Maximum number of HARQ retransmission | 4 |