3GPP TSG RAN WG1 Meeting #81	  R1-153033

Fukuoka, Japan, 25th - 29th May 2015

Source:	CMCC

Title:	Updated evaluation of feedback based on hybrid RS

Agenda Item:	6.2.5.2.2

Document for:	Discussion and Decision

- Introduction

Feedback enhancement has been the focus of MIMO enhancement since Rel-10. In the R13 3D-MIMO SI, similarly feedback could be the main discussion topic. From our perspective, the major issue for 3D-MIMO feedback enhancement is that the antenna configurations are very diverse and its number is huge, which causes much difficulty in feedback design. This contribution gives the simulation results of the feedback framework based on hybrid RS discussed in contribution [1].

- Feedback based on hybrid RS

As discussed in [1],  the feedback using a long-term CSI and a short-term CSI is proposed in order to reduce CSI-RS overhead and enable universal feedback for diverse antenna configurations. An example of hybrid RS procedure is as follows :

(a) Hybrid RS operation in TDD                                   (b) Hybrid RS operation in FDD

Figure 1. Procedure for  hybrid RS operation in TDD and FDD system

The following working procedure could be universal to different eNB antenna configurations :

Step 1: eNB transmits long-term RI-RS which is mainly for the purpose of reducing overhead of short-term CSI-RS. Example of such long-term RI-RS could be transmitted from two far-apart antennas to ensure RI is properly measured. It should be noted that W1 could be measured based on long-term reciprocity so it should be FFS if long-term RI-RS needs to support W1 measurement.

Step 2: UE reports long term channel information (e.g., RI) and SRS to eNB.

- In TDD system, the short-term channel reciprocity can be used.

- In FDD system, the long-term channel reciprocity may be used.

Step 3: eNB transmits precoded short-term CSI-RS to UE. One simple strategy is:

- If UE report RI = 2, transmit two port short-term CSI-RS to UE, each CSI-RS port corresponds to one eNB antenna polarization (and beamformed based on long-term direction information W1); UE would report co-phase between polarizations and two CQIs for two codewords.

- if UE report RI = 1,

  - In FDD system, transmit two port short-term CSI-RS to UE, each CSI-RS port corresponds to one eNB antenna polarization (and beamformed based on long-term direction information W1); UE would report co-phase between polarizations and one CQI for the codewords.

  - In TDD system, transmit one port short-term CSI-RS to UE, the CSI-RS port corresponds to both polarizations and beamformed based on CSI obtained from 1Tx SRS from UE (partial reciprocity)

Step 4: UE reports CQI calculated based on configured short-term CSI-RS and reported RI.

Step 5: PDSCH transmission.

The above procedure applies for diverse antenna configurations in 3D-MIMO, and could be even applicable to future massive MIMO in higher frequency bands.

The characteristics of double CSI-RS is summarized in the follownig table.

Table 1.  Double CSI-RS characteristics

|  | Long-term CSI-RS | Short-term CSI-RS |
| --- | --- | --- |
| Number of ports | Equal to the number of UE Rx antenna | Less than reported RI |
| Directionality | Cell-specific, broadcast | UE-specific, beamformed |
| Example periodicity | 160 ms | 5 ms |

Because short-term CSI-RS is UE specific, its overhead would be proportional to number of UEs. However based on current FTP model, the concurrent user is not more than 4 for typical data load; assuming each UE has two 2Rx antenna, then the maximum number of CSI-RS port for each UE is 2, which translates to the same overhead as current 8 CSI-RS ports. The total CSI-RS overhead, even in the extreme case, will not exceed totally 16 CSI-RS ports, which is quite acceptable in current LTE system.

- Simulation results of feedback based on hybrid RS

Performance evaluation results of feedback based on hybrid RS are shown in this section, while beamformed CSI-RS is applied as the baseline in FDD system and single layer transmission using channel reciprocity is applied as the baseline in TDD system. 3D-UMi with 200m ISD and antenna configuration (8,4,2,64) with one-to-one mapping are applied in the simulation. Assuming there are total 8 beams for beamformed CSI-RS, and each beam can transmit 8-port CSI-RS every 5ms. For hybrid RS scheme, eNB transmits long-term CSI-RS every 160ms, and transmits 2-port short-term CSI-RS every 5ms. UE only has one transmit antenna for SRS transmitting, and up to 2-layer could be transmitted per UE in hybrid RS scheme. Assume there are 3 symbols for DL CCHs, 2 CRS ports and DMRS with 12 REs per PRB. Therefore, the overhead is calculated in Table 2, where NUE represents the number of concurrent users.

Table 2. Overhead Comparison

|  |  | TDD |  | FDD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline | Hybrid RS | Baseline | UL overhead compression | Hybrid RS |
| DL (RE/subframe) | DL CCHs + CRS + DMRS | 60 | 60 | 60 |  | 60 |
|  | CSI-RS | 0 | 64/160+ NUE*2/5 | 8*8/5 |  | 64/160+ NUE*2/5 |
| UL (bit/subframe) | RI | 0 | 1/160 | 1/160 | (1+3+4)/160 | 1/160 |
|  | Co-phase (or PMI) | 0 | 1/5 | max. 8*8/5 | 4/5 | 1/5 |

We can see that, hybrid RS scheme can reduce a lot of PMI feedback and CSI-RS overhead in FDD system. The PMI feedback overhead is 1/64 of baseline, and CSI-RS overhead is 16% of baseline assuming 4 concurrent users in system. In order to reduce feedback overhead of the baseline scheme in FDD system, the 8 beams from vertical direction could be long-term selected using 3 bits and the PMI feedback could be 4-bit long-term plus 4-bit short term. Thereby, the feedback overhead can be reduced to (1+3+4)/160+4/5, which is still 4 times larger than that of Hybrid RS scheme. Further more hybrid RS scheme can achieve dual-layer transmission in TDD system compared to the baseline transmission schemes.

Table 3 and 4 give the system level simulation results of the hybrid RS using Full Buffer traffic model and FTP traffic model in both TDD and FDD system.

Table 3. System level simulation results using full buffer traffic model

|  | TDD |  | FDD |  |
| --- | --- | --- | --- | --- |
|  | Baseline | Hybrid RS | Baseline | Hybrid RS |
| Cell average (bps/Hz) | 2.337 | 2.796 | 2.660 | 2.819 |
| Cell edge (bps/Hz) | 0.1634 | 0.1557 | 0.1068 | 0.1052 |

Table 4. System level simulation results using FTP traffic model (λ=3.3)

|  | TDD |  |
| --- | --- | --- |
|  | Baseline | Hybrid RS |
| 5% UPT(Mbps) | 7.98 | 7.79 |
| 50% UPT(Mbps) | 20.93 | 25.59 |
| Mean UPT(Mbps) | 19.22 | 27.89 |
| RU | 46% | 35% |

From Table 3, we can see that in TDD system, hybrid RS scheme can improve average cell throughput by 20% with 4% performance loss in cell-edge. In FDD system, average cell throughput is increased by 6% and cell edge throughput is decreased by 2% while using hybrid RS scheme. Besides, hybrid RS can reduce a lot feedback overhead in FDD system.

From Table 4, in TDD system, although 2% performance loss is observed in 5% UPT, hybrid RS scheme can still improve 50% and mean UPT by 22% and 45%, respectively. Moreover, RU is reduced by 11% while adopting hybrid RS scheme.

- Conclusions

In this contribution, we give the system level simulation results of feedback framework based on hybrid RS, which is potentially applicable to diverse antenna configurations. The simulation results using full buffer traffic model show 20% average cell throughput improvement could be obtained in TDD system. In FDD system, average cell throughput is increased by 6% and cell edge throughput is decreased by 2%. Besides, hybrid RS can reduce a lot feedback overhead in FDD system. When FTP model is used in TDD system, although 2% performance loss is observed in 5% UPT, hybrid RS scheme can still improve 50% and mean UPT by 22% and 45%, respectively. Moreover, RU is reduced by 11% while adopting hybrid RS scheme.

References

- R1-150448, “Feedback framework for diverse 3D-MIMO antenna configurations”, CMCC, 3GPP TSG RAN WG1 Meeting #80, Athens, Greece, 9 – 13 February, 2015.

Annex

Table 1: 3D-MIMO Evaluation Assumptions

| Parameters | Values |
| --- | --- |
| Homogeneous scenarios | 3D-Umi ISD 200m |
| Polarized antenna modeling | Model-2 from 36.873 |
| Traffic model | Full Buffer, FTP 1 |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 3D-Umi ISD 200m: 2GHz |
| Downtilt | 3D-Umi ISD 200m: 100 degree |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions, |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Duplex Mode | TDD and FDD |
| Feedback | PUSCH 3-0 for reciprocity based operation |
| SRS | CQI reporting triggered per 5ms |
| Transmission scheme | Feedback delay is 5 ms |
| SRS | 1Tx, 5 ms periodicity, wideband |
| Transmission scheme | TM10, single CSI process, dynamic SU/MU-MIMO with rank adaptation (no CoMP) |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12REs per PRB |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |

Table 2: BS Antenna Configuration

| Parameter | Values |
| --- | --- |
| Number of columns (N) | 4 |
| Antenna Polarization (P) | P = 2: cross-pol (eNB: +/- 45 deg, UE: 0/90 deg) |
| Horizontal antenna element spacing dH | 0.5λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| Vertical antenna element spacing and Number of antenna elements with the same polarization in each column (dV , M) | (0.8 λ, 8) |