3GPP TSG RAN WG1 Meeting #81			R1-153383

Fukuoka, Japan, 25th – 29th May 2015

Agenda Item:	6.2.5.1

Source: 	CATR

Title: 	Baseline performance of Elevation Beamforming/Full-Dimension (FD) for LTE

Document for:	Discussion and Decision

Introduction

In RAN#65 meeting, the new study item of elevation beamforming/Full-Dimension (FD) MIMO for LTE [1] was approved. In RAN1# 80 meeting, Following four categories of baseline (a.k.a. implementation based enhancement) schemes are captured in TR 36.897 based on RAN1#80 contributions:

- Category 1:  Sectorization (in one or both of vertical and horizontal domains) with different cell-ID for each sector

- Category 2:  Virtual sectorization using one or more beamformed CSI-RS resource(s) with a single cell-ID (single sector as a special case)

- Category 3:  Kronecker precoding with 2 CSI processes

- Category 4:  SRS based precoding scheme in TDD

In this contribution, we provide evaluation results of category2 with full buffer traffic model based on the agreed evaluation assumptions.

Category 2 Evaluation Results

The details of baseline case for non-reciprocity operation are summarized in Table-1 and the other parameters are summarized in annex.

Table 1 Details of baseline case for non-reciprocity operation

|  | Case1 | Case2 |
| --- | --- | --- |
| Scenarios | Scenarios-1: 3D UMa with 500m ISD at carrier frequency 2GHz | Scenarios-1: 3D UMa with 500m ISD at carrier frequency 2GHz |
| Antenna array configuration (M, N, P, Q) | (8,4,2,16) | (8,4,2,32) |
| TXRU virtualization weights | 2 fully connected TXRUs per polarization in vertical dimension. each TXRU virtualized by an 8-element DFT weight with the downtilt values [80,100] | 2 fully connected TXRUs per polarization in vertical dimension. each TXRU virtualized by an 8-element DFT weight with the downtilt values [80,100] |
| CSI-RS to TXRU virtualization | One to one mapping. two CSI-RS resources each comprising of 8 horizontal CSI-RS ports with the downtilt values [80,100] | One to one mapping. two CSI-RS resources each comprising of 8 horizontal CSI-RS ports with the downtilt values [80,100] |
| CSI-RS port indexing | As in Rel.10, port 15-18 are co-polarized and port 19-22 are co-polarized. | As in Rel.10, port 15-18 are co-polarized and port 19-22 are co-polarized. |
| Cell association weights | CRS port 0 associated to a single TXRU with weights [1,0,0,...,0] or [0,1,0,…,0] | CRS port 0 associated to a single TXRU with weights [1,0,0,...,0] or [0,1,0,…,0] |
| Cell-association method | RSRP on CRS port 0 | RSRP on CRS port 0 |
| SRS configurations – number of SRS ports, periodicity, SRS bandwidth | none | none |

The details of the simulation assumptions are listed in Annex. The simulation results with full-buffer traffic model and FTP model are given in Table 1 and table 2.

Table 1: Simulation results of case 1 and case 2 with full-buffer model

| Configuration | Schemes |  | Cell edge user SE (bps/Hz/user) | Cell average SE (bps/Hz) |
| --- | --- | --- | --- | --- |
| 3D-UMA |  | 16TXRU | 0.08 | 3.08 |
| 3D-UMI |  | 16TXRU | 0.09 | 3.68 |

Table 2: simulation results of 16 TXRU and 32 TXRU with FTP channel model

| Scenarios | Traffic load | 16TXRU |  |  | 32TXRU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% UPT (Mbps) | 50% UPT (Mbps) | Mean UPT (Mbps) | 5% UPT (Mbps) | 50% UPT (Mbps) | Mean UPT (Mbps) |
| 3D-UMa | 20% | 10.45 | 24.12 | 25.47 | 11.03 | 25.24 | 27.03 |
|  | 50% | 7.16 | 20 | 21.69 | 8.26 | 21.38 | 23.24 |
|  | 70% | 4.81 | 15.94 | 17.97 | 5.52 | 17.39 | 18.5 |

Summary

In this contribution, we provide category 2 baseline evaluation results of 3D-UMa and 3D-UMi with full buffer and FTP model.

References

- RP-141644, “New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia Networks.

- 3GPP RAN1 #80 Chairman’s notes

- 3GPP TR 36.873, “Study on 3D channel model for LTE,” V2.0.0, Release 12, Mar. 2014.

Annex

Table 2: 3D-MIMO Evaluation Assumptions for Baseline Case

| Parameters | Values |
| --- | --- |
| Polarized antenna modeling | Model-2 from 36.873 |
| Number of users per cell | 10 |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Duplex Mode | FDD |
| Feedback | PUSCH 3-2 for non-reciprocity operation |
|  | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
|  | Rel-10 8Tx codebook based for non-reciprocity based operation |
| Transmission scheme | TM10, SU-MIMO with rank adaptation (no CoMP) |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB and CSI-RS overhead |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| CSI-RS periodicity | 5ms |