3GPP TSG RAN WG1 Meeting #82	      R1-154016

Beijing, China, 24th - 28th August 2015

Source:	Panasonic

Title: 	MIB and SIB for Rel.13 MTC

Agenda Item:		7.2.1.7

Document for:	Discussion and Decision

# Introduction

This document captures the PBCH content proposal in R1-152690 and SIB transmission proposal in R1-152691.

# Discussion

PBCH content

Related to the discussion on the determination of the start OFDM symbol in RAN1#80, whether unused bits in PBCH are used or not was discussed. Some offline discussion was held and report is provided in [1]. The same PBCH is used for both current UEs and Rel.13 MTC UEs. Therefore, backward compatibility is required to be kept.

According to RRC spec TS36.331 [2], PBCH content is defined as MIB (MasterInformationBlock). Ten spare bits are available. These 10 bits were intentionally kept reserved for future usage [3].

We propose 5 bits in reserved 10 bits in PBCH are used for the indication of TBS and time/frequency resource usage of MTC SIB1 and not to use "Physical downlink control channel for MTC". Using "Physical downlink control channel for MTC" for MTC SIB1 requires additional resources for Rel. 13 MTC and further delays the reception of MTC SIB1. In our view, 5 bits can be sufficient for the indication of SIB1 location. The table 1 is example of 5 bits usage in PBCH.

Regarding MTC SIB1 transport block size, we think 3 sizes could be sufficient. We are tempted to only one size for Rel.13 MTC SIB1 TBS but thinking for future usage/extension, 3 sizes are proposed.

Time location of MTC SIB1 agreement situation in the last RAN1 meeting is " Possible subframes are {0,4,5,9} for FDD and {0,5} for TDD. FFS subframes {1,6} for TDD. ". We don't think MTC SIB1 is located on every possible subframes. . As for MTC SIB1 resource utilization, once in every 20, 40 or 80ms order could be reasonable number in order to set total number of MTC SIB overhead less than 5%. Note that once in every 20, 40 or 80ms order does not mean to transmit exactly once in every 20, 40 or 80ms. It might be two or four subframes in every 20, 40 or 80ms. By taking into account frequency hopping, multiple channel estimation gain and DRX operation, how to transmit MTC SIB1 needs further discussion. The reason to have different TBS in the same overhead is that SIB1 is constant transmission and the reception delay is up to UE implementation. Larger TBS just means more delay if the resource utilization is same.

Table 1: Example of 5 bits usage in PBCH reserved bits

| 5 bits reserved bits value | Transport block size of MTC SIB1 | SIB1 resource utilization | Time/frequency location and frequency hopping |
| --- | --- | --- | --- |
| 0 | No Rel.13 MTC support | No Rel.13 MTC support | No Rel.13 MTC support |
| 1 | Small size | Once in every 80ms order | Location X. No hopping |
| 2 | Medium size | Once in every 80ms order | Location X. No hopping |
| 3 | Large size | Once in every 80ms order | Location X. No hopping |
| 4 | Small size | Once in every 80ms order | Location X. With hopping |
| 5 | Medium size | Once in every 80ms order | Location X. With hopping |
| 6 | Large size | Once in every 80ms order | Location X. With hopping |
| 7 | Small size | Once in every 40ms order | Location Y. No hopping |
| 8 | Medium size | Once in every 40ms order | Location Y. No hopping |
| 9 | Large size | Once in every 40ms order | Location Y. No hopping |
| 10 | Small size | Once in every 40ms order | Location Y. With hopping |
| 11 | Medium size | Once in every 40ms order | Location Y. With hopping |
| 12 | Large size | Once in every 40ms order | Location Y. With hopping |
| 13 | Small size | Once in every 40ms order | Location Y. With hopping |
| 14 | Medium size | Once in every 40ms order | Location Y. With hopping |
| 15 | Large size | Once in every 20ms order | Location Y. With hopping |
| 16 | Small size | Once in every 20ms order | Location Y. With hopping |
| 24 | Large size | Once in every 20ms order | Location Z. With hopping |
| 25 | Small size | Once in every 20ms order | Location Z. With hopping |
| 26 | Medium size | Once in every 20ms order | Location Z. With hopping |
| 27 | Large size | Once in every 20ms order | Location Z. With hopping |
| 28 to 31 | Reserved for future usage | Reserved for future usage | Reserved for future usage |

We propose following.

Proposal 1: 5 bits in current PBCH is used for TBS, time and frequency position of MTC SIB1.

System information

We propose the narrow band containing SIB1 and all the other SIBs are called as Primary narrow band. We further propose the narrowband not to contain SIBs is called as Secondary narrow band. A cell can have multiple secondary narrow bands.

Primary and secondary narrowband hopping is based on the modification of PUSCH sub-band-based frequency hopping pattern [4]. The Pseudo-random sequence generator for these frequency hopping patterns is initialized by cell ID, SFN and the parameter given as the time/frequency location and frequency hopping by PBCH.

Although large coverage case is noise/power limited deployment, no or small coverage case may be deployed in interference limited condition [5]. Narrow band hopping is used to alleviate the lack of frequency and spatial diversity for enhanced coverage. By adjusting the Pseudo-random sequence generator by the parameter given by PBCH, the amount of the collision among neighbor cells can be more manageable even PCI is same. Example that non-enhanced coverage does not collide PCIs but enhanced coverage channels collide PCIs is described in Fig.1 where the situation is different upper and under the ground. At upper the ground, PCIs separate neighbor cells as legacy manner. In under the ground with enhanced coverage, the interfered cells are different from cells on the ground. When the PCI coordination is not worked, the Pseudo-random sequence of frequency hopping can manage interference among cells with PCID=3.

Fig.1 Example of common channel collision when coverage is extended to under the ground

Based on the above, we propose following.

Proposal 2: To define Primary narrow band which contains SIB1 and the other SIBs. A cell can have only one primary narrow band.

Proposal 3: To define Secondary narrow band which does not contain SIB1 and the other SIBs. A cell can have multiple secondary narrow band.

Proposal 4: Pseudo-random sequence generator for primary and secondary hopping pattern is initialized by cell ID, SFN and the parameter given by PBCH.

# Conclusion

We discussed PBCH contents and SIB transmission. We propose following.

Proposal 1: 5 bits in current PBCH is used for TBS, time and frequency position of MTC SIB1.

Proposal 2: To define Primary narrow band which contains SIB1 and the other SIBs. A cell can have only one primary narrow band.

Proposal 3: To define Secondary narrow band which does not contain SIB1 and the other SIBs. A cell can have multiple secondary narrow band.

Proposal 4: Pseudo-random sequence generator for primary and secondary hopping pattern is initialized by cell ID, SFN and the parameter given by PBCH.

# Reference

- R1-150865, "MIB contents offline discussion", Panasonic, Nokia Networks, Nokia Corporation

- TS36.331

- R1-084063, " LS on transport block size on BCH", TSG-RAN WG1

- R1-153959, “Design of frequency hopping for MTC,” Panasonic

- R1-154017, " MTC terminology on normal coverage", Panasonic