3GPP TSG RAN WG1 Meeting #82bis	R1- 155074

Malmo, Sweden, Oct. 5 – 9, 2015

Agenda Item:		7.2.7.1

Source:		Huawei, HiSilicon

Title:		System-level performance of MUST schemes

Document for:		Discussion/Decision

# Introduction

At the RAN#81 and 82 meeting, the evaluation assumptions regarding to downlink multiuser superposition transmission were agreed [1][2][3]. In this contribution, we present the simulation results for MUST schemes of MUST category 1, 2 and 3 [4] with different UE receiver assumptions according to evaluation assumption agreements of the MUST deployment scenario 1.

# System-Level simulation assumptions

## UE receiver assumptions

The assumptions for the inter-cell interference and inter spatial layer interference receiver are kept the same for different MUST schemes and the baseline scheme. Hard CW-IC receiver could be applied to all MUST category 1, 2 and 3. For MUST category 2 and 3, since the multiple users are superposed with gray mapping, it is also possible to directly apply the R-ML receiver to superposition layer interference suppression instead of CW-IC receiver. In addition, MUST category 2 performances with R-ML receiver and hard CW-IC receivers are both presented.

## Specific system settings for the simulation

More detailed simulation assumptions are provided in the appendix according to [2, 3], and some of the parameters are highlighted in this section. In the following sections, the simulation results of MUST scenario 1 is presented for 2 TX and 2 RX antenna configurations for transmission mode 4. For the CSI feedback, the existing channel state information (CSI) feedback including RI, PMI, and CQI is used where wideband PMI/RI and subband CQI are reported to the eNB by the UE. 3km/h UE velocity is assumed for the simulations. For traffic model, both full buffer and burst buffer are assumed; and for the burst buffer traffic, FTP traffic model I is applied and the file size is 100KB. The channel estimation error is also considered. In the simulation, the simulation time is extended to 30 seconds for the burst buffer traffic.

# High level criteria for UE selection and scheduling

## Preparation and assumptions

UE proportional fair weighting factor calculation is wi=1/Ri', where Ri' is the historic average throughput for user i in the past T subframes up to the current TTI. For the scheduling of the current TTI, it is assumed that there are M users in the queue ready to be scheduled and there are S subbands available. For each user m, the wideband PMI is vm and the corresponding reported CQI on subband s is γm,s .

## Superposition PF metric calculation

For user m and n pairing, and power allocation factor αj, j=1,2,….J for subband s, the metric Em,n,j,s is calculated as Em,n,j,s=wmRm,j,s+wnRn,j,s if the PMI of the two users are the same, where Rm,j,s and Rn,j,s are the payload could be carried in the subband s for user m and n after pairing with power allocation factor αj. And for each subband and for each user m, the single user scheduling metric is also calculated as Em,s=Rm,0,s/Rm', where Rm,0,s is the payload that only user m is scheduled in the subband s.

## Users candidates and power allocation selection

For each subband, the largest PF value over all the PF values (Em,n,j,s and Rm,0,s) is selected as well as the corresponding user candidate(s)  and the power allocation candidate. An example of the selection results for one TTI may be given as the following. (Example: user #2 could be far user in subband #1 and single user in subband #2.)

Fig.1 example of the user pairing status

## User/power reselection

When the optimum pairing is done, user reselection and power reallocation will be performed according the LTE specification limitations, where the power and near-far status of one UE should be the same for all of its allocated resource blocks. The final scheduling results for the example in Fig.1 are shown in Fig.2.

Fig.2. the final scheduling results for the example above

# System level simulation results

In this section, the performances of NOMA of MUST category 1, SOMA of MUST category 2 and REMA of MUST category 3 [5] are provided, where CW-IC receiver is employed to all three MUST categories, R-ML receiver is employed to MUST category 2, and MMSE-IRC is employed to detect the composite constellation for MUST category 3.  Subband scheduling is employed to MUST category 1 and 2, while wideband scheduling to MUST category 3.

## MUST category 1&2 with hard CW-IC receiver

The simulation results are in the following tables

TABLE I simulation results for MUST category 1&2 with hard CW-IC receiver of full buffer

| Cell average gain (%) | Cell edge gain (%) |
| --- | --- |
| 13.4 | 16.6 |

TABLE II simulation results for MUST category 1&2 with hard CW-IC receiver of burst buffer

| RU (%) | Throughput gain (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.2 | 2.3 | 3.5 | 3.7 | 0.0 | 99.7 |
| 87.8 | 11.8 | 15.7 | 16.5 | 6.4 | 99.1 |

Observation 1:   MUST category 1&2 have the same system level performance when hard CW-IC receiver is applied for both of the schemes due to the same L2S modeling of hard CW-IC for MUST category 1&2.

Observation 2: With MUST category 1&2 and hard CW-IC receiver used at the near UE, ~14% cell average and ~17% cell edge throughput gain are achieved for full buffer traffic.

Observation 3: For burst buffer traffic model, the gain varies according to the traffic load. Maximally ~12% average user throughput gain and 16% for cell edge user throughput gain can be achieved for subband scheduling. The gain increases note worthily as the resource utilization (RU) increases.

Observation 4: The SCT to OCT ratio for the simulated cases are very close to 1, which implies that there are almost no dropped packets in the simulations.

## MUST category 2 performance comparison between using R-ML and hard CW-IC receivers

The simulation results are in the following tables.

TABLE III simulation results for MUST category 2 with R-ML receiver of full buffer

| Cell avg. gain (%) | Cell edge gain (%) | Cell avg. loss over Hard CW-IC (%) | Cell edge loss over Hard CW-IC (%) |
| --- | --- | --- | --- |
| 12.7 | 16.1 | 0.6 | 0.4 |

TABLE IV simulation results for MUST category 2 with R-ML receiver of burst buffer

| RU (%) | UPT gain (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.3 | 1.9 | 3.4 | 3.6 | 0.0 | 99.7 |
| 88.1 | 11.1 | 14.9 | 16.4 | 5.7 | 99.2 |

TABLE V The UPT difference between CW-IC and R-ML receiver for MUST category 2 of burst buffer

| RU (%) | UPT loss of R-ML over Hard CW-IC (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.3 | 0.4 | 0.1 | 0.1 | 0.0 | 99.7 |
| 88.1 | 0.7 | 0.8 | 0.1 | 0.7 | 99.2 |

Observation 5: For MUST category 1&2, with the R-ML receiver at the near UE for superposition coding, the performance is slightly degraded compared with hard CW-IC receiver. But overall the R-ML and the hard CW-IC receivers lead to very similar performance for MUST category 2.

## MUST category 3 with wideband scheduling

In this section, SLS results of MUST category 3 with wideband scheduling are presented. The parameters used for system-level simulation of MUST category 3 are summarized in Table VI. For parameters not listed in Table VI, the values reported in Appendix have been used.

Table VI. Simulation parameters

| Parameter | Value |
| --- | --- |
| Traffic model | Full-buffer |
| N. users per cell | 10, 20 |
| Maximum TX rank | 1 |
| Scheduling algorithm | PF, wide-band |
| UE pairing | Same rank, same PMI |
| CSI feedback | Wide-band CQI/PMI/RI |
| PMI/RI feedback period | 40ms |
| RX EVM | 0% |

In Table VII, preliminary SLS results for MUST category 3 [4] are shown. The cell-average throughput gain over baseline is obtained for rank-1 transmission. The considered UE receivers are MMSE-IRC and CW-IC.

Table VII. Cell-average throughput gains of MUST category 3 over baseline.

| N. UEs/cell | MMSE-IRC | CW-IC |
| --- | --- | --- |
| 10 | 8.01% | 11.8% |
| 20 | 8.54% | 14.79% |

Observation 6: For MUST category 3 with the full buffer traffic model and wide-band scheduling, the cell-average throughput gain of MUST category 3 without CW-IC is about 8.5% and about 15% with CW-IC.

## Number of users scheduled

Fig.2. distribution of the number users scheduled in one TTI

The above figure depicts the statistics of the number of users that are scheduled in one TTI, for baseline and  for MUST category 2 schemes, for 88.1% resource utilization case. And it can be concluded for MUST category 1&2 schemes that the TTI with larger number of users scheduled is with larger probability than baseline case. In addition, the probability of number of scheduled users more than 8 is very low (0.8%) for MUST. With the 10MHz bandwidth used in the simulation, the available CCEs (around 40CCEs) can easily accommodate 8 and more than 8 PDCCHs.

Observation 7: Without MUST, the maximum number of scheduled users per TTI is 8. With MUST, the probability of number of scheduled users more than 8 is very low (0.8%) for high traffic load (RU = 88.1%). The PDCCH region can easily accommodate the needed PDCCHs.

# Conclusions

In this contribution, the system evaluation results on MUST schemes MUST category 1, 2 and 3 are provided. The followings are observed:

Observation 1:   MUST category 1&2 have the same system level performance when hard CW-IC receiver is applied for both of the schemes due to the same L2S modeling of hard CW-IC for MUST category 1&2.

Observation 2: With MUST category 1&2 andhard CW-IC receiver used at the near UE, ~14% cell average and ~17% cell edge throughput gain are achieved for full buffer traffic.

Observation 3: For burst buffer traffic model, the gain varies according to the traffic load. Maximally ~12% average user throughput gain and 16% for cell edge user throughput gain can be achieved for subband scheduling. The gain increases note worthily as the resource utilization (RU) increases.

Observation 4: The SCT to OCT ratio for the simulated cases are very close to 1, which implies that there are almost no dropped packets in the simulations.

Observation 5: For MUST category 1&2, with the R-ML receiver at the near UE for superposition coding, the performance is slightly degraded compared with hard CW-IC receiver. But overall the R-ML and the hard CW-IC receivers lead to very similar performance for MUST category 2.

Observation 6: For MUST category 3 with the full buffer traffic model and wide-band scheduling, the cell-average throughput gain of MUST category 3 without CW-IC is about 8.5% and about 15% with CW-IC.

Observation 7: Without MUST, the maximum number of scheduled users per TTI is 8. With MUST, the probability of number of scheduled users more than 8 is very low (0.8%) for high traffic load (RU = 88.1%). The PDCCH region can easily accommodate the needed PDCCHs.

# References

3GPP, Draft_Minutes_report_RAN1#81_v020 – final, Japan, May, 2015.

MediaTek, “Summary of System-level Evaluation Assumptions for MUST,” R1-153626, Japan, Apr. 2015.

3GPP, Chairman's_Notes_RAN1_82_-_final, Beijing, China, August, 2015

Huawei, HiSilicon, MediaTek, NTT DOCOMO, Sony, CHTTL, HTC, CATR, OPPO, ITRI, “TP for classification of MUST schemes”, RP-154999, Beijing, China, Aug. 24 -28, 2015.

Huawei, HiSilicon, “Classification of MUST schemes”, R1-153798, Beijing, China, Aug. 24-28, 2015.

Huawei, HiSilicon, “Link to system modeling for superposition transmission”, R1-151873, Belgrade, Serbia, Apr., 2015.

# Appendix: detailed simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario | MUST Scenario 1 |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0GHz |
| Total BS TX power | 46dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna pattern | 3D |
| Antenna Height: | 25m |
| Antenna gain + connector loss | 17 dBi |
| UE antenna Height | 1.5m |
| Antenna gain of UE | 0 dBi |
| Antenna configuration | 2x2, cross-polarized, 0.5-wave length between antenna groups |
| Transmission mode | TM4 |
| Traffic model | Full buffer (10/Cell) and burst buffer (FTP mode I of 100KB file size) |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| UE receiver | In baseline, MMSE-IRC for inter-cell interference suppression MMSE for inter-spatial layer interference suppression for SU-MIMO In MUST, for all users, MMSE-IRC is assumed for inter-cell interference suppression For MUST near users, CW-IC is assumed for intra spatial layer interference cancellation for both MUST category 1&2, R-ML is also applied for MUST2 for comparison, and MMSE is assumed for inter spatial layer interference suppression for SU-MIMO For other users, MMSE-IRC is assumed for inter spatial layer interference suppression. |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Unified handover margin | 3dB |
| Overhead | 3 symbols for DL CCHs |
| Network synchronization | Synchronized |
| Performance metrics | Average cell throughput, 5% cell edge throughput |
| Scheduling assumptions | No MU-MIMO is considered for 2X2 antenna configuration |
| Feedback assumptions | Non-ideal CRS channel/interference estimation Release 12 CSI feedback schemes Feedback delay 5ms Feedback periodicity 5ms |
| Receiver impairment modeling for demodulation | Non-ideal CRS channel estimation |
| EVM | TX EVM: 8%, UE RX EVM: 4% |