3GPP TSG RAN WG1 Meeting #82bis                                                                         	                    R1-155343

Malmö, Sweden, 5th – 9th October 2015

Source:	Panasonic

Title: 	Multiple subframe code spreading for MTC UEs

Agenda Item:		7.2.1.4

Document for:	Discussion and Decision

Introduction

This contribution proposes to have multiple subframe code spreading on top of the repetition for PUSCH and PUCCH. This contribution also describes the system design of multiple subframe code spreading.

Multiple subframe code spreading

Concept of multiple subframe codes spreading

In order to achieve 15 dB coverage enhancement, repetition over multiple subframes is one of necessary solutions. Repetition degrades significant spectral efficiency for both control and data channels since more physical resources are occupied. Techniques to mitigate the loss by repetition are necessary.

One method to mitigate repetition loss is multiple subframe code spreading on the top of the repetition. Fig. 1 illustrates signal structure of multiple subframe code spreading. Each subframe is repeated over multiple subframes and then, spread by one of orthogonal spreading sequences over multiple subframes.

Fig.1	Multiple subframe code spreading.

Multiple subframe code spreading on top of repetition can mitigate the spectral efficiency loss incurred by repetition. To apply multiple subframe code spreading especially to the uplink data and control channel (such as PUSCH and PUCCH) can provide more resources. Using NSF orthogonal spreading codes, up to NSF UEs/signals can be simultaneously transmitted within the same resource block over the repetition period. This can minimize the spectral efficiency degradation caused by repetition. A smaller granularity than 1 PRB transmission may have a similar effect on the spectral efficiency, but multiple subframe code spreading allows to keep the same PUSCH/PUCCH structure.

Observation 1: Multiple subframe code spreading on top of repetition can be utilized to mitigate the spectral efficiency loss incurred by repetition.

Design of multiple subframe codes spreading

In RAN1#80 [1], RAN1 has agreed that multiple repetition levels in time domain are supported for enhanced coverage transmission. In order to realize to multiplex UEs with multiple repetition levels by multiple subframe code spreading, to have a common spreading factor (as shown in Fig. 2) would be reasonable. For larger repetition, the same spreading code sequence could be repeatedly applied.

Fig.2	The use of common spreading factor.

In RAN1#80 [1], to support cross-subframe channel estimation was agreed for uplink data channel. In order to allow cross-subframe channel estimation, PRB position is the same during at least X subframes. Larger number of X could face the issue on frequency error [2]. Then, relatively smaller number of X, such as X = 4 or 8 would be reasonable. Multiple subframe code spreading is suited to the cross-subframe channel estimation as the receiver operation is almost the same as symbol level combining with cross-subframe channel estimation (Detailed description of receiver operation is shown in Appendix A). Therefore, we propose to support multiple subframe code spreading with a common spreading factor X = 4 or 8 as well as cross-subframe channel estimation with symbol level combining. Although both X = 4 or 8 are valid, based on the evaluation of PUCCH [3], we propose to use X = 4. It should be noted that multiple subframe code spreading can also be workable when soft bit combining although some more effort for specification is required. On the other hand, when symbol level combining is supported, there could be almost no additional effort to support for multiple subframe code spreading.

Observation 2: Multiple subframe code spreading on top of repetition is suited to symbol level combining with cross-subframe channel estimation.

In RAN1#80 [1], to support frequency hopping over the system bandwidth was also agreed for uplink data channel. Multiple subframe code spreading using a common spreading factor is also suited to the frequency hopping over the system bandwidth, as orthogonal property among different spreading codes can be kept within hopping interval.

Observation 3: Multiple subframe code spreading using a common spreading factor is suited to frequency hopping over the system bandwidth.

We had evaluated the link level performance of applying multiple subframe code spreading to PUSCH [4] and showed that multiple subframe code spreading can increase PUSCH capacity without negatively effecting coverage enhancement. Based on above discussion and our previous evaluation, we propose:

Proposal 1: Coverage enhancement for PUSCH and PUCCH supports multiple subframe code spreading using a common spreading factor X = 4 as well as cross-subframe channel estimation.

Indication of spreading code sequence

There are two options for the indication of spreading code sequence:

- Dynamic indication: Spreading code sequence is indication by DCI. Assuming common spreading factor is 4 or 8, 2 or 3 bits are needed in DCI. Although adding new field for spreading code indication causes the increase of DCI overhead, some fields in DCI that are expected not to be used for MTC coverage enhancement, would be replaced by spreading code indication. For example, there is no need to support SU-MIMO for MTC UEs or UE in coverage enhancement, cyclic shift filed in uplink-related DCI format could be reused for code indication for multiple subframe code spreading.

- Semi-static indication: Spreading code sequence can also be indicated via higher layer signalling. When the spectral efficiency is not limiting factor, the network side may not implement the receiver corresponding to multiple subframe code spreading on top of repetition. Even such situation, to configure the spreading sequence like all +1 could receive UE’s transmission. No loss from DCI size overhead is incurred. Other option would be spreading sequence is indicated in an implicit manner such as linked to UE-ID (RNTI).

Proposal 2: Dynamic or semi-static spreading code indication should be specified for multiple subframe code spreading.

Conclusion

In this contribution, we discussed multiple subframe code spreading on top of the repetition and showed some simulation results when using repetition and multiple subframe code spreading on PUSCH. Based on the discussion presented, we summarize our views through the following observations and proposal:

Observation 1: Multiple subframe code spreading on top of repetition can be utilized to mitigate the spectral efficiency loss incurred by repetition.

Observation 2: Multiple subframe code spreading on top of repetition is suited to symbol level combining with cross-subframe channel estimation.

Observation 3: Multiple subframe code spreading using a common spreading factor is suited to frequency hopping over the system bandwidth.

Proposal 1: Coverage enhancement for PUSCH and PUCCH supports multiple subframe code spreading using a common spreading factor X = 4 as well as cross-subframe channel estimation.

Proposal 2: Dynamic or semi-static spreading code indication should be specified for multiple subframe code spreading.

- Reference

- 3GPP RAN1#80, Chairman’s note

- R1-133587,	“Implementation Considerations for Coverage Enhancements for MTC,” Qualcomm Incorporated

- R1-151668, “Discussion and performance evaluation on PUCCH for MTC UEs”, Panasonic

- R1-153969, “Multiple subframe code spreading for MTC UEs” Panasonic

- Appendix A: System model of PUSCH repetition with multiple subframe code spreading

Fig.6 illustrates the system model of PUSCH repetition with multiple subframe code spreading. At the UE side, subframe-level orthogonal code spreading is applied

,																										(1)

where Si(n, k) is the frequency-domain transmitted symbol after multiple subframe code spreading at k-th subcarrier of n-th symbol in i-th (i = 0~NRep-1) subframe for PUSCH repetition. NRep denotes the number of repetitions. D(n, k) is the frequency-domain transmitted symbol at k-th subcarrier of n-th symbol and it is the same over the repetition period (for i = 0~NRep-1). Cu(i), i = 0~NRep-1, is the u-th multiple subframe spreading code sequence. When the multiple subframe spreading factor is NSF, multiple subframe spreading code sequence is given by .

At the receiver of eNodeB, subframe-level despreading and symbol combining are carried out by

,																						(2)

where Rcomb,j(n, k), j = 0~NRep/NSF-1, is the frequency-domain received signal after subframe-level despreading and symbol combining. Ri(n, k) is the frequency-domain received signal at k-th subcarrier of n-th symbol in i-th subframe for PUSCH repetition. After despreading and symbol combining, channel estimation is carried out by using the combined received reference symbol of Rcomb,j(n, k) and then, MMSE-FDE is carried out by using channel estimate as

,																							(3)

where Wj(n,k) is the frequency domain equalization weight. After FDE, NRep/NSF equalized received signals are combined and the frequency-domain received signal after repetition combining is given by

.																									(4)

The above combined symbols are input to the SC-FDMA demodulation.

Fig.6	System model of PUSCH repetition with multiple subframe code spreading.