3GPP TSG RAN WG1 Meeting #86	R1-166096

Gothenburg, Sweden, Aug 22-26, 2016

Agenda Item:	8.1.2.2

Source:	Huawei, HiSilicon

Title:	SLS methodology for MA evaluation in mMTC scenario

Document for:	Discussion and decision

Introduction

In RAN1#84bis meeting [1], it was agreed that

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases;

- SLS is used for comparison of proposals, and verification with traffic/scheduling/multi-cell interference dynamics.

In order to study the system-level performances of the various non-orthogonal multiple access schemes, the methodology should first be agreed in order to facilitate meaningful and fair comparisons. In this contribution, we focus on the system level simulation methodology and in particular on the physical layer abstraction methodology for the uplink. When applied for the mMTC scenario, we described how to compare the main performance metric of connection efficiency and connection density.

Connection efficiency

The connection density/connection efficiency is the main KPI for the mMTC scenario [2]. The evaluation needs to show what connection density could guarantee a system outage rate (packet drop rate, PDR).

It is noted that the connection density would results in a system traffic load. When the connection density is increased, the traffic load would be increased accordingly. Assume the device number is N in the system per km2 and the average inter-packet arrival time is Tpackect (in second), the system traffic loading rate is given by L= N / Tpackect (in packets/km2/s).

Figure.1 Schematic diagram of PDR vs. system traffic loading

The evaluation would produce the PDR vs number of devices N per km2. It could in turn translate into PDR vs system traffic loading as shown in Figure 1. To guarantee a PDR requirement α%, the maximum allowed traffic loading rate L for candidate scheme can be observed, as illustrated in Figure 1. From this figure, one could found that Scheme 2 could support higher traffic loading, and consequently higher connection density / connection efficiency than Scheme 1. In [3], this figure is used to present the evaluation results. The connection density could be further derived by the relationship of traffic loading and the number of devices per km2, as described in [4].

Proposal 1: The packet drop rate vs system traffic loading curve could be employed to observe the connection density capability of different schemes.

Physical layer abstraction method

In this section, a new PHY layer abstraction approach is proposed for SLS evaluation of non-orthogonal multiple access schemes. This method is characterized by both its simplicity and accuracy, with specified lookup tables of parameters. The general approach is to calculate the post-processing-SINR (pp-SNR) based on the perfect interference cancellation (PIC) receiver bound on each resource element (RE) or RE groups, and map to the effective SNRs by using specific effective SNR mapping (ESM) functions with recommended parameters. The estimated BLERs can be obtained by looking up the effective SNRs in a pre-stored SISO AWGN link performance table.

Such PHY abstraction of non-linear receiver for non-orthogonal multiple access is actually similar to that of non-linear detector for MIMO. In general, it contains the following four steps:

- Step 1: Express the transmission by a MIMO system model [5]

y=Hx+w,                                                                      (4)

where y is the received symbol vector, H is the effective channel matrix, taking into account of real channel realizations and the user signatures. x is the transmitted symbol vector, and w is the AWGN noise vector plus the inter-cell interference with covariance matrix Rww.

- Step 2: Apply one receiver bound [6] , which is the PIC bound, to calculate the pp-SINR of the nth user or data layer for each RE or RE groups (assume each RE group contains K REs)

γnPIC=hnHRww-1hn,                                                             (5)

where hn is the effective channel vector of the nth user and Rww is the covariance matrix of AWGN noise plus inter-cell interferences. The calculation of pp-SNRs relies on the modeling of receiver type and inter-cell interference.

- Step 3: Calculate the effective SNR by using the pp-SINRs and the curve fitting parameter β obtained from link level training, such as

γeff=f-11Kk=1Kf(γk;β),                         (6)

where f is the ESM function. Both Shannon capacity ESM function

f(γk;β)=log2(1+βγnPIC),                                                   (7)

and mutual information ESM function

f(γk;β)=MI(βγnPIC),                                                     (8)

are considered, where MI(⋅) is a function of calculating the constellation constrained capacity of       received symbols [7].

- Step 4: Get the BLER value by looking up the SISO AWGN link performance table with the derived effective SINR value.

Training of parameter β

The training method is to compare the BLERs from link-level simulations and the estimated ones with certain amount of β values for a long term average of channels.  For each of the β tested, the BLER mean square errors (MSE) from real link-level simulations (BLERsim,l) and the proposed estimation method (BLERest,l) can be calculated by

MSE=1Ll=1Llog10BLERsim,l-log10BLERest,l2,                             (9)

where L samples of interested noise powers are assumed. The recommended β can be selected based on the MSE calculation. Note that the β values for different Channels (such as Umi, TDL-A, TDL-C, and etc.) and antenna configurations (such as 1T2R and 1T4R) are similar, and the final results are averaged among them. The dimension of β will be (1) Number of active UEs; (2) MCS selection.

Numerical verifications of the proposed physical layer abstraction method are presented in the appendix.

Observation 1: The proposed physical layer abstraction method accuracy has been verified through estimation of the link-level performances of MAs with R-ML receiver and MMSE-SIC receiver.

Proposal 2:  The proposed physical layer abstraction method is adopted as one of the approaches for UL SLS evaluation of different multiple access schemes.

Example with Sparse Coding

In the first step of Section 3, different schemes can be modelled by different signatures (containing the information of codebooks/sequences/interleavers). Taking SCMA as the example, 6 SCMA signatures are provided in a signature matrix below, with spreading factor equal to 4,

SSCMA=101001100110010101011010,                                                               (10)

where every “1” element in each column implies the spreading symbol position over 4 REs. For UL transmission, UEs will be allocated by BS with specific signatures in granted case, or they can randomly select their signatures from the signature pool in grant-free case.

Given the receiver antenna number Nr and the spreading signature Sn for the nth user within total N users, the system model of UL SCMA SIMO scenario is given by

y=n=1Nhnxn+w=Hx+w,                                                         (11)

where y is a MNr×1 vector of received symbol, x is a N×1 vector of transmitted symbols, hn denotes the effect channel of the nth user, taking into account of real channel realizations and the user signatures with hn=hn,1T, ⋯,hn,NrTT,  hn,r= hn,r⊙Sn, and H=[h1, ⋯,hN]. w represents the AWGN noise plus inter-cell interference vector with covariance matrix Rww.

Conclusions

In mMTC evaluation, to compare the connection efficiency for different schemes, we propose

Proposal 1: The packet drop rate vs system traffic loading curve could be employed to observe the connection density capability of different schemes.

For link to system mapping, we have

Observation 1: The proposed physical layer abstraction method accuracy has been verified through estimation of the link-level performances of MAs with R-ML receiver and MMSE-SIC receiver.

Proposal 2:  The proposed physical layer abstraction method is adopted as one of the approaches for UL SLS evaluation of different multiple access schemes.

References

[1] 3GPP Chairman Notes, RAN1#84bis, Busan, Korea, April 2016.

[2] 	R1-164370, “SLS evaluation methodology for grant-free based multiple access,” RAN1#85, Huawei, HiSilicon, Nanjing, China, May 23-27, 2016.

[3] R1-166097, “SLS Results for MA evaluation in mMTC scenario,” RAN1#86, Huawei, HiSilicon, Gothenburg, Sweden, August 2016.

[4] R1-166111, “On traffic model and evaluation method for mMTC evaluation,” RAN1#86, Huawei, HiSilicon, Gothenburg, Sweden, August 2016.

[5] 3GPP 36.866, “Study on network-assisted interference cancellation and suppression (NAICS) for LTE (Release 12)”.

[6] 3GPP 36.859, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE (Release 13)”.

[7] IEEE 802.16m, Evaluation Methodology Document (EMD)

- Appendix: Verification of the PHY Abstraction Method

Table A-1. System parameters for verification

| Parameters | Values |
| --- | --- |
| MA schemes | SCMA, LDS, MC-CDMA |
| Bandwidth | 4 PRB |
| MCS | SCMA: 8 points codebook, CR=1/3, 1/4… LDS: QPSK, CR=1/2 MC-CDMA: QPSK, CR=1/2 |
| Channel Model | ITU-UMi 3km/h |
| Antenna Configuration | 1T2R |
| Channel Estimation | Perfect |
| Num. of UEs | [1 4 6] |
| Receiver type in LLS | SCMA: MPA LDS: MPA MC-CDMA: MMSE-SIC |

SCMA (MPA receiver)

Shannon capacity ESM, CR=1/2, 8 points codebook

1UE,  β=1                                      4UE, β=0.97                                     6UE, β=0.95

MIESM, 8 points codebook

4 UE

Code rate= 1/4,  β=0.94                   Code rate= 1/3,  β=0.94               Code rate= 1/2,  β=0.94

6 UE

Code rate= 1/4,  β=0.92                        Code rate= 1/3,  β=0.92                   Code rate= 1/2,  β=0.92

Note here for MIESM, the β value is not sensitive to the code rate.

LDS (MPA receiver)

Shannon capacity ESM, CR=1/2, QPSK

1UE,  β=0.97                                 4UE,  β=0.95                                       6UE,  β=0.93

MIESM, CR=1/2, QPSK

1UE,  β=0.98                                            4UE,  β=0.98                                             6UE,  β=0.96

MC-CDMA (MMSE-SIC receiver)

Shannon capacity ESM, CR=1/2, QPSK

1UE,  β=0.98                                    4UE,  β=0.95                                        6UE,  β=0.92

MIESM, CR = 1/2, QPSK

1UE,  β=0.97                                4UE,  β=0.94                                  6UE,  β=0.92