3GPP TSG-RAN WG1 #86	                            　　	                                   R1-166573

Göteborg, Sweden, 22 – 26 Aug., 2016

Source:	Mitsubishi Electric

Title:	Maximum number of multiplexed layers for NR MIMO

Agenda Item:	8.1.5 Multi-antenna scheme for new radio interface

Document for: 	Discussion/Decision

Introduction

In RAN1 #85, the following agreement was made [1],

Agreements: - The following DL multi-antenna transmissions to be studied for NR - Closed-loop/(Semi)Open-loop spatial multiplexing - Single/Multi-point transmissions - SU/MU-MIMO - Transmit diversity, - e.g., Single/Multi panel spatial diversity - Combination of above techniques - Other DL multi-antenna transmissions and related techniques are not precluded - This does not imply that used transmission technique needs to be known to the UE

In this contribution, analysis on spectral efficiency for several values of multiplexed layers is conducted and simulation results are shown to demonstrate speed tolerance of different number of multiplexed layers. We recommend that 16 layer-multiplexing is supported for both SU-MIMO and MU-MIMO.

Motivation for considering higher number of multiplexes

In LTE, up to 8 layers are supported for quasi-orthogonal multiplexing. In [2], the target spectral efficiency and peak data rate for downlink are set at 30bps/Hz and 20Gbps, respectively. In this contribution, it will be shown that 16-layer multiplexing, combined with the numerology options discussed in RAN1, can achieve the target values. Number of multiplexing layers will become an important feature especially when serving multiple UEs in crowded scenarios described in [2]. In RAN1#85, there are contributions [3, 4, 5] that have recommended the number of layers for NR MIMO. In [6], several MIMO technologies are proposed for NR. In the following sections, simulation results and analysis will be shown to demonstrate the benefits obtained from 16-layer multiplexing for both DL SU-MIMO and DL MU-MIMO.

Spectral efficiency, operation SNR and tolerance against mobility

In Figure 1 and Figure 2, average sum-rate curves for two configurations with different normalized fading rates are shown. The simulation parameters are shown in Table 1. In the simulation, i.i.d channels with ideal channel estimation are assumed. Channels vary during the interval between the precoding weight matrix determination and the actual DL transmission. It is assumed that perfect knowledge of the channel is available at both transmitter and receiver. In Table 2, target SNR to achieve 30bps/Hz is shown. At high SNR, at the maximum spectral efficiency of 4bps/Hz per layer, at least 8 layers are required to reach the target value. If the number of layers is increased, the maximum spectral efficiency per layer and required SNR to reach the target value can be decreased. It should also be noticed from Figure 2 that 16-layer multiplexing brings more tolerance to the Doppler spread to yield 30bps/Hz. Thus, in NR massive MIMO, up to 16-layer multiplexing should be considered for MU-MIMO to meet the target spectral efficiency of 30bps/Hz.

Table 1 Simulation parameters

| Center Frequency | 30GHz |
| --- | --- |
| Number of antennas at TX | 8,16 |
| Number of UEs | 8 |
| Number of antennas at UE | 1,2 |
| Channel model | i.i.d, Jakes model |
| Precoding method | Block diagonalization, SVD |
| Channel estimation | Ideal |
| Transmission efficiency | 0.7 |
| Duration between start of DL transmission and precoding matrix determination, ∆T | 0.25ms |

Figure 1 : Sum rate vs. SNR for 8 stream multiplexing, 8UEs, 1 stream per UE

Figure 2 : Sum rate vs. SNR for 16 stream multiplexing, 8 UEs and 2 streams per UE

Table 2 : Required SNR with 8 or 16 received antennas

| Number of multiplexed layers | Average efficiency per layer for 30bps/Hz [bps/Hz] | Target average SNR for 30bps/Hz [dB] |
| --- | --- | --- |
| 8 | 3.75 | 21 |
| 16 | 1.875 | 12 |

Observation 1: The target spectral efficiency can be reached at lower SNR than 8-layer multiplexing when 16-layer multiplexing is used for MU-MIMO

Observation 2: Additional multiplexed layers provide robustness against mobility

Peak spectral efficiency and peak data rate calculation with numerology discussed in RAN1#85 for downlink for above 6GHz

Spectral efficiency

In this section, we use the formula used in [5] to calculate the peak spectral efficiency,

PeakSE=NumLayers×log2ModOrder×NumSCsPerSymbol×NumSymbolsPerTTI×1-OverheadFracTTIDurationMs×CCBWMHz×1000

Definitions of the variables in the equation above and numerology options considered in this contribution are summarized in Table 3 and Table 4, respectively. The analysis is limited to above 6GHz, downlink only.

In Table 5, the peak spectral efficiency achieved for 60kHz and 75kHz spacing is shown. We note that the numerology options in this analysis, such as 14 symbols per TTI, are chosen as examples. The aim of the analysis here is not to mandate the numerology options in the table or preclude other candidate options. The analysis here can easily be extended to other options such as the options discussed in our companion contribution [7].

In this calculation, OverheadFrac is set at 0.30. The parameters which achieve the target spectral efficiency are highlighted by yellow in Table 5.

Table 3 Parameter descriptions [5]

| NumLayers | Number of multiplexed layers |
| --- | --- |
| SCSpacingKhz | Carrier spacing in kHz |
| ModOrder | Modulation order |
| NumSCsPerSymbol | Number of carriers per OFDM symbol |
| NumSymbolsPerTTI | Number of OFDM symbols per TTI |
| OverheadFrac | Overhead fraction |
| TTIDuationMs | TTI duration in ms |
| CCBWMHz | Component carrier BW in MHz |

Table 4 Parameters for two numerologies

| # | NumSCsPerSymbol | SCSpacingKhz | NumSymbolsPerTTI | TTIDurationMs | CCBWMhz |
| --- | --- | --- | --- | --- | --- |
| 1 | 1200 | 60 | 14 | 0.25 | 80 |
| 2 | 1200 | 75 | 14 | 0.2 | 100 |

From Table 5, it is clear that 30bps/Hz can be reached with 16-QAM with 16-layer multiplexing while the same target can be reached with 256QAM with 8-layer multiplexing. From the result, it is clear that the target value can be reached with 16-layer multiplexing at lower modulation order, allowing operation at lower SNR.

Table 5 Peak spectral efficiency in bps/Hz when carrier spacing is an integer multiple of 15kHz (i.e., 15kHz, 60kHz, 75kHz)

|  |  | ModOrder |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 16 | 64 | 256 |
| NumLayers | 2 | 2.35 | 4.70 | 7.09 | 9.41 |
|  | 4 | 4.70 | 9.41 | 14.11 | 18.82 |
|  | 8 | 9.41 | 18.81 | 28.22 | 37.63 |
|  | 16 | 18.82 | 37.63 | 56.48 | 75.26 |

Observation 3: 16-layer multiplexing achieves the target peak spectral efficiency, 30bps/Hz, with the modulation order of 16

Peak data rate

In the following, peak data rate is computed for two values of the system bandwidth. For 75kHz carrier spacing, BW=1GHz and 500MHz are considered. For 60kHz, BW=960MHz and 480MHz are considered. The bandwidth 500MHz and 480MHz are chosen for the following reasons. While multiple bands with bandwidth of several GHz are considered for further studies in WRC19 from 24GHz – 40GHz and 66GHz – 86 GHz [2], available bandwidth per band per operator may be reduced to several hundred MHz if the same band is shared by several operators. Thus, 500MHz or 480Mhz can be considered as a practical value for bandwidth for NR for above 6GHz bands. The combinations of number of layers and modulation order that can yield the value over the target peak data rate are shown in red characters.

Table 6 Peak data rate in Gbps when carrier spacing is 75kHz, BW=1GHz

|  |  | ModOrder |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 16 | 64 | 256 |
| NumLayers | 2 | 2.35 | 4.70 | 7.09 | 9.41 |
|  | 4 | 4.70 | 9.41 | 14.11 | 18.82 |
|  | 8 | 9.41 | 18.81 | 28.22 | 37.63 |
|  | 16 | 18.82 | 37.63 | 56.48 | 75.26 |

Table 7 Peak data rate in Gbps when carrier spacing is 75kHz, BW=500MHz

|  |  | ModOrder |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 16 | 64 | 256 |
| NumLayers | 2 | 1.18 | 2.35 | 3.55 | 4.71 |
|  | 4 | 2.35 | 4.71 | 7.06 | 9.41 |
|  | 8 | 4.71 | 9.41 | 14.11 | 18.82 |
|  | 16 | 9.41 | 18.82 | 28.24 | 37.63 |

Table 8 Peak data rate in Gbps when carrier spacing is 60kHz, BW=960MHz

|  |  | ModOrder |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 16 | 64 | 256 |
| NumLayers | 2 | 2.26 | 4.51 | 6.80 | 9.03 |
|  | 4 | 4.51 | 9.03 | 13.55 | 18.07 |
|  | 8 | 9.03 | 18.06 | 27.09 | 36.12 |
|  | 16 | 18.07 | 36.12 | 54.22 | 72.25 |

Table 9 Peak data rate in Gbps when carrier spacing is 60kHz, BW=480MHz

|  |  | ModOrder |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 16 | 64 | 256 |
| NumLayers | 2 | 1.13 | 2.26 | 3.40 | 4.52 |
|  | 4 | 2.26 | 4.52 | 6.77 | 9.03 |
|  | 8 | 4.52 | 9.03 | 13.54 | 18.06 |
|  | 16 | 9.03 | 18.06 | 27.11 | 36.12 |

Observation 4: 16-layer multiplexing achieves the target peak data rate with the modulation order of 16 when near -1GHz bandwidth is considered.

Observation 5: With near-500MHz bandwidth, the target data rate can be reached by using 16-layer multiplexing.

Conclusion

In this contribution, we made the following observations:

Observation 1: The target spectral efficiency can be reached at lower SNR than 8-layer multiplexing when 16-layer multiplexing is used for MU-MIMO

Observation 2: Additional multiplexed layers provide robustness against mobility

Observation 3: 16-layer multiplexing achieves the target peak spectral efficiency, 30bps/Hz, with the modulation order of 16

Observation 4: 16-layer multiplexing achieves the target peak data rate 20Gbps with the modulation order of 16 when near -1GHz bandwidth is considered.

Observation 5: With near-500MHz system bandwidth, the target data rate can be reached by using 16-layer multiplexing.

Based on the above observations, we make the following proposal.

Proposal: Support 16-layer multiplexing for both DL SU-MIMO and DL MU-MIMO for NR-MIMO to meet the target spectral efficiency and peak data rate for above 6 GHz.

- References

- [1]	Chairman’s note, 3GPP RAN1 #85, Nanjing, China, May, 2016

- [2]	TR 38.913, V0.3.0, “Study on Scenarios and Requirements for Next Generation Access Technologies”, Mar. 2016.

- [3]	R1-164373, “Evaluation Methodology for Massive MIMO in New Radio”, Huawei, , 3GPP RAN1 #85, Nanjing, China, May 2016.

- [4]	R1-164015, “DL MIMO framework for NR”, Samsung, 3GPP RAN1 #85, Nanjing, China, May, 2016.

- [5]	R1-163985, “Discussion on achieving data rate and spectral efficiency KPIs for NR”, Samsung, 3GPP RAN1 #85, Nanjing, China, May, 2016.

- [6]	R1-164027, “Design of massive MIMO for new radio”, Mitsubishi Electric, 3GPP RAN #85, Nanjing, China, May 2016.

- [7]	R1-166225, “Considerations on numerology for support of flexible guard lengths,” Mitsubishi Electric, RAN1#86, Goteborg, Sweden, August 2016.