3GPP TSG RAN WG1 meeting #86bis	R1-1608922

Lisbon, Portugal, 10th - 14th October, 2016

Agenda Item:	8.1.3.1

Source:	Spreadtrum Communications

Title:	Performance analysis of channel coding schemes for NR

Document for:	Discussion and decision

Introduction

In RAN1 #86, different channel coding schemes are compared in terms of the performance, complexity, and flexibility. It is agreed that the eMBB data channel coding scheme will be chosen at RAN1#86bis and companies are encouraged to continue analysis and comparison in order to inform the final decision at RAN1#86bis. In this contribution, we evaluate the performance of the main candidates in the eMBB usage scenario and URLLC/mMTC scenario. Simulation parameters can be found in Appendix.

Performance evaluation in the eMBB scenario

The main simulation parameters for NR are shown in Table 1 and 2.

For Polar code, the information bit position set is selected by Gaussian approximation method mentioned in [1]. Noted that, before the information bits are encoded into Polar codeword, CRC check bits are added. Correspondently, the decoding algorithm of Polar code is CRC aided SCL algorithm with list size 32. For both QPSK and 64QAM in eMBB scenario, 8 bit CRC is added when information bit length is 100 and 400, while 24 bit CRC is added for other information bit length. Random interleaver is used to break the burst error.

For LDPC code, we adopt the LDPC code construction matrix and the rate matching method in [2]. The layered min-sum decoding algorithm with 15 iterations is used.

For Turbo code, we evaluate LTE Turbo codes with its rate matching method. No CRC is added and Max-log-MAP decoding algorithm with 8 iterations is used.

Fig.1 gives the BLER performance of the three channel coding schemes in eMBB scenario in QPSK. Here, the figures take 1/5, 1/2 and 8/9 to represent low medium and high code rate respectively.

Fig.1. Channel coding performance for eMBB in QPSK

We can see from Fig.1 that, with low coding rate at 1/5, LDPC equals to Turbo with short and medium code length, and LDPC gains 0.2-0.3dB at high code length. While Polar code is better than Turbo and LDPC at low SNR region, but with slow BLER decrease speed than LDPC and Turbo.

For medium coding rate at 1/2, Polar is better than turbo and LDPC obviously.

For high code rate at 8/9, LDPC is better than Polar and Turbo code at large code length, and worse than Polar and Turbo for short code length. While Turbo code has the error floor phenomenon which is not suitable for eMBB scenario.

As for 64QAM, the comparison among Polar code, LDPC code and Turbo code are shown in Fig.2.

Fig.2. Channel coding performance for eMBB in 64QAM

We can see from Fig.2 that, with low code rate at 1/5, LDPC is better than Turbo and Polar code, Polar code only better than LDPC at very high BLER region and the drop speed of LDPC is also larger than Polar code.

For medium rate at 1/2, Turbo code becomes the best code scheme for large code length, and Polar can  obtain 1.5dB than LDPC for short code length and 0.5dB for large code length.

For high rate at 8/9, LDPC is the better than Turbo and Polar code again, and the curve drop speed is also the fastest among the three code schemes.

Observation 1: In eMBB scenario with QPSK and 64QAM, Turbo is not suitable to be the channel coding scheme for the error floor phenomenon, Polar code can obtain SNR gain at short code lengths., while LDPC can be the proper choice for the best performance with high code rate and large code length.

Performance evaluation in the URLLC/mMTC scenario

In URLLC/mMTC scenario, the encoding and decoding algorithm of Polar code and Turbo code are the same as that in eMBB scenario. While for Polar code, 8 bit CRC is added for information bit length 20 and 40, 24 bit CRC is added for information bit length 200 and 600, and 16 bit CRC is added for information bit length 1000.

For TBCC code, 16 bit CRC is added and the list Viterbi decoding algorithm with list size 8 is used.

Fig.3 gives the BLER performance of the three channel coding schemes in URLLC/mMTC scenario in QPSK.

Fig.3. Channel coding performance for URLLC/mMTC in QPSK

We can see from Fig.3 that, with low code rate at 1/12, Polar code is obviously better than turbo and TBCC code whether for short code length or large code length. The SNR gain can be 1-2dB for Turbo code and 4-5dB for TBCC code.

For medium and high code rate at 1/6 and 1/3, the same conclusion can be drawn. While as the code rate becomes larger, the SNR gain between Polar and the other two code are turning smaller.

As for 64QAM, the comparisons among Polar code, LDPC code and Turbo code are shown in Fig.4.

Fig.4. Channel coding performance for URLLC/mMTC in 16QAM

We can see from Fig.4 that, with the three code rate of 1/12, 1/6 and 1/3 in 16QAM, the BLER curves have the trends as that for QPSK. Polar is still the best among the three coding schemes. And the SNR gain between Polar and the other two code are getting smaller as the code rate becomes larger.

Observation 2: In URLLC/mMTC scenario, Polar code has the best BLER performance, and Polar code can be the proper channel coding scheme at least for low code length.

Conclusions

In this contribution, we present initial simulation results of channel coding schemes for NR usage scenario. Based on the above discussion, we have the following observations:

Observation 1: In eMBB scenario with QPSK and 64QAM, Turbo is not suitable to be the channel coding scheme for the error floor phenomenon, Polar code can obtain SNR gain at short code lengths, while LDPC can be the proper choice for the best performance with high code rate and large code length.

Observation 2: In URLLC/mMTC scenario, Polar code has the best BLER performance, and Polar code can be the proper channel coding scheme at least for low code length.

References

R1-164039, “Polar codes - encoding and decoding”, Huawei, HiSilicon, 3GPP TSG RAN WG1 #85, Nanjing, China, 23th – 27th May 2016.

R1-164008, "Preliminary evaluation results on new channel coding scheme for NR", Samsung,  3GPP TSG RAN WG1 #85, Nanjing, China, 23th – 27th May 2016.

Appendix

Table 1: Simulation assumption for the eMBB scenario

| Channel | AWGN |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK, 64QAM |  |  |
| Coding Scheme | Turbo | LDPC | Polar |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |  |
| Info. block length (bits w/o CRC) | 100, 400, 1000, 2000, 4000 |  |  |
| Decoding algorithm | Max-log-MAP | Layered min-sum | List-32 |
| Iteration number | 8 | 15 | / |
| CRC | no | no | 8/24 |

Table 2: Simulation assumption for the URLLC/mMTC scenario

| Channel | AWGN |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK, 16QAM |  |  |
| Coding Scheme | Turbo | TBCC | Polar |
| Code rate | 1/12,1/6,1/3 |  |  |
| Info. block length (bits w/o CRC) | 20, 40,200,600,1000 |  |  |
| Decoding algorithm | Max-log-MAP | List Viterbi list-8 | List-32 |
| Iteration number | 8 | / | / |
| CRC | no | 16 | 8/16/24 |