3GPP TSG RAN WG1 #86b                                       R1-1609104

# Lisbon, Portugal 10th - 14th October 2016

Agenda item:	8.1.4.2

Source:	Samsung

Title:	Evaluation assumptions for NR coordinated interference cancellation based on advanced receivers

Document for:	Discussion and decision

Introduction

In RAN1#86, the following agreement on NR network coordination aspects was made [1].

Agreements: - Study to support various interference management schemes: - Interference management over different time scales: - Semi-static/preconfigured interference management - Dynamic interference management - Interference management where signals/channels from/to UE(s) is - Transmitted from/to multiple TRPs - Transmitted from/to single TRP - The above study should consider: - Forward compatibility, e.g., for future introduction of additional interference management schemes (if any) - Low and high NR frequencies - Take into account backhaul/fronthaul latency constraints - Both TDD and FDD - Both data and control channels - Interference measurement/reporting - Taking into account interference management for advanced receivers - Taking into account various scenarios - Taking into account beam management, different antenna structures, etc.

This contribution provides evaluation assumptions for inter-cell interference management based on advanced receivers in a coordinated manner related to the highlighted part of the agreement.

Proposals on link-level evaluation assumptions for coordinated interference cancellation

Our companion contribution [2] discusses the necessity of studying coordinated interference cancellation (interference management based on advanced receivers with various modulation mapping and codeword-to-layer (CW2L) mapping), which aims to meet the cell edge requirement of New Radio (NR). In order to have common understanding of link-level simulation (LLS) evaluations among companies, LLS evaluation assumptions used in Release-12 NAICS may be a good starting point, since many companies were already familiar with them. However, since some parameters, such as channel model, and antenna configuration, pattern, and orientation of BS and UE, may affect the performance verification, we propose to update at least these parameters based on email discussion [86-20] for NR MIMO calibration.

Proposal 1: Link-level simulation assumptions of NAICS (TR 36.866) can be a starting point for the evaluation of advanced receivers in NR network coordination with the following updates:

- Carrier frequency

- System bandwidth

- BS antenna configuration

- BS antenna pattern

- BS array orientation

- UE antenna configuration

- UE antenna pattern

- UE array orientation

- Channel model

- Coded modulation

- Etc.

Proposal 2: For further refinement on proposal 1, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted.

A detailed example for the proposed link-level simulation assumptions for coordinated interference cancellation is provided in Appendix 1.

Proposals on system-level evaluation assumptions for coordinated interference cancellation

We propose to use the same deployment scenarios described in Appendix 2 and system-level simulation assumptions discussed in our companion contribution [3] except for the parameter of UE receiver, for which further advanced receivers other than NAICS receivers are not precluded, since the performances are evaluated for coordinated interference cancellation based on advanced receivers.

Proposal 3: For NR coordinated interference cancellation schemes, it is recommended that the following three deployment scenarios be evaluated in NR study item:

- Indoor hotspot, dense urban, urban micro

Proposal 4: Simulation assumptions of FeCoMP (TR 36.741) can be a starting point for the evaluation of NR coordinated interference cancellation schemes with following refinement

- Carrier frequency

- Subcarrier spacing

- System bandwidth

- Coordination cluster size

- Channel model

- TRP and UE antenna configuration

- UE mobility (UE movement, UE rotation, …)

- Baseline scheme

- UE receiver

- Etc.

Proposal 5: For further refinement on proposal 4, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted

A detailed example for the proposed system-level simulation assumptions for coordinated interference cancellation is provided in Appendix 3.

Conclusion

This contribution discusses link-level and system-level simulation assumptions for coordinated interference cancellation and provides the following proposals.

Proposal 1: Link-level simulation assumptions of NAICS (TR 36.866) can be a starting point for the evaluation of advanced receivers in NR network coordination with the following updates:

- Carrier frequency

- System bandwidth

- BS antenna configuration

- BS antenna pattern

- BS array orientation

- UE antenna configuration

- UE antenna pattern

- UE array orientation

- Channel model

- Coded modulation

- Etc.

Proposal 2: For further refinement on proposal 1, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted.

Proposal 3: For NR coordinated interference cancellation schemes, it is recommended that the following three deployment scenarios be evaluated in NR study item:

- Indoor hotspot, dense urban, urban micro

Proposal 4: Simulation assumptions of FeCoMP (TR 36.741) can be a starting point for the evaluation of NR coordinated interference cancellation schemes with following refinement

- Carrier frequency

- Subcarrier spacing

- System bandwidth

- Coordination cluster size

- Channel model

- TRP and UE antenna configuration

- UE mobility (UE movement, UE rotation, …)

- Baseline scheme

- UE receiver

- Etc.

Proposal 5: For further refinement on proposal 4, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted

References

- 3GPP, RAN#186, Chairman’s Notes

- R1-1609103, Samsung, Coordinated interference cancellation for NR

- R1-1609102, Samsung, Evaluation assumptions for NR network coordination

- Appendix 1

| Parameters | Values |
| --- | --- |
| Carrier frequency | 4GHz, 30GHz |
| Waveform | CP-OFDM |
| Channel coding | LTE Turbo code (Baseline) |
| System bandwidth | Refer to [86-20] |
| Total allocated bandwidth for transmission | Companies need to report this value |
| BS antenna configuration | Refer to [86-20] |
| BS antenna pattern |  |
| BS array orientation |  |
| UE antenna configuration |  |
| UE antenna pattern |  |
| UE array orientation |  |
| Number of interfering cells | 1 or 2 |
| SNR/INR level | Companies need to report this value |
| MIMO mode | SU-MIMO up to rank 2 |
| MIMO correlation | Companies need to report this value |
| Modulation | 4-QAM / 16-QAM / 64-QAM |
| Coded modulation | Companies need to report this |
| MCS | Companies need to report this value |
| Control overhead | Realistic |
| Channel estimation | Realistic |
| Channel Model | Refer to [86-20] |

- Appendix 2

| Scenarios | Carrier Freq. [GHz] | Layout | ISD | UE speed | Reference | NW coordination efficiency |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot | ~ {4, 30, 70} | 1 layer | 20m | 3km/h | FeCoMP | High |
| Dense urban | ~ {4 + 30} | 2 layers | 200m for Macro | 3km/h (80%), 30km/h (20%) | CoMP, FeCoMP | High |
| Rural | ~ {0.7, 2, 4} | 1 layer | {1732, 5000}m | 3km/h (50%), 120km/h (50%) | - | Low |
| Urban macro | ~ {2, 4, 30} | 1 layer | 500m | 3km/h (80%), 30km/h (20%) | CoMP, FeCoMP | High |
| High speed (trains) | ~ {4, 30, 70} | Macro (+ relay) | 1732m | Up to 500km/h | - | Med |
| Extreme long range | ~ {0.7, <3} | 1 layer | Isolated | Up to 160km/h | - | Low |
| Massive connection | {0.7, 2.1(opt.)} | 1 layer | {500, 1732}m | 3km/h (80 or 100%), 100km/h (20 or 0%) | - | Med |
| Highway | < 6 (~ 6) | Macro (+ RSU) | {1732, 500(opt)}m for Macro {50, 100}m for RSU | 100~300km/h | - | Med |
| Urban grid | < 6 (~ 6) | Macro (+ RSU) | 500m for Macro {50, 100}m for RSU | 15~120km/h [TBD] Ped./bicycle | - | Med |
| Air to Ground | < [4] | 1 (+ relay) | - | [1000]km/h | - | Low |
| Light aircraft | < [4] | 1 (+ relay) | TBD | Up to [370]km/h | - | Med |
| Satellite | 2~50 | - | - | - | - | Low |

Table 1. Summary on NR deployment scenarios

- Appendix 3

| Parameters | Indoor Hotspot | Urban Macro | Dense urban |
| --- | --- | --- | --- |
| Layout | Refer to TR 38.802 |  |  |
| ISD |  |  |  |
| Minimum distances |  |  |  |
| Carrier frequency | {4, 30}GHz | {4, 30}GHz | Macro layer: 4GHz Small cell layer: 4GHz (co-channel) 30GHz (non co-channel) |
| Coordination cluster size for ideal backhaul | All sites | 3 macro sites, 7 macro sites is optional, other coordination cluster size are not precluded | 3 macro sites with 3*3*N small cell TRPs 1 macro sites with 1*3*N small cell TRPs 7 macro sites with 7*3*N small cell TRPs is optional, other coordination cluster size are not precluded |
| Subcarrier spacing | 15kHz for CF=4GHz, 60kHz for CF=30GHz |  |  |
| System Bandwidth | 20MHz for CF=4GHz, 40MHz for CF=30GHz |  |  |
| Channel model | Refer to TR 38.802 |  |  |
| TRP antenna configurations (M,N,P,Mg,Ng), (dH,dV,dH,g,dV,g) | Baseline refer to [86-20] Other configurations in TR 38.802 are not precluded |  |  |
| TRP Tx power | Refer to TR 38.802 |  |  |
| TRP antenna pattern |  |  |  |
| TRP antenna height |  |  |  |
| Small cell TRP dropping |  |  |  |
| UE antenna height/UE dropping |  |  |  |
| Association of UE to TRP | Association method (including CRE) should be reported |  |  |
| Maximum CoMP measurement set size | Baseline 3TPs. If a different value is used, it should be indicated. |  |  |
| UE antenna configuration (M,N,P,Mg,Ng), (dH,dV,dH,g,dV,g) | Refer to [86-20] |  |  |
| UE antenna pattern | Refer to TR 38.802 |  |  |
| UE array orientation | Refer to [86-20] |  |  |
| UE receiver noise figure | Below 6GHz: 9 dB Above 6GHz: 10dB |  |  |
| Traffic model | FTP traffic model 1 as a baseline, S = 0.1Mbytes (optional) or 0.5Mbytes |  |  |
| Traffic load (Resource utilization) | <5%, 20%, 40%, 70%, Optional 80% (S=0.1Mbytes) |  |  |
| UE receiver | MMSE-IRC as the baseline receiver (other advanced NAICS receivers and further advanced receivers are not precluded) |  |  |
| Feedback assumption | Realistic |  |  |
| Transmission scheme |  |  |  |
| RS modelling |  |  |  |
| Channel estimation |  |  |  |
| Overhead modelling |  |  |  |
| Handover margin | 3dB |  |  |
| Backhaul link delay | 0ms, 2ms (optional), 5ms, 50ms |  |  |
| Baseline scheme | Provided by each company |  |  |