3GPP TSG RAN WG1 Meeting #86bis                       R1-1609223

Lisbon, Portugal 10th - 14th October 2016

Agenda Item:	8.1.1.2

Source: 	LG Electronics

Title: 	Further evaluation results of NCMA in UL LLS

Document for:	Discussion

Introduction

In general, for the conventional DL/UL multiple access (MA) scheme, three design criteria have been considered in terms of spectral efficiency, energy efficiency and system complexity. In NR (New RAT) design, three use cases are considered as eMBB, mMTC, and URLLC with several KPIs for IMT 2020 RIT submission [1]. Specially, some KPIs such as massive connectivity, spectral efficiency and latency, might be enormously challenging. As the first step toward NR design, the non-orthogonal multiple access (NoMA) schemes for NR are discussed and further UL LLS evaluation results for NoMA schemes are presented.

Followings are the observations, agreements and conclusion on the multiple access of NR drawn by the previous RAN1 meetings:

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- NR should target to support UL non-orthogonal multiple access, in addition to the orthogonal approach, targeting at least for mMTC

- NR should target to support UL “autonomous/grant-free/contention based” at least for mMTC

Conclusion:

- In RAN1 discussion for MA, grant-free is used to represent “autonomous/grant-free/contention based”

Non-orthogonal Coded Multiple Access (NCMA)

The NoMA schemes have been proposed in order to increase connectivity or the system throughput and the proposed schemes are based on the non-orthogonal spreading code, different spatial resource or different power. NoMA schemes are expected to provide improved connectivity or increased system throughput compared to the conventional OMA schemes under specific environment with system optimizations (e.g. power allocation or user scheduling). However, NoMA schemes have some drawbacks, such as scheduling complexity, encoding/decoding complexity, loss of BLER, and limited environments.

In the NoMA schemes, multiuser interference (MUI) is inherently induced. We suggest spreading based non-orthogonal coded multiple access (NCMA) scheme in [2]. This scheme approaches minimization of MUI theoretically based on the spreading coded. Here, we define the spreading code as ‘Grassmannian Sequence’ [3][4] and the sequence and quantized version of the sequence can be obtained as follows.

- Grassmannian Sequence

  - Each complex spreading sequence of this sequence set is generated by Grassmannian line packing problem. Let the Grassmannian sequence set defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor. Then, the sequence design problem can be posed in terms of maximizing the minimum chordal distance between sequence pairs: minGmax1≤k<j≤K1-sk*∙sj2, where sk* is the conjugate sequence of sk.

  - Spreading sequence set: G=s(1)…s(K)

- M-QAM quantized Grassmannian Sequence

  - Each complex coefficient of this sequence (which is generated by the Grassmannian sequence) is quantized by M-QAM constellations. Then, the M-QAM quantized Grassmannian sequence set is defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor.

  - Complex coefficient: si(j)=c, i=1≤i≤N, i=1≤j≤K, c∈the set of M-QAM constellations

  - Spreading sequence set: G=s(1)…s(K)

Note that the spreading code design mentioned in [2] is one of the NCMA schemes.

Evaluation Results

  - Simulation Parameters

- In this chapter, we provide LLS evaluation and the simulation parameters are represented as shown below:

Table I. UL Link Level Simulation Assumptions

| Waveform | OFDM |
| --- | --- |
| Multiple Access | OFDMA w/ Localized Allocation NCMA w/ Non-orthogonal Code via 64QAM-quantized Grassmannian Sequence [Appendix] |
| Carrier Frequency | 2GHz |
| Subcarrier Spacing | 15kHz |
| System Bandwidth / FFT Size | 10MHz / 1024 |
| Transmission Bandwidth | 4PRBs |
| Antenna Configuration | 1T2R |
| Target Spectral Efficiency | Per UE spectral efficiency: 0.05, 0.2 |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiple UEs | 4, 6, 8 |
| Channel Coding | Turbo Coding (TC); maximum mother code rate = 1/4 |
| Modulation | QPSK, 16QAM |
| MA Signature Selection | Fixed Allocation |
| Interference Cancellation | Successive Interference Cancellation (SIC) ; Codeword level IC is assumed, but CRC check is not assumed. |
| Channel Model | TDL-C (300ns), mobility-3km/h |
| Channel Estimation | Ideal (Perfect) / Realistic (Least Square) – LTE UL DMRS w/ Zadoff-Chu |

  - Block Error Rate

In this section, the link level simulation results of NCMA via 64QAM-quantized Grassmannian Sequence are compared with the baseline OFDMA scheme in terms of block error rate (BLER), SNR gain and MCL with multiuser connectivity.

Figures 1 and 2 represent the BLER comparisons for SE per UE = 0.2 and 0.05 [bps/Hz], respectively.

(a) 4 UEs w/ Ideal Channel Estimation                 (b) 4 UEs w/ Realistic Channel Estimation

(c) 6 UEs w/ Ideal Channel Estimation                 (d) 6 UEs w/ Realistic Channel Estimation

(e) 8 UEs w/ Ideal Channel Estimation                 (f) 8 UEs w/ Realistic Channel Estimation

Figure 1. BLER comparisons for SE per UE = 0.2 [bps/Hz]

(a) 4 UEs w/ Ideal Channel Estimation                 (b) 4 UEs w/ Realistic Channel Estimation

(c) 6 UEs w/ Ideal Channel Estimation                 (d) 6 UEs w/ Realistic Channel Estimation

(e) 8 UEs w/ Ideal Channel Estimation                 (f) 8 UEs w/ Realistic Channel Estimation

Figure 2. BLER comparisons for SE per UE = 0.05 [bps/Hz]

In Table II, the SNR gains of NCMA over OFDMA under different scenarios are summarized.

Table II. NCMA SNR gains in dB over OFDMA @ BLER=0.1

| SE per UE (bps/Hz) | Number of UEs | Baseline | Ideal CE | Realistic CE |
| --- | --- | --- | --- | --- |
| 0.2 | 4 | OFDMA, QPSK, code rate=0.5 | 1.7 | 0.3 |
|  | 6 | OFDMA, 16QAM, code rate=0.375 | 2.75 | 1.1 |
|  | 8 | OFDMA, 16QAM, code rate=0.5 | 3.6 | 1.7 |
| 0.05 | 4 | OFDMA, QPSK, code rate=0.125 | 0.65 | -1.4 |
|  | 6 | OFDMA, QPSK, code rate=0.1875 | 0.65 | -1.7 |
|  | 8 | OFDMA, QPSK, code rate=0.25 | 0.6 | -1.9 |

Note that the performance of OFDMA is represented as the best performance of OFDMA among multiple MCS levels for each case. NCMA shows gain over OFDMA under SE with 0.2[bps/Hz] but OFDMA outperforms NCMA under low SE(0.05 bps/Hz) environment under realistic channel estimation.

In another approach, above results show that the performance gap of OFDMA between the cases of realistic channel estimation and ideal channel estimation is up to 1dB. Otherwise, the performance gap of NCMA between the cases is up to 3.5dB. So, the results represent that the performance of NoMA schemes is closely related with the channel estimation performance. In other words, considering the realistic channel estimation, performance of MUD for NoMA scheme can be degraded. Thus, to design the NoMA schemes in NR, the realistic channel estimation should be considered.

Observation 1: MUD performance of NoMA scheme depends on the channel estimation performance.

Observation 2: MUD performance of NoMA is degraded under real channel estimation.

Proposal 1: The realistic channel estimation should be considered for the NoMA design in NR.

Table III. MCL comparisons for SE per UE =0.2 @ BLER=0.1 with Ideal Channel Estimation

| Scheme |  | OFDMA |  |  | NCMA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Overloading (%) |  | 400 | 600 | 800 | 400 | 600 | 800 |
| SE(bps/Hz) |  | 0.8 | 1.2 | 1.6 | 0.8 | 1.2 | 1.6 |
| Tx | (1) Tx Power (dBm) | 23 |  |  |  |  |  |
| Rx | (2) Thermal noise density (dBm/Hz) | -174 |  |  |  |  |  |
|  | (3) eNB receiver noise figure (dB) | 3 |  |  |  |  |  |
|  | (4) Interference margin (dB) | 0 |  |  |  |  |  |
|  | (5) Occupied channel bandwidth (Hz) | 180k | 120k | 90k | 720k | 720k | 720k |
|  | (6) Effective noise power = (2)+(3)+(4)+10 log ((5)) (dBm) | -188.447 | -120.208 | -121.458 | -112.427 | -112.427 | -112.427 |
|  | (7) Required SINR (dB) | 4.3 | 7.3 | 9.6 | -3.4 | -3.23 | -3 |
|  | (8) Receiver sensitity =(6)+(7) (dBm) | -114.147 | -112.908 | -111.858 | -115.827 | -115.657 | -115.427 |
|  | (9) Receiver processing gain | 0 |  |  |  |  |  |
| MCL | (10) MCL = (1) - (8) + (9) (dB) | 137.1473 | 135.9082 | 134.8576 | 138.8267 | 138.6567 | 138.4267 |

Table IV. MCL comparisons for SE per UE =0.05 @ BLER=0.1 with Ideal Channel Estimation

| Scheme |  | OFDMA |  |  | NCMA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Overloading (%) |  | 400 | 600 | 800 | 400 | 600 | 800 |
| SE(bps/Hz) |  | 0.2 | 0.3 | 0.4 | 0.2 | 0.3 | 0.4 |
| Tx | (1) Tx Power (dBm) | 23 |  |  |  |  |  |
| Rx | (2) Thermal noise density (dBm/Hz) | -174 |  |  |  |  |  |
|  | (3) eNB receiver noise figure (dB) | 3 |  |  |  |  |  |
|  | (4) Interference margin (dB) | 0 |  |  |  |  |  |
|  | (5) Occupied channel bandwidth (Hz) | 180k | 120k | 90k | 720k | 720k | 720k |
|  | (6) Effective noise power = (2)+(3)+(4)+10 log ((5)) (dBm) | -118.447 | -120.208 | -121.458 | -112.427 | -112.427 | -112.427 |
|  | (7) Required SINR (dB) | -3.05 | -1.15 | 0.1 | -9.7 | -9.6 | -9.5 |
|  | (8) Receiver sensitity =(6)+(7) (dBm) | -121.497 | -121.358 | -121.358 | -122.127 | -122.027 | -121.927 |
|  | (9) Receiver processing gain | 0 |  |  |  |  |  |
| MCL | (10) MCL = (1) - (8) + (9) (dB) | 144.4973 | 144.3582 | 144.3576 | 145.1267 | 145.0267 | 144.9267 |

Tables III and IV represent the MCL comparisons for SE per UE=0.2 and 0.05 [bps/Hz] at the BLER=0.1 with the ideal channel estimation, respectively. As shown in these tables, the NCMA can provide better MCL compared to OFDMA. Based on Tables II, III and IV, the performance gain of NCMA over OFDMA can be achieved as the supported number of users and target spectrum efficiency increases. Therefore, NCMA based on MQAM-quantized Grassmannian Sequence is considered for the NoMA design in NR.

Observation 3: The gain of the spreading based NCMA over OFDMA increases as the supported number of users and target spectrum efficiency increases.

Proposal 2: The spreading based NCMA based on MQAM-quantized Grassmannian Sequence is considered for the NoMA design in NR.

Summary

In this document, we presented further evaluation results for the spreading based NCMA. Following the results, our observation and proposal can be summarized as below.

Observation 1: MUD performance of NoMA scheme depends on the channel estimation performance.

Observation 2: MUD performance of NoMA is degraded under real channel estimation.

Observation 3: The gain of the spreading based NCMA over OFDMA increases as the supported number of users and target spectrum efficiency increases.

Proposal 1: The realistic channel estimation should be considered for the NoMA design in NR.

Proposal 2: The spreading based NCMA based on MQAM-quantized Grassmannian Sequence is considered for the NoMA design in NR.

Reference

TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

R1-162517, LG Electronics, “Considerations on DL-UL multiple access for NR”

R1-166874, LG Electronics, “Discussion on Non-orthogonal Spreading Sequences for NoMA”

R1-1609230, LG Electronics, “Discussion on Non-orthogonal Spreading Sequence and Codebooks for NoMA”

Appendix

- Examples of 64QAM-quantized Grassmannian Sequence based spreading codebook for minimizing the MUI are represented as follows:

Table V. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 2

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 2 | 5+5i5+5i5+5i-5-5i*Pno,2,2 |
| 4 | -3+5i-3+7i7-3i-3+1i-1-3i-7-3i7-3i5+3i*Pno,2,4 |
| 6 | -3+1i-7-3i7-3i-5+3i-1-5i-5+3i-1-7i-5-5i-1-7i-7+1i-1-1i3-3i*Pno,2,6 |
| 8 | -3-7i-7-1i-1-3i1-5i-3-1i-1-5i-1+7i-5-7i-3-1i-7+1i1-7i3+1i-3-7i1+5i-5+1i-1-7i*Pno,2,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.

Table VI. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 4

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 4 | 5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i-5-5i-5-5i-5-5i5+5i*Pno,4,4 |
| 6 | -7+3i-3+1i-5+1i-1-5i3+1i-1-7i1-7i-7+3i-1-7i-1+1i-3+1i3+5i-1-3i-1+7i-3+1i-5+5i-7+5i-3+1i-3-7i5+7i3+3i5-7i-1+3i1+3i*Pno,4,6 |
| 8 | -5-3i-7+1i1-5i1+3i-1-3i-7-3i-7+7i5-1i-1-3i-5-3i7-3i3+1i-5-3i-7+7i-1+5i-5+5i-7-3i1-7i1-3i1-7i-1+3i7+1i-5+7i-3+5i-5+7i-3-5i1+1i-7+7i-5+3i-1-1i7-5i3+1i*Pno,4,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.