3GPP TSG RAN WG1 Meeting #86bis	R1-1609439

Lisbon, Portugal, October 10th - 14th, 2016

Agenda Item:	8.1.3.1

Source:	Huawei, HiSilicon

Title:	Further results on channel coding schemes for NR

Document for:	Discussion and decision

# Introduction

This contribution evaluates the performance and complexity of the polar code [1] and E-TBCC (Enhanced-Tail-Biting-Convolutional-Code) [2].

# Code Construction

E-TBCC

In order to improve the performance for lower code rates and small block lengths, three schemes are considered [1][2]:

- TBCC Scheme-1 (64-state) [2]:  be enhanced by extending the mother code rate from 1/3 to lower code rates and the list of polynomials shown in Table 1.

- TBCC Scheme-2 (256-state) [2]: be enhanced by adopting convlutional code of larger constraint length and by extending the mother code rate from 1/3 to lower code rates, and the first 3 polynomials [561, 753, 712] (octal) inTable 2.

Note that Scheme-2 has the optimal performance boundary of the E-TBCC.

Table 1 Nested polynomials for TBCC with constraint length 𝛎+1=7

| n | Polynomial | df | Adf | Cdf |
| --- | --- | --- | --- | --- |
| 1 | 133 | - | - | - |
| 2 | 171 | 10 | 11 | 36 |
| 3 | 165 | 15 | 3 | 7 |
| 4 | 117 | 20 | 2 | 3 |
| 5 | 135 | 25 | 1 | 1 |
| 6 | 157 | 30 | 1 | 2 |
| 7 | 135 | 36 | 4 | 8 |
| 8 | 123 | 40 | 1 | 1 |
| 9 | 173 | 46 | 3 | 6 |
| 10 | 135 | 51 | 2 | 4 |
| 11 | 171 | 56 | 2 | 3 |
| 12 | 135 | 61 | 1 | 1 |

Table 2 Nested polynomials for TBCC with constraint length 𝛎+1=9

| n | Polynomial | df | Adf | Cdf |
| --- | --- | --- | --- | --- |
| 1 | 561 | - | - | - |
| 2 | 753 | 12 | 11 | 33 |
| 3 | 715 | 18 | 4 | 11 |
| 4 | 517 | 24 | 2 | 3 |
| 5 | 675 | 30 | 1 | 2 |
| 6 | 513 | 36 | 1 | 1 |
| 7 | 657 | 43 | 2 | 4 |
| 8 | 745 | 49 | 1 | 1 |
| 9 | 753 | 56 | 4 | 8 |
| 10 | 473 | 62 | 3 | 6 |
| 11 | 745 | 68 | 2 | 3 |
| 12 | 753 | 74 | 1 | 2 |

- PC (parity-check)-Based polar code construction, in which some parity-check bits are put on the PC (parity-check)-frozen-bit set. These parity-check bits would replace the CRC bits to help to correct the error. The scheme illustrated in Figure 1. The general rules to select the PC-frozen-bit set and to establish the PC function are deterministic and straightforward and detailed in [1].

Figure 1. Parity-Check-Based polar code

# Performance Comparison

To avoid any degradation of blind detection based on the masked CRC bits, neither decoder uses the CRC-bits for error correction. Therefore, Viterbi decoder is used for the E-TBCC, while a PC-SCL decoder (List=8 and List=32) is used for the PC-Based Polar code.

## Simulation with LTE-PDCCH parameter

Figure 2.	 Code length (N=288 bits), info block length (K=28/43/67 bits) , PC-Based Polar vs E-TBCC (Scheme-1)

Figure 3.	 Info block length (K=120 bits)  PC-Based Polar vs E-TBCC (Scheme-1/Scheme-2)

Figure 4.	 Code length (N=576 bits)  PC-Based Polar vs E-TBCC (Scheme-2)

From the results in Figures 2,3 and 4, it is observed the following:

Observation-1: The performance of PC-SCL-8 polar decoder is comparable with E-TBCC Viterbi-decoder of E-TBCC for K( info. block size) <=40 bits, and PC-SCL-8 outperforms E-TBCC for K> 40 bits.

Observation-2:  PC-SCL-32 decoder gain compared to Viterbi E- TBCC decoder is over 1dB gain.

Observation-3:  When the code length increases, the E-TBCC Viterbi decoder performance degrades

## Simulation with Simulation Assumptions in [4]

Figure 5.	 Info block length (K=80-bit), R= 1/3~1/9,  PC-Based Polar vs E-TBCC (Scheme-1/Scheme-2)

Figure 6.	 Info block length (K=120-bit), R= 1/3~1/9, PC-Based Polar vs E-TBCC (Scheme-1/Scheme-2)

From the results in Figures 5 and 6, it is observed the following:

Observation-4:  For K>120 bits, PC-SCL-8 polar decoder outperforms the Scheme-2 E-TBCC, with gain of up to ~0.8dB at BLER = 0.001.

Observation-5:  PC-SCL-32 decoder gain compared to Scheme-2 E-TBCC  can be up to ~1.5dB.

# Complexity Comparison

The computational complexity is based on the analysis in [3]. As an example, Table 3 illustrates the complexity for an information block length of 40-bit and code block length of 512.

Table 3 Computational Complexity

| PC-Based Polar Code with SC-SCL decoder |  |  | E-TBCC by a Viterbi decoder |  |  |
| --- | --- | --- | --- | --- | --- |
| List = 32 | List = 8 | List = 4 | Scheme-1(64-state, R=1/3) | Scheme-2(256-state, R=1/6) | Scheme-2(256-state, R=1/9) |
| ~178K | ~43K | ~21K | ~33K | ~614K | ~860K |

It is observed that

Observation-6: Viterbi decoder of Scheme-1 E-TBCC with R=1/3 has a comparable complexity as PC-SCL-8 decoder.

Observation-7: Viterbi decoder of Scheme-2 E-TBCC has much higher complexity than PC-SCL-32 decoder.

# Conclusions

Observation-1: The performance of PC-SCL-8 polar decoder is comparable with E-TBCC Viterbi-decoder of E-TBCC for K( info. block size) <=40 bits, and PC-SCL-8 outperforms E-TBCC for K> 40 bits.

Observation-2:  PC-SCL-32 decoder gain compared to Viterbi E- TBCC decoder is over 1dB gain.

Observation-3:  When the code length increases, the E-TBCC Viterbi decoder performance degrades rapidly and the coding gain of Polar vs E-TBCC can be up to ~2dB.

Observation-4:  For K>120 bits, PC-SCL-8 polar decoder outperforms the Scheme-2 E-TBCC, with gain of up to ~0.8dB at BLER = 0.001.

Observation-5:  PC-SCL-32 decoder gain compared to Scheme-2 E-TBCC can be up to ~1.5dB.

Observation-6: Viterbi decoder of Scheme-1 E-TBCC with R=1/3 has a comparable complexity as PC-SCL-8 decoder.

Observation-7: Viterbi decoder of Scheme-2 E-TBCC has much higher complexity than PC-SCL-32 decoder.

# References

- R1-1608862, “Polar Code Construction for NR”, Huawei,  HiSilicon

- R1-166926, “Further Discussion on TBCC Complexity and Enhancement”, Ericsson

- R1-164040, “ On latency and complexity” , Huawei,  HiSilicon

- Chairman’s notes, 3GPP TSG RAN WG1 #86bis meeting.