3GPP TSG-RAN WG1 #86  													                           R1- 1609466

Source: 	Intel Corporation

Title:	Remaining Details of open-loop techniques

Agenda item:	    7.2.4.3

Document for:	Discussion and Decision

# Introduction

In the RAN 1 #86 meeting the specification enhancement on DMRS based semi-open-loop transmission was agreed [1], the following areas are included.

- Support RE level transmitter operation on PDSCH for both CSI reporting and PDSCH transmission

  - Down selection from the following transmission schemes in RAN1#86bis, including possible combination

    - Precoder cycling

    - Tx diversity

    - LD-CDD

    - Layer Permutation

In this contribution we provide our views on the required specification enhancements for supporting DMRS based semi-open-loop transmission schemes.

# Discussion

Since Rel-8, the open-loop technique has been adopted. In TM3, when rank v is greater than 1, the precoder for PDSCH with large delay CDD can be changed every v subcarriers. For every v contiguous subcarriers, the same random precoders can be used to form v beams. Within these subcarriers, the CDD matrix can change the mapping of v intermixed data symbols to the v beams, which makes the channel qualities of the TB close to each other. When rank v is equal to 1, the transmit diversity is used, where the SFBC is applied to 2 or 4 consecutive REs. As the TM3 is CRS based, it cannot support open-loop scheme for eNBs with more than 4 antenna ports. In addition since the DMRS based transmission schemes are the baseline for FD-MIMO systems, DMRS based open loop scheme is the only one way to implement the open-loop transmission.

## 2.1 Transmission Method

For rank 1 transmission, either transmit diversity or precoder cycling can be utilized to support open loop transmission. However for transmit diversity, due to requirement for even number of REs, the transmission on some OFDM symbols may not be possible. In this case due to presence of orphan RE(s), the PDSCH performance may be impacted. Comparing to transmit diversity, the precoder cycling doesn’t have such requirement and therefore is more preferred.

For rank>1 transmission, the precoder cycling can be applied to enable the beam diversity. As observed in [2], compared to RB level precoder cycling, the performance for the RE level precoder cycling with large delay CDD can provide better performance, especially for lower SINR case. Further from [3] it can be observed that significant performance gain can be achieved from RB level precoder cycling with per subcarrier large delay CDD, compared to that without large delay CDD.

Link level simulations are provided to evaluate the performance difference for RE or RB level, precoder cycling or precoder cycling +LD-CDD semi-OL transmission schemes. Detailed simulation assumptions can be found in [4] and also listed in the Appendix for easy reference. Fore valuation 6 DMRS antenna ports and rank 2 PDSCH transmission were assumed. In this case the number of precoders within a PRBs is 3. The selected precoders were generated randomly to provide different transmitting directions. In one subframe, the three precoders can be cycled in the RE level (4REs) or RB level.

Figure 1 shows the link level performance of the rank 2 transmission for the UE speed of 30km/h, 60km/h and 120 km/h. RE and RB level precoder cycling with/without LD-CDD. It can be observed that 1-2dB performance gain can be achieved from LD-CDD compared to the case without LD-CDD. And compared with RB level precoder cycling, RE level precoder cycling is slightly preferred.

Figure 1: 8Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

Observation 1: 1-2dB performance gain can be observed from the LD-CDD compared to the case without LD-CDD, and the performance of RE-level precoder cycling is better than RB-level precoder cycling.

Proposal 1: for rank 1 transmission, compared to transmit diversity, per RE precoder cycling based transmission is slightly preferred as there would be no orphan RE issue; for rank>1 transmission, to obtain better performance, precoder cycling with large delay CDD based data transmission should be applied, and RE level precoder cycling is preferred.

## 2.2 CSI reporting scheme

For the semi-open-loop based DMRS, the precoder can be given as W=W1W2, where W1 indicates the long-term and wideband precoder and W2 denotes the short-term precoder which can be one beam selected from a set of beams. The W1 information in CSI report represents the long-term transmission direction, while W2 information represents the short-term transmission direction. In the scenarios with UE mobility the accuracy of W2 cannot be sufficient enough to guarantee the performance and the precoding cycling across W2 is required.  Since the reported W2 matrix is not used for the transmission, the overhead of the uplink control information can get redcued by not transmitting the feedback of W2.

The calculation of RI and CQI, conditioned on the selected long-term transmission direction W1, the UE could assume the precoder cycling is applied for W2. In tis case the reported RI and CQI can reflect the channel state information taking onto account with diversity gain provided by W2 cycling.

Proposal 2: CSI reporting for semi-open loop schemes can include feedback of the long-term transmission direction represent by W1as well as the RI and CQI estimated based on precoder cycling.

# Conclusions

In this contribution we have provided our views on specification enhancements for DMRS based semi-open loop transmission. From the discussion, we have the following proposals.

Observation 1: 1-2dB performance gain can be observed from the LD-CDD compared to the case without LD-CDD, and the performance of RE-level precoder cycling is better than RB-level precoder cycling.

Proposal 1: for rank 1 transmission, compared to transmit diversity, per RE precoder cycling based transmission is slightly preferred as there would be no orphan RE issue; for rank>1 transmission, to obtain better performance, precoder cycling with large delay CDD based data transmission should be applied, and RE level precoder cycling is preferred.

Proposal 2: CSI reporting for semi-open loop schemes can include feedback of the long-term transmission direction represent by W1as well as the RI and CQI estimated based on precoder cycling.

# References

- 3GPP Chairman's Notes RAN1#86

- R1-164433, “Discussion on DMRS based OL and Semi-OL Transmission”, Qualcomm

- R1-164151, “Evaluation of open loop schemes”, Intel

- R1-163832,”WF on Evaluation Assumptions for DMRS based Semi-OL(Office Agreement)”, Samsung

# Appendix – link level simulation assumptions

Table 2 Simulation Assumptions

| Parameters | Value |
| --- | --- |
| System bandwidth | 10MHz (50RBs) |
| Antenna configuration | 8 ports 2 Rx antennas (cross-polarization) |
| Channel model | 3D UMa 500m |
| UE velocity | 30 km/h, 60 km/h, and/or 120km/h |
| Carrier Frequency | 2GHz |
| Overhead | 2 PDCCH symbols per sub-frame |
| Modulation, code rates | Based link adaptation |
| HARQ | HARQ with max 4 transmissions |
| Number of allocated PRBs | 20 PRBs |
| CSI feedback | Open loop with CDD: TM3 like feedback Open loop without CDD: TM9 like feedback Close loop: TM9 Semi-open loop: TM9 like feedback |
| Feedback delay | 5 msec |
| Feedback periodicity | 5 and/or 10 msec |
| Channel estimation for demodulation and CSI estimation | Non-ideal |
| Receiver | MMSE-IRC |
| Rank adaptation | NO |
| Baseline | TM 9 based closed loop transmission |