3GPP TSG RAN WG1 Meeting #86bis               													R1-1609731

Lisbon, Portugal 10th - 14th October 2016

Source: 	Ericsson

Title:	V2V Signals Transmission Dropping for SLSS Reception

Agenda Item:	7.2.1.5.3

Document for:	Discussion and Decision

# Introduction

In order to detect an incoming SLSS transmission and identify a new Sync Ref UE, the UE needs to periodically drop ProSe transmissions and scan the radio environment. The corresponding behaviour was specified in Rel-12 for Any Cell state [1]:

### 11.5.2	Selection/Reselection to intra-frequency SyncRef UE

#### […]

The UE shall be able to identify newly detectable SyncRef UE within Tdetect,SyncRef UE seconds if SyncRef UE meets the selection / reselection criterion defined in TS 36.331.

[…]

Tdetect,SyncRef UE is defined as 20 seconds at SCH Es/Iot ≥ [-4 dB], provided that the ProSe UE is allowed to drop or delay a maximum of 2% of its ProSe Direct Communication transmissions for the purpose of SyncRef UE selection / reselection.

The UE capable of ProSe Direct Communication shall be capable of performing S-RSRP measurements for [6] identified ProSe synchronization sources with the measurement period of [400] ms.

The above requirements are based on 40ms SLSS periodicity. For V2V, RAN1 has identified two important changes to the UE behaviour compared to Rel-12:

- The possibility to synchronize to GNSS instead of SLSS

- Increased SLSS periodicity to 200ms (working assumption).

In light of the above agreements we believe that a revision of the UE behaviour for sync acquisition is required in order to fulfil service requirements for V2V.

# V2V Sidelink Signals Dropping for SLSS Detection

According to the Rel-12 behavior the UE needs to scan over a long time period in order to detect a new potential sync Ref UE. Such period amounts to 400ms every 20s, under the understanding that a UE needs to receive up to 10 SLSS/PSBCH repetitions before successfully synchronizing.

In V2V working assumption the SLSS periodicity is increased to 200ms, 5 times larger than for ProSe, while SLSS is substantially the same as in Rel-12. Please note that Ericsson is proposing to change such working assumption to 256ms [2]. This implies that, in order to obtain similar SLSS detection accuracy as for Rel-12, a UE would need to drop 5x400=2s of sidelink transmissions every 20s, which implies that a V2V UE would be unable to fulfill service requirements.

Observation:

- In order to obtain similar SLSS detection accuracy as for Rel-12 a UE would need to drop 2s of sidelink transmissions every 20s, which implies that a V2V UE would be unable to fulfill service requirements.

We observe that, differently from Rel-12, a V2V UE may synchronize to GNSS, reducing the need to synchronize to another UE if GNSS has the highest synchronization priority. Actually, whenever a UE is synchronized to GNSS and provided that GNSS has higher priority than SLSS-based synchronization, there is no obvious reason why the UE should drop sidelink transmissions for monitoring SLSS.

Observation:

- In some cases a V2V UE does not need to continuously search for SLSS

  - E.g., when GNSS has higher synchronization priority than SLSS-based synchronization and the UE is accurately synchronized to GNSS.

We propose the following behavior for the V2V UE silence period:

Proposal:

- A V2V UE is not allowed to drop sidelink transmissions for the purpose of detection of new Sync Ref UEs if GNSS is (pre)configured to have higher synchronization priority than SLSS-based synchronization and the UE is accurately synchronized to GNSS.

  - Otherwise, the UE is allowed to drop a (FFS in RAN4) fraction of sidelink transmissions for the purpose of SLSS detection from new sync Ref UEs.

  - FFS in RAN4 the definition of “accurately synchronized to GNSS”.

# Conclusions

In this contribution we discuss the use of silent periods for V2V sidelink:

Observations:

- In order to obtain similar SLSS detection accuracy as for Rel-12 a UE would need to drop 2s of sidelink transmissions every 20s, which implies that a V2V UE would be unable to fulfill service requirements.

- In some cases a V2V UE does not need to continuously search for SLSS

  - E.g., when GNSS has higher synchronization priority than SLSS-based synchronization and the UE is accurately synchronized to GNSS.

Proposal:

- A V2V UE is not allowed to drop sidelink transmissions if GNSS is (pre)configured to have higher synchronization priority than SLSS-based synchronization and the UE is accurately synchronized to GNSS.

  - Otherwise, the UE is allowed to drop a (FFS in RAN4) fraction of sidelink transmissions for the purpose of SLSS detection from new sync Ref UEs.

  - FFS in RAN4 the definition of “accurately synchronized to GNSS”.

# References

[1] 36.133, 3GPP

[2] R1-166960, Distributed synchronization procedure for V2V over PC5, Ericsson