3GPP TSG-RAN WG1 #86bis	R1-1609841

Lisbon, Portugal, 10th-14th October, 2016

Source:	Ericsson

Title:	On Supported CSI-RS Density Values for Class A in eFD-MIMO

Agenda Item:	7.2.2.4

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following agreements were made with regards to overhead reduction for Class A CSI-RS in eFD-MIMO:

Agreement:

At least for Class A NZP CSI-RS with more than 16 CSI-RS ports:

- All ports in a CSI-RS resource are transmitted within the same subframe

  - i.e. CSI-RS overhead reduction is done in the frequency domain

- CSI-RS density d ∈ {1,1/2, and at least one other value <= 1/3} RE/RB/port

  - Other values of d are not precluded (e.g. 2/3, ¾)

  - FFS whether different ports in a CSI-RS resource may have different densities

- FFS PDSCH rate matching in the REs in PRBs with no CSI-RS ports within a group

  - Opt-1: comb like transmission

  - Opt-2: frequency domain measurement restriction

- FFS the detailed signalling design

In this contribution, we present simulation results on the different CSI-RS densities (i.e., d values) that should be supported in addition CSI-RS densities of 1 and 1/2.  Based on our results, we propose an additional density value that should be supported in Rel-14 eFD-MIMO for Class A CSI-RS.

# Results with Different CSI-RS Densities

In this section, we evaluate the performance of Class A CSI-RS schemes with frequency domain overhead reduction.  We performed system level simulations using a 32 port 8x4 dual polarized array with 2x1 subarray virtualization.  All 32 CSI-RS ports are located in the same set of PRBs (e.g., the even PRBs) with different CSI-RS densities.  The 32 port CSI-RS resource is attained by aggregating four 8-port CSI-RS configurations with CDM-4.  Detailed simulation parameters and assumptions are given in the Appendix A.

The performance comparisons between different CSI-RS densities is given in Figures 1-2 for 3D-UMi and 3D-UMa where a CSI-RS reuse factor of 3 is assumed.  The performance gains shown in Figures 1-2 are relative to a reuse-3 32 port CSI-RS design with 1 RE/RB/port (see Appendix B for absolute throughput value comparisons).  From the figure, it is seen that a configuration with CSI-RS density 1/3 can outperform a configuration with CSI-RS density 1/2 in terms of cell edge and mean user throughput particularly in the case of 3D-UMi.  However, reducing the CSI-RS density further (i.e., density values of 1/4 and 1/5) generally results in performance losses.  Particularly, the cell edge performance degrades significantly in the case of 3D-UMa.

With CSI-RS reuse 3, a Class A 32-port configuration with CSI-RS density 1/3 can outperform a configuration with CSI-RS density 1/2 in terms of cell edge and mean user throughput particularly in the case of 3D-UMi.

With CSI-RS reuse 3, a Class A 32-port configuration with CSI-RS densities 1/4 and 1/5 result in performance losses compared to a 32-port configuration with CSI-RS density 1/3 particularly in the case of 3D-UMa.

Hence, given these observations, we propose the following:

For Class A eFD-MIMO, a CSI-RS density value of 1/3 should be supported in addition to CSI-RS density values of 1 and 1/2.

Figure 1: Performance comparison between different CSI-RS densities for 3D-UMi

Figure 2: Performance comparison between different CSI-RS densities for 3D-UMa

# Conclusions

In this contribution, we presented simulation results on the different CSI-RS densities (i.e., d values) that should be supported in addition to CSI-RS densities of 1 and 1/2.  Based on our results, we made the following observations and proposal:

Observation 1	With CSI-RS reuse 3, a Class A 32-port configuration with CSI-RS density 1/3 can outperform a configuration with CSI-RS density 1/2 in terms of cell edge and mean user throughput particularly in the case of 3D-UMi.

Observation 2	With CSI-RS reuse 3, a Class A 32-port configuration with CSI-RS densities 1/4 and 1/5 result in performance losses compared to a 32-port configuration with CSI-RS density 1/3 particularly in the case of 3D-UMa.

Proposal 1	For Class A eFD-MIMO, a CSI-RS density value of 1/3 should be supported in addition to CSI-RS density values of 1 and 1/2.

# Appendix A

| Simulation parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD, 3D UMa 500m ISD |
| Antenna Configurations | 8x4 with 2x1 virtualization tilt: 130° for 3D-UMi and 122° for 3D-UMa |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (3D-UMi), 46 dBm (3D-UMa) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| CRS interference | 2 CRS ports, Colliding CRS. |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modelled. For reuse factor 3, sectors with the same orientation (i.e., 0°, 120°, 240°) are grouped together. |
| Codebook | 2D Grid of Beams based on DFT |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |

# Appendix B

Table 1.  Performance comparison between different CSI-RS densities with 50% RU in 3D-UMi

Table 2.  Performance comparison between different CSI-RS densities with 70% RU in 3D-UMi

Table 3.  Performance comparison between different CSI-RS densities with 50% RU in 3D-UMa

Table 4.  Performance comparison between different CSI-RS densities with 70% RU in 3D-UMa