3GPP TSG RAN WG1 Meeting #88	R1-1701752

Athens, Greece, February 13-17, 2017

Agenda Item:	7.2.4.1.1

Source:	Huawei, HiSilicon

Title:	Remaining details of OTDOA assistance data

Document for:	Discussion and decision

# Introduction

In RAN1#87, the following agreements were made:

Agreements

- NPRS sequence is determined based on a subsequence of length-2 truncated from PRS sequence introduced in Rel-14 for further indoor positioning enhancements.

  - For stand-alone and guard band operations, the length-2 NPRS sequence is the central 2 elements of Rel-14 PRS sequence.

  - For in-band operation, the length-2 NPRS sequence is truncated from Rel-14 PRS sequence corresponding to the LTE PRB of the NB-IoT carrier.

For in-band scenario,

- If NPRS subframe configuration is part A or Part A + Part B, legacy LTE PRS pattern in one PRB is adopted

- The NPRS location in slot is generated by the formula below

where ns is the slot number within a radio frame; l is the OFDM symbol number within slot ns; k is the subcarrier index within LTE bandwidth which is used for NB-IoT NPRS transmission; nPRB is the PRB index within LTE bandwidth which is used as NB-IoT NPRS PRB; vshift is the cell-specific frequency shift generated by , where  equals  unless configured by higher layers.

- Note:  It is assumed vshift, , , and PBCH antenna ports number will be signaled by higher layers.

- If NPRS subframe configuration is part B only, NPRS is punctured in OFDM symbols 5 and 6 in each slot

For Part-B NB-IoT specific assistance information:

- Number of subframes of NPRS in one occasion is NPRS {10, 20, 40, 80, 160, 320, 640, 1280}

- Periodicity of NPRS occasion TPRS: 160ms, 320ms, 640ms, 1280ms.

- Valid configurations are those satisfying NPRS <= TPRS

- For a given periodicity of NPRS occasion, the starting subframe offset of NPRS occasion = αTPRS, α∈{0,18,28,38,48,58,68,78}

In this contribution, remaining details of OTDOA assistance information are discussed.

# Indication of the sequence for NPRS

According to the agreements, for in-band operation, the NPRS sequence is truncated from Rel-14 PRS sequence. The mapping of sequence to PRB for LTE PRS is similar to that for CRS. Therefore, to indicate the sequence for NPRS, the indication of CRS sequence for an in-band NB-IoT carrier can be reused, i.e. introducing an LPP assistance information parameter NPRS-SequenceInfo to indicate the PRB index nPRB for the NPRS. Considering the NPRS transmission can be configured in any PRB in the LTE transmission bandwidth, the value range of the indication needs to be wider than for CRS, as shown in Table 1.

Table 1	Definition of NPRS-SequenceInfo

| NPRS-SequenceInfo | E-UTRA PRB index for odd number of | ###### NPRS-SequenceInfo | ###### E-UTRA PRB index for even number of |
| --- | --- | --- | --- |
| 0 - 74 | -37, -36, …, 37 | 75 - 174 | -50, -49, …, 49 |

Proposal 1: For each NB-IoT carrier configured for NPRS in in-band mode, a parameter NPRS-SequenceInfo in OTDOA assistance information indicates the sequence used by the NPRS.

# Collision avoidance between SIBx and NPRS

In an NB-IoT carrier, SIB1-NB transmission occurs in subframe #4 in every 2nd radio frame, and may collide with the NPRS transmission. An example of such collision is shown in Fig.1, where SIB1-NB is transmitted with 8 repetitions, and NPRS is configured by Part B only. In some cases, such collisions can be avoided by scheduling, i.e. place the NPRS transmission into the radio frames not containing SIB1-NB through selecting proper parameters, e.g. starting subframe offset for NPRS. However, if SIB1-NB has 16 repetitions or NPRS has a periodicity of 160ms, such scheduling methods do not work. If the collision occurs, to maintain the backward compatibility, the NPRS will not be transmitted in the collided subframe. However, it will degrade the performance of RSTD measurement as the subframes of SIB1-NB of neighbor cells are not known by UE. Therefore, in order to mitigate the problem, it is suggested to indicate the exact time resource allocation of SIB1-NB of neighbor cells to UE, i.e. the repetition number {4, 8, 16} included in the higher parameter schedulingInfoSIB1. If the NB-IoT carrier is a non-anchor carrier that does not carry SIB1-NB, the repetition number of SIB1-NB can be indicated as 0.

Figure 1	The collision between SIB1-NB and NPRS configured by Part B only

Proposal 2: The number of repetitions of SIB1-NB for each NB-IoT carrier configured for NPRS is indicated in OTDOA assistance information, from the set {0, 4, 8, 16}, with 0 denoting a non-anchor carrier.

For the SIBs other than SIB1-NB, it can be expected that the collision with NPRS may also occur. Similar with the SIB1-NB, scheduling can also be used for other SIBs to avoid the collision. It should be noted that, the gap between two transmissions of an SI-message is determined by the SI-window, SI periodicity and SI repetition pattern, which is much longer than that of SIB1-NB (i.e. 20ms). Therefore, there will be more unoccupied time resources for the network to schedule the NPRS transmission in. And it can be expected that the scheduling method to avoid collision will be more efficient for the other SIBs.

In Fig.2, two examples of NPRS scheduling to avoid collision with other SIBs are shown. In the first example, 4 SI-messages shown with colored boxes are transmitted with the same SI periodicity of 1280ms and the length of SI window is 160ms. There are multiple radio frames at the end of each SI period left for the network to schedule the NPRS transmission. In the second example, it is shown that the NPRS can also be scheduled in the radio frames between two adjacent transmissions of one SIB message, where the SIB is transmitted in every 4th radio frame in an SI window.

Figure 2	The scheduling of NPRS to avoid collision with other SIBs

Like SIB1-NB, indicating the time resource allocation of the other SIBs can also be applied to mitigate the impact of the collision. However, for each cell, there are multiple SIBs and each has its own scheduling information. If the time resource allocations of all the other SIBs are indicated, significantly more signaling overhead will be introduced.

Therefore, and considering that other SIBs are more flexibly scheduled than SIB1-NB, it is suggested to avoid the collision between SIBs other than SIB1-NB and NPRS by network scheduling.

Proposal 3: Collision between NPRS and SIBs other than SIB1-NB is assumed to be avoided by network scheduling.

# Indication of SFN

In time asynchronous network deployments, SFN alignment between two cells cannot be assumed. An example of this case is shown in Fig. 3, where there is a time offset between the beginning of frames with SFN = 0 of two cells. In the example, the same parameters of Part B NPRS subframe configuration are assumed for the two cells, where the periodicity is 1280 ms, the number of subframes is 160, and the starting subframe offset is 0.

When UE receives the NPRS subframe configuration of Cell 1, it cannot be sure about the exact time resource of NPRS in each occasion. Therefore, the UE has to blindly search the NPRS, and estimate the arrival timing after the threshold is met. It imposes a high requirement on the memory and processing capability for UE. However, if the time offset between the two cells is indicated to UE, it can speed up the completion and improve the accuracy performance of the arrival time estimation, because blind search in a large time window is not needed. The analysis is made for Part B only, but it can be understood that it is also applicable for the case of Part-A+Part-B.

Observation 1: When Part B only or Part A+Part B is configured, if the time offset between two asynchronous cells is known by UE, the RSTD measurement performance can be improved.

Figure 3	An example of asynchronous deployment of two cells

If subframes #4 or #9 are configured for NPRS by Part A and/or Part B, according to the agreements, NPRS can only occur when there is no NSSS or SIB1-NB. If the time resource of NSSS or SIB1-NB transmission is not known, blind search has to be performed by UE to find the NPRS transmission and obtain a correct measurement. For the NPRS in the subframe #9, 20ms search window is needed, while for the NPRS in the subframe #4, an even larger search window is needed. Therefore, it is needed to indicate the time offset to remove the uncertainty of NPRS transmission in subframe #4 or #9 at UE side.

Observation 2: When subframe #4 or #9 is configured for NPRS transmission, it is beneficial to remove the uncertainty of NPRS transmission timing in neighbor cells.

In LPP, slotNumberOffset indicates the slot number offset between a neighbor cell and a reference cell. The value set of slotNumberOffset is {0, 1, …, 19}, which is not enough to indicate the time offset. It is proposed to also indicate the SFN level offset between two cells. Then the slotNumberOffset together with the SFN offset can be used to indicate the exact time offset.

It would be possible to indicate SFN offset as the number of full radio frames Nrf counted from the beginning of the radio frame with SFN#0 of the reference cell to the beginning of the closest subsequent radio frame with SFN#0 of the neighbor cell. However, the value set of Nrf is {0, 1, …, 1023}, which will require much signaling overhead.

As the maximum periodicity of Part B is 1280 ms and 8 starting subframe offsets αTPRS (α∈{0,18,28,38,48,58,68,78}) can be signaled, it is enough for the SFN offset to indicate the difference of radio frames between the reference and neighbor cells within a period of 128/8 = 16 radio frames. However, for the SIB1-NB, the transmission gap between two repetitions is related to the number of repetitions. When the number of repetitions of SIB1-NB is 4, there will be a maximum transmission gap as 64 radio frames. Therefore, for SIB1-NB, the SFN offset only needs to indicate the difference of radio frames between the reference and neighbor cells with a period of 64 radio frames. Based on the analysis, the relevant SFN offset is:

SFN offset = Nrf mod 64

The value set of SFN offset is {0, 1, …, 63}, which minimizes the signaling overhead.

Proposal 4: For each NB-IoT carrier configured for NPRS, SFN offset {0, 1, … 63} with respect to the reference cell is indicated in OTDOA assistance information.

# Conclusions

In this contribution, remaining details of OTDOA assistance information were discussed. Following proposals are proposed,

Proposal 1: For each NB-IoT carrier configured for NPRS in in-band mode, a parameter NPRS-SequenceInfo in OTDOA assistance information indicates the sequence used by the NPRS.

Proposal 2: The number of repetitions of SIB1-NB for each NB-IoT carrier configured for NPRS is indicated in OTDOA assistance information, from the set {0, 4, 8, 16}, with 0 denoting a non-anchor carrier.

Proposal 3: Collision between NPRS and SIBs other than SIB1-NB is assumed to be avoided by network scheduling.

Proposal 4: For each NB-IoT carrier configured for NPRS, SFN offset {0, 1, … 63} with respect to the reference cell is indicated in OTDOA assistance information.

Observation 1: When Part B only or Part A+Part B is configured, if the time offset between two asynchronous cells is known by UE, the RSTD measurement performance can be improved .

Observation 2: When subframe #4 or #9 is configured for NPRS transmission, it is beneficial to remove the uncertainty of NPRS transmission timing in neighbor cells.

# References

RAN1#87 Chairman’s Notes, Reno, USA, 14th – 18th November, 2016.