3GPP TSG-RAN WG1 Meeting #88	R1-1702018

Athens, Greece, February 13 – 17, 2017

Source:	Xinwei

Title:	Remaining details of DCI design

Agenda Item:	7.2.1.1

Document for:	Discussion

Introduction

DCI design for dynamic scheduling and UL/SL SPS was discussed in RAN1 #87 and #86bis meetings with the following agreements:

# Agreements: - If the DCI for SL SPS or dynamic scheduling is transmitted in subframe n and the offset value (in two-bit field) is set to m, the time location of initial transmission is the first subframe included in the resource pool that occurs at or after subframe n+4+m. - Offset value is one of {0, 1, 2, 3}. - Applicable to both SL SPS and dynamic scheduling. - Subframe n can be any of DL subframes or special subframes in TDD Uu carrier.

And

Agreements: - An SL SPS RNTI different from SL dynamic scheduling RNTI is defined. - The following two fields are additionally included in the existing dynamic scheduling DCI (i.e., DCI 5A). - Working assumption: - SL SPS configuration index : 3 bits - Activation/release indication : 1 bit - If the number of information bits in SL SPS DCI format mapped onto a given search space is less than the payload size of DCI format 0 mapped onto the same search space, zeros shall be appended to SL SPS DCI format until the payload size equals that of DCI format 0 including any padding bits appended to DCI format 0. - FFS whether the size of SL SPS DCI format can be larger than the size of DCI format 0 on the same search space. Agreements: - The following existing field in DCI 0 (for the legacy UL SPS activation/release) is reused to indicate V2X UL SPS configuration index. - Working assumption: Cyclic shift DM RS (3bits) - TPC command (2 bits) can be considered as an alternative if this working assumption is not confirmed. - Each UL SPS DCI contains a single SPS configuration index.

In this contribution, we discuss some remaining issues for DCI design.

DCI design

Due to the existence of up to 8 SPS configurations agreed by RAN2, 3 bits of SL SPS configuration index field should be included in the dynamic scheduling DCI format 5A. The active/release mechanism for SL transmission should be flexible to support traffic change, so both implicit and explicit release should be supported, and 1 bit activation/release should also be included in DCI 5A.

Similarly the UL SPS configuration index field should be indicated in DCI format 0. Considering that all fields in DCI format 0 except for TPC and cyclic shift DM RS are used in V2X UL SPS scheduling, the 3-bit CS DMRS field should be reused to indicate V2X UL SPS configuration index.

Therefore, we propose to confirm the following working assumptions.

Proposal 1: Confirm the working assumption：

- The following two fields are additionally included in the existing dynamic scheduling DCI (i.e., DCI 5A):

  - SL SPS configuration index: 3 bits

  - Activation/release indication: 1 bit

- The following existing field in DCI 0 (for the legacy UL SPS activation/release) is reused to indicate V2X UL SPS configuration index: Cyclic shift DM RS (3bits).

The maximum payload size of DCI format 5A used for V2X SL SPS scheduling is calculated in Table 1 with all different system bandwidths. In this table the sizes of lowest index of sub-channel field and frequency resource location field are assumed variable depending on system bandwidth.

As a comparison, the payload size of current DCI format 0 under corresponding system bandwidths is calculated in Table 2.

Table 1 Size of SL SPS scheduling DCI format 5A

| Bandwidth | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CIF | 0 or 3 |  |  |  |  |  |
| Lowest index of sub-channel | 0 | 2 | 3 | 4 | 4 | 5 |
| Frequency resource location | 0 | 3 | 4 | 6 | 7 | 8 |
| Time gap between initial tx and retx | 4 |  |  |  |  |  |
| SPS configuration index | 3 |  |  |  |  |  |
| SPS activation/release | 1 |  |  |  |  |  |
| Maximum size | 11 | 16 | 18 | 21 | 22 | 24 |

Table 2 Size of DCI format 0

| Bandwidth | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CIF | 0 or 3 |  |  |  |  |  |
| Format 0/1A flag | 1 |  |  |  |  |  |
| Hopping flag | 1 |  |  |  |  |  |
| MCS | 5 |  |  |  |  |  |
| NDI | 1 |  |  |  |  |  |
| TPC | 2 |  |  |  |  |  |
| CS DMRS | 3 |  |  |  |  |  |
| CQI request | 1 |  |  |  |  |  |
| SRS request | 0 or 1 |  |  |  |  |  |
| RA type | 0 or 1 |  |  |  |  |  |
| RB assignment | 5 | 7 | 9 | 11 | 12 | 13 |
| Minimum size | 19 | 21 | 23 | 25 | 26 | 27 |

Before any padding bit introduced, for DCI format 0 and SL SPS DCI format 5A that scheduling resources on the same bandwidth, the maximum size of DCI format 5A is always smaller than the minimum size of DCI format 0 mapped onto the same search space.

Observation 1: The payload size of SL SPS DCI is always not larger than the size of DCI format 0 mapped onto the same search space under same scheduling bandwidth.

When cross-carrier scheduling is introduced, payload size of DCI format 5A scheduling resources on large bandwidth may exceed size of DCI format 0 based on current mechanism. Consider that different scheduling bandwidth might be introduced and to decide when the size of DCI format 5A is not larger than DCI format 0 is complicated, and when size of DCI format 5A is larger, padding of DCI format 0 is not recommended, we propose to control the length of DCI format 5A by scheduling a limited set of sub-channel related parameters, such as restricting available number or size of scheduled sub-channels. With this solution the size of DCI format 5A is always not larger than DCI format 0.

Proposal 2: For DCI format 5A in cross-carrier case, restrict the value range of sub-channel related parameters such as scheduled number of sub-channels or sub-channel size to keep size of DCI format 5A not larger than DCI format 0.

Conclusions

Proposal 1: Confirm the working assumption：

- The following two fields are additionally included in the existing dynamic scheduling DCI (i.e., DCI 5A):

  - SL SPS configuration index: 3 bits

  - Activation/release indication: 1 bit

- The following existing field in DCI 0 (for the legacy UL SPS activation/release) is reused to indicate V2X UL SPS configuration index: Cyclic shift DM RS (3bits).

Observation 1: The payload size of SL SPS DCI is always not larger than the size of DCI format 0 mapped onto the same search space under same scheduling bandwidth.

Proposal 2: For DCI format 5A in cross-carrier case, restrict the value range of sub-channel related parameters such as scheduled number of sub-channels or sub-channel size to keep size of DCI format 5A not larger than DCI format 0.

References

[1] Draft minutes report of RAN1 #87