3GPP TSG RAN WG1 Meeting	#88		   					 R1-1702330

Athens, Greece, 13th-17th February 2017

Agenda Item:	8.1.2.4.1

Source:	InterDigital Communications

Title:	On CSI-RS Design for CSI Acquisition

Document for:	Discussion

# Introduction

In RAN1 NR-Adhoc meeting, RAN1 made the following agreements as a progress for CSI-RS design [1]:

Agreements:

- The CSI-RS RE mapping pattern of one N-port CSI-RS resource is composed of one or multiple CSI-RS RE mapping patterns of CSI-RS resources of equal or smaller number of ports, [e.g., 2, 4, or 8]

  - A CSI-RS RE mapping pattern is defined within a slot

  - FFS: A CSI-RS RE mapping pattern can span multiple configurable consecutive/non-consecutive OFDM symbols

  - FFS on mapping of ports to the CSI-RS RE mapping pattern

  - Density per port in terms of RE per port per PRB is configurable supports for density greater than 1 is not precluded

Agreements:

- For periodic CSI-RS,

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

  - Aperiodic CSI reporting is triggered by DCI

    - FFS: Necessity of additional signaling with MAC CE

- For semi-persistent CSI-RS,

  - Periodic CSI reporting is not supported.

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission

  - Aperiodic CSI reporting is triggered by DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Necessity of additional signaling with MAC CE

- For aperiodic CSI-RS,

  - Periodic [and semi-persistent] CSI reporting is not supported

  - Aperiodic CSI reporting is triggered by DCI

    - Aperiodic CSI-RS is triggered by DCI and/or MAC CE

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission, e.g., common DCI signaling between CSI reporting and CSI-RS transmission

    - FFS: Necessity of additional signaling with MAC CE

- Note that further down selection can be done later between MAC CE and DCI in above bullets

- Note that it is possible to dynamically trigger RS and reports through links in the measurement setting

In this contribution, we discuss further details on design considerations for CSI-RS.

# Considerations

The configurable CSI-RS density has been agreed as the RS overhead gets increased for a higher number of CSI-RS ports with a UE-specific CSI-RS configuration. Therefore, reducing the RS overhead while increasing CSI-RS pattern reuse to support a larger number of UEs may help the system performance, although it slightly degrades the CSI reporting accuracy.

In LTE, the CSI-RS density of 1RE/port/RB has been supported as a maximum and the density can be further reduced by nulling of CSI-RS in a PRB level. It has been observed that there is a marginal performance difference between 1RE/port/RB and a higher CSI-RS density. This is mainly due to implicit CSI feedback has been used in LTE. Since two CSI feedback types are supported and Type II CSI requires high resolution, a higher density than 1RE/port/RB may be considered and carefully investigated at least for Type II CSI feedback in terms of system performance and RS overhead.

As the PRB-level nulling of CSI-RS is evaluated and supported in LTE already, at least PRB-level nulling of CSI-RS to support the configurable CSI-RS density should be supported. The configurable CSI-RS density in time domain can be further studied.

Proposal-1: support at least PRB-level nulling of CSI-RS to configure the density

Multiple OFDM symbols can be used to multiplex antenna ports in a slot, however, a code spreading has to be used across antenna ports located in a different OFDM symbol to support a full power transmission. For example, orthogonal cover code (OCC) in time has been used for the CSI-RS antenna ports so that all antenna ports are multiplexed in FDM and CDM within an OFDM symbol, thus allowing a full power transmission.

For better coverage of CSI-RS, the full power transmission should be considered as one of design criteria for CSI-RS pattern design.

Proposal-2: CSI-RS design should support full power transmission always

It has been agreed to support both aperiodic and semi-persistent NZP-CSI-RS transmission, where the aperiodic and semi-persistent NZP-CSI-RS is a UE-specific configuration and triggered dynamically. Since the wideband NZP-CSI-RS transmission is supported, the NZP-CSI-RSs triggered for other UEs in a slot may interfere NR-PDCCH and/or NR-PDSCH reception in the slot. The performance impacts from the NZP-CSI-RS interference for PDSCH reception has been evaluated and investigated in LTE and RAN1 decided to support rate-matching around the NZP-CSI-RS REs for PDSCH.

As similar to LTE, in order to minimize the performance impacts from NZP-CSI-RS, at least NR-PDSCH RE muting (e.g., rate-matching) of aperiodic and semi-persistent NZP-CSI-RS should be supported. Therefore, aperiodic and semi-persistent ZP-CSI-RS should be introduced in NR.

Proposal-3: support aperiodic and semi-persistent ZP-CSI-RS for NR-PDSCH RE muting

A CSI-RS resource pool is used in NR and a UE may be configured and/or indicated to measure one or more CSI-RS resources in the CSI-RS resource pool. Since the configuration and indication is a UE-specific, the active aperiodic and/or semi-persistent NZP-CSI-RS set may be dynamically changed from a slot to another. A UE scheduled for NR-PDSCH reception with and without aperiodic and/or semi-persistent NZP-CSI-RS may need to know the active set of aperiodic and/or semi-persistent NZP-CSI-RS configurations in the slot for rate-matching purpose.

Since the active set of aperiodic and/or semi-persistent NZP-CSI-RS configurations are common for all UEs scheduled in the slot, it can be dynamically indicated in a common DCI.

Proposal-4: active set of aperiodic and/or semi-persistent ZP-CSI-RS is indicated in each slot

# Summary

This contribution discussed CSI-RS design considerations in NR. Based on the discussions, we propose followings:

Proposal-1: support at least PRB-level nulling of CSI-RS to configure the density

Proposal-2: CSI-RS design should support full power transmission always

Proposal-3: support aperiodic and semi-persistent ZP-CSI-RS for NR-PDSCH RE muting

Proposal-4: active set of aperiodic and/or semi-persistent ZP-CSI-RS is indicated in each slot

# References

RAN1 Chairman’s Notes, 3GPP TSG-RAN WG1 NR-Adhoc