3GPP TSG RAN WG1 Meeting #88                                           R1-1702545

Athens, Greece 13th - 17th February 2017

Agenda item:    7.2.3.4

Source:             Qualcomm Incorporated

Title:                  VoLTE enhancements for eMTC

Document for:  Discussion and Decision

- Introduction

In RAN#73 the FeMTC WI [1] was updated to include VoLTE enhancements for HD-FDD, which may be achieved by reducing DL repetitions, including new repetition factors, and adjusted scheduling delays.

In RAN1#86b the following agreements were reached:

Agreement:

- RAN1 design will assume delay budget relaxation for UL voice packets

- Prioritize consideration of the following in VoLTE enhancements in FeMTC for BL/CE UE considering overall system impacts:

  - New number(s) of repetitions for PUSCH

  - Adjusted scheduling relationships between physical channels

  - SRS coverage enhancement

  - Discontinuous PUSCH transmission

- SPS and non-SPS cases should both be considered

In this contribution we provide our views on how VoLTE can be enhanced for operation with repetitions.

- Modulation optimization for VoLTE

The LTE TBS table has a special entry for TBS 328 (Figure 1) that optimizes the PUSCH performance when using to repetitions (TTI bundling) or retransmissions by resorting to the use of QPSK modulation. In this case, the TBS is not self-decodable within a single subframe (due to coding rate greater than 1), but after the first retransmission the QPSK performance is better than that of 16-QAM. This table entry is especially selected to meet the TBS needed for codec AMR-WB 12.65k [1].

Figure 1 TBS table with special entry for 328, corresponding to rate 12.65k

Other codec modes, however, are not optimized for. For example, in Table 1 we show the TBS that should be used for different AMR modes, adjusted to available TBS sizes. For example, if we have a UE transmitting PUSCH with AMR 4,75k (TBS of 208), in the current TBS table it would need to use ITBS=12, which requires 16-QAM.

Table 1 Transport block sizes for different codec rates

| Frame content (AMR mode, comfort noise, or other) | Size adjusted to available TBS sizes |
| --- | --- |
| AMR 4,75 kbit/s | 208 |
| AMR 5,15 kbit/s | 208 |
| AMR 5,90 kbit/s | 208 |
| AMR 6,70 kbit/s (PDC-EFR) | 224 |
| AMR 7,40 kbit/s (TDMA-EFR) | 256 |
| AMR 7,95 kbit/s | 256 |
| AMR 10,2 kbit/s | 296 |
| AMR SID | 136 |
| AMR-WB 6.60 kbit/s | 208 |
| EVS 13.2 Channel Aware mode | 328 |
| EVS 7.2 | 208 |

In Figure 2 we show the performance different of TBS 208 for different number of repetitions when transmitted with 16-QAM or QPSK. We observe that the performance with QPSK is better than that for 16-QAM (~1dB for moderate SNR values).

Figure 2 TBS 208, 1PRB QPSK (dashed) vs 16-QAM (solid) for 1,2,4,…,32 repetitions. 2 Rx antennas, EPA5

Observation 1: For some TBS values, using QPSK instead of 16-QAM may increase performance when operating with repetitions.

Additionally, the maximum power reduction (MPR) also depends on the modulation scheme. The MPR for 1 PRB QPSK is 0dB, and for 16-QAM is 1dB. This can add up an additional 1dB gain

Observation 2: The MPR for QPSK/1PRB is 0dB, and for 16-QAM is 1dB. For the same link level performance, QPSK can provide an additional ~1dB gain over 16-QAM.

In view of these observations, it seems reasonable to optimize the modulation scheme used for PUSCH in VoLTE. This would lead to increased capacity (for the same coverage) or increased coverage (for the same capacity) compared with a sub-optimal modulation scheme.

- Proposed solution

The solution for modulation enhancements should meet the following requirements:

- Adapted to multiple codecs and conditions: The PHY may not know what codec the UE is operating on at a given moment, so the solution should be codec-agnostic. Also, the TBS may change depending on the availability of RoHC, among other factors. The flexibility of the solution should be adapted to these cases.

- Forward compatible: Considering a number of codecs for which we optimize the modulation may not be forward-looking, since advances in multimedia/coding may lead to a different set of TBS in a few years. The solution should be flexible enough to allow for future codecs to be successfully deployed without a need to change the PHY/TBS parameters.

In view of these two requirements, it does not seem feasible to add additional entries to the TBS table (like 328), since this would require selecting a subset of the TBS we want to optimize for, and thus is not flexible.

We propose to add a field in DCI that overrides the default modulation scheme. This field can be 1-bit that operates as follows:

- If the modulation order is Qm=2

  - Then the field is reserved.

- If the modulation is greater than 2

  - Override the modulation order with Qm-2 (i.e., 16-QAM QPSK)

This field would be present depending on RRC configuration. For example, there can be an explicit RRC parameter to enable this bit, or it can depend on whether other “VoLTE enhancement” is enabled (e.g. new set of repetitions).

Proposal 1: Include an additional 1-bit field in the uplink DCI to override the modulation scheme as follows:

  - If the modulation order is Qm=2 , then the field is reserved.

  - If the modulation is greater than 2, then override the modulation order with Qm-2 (i.e., 16-QAM QPSK)

Proposal 2: This field is enabled by:

- Option 1: New RRC parameter

- Option 2: Same RRC parameter that enables the adjusted set of repetitions

Although the benefit of these modulation enhancements is clear for PUSCH, they can also be applied to PDSCH to increase link performance, although there is no additional advantage for the MPR.

Proposal 3: Consider following a similar approach for PDSCH to reduce the number of repetitions required to reach a given BLER.

- Summary

Observation 1: For some TBS values, using QPSK instead of 16-QAM may increase performance when operating with repetitions.

Observation 2: The MPR for QPSK/1PRB is 0dB, and for 16-QAM is 1dB. For the same link level performance, QPSK can provide an additional ~1dB gain over 16-QAM.

Proposal 1: Include an additional 1-bit field in the uplink DCI to override the modulation scheme as follows:

  - If the modulation order is Qm=2 , then the field is reserved.

  - If the modulation is greater than 2, then override the modulation order with Qm-2 (i.e., 16-QAM QPSK)

Proposal 2: This field is enabled by:

- Option 1: New RRC parameter

- Option 2: Same RRC parameter that enables the adjusted set of repetitions

Proposal 3: Consider following a similar approach for PDSCH to reduce the number of repetitions required to reach a given BLER.

References

[1] RP-161464, Revised WID for Further Enhanced MTC for LTE, Ericsson, RAN#73