3GPP TSG RAN WG1 Meeting #88                                               	R1-1703377

Athens, Greece, 13th – 17th February 2017

Source: 	Hughes Network Systems,

Title:	Reduced PAPR APSK Constellations

Agenda Item:	8.1.5 Modulation

Document for:	Discussion and Decision

# Introduction

In 3GPP RAN1 #86bis, it was agreed that [1]:

Agreements:

- The same constellation mapping as used in LTE (i.e. QPSK, 16QAM, 64QAM and 256QAM) is introduced, while not precluding other constellation mappings

  - Note that there might be possibility to exclude some of above constellation mapping based on the further study

- Enhancement modulation schemes for further study include

  - Higher order modulation in conjunction with MIMO

  - Constellation mapping among subcarriers

  - Other constellations (e.g., non-uniform QAM)

  - Coded modulations

  - Spatial modulation

  - Mappings of bits to symbol(s)

  - Rotated-QAM up to BPSK, QPSK

    - kπ-QAM (0<k<=1)

      - FFS k (e.g., k = 0.5 for BPSK, 0.25 for QPSK)

  - Constellation Interpolation

  - Note: Other modulation schemes or combinations of the above schemes are not precluded

  - Note: Proponents should describe the details of the receivers

This contribution proposes APSK constellations with reduced PAPR.  Moreover the proposed constellations improve the PER vs SNR performance not only on nonlinear channels, but on linear channels as well.

# Discussion

One well-known method to reduce the PAPR of QAM constellations is using signal shaping through a shaping code. The disadvantage of this technique is the added receiver complexity due to the shaping decoder and the reduced rate due to the shaping code. An alternative method is to use APSK constellations. APSK constellations reduce PAPR without any additional shaping code. Moreover as shown in the figures below, they offer performance improvements even on linear channels. The proposed 16APSK, 64APSK, and 256APSK constellations are depicted below in Figures 1 through 3. These constellations have appeared in various standards including DVB-S2X [2] and DTMB-A [3]. Their capacity comparisons with respect to QAM constellations on linear channels are shown in Figures 4 through 6, and performance gains at representative bandwidth efficiency regions are shown in Table 1. In addition to the performance gains on linear channels shown in Table 1, the proposed constellations also have reduced PAPR as shown in Table 2, which would lead to additional gains on nonlinear channels where high power amplifiers (HPA) are operated close to saturation.

Figure 1. 16APSK Constellation

Figure 2. 64APSK Constellation

Figure 3. 256APSK Constellation (Gray Labeling)

Figure 4. Linear Channel Capacity Comparison of 16-APSK and 16-QAM

Figure 5. Linear Channel Capacity Comparison of 64-APSK and 64-QAM

Figure 6. Linear Channel Capacity Comparison of 256-APSK and 256-QAM

Table 1. Capacity Gains of APSK Constellations with respect to QAM on Linear Channels

| Constellation Order | Capacity Improvement on Linear Channel |
| --- | --- |
| 16 | 0.3 dB (@2.5 bits/sym.) |
| 64 | 0.5 dB (@3.5 bits/sym.) |
| 256 | 0.6 dB (@4.5 bits/sym.) |

Table 2. PAPR Reduction of APSK Constellations with respect to QAM

| Constellation Order | PAPR Reduction |
| --- | --- |
| 16 | 0.36 dB |
| 64 | 0.83 dB |
| 256 | 0.41 dB |

Observation 1: Proposed APSK constellations offer performance advantage with respect to QAM constellations even on linear channels at typical operating bandwidth efficiency regions.

Observation 2: Moreover, proposed APSK constellations have reduced PAPR compared to QAM constellations which would lead to more performance gains on nonlinear channels where HPA is operated close to saturation.

Proposal 1: NR supports the described APSK constellations for improved performance.

# Summary

This contribution proposes APSK constellations already adopted by various standardization bodies for improved performance on linear and nonlinear channels.

Proposal 1: NR supports the described APSK constellations for improved performance.

# References

[1] 3GPP RAN1 86bis, “Chairman’s Notes RAN1_86bis” Lisbon, Portugal, Oct 2016.

[2] ETSI EN 302 307-2 v1.1.1 Digital Video Broadcasting (DVB); Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering, and other broadband satellite applications; Part 2: DVB-S2 Extensions (DVB-S2X)

[3] Error correction, data framing, modulation and emission methods for digital terrestrial television broadcasting, Recommendation ITU-R BT. 1306-7, June 2015