3GPP TSG-RAN WG1 Meeting #89	R1-1706883

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	6.2.6.3

Source:	Ericsson

Title:	Uplink HARQ-ACK feedback for MTC

Document for:	Decision

# Introduction

The Rel-15 WI on “Even further enhanced MTC for LTE” [1] has the following WI objective for machine-type communications for BL/CE UEs:

- Power consumption reduction for physical channels [RAN1 lead, RAN2, RAN4]

  - Study and, if found beneficial for connected mode, specify physical signal/channel/DCI for HARQ-ACK feedback in DL for data transmission in UL.

RAN1#88bis made the following agreements:

- For the purpose of evaluations of HARQ-ACK feedback for UE power saving:

  - At least the following two options are considered for HARQ-ACK feedback channel:

    - Option 1: One HARQ-ACK feedback channel for multiple UE PUSCHs in one transmission

    - Option 2: One HARQ-ACK feedback channel for a single UE PUSCH in one transmission

  - Detailed design on HARQ-feedback channel FFS among at least the following options:

    - PHICH-like channel

    - MPDCCH with reduced DCI size(s)

    - MPDCCH with existing DCI size(s)

  - Early termination of PUSCH transmission and/or MPDCCH monitoring are considered.

    - The detailed solutions can be different for HD-FDD and FD-FDD/TDD.

- RAN1#88bis also agreed the evaluation assumptions and performance metrics in [2].

In this contribution we focus on the case highlighted above, i.e. the use of MPDCCH with existing DCI sizes for indication of HARQ-ACK feedback to a single UE for a PUSCH transmission from a single UE, for the purpose of saving power by terminating the MPDCCH monitoring earlier compared to Rel-13/14 operation.

# Discussion

The uplink HARQ operation for BL/CE UEs is asynchronous [3] and there is no explicit positive HARQ-ACK feedback sent from eNB to let the UE know whether an uplink data transmission on PUSCH was successfully received or not. When the uplink traffic is relatively frequent, the UE receives an implicit positive HARQ-ACK feedback for the previous uplink data transmission when it is scheduled with a new uplink data transmission using the same PUSCH HARQ process with the new data indicator (NDI) bit toggled.

However, when the uplink traffic is relatively infrequent, the UE may not receive any implicit positive HARQ-ACK feedback for an uplink data transmission since there is no immediate next uplink data transmission with the NDI bit toggled. The UE will wait until the higher layer uplink HARQ retransmission timers (UL HARQ RTT timer and drx-ULRetransmissionTimer) expire [3], which could take a long time compared to the time scale of the physical layer procedures.

This means that the UE may have to keep its receiver circuitry on when it could have shut it off earlier if it could receive a positive HARQ-ACK feedback in the form of an UL grant from eNB to let the UE know that it does not need to stay awake for potential HARQ retransmissions. This would allow the UE to go to sleep earlier when it is configured with DRX operation in RRC connected mode.

This may also be relevant for the case when the BL/CE UE is being released from RRC connected mode to RRC idle mode. When the UE receives an RRCConnectionRelease message, it sends a corresponding RLC-ACK on PUSCH and waits for ”1.25 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier” [4]. It may be possible to reduce the waiting time if the lower layers can indicate the receipt of the message earlier through the reception of a positive HARQ-ACK feedback from eNB.

Positive HARQ-ACK feedback signaling may allow the UE to go to sleep earlier in DRX operation in connected mode or during release from connected to idle mode.

Sections 2.1 and 2.2 in this contribution describe how signaling for positive HARQ-ACK feedback to a single UE using MPDCCH with existing DCI sizes can be introduced in CE modes A and B.

The impacts on implementations and specifications from introducing HARQ-ACK feedback for a single UE using MPDCCH with existing DCI sizes can be expected to be relatively small compared to the other options considered by RAN1.

If uplink HARQ feedback signaling is introduced in downlink, it is desired that the UE indicates with capability signaling whether it supports the new signaling or not, so that the network can make an informed decision regarding whether to spend downlink resources on transmitting the feedback signal or not.

If uplink HARQ-ACK feedback signaling is introduced in downlink, it should be an optional UE capability with UE capability signaling to the network.

## HARQ-ACK for single UE using existing DCI size in CE mode A

For BL/CE UEs, UL grants are scheduled using DCI format 6-0A in CE mode A [5]. A resource block assignment field in the DCI indicates the PUSCH PRB allocation. The field consists of {5, 6, 7, 8, 9, 9} bits for a system bandwidth of {1.4, 3, 5, 10, 15, 20} MHz. In CE mode B, all possible bit combinations are used, but in CE mode A, only some of the possible bit combinations are used [6][7]. For all possible configurations, at least 11 values are unused in CE mode A, as shown in Table 1.

Table 1: PUSCH resource allocation indices in CE mode A

| Configured maximum PUSCH channel bandwidth | System bandwidth | Number of unused values in resource block assignment field in DCI format 6-0A |
| --- | --- | --- |
| 1.4 MHz | 1.4 MHz | 11 |
|  | 3 MHz | 2 * 11 = 22 |
|  | 5 MHz | 4 * 11 = 44 |
|  | 10 MHz | 8 * 11 = 88 |
|  | 15 MHz | 16 * 11 = 176 |
|  | 20 MHz | 16 * 11 = 176 |
| 5 MHz | 1.4 MHz | 11 – 0 = 11 |
|  | 3 MHz | 22 – 3 = 19 |
|  | 5 MHz | 44 – 19 = 25 |
|  | 10 MHz | 88 – 59 = 29 |
|  | 15 MHz | 176 – 99 = 77 |
|  | 20 MHz | 176 – 139 = 37 |

Some of these at least 11 unused values could be used to indicate uplink HARQ-ACK feedback for the up to 8 uplink HARQ processes available in CE mode A according to Table 2.

Table 2: Uplink HARQ-ACK feedback in CE mode A

| Value | Meaning |
| --- | --- |
| Unused value 1 | Positive HARQ-ACK for UL HARQ process #0 |
| Unused value 2 | Positive HARQ-ACK for UL HARQ process #1 |
| Unused value 3 | Positive HARQ-ACK for UL HARQ process #2 |
| Unused value 4 | Positive HARQ-ACK for UL HARQ process #3 |
| Unused value 5 | Positive HARQ-ACK for UL HARQ process #4 |
| Unused value 6 | Positive HARQ-ACK for UL HARQ process #5 |
| Unused value 7 | Positive HARQ-ACK for UL HARQ process #6 |
| Unused value 8 | Positive HARQ-ACK for UL HARQ process #7 |
| Unused value 9 | Positive HARQ-ACK for all UL HARQ processes |

Consider introducing signaling of positive HARQ-ACK feedback to a single UE using existing DCI size in CE mode A using unused bit combinations in the resource block assignment field in DCI format 6-0A.

## HARQ-ACK for single UE using existing DCI size in CE mode B

For BL/CE UEs, UL grants are scheduled using DCI format 6-0B in CE mode B [5]. A modulation and coding scheme (MCS) index is indicated by a 4-bit field in the DCI. The MCS index is used to determine the modulation order and transport block size (TBS) index. In CE mode A, all MCS indices are used. However, for CE mode B, MCS indices larger than 10 are not used [6], meaning that 5 values are unused in CE mode B, as shown in Table 3.

Table 3: PUSCH MCS indices in CE mode B

| MCS index | Modulation order | TBS index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | Unused in CE mode B |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |

Some of these 5 unused values could be used to indicate uplink HARQ-ACK feedback for the up to 2 uplink HARQ processes available in CE mode B according to Table 4.

Table 4: Uplink HARQ-ACK feedback in CE mode B

| MCS index | Modulation order | TBS index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | Positive HARQ-ACK for UL HARQ process #0 |  |
| 12 | Positive HARQ-ACK for UL HARQ process #1 |  |
| 13 | Positive HARQ-ACK for all UL HARQ processes |  |
| 14 | Unused in CE mode B |  |
| 15 |  |  |

Consider introducing signaling of positive HARQ-ACK feedback to a single UE using existing DCI size in CE mode B using unused bit combinations in the MCS field in DCI format 6-0B.

# Conclusion

We made the following observations:

Observation 1	Positive HARQ-ACK feedback signaling may allow the UE to go to sleep earlier in DRX operation in connected mode or during release from connected to idle mode.

Observation 2	The impacts on implementations and specifications from introducing HARQ-ACK feedback for a single UE using MPDCCH with existing DCI sizes can be expected to be relatively small compared to the other options considered by RAN1.

We propose the following:

Proposal 1	If uplink HARQ-ACK feedback signaling is introduced in downlink, it should be an optional UE capability with UE capability signaling to the network.

Proposal 2	Consider introducing signaling of positive HARQ-ACK feedback to a single UE using existing DCI size in CE mode A using unused bit combinations in the resource block assignment field in DCI format 6-0A.

Proposal 3	Consider introducing signaling of positive HARQ-ACK feedback to a single UE using existing DCI size in CE mode B using unused bit combinations in the MCS field in DCI format 6-0B.

Further study of the potential gains and impacts is needed before final decisions are made.

# References

- RP-170732, “New WID on Even further enhanced MTC for LTE”, RAN#75, March 2017.

- R1-1706805, “Way forward on Uplink HARQ-ACK feedback”, Ericsson

- TS 36.321 V14.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification (Release 14)”, March 2017.

- TS 36.331 V14.2.2, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 14)”, March 2017.

- TS 36.212 V14.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding (Release 14)”, March 2017.

- TS 36.213 V14.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 14)”, March 2017.

- R1-1703969, “WF on Resource Allocation for FeMTC UEs with 5MHz PUSCH Channel Bandwidth”, Huawei, HiSilicon, Ericsson, Qualcomm, Softbank, Lenovo, Motorola Mobility, Sharp, RAN1#88, February 2017.