3GPP TSG RAN WG1 Meeting #89	R1-1708247

Hangzhou, China 15th – 19th May 2017

Agenda Item:	6.2.7.1.1

Source: 	SONY

Title:	Power consumption evaluation of wake-up signal technique for feNBIoT

Document for:	Discussion / Decision

# Introduction

In RAN1#88bis, the following agreements on downlink channel power efficiency were reached [1]:

- Techniques to be evaluated:

  - Wake-up signal/channel (either relying or not relying on DL synchronization)

  - Go-to-sleep signal/channel (either relying or not relying on DL synchronization)

  - Compact DCI

  - Reduced-bandwidth NPDCCH

  - Dynamic UESS periodicity

- The evaluation of the above techniques to be based on the simulation assumptions and metrics in [2].

According to these agreements, the proposed performance metrics for the evaluation of the above techniques are:

  - Power efficiency, PcandidatePref, in terms of % (reduced power consumption)

  - Miss and false detection rate  in terms of % (expressed as 100 – percentage of successful detection and false detection events)

  - System overhead in terms of % (as used part of all Res per radio frame),

And the proposed reference power consumption for a NB-IoT UE in different operational states: receive, light sleep, and deep sleep are 100, 1, and 0.015 [units/msec] respectively.

This contribution provides an evaluation of the first technique (wake up signal) based on their power consumption performance. In our evaluation, we follow the reference power consumption model of a UE in different states and use the proposed numerical values and assumptions.

# Power consumption calculation

We first calculate the total energy consumption for the reference scenario, i.e., a UE monitors the NPDCCH transmission, but does not receive a NPDSCH, for the following scenario:

- idle mode when UE in DRX and eDRX, (while we focus on idle mode our analysis can also be used for connected mode).

We then analyse the impact on the total power consumption from adding our candidate techniques to the reference scenario.

Reference scenario

Let us start with the power calculation of the reference scenario

Pref=ErefTeDRXCycle 	(1)

in idle mode and for one eDRX cycle. As shown in Fig.1 the total energy consumption Eref is the sum of the energy consumption during the paging transmission window (PTW), and energy consumption of a UE in deep sleep,

Eref=Eptw+(TeDRXcycle-Tptw)PDS.	(2)

Figure 1: Idle mode NPDCCH monitoring (X% of paging belongs to another UE(s))

During PTW, the UE needs to periodically, with a period equal to DRX-cycle, activate its receiver to monitor NPDCCH in paging occasions for a paging indicator from eNB. The UE needs to wake-up slightly earlier prior to its paging occasions for time/frequency synchronization purposes and also to re-confirm the serving cell. The CRC of the paging indicator is scrambled by P-RNTI, commonly used for all UEs, to identify if there exists a paging message. Therefore, only after the UE successfully decodes the NPDCCH and the subsequent NPDSCH (not shown in the figure), the UE realizes that the paging can belong to other UEs. In this case, the UE has performed an unnecessary operation by overhearing an NPDCCH and NPDSCH targeted to other UEs. Additionally, in extended coverage conditions, the paging indicator is transmitted with a certain number of repetitions to accumulate enough energy to decode the NPDCCH.

Taking into account the UE operation in all the states mentioned above, the total energy consumption during PTW, Eptw, becomes the sum of the periodic energy cost of the receiver ramp-up, sync., NPDCCH monitoring (including repetition for CE and the cost of overhearing), receiver ramp-down and the energy consumption in light sleep mode. Note that the energy consumption of receiver ramp-up/ramp-down and synchronization can be different when the UE wakes up from deep sleep (DS) and light sleep (LS).

Eptw=TDSruPDSru+TDSsyncPrx+(Npo-1)(TLSruPLSru+TLSsyncPrx+TLSrdPLSrd)+Npo(1-x) R TpoPrx +Npo x R TpoPrx+Npo(Tptw-TDSru-TDSsync-(Npo-1)(TLSru+TLSsync+TLSrd)-NpoR Tpo-TDSrd)PLS+TDSrdPDSrd.        		(3)

In this equation, R denotes the highest number of repetitions to reflect the maximum extended coverage used in a cell, Npo the number of paging occasions in PTW, and x the percentage that paging belongs to another UE(s). The time spent in a certain operational state and the corresponding power consumption are denoted by Tji and Pji, respectively. More detailed parameters description can be found in the Appendix. Replacing equations (3) and (2) back in equation (1) we calculate the total power consumption of the reference scenario.

Wake-up signal/channel (either relying or not relying on DL synchronization)

In order to improve energy efficiency of the paging in idle mode, the use of a wake-up signal to indicate the presence of the NPDCCH is introduced [3].

Figure 2: Idle mode processing with wake-up signal (X% of pagings belongs to another UE(s))

The wake-up signal has a length much shorter than NPDCCH, carries very limited information related to the target UE (or group of UEs) and is transmitted prior to the NPDCCH transmission, as shown in Fig. 2. As opposed to legacy NB-IoT (the reference scenario), a UE continues to decode the subsequent NPDCCH transmission only if it detects a wake-up signal carrying its identity. By doing this, the UE can save energy by both increasing the “sleep time” period and avoiding unnecessary NPDCCH decoding.

Taking into account the energy saving mechanism introduced above, the total energy consumption during one eDRX cycle becomes

Pcandidate=EcandidateTeDRXCycle	(4)

where

Ecandidate=EptwWUS+(TeDRXcycle-Tptw)PDS.	(5)

In equation (5) the first term EptwWUS  denotes the energy consumption during PTW when wake-up signal technique is used and the second term energy consumption of a UE in deep sleep. Similar to the reference scenario, the total energy consumption of wake-up signal technique in PTW, EptwWUS, is the sum of periodic energy cost of the receiver ramp-up, wake-up signal monitoring, NPDCCH decoding (only if a wake-up signal is detected), the receiver ramp-down and the energy cost in light sleep mode. Note that the wake-up signal provides both synchronization, when waking up from deep and light sleep, and coverage enhancement, under extended coverage conditions. Moreover, while a wake-up signal miss detection will not directly influence the UE power consumption, a wake-up signal false-alarm leads to an unnecessary decoding of the NPDCCH and extra energy cost in UE. Taking into account all the above energy cost EptwWUS becomes

Eptwwus=TDSruPDSru+TDSwusPrx+(Npo-1)(TLSruPLSru+TLSwusPrx+TLSrdPLSrd)+Npo(1-x) R TpoPrx +pfa Npo x R TpoPrx+Npo(Tptw-TDSru-TDSwus-(Npo-1)(TLSru+TLSwus+TLSrd)-Npo1-xR Tpo-pfa Npo x R Tpo-TDSrd)PLS+TDSrdPDSrd ,   	(6)

Again replacing equations (6) and (5) back in (4) we calculate the total power consumption of our candidate technique.

# Performance Evaluation

We evaluate the power consumption performance of our candidate approach. i.e., a use of wake-up signal/channel technique, by calculating the percentage of reduced power consumption

RePo=PcandidatePref,

for the scenarios listed in Table 1. Power consumption parameters needed to calculate Pcandidate (equation (3)) and Pref (equation (1)) are replaced by the nominal values proposed in [2]. Time interval parameters are replaced by nominal values listed in Table 2.

Table 1 – Studied scenarios

|  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| eDRX-cycle (TeDRXcycle) | 87.38 min | 87.38 min | 87.38 min | 40.96 sec** |
| DRX-cycle | 2.56 sec | 2.56 sec | 2.56 sec | 1.28 sec |
| Paging transmission window (Tptw) | 20.48 sec | 20.48 sec | 20.48 sec | 10.24 sec* |
| False-alarm probability (pfa) | 0 | 0 | 0.01 | 0.01 |
| x % of the paging belong to another UE | 0 | 90% | 90% | 90% |

* The number of paging occasions is the same as the other cases.

**It also covers connected mode

Table 2 – Time interval when UE operates in different states

| Parameter |  | Value | Description |
| --- | --- | --- | --- |
| NPDCCH | MCL =144 | 1 msec (One repetition) | Total time R Tpo spent to decode NPDCCH |
|  | MCL =164 | 27 msec (96 repetitions) |  |
| Sync | MCL =144 | 2 msec (rely on DL synchronization) | Time interval spent to re-acquire frequency/time synchronization. (reference scenario) |
|  |  | 80 msec (not rely on DL synchronization) |  |
|  | MCL =164 | 480 msec (extended coverage, not rely on DL synchronization) |  |
| Wake-up signal* | MCL =144 | 1/14 msec (one OFDM symbol and 1 RB, light sleep) | Time interval spent to synchronize and to indicate there is a paging message available. (wake-up signal technique) |
|  |  | 1 msec (one subframe and 1 RB, deep sleep) |  |
|  | MCL =164 | 1 msec (one subframe and 1 RB, extended coverage) |  |
| Ramp-up/ramp-down from/to deep sleep |  | 150/14 msec |  |
| Ramp-up/ramp-down from/to light sleep |  | 14/6 msec |  |

* In our proposal, the wake-up signal length is a function of DRX length and coverage conditions. For details we refer to Section 4.

The resulting reduced power consumption for the studied scenarios is shown in Fig. 3. As we observe the use of a wake-up signal before paging occasions can lead to a reduced power consumption of 92% in good coverage and 29% in extended coverage conditions. This translates to 6% and 71% energy saving at the UE in good coverage and extended coverage conditions, respectively. As we can see, the power saving is more pronounced for the extended coverage scenario and when the number of occasions that paging belongs to another UE(s) increases. Note that the energy saving would be higher if the analysis also included the reading of NPDSCH when other UEs are paged.

Observation 1: A use of a wake-up signal before paging occasions can lead to a reduced power consumption (compared to the reference scenario). The power saving is more pronounced for extended coverage scenario and when the number of occasions that paging belongs to another UE(s) increases.

Proposal 1: A wake-up signal, as a new signal/channel, needs to be used with the purpose to detect the presence of the subsequent NPDCCH.

# Wake-up Signal Design

The wake-up signal should be designed with certain properties. Specifically, in order to be decoded in a short time interval, it should carry a minimum of information. We consider the wake-up signal only represents an identifier. The identifier may not necessarily correspond to a single UE, but may correspond to a group of UEs. A UE that can detect that wake-up signal with a matching identifier should expect a subsequent transmission (e.g. NPDCCH and NPDSCH). Moreover, the wake-up signal is used to re-acquire time/frequency synchronization when waking up from a deep or light sleep mode. The coverage range of wake-up signal transmission should be considered as well. Therefore, the wake-up signal length can be adaptively selected to provide sufficient synchronization in different DRX modes and under different coverage conditions.

The Wake-up signal can be generated using certain known sequences (e.g. Zadoff-Chu sequence, M-sequence, etc) or a random bit sequence. At the receiver side, the detection complexity which usually relates to the UE power consumption should be kept low, significantly lower than the complexity of NPDCCH itself. From the network side, it is important to keep the network utilization as optimized as possible, e.g., the number of physical resources should be kept minimum and the possibility on the usage of common resources for more than one UE.

Proposal 2: The wake-up signal is used to indicate whether there is a paging message for a UE and also used for synchronization purposes when a UE wakes up from deep sleep or light sleep. The length of the wake-up signal is a function of DRX length and coverage conditions and is selected according to the DRX time interval and/or coverage conditions.

Proposal 3: When designing a wake-up signal, we also need to consider how wake-up signal properties influences the UE detection complexity, the required size/resource allocation of the wake-up signal transmission, targeted received signal sensitivity, and the wake-up signal detection performance.

# Conclusion

In this contribution we evaluated energy efficiency of wake-up signal technique.

Observation 1: A use of a wake-up signal before paging occasions can lead to a reduced power consumption (compared to the reference scenario). The power saving is more pronounced for extended coverage scenario and when the number of occasions that paging belongs to another UE(s) increases.

Proposal 1: A wake-up signal, as a new signal/channel, needs to be used with the purpose to detect the presence of the subsequent NPDCCH.

Proposal 2: The wake-up signal is used to indicate whether there is a paging message for a UE and also used for synchronization purposes when a UE wakes up from deep sleep or light sleep. The length of the wakeup signal is a function of DRX length and coverage conditions and is selected according to the DRX time interval and/or coverage conditions.

Proposal 3: When designing a wake-up signal, we also need to consider how wake-up signal properties influences the UE detection complexity, the required size/resource allocation of the wake-up signal transmission, targeted received signal sensitivity, and the wake-up signal detection performance.

# References

- “Chairman's Notes RAN1_88bis_final”, Spokane, April 2017

- R1-1706777 – “Way forward on simulation assumption on signal for power consumption reduction, Huawei, Hisilicon, Ericsson, Spokane, April 2017

- R1-1705203 – “NB-IoT UE Power Consumption Reduction in Idle Mode Paging, Sony, Spokane, April 2017.

# Appendix

Table 3 – Parameter description

| Variable | Description | Variable | Description |
| --- | --- | --- | --- |
| TDSru | UE’s receiver ramp-up time from deep sleep | PDSru | Receiver ramp-up power consumption in deep sleep |
| TDSrd | UE’s receiver ramp-down time to deep sleep | PDSrd | Receiver ramp-down power consumption in deep sleep |
| TLSru | UE’s receiver ramp-up time from light sleep | PLSru | Receiver ramp-up power consumption in light sleep |
| TLSrd | UE’s receiver ramp-down time to light sleep | PLSrd | Receiver ramp-down power consumption in light sleep |
| TDSsync | Synchronization time when UE wakes up from deep sleep | Npo | Number pf paging occasions in one PTW |
| TLSsync | Synchronization time when UE wakes up from light sleep | Prx | Receiver power consumption |
| TDSwus | Wake-up signal time when UE wakes up from deep sleep | PDS | Deep sleep power consumption |
| TLSwus | Wake-up signal time when UE wakes up from deep sleep | PLS | Light sleep power consumption |
| Tpo | Paging occasion length | x | Percentage that paging belongs to another UE(s) in one PTW |
| Tptw | Paging transmission window length | R | Repetition |
| TeDRX-cycle | eDRX-cycle length | pfa | Wake-up signal false-alarm probability |