3GPP TSG RAN WG1 Meeting #89             　　	                      R1- 1709027

Hangzhou, P.R. China 15th – 19th May 2017

Source:	Mitsubishi Electric

Title:	DMRS designs for NR MIMO: evaluation results

Agenda Item:	7.1.2.4.2 DMRS

Document for: 	Discussion/Decision

Introduction

In the RAN1#88bis meeting, the following agreements were made [1]:

R1-1706310 WF on the number of DL orthogonal DM-RS ports Huawei, HiSilicon, AT&T, IITM, Softbank, CeWiT, CMCC, IITH, Tejas Networks, MediaTek, CATT, CATR, Nokia, ASB, Mitsubishi Electric, NTT DOCOMO, Fujitsu, China Unicom, Deutsche Telekom, KDDI, TELECOM ITALIA, Xinwei - Confirm the following working assumption: - Support at least the following design of DL DM-RS for data channels - Support the maximal 12 orthogonal DL DMRS ports for MU-MIMO

R1-1706543 Additional DMRS structure Samsung, Qualcomm, ETRI, CATT, Ericsson, vivo, KT Corp R1-1706816 Additional DMRS structure Samsung, Qualcomm Incorporated, ETRI, CATT, Ericsson, vivo, KT Corp, Nokia, Alcatel-Lucent Shanghai Bell, MediaTek Inc., NTT DOCOMO, INC Agreements: - Companies are encouraged to perform further evaluations on additional DM-RS symbols, using same or lower density compared with front loaded DM-RS, and also identifying use cases associated with the operation - Aim to decide in the next meeting whether to support same density only, or lower density only, or both - FFS at least CP-OFDM, frequency domain density of front loaded DMRS is configurable.

In addition, the following agreement was made regarding CSI-RS density.

R1-1706441 WF on CSI-RS density LG Electronics, InterDigital, Intel, ITRI Agreements: - At least CSI-RS for CSI acquisition, NR supports CSI-RS density d RE/RB/port for x-port CSI-RS - Value(s) of d are at least d=1,1/2. - For d<1, PRB-level comb-type transmission is supported. - FFS whether offset value(s) can be the same or different across antenna ports - FFS on supporting d>1 in the consideration of use case, e.g., NZP CSI-RS for IMR. - FFS on the supported combinations of value(s) of x and d.

Based on the agreements above, we make a proposal on density-reduced DMRS placement.

DMRS evaluation results: equal vs. unequal density

In our companion contribution [2], we have shown four different designs for DMRS. For reference, the configurations are shown in Figure 9 through Figure 12 in the appendix. The simulation is conducted in the high speed train scenario at 30GHz. The simulation parameters are shown in Table 1 in the Appendix. We note that AFC using the CP from OFDM is implemented in the simulation. We also note that three interpolation methods are used in the simulation. Spectral efficiency performance for K=7dB is shown in Figure 1 through Figure 3 for different interpolation methods. It is clear from the figures that interpolating in both time and frequency domain does not yield satisfactory performance for configuration for “equal density 1”. On the other hand, averaging channel estimates in both time and frequency domain yield the best performance among three interpolation methods. This is due to the strong LoS component which does not introduce selectivity in both frequency and time domain. In addition, it is clear that equal or unequal density configuration do not make a big difference in the performance.

Spectral efficiency performance for K=13.3dB is shown in Figure 4 and Figure 6. Similar to the result when K=7dB, averaging in both time and frequency domain yields the best spectral efficiency performance. In addition, it is clear from the figures that equal or unequal density configuration do not make a big difference in the performance. Thus, as mentioned in our companion contribution [2], for ease of signaling, the equal density configuration is recommended.

Observation 1: In the high speed train scenario, equal or unequal density of DMRS configuration does not have significant impact on the spectral efficiency performance

Figure 1 Time and frequency domain interpolation, K=7dB

Figure 2 Time domain interpolation, Frequency domain averaging, K=7dB

Figure 3 Time and Frequency domain averaging, K=7dB

Figure 4 Time and frequency domain interpolation, K=13.3dB

Figure 5 Time domain interpolation, Frequency domain averaging, K=13.3dB

Figure 6 Time and Frequency domain averaging, K=13.3dB

Evaluation results: density reduction

In this section, link level evaluation results are presented to demonstrate the performance of the proposed reduced density DMRS design. For convenience, the proposed reduced DMRS density design is shown in Figure 13 in the appendix. In Figure 7, throughput performance at 4GHz of both reduced and full density DMRS configurations are shown. The simulation assumption for the link level simulation is shown in Table 2. In Figure 7, small degradation in the throughput performance is observed at higher CNR by reducing the density to 50%. In Figure 8 and Figure 9, throughput performances at 30GHz for modulation up to 64QAM and 256QM respectively, are shown. It is clear from the figures that similar to the 4GHz evaluation result, small degradation is observed at higher CNR.

At 30GHz, small performance degradation is observed when up to 64QAM is used in the evaluation. The difference in performance between d=1 and d=1/2 is noticeable at higher order modulation, i.e., 256QAM. These degradations are due to selectivity in the channel. Different patterns from the one considered in our companion contribution [2] can be considered for performance improvement.

Observation 2: With the distributed DMRS configuration, degradation in throughput performance is observed at high CNR

Figure 7 Throughput performance of reduced density DMRS, 4GHz

Figure 8 Throughput performance of reduced density DMRS with modulation up to 64QAM, 30GHz

Figure 9 Throughput performance of reduced density DMRS with modulation up to 256QAM, 30GHz

Conclusion

Observation 1: In the high speed train scenario, equal or unequal density of DMRS configuration does not have significant impact on the spectral efficiency performance

Observation 2: With the distributed DMRS configuration, degradation in throughput performance is observed at high CNR

In this contribution, the following proposal is made.

Proposal 1: Consider the equal DMRS density design

Proposal 2: Consider multiple RB designs for reduced DMRS density

- References

- [1]	Draft Minutes Report of 3GPP TSG RAN WG1 #88bis, Spokane, USA, Apr. 2017

- [2]	R1-1709023, “DMRS designs for NR MIMO: views on density reduction for DMRS,” Mitsubishi Electric, Hangzhou, China, May, 2017.

- [3]	R1-168174, “WF on CDL-D model for high speed train scenario at 30GHz for RRH and relay,” Mitsubishi Electric, ETRI, Ericsson, 3GPP RAN1#86, Göteborg, Sweden, Aug., 2016

- Appendix

| Figure 10 Equal density, 12 DMRS symbols over 2 OFDM symbols (Equal density 1) | Figure 11 Equal density : 12 DMRS symbols over 4 OFDM symbols (Equal density 2) |
| --- | --- |

| Figure 12 Unequal density, 12 DMRS symbols over 4 OFDM symbols (Unequal density 1) | Figure 13 Unequal density : 12 DMRS symbols over 3 OFDM symbols (Unequal density 2) |
| --- | --- |

Figure 14 Reduced DMRS density configuration with d=1/2

Table 1 Evaluation assumptions for equal vs unequal density in high speed train scenario

| FEC, modulation | Turbo code, 256QAM, 64QAM, 16QAM, QPSK |
| --- | --- |
| Channel estimation | OFDM: Estimated with RS |
| Channel model | CDL-D shown in [3], (ASD, ASA, ZSA, ZSD)=(5, 15, 1, 5), K-factor=7.0dB, 13.3dB |
| Center frequency, simulation BW, carrier separation | 30GHz, 80MHz, 60kHz |
| Relay and RRH antenna elements | 128 |
| Antenna architecture for RRH and relay | (8,16,1,1,1) |
| Moving direction of the train | Moving at direction of ϕ=0o, θ=90o |
| RRH antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max= 8 dBi |
| Relay antenna element radiation pattern | AE,Vθ''=-min12θ''-900θ3dB2,SLAV, θ3dB=650, SLAV=30 AE,Hφ''=-min12φ''φ3dB2,Am, φ3dB=650, Am=30 A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am GE,max=8 dBi |
| Phase noise model | BS model in R1-164041 |
| AFC | CP is used for AFC for OFDM |
| Interpolation | Time and frequency domain averaging, time and frequency domain interpolation, frequency domain averaging and time domain interpolation |
| Number of data symbols, IDFT | 1200, 2048 samples |
| CP size for OFDM and DFT-s-OFDM | 144 samples |
| Maximum speed | 600 km/h |
| Number of OFDM symbols | 14 |

Table 2 Evaluation assumptions for DMRS reduced density reduction, link level simulation, 4GHz

| FEC, modulation | Turbo code, QPSK, 16QAM, 64QAM, 256QAM |
| --- | --- |
| Channel estimation | OFDM: Estimated with DMRS |
| Channel model | CDL-B DS=30ns |
| Center frequency, simulation BW, carrier separation | 4GHz, 80MHz, 60kHz |
| Mobility | 30 km/hr |
| Number of antennas at TRP | 16 |
| Number of antennas at UE | 4 |

Table 3 Evaluation assumptions for DMRS reduced density reduction, link level simulation, 30GHz

| FEC | Turbo code, QPSK, 16QAM, 64QAM, 256QAM |
| --- | --- |
| Channel estimation | OFDM: Estimated with DMRS |
| Channel model | CDL-B DS=30ns |
| Center frequency, simulation BW, carrier separation | 30GHz, 80MHz, 60kHz |
| Mobility | 3 km/hr |
| Antenna architecture for BS | (8,32,2,1,1) |
| Antenna architecture for UE | (8,4,1,1,2) |